THE COUNCIL RECOGNISING that there is an increase in new and emerging uses of ocean and coastal waters, subsoil and seabed, an increase of seaborne trade, increasing demands of energy resources, increasing recreational use and increasing pollution threats from both ocean uses and an expanding of coastal population.

RECOGNISING ALSO the number and types of man-made structures being built in the maritime environment are increasing.

RECOGNISING ALSO the need to provide consistency in marking different types of offshore structures which may be a danger to navigation,

RECOGNISING ALSO that it is a matter for a National Authority to decide on whether a man-made structure needs to be marked, depending on the risk involved an the level of traffic

RECOGNISING ALSO that IMO Resolution A.672(16), dated 6th December 1989, established Guidelines and Standards for the Removal of Offshore Installations and Structures on the Continental Shelf and in the Exclusive Economic Zone, which incorporate requirements for such installations and structures, whilst being removed, to be marked in accordance with IALA Recommendations.

RECOGNISING FURTHER that marking is to improve the safety to navigation and protect the structures themselves,

HAVING CONSIDERED the proposals by the IALA ANM Committee,

ADOPTS the recommendation on the Marking of Man-Made Structures as set out in the following Sections.

THE COUNCIL, NOTING that the content of recommendations for marking as outlined in IALA Recommendations, O-114, O-116, O-117 and O-131 (titels) have been consolidated as separate sections within this Recommendation,

RECOMMENDS that Members ensure that the marking of man-made structures conforms to the standards and practices specified in the following Sections to this recommendation and

REVOKES IALA Recommendations, O-114, O-116, O-117 and O-131

- General considerations

There has been an increasing development in man-made structures at sea, which may affect shipping.  These structures can be: isolated or in groups, close or far away from the shore, small or big-sized, and close or far apart from commercial navigation zones.

IALA is monitoring the developments of these structures and will continue to create and update documentation as required to ensure clear and unambiguous marking of waterways for safe navigation, protection of the environment and protection of the structures themselves.  Authorities facing problems in this field are invited to bring them to the attention of IALA to obtain advice on current practice.

An Inventory of Man-Made Offshore Structures can be found in Annex I.

The following sections of this Document detail the updated IALA Recommendations for the marking of each offshore structure considered in Annex I.

Section 5 draws attention to the need for the basic marking to be reviewed from time to time.

The marking of offshore structures should be implemented using the following recommendations as a minimum requirement.

  - Scope

This recommendations are for the guidance of stakeholders such as National Administrations, Lighthouse Authorities, Aviation Authorities and other competent Authorities, Aids to Navigation providers, and the Contractors and Developers involved in each type of the structures to be mentioned in the following sections.

  - Field of Application

These Recommendations apply to all structures fixed in position temporarily or permanently which extend above the surface of the sea and which are obstructions to navigation, e.g. structures used for drilling or exploring for minerals, oil production platforms, oil well protective jackets or ocean data platforms.  Structures specially established as aids to navigation are excluded.

  - Information

Responsible authorities must ensure that shipping will be informed of all marking in accordance with these Recommendations, promulgated through the proper navigational publications and by broadcast navigational warnings where applicable.

- marking of offshore STRUCTURES

As it becomes evident in Annex 1 Man Made Offshore structures present very different characteristics between each other. Therefore for the sake of the signalization these structures they have been grouped according the object that fulfils the structure. Therefore in section 2.2 is presented the suggested marking for Aquaculture Farms; in Section 2.3 is presented the suggested marking for Wind farms; in Section 2.4 is presented the suggested marking for Wave and Tidal Energy Devices. All other Man Made Offshore Structures listed in Annex 1 should be marked according Section 2.1 as Offshore Structures in General

It is important to consider that marking of offshore structures is important during the different phases of their lives, i.e. construction of the structure, operation and also when the operation has finished and the structure remains an hazard to navigation.

  - MARKING OF OFFSHORE STRUCTURES IN GENERAL

    - General

Offshore Structures in General are those that can be found in Annex I and are not included in Sections 2.2 to 2.4

Consultation between the stakeholders such as Developers, National Administrations,  Aids to Navigation Authorities, Competent Authorities and Contractors should take place at an early stage.  In general, development of all structures mentioned in this Section should not prejudice the safe use of Traffic Separation Schemes, Inshore Traffic Zones, recognised sea lanes and safe access to anchorages, harbours and places of refuge. On a case-by-case basis, National Authorities may consider establishing Exclusion or Safety Zones, which would prohibit or restrict vessels from entering groups of Offshore Structures in General.  Such information should be shown on the navigation chart, as appropriate.

    - Marking

The Offshore Structures mentioned in this Section should be marked as a single unit or a block or field as follows:

- Any structure shall be marked at night by one or more white lights so constructed and fixed as to ensure that at least one light is visible upon approaching the structure from any direction.

- The lights shall be placed not less than 6m and not more than 30m above MHWS with a minimum effective intensity of 1400 candelas. The lights shall be operated in unison with a flashing character according to Morse letter « U » (    ) and with a maximum period of 15 seconds. The vertical distribution of the projected beam shall be such that the light will be visible from the immediate vicinity of the structure to the maximum luminous range of the light.

- The horizontal and vertical extremities of the structure shall be adequately marked in a manner determined by the Authority and in conformity with the requirements of air navigation regulations.

- Each structure shall, where practicable, display identification panels with black letters or numbers 1 m high on a yellow background visible in all directions. These panels shall be easily visible in daylight as well as at night, either by the use of illumination or retro-reflecting material.

- Each structure shall carry one or more sound signals so constructed and fixed as to be audible upon approaching the structure from any direction.

- The sound signals shall be placed not less than 6m and not more than 30m above MHWS with an IALA usual range of at least 2 miles.  The character shall be rhythmic blasts corresponding to Morse letter « U » (    ) every 30 seconds.

- The minimum duration of the short blast shall be 0.75 seconds. The sound signals shall be operated when the meteorological visibility is two miles or less.

- Where there is a requirement to identify a particular structure, a radar beacon may be fitted. The range and code shall be determined by the authority. Any radar beacon on a temporary uncharted structure shall be coded D (     ).

- Where a number of structures are situated so that the safety of navigation in the area may be secured without each of the structures being individually equipped with lights and sound signals in accordance with these recommendations, or where the Authority considers that local conditions permit a relaxation of the requirements for the intensity of the light, the Authority shall determine what marking shall be applied.

- Wherever deemed necessary by the authority, buoys or beacons shall be placed to mark the perimeter of a group of structures, or to mark channels through a group of structures, or to mark any fixed structure while being erected or dismantled. The characteristics of such marks shall be determined by the Authority in accordance with the IALA Maritime Buoyage System.

- Where underwater obstructions, such as submerged wells or pipelines, exist in depths of water so as to be a hazard to surface borne vessels, or are otherwise required, they should be adequately marked in accordance with the IALA Maritime Buoyage System.

    - Considerations During Construction / Decommissioning

During the construction / decommissioning of Offshore Structures in General, working areas should be established and marked in accordance with the IALA Maritime Buoyage System (MBS).  National Authorities should also consider the use of guard ships in areas of high traffic density.

Notices to Mariners, Radio Navigational Warnings – NAVTEX and/or broadcast warnings must be promulgated in advance of and during any offshore wave and/or tidal energy extraction device construction.

Where individual Offshore Structures in General extend above the surface, careful consideration needs to be given to any additional temporary marking that may be required during the construction / decommissioning phase.

When decommissioning such devices, the Authority should ensure that the operator / contractor is obliged to remove all obstruction so the sea bed is returned to its original depth and topography. In the event that any residue or obstruction remains that, in the opinion of the Aids to Navigation Authority, constitutes a danger to navigation, then the residue or obstruction shall be marked according to the authority’s requirements.

    - Additional marking

The authority may, in special cases due to navigational reasons, require additional marking equipment such as:

- A high intensity light with specific character

- Maritime radio beacon with specially determined range, frequency and character.

- Radar beacon with specific range and character

- Buoys in accordance with the IALA Maritime Buoyage System

  - MARKING OF OFFSHORE AQUACULTURE FARMS

    - General

Aquaculture farms are defined in Annex I. When an aquaculture farm/s is considered to represent a danger to navigation,  it should be marked in accordance with the IALA Maritime Buoyage System, using special marks, lateral or cardinal marks, or a combination thereof.  The use of electronic AtoNs such as racons or AIS may also be considered. The farm (or group of farms) should be marked depending on their size, extent and location.  In some cases, it may be sufficient to mark only part of the perimeter or the centre.

Administrations should bear in mind that the marking recommendations herein may be adjusted considering traffic density, proximity to ports, proximity to dangers, tidal considerations and other factors.

  - Aquaculture farms are normally marked by special marks.

  - If there is a requirement for vessel traffic between aquaculture farms, then such a channel should normally be marked with lateral marks.

  - If the prevailing situation warrants it, cardinal marking alone may be used to direct mariners away from the aquaculture farm.

  - To improve the effectiveness of the lighting, and taking into consideration background lighting, synchronisation of the various lights should be considered.

  - To improve the radar target and the visibility of the aquaculture farm, radar reflectors and reflective material should be considered.

Examples can be found in Section 3.2.2. The marking denoted in those examples is the minimum recommended marking arrangement with special marks

    - Marking Examples

Aquaculture farms can be of various regular and irregular shapes and sizes but are usually either Rectangular or Circular.

      - Rectangular aquaculture farms

Should be marked according to the length of the sides.

Example 1

Size:	side A and side B    500 metres.

Minimum Marking Arrangement:

A light in the middle of the aquaculture farm.

A

B

Example 2

Size:

side A (sea side and coast side)    2500 metres

side B (lateral side)    500 metres.

Minimum Marking Arrangement:

A light on each sea corner and a daymark on each coast corner.

A

B

Example 3

Size:

side A (sea side and coast side)    500 metres

side B (lateral side)    2500 metres.

Minimum Marking Arrangement:

A light on one of the sea corners, a light on the diagonally opposite coast corner and daymarks on the other two corners.

A

B

Example 4

Size:	side A and side B  500 and    2500 metres
such as A  B ≤ 2.250.000 metres2

Minimum Marking Arrangement:

Lights on diagonally opposite corners and daymarks on the other two corners.

A

A

B

Example 5

Size:	side A and side B  900 and    2.500 metres
such as A  B  2.250.000 metres2 and  6.250.000 metres2

Minimum Marking Arrangement:

A light at each corner of the aquaculture farm.

A

B

      - Circular aquaculture farms

Should be marked according to the length of their radius.

Example 1

Size:			radius    250 metres.

Minimum Marking Arrangement:

A light in the centre of the aquaculture farm.

Example 2

Size:			radius   250 and    1000 metres.

Minimum Marking Arrangement:

Two lights, 180 degrees apart on the circumference of the aquaculture farm and two daymarks on the circumference positioned 90 degrees to the lights.

Example 3

Size:		radius   1000 and    2000 metres.

Minimum Marking Arrangement:

Three lights, 120 degrees apart on the circumference of the aquaculture farm.

120°

Example 4

Size:		radius   2000 metres.

Minimum Marking Arrangement:

Three lights, 120 degrees apart on the circumference of the aquaculture farm and three daymarks on the circumference positioned 60 degrees to the lights.

 60°

  - MARKING OF OFFSHORE WINDFARMS

    - General

When mentioning Windfarms, the following are included: Meteorological Mast, Wind Generator and Wind Farm, as defined in Annex I.

Consultation between the stakeholders such as Developers, National Administrations, Lighthouse Authorities, Aviation Authorities, AtoN providers, Competent Authorities and wind farm contractors and developers should take place at an early stage.  In general, development of offshore energy structures or wind farms should not prejudice the safe use of Traffic Separation Schemes, Inshore Traffic Zones, recognised sea lanes and safe access to anchorages, harbours and places of refuge.  On a case-by-case basis, National Authorities may consider establishing Exclusion or Safety Zones, which would prohibit or restrict vessels from entering wind farms.  Such information should be shown on the navigation chart, as appropriate.

In order to avoid confusion from a proliferation of Aids to Navigation in a high-density wind farm, full consideration should be given to the use of synchronised lighting, different light characters and varied light ranges.

Some IALA members have carried out trials on wind farms to identify if interference to radar, radio navigation and radio communications is experienced.  Trials indicate that wind farm structures affect shipborne and shorebased radar systems.  This interference returned radar responses strong enough to produce interfering side lobe, multiple and reflected echoes.  Bearing discrimination was also reduced by the magnitude of the response.  It has been determined that passage close to a wind farm boundary, or within the wind farm itself, could affect the vessel’s ability to fully comply with the International Regulations for the Prevention of Collisions at Sea.  Administrations / developers should keep this information in mind when designing wind farms, and they may wish to carry out individual trials to verify the impact of the wind farm on navigation.

There has been some evidence that scouring at the bases of wind generators in areas of strong tides or currents has resulted in significant deposits of material in other locations.  Some authorities have insisted on fitting depth monitoring devices to wind generators to measure scour.  This may need to be considered when approving wind farm proposals/locations.

    - Marking of Individual Structures (Wind Turbines)

The tower of every wind generator should be painted yellow all round from the level of Highest Astronomical Tide (HAT) to 15 metres or the height of the Aid to Navigation, if fitted, whichever is greater.

Alternative marking may include horizontal yellow bands of not less than 2 metres in height and separation.

Consideration may be given to the use of additional retro reflective material.

Due to the increased danger posed by an isolated structure, it should be lighted with a white light flashing Morse code "U".

      - Aids to Navigation for marking Individual Structures

The Aids to Navigation on the structure of a wind generator should be mounted below the lowest point of the arc of the rotor blades.  They should be exhibited at a height above the level of the Highest Astronomical Tide (HAT) of not less than 6 metres or more than 15 metres.  Aids to Navigation on wind turbines should comply with IALA Recommendations and have an availability of not less than 99.0% (IALA Category 2).

Sample marking of individual wind turbines

HAT

TT

15 metres

HAT

15 metres

Height of AtoN,

if fitted

    - Marking of Groups of Structures (Wind Farms)

A Significant Peripheral Structure (SPS) is the “corner” or other significant point on the periphery of the wind farm.  Every individual SPS should be fitted with lights visible from all directions in the horizontal plane.  These lights should be synchronized to display an IALA “special mark” characteristic, flashing yellow, with a range of not less than five (5) nautical miles.

As a minimum, lights on individual SPSs should exhibit synchronised flashing characteristics, however Administrations should consider the synchronisation of all SPSs.  In the case of a large or extended wind farm, the distance between SPSs should not normally exceed three (3) nautical miles.

Selected intermediate structures on the periphery of a wind farm other than the SPSs, should be marked with flashing yellow lights which are visible to the mariner from all directions in the horizontal plane.  The flash character of these lights should be distinctly different from those displayed on the SPSs, with a range of not less than two (2) nautical miles.  The lateral distance between such lit structures or the nearest SPS should not exceed two (2) nautical miles.

      - Aids to Navigation for marking Wind Farms

In addition to the use of lights for marking the SPSs and selected intermediate peripheral structures of a wind farm, further consideration should be given to the use of:

- Lighting all peripheral structures;

- Lighting all structures within the wind farm;

- Racons;

- Radar Reflectors and Radar Target Enhancers; and/or

- AIS as an Aid to Navigation (as per IALA Recommendation A-126).

It is important that these AtoNs be used with care to mark the grouping of wind generators.

Consideration may be given to the provision of sound signals where appropriate, taking into account the prevailing visibility, topography and vessel traffic conditions.  The typical range of such a sound signal should not be less than two (2) nautical miles.

Sample marking of wind farm

SPS - lights visible from all directions in the horizontal plane.  These lights should be synchronized to display an IALA “special mark” characteristic, flashing yellow, with a range of not less than five (5) nautical miles

Intermediate structures on the periphery of a wind farm other than the SPSs - marked with flashing yellow lights which are visible to the mariner from all directions in the horizontal plane with a flash character distinctly different from those displayed on the SPSs and with a range of not less than two (2) nautical miles

< 2 nm

< 3 nm

    - Considerations During Construction

During the construction of an offshore wind farm, working areas should be established and marked in accordance with the IALA Maritime Buoyage System (MBS).  National Authorities should also consider the use of guard ships in areas of high traffic density.

Notices to Mariners, Radio Navigational Warnings and Notices to Airmen must be promulgated in advance of and during any offshore wind farm construction.

Power cables between wind generators, between wind generators and the transformer station, and between the transformer station and the shore should be sufficiently trenched to avoid exposure from scouring / sand migration or trawling activities.

    - Additional Considerations

Depending on the marking, lighting and lateral separation of the peripheral structures, the additional marking of the individual structures within a wind farm may be considered as follows:

- Lighting of each structure

- Individual structures unlighted with retro-reflective areas.

- Individual structures illuminated with down-lights on ladders and access platforms.

- Use of flashing yellow lights with a range of not less than two (2) nautical miles.

- Identifying numbers on each individual structure, either lit or unlit.

An Electrical transformer station or a meteorological or wind measuring mast, if considered to be a composite part of the wind farm, should be included as part of the overall wind farm marking. If not considered to be within the wind farm block it should be marked as an offshore structure. (i.e. a white light flashing Morse code "U").

As far as practicable, Aeronautical obstruction warning lights fitted to the tops of wind generators should not be visible below the horizontal plane of these lights.  Aviation Authorities should be consulted regarding the specification of such lights.

  - MARKING OF OFFSHORE WAVE AND TIDAL ENERGY DEVICES

    - General

Wave and Tidal Energy Devices include: Tidal Generator, Tidal Generator field, Wave Generator, Wave Generator field, as defined in Annex I.

In general, any risk assessment of offshore wave and tidal extraction devices will likely determine that, when compared to gas and oil structures, there is a lower risk of either pollution or loss of life should a vessel foul such an installation. Consequently, the marking requirements can be mitigated.  It should be born in mind that many wave and tidal devices are low freeboard floating structures that are moored to the seabed. They may be moored in deep or shallow water and some may be located on the seabed or just below the surface.  Surface piercing and subsurface elements may extend laterally beyond the surface elements.  This could include shared moorings and mid-water connections between units which may also carry electricity, control signals, hydraulics or pneumatics associated with the units. It should also be noted that many tidal concepts have fast-moving sub-surface elements such as whirling blades, and these should be taken into account when identifying the marking requirements.

Consultation between the stakeholders such as Developers, National Administrations,  Aids to Navigation Authorities, Competent Authorities and wave and tidal contractors should take place at an early stage.  In general, development of offshore energy structures should not prejudice the safe use of Traffic Separation Schemes, Inshore Traffic Zones, recognised sea lanes and safe access to anchorages, harbours and places of refuge. On a case-by-case basis, National Authorities may consider establishing Exclusion or Safety Zones, which would prohibit or restrict vessels from entering Offshore Wave and Tidal Energy fields.  Such information should be shown on the navigation chart, as appropriate.

In order to avoid confusion from a proliferation of Aids to Navigation in a high-density wave and tidal energy extraction field, full consideration should be given to the use of synchronised lighting, different light characters and varied light ranges.

There has been some evidence that sea-bed scouring at the bases of offshore renewable energy installations in areas of strong tides or currents has resulted in significant deposits of material in other locations.  Some authorities have insisted on fitting depth monitoring devices to such installations to measure scour.  This may need to be considered when approving wave and tidal energy extraction proposals/locations.

    - Marking

Wave and Tidal energy extraction devices should be marked as a single unit or as a block or field as follows:

- When structures are fixed to the seabed and extend above the surface, they should be marked in accordance with the recommendations contained in Section 3.3, Marking of Offshore Windfarms.

- Areas containing surface or sub-surface energy extraction devices (wave and/or tidal) should be marked by appropriate navigation buoys in accordance with the IALA Buoyage System, fitted with the corresponding topmarks and lights. In addition, active or passive radar reflectors, retro reflecting material, racons and/or AIS transponders should be fitted as the level of traffic and degree of risk requires.

- The boundaries of the wave and tidal energy extraction field should be marked by Navigational Lighted Buoys, so as to be visible to the Mariner from all relevant directions in the horizontal plane, by day and by night. Taking the results of a risk assessment into account, lights should have a nominal range of at least 5 (five) nautical miles. The Northerly, Easterly, Southerly and Westerly boundaries should normally be marked with the appropriate IALA Cardinal mark. However, depending on the shape and size of the field, there may be a need to deploy lateral or special marks.

- In the case of a large or extended energy extraction field, the distance between navigation buoys that mark the boundary should not normally exceed 3 (three) nautical miles.

- Taking into account environmental considerations, individual wave and tidal energy devices within a field which extend above the surface should be painted yellow above the waterline.  Depending on the boundary marking, individual devices within the field need not be marked. However, if marked, they should have flashing yellow lights so as to be visible to the mariner from all relevant directions in the horizontal plane. The flash character of such lights should be sufficiently different from those displayed on the boundary lights with a range of not less than 2 nautical miles.

- Consideration should be given to the provision of AIS as an Aid to Navigation (IALA Recommendation A-126) on selected peripheral wave and/or tidal energy devices.

- A single wave and/or tidal energy extraction structure, standing alone, that extends above the surface should be painted black, with red horizontal bands, and should be marked as an Isolated Danger as described in the IALA Maritime Buoyage System.

- In the case of a single wave and/or tidal energy device which is not visible above the surface but is considered to be a hazard to surface navigation, it should be marked by an IALA special mark yellow buoy with flashing yellow light with a range of not less than 5 nautical miles, in accordance with the IALA Buoyage System.  It should also be noted that many tidal concepts have fast-moving sub-surface elements such as whirling blades.

- The Aids to Navigation described herein should comply with IALA Recommendations and have an appropriate availability, normally not less than 99.0% (IALA Category 2).

- The relevant Hydrographic Office should be informed of the establishment of an energy extraction device or field, to permit appropriate charting of same.

- Notices to Mariners should be issued to publicise the establishment of a wave and/or tidal energy device or field.  The Notice to Mariners should include the marking, location and extent of such devices/fields.

    - Considerations During Construction / Decommissioning

During the construction / decommissioning of an offshore wave and/or tidal energy extraction device or field, working areas should be established and marked in accordance with the IALA Maritime Buoyage System (MBS).  National Authorities should also consider the use of guard ships in areas of high traffic density.

Notices to Mariners, Radio Navigational Warnings – NAVTEX and/or broadcast warnings must be promulgated in advance of and during any offshore wave and/or tidal energy extraction device construction.

During construction, power cables between wave and tidal generators, between such generators and the transformer station, and between the transformer station and the shore should be sufficiently trenched to avoid exposure from scouring / sand migration or trawling activities.

Where individual wave and/or tidal energy devices extend above the surface careful consideration needs to be given to any additional temporary marking that may be required during the construction / decommissioning phase.

When decommissioning such devices, the Authority should ensure that the operator / contractor is obliged to remove all obstruction so the sea bed is returned to its original depth and topography. In the event that any residue or obstruction remains that, in the opinion of the Aids to Navigation Authority, constitutes a danger to navigation, then the residue or obstruction shall be marked according to the authority’s requirements.

    - Contingency Plans

- Operators of wave and/or tidal energy extraction devices or fields should develop contingency plans and emergency response plans which address the possibility of individual devices breaking loose and becoming floating hazards. Automatic location and tracking devices should be considered.

- Developers and/or operators should have a reliable maintenance and casualty response regime in place to ensure the required availability targets are met. This will include having the necessary AtoN spares on hand, with provision made at the design stage, where necessary, to ensure safe access.

    - Additional Considerations

Depending on the marking, lighting and lateral separation of the field boundary, the additional marking of the individual structures within an energy extraction field, visible above the surface of the sea, may be considered as follows:

- Lighting of each structure.

- Individual structures unlighted with retro-reflective areas.

- Individual structures illuminated with down-lights on ladders and access platforms.

- Use of flashing yellow lights with a range of not less than two (2) nautical miles.

- Identifying numbers on each individual structure, either lit or unlit.

An electrical transformer station or other structure, if considered to be a composite part of the energy extraction field, should be included as part of the overall marking. If not considered to be within the boundaries of the field, it should be marked as a single stand alone device as described in Section 3.4.2 (6 and 7 refers).

- FUTURE REQUIREMENTS

IALA is aware that there are an increasing number of structures which may affect shipping.

It is therefore recommended that Authorities should continuously monitor these Recommendations to ensure that any navigational problems caused by offshore structures are solved in a satisfactory manner.

Authorities facing problems in this field are invited to bring them to the attention of IALA to obtain advice based on current practice.

IALA foresees the need to re-examine these Recommendations from time to time, and if necessary to update them.

- glossary

Appropriate National Authority: the proper National Authority for determining the marking of offshore structures – hereafter referred to as the Authority.

Effective intensity – the effective intensity of rhythmic lights shall be calculated in accordance with the IALA « Recommendations for the calculation of the effective intensity of a rhythmic light », November 1980, using the Schmidt-Clausen method of calculation, where this is applicable for the apparatus concerned.

Energy Extraction Device (EED)  -  a wave or tidal generator.

Mean High Water Springs – hereafter referred to as MHWS.

Nautical mile – 1852m – hereafter referred to as a mile.

Significant Peripheral Structure (SPS) -  the “corner” wind generator on a rectangular wind farm or other significant point on the periphery of a wind farm.

Transformer Station (hub) - a special structure within or outside the wave and/or tidal energy extraction field and/or wind farm to which the individual generators are connected via a power cable. Power is transferred ashore from the transformer station by submarine cable.  A ‘hub’ may be a separate fixed or floating platform, a unit very similar to the generators but carrying additional power conversion equipment.

Usual range – the usual range of the fog signal shall be calculated in accordance with IALA Recommendation E-109, on the calculation of the range of a sound signal, dated May, 1998.