German Federal Waterways Administration

ANM12/6/1

Traffic Technologies Centre

Frank Hermann

Input to IALA Document ANM11/WG1/task1

‘draft Guideline (Manual) on a minimum comprehensive mix of AtoN in fairways/waterways including dredged channels and canals’

Remarks on 3.3.1 Buoys and Beacons / page 12

starting with ‘The observation distance ... ‘

ending with ‘Night time visibility, nominal range, tbd’

1.	General remarks

The German Traffic Technologies Centre has done some investigations on the visual performance of lights and daymarks. The results were drawn into IALA EEP committee and IALA expert group on lights (see EEP 10 and EEP 11).

Some of the results will be used in an ‘IALA Guideline On Daymarks For Floating Aids To Navigation’, which is currently in a very early status. In addition there will be work on the visual perception of lights including effective intensity and conspicuity in IALA EEP or sub groups.

Three Qualities of visual perception

(1) Object is visible (Pure visibility)

The buoy can be seen but appears still too small to be recognized as a certain type of buoy according to IALA buoyage system.

(2) Object can be identified (identification): The buoy can be identified to be a certain type of buoy of the IALA buoyage system.

(3) Object is conspicuous (Conspicuity): The buoy appears outstanding in a complex visual scene.

To work properly as an aids-to-navigation it is necessary that the buoy satisfies the requirement (2) ‘Identification’.

2.	Daytime

At daytime the identification of a buoy according to IALA buoyage system is linked to the visual information below:

(A) Colour

(B) Shape (simple shapes: Cone, cylinder, ball / arrangement of simple shapes)

(C) Combination of Colour and Shape (e.g. stripes of different colours)

To describe the visual performance it becomes evident to distinguish between a visible (1) and an identified (2) object.

In the draft it is stated that ‘The observation distance at daytime depends on the dimensions of the buoy.’

The term ‘observation distance’ is not defined according to the visual performance of the object.

Further on the undefined distance is stated to be dependent on the size of the buoy only. This is not true.

The visual performance and so any kind of a ‘maximum useful observation distance’ depends on: size, luminance factor, shape, colour, background, meteorological and geographical conditions. This can already be found in the draft ‘IALA Guideline On Daymarks For Floating Aids To Navigation’ (EEP11).

3. 	Nighttime

The nominal luminous range is a historical term. However it has to be supported for the description of lights (industrial products, list of lights, charts).

The nominal luminous range is based on a meteorological visibility of 10 M.

In general the nominal luminous range is not the design parameter for a light (see draft IALA-Rec E200-2 / EEP 11 input).

Example:

The lighted buoys of Germany are based on a meteorological visibility of 4.3 M. The luminous intensity is therefore up to 8-times higher compared to a design visibility of 10 M.

For fixed lights the design visibility is even lower in Germany. Values between 2.2 M and 4.3 M are used depending on the region.

In the current discussion the influence of background illumination and rival lights is considered to maintain a sufficient visual performance (‘conspicuity models’).

In many cases the only way to maintain the required performance is to increase intensity even more.

In contrast to this the use of the nominal luminous range to design a light would reduce the visual performance severely.

4.	Conclusion

There is a conflict between the statements in 3.3.1 of the draft (ANM) and the results on the quality of visual perception already available (EEP).

In detail the formula Ld = ~ H / tan 3’ cannot be used to describe the visual performance of a buoy at daytime and the nominal luminous range cannot be used as a design parameter of a light at night time.

I recommend a further discussion between the members of ANM and EEP involved in this issue. To start a discussion I recommend to provide definitions to make the discussion clear. Possible definitions are ‘observation distance’, ‘maximum useful observation distance’, ‘Daytime range’, ‘design parameter’, ‘concurrent or rival lights’.

Frank Hermann

German Traffic Technologies Centre