Potential causes of interference with GPS Synchronisation

- Test the almanac theory and check the GPS antenna to ensure it is not being saturated/affected by external radio transmissions

- Check for problems in the firmware by testing the GPS unit with the lantern and not just by testing the GPS unit alone.

- Firmware bugs can prevent the lanterns working properly with the GPS unit and also possible algorithm issues.

- The GPS units might not be acquiring the satellites and not receiving a time stamp. This could be caused by shielding or marginal visibility of the antenna.

- It is normal for the GPS sync to occur periodically in order to save energy. It can take up to 2 minutes for the GPS unit to acquire the time signal and if the GPS units do not acquire the time signal within the given slot the lights will not sync.  The period needed to acquire the time signal can vary and may be the reason the lights are going in and out of sync. When the GPS is turned on it will take considerably more time to acquire data as it may be downloading updated almanacs

- The problem may not be the same for each manufacturer but is likely to be a combination of application, design, and software.

- High wattage lamps can introduce noise on the Pulse Per Second signal which retriggers the timing at wrong times. A filter on the PPS line should fix this problem

- The Trimble antenna can lose it's settings and revert back to it's default settings, which might not have the Seconds of the Week data needed for synchronization.  Upgrading the firmware in the Trimble antenna should fix this problem.

- Radio transmission in close proximity of the lanterns could interfere with the GPS receiver and cause loss of satellites.

- Some software designs accept time data when at least one satellite is seen whereas other manufacturers wait for 3 or more satellites before accepting the time data. Problems can occur if only part of the sky is seen by the antenna (as in bridge lights) or if nearby radio transmissions cause interference.

- Ships running S-band radar can interfere with the GPS signal which is a problem with all GPS receivers used for any purpose. In some instances GPS receivers have been ruined by the strong RF signal from S-Band radars. All new shipboard IMO-GPS receivers have additional filtering to alleviate this problem. New receivers being used for other GPS uses are now being filtered to address this problem.

Typical problems include:

1) a ship comes into port running S-Band radar and disturbs the GPS receivers. GPS returns to normal when ship docks and turns off radars. Some ports have made it a rule that ships must turn off S-Band radars when entering port for this reason.

2) a ship anchors near GPS Synchronised lights and leaves its S-Band radar running. Lights Synchronise normally when the ship leaves.

Note:

a) Strong S-Band radars have also been known to interrupt cell phone operation and destroy sensitive cell phone receivers. Service providers have added special filtering to cell phone towers installed near ports and waterways where S-Band radars would be used. This is another reason why S-Band radar usage has been banned in many ports.

b) The amount of metal surrounding the light has also been known to influence the receiver and some authorities have increased the height of their lanterns/structures.

c) It would also seem that some ships running closer than 20 meters to the lantern have a higher probability of setting it out of sync. The issues do not seem to appear in low traffic ports, on wooden structures, or at other installations such as fish farms.