IALA e-Nav Committee – eNAV 5 (15-19 Sept 08)

WG 1 – Working Paper

Priority Analysis for User Needs

DRAFT – Working Paper

Introduction

Under the draft framework for the implementation process for the e-Navigation strategy, the IMO have requested that “the initial user needs should be further reviewed and prioritized by 2009”.

WG1 therefore discussed approaches to the prioritisation of the high level user needs identified in the draft strategic plan (section 8). The WG commenced by conducting a ‘benefits analysis assessment’ based on the stated benefits of e-Navigation as listed in the same strategy (section 6). These benefits included those for improved safety; environmental protection; security and efficiency. This benefits analysis was further broken down by main user groups comprising: ship’s crews; VTS; SAR; pilots; administrations and shipowners.

The aim of the assessment was to identify how important the stated benefits of e-Navigation are to each stakeholder group, and what impact the implementation of e-Navigation will have for each stakeholder group upon realising these benefits.

Participants in this assessment were representative of a wide range of maritime stakeholder including those under review. Annex 1 contains the raw data from this assessment.

Findings:

- Prioritisation of needs without a rigid specification of attributes based on effect, such as cost or risk and resulted in a relatively subjective output. Wider consultation on the effects of the stated benefits would help reduce subjectivity.

- There are clear benefits for all stakeholders from the effective implementation of e-Navigation, although from different perspectives.

- All sectors of shipping stand to gain: All from improved safety and security; some will benefit from improvements in efficiency and effectiveness, including logistic management and emission reduction.

- It will soon be necessary to identify which stakeholders will be required to make investments in e-Navigation in relation to where the benefits will be achieved. It was noted however that much of the needed infrastructure investment, particularly in the early stages, could be achieved by reallocation from existing budgets.

Conclusions:

- It was noted that governments and authorities need to be kept informed as to the potential impact and requirements of e-Navigation so that advanced planning and budgetary issues can be addressed.

- Frontline operators such as ship’s crews, pilots, VTS and SAR personnel will benefit the most from operational improvements gained from effective and coordinated displays, streamlined data exchange and communications, and decision support systems.

- The possible benefits to administrations and shipowners will tend to benefit from the long term effects of fewer incidents and the resultant cost savings from reduced cost of insurance and incident response, environmental impact and the potential improvements in effective and efficient ship movements.

- The need for improved information flow and system integration between all stakeholders was recognised as a high priority, however system boundaries will need to be carefully managed and the implementation should be done in a phased approach. Improving the effectiveness of transmission and presentation of safety related information, such as Navtex navigational warnings, was considered a high priority.

- Improved decision support systems were considered paramount to reducing error and improving safety. Improvement of critical safety alarms and alerts, such as are being developed by the IMO IBS CG, were considered a fundamental starting point for reducing burdens and improving decision making.

- A common approach to navigation displays for ship’s crews and pilots, such as S-Mode, was considered to be a high priority to gain benefits from improved decision making based on more effective operation and from more effective training. Note that S-Mode is only envisioned for the ship environment.

- Aspects of having a Common Operating Picture (with consideration for the need for a User Defined Operating Picture) were considered a priority so as to realise improvements in the operation of VTS, SAR and incident response activities across a wide range of shore based authorities on a national, regional and international basis in support of safety, security and the protection of the marine environment. Note that COP and UDOP are only envisioned for the shore based environment.

- System integrity and resilience was seen as absolutely critical.

- e-Navigation has the potential to improve teamwork within a ship, shore facility, and across remote users through improved data exchange and communication techniques. However, this improvement would only be recognised if training and procedures were developed and implemented in conjunction with user needs.

Recommendations:

The following recommendations pertain to the prioritisation of user needs as stated and agreed in the IMO Draft Strategy for e-Navigation (section 8) (Annex II).

- Noting the long lead-time requirements on government agencies tasked with providing maritime and shore services (hydrographic surveys and charting, MSI and integrated coastal management, position fixing and communications infrastructure, etc…) , it is recommended that potential e-Navigation requirements for such services be communicated to them at an early stage through the relevant international organisations.

- The development of a “Common Maritime Information/Data Structure” is a high priority, as it forms an essential foundation upon which e-Navigation needs to be developed. A common maritime information/data structure will help facilitate effective data exchange and communication, improved decisions based on up to date information, and foster a sharing of information to improve domain awareness, risk assessment and response effectiveness.

- The transmission to vessels of Maritime Safety Information (MSI) should be integrated and presented on vessel’s navigation displays in a clear and unambiguous manner taking into account the best practice for graphical displays and the human element. Such information should be accessible via an appropriate navigation display (e.g. ECDIS, INS) and be integrated into an alert management system such as that being developed within the IMO INS standard.

- A phased approach to report transmissions between ship and shore, shore and ship and shore to shore should be established based on the goal of reducing the administrative workload to the operators from the frequency of transmitting routine messages.

- A phased approach to developing a common operating picture of the maritime domain awareness should be developed to promote improved safety, security and incident response across a national, regional and international basis.

- An S-Mode should be developed for shipboard displays to ensure that all ship’s crew and pilots are better able to execute critical tasks to support safe navigation..

Annex I

Benefits Analysis Assessment

| Benefits |  | Mariners | VTS | SAR | Pilots | Administrations | Ship owners |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Safety |  |  |  |  |  |  |  |
|  | Decision support | 4 | 4 | 5 | 4 | 2 | 4 |
|  | Indications/alerts | 4 | 4 | 2 | 2 | 2 | 1 |
|  | ENC coverage | 5 | 2 | 2 | 2 | 2 | 4 |
|  | S-Mode (or VTS common operating picture) | 5 | 3 | 5 | 5 | 4 | 5 |
|  | System resilience | 4 | 4 | 5 | 4 | 5 | 4 |
|  | Integration (ship/shore) | 3 | 5 | 5 | 4 | 5 | 3 |
| Protection of the environment |  |  |  |  |  |  |  |
|  | Reduce incidents | 4 | 4 | 2 | 4 | 5 | 4 |
|  | Reduce emissions | 2 | 2 | 1 | 1 | 4 | 5 |
|  | Incident response | 2 | 4 | 5 | 2 | 4 | 4 |
| Security |  |  |  |  |  |  |  |
|  | Passive shore domain awareness | 2 | 3 | 4 | 4 | 4 | 3 |
| Efficiency |  |  |  |  |  |  |  |
|  | Speed of type approval | 1 | 1 | 1 | 1 | 4 | 1 |
|  | Standard reporting | 5 | 5 | 2 | 3 | 4 | 5 |
|  | Better watchkeeping | 5 | 4 | 4 | 3 | 3 | 4 |
|  | Existing system integration (including equipment compatibility and interoperability (includes data exchange) and meeting of user requirements | 4 | 4 | 5 | 4 | 3 | 4 |
| Teamwork |  |  |  |  |  |  |  |
|  | Improved Human Resource Management (teamwork) | 4 | 4 | 4 | 4 | 3 | 3 |

The following rankings have been used to prioritise the potential benefits of e-Navigation when compared with the current general situation pertaining to the facilitation of safe navigation:

1. Low priority

2. Low to medium priority

3. Medium priority

4. Medium to high priority

5. High priority

Annex II

Extract from Nav 54/25 Annex 12

6 BENEFITS OF E-NAVIGATION

6.1 The main broad benefits of e-navigation are expected to be:

.1 improved safety, through promotion of standards in safe navigation supported by:

.1 improved decision support enabling the mariner and competent authorities ashore to select relevant unambiguous information pertinent to the prevailing circumstances;

.2 a reduction in human error through provision of automatic indicators, warnings and fail-safe methods;

.3 improved coverage and availability of consistent quality Electronic Navigational Charts (ENCs);

.4 introduction of standardized equipment with an S-Mode* option but without restricting the ability of manufacturers to innovate;

.5 enhanced navigation system resilience, leading to improved reliability and integrity; and

.6 better integration of ship and shore-based systems; leading to better utilization of all human resources;

.2 better protection of the environment both by:

.1 improving navigation safety as above, thereby reducing the risk of collisions and groundings and the associated spillages and pollution;

.2 reducing emissions by using optimum routes and speeds; and

.3 enhancement of ability and capacity in responding and handling of emergencies such as oil spills;

.3 augmented security by enabling silent operation mode for shore-based stakeholders for domain surveillance and monitoring;

.4 higher efficiency and reduced costs enabled by:

.1 global standardization and type approval of equipment augmented by a “fast track” change management process (in relation to technical standards for equipment);

.2 automated and standardized reporting procedures, leading to reduced administrative overhead;

.3 improved bridge efficiency allowing watch keepers to maximize time to keeping a proper lookout and embrace existing good practice, e.g., using more than one method to ascertain the ship's position; and

.4 integration of systems that are already in place, precipitating the efficient and coherent use of new equipment that meets all user requirements;

.5 improved human resource management by enhancing the experience and status of the bridge team.

7 BASIC REQUIREMENTS FOR THE IMPLEMENTATION AND OPERATION OF E-NAVIGATION

7.1 To attain these benefits, a number of basic requirements should be fulfilled as enablers to the implementation and operation of e-navigation. In particular:

.1 implementation of e-navigation should be based on user needs not technology-driven and over-reliance should not be placed on technology to avoid, for example:

.1 system failures causing delays because the ship is now deemed unseaworthy;

.2 loss of basic good seamanship by crews;

.3 inappropriate substitution of the human element by technology; and

.4 degradation of bridge resource management and best practices by the crew;

.2 operating procedures should be put in place and kept under review, most notably in relation to the human/machine interface, the training and development of mariners and the roles, responsibilities and accountabilities of ship- and shore-based users;

.3 the mariner should continue to play the core role in decision making even as the supporting role of the shore-based users increases;

.4 human factors and ergonomics should be core to the system design to ensure optimum integration including the Human Machine Interface (HMI), presentation and scope of information avoiding overload, assurance of integrity and adequate training;

.5 adequate resources should be made available and assured both for e-navigation itself and the necessary enablers such as training and radio-spectrum;

.6 implementation should be measured and not over-hasty; and

.7 costs should not be excessive.

8 POTENTIAL USERS OF E-NAVIGATION AND THEIR HIGH-LEVEL NEEDS

8.1 A significant number of potential ship and shore-based users of e-navigation have been identified and are summarized at annex 2.

8.2 A methodology was used to capture evolving user needs. It was based on the elements contained within the accepted definition of e-navigation and applied templates to define specific user needs based on the harmonized collection, integration, exchange, presentation, analysis and human element aspects for all users. Following extensive feedback from Member States, other maritime organizations, and interested parties, an analysis was conducted resulting in the identification of high-level generic user needs for both ship and shore users. Thus the needs of a typical SOLAS ship and a generic shore authority have been used as the basis for the identification of the high-level user needs reproduced below. A more detailed user needs may have to be identified as a part of the implementation plan.

.1 Common Maritime Information/Data Structure

Mariners require information pertaining to the planning and execution of voyages, the assessment of navigation risk and compliance with regulation. This information should be accessible from a single integrated system. Shore users require information pertaining to their maritime domain, including static and dynamic information on vessels and their voyages. This information should be provided in an internationally agreed common data structure. Such a data structure is essential for the sharing of information amongst shore authorities on a regional and international basis.

.2 Automated and Standardized Reporting Functions

E-navigation should provide automated and standardized reporting functions for optimal communication of ship and voyage information. This includes safety-related information that is transmitted ashore, sent from shore to shipborne users and information pertaining to security and environmental protection to be communicated amongst all users. Reporting requirements should be automated or pre-prepared to the extent possible both in terms of content and communications technology. Information exchange should be harmonized and simplified to reduce reporting requirements. It is recognized that security, legal and commercial issues will have to be considered in addressing communications needs.

.3 Effective and Robust communications

A clear need was expressed for there to be an effective and robust means of communications for ship and shore users. Shore-based users require an effective means of communicating with vessels to facilitate safety, security and environmental protection and to provide operational information. To be effective, communication with and between vessels should make best use of audio/visual aids and standard phrases to minimize linguistic challenges and distractions to operators.

.4 Human Centred Presentation Needs

Navigation displays should be designed to clearly indicate risk and to optimize support for decision making. There is a need for an integrated “alert management system” as contained in the revised recommendation on performance standards for Integrated Navigation Systems (INS) (resolution MSC.252(83)). Consideration should be given to the use of decision support systems that offer suggested responses to certain alerts, and the integration of navigation alerts on board ships within a whole ship alert management system. Users require uniform and consistent presentations and operation functionality to enhance the effectiveness of internationally standardized training, certification and familiarization. The concept of S-Mode has been widely supported as an application on board ship during the work of the Correspondence Group. Shore users require displays that are fully flexible supporting both a Common Operating Picture (COP) and a User Defined Operating Picture (UDOP) with layered and/or tabulated displays. All displays should be designed to limit the possibility of confusion and misinterpretation when sharing safety-related information. E-navigation systems should be designed to engage and motivate the user while managing workload.

.5 Human Machine Interface

As electronic systems take on a greater role, facilities need to be developed for the capture and presentation of information from visual observations, as well as user knowledge and experience. The presentation of information for all users should be designed to reduce “single person errors” and enhance team operations. There is a clear need for the application of ergonomic principles both in the physical layout of equipment and in the use of light, colours, symbology and language.

.6 Data and System Integrity

E-navigation systems should be resilient and take into account issues of data validity, plausibility and integrity for the systems to be robust, reliable and dependable. Requirements for redundancy, particularly in relation to position fixing systems, should be considered.

.7 Analysis

E-navigation systems should support good decision making, improve performance and prevent single person error. To do so, shipboard systems should include analysis functions that support the user in complying with regulations, voyage planning, risk assessment, and avoiding collisions and groundings including the calculation of Under Keel Clearance (UKC) and air draughts. Shore-based systems should support environmental impact analysis, forward planning of vessel movements, hazard/risk assessment, reporting indicators and incident prevention.

Consideration should also be given to the use of analysis for incident response and recovery, risk assessment and response planning, environment protection measures, incident detection and prevention, risk mitigation, preparedness, resource (e.g., asset) management and communication.

.8 Implementation Issues

Best practices, training and familiarization relating to aspects of e-navigation for all users should be effective and established in advance of technical implementation. The use of simulation to establish training needs and assess its effectiveness is endorsed. E-navigation should as far as practical be compatible forwards and backwards and support integration with equipment and systems made mandatory under international and national carriage requirements and performance standards. The highest level of interoperability between e-navigation and external systems should be sought where practicable.

A Common Operating Picture (COP) is single similar display of relevant (operational) information (e.g. position of vessels and resources, position and status of important infrastructure such as hydrographic features and Aids to Navigation, etc.) shared by more than one decision maker. A common operating picture facilitates collaborative planning of operations and assists all echelons to achieve situational awareness. It should be noted that COP is only envisioned for the shore based environment.

A User Defined Operating Picture (UDOP) is a display based on a COP, customised by the operator to display specific information or information in a specific layout to assist with the particular tasks at hand. Marine Information Overlays (MIO) may be used to enhance decision making for particular tasks.

This common infrastructure has to be developed alongside the Marine Spatial Data Infrastrusture (MSDI) currently under implementation amongst advanced Hydrographic Offices such as the UKHO, SHOM, and others.

This could have consequences on current ENC developments such as SCAMIN (the equivalent of scale in a traditional paper chart)

S-Mode is the proposed functionality for shipborne navigation displays using a standard, default presentation menu system and interface.