AISM Association of Internationale de Signalisation Maritime  IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

IALA Recommendation e-NAV-101

On

The e-Navigation Architecture

- the Shore-based Perspective

Edition 1.0

Draft as of 2008-04-18

20ter, rue Schnapper     -     78100     Saint Germain en Laye        -       France

Telephone +33 1 34 51 70 0 Telefax +33 1 34 51 82 05 Telex 695499 ialaism f

E-mail          :         iala-aism@wanadoo.fr      Internet      :      hjttp://iala-aism.org

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

IALA Recommendation on 
The e-Navigation Architecture – 
The Shore-based Perspective

(Recommendation e-Nav-101)

THE COUNCIL

RECALLING that one of the aims of the Association is to foster safe, economic and efficient movement of vessels and the protection of the environment through the improvement and harmonisation of aids to navigation, vessel traffic services and other means world-wide.

NOTING

- IMO’s strategic plan regarding e-Navigation;

- IMO has expressed an interest in the contribution of IALA to the work on e-Navigation;

- the IALA “Proposed e-Navigation Strategy to IMO” document submitted to the IMO Correspondence Group on e-Navigation for their preparation for NAV54;

[this section should be updated following NAV54]

- NOTING ALSO the work carried out by:

- IMO’s Correspondence Group on e-Navigation;

- the e-Navigation Committee and the VTS Committee regarding user requirements within the scope of IALA;

- IALA in developing a [working] definition of e-Navigation, amended and adopted by IMO; [this sentence should be updated following NAV54/MSC85]

RECOGNISING that the e-Navigation architecture will assist in the development and maintenance of peer-to-peer applications for ship to ship, ship to shore, shore to ship, shore to shore, with respect to the international harmonisation and international standardisation, in particular in the following fields

- technical shore-based e-Navigation services;

- technical means for communication;

- data modelling and referential data;

- Human Machine Interface presentations;

RECOGNISING ALSO that the e-Navigation architecture will assist administrations in the efficient deployment of their services to the mariner and to the maritime community.

RECOMMENDS

that National Members and other appropriate authorities providing marine aids to navigation services adopt the e-Navigation design and implementation principles in accordance with the methodology set out in the Annex to this recommendation.

*    *    *

Annex

The e-Navigation Architecture –

the Shore-based Perspective

Table of Contents

Document Revisions	2

1	Overview	6

2	Background	7

2.1	IALA’s general mandate	7

2.2	IMO’s e-Navigation concept	7

2.3	Driving forces leading to the development of the present e-Nav concept	8

2.4	IALA’s role in the e-Navigation concept	9

2.5	Consequences of IMO’s definition of the e-Navigation concept	9

3	The application-to-application information flow	10

4	From e-Nav concept to a technical e-Nav architecture	11

4.1	The technical point of view - introduction	11

4.2	The e-Nav architecture	11

4.3	Functional and physical links	14

4.4	IALA’s role applied to the e-Nav architecture	15

5	The common shore-based e-Nav system architecture	16

5.1	Introduction	16

5.2	Thinking in terms of information flow, peer-to-peer user applications, and user interfaces	17

5.3	What (for) instead of how! - the paradigm for the common shore-based e-Nav system	21

5.4	The shore-based e-Nav system layout	23

6	Dependency on external systems and on infrastructure	23

6.1	Dependency on GNSS for position and time	23

6.2	Dependency on infrastructure	24

7	IALA e-Navigation documentation framework	24

8	Glossary of terms	26

- Overview

This Annex describes the e-Navigation architecture from a shore-based perspective and thus provides guidance to IALA membership regarding methodology.

To that end, this Annex first provides background information on IMO’s e-Navigation concept, the driving forces which lead to its development, and IALA’s role in the e-Navigation concept. Within this context, the dual nature of IALA’s commitment to IMO’s e-Navigation concept is reflected: While there has been a long standing responsibility for architectural considerations of shore-based systems within IALA’s existing mandate, IMO’s e-Navigation concepts adds new dimensions to those architectural considerations.

The Annex goes on to introduce important principles of the e-Navigation architecture, such as the orientation towards application-to-application information flow instead of technology orientation. The Annex then deduces an e-Navigation architecture in system engineering terms from the e-Navigation concept, and establishes additional principles of the e-Navigation architecture. In particular a number of concepts are introduced and explained, namely

- encapsulation,

- co-operative peer-to-peer user application interaction,

- functional interfaces to users,

- functional and physical links ship to ship, ship to shore, shore to ship and shore to shore, and

- technical e-Navigation services.

This introduction of important principles eventually culminates in one concluding figure (Figure 3), which is a fairly complete and comprehensive engineering representation of the e-Navigation architecture.

After the principles of the e-Navigation architecture have been introduced, this Annex applies them to appropriate technologies.  It also describes how it is possible to seamlessly derive more concrete technical requirements for the modules and components of the shore-based e-Navigation system from stated user requirements (“top down approach”). It also takes into account the requirements of life-cycle management at the appropriate stage.

Eventually, the dependencies of the e-Navigation architecture on the World Wide Radionavigation System (WWRNS) and on the infrastructure are reflected.

The Annex concludes with a discussion of the bearing of the e-Navigation architecture on IALA documentation and provides an “e-Navigation compliant” structure for the IALA documentation as an overview.

- Background

  - IALA’s general mandate

The International Association of Marine Aids to Navigation and Lighthouse Authorities (IALA) was formed in 1957 as a non-government, non-profit making, technical association that provides a framework for aids to navigation authorities, manufacturers and consultants from all parts of the world to work with a common effort to:

- harmonise standards for aids to navigation systems worldwide;

- facilitate the safe and efficient movement of shipping, and;

- enhance the protection of the maritime environment.

The functions of IALA include, among other things:

- developing international cooperation by promoting close working relationships and assistance between members;

- collecting and circulating information on communicating recent developments and matters of common interest;

- liaison with relevant inter-governmental, international and other organisations.  For example, the International Maritime Organisation (IMO), the International Hydrographic Organisation (IHO), the Commission on Illumination CIE, and the International Telecommunications Union (ITU);

- liaison with organisations representing the aids to navigation users;

- address emerging navigational technologies, hydrographic matters and vessel traffic management;

- provide specialist advice or assistance on aids to navigation issues (including technical, organisational or training matters);

- encouraging IALA members to develop policies that address the social and environmental issues associated with establishing and operating aids to navigation.  This includes issues such as use of aids to navigation as a base for the collection of data or other governmental or commercial services.

  - IMO’s e-Navigation concept

IMO (MSC 81) accepted a proposal to add a new item on the development of an e-Navigation strategy to its work programme.  The NAV and COMSAR Sub-Committees were asked to consider the issue.

IMO (NAV 53) adopted the following definition of e-Navigation :

“e-Navigation is the harmonised collection, integration, exchange, presentation and analysis of maritime information onboard and ashore by electronic means to enhance berth to berth navigation and related services, for safety and security at sea and protection of the marine environment”

The IMO Correspondence Group on e-Navigation gave the following description about the e-Navigation concept in its report to NAV 54:

“e-Navigation is a global concept that will be implemented and operated at global, regional and local levels across all user groups. A clear ownership and control (regulatory) structure needs to be put in place to ensure coherence and harmonisation, and facilitate seamlessness. This means that the governance of the e-Navigation concept must reside in a single institution that has the technical, operational and legal competences needed to define and enforce the overarching framework with implementation, operation and enforcement taking place at the appropriate level – global, regional, national or local – within that framework. This approach does not mean that the governing organisation has to carry out all tasks in-house – it can delegate as appropriate to competent bodies.”

  - Driving forces leading to the development of the present e-Nav concept

Driving forces that prompted the development of an e-Nav concept are:

- Increased vessel traffic density and/or increased tonnage

- Increased demand for improved information processing in maritime services: The maritime community is facing an increased demand for improved maritime services in order to enhance safety and security at sea, efficiency, and protection of the marine environment.

- Increased demand for improved efficiency in berth-to-berth operations of shipping: This involves an increased demand for simplification of information sharing between information users and information providers.

- Increased demand for communication: The maritime community requires simple and efficient communication systems that ensure confidentiality, integrity and availability of information being transmitted and received ship to ship, ship to shore, shore to ship, and shore to shore. Also, there is an increase demand for communication to Search-and-Rescue and law enforcement aircrafts.

- Advent of digital information technologies: Most information is now available in digital format both onboard ships and within shore-based systems. It is also a requirement that more data is collected and stored in order to facilitate data comparison, data exchange and statistic evaluation.

- Globalisation: The maritime community has identified the need to track vessels from berth-to-berth for strategic vessel management, Search and Rescue, and security. This has resulted also in an increased demand for international data exchange.

- Extended area coverage: There is a requirement for extended area coverage which can be fulfilled more readily by e-Nav technologies.

- Imposed reduction in staffing level: Due to a steady reduction in staffing level, imposed by national governments, administrations require an e-Nav concept

- which would allow for a minimum of technical services to support in terms of both investment and maintenance and

- which would support more efficient life cycle management of the technical services in place.

- Cost/benefit: The maritime community is striving for increased efficiency and improved cost/benefit ratios in their operation. Also, administrations face an increased public awareness for cost/benefit of their operation.

Immediate consequences of the envisaged e-Navigation concept are the following, which in turn may become driving forces themselves.

- International standardisation: International standardisation has been recognised as a state-of-the-art description for technology. Hence, there is an increased need for information and documentation for a common, international and public understanding of system functions that constitute the e-Nav concept.

- Open architecture: When creating an e-Navigaton architecture and supporting technical services, modular and open system design principles should be applied (“plug-and-play”). Open system architectures are more scaleable and better maintainable. When individual components need to be replaced due to life-cycle management, there is a requirement to maintain the functional requirements of a shore-based system.

- Increased degree of automation: An increased degree of automation is requested to assist shore-based operators and maintenance personnel.

  - IALA’s role in the e-Navigation concept

Because of its general mandate, IALA has a responsibility to the maritime community to support national members providing shore-based services (ship to shore, shore to ship and shore to shore).

At the IMO Sub-Committee on Navigation’s meeting in July 2007, NAV53, several delegations reiterated the role that IALA could play in the development of e-Navigation, particularly the shore aspects of aids to navigation.  They also pointed out that IALA has created an e-Navigation Committee at its 2006 Conference in Shanghai. The conclusion was that IALA will play a key role, working closely with IMO, IHO and other relevant international organisations in developing the e-navigation concept.

Also in July 2007, at the IALA e-Navigation Seminar the Secretary General of IMO gave a keynote address, stressing that IMO has clearly identified the importance of a common architecture for e-Navigation. The Secretary General of IMO also stressed, “that IALA’s participation in the work of IMO on this subject is vital, and will be become more so as the concept takes shape and eventually near fruition”.

Hence, IMO has invited IALA and other international organisations to participate in its work and provide relevant input.

  - Consequences of IMO’s definition of the e-Navigation concept

From the above IMO’s definition of e-Navigation, the following points are understood:

- Operational requirements (user requirements) need to be analysed in the framework of the e-Nav architecture, i.e. internationally analysed and consolidated operational requirements need to be represented in a way meaningful to the individual parts of the e-Nav architecture.

- Three mutually interacting parts of an e-Nav architecture can be identified:

- shipboard integration of information/data processing devices;

- application-to-application data exchange in a peer-to-peer fashion;

- shore-based e-Nav system architecture that integrates a variety of shore-based technologies and data processing devices;

- Since the e-Nav concept is envisaged over a ship’s complete voyage from berth-to-berth, the e-Nav concept requires global commonality of the shore-based e-Nav system architecture of different national competent authorities. There needs also to be a certain degree of international standardisation.

- Common service levels and standardised minimum capabilities are implied for

- the participating shipboard integrated equipment

- the application-to-application data exchange means and

- the services provided from applications ashore.

- The application-to-application information flow

The key element of the e-Nav concept is considered in terms of information flow between applications, which constitute sources and sinks for information, or both simultaneously.

In addition, storage of information and /or data over longer periods of time together with the  retrieval of that information / data are part of the e-Nav concept.

To achieve that end, operational requirements (user requirements) can be represented in terms of appropriate state-of-the-art data models with abstract descriptions of the processes and of the functionality which are required to properly process the data to arrive at meaningful information for the stakeholders involved.

Furthermore, the co-operation in data exchange ship to ship, ship to shore, shore to ship, shore to shore and other users is at the core of the e-Nav architecture. Hence the co-operative nature is a core value of e-Nav concept.

To provide seamless service, the existing non-co-operative services and aspects must be integrated in this predominantly co-operative e-Nav architecture by proper engineering models.

It is appropriate, when considering abstract flow of information between applications, to use the term of functional links between participating applications..

The above abstract statements constitute the core values of the e-Nav concept in analysis and design. Eventually the data exchange and the processes and functionalities using that data need to be manifested in physical links, e.g. a physical communication link, and physical entities and devices, e. g. computer systems. Hence, requirements and limitations stemming from that physical world will have their impact on the system architecture.

Finally, the different dimensions of application-to-application information flow need to be considered to understand the e-Nav concept. These different dimensions include the following:

- Location of the participating applications: on board ships, on board floating AtoNs, onboard aircrafts interacting with the maritime community and on shore, etc. This dimension highlights the requirement of a high connectivity within the e-Nav concept.

- Distributed responsibilities of stake holders in the e-Nav concept, e.g. different authorities ashore operating in the same area, but with different tasks.

- The large variety of the nature and amount of data to be exchanged between applications will result in a large variety of appropriate technologies to be considered.

- Quality requirements: The attributes of accuracy, integrity, reliability, continuity, time behaviour etc. need to be defined when analysing the operational requirements (user requirements).

- From e-Nav concept to a technical e-Nav architecture

  - The technical point of view - introduction

In the previous chapter e-Navigation was described as a concept in operational and management terms. In order to implement the e-Nav concept technical solutions need to be developed and deployed. These technical solutions need to fulfil both the operational requirements and the management goals simultaneously.

Also, a seamless and transparent, top-down, step-wise development of the technical solution from the operational requirements is essential. This derivation would need to take into account the above life-cycle-management goals as well.

It is evident that due to the overarching and complex nature of the e-Nav concept, the technical solution is not a simple one. By means of introduction, several steps are required to arrive at a suitable technical solution. The first steps are being taken in this IALA Recommendation, while consecutive IALA Recommendations will build upon this.

The very first step to derive a technical solution from the abstract e-Nav concept detailed already, is to look at the e-Nav architecture from a technical point of view, i.e. correlate the abstract e-Nav concept with some well known technical concepts, systems and components while keeping in mind the whole picture and preserving the abstract qualities of the e-Nav concept. This step is examined in this chapter.

  - The e-Nav architecture

All the aspects introduced in the preceeding chapters plus some others to be introduced need to be considered when developing the e-Nav architecture.

One resulting representation of the e-Nav architecture is shown in Figure 1. The shipboard entities, the physical link(s) and the shore-based entities are included in this representation of the e-Nav architecture.

On the left within Figure 1 is represented, for simplicity’s sake, a single “ship technology environment”. From the e-Nav concept’s perspective the relevant devices within the ship technology environment are the transceiver station, the data sources and the data sinks connected to the transceiver station, the Integrated Navigation System (INS) and the Integrated Bridge System (IBS). The transceiver station is shown as a single station for simplicity’s sake, although there may be several transceiver stations.

The transceiver station interfaces with the physical link(s) to the appropriate technical e-Nav services ashore. Examples of physical links are introduced in the next section.

The shore-based technical e-Nav services, in their totality and by their interactions, provide the interfaces of the shore-based user applications to the physical link(s). They also encapsulate their technology  the whole of the common shore-based e-Nav system architecture. The encapsulation principle hides the technology’s sophistication from the shore-based e-Nav system as a whole and thus reduces complexity. In Figure 1, the entities which are involved with the specifics of the link technology  are confined by the dotted line. Amongst other benefits, it allows for parallel work of the appropriate experts in the particular technology of a given physical link, provided the functional interfaces of the shore-based technical e-Nav services are well defined.

For the precise technical structure of the shore-base technical e-Nav services see the common shore-base e-Navigation system architecture in IALA Recommendation 0201.

The shore-based technical e-Nav services use data transfer network(s), which distribute(s) the data used by the shore-based technical e-Nav services. The requirements for data transfer are dependent on the specifics of the technology of the technical shore-based e-Nav service. For example, a shore-based Radar Service would require different data transfer network capabilities than a shore-based AIS Service. Hence, the individual requirements for data transfer network of a shore-based technical e-Nav service are also encapsulated. This also reduces complexity.

It should be noted, that the shore-based technical e-Nav services mutually exchange data between them at Nodes or Node sites by means of local area networking.

The operators ashore, e. g. VTS operators, need the user applications to perform their tasks in co-operation with shipboard applications. From their perspective, it is not the physical links nor the shore-based technical e-Nav services that matter – they have been encapsulated for that reason. It is the functional links between the shore-based user applications at hand and the peer shipboard applications which matter to the users on both sides.

Figure 1 shows IMO’s World Wide Radio Navigation System (WWRNS), which includes GNSS, being presented as a system “external” to the e-Navigation architecture providing position and time information. A further discussion of that dependency is contained in a chapter below.

IBS

World Wide Radionavigation System (WWRS) of IMO, including GNSS, GNSS augmentation and terrestrial backup

Appli-cation 1

Application 2

Application 3

Application 4

Link technology (encapsulated)

Transceiver station

Ship’s

sensors

Shipboard

Applications

INS

technical

e-Nav

ser-vices

Application-to-

application

(peer-to-peer)

functional

connection

Physical

Link (e.g. radio link)

other

ships

other

ships

mariner

Shore-based

users

Figure 1: 
e-Nav architecture

Ship technology 
environment

  - Functional and physical links

    - The relationship between functional and physical links

The bold dotted arrow in Figure 1 is an example of one functional connection for data exchange between “Application 4” ashore and the “ship’s technology environment” (data sink), and vice versa. The physical path of the data exchange uses the physical links and the various physical interfaces between “Application 4” ashore and the “ship’s technology environment” , as indicated by the small arrows. The functional connection is the abstract statement in regard to requirement analysis for the application-to-application [information/data] exchange. The physical path may take a completely and more sophisticated path. To use the abstraction of the functional connection hence facilitates the requirement analysis.

    - Relevant physical link technologies

Physical links between (fixed) shore and (mobile) shipboard equipment each employ one or more appropriate mediums such as radio waves or light.

Figure 2a shows a variety of physical link technologies which are relevant for the e-Nav architecture. It also shows that well-known technologies such as VTS radar can be modelled as a physical link: the data exchanged is the ship’s radar echo signature. Figure 2b shows that visual or traditional Aids-to-Navigations (either fixed or floating) can also be modelled in the same way. In fact, “visual” Aids-to-Navigation may employ a variety of state-of-the-art physical link technologies simultaneously, not just light.

Every technical e-Nav service of the shore-based e-Navigation system architecture employs at least one physical link technology.

Figure 2a/2b: Variety of physical link technologies relevant for the e-Nav architecture

  - IALA’s role applied to the e-Nav architecture

All Aids to navigation can be represented digitally allowing full evaluation and planning of Aids to Navigation with respect to e-Nav.  IALA members have responsibility for Maritime Aids to Navigation, including VTS, hence IALA has a significant role to play in the development of the e-Nav concept.  IALA’s role regarding the e-Navigation architecture can be derived in terms of the entities involved. Based on IALA’s existing mandate, IALA would thus concern itself with

- the shore-based aspects in general, including the definition of the relevant shore-based user requirements, as well as training aspects of shore-based personnel;

- the relevant shore-based user applications, e. g. VTS applications;

- the variety of individual shore-based e-Nav services, such as the Radar Service, the AIS Service, the traditional Aids-to-Navigation services etc;

- the link technologies:

- The AIS is a point in case: ITU has requested IALA to maintain the AIS VHF data link (Compare Recommendation ITU-R M.1371).

- Another point in case may be the re-engineering of existing radio navigation systems, such as the MF DGNSS system (IALA Radio Beacon system), with the goal to maximise the data transfer capacity of that radio link.

- Yet another point in case may be the co-ordinated development and use of optimised existing and genuinely new radio communication systems shore to ship/ship to shore, as well as the development of the IALA World Wide Radio Communication Plan.

- the dependency and therefore contribution to the World Wide Radio Navigation System: contribution in terms of terrestrial augmentation and/or backup systems, such as the IALA developed MF DGNSS system. The development of the IALA World Wide Radio Navigation Plan (WWRNP) is another point in case.

There is also an involvement by IALA in the shipboard transceiver station definition, since it cannot be treated independently from its associated physical link technology. Again AIS is a point in case: IALA initiated and still takes an active role in the definition of mobile AIS stations, which were or are still under consideration at IEC, to the extent of forming a joint IEC/IALA working group in this regard.

- The common shore-based e-Nav system architecture

  - Introduction

The previous chapter introduced the first step of how to arrive at a technical solution for the quite abstract e-Nav concept, starting from user requirements by top-down seamless derivation and taking also into account the management goals stated above. Also, the role of IALA was introduced in this regard, thus introducing a specific shore-based point of view.

Also, an e-Nav architecture was described in more technical terms, part of which are well-known, part of which are necessary by the e-Nav concept. The e-Nav architecture consists of three main columns:

- Shipboard equipment and shipboard applications

- Shore-based technical e-Nav services and their components, including requirements for data transfer networking, and shore-based user applications;

- Radio links ship-to-shore/shore-to-ship as physical links.

Thus, certain important concepts of the e-Nav architecture have been introduced, from a technical point of view.

It is an essential requirement, that any part of the e-Nav architecture should be developed in accordance with top-down user requirements, while also fulfilling the management goals. To eventually arrive at a seamless derivation of requirements for shore-based technical e-Nav services and their components from top-down user requirements, by means of introduction, it is now necessary to make a second step (the first step was done in the previous chapter): It is now necessary to step back once again from the concrete physical entities involved. It is necessary to retract into the abstract once again since the seamless derivation of specific requirements for physical entities from the (abstract) user requirements is an abstract process. This is done in this chapter.

Also, when introducing the shore-based technical e-Nav services in the previous chapter it were allusions to a shore-based system architecture, which would be required to allow for dynamic and flexible interaction of the shore-based technical e-Nav services in a proper way, to the ultimate benefit of the user applications. This chapter focuses on that shore-side system architecture but necessarily takes also into account the e-Nav architecture as introduced in the previous chapter.

  - Thinking in terms of information flow, peer-to-peer user applications, and user interfaces

User and operational stake holders in general, within the e-Nav concept, should think in terms of information flow, peer-to-peer user applications, and user interfaces, as follows:

User requirements can be described in terms of information and functionality delivered at the Human-Machine-Interfaces of the user applications, both shipboard and ashore, and by the functional links in-between those peer-to-peer applications.

This is illustrated by Figure 3 (explanation follows).

Figure 3: The e-Navigation architecture in terms of information flow, peer-to-peer user applications, and user interfaces

In Figure 3 the following graphical symbols are used:

- interfaces between entities involved are symbolised by a little circle and a line leading to the system which provides the interface.

- technical systems are symbolised by cubes, which expresses the encapsulation principle explained above (“black boxes”).

- Human-Machine-Interfaces, indicated by the acronym HMI, and Machine-Machine-Interfaces, i.e. mainly IT or computer interfaces, indicated by the acronym M2M.

- The arrows with the dotted lines indicate requirements, which are put forward by the entity at which the dotted line starts.

- Requirement arrow(s) always point to interfaces (circles). This is to indicate, that the interface(s) fulfil the requirement(s).

The following conclusions can be drawn from Figure 3:

- When designing and describing operational e-Nav applications, they should be described as peer-to-peer applications using the information flow concept:

- (Ultimate) sources of data items stemming from those sources to the information flow, i.e. source data, should be identified.

- Likewise, (ultimate) sinks for data items, required by a particular sink of the information flow, i.e. recipient data, should be identified.

- When there is intermediate processing of data items involved, the appropriate algorithms should be stated on an operational level (not in technical detail).

- For all data items should be described with their attributes relevant to the operational e-Nav application.

- Necessary operational constraints (such as permissible min/max values) should be stated as additional attributes to the data items. The number of operational constraints should be minimised, however, to allow for maximum innovation when designing technical solutions.

- Figure 3 also represents a clean-cut statement regarding the distribution of responsibilities between operational stake-holders and engineers:

- Operational stake-holders state the operational requirements (symbolised in Figure 3 by dotted arrows), in particular at the user interfaces.

- Engineers analyse the operational requirements and take into account the management goals, in particular considering the life-cycle-management aspects of any system or component. The result of their analysis is an engineering-like representation of the operational requirements together with management requirements.

- This engineering-like representation of the analysis of operational and management requirements is a stand-alone document, which may and necessarily often will assume a different format than the documentation of the operational and of the management requirements. Thus, it can and should be verified and validated by operational stake-holders and management. This engineering-like representation constitutes a kind of “Service Level Agreement” between the operational stakeholders and the engineers, i.e. a Service Level Agreement at the user interfaces.

- Engineers eventually provide the appropriate technical Human-Machine-Interface (HMI), which fulfils those operational requirements.

- A Human-Machine-Interface (HMI) can be any kind of appropriate combination of displays, keyboards, voice interfaces (microphone / loudspeaker), and other human interaction devices. The suite of those devices at one operational working position is called the Operational Presentation Surface (OPS). This term uses the concepts of modern computer interfacing technologies, such as window techniques.

- The distribution of responsibilities is reflected in an appropriate documenta-tion framework. As far as IALA documentation is concerned, this is further developed in the last chapter.

- It should also be noted, that Figure 3 returns the usage of the term “VTS” completely to the operational realm and also helps to resolve the perceived contention between the two terms “VTS” and “VTMIS” as follows:

- “Vessel Traffic Services” (VTS) strictly speaking is an operational term, as the term implies. It comprises the three individual and well-established operational services: Traffic Information, Navigational Assistance and Traffic Control.

- Over time, the term VTS (and only in its abbreviated form for that matter) came to be used as a synonym for the technical system, which supports the above operational services. Common usage was to consider a “VTS” as a well defined set of certain technologies, in particular radar and VHF analogue voice communication. When referring to the technical equipment needed to support the Vessel Traffic Services, the technical term VTS System should be used.

- The recognition of the VTS as an “Aid-to-Navigation” by IMO does not contradict this, when considered at the functional level.

- With the advent of modern digital technology on a broader scale, in particular with the advent of AIS and Information Technology (IT) in general, the commonly used term became questionable, i.e. a modern VTS System will often integrate more than “just” radar and VHF analogue voice communication. Also the need to exchange data between different shore authorities was identified for various purposes, i.e. additional tasks developed over time.  It was felt, that the term “VTS” was too narrow to convey the full picture. This gave rise to the introduction of the term “Vessel Traffic Management and Information System” (VTMIS).

- It should noted, that “VTMIS” as spelt out above is a designator for a technical system. To keep the discussion on the appropriate level - i.e. on the operational level – “VTMIS” should be spelled out “Vessel Traffic Management and Information Services”, thereby indicating the operational services involved.

- Figure 3 now renders both concepts possible on the same level, i.e. the operational level. Both the Vessel Traffic Services and the Vessel Traffic Management and Information Services, when understood as operational services and not as technical solutions, would require the appropriate user interfaces by “the technical system”. Since the technical system may and should be construed as a “black box” by the operational stake holders, the actual technologies which are employed within that “system”, are irrelevant to the operational services as long as they fulfil the operational requirements.

- Hence, the above terminology discussion “VTS” / “VTMIS” can and should be resolved on operational grounds alone, namely as a well established understanding of sets of operational services provided, which may or may not overlap.

  - What (for) instead of how! - the paradigm for the common shore-based e-Nav system

Based on the definition of the operational requirements as described above, these stated and internationally agreed user requirements can then be translated into…

- functional analysis documentation of the requirements using their engineering analysing tools as explained, followed by

- functional descriptions of shore-based technical e-Nav services with definitions of the physical links (“signal-in-air”) together with functional descriptions for shipboard devices

… while all the time taking into account the additional management goals in this process, such as life cycle management requirements.

There are state-of-the-art engineering tools available to facilitate that work, such as the Object-oriented Engineering Process (OEP) and the use case methodology. That methodology toolbox perfectly supports IMO’s e-Navigation concept.

The most important part of the paradigm of the common shore-based e-Nav system is the Object-oriented Engineering Process (OEP) (see Figure 4 below). It describes the methodology applied to the engineering task at hand in the following step-by-step process.

- List and specify all primary operational requirements for the common shore-based e-Nav system as a whole.

- List and specify the secondary requirements for the common shore-based e-Nav system as a whole from a management and technical point of view, such as life-cycle-management requirements and resources usage requirements.

- Use state-of-the-art engineering tools to do an engineering analysis of requirements to the shore-based e-Nav system.

The well attested use case approach provides such state-of-the-art engineering methodology. [It is a point in case that IMO’s defines e-Navigation in accordance with the use case concept: “e-Navigation is the harmonised collection, integration, exchange, presentation and analysis of maritime information onboard and ashore (…).”

- The five keywords highlighted above describe what tasks are to be performed with maritime information, are indeed use case designations. (The five keywords are the core of the definition the remainder of which is only adding additional details.)

- It should further be noted, that IMO’s definition explicitly discerns the use cases for the shipboard side and the shore side (also highlighted above).

- Hence, it can be concluded that IMO’s definition of e-Navigation is fully compatible with state-of-the-art engineering tools for requirement analysis, and vice versa.]

- Derive the essential system requirements by exploitation of similarities between primary of different users and also of secondary requirements as identified above.

- Design a common shore-based e-Nav system layout predominantly using individual e-Nav services as building blocks.

- For each and every essential system requirement identify the interactions of the relevant individual e-Nav services needed to fulfil that specific essential system requirement.

- Draft a precise functional description of an individual technology as an e-Nav service in accordance with an generic e-Nav service model. Include tertiary requirements now.

- Derive all component requirements for all relevant components of an individual e-Nav service (primary, secondary, tertiary).

- Capture all the above descriptions in a documentation system and prepare for the submission to Quality Management audits.

Figure 4: Engineering analysis of requirements to the shore-based e-Nav system

  - The shore-based e-Nav system layout

The shore-based e-Nav system layout is described in detail in IALA Recommendation e-NAV 201.

- Dependency on external systems and on infrastructure

  - Dependency on GNSS for position and time

As indicated in Figure 1, there is a dependency of the e-Nav Concept on external systems for position fixing and for timing. This may pose a certain vulnerability for all e-Nav applications, since dynamic position information is involved in most e-Nav applications and time information is required in every e-Nav application. Hence, mitigation methods are important.

  - Dependency on infrastructure

Though functionally apparently “invisible”, the supporting infrastructure as a module internal to the common shore-based e-Nav system architecture has an ever increasing importance for the proper and efficient function of the e-Nav applications. Hence, the dependency of the common shore-based e-Nav system on its infrastructure.

The infrastructure consists of supporting devices, which e.g. allow application software to be executed (i.e. computers, computer periphery, operation systems, local area network components, etc.), provide a reliable energy supply, provide temperature control, provide housing for components, protection against harmful interference, tampering, unauthorised access, environmental conditions, etc.

Infrastructure becomes more and more sophisticated and in itself subject to increasing requirements. Reduction of energy consumption, environmental friendliness, centralised management, maintenance cost, staffing level, staff responsibilities, and reliability level are just a few key words in this regard.

When setting up a shore-based e-Nav system, the infrastructure should be planned carefully to mitigate harmful consequences of the dependency of the shore-based e-Nav system on its infrastructure.

- IALA e-Navigation documentation framework

This recommendation fits into the IALA e-Navigation documentation framework as indicated in the following figure.

| e-Nav Recommendations on Generalities of the shore-based e-Nav system architecture |  |  |  |  |
| --- | --- | --- | --- | --- |
| Table of Contents for “e-Navigation compliant” IALA Documentation |  |  |  |  |
| Shore-based e-Nav system terminology (= glossary) |  |  |  |  |
| e-Nav Recommendations on e-Nav conceptional aspects and user requirements viewed from ashore |  |  |  |  |
| The e-Nav Architecture – the Shore-Based Perspective (= this Recommendation) |  |  |  |  |
| e-Nav system requirement analysis and e-Nav applications descriptions: arranged in fields of e-Nav applications as defined by OPS WG |  |  |  |  |
| e-Nav Recommendations on technical generics of the common shore-based e-Nav system architecture |  |  |  |  |
| common shore-based e-Nav system architecture |  |  |  |  |
| technical e-Nav services (engineering model) |  |  |  |  |
| The Universal Maritime Data Model and the data model of the common shore-based e-Nav system architecture |  |  |  |  |
| Interface considerations with shore-based systems and between shore-based systems |  |  |  |  |
| Infrastructure considerations |  |  |  |  |
| Life-Cycle-Management Considerations: Technical Operation Personnel + Tasks/ Technical Development Personnel + Tasks |  |  |  |  |
| Quality Management and Certification Considerations |  |  |  |  |
| e-Nav Recommendations on shore-based e-Nav applications mapping to e-Nav services involved - examples |  |  |  |  |
| Shore-based e-Nav applications involving “visual” Aids-to-Navigation |  |  |  |  |
| Shore-based e-Nav applications involving VTS |  |  |  |  |
| … |  |  |  |  |
| e-Nav Recommendations on technology-specific e-Nav services - examples |  |  |  |  |
| AIS Service | Radar Service | “Visual” AtoN Service | User Interaction Service | … other technical e-Nav services … |

Figure 5: [Future] IALA e-Nav documentation layout

- Glossary of terms

e-Navigation concept: a concept devised to provide a comprehensive, standardised ship-to-ship, ship-to-shore, shore-to-ship and shore-to-shore information exchange environment.

e-Navigation architecture: a description of all the actors, i.e. users and systems, functions and processes, technical services, interactions of entities involved, information and data being an integral part of the e-NAV concept.

common shore-based e-Navigation system architecture: a subset of the e-Navigation architecture, composed of only the shore-based part, which is exploiting international commonalities of such shore-based systems of IALA national members.

(technical) e-Nav service: fundamental building block of the common shore-based e-Navigation system.  For the (technical) e-Nav service there is a generic, international and abstract description without reference to a specific physical implementation or a specific technology. This generic description exploits the architectural commonalities of various technologies, however.

Specific (technical) e-Nav services: technology-specific e-Nav services like the AIS Service, the Radar Service, the DGNSS Correction Service, the User Interaction Service (to e.g. display maritime traffic situational image), etc.

functional link: the functional implementation of a communication link between two e-Nav actors.

physical link: the physical implementation of a communication link between two e-Nav actors.

peer-to-peer connection: describes communication from the originator/sender (information source) to the recipient (information sink), without reference to the communication medium(s).

Due to that development it is strictly speaking a misnomer to use the word “visual” aids-to-navigation. Since “visual” aids-to-navigation is a well-known term, it will be used throughout the description of the e-Nav concept and the common shore-based e-Nav system architecture.

Primary aspects are those, which justify the existence of the system component under consideration. Secondary aspects are those, which arise only as “second thoughts”, e.g.: To replay data, there must have been a data logging beforehand. Tertiary are those, which arise only when considering faulty behavior or optimisation of basically healthy functional concepts.