ANM12/11/4

| IALA 20 ter Rue Schnapper 78100 St Germain-en-laye France |  |
| --- | --- |

| Your ref.: ANM 12 | Our ref.: 07/04670-4 | Archive no: 622.3 | Officer in charge: B E Krosness | Date: 2008-09-29 |
| --- | --- | --- | --- | --- |

Dear Sirs,

Considerations by NCA on the development of consolidated guidance for the marking of different types of offshore structures, including their impact on AtoN, radar and communications

We refer to your message of 2008-09-03 regarding Task 8 "Develop consolidated guidance for the marking of different types of offshore structures, including their impact on AtoN, radar and communications" of the ANM Committee work programme.

In our consideration of our use of the O-114, O-116, O-117 and O-131 recommendations we find that adhering to these does not necessarily fully meet the users need for visual navigational guidance, and also that these to some extent might result in the provision of more navigational guidance than necessary. These recommendations may have been developed at a time with different user requirements, and not been amended as necessary to cater for the present requirements, taking the development of navigation technology into account. Specifically there are, in our view, unfavourable limitations of light characters for special marks in the Maritime Buoyage System, if fully adhered to, and a need to reconsider the requirements for especially audible and to some extent also visual aids.

We are not sure whether the marking of different types of offshore structures in all instances could be classified as an Aid to Navigation. Therefore it is of interest to go back to a definition (Bowditch) of such aids, e.g.:

"The term aid to navigation means any device external to a vessel intended to be of assistance to a navigator in his determination of position or safe course, or to provide him with a warning to dangers or obstructions to navigation."

Having defined what an aid to navigation is, we should look at the means that are available to assist a navigator to determine a position or safe course. Thereafter we should establish what would be a safe and cost effective way to carry out ship-, vessel- or boat-navigation. In doing this it is essential to consult with carriage requirements for shipborne navigational systems and equipment. For ships where the SOLAS convention applies this can be found in Chapter V: Safety of navigation Regulation 19. For fishing vessels and boats other regulations would apply. It should be noted that the current carriage requirements and requirements that are expected in the near future (e.g. ECDIS) for SOLAS ships is the result of a process where mitigation of risk has been an important factor.

In general it is our view that the development, in both regulatory tools and in means available, have led to audible aids becoming more or less obsolete, while visual aids are still very important, although at a shorter range than before. Further we believe that radar aids, as it have for a long time, and AIS will be an important part of future means. The presentation of aids to navigation on the ECDIS both with respect to accuracy and legibility is also an important matter which should be kept under observation.

In practice we would suggest that it be considered to merge, and revise and update as necessary, the recommendations with two recommendations as the end result:

- recommendation on the marking of offshore structures, including renewable and non-renewable energy installations, and

- recommendation on the marking of aquaculture farms

It could be an option to have separate recommendations for renewable and non-renewable energy installations respectively.

We would also like to make some comments on each of the recommendations:

Re. O-114 Recommendation on the marking of offshore structures

From the introduction it can be found that this recommendation originates back to 1965. The present recommendation from 1998 is mostly adequate, but there may in part be reasons to amend the technical requirements such as:

- A range instead of intensity could be described for the light. The power of the light in candelas would then have to be adjusted according to the atmospheric conditions in the region as appropriate. The recommended range could for example be either 12 or 15 nautical miles or somewhere between.

- The description of the recommended marking of the vertical extremities of the structure could be relaxed. That is, it should be taken into account that the installations in general are very well lit, and therefore this kind of marking in some instances may be superfluous.

- The wording "shall" carry one or more sound signals is to absolute and in our view the recommendation need to be updated regarding this type of AtoN.

- The recommendation on the use of AIS should also be considered.

Re. O-116 Recommendation on the marking of aquaculture farms

As this recommendation is of recent date and includes different marking options we see no immediate need for revision.

It should be stressed that marking of the aquaculture farms in the coastal waters and close to the fairways is an important issue for us. Aquaculture farms represent a considerable risk for high speed crafts and other vessels navigation. Further the escape of fish from such farms, which is the probable result of a collision, poses a major environmental threat.

Responding to an initiative from the Norwegian Directorate of Fisheries we have in 2008 introduced new regulations for the marking of aquaculture farms. In the regulations it is referred to IALA guidelines and recommendations. After thorough consultation with the users it was decided to use the following light signal:

"The light signal shall have yellow colour with a 2 second interval, of which 1.25 seconds light and 0.75 second darkness, named Oc Y 2s, which is a duty cycle of 62.5 %.

The range of the light in darkness shall be 3 nautical miles

The colour shall be in accordance with recommendations given by IALA, and the range shall similarly be calculated after a recommendation given by IALA.

The Norwegian coastal administration can in special circumstances approve that the character with a period within 2 seconds, with a lower duty- cycle or that a range of 2 nautical miles is used."

This is concurrent with recommendations after two very serious high speed craft accidents in Norway, namely the "SeaCat" accident in 1991 and the "Sleipner" accident in 1999.

Re. O-117 Recommendation on the marking of offshore wind farms

We have found that when devising a marking scheme for wind farms both marking as recommended for a wind farm, marking in accordance with the MBS, floodlighting of individual installations etc. should be considered. From an environmental perspective the marking should also not be able to be perceived as light pollution.

The present recommendation should preferably be reviewed and eventually revised if found necessary.

Re. O-131 Recommendation on the marking of offshore wave and tidal energy devices

This recommendation should also be reviewed as the O-117 recommendation and eventually revised if found necessary.

In conclusion it is our view that the IALA recommendations should recommend what is needed to ensure safety of navigation. If anything in excess is recommended it will incur unnecessary costs, in some instances engineering works that is more troublesome to carry out, and may contribute to bringing the regulatory regime in disregard.

| Arve Dimmen | Bjoern Erik Krosness |
| --- | --- |
| Head of division | Principal Engineer |