REPORT

on

the 54th session of the IMO Sub-Committee

on Safety of Navigation (NAV)

Contents

# 1.	Introduction

2.	e-Navigation

# 3.	Ships’ routeing and related matters

3.1	AIS information and mandatory ship reporting system

## 3.2	Traffic Separation Schemes (TSS) and other routeing measures

3.3.	Mandatory ship reporting system

4.	Electronic Chart Display and Information System (ECDIS)

5.	Shipborne Automatic Identification System matters

5.1	Impact of Resolution MEPC.118(52) upon existing AIS shipboard installations

5.2	Satellite detection of AIS

5.3	Measures to minimize incorrect data transmission by AIS equipment

5.4	Revision of the Guidance on the application of AIS binary messages

5.5	Presentation symbol for AIS-SART

6.	Right of way of vessels over pleasure craft

7.	Other items

7.1	Colour specification of lights

7.2	Improved safety of pilot transfer arrangements

7.3	Codes, recommendations, guidelines of non-mandatory instruments

7.4	Worldwide radionavigation system

## 8.	Recommended actions for IALA

## 8.1.	the Council

8.2.		the e-Navigation Committee

8.3.    	the VTS Committee

8.4.		the Aids to Navigation Management Committee

8.5		the Engineering, Environment and Preservation Committee

8.6		the Pilotage Authority Forum

# 9.	Date of the next session.

### A N N E X

Sub-Committee on Safety of Navigation (NAV) – 55th session - Agenda proposed to the Maritime Safety Committee

*  *  *

1.	Introduction

The 54th session of the IMO Sub-Committee on Safety of Navigation was held at the IMO headquarters in London during the period 30 June to 4 July, 2008. It was attended by representatives from 67 Member States and 23 Observer Organizations. The Secretary General and Jean-Charles Leclair were representing the Association.

The most important item discussed during the week was the concept of e-Navigation, its definition and the way to adopt an overall strategy to cover all of its aspects. IALA was very involved in the discussion and, from the beginning, has taken an active role in the development of the IMO work on this subject.  The Sub-Committee agreed on the strategy of e-Navigation, very close to the draft established by IALA, and proposed to the Maritime Safety Committee a strategy implementation framework inviting again the international organizations involved, in particular the Association, to continue to participate in the implementation of e-navigation.

Furthermore, after years of discussion, the Sub-Committee succeeded to reach a consensus on mandatory carriage requirement for ECDIS for all passenger, tanker and cargo ships, but with a calendar of implementation extended up to 2018.

During the session, at the request of the chairman, Mr. K. Polderman from The Netherlands, the Sub-Committee was invited to elect a new chairman for the year 2009. Therefore, the Sub-Committee elected Mr. J.M. Sollosi from the United States, who is also the chairman of the IALA VTS Committee. The Sub-Committee also elected a new vice-chairman, Mr. Raja Datuk Malik from Malaysia.

2.	e-Navigation

At its 81st session (May 2006), the Maritime Safety Committee had instructed the NAV Sub-Committee, together with COMSAR, to develop an e-navigation strategy with a target completion date of 2008. As a first step, NAV 52 established a correspondence group, under the co-ordination of the United Kingdom, to develop a clear definition and objectives for the concept of e-navigation. IALA and IHO were particularly invited to participate to the work of IMO on the matter and to provide relevant inputs.

The Correspondence Group submitted a first document to NAV 53 and the Sub-Committee agreed on the definition of e-navigation, as proposed to the Correspondence Group by IALA, and on the core objectives of the concept. It also decided that the e-Navigation strategy should be user driven and that it was first necessary to identify and define the user requirements before considering any technology standards.

NAV 53 decided to re-establish the intersessional Correspondence Group with new terms of reference. NAV 53 also instructed the Correspondence Group to submit a document to COMSAR 12, raising specific questions that should be addressed by COMSAR and also to prepare a final comprehensive report for consideration by NAV 54.

Therefore, the Correspondence Group submitted two documents to the Sub-Committee; the first one, mostly inspired by IALA works, was the proposed high level strategy for e-Navigation and contained advises on the methodology adopted to identify user needs and also propositions for an implementation strategy for e-Navigation. The second document provided supplementary information developed by the Group as well as a potential program for implementing e-Navigation.

The Sub-Committee using the proposals from the correspondence group agreed two documents for submission to the Maritime Safety Committee at its 85th session (December 2008).

The first one (15 pages) presents the Strategy for the implementation of e-Navigation. Its content is as follow:

- Definition and scope

- The need for e-Navigation

- The case for e-Navigation

- Vision for e-Navigation

- Core objectives of e-Navigation

- Benefits of e-Navigation

- Basic requirements for the implementation and operation of e-Navigation

- Potential users of e-Navigation and their high level needs

- Key strategy elements and implementation

Annex 1: Responsibilities for ownership and control of the e-Navigation concept by IMO

Annex 2: Potential e-Navigation users

The second document proposes the framework for the implementation process for the e-Navigation and describes the different steps: strategy implementation plan, user needs, architecture, gap analysis, cost-benefit and risk analysis, implementation plan. The process is illustrated in the following figure:

The document NAV 54/WP.6, containing the two documents described above, will be provided to the e-Navigation Committee and the VTS Committee.

When the Sub-Committee invited the Committee to approve the two documents, it also agreed to invite the Committee to request other international organizations to participate in the implementation of e-navigation and thanks them, in particular IALA, for the efforts they already dedicated to the project.

In light of the foregoing, the Sub-Committee further agreed that, subject to the Committee’s approval of the e-navigation strategy, it would be necessary to have a corresponding item on the Sub-Committee’s work program to implement it. Accordingly, the Sub-Committee invited the Committee to amend the title of this work program item as “Implementation of an e-navigation strategy” with four sessions to complete the work.

3.	Ships’ routeing and related matters

3.1	AIS information and mandatory ship reporting system

During the discussions on ship routeing and ship reporting system, the Sub-Committee discussed the issue of the increasing number of mandatory ship reporting systems and whether AIS and/or LRIT could be used to satisfy the reporting requirements in such systems. The Sub-Committee agreed that:

- AIS and LRIT could not in all instances take the place of manual reporting in ship reporting systems. Manual reporting might have benefits for instance, when the mariner has to take affirmative action to report, it ensured that the person on the bridge was more alert and could be made aware of issues when entering the reporting area. Additionally, since regulation SOLAS chapter V, regulation 11, subparagraph (g) required that a system be capable of interaction, a ship reporting system might provide the mariner with valuable information, when entering a reporting area;

- there might be problems with completely replacing manual reporting within a ship reporting system with AIS because the coverage of AIS was “line of sight” on the VHF range and there were areas where AIS shoreside coverage was incomplete or non-existent; and

- Member Governments should, when considering the development of new mandatory ship reporting system proposals, consider the following issues: (a) the technological developments, in particular AIS and LRIT, and whether such developments should be implemented in the ship reporting system; and (b) there might be areas where critical navigation decisions needed to be made and therefore manual reporting in such areas might distract the mariner from these decisions.

In considering the way forward on this issue, the Sub-Committee agreed that any review should be done in the context of SOLAS chapter V, regulation 11. Furthermore, that it was premature for the Sub-Committee to undertake a full-scale review of all mandatory reporting systems as AIS and LRIT were still under development.

3.2	Traffic Separation Schemes (TSS) and other routeing measures

The Sub-Committee approved and invited the Maritime Safety Committee to adopt:

- New Traffic Separation Scheme “In the Approaches to the Port of Thessaloniki”

- New Traffic Separation Schemes for the “Aland Sea”

- New Traffic Separation Scheme in “Liverpool Bay”

- Amendments to the existing traffic separation scheme “In the approach to Boston, Massachusetts”

- Amendments to the existing Traffic Separation Scheme “Off Land’s End, between Seven Stones and Longships”

- Amendments to the existing Traffic Separation Scheme “In the Approaches to the River Humber”

- Amendments to the existing Traffic Separation Scheme “At Hatter Barn”

- Establishment of a recommendatory seasonal Area to be Avoided (ATBA) “In the Great South Channel”, off the East coast of the United States

- Establishment of an Area to be Avoided  (ATBA) and two Mandatory No Anchoring Areas in the Western North Atlantic Ocean, off the coast of the United States

- Deep-water route leading to the Aland Sea

- Two-way route leading to the Aland Sea

- Area to be Avoided (ATBA) in “Liverpool Bay”

3.3.	Mandatory ship reporting system

The Sub-Committee approved and invited the Committee to adopt:

- New mandatory ship reporting system “Off the Coast of Portugal – COPREP”

- Amendments to the existing ship reporting system for the Papahanaumokuakea Marine National Monument, “CORAL SHIREP”

4.	Electronic Chart Display and Information System (ECDIS)

At its 51st session, the Sub-Committee started to discuss the question of mandatory carriage of ECDIS on ships other than passenger vessels and high speed crafts, for which it is already compulsory. At its 52nd and 53rd sessions, discussions continued and two FSA studies were produced by Denmark together with Norway and by Japan. Both concluded that the mandatory carriage requirement for ECDIS to cargo ships was justified as being cost-effective if the ships sailed in routes or sea areas where suitable scaled ENCs are available. However, the majority of the delegations had been of the view that ENC coverage was a necessary prerequisite for the introduction of a mandatory carriage requirement of ECDIS, even if it does not mean a 100% ENC coverage.

At this 54th session IHO gave updated information on the availability of ENCs and on the development of the IHO on line catalogue of available official. In particular, the Organization confirmed that ENC production is constantly increasing and will continue to do so. Factual figures and HO official planning reports showed that the world’s major trading routes and ports will be covered by 2010.

After new extensive debates, it appeared that, this time, a large majority of those who spoke on the issue was supporting, in principle, a mandatory carriage requirement for ECDIS. However, the consensus was only possible with minimum constraints regarding ship size and type and the relevant time frames. The Sub-Committee finally agreed the following new regulation and invited the Maritime Safety Committee to approve the draft amendments to SOLAS regulation V/19:

“Chapter V

Safety of Navigation

Regulation 19

Carriage requirements for shipborne navigational systems and equipment

2.10 	Ships engaged on international voyages shall be fitted with an Electronic Chart Display and Information System (ECDIS) as follows:

.1 passenger ships of 500 gross tonnage and upwards constructed on or after 1 July 2012;

.2 tankers of 3,000 gross tonnage an upwards constructed on or after 1 July 2012;

.3 cargo ships, other than tankers, of 10,000 gross tonnage and upwards constructed on or after 1 July 2013;

.4 cargo ships, other than tankers, of 3,000 gross tonnage and upwards but less than 10,000 gross tonnage constructed on or after 1 July 2014;

.5 passenger ships of 500 gross tonnage and upwards constructed before 1 July 2012, not later than the first safety equipment survey on or after 1 July 2014;

.6 tankers of 3,000 gross tonnage and upwards constructed before 1 July 2012, not later than the first safety equipment survey on or after 1 July 2015;

.7 cargo ships, other than tankers, of 50,000 gross tonnage and upwards constructed before 1 July 2013, not later than the first safety equipment survey on or after 1 July 2016;

.8 cargo ships, other than tankers, of 20,000 gross tonnage and upwards but less than 50,000 gross tonnage constructed before 1 July 2013, not later than the first safety equipment survey on or after 1 July 2017;

9 cargo ships, other than tankers, of 10,000 gross tonnage and upwards but less than 20,000 gross tonnage constructed before 1 July 2013, not later than the first safety equipment survey on or after 1 July 2018.

2.11 	Administrations may exempt ships from the application of the requirements of paragraph 2.10 when such ships will be taken permanently out of service within two years after the implementation date specified in sub-paragraphs .5 to .9 of paragraph 2.10 above.”

5.	Shipborne Automatic Identification System matters

5.1	Impact of Resolution MEPC.118(52) upon existing AIS shipboard installations

The Sub-Committee noted that MSC 84 had referred document MSC 84/23/1 (IALA), relating to the need for possible modification of hardware of all AIS units following the entry into force of the 2004 amendments to MARPOL Annex II on 1 January 2007, to NAV 54 for consideration and instructed it to report the outcome to MEPC 58 and MSC 85.

After careful consideration, the Sub-Committee agreed with the solution proposed by IALA and ITU and decided to amend the SN.1/Circ.227 on Guidelines for the installation of a shipborne automatic identification system (annex 2). Indeed, The Sub-Committee noted that the old and new categories were broadly equivalent, and therefore it was sufficient to revise the reference documentation SN/Circ.227, annex 2 to reflect the new classification letters corresponding to the same digits as currently in use by the AIS shipborne equipment. Practically this means that the reference hazard or pollutant categories A, B, C and D are changed to the hazard or pollutant categories X, Y, Z and OS, by using the same digits 1, 2, 3 and 4.

The Sub-Committee finalized a draft liaison statement to ITU, IEC and IALA informing them of the change to hazard or pollutant categories and inviting them to note this in any future revision of their documentation.

5.2	Satellite detection of AIS

The Sub-Committee’s attention was brought on the last paragraph of a liaison statement from ITU-R WP5B’s February 2008 meeting to IMO, IALA and CIRM, concerning the Preliminary draft new report ITU-R M.(SAT-AIS) on Improved satellite detection of AIS. In particular, ITU advised that, to effect the operation of this improved satellite detection, IMO was invited to consider the need to specify modifications to the shipborne AIS Class A equipment.

The Sub-Committee noted that, up to now, the issue of satellite detection of AIS as such

had never been discussed in detail by the Committee or one of its relevant Sub-Committees

e.g., COMSAR or NAV and as such, there was no policy direction on this issue.

The Sub-Committee also noted advice on this issue received from the Joint IMO/ITU Experts Group on Maritime radiocommunication matters. The Experts Group considered it to be of importance for IMO to firstly make a clear decision whether it was supporting this issue, or whether there was no interest. Secondly, it was considered of utmost importance for IMO to inform ITU quickly of its policy direction in this regard. The Group further was of the view that there would be a need to revise the Performance Standards for AIS Class A equipment, when there would be a decision for a mandatory requirement. As a result of this, every ship would then need a software upgrade. Finally the Group advised that, in principle, everyone who would be able to receive these signals could use the information collected, also for commercial activities. In this regard, the Group recalled that MSC 79 had considered matters relating to freely available AIS generated ship data and the attendant security risks. The Committee agreed at that time that the publication on the world-wide web or elsewhere of AIS data transmitted by ships could be detrimental to the safety and security of ships and port facilities and was undermining the efforts of the Organization and its Member States to enhance the safety of navigation and security in the international maritime transport sector.

The Sub-Committee was of the opinion that for the present it was premature to further address the issue of satellite detection of AIS and agreed to:

.1 bring the matter to the attention of the Maritime Safety Committee with the aim of getting a clear policy direction; and

.2 send an interim liaison statement to ITU explaining that further discussion in IMO

was needed before any guidance on this issue could be given; the liaison statement should be sent as soon as possible in order to inform ITU-R WP5B in time for its next meeting to be held in October this year.

The Sub-Committee invited the Committee to take a clear decision on whether it was supporting the issue of satellite detection of AIS, taking into account that:

.1 in principle, everyone who would be able to receive these signals could use the information collected, also for commercial activities; and

.2 there might be a need to subsequently specify modifications to the shipborne AIS Class A equipment.

The Sub-Committee invited Member Governments also to submit proposals on the issue to MSC 85, as appropriate and instructed the Secretariat to send a liaison statement to ITU-R WP 5B, copied to IALA and CIRM.

5.3	Measures to minimize incorrect data transmission by AIS equipment

This item was still on the agenda of the Sub-Committee as requested by the Committee. The IALA delegation reminded that the Association had submitted to NAV 53 an information document reporting the result of a survey on AIS incorrect transmissions. It also recalled in this context about the recently adopted MSC Circular (MSC.1/Circ.1252) on Guidelines on annual testing of the Automatic Identification System (AIS) aimed at minimizing incorrect data transmissions by AIS equipment.

The Sub-Committee, noting that no other substantial proposals had been submitted on this issue to this session for consideration, agreed to defer further consideration of this item to NAV 55.

5.4	Revision of the Guidance on the application of AIS binary messages

As proposed by Sweden, the Maritime Safety Committee requested the NAV Sub-Committee “to review the Guidance on the application of AIS binary messages (SN/Circ.236) on the basis of operational needs and experience gained and taking into account existing technical limitations, to facilitate an effective and appropriate use of AIS binary messages and protect the main function of AIS.”

After a detailed discussion and information provided by Sweden, Germany and Japan, the Sub-Committee agreed to establish an intersessional Correspondence Group under the coordination of Sweden to make progress on the issue and report to NAV 55.

The Correspondence Group should evaluate the use of binary messages in the trial period, as identified in SN/Circ.236, and select and propose a revised set of AIS binary messages for international use. The Correspondence Group should in the selection of messages consider:

.1 the operational need;

.2 other/existing methods to fulfill the need;

.3 user interface onboard, both for presentation and input of information;

.4 the technical limitations; and

.5 graphical presentation of AIS binary messages.

The Correspondence Group should develop guidance for the use of AIS binary messages to ensure that the main functions of AIS were not impaired due to overloading of the VDL. In developing the guidance, consideration should be given to the use of binary messages defined by IMO as well as binary messages developed regionally. The Correspondence Group should further evaluate the need for additional means of communication, given the increasing demand for digital communication between shore and ships.

5.5	Presentation symbol for AIS-SART

As proposed by IEC, the Sub-Committee agreed to add a new symbol for the presentation of an AIS Search and Rescue Transmitter (AIS-SART) to table 3 of the appendix of annex 1 of the guidelines for the presentation of navigation-related symbols, terms and abbreviations (SN.1/Circ.243/Add.1).

(a circle containing a cross drawn with solid lines)

Regarding the other proposals from IEC (real and virtual AtoN symbols), the IALA delegation indicated that the Association could make further proposals to the next session taking into account different trials made, in particular in Japan.

6.	Right of way of vessels over pleasure craft

The Sub-Committee discussed a proposal from Italy to amend the COLREG Convention to give right of way of vessels over pleasure craft in areas with high density of pleasure crafts and where it is difficult to operate safely for large vessels, as some port entrances for instance.

A large number of delegations including observers spoke on the issue. In particular, the IALA delegation relayed the opinion expressed by the VTS Committee that “VTS, with a mandate to enforce compliance with the COLREGS, cannot always make a distinction between vessels being operated as pleasure craft and vessels being operated in other service”.  Finally, most of the delegations were not in favor of amending COLREG and instead favored a local approach based on national regulations, educational awareness campaigns and other practical measures aimed at recreational craft.

7.	Other items

7.1	Colour specification of lights

At NAV 52, the Norvegian delegation had proposed to amend the COLREG Convention on the colour of lights to take into account the revision of standards as decided by the International Commission on Illumination (CIE). After due consideration, at this session, the delegation of Norway stated that when raising the issue under consideration, it had been of the opinion that due to the revision of the CIE standard on Colours of Signal Lights there was an inconsistency between part 1 and part 2 of section 7 in annex I of COLREG. However, having been made aware that part 1 of section 7 did not refer to the CIE standard in its entirety,but only to the diagram specified for each colour, and as the corner coordinates in part 2 of section 7 were within the boundaries of the diagram, Norway no longer saw any inconsistencies within the regulation and thus withdrew the proposal to amend COLREG.

7.2	Improved safety of pilot transfer arrangements

The Sub-Committee considered a proposal from Brazil, the United States and IMPA outlining the fact that despite ever increasing safety efforts, pilots continued to lose their lives or suffer serious injury in the course of transferring to ships from pilot launches and cutters, using ladders. The proposal suggested to amend SOLAS regulation V/23 and/or resolution A.889(21) to improve the safety of pilot transfer operations using ladders.

Accordingly, the Sub-Committee agreed to the establishment of a correspondence group, under the coordination of the United States, to consider the proposal and to report to NAV 55.

7.3	Codes, recommendations, guidelines of non-mandatory instruments

At the request of the Maritime Safety Committee, the Sub-Committee considered a document prepared by the IMO Secretariat containing the list of codes, recommendations, guidelines and other safety and security related non-mandatory instruments relevant to the work of the Sub-Committee for review.

Due to the limited time available during the session, the Sub-Committee decided to establish a correspondence group to review the existing related documents and to develop proposals for revision or deletion where appropriate. IALA was more particularly requested to review the circulars SN/Circ.105 “IALA Buoyage System”, SN/Circ.107 “Maritime Buoyage System” and SN/Circ.120 “IALA Maritime Buoyage System agreement”.

7.4	Worldwide radionavigation system

The observer from the European Commission provided an update on the progress concerning the European Galileo navigation satellite programme. He stated that the European GNSS Supervisory Authority (GSA) had already started the certification process for GALILEO. The first phase of the programme to validate the technical options for the system and to create the conditions necessary for the rapid deployment of the infrastructure was now well underway with the successful launch of the second and last of two test satellites, GIOVE B, on 27 Apri1 2008. The full operational capacity of GALILEO was expected for 2013.

EGNOS, the former testbed of GALILEO, had now become a complete separate product, forming a so-called augmentation system as was WAAS in the United States and would be commercially exploited starting in 2009 as it was already technically operational. EGNOS was using the GPS signal, two complementary satellites and ground computations to improve the accuracy of the GPS information. It covered the whole of Europe and its coastal waters, would be extended in the near future to the whole of the Mediterranean Sea, and might later cover also Africa. EGNOS should remain operational for at least 20 years.

8.	Recommended actions for IALA

It is recommended that:

8.1.	the Council

- note the election of Mr. Mike Sollosi as chairman of the Sub-Committee on Safety of Navigation (Introduction)

- note the adoption of the strategy for e-Navigation and the framework for the implementation process for the e-Navigation concept (item 2)

- note the decision on mandatory carriage of ECDIS and the implementation timeframe (item 4)

- note the request addressed to MSC 85 to open discussion on the opportunity to develop satellite detection of AIS (item 5.2)

- note the decision not to amend COLREG to give more way of vessels over pleasure crafts (item 6).

- note the information provided on the development of Galileo and EGNOS (item 7.4).

8.2.		the e-Navigation Committee

- note the adoption of the strategy for e-Navigation and the framework for the implementation process for the e-Navigation concept – (References item 2 and document NAV 54 WP.6))

- note the discussion on AIS information and mandatory ship reporting system (item 3.1)

- note the decision on mandatory carriage of ECDIS and the implementation timeframe (item 4)

- note the decision regarding the IALA submission to MSC 84 on impact of Resolution MEPC.118(52) upon existing AIS shipborne installation (item 5.1)

- note the request addressed to MSC 85 to open discussion on the opportunity to develop satellite detection of AIS – If needed a brief could be prepared for the IALA delegation (item 5.2)

- note the discussion on minimizing incorrect data transmission by AIS equipment and, if needed, prepare a submission to NAV 55 (item 5.3)

- revision of the Guidance on the application of AIS binary messages: provide input to the correspondence group (item5.4)

- note the decision on presentation symbol for AIS SART and prepare a submission on real and virtual AtoN symbols to NAV 55 (item 5.5)

- note the information provided on the development of Galileo and EGNOS (item 7.4).

8.3.    the VTS Committee

- note the election of Mr. Mike Sollosi as chairman of the Sub-Committee on Safety of Navigation (Introduction)

- note the adoption of the strategy for e-Navigation and the framework for the implementation process for the e-Navigation concept – (References item 2 and document NAV 54 WP.6))

- note the discussion on AIS information and mandatory ship reporting system (item 3.1)

- note the discussion on minimizing incorrect data transmission by AIS equipment and, if needed, prepare a submission to NAV 55 (item 5.3)

- note the decision on presentation symbol for AIS SART and prepare a submission on real and virtual AtoN symbols to NAV 55 (item 5.5)

- note the decision not to amend COLREG to give more way of vessels over pleasure crafts (item 6).

8.4.		the Aids to Navigation Management Committee

- note the adoption of the strategy for e-Navigation and the framework for the implementation process for the e-Navigation concept – (References item 2 and document NAV 54 WP.6))

- note the discussion on AIS information and mandatory ship reporting system (item 3.1)

- revision of the Guidance on the application of AIS binary messages: provide input to the correspondence group (item5.4)

- note the decision on presentation symbol for AIS SART and prepare a submission on real and virtual AtoN symbols to NAV 55 (item 5.5)

- note the review of codes, recommendations and guidelines of non-mandatory instruments and the request to assess the SN Circ. 105, 107 and 120 (item 7.3).

- note the information provided on the development of Galileo and EGNOS (item 7.4).

8.5 	the Engineering, Environment and Preservation Committee

- note the discussion on colour specification lights (item7.1)

- note the review of codes, recommendations and guidelines of non-mandatory instruments and the request to assess the SN Circ. 105, 107 and 120 (item 7.3).

8.6	the Pilot Authority Forum

- note the decision to develop SOLAS amendment on pilot transfer arrangements (item 7.2)

9.	Date of the next session.

The 55th session of the NAV Sub-Committee is tentatively scheduled to be held from 27 to 31 July 2009 in London.

The IMO Secretariat should receive documents intended for consideration by the Sub-Committee approximately as follows:

(a)	those containing more than 6 pages of  text (bulky documents) and those which 	require	 action or decision by the 25th of April 2009;

(b)	other documents by the 21th of May 2009.

The proposed agenda for the session is attached as annex.

* * *

J.Ch. Leclair

Accredited Representative of IALA to IMO,

14 July 2008.

### A N N E X

Sub-Committee on Safety of Navigation (NAV) – 55th session - Agenda proposed to the Maritime Safety Committee

Opening of the session

1	Adoption of the agenda

2	Decisions of other IMO bodies

3 	Routeing of ships, ship reporting and related matters

4	Development of guidelines for IBS, including performance standards for bridge alert management

5	Guidelines for consideration of requests for safety zones larger than 500 metres around artificial islands, installations and structures in the EEZ

6	Amendments to the performance standards for VDR and S-VDR

7	Development of procedures for updating shipborne navigation and communication equipment

8	Safety provisions applicable to tenders operating from passenger ships

9	ITU matters, including Radiocommunication ITU-R Study Group 5 matters

10	Code of conduct during demonstrations/campaigns against ships on high seas

11	Measures to minimize incorrect data transmissions by AIS equipment

12	Implementation of an e-navigation strategy

13	Guidelines on the lay-out and ergonomic design of safety centers on passenger ships

14 	Review of vague expressions in SOLAS regulation V/22

15	Revision of the Guidance on the application of AIS binary message

16	Improved safety of pilot transfer arrangements

17	Casualty analysis

18	Consideration of IACS unified interpretation

19	Work program and agenda for NAV 56

20	Election of Chairman and Vice-Chairman for 2010

21	Any other business

22	Report to the Maritime Safety Committee

__________

Coordinator:

Mr. Rolf Zetterberg

Senior Technical Advisor, MSc

Ship Traffic Division

Swedish Maritime Safety Inspectorate

Östra Promenaden 7

SE-601 78 Norrköping

Sweden

Telephone: +46 11 19 15 12

Fax: +46 11 19 12 30

Mobile: +46 708 19 15 12

E-mail: inspektion@sjofartsverket.se

Email directly: rolf.zetterberg@sjofartsverket.se