Chapter on e-Navigation

for the

IALA NAVGUIDE

2010 Edition

Prepared by:   Nick Ward and Mahesh Alimchandani

Table of Contents

1	INTRODUCTION	4

2	BACKGROUND	4

3	KEY ELEMENTS	5

3.1	Definition	5

3.2	A vision of e-Navigation	5

4	STRATEGY AND IMPLEMENTATION	6

High Level User Needs	6

5	IALA’s ROLE	8

6	ARCHITECTURE	9

7	TECHNOLOGY FOR E-NAVIGATION	12

7.1	IALA Plan	12

7.2	Electronic Position Fixing Systems	12

7.2.1	Global Navigation Satellite Systems (GNSS)	12

7.3	Differential GNSS	15

7.3.1	IALA Beacon DGNSS	16

7.4	RAIM	18

7.5	Terrestrial – eLoran	18

7.5.1	Introduction	18

7.5.2	Background	18

7.5.3	eLoran Performance	19

7.5.4	Core eLoran elements	19

7.5.5	Compatibility between eLoran and Loran-C	19

7.5.6	eLoran as a viable backup to GNSS	20

7.7	Radar Aids to Navigation	20

7.7.1	New Technology Radars	20

7.7.2	Radar Reflectors	21

7.7.3	Radar Target Enhancers	21

7.7.4	Radar Beacon (Racon)	21

7.7.5	Frequency-Agile Racon	22

7.7.6   Performance Criteria	23

7.7.7	Technical Considerations	23

7.7.8	  Use with New Technology Radars	23

7.8	Non-radio positioning (Inertial)	24

8	Communications	24

8.1	IALA Plan	24

8.2	Long Range Identification and Tracking	25

8.2.1	Introduction	25

8.2.2	LRIT Concept	25

8.2.3	LRIT Performance Standards and Functional Requirements	25

9	AIS	27

9.1	Overview	27

9.2	Purpose & Function	27

9.3	System Characteristics	28

9.3.1	Frequencies and capacity	28

9.3.2	Shipboard AIS	29

9.3.3	Information available	29

9.3.4	Shore based AIS	30

9.4	AIS as an aid to navigation	30

9.5	Carriage Requirements	31

9.6	Cautions when using AIS	31

9.7	Strategic Applications of AIS	31

9.8	International Standards, Recommendations and Guidelines	32

9.8.1	IMO Documentation	32

9.8.2	IALA Documentation	32

9.8.3	ITU Documentation	33

9.8.4	IEC Standards	33

10	ECDIS	34

10.1	Electronic Chart Display and Information System (ECDIS)	34

10.2	Performance Standards	34

10.3	Performance Elements	34

11	Maritime Information Systems	35

11.1	Maritime Information Structure	35

11.2	AtoN Attribute Information	35

11.3	Meteorological & Hydrological Information	35

11.4	Intergovernmental Oceanographic Commission	36

Chapter XX 	e-Navigation

# INTRODUCTION

e-Navigation is an IMO-led broad strategic vision for the harmonisation of marine navigation systems and supporting shore services, underpinned by user needs.

The concept involves the utilization and integration of all available navigational tools to secure a greater level of safety and accident prevention.  Implementation of e-Navigation will, at the same time, deliver substantial operating efficiencies with resulting commercial benefits.

e-Navigation will incorporate the use of new technologies in a structured way and ensure that their use is compliant with the various electronic navigational and communication technologies and services that are already available.

It should be noted that without e-Navigation, the multiplicity of systems and equipment will continue to evolve, with varying degrees of effectiveness.  e-Navigation is an opportunity to optimise these developments and ensure the focus of future developments is based on a holistic approach to safe berth-to-berth navigation.

# BACKGROUND

Rising trends of marine accidents both in terms of numbers and costs are mainly

associated with collisions and groundings. There are numerous examples of collisions and

groundings that might have been avoided had there been suitable input to the navigation

decision-making process.

Research indicates that around 60% of collisions and groundings are caused by direct

human error. Despite advances in bridge resource management training, it seems that the

majority of watchkeeping officers make critical decisions for navigation and collision avoidance in isolation, due to a general reduction in manning.

In human reliability analysis terms, the presence of someone checking the decision-making

process improves reliability by a factor of 10. If e-navigation could assist in improving this aspect, both by well-designed onboard systems and closer cooperation with vessel traffic

management (VTM) instruments and systems, risk of collisions and grounding and their inherent liabilities could be dramatically reduced.

However, although e-navigation may be able to improve the situations described above,

there is also a need to recognize the role of the practice of good seamanship, the provision of

suitable training and the use of procedures.

In 2006, seven IMO Member States made a joint submission to the Maritime Safety Committee to “develop a strategic vision for the utilization of existing and new navigational tools, in particular electronic tools, in a holistic and systematic manner.”

# KEY ELEMENTS

## Definition

The definition of e-Navigation adopted by IMO is:

“e-Navigation is the harmonised collection, integration, exchange, presentation and analysis of maritime information onboard and ashore by electronic means to enhance berth to berth navigation and related services, for safety and security at sea and protection of the marine environment”

WHAT DOES THE “e” IN e-NAVIGATION STAND FOR?

It is generally accepted that the IMO concept of “e-Navigation” can be thought of as a brand, without the need for “e” to be specifically defined.

The concept of e-Navigation was proposed by IMO Member States in 2006 as a process for the harmonisation, collection, integration, exchange and presentation of maritime information. As such, the “e” could have stood for “enhanced” or “electronic”, but this would unnecessarily limit what can be done within e-Navigation. It must be noted that generic electronic marine navigation already exists in many forms and should not be confused with this specific IMO initiative.

## A vision for e-Navigation

A vision of e-navigation is embedded in the following general expectations for the

onboard, ashore and communications elements:

.1 On board

Navigation systems that benefit from the integration of own ship sensors,

supporting information, a standard user interface, and a comprehensive system for

managing guard zones and alerts. Core elements of such a system will include,

actively engaging the mariner in the process of navigation to carry out his/her

duties in a most efficient manner, while preventing distraction and overburdening;

.2 Ashore

The management of vessel traffic and related services from ashore enhanced

through better provision, coordination, and exchange of comprehensive data in

formats that will be more easily understood and utilized by shore-based operators

in support of vessel safety and efficiency; and

.3 Communications

An infrastructure providing authorized seamless information transfer on board

ship, between ships, between ship and shore and between shore authorities and

other parties with many related benefits.

# STRATEGY AND IMPLEMENTATION

In 2008, IMO approved the development of an e-Navigation Strategy Implementation Plan.  This includes the development of a technical architecture, gap analysis, cost benefit analysis and the creation of a detailed implementation plan.  A structured and phased approach is required to capture evolving user needs, making use of the existing agreed methodology.

The Strategy Implementation Plan includes priorities for deliverables, a schedule for implementation and provision for the continual assessment of user needs.   Implementation will be a phased, iterative process.  The architecture for e-navigation will encompass hardware, data, information, communications technology and software.  It will be based on a modular and scalable concept and will cater for continued development and enhancements. The initial architecture is to be ready for a coordinated review by 2009 and completed by 2010.

The deployment of new technologies is to be based on a systematic assessment of how the technology can best meet defined and evolving user needs within the e-navigation concept.

The initial gap analyses are to be focused on technical, regulatory, operational and training aspects and are to be completed by 2010.

Cost-benefit and risk analyses will be used to support strategic decisions, as to when certain functions need to be enabled. The analyses will address financial and economic aspects, as well as assess the impact on safety, security and the environment. The aim is to complete this by the year 2011.

Implementation of the e-navigation plan is due to begin in 2012 and will include: identification of responsibilities of the appropriate organizations/parties; transition planning; and a phased implementation schedule, along with possible roadmaps to clarify common understanding necessary for implementation.

High Level User NeedsThe IALA methodology was used to capture evolving user needs. It was based on the elements contained within the accepted definition of e-navigation and applied templates to define specific user needs based on the harmonized collection, integration, exchange, presentation, analysis and human element aspects for all users. Following extensive feedback from Member States, other maritime organizations, and interested parties, an analysis was conducted resulting in the identification of high-level generic user needs for both ship and shore users. Thus the needs of a typical SOLAS ship and a generic shore authority have been used as the basis for the identification of the high-level user needs reproduced below. A more detailed user needs may have to be identified as a part of the implementation plan.

.1 Common Maritime Information/Data Structure

Mariners require information pertaining to the planning and execution of voyages,

the assessment of navigation risk and compliance with regulation. This

information should be accessible from a single integrated system. Shore users

require information pertaining to their maritime domain, including static and

dynamic information on vessels and their voyages. This information should be

provided in an internationally agreed common data structure. Such a data

structure is essential for the sharing of information amongst shore authorities on a

regional and international basis.

.2 Automated and Standardized Reporting Functions

E-navigation should provide automated and standardized reporting functions for

optimal communication of ship and voyage information. This includes

safety-related information that is transmitted ashore, sent from shore to shipborne

users and information pertaining to security and environmental protection to be

communicated amongst all users. Reporting requirements should be automated or

pre-prepared to the extent possible both in terms of content and communications

technology. Information exchange should be harmonized and simplified to reduce

reporting requirements. It is recognized that security, legal and commercial issues

will have to be considered in addressing communications needs.

.3 Effective and Robust communications

A clear need was expressed for there to be an effective and robust means of

communications for ship and shore users. Shore-based users require an effective

means of communicating with vessels to facilitate safety, security and

environmental protection and to provide operational information. To be effective,

communication with and between vessels should make best use of audio/visual

aids and standard phrases to minimize linguistic challenges and distractions to

operators.

.4 Human Centred Presentation Needs

Navigation displays should be designed to clearly indicate risk and to optimize

support for decision making. There is a need for an integrated “alert management

system” as contained in the revised recommendation on performance standards for

Integrated Navigation Systems (INS) (resolution MSC.252(83)). Consideration

should be given to the use of decision support systems that offer suggested

responses to certain alerts, and the integration of navigation alerts on board ships

within a whole ship alert management system. Users require uniform and

consistent presentations and operation functionality to enhance the effectiveness

of internationally standardized training, certification and familiarization. The

concept of S-Mode has been widely supported as an application on board ship

during the work of the Correspondence Group. Shore users require displays that

are fully flexible supporting both a Common Operating Picture (COP) and a User

Defined Operating Picture (UDOP) with layered and/or tabulated displays. All

displays should be designed to limit the possibility of confusion and

misinterpretation when sharing safety-related information. E-navigation systems

should be designed to engage and motivate the user while managing workload.

.5 Human Machine Interface

As electronic systems take on a greater role, facilities need to be developed for the

capture and presentation of information from visual observations, as well as user

knowledge and experience. The presentation of information for all users should

be designed to reduce “single person errors” and enhance team operations. There

is a clear need for the application of ergonomic principles both in the physical

layout of equipment and in the use of light, colours, symbology and language.

.6 Data and System Integrity

E-navigation systems should be resilient and take into account issues of data

validity, plausibility and integrity for the systems to be robust, reliable and

dependable. Requirements for redundancy, particularly in relation to position

fixing systems, should be considered.

.7 Analysis

E-navigation systems should support good decision making, improve performance

and prevent single person error. To do so, shipboard systems should include

analysis functions that support the user in complying with regulations, voyage

planning, risk assessment, and avoiding collisions and groundings including the

calculation of Under Keel Clearance (UKC) and air draughts. Shore-based

systems should support environmental impact analysis, forward planning of vessel

movements, hazard/risk assessment, reporting indicators and incident prevention.

Consideration should also be given to the use of analysis for incident response and

recovery, risk assessment and response planning, environment protection

measures, incident detection and prevention, risk mitigation, preparedness,

resource (e.g., asset) management and communication.

.8 Implementation Issues

Best practices, training and familiarization relating to aspects of e-navigation for

all users should be effective and established in advance of technical

implementation. The use of simulation to establish training needs and assess its

effectiveness is endorsed. E-navigation should as far as practical be compatible

forwards and backwards and support integration with equipment and systems

made mandatory under international and national carriage requirements and

performance standards. The highest level of interoperability between e-navigation

and external systems should be sought where practicable.

# IALA’s ROLE

IALA’s e-Navigation Committee has contributed substantially to the formulation of the IMO Strategy for the implementation of e-Navigation and to the Strategy Implementation Plan. The working groups of the e-NAV Committee are developing shore user requirements, information systems and data structures, a World Wide Radio Navigation Plan, a Radio Communications Plan, a future second generation AIS and a shore-side architecture, together with a documentation structure to encompass the whole of e-Navigation.

# ARCHITECTURE

A representation of the ship to shore portion of the  proposed e-navigation architecture being developed by IALA, under the IMO e-navigation agenda, is shown in the figure overleaf.  It includes the shipboard entities, the physical link(s) and the shore-based entities.

UMDM

UMDM

Ship

environment

IBS

World Wide Radionavigation System (WWRNS) of IMO, including GNSS, GNSS augmentation and terrestrial backup

Transceiver station

Ship’s

sensors

Shipboard

Applications

INS

Shore-based

technical

e-Navigation

services

(= shore-based

applications)

Application-to-

application

functional

connection

Physical

Link (e.g. radio link)

other

ships

other

ships

mariner

Shore- based users

(e.g. VTS Operators)

Shore

environment

From an e-Navigation perspective, the relevant devices within the ship environment are the transceiver station, the data sources and the data sinks connected to the transceiver station,  the Integrated Navigation System (INS) and the Integrated Bridge System (IBS). Only one transceiver station is shown for simplicity, although there may be several.

The shipboard transceiver station interfaces with the physical link(s) to the appropriate technical e-Navigation services ashore.

The shore-based technical e-Navigation services provide the interfaces of the shore-based user applications to the physical link(s). The data transfer network(s) depend on the application, for example, a shore-based radar service would require different data transfer network capabilities to a shore-based AIS service.

The figure shows the IMO World Wide Radio Navigation System (WWRNS) as a system “external” to the e-Navigation architecture, providing position and time information. The WWRNS includes GNSS, augmentation systems and terrestrial backup radio navigation systems.

# TECHNOLOGY FOR e-NAVIGATION

Many sub-systems or components will have to be developed, enhanced or provided to realise the full integrated concept of e-Navigation. Crucial among these are Positioning, Navigation and Timing (PNT), communications and information systems.  It should be noted that not all of the following technologies or systems will be needed within the e-Navigation concept.

## IALA Plan

IALA has developed a World Wide Radio Navigation Plan (WWRNP) aimed at providing the WWRNS to support e-Navigation. One key concept in this plan is the separation of the generation of correction data from the means of transmission, to facilitate broadcasting by a variety of methods. This could lead to the integration of terrestrial systems (DGNSS beacons, e-Loran, AIS) to provide shared data channels and common correction sources, as well as additional ranging signals, contributing to a redundant position-fixing solution, complementary to, but independent of GNSS.

Future standards for position-fixing systems should be considered in the context of position-fixing requirements for e-Navigation. This WWRNP could be the basis for a submission to IMO as a contribution to the WWRNS. The plan provides guidance to IALA members regarding potential future developments, which will enable members to identify areas requiring resource allocation and research activity.

## Electronic Position Fixing Systems

### Global Navigation Satellite Systems (GNSS)

Global Navigation Satellite System (GNSS) is a generic term for a satellite system that provides a world-wide position determination, time and velocity capability, for multi-modal use.

GNSS is based on a constellation of active satellites, which continuously transmit coded signals in one or more frequency bands. These signals can be received by users anywhere on the earth’s surface to determine their position and velocity in real time, based on ranging measurements.

If a GNSS conforms to IMO Resolution A.953(23) for a World-Wide Radionavigation System (WWRNS), the receivers of that GNSS will satisfy the IMO carriage requirements for position fixing equipment referred to in Chapter V of the SOLAS Convention.

Several Global Navigation Satellite Systems (GNSS) have been deployed, fully or partially, or are under development.

Since 1996, the US “Navstar” Global Positioning System (GPS) and the Russian Global Orbiting Navigation Satellite System (GLONASS) have been recognised as components of the WWRNS.  In the future, GNSS will include other systems such as the “GALILEO”, a system currently under construction by the European Union, and “COMPASS”, currently under construction by China. Furthermore, regional GNSS components like “QZSS” from Japan and IRNSS from India are planned to become operational in the next few years.

#### GPS

The Global Positioning System, Standard Positioning Service (GPS SPS) is a three-dimensional positioning, three-dimensional velocity and time system that became fully operational in 1995.  The system is operated by the United States Air Force on behalf of the United States Government.

The U.S. Government provides two levels of GPS service. The Precise Positioning Service (PPS) provides full system accuracy to designated users. The Standard Positioning Service (SPS) provides accurate positioning to all users.

The GPS has three major segments: space, control, and user. The GPS Space Segment consists of a nominal constellation of 24 satellites in six orbital planes. The satellites operate in circular 20,200 km (10,900 nm) orbits at an inclination angle of 55 degrees and with a 12-hour period.

The GPS SPS is available on a non-discriminatory basis and free of direct user fees to all users with an appropriate receiver.  The service satisfies the requirements for general navigation and harbour approach with a horizontal position accuracy of 9 metres (95% probability)2.

GPS receivers, in combination with other equipment, are able to provide:

- absolute positioning

- relative positioning (this can be further processed to derive speed over ground (SOG) , course over ground (COG), etc.)

- timing

This information may refer to a stationary observer (static positioning) or to a moving observer (kinematic positioning).

There are 31 usable satellites in the constellation at present. A modernization program aims to improve the accuracy and availability for all users and involves new ground stations, new satellites, and four additional navigation signals: three new civilian signals known as L2C, L5 and L1C and a new military code called M-Code. Full Operational Capability of the L2C code is not expected before 2014 and for L5 before 2016.

The next generation of satellites, GPS III, is in its definition phase (timeframe to 2021).

Further information on GPS can be found on the USCG NAVCEN website http://www.navcen.uscg.gov.  The website also has a link to the latest United States Federal Radionavigation Plan that provides a comprehensive account of current and future developments for GPS.

#### GLONASS

The Global Navigation Satellite System (GLONASS) is a three-dimensional positioning, velocity and time system managed by the Russian Space Agency for the Russian Federation.

GLONASS has a similar potential user community to GPS SPS.  It is available on a non-discriminatory basis and free of direct user fees to all users with an appropriate receiver.  With a full complement of 24 satellites, the service would satisfy the requirements for general navigation and give a horizontal position accuracy of 45 metres (95% probability). Further satellite launches planned for 2009-10 would be expected to restore the full complement of satellites.

Recent launches have included the improved GLONASS M satellites with a second civil signal. The new GLONASS-K with a third civil signal on L3, was due to start launching in 2009. It will also carry differential corrections, integrity information and search and rescue functions. Present satellites use Frequency Division Multiple Access, but the future GLONASS K-M system, which is at the requirement definition stage, is planned to include a code division multiple access (CDMA) signal, inter-operable with GPS.

Further information on GLONASS and future developments can be found on the Russian Space Agency, Information Analytical Centre website  http://www.glonass-ianc.rsa.ru

#### Galileo

Galileo, the future European satellite navigation system, will be under civilian control.

Galileo is expected to have a constellation of 30 satellites, of which 27 will be operational, with three active spares. These will be positioned in a Medium Earth Orbit (MEO) with an inclination of 56o to the equator. These satellites will provide dual frequencies as standard, enabling users to calculate their positions with metre level accuracy.

Galileo will provide five levels of service, each with their own performance standards: Open Service (OS), Safety-of-Life Service (SoL), Commercial Service (CS), Public Regulated Service (PRS) and Search and Rescue Service (SAR).  Galileo is expected to provide timely warnings of integrity failure, within a few seconds.  Among its novel features will be a system to relay distress messages to COSPAS-SARSAT service centres, while keeping the user informed.

Two test-bed satellites, GIOVE A and GIOVE B are in orbit, providing signals for test purposes, including navigation signals. The first launch of an operational Galileo satellite is scheduled for 2010 and full operational capacity is planned for 2013. Agreement has been reached with the US on interoperability and cooperation with GPS.

Further information on Galileo can be found on the Internet http://europa.eu.int/comm/dgs/energy_transport/galileo

#### Beidou/Compass

China is currently engaged in the development of a global system, complemented by a regional component. The first four COMPASS satellites were GEOs (Geo-stationary Orbit).  Then, in April 2007, the first MEO (Medium Earth Orbit) was launched.

The system currently uses two GEOs, with a third as a backup, to provide 100 m horizontal accuracy. Beidou 2 may be a full 30 MEO constellation, or 4 GEOs and 12 MEOs, or some other combination of MEOs, GEOs and inclined geo-stationary orbit satellites (IGSO), with a proposed operational date of 2015.

#### QZSS

Japan is developing a Quasi-Zenith Satellite System (QZSS).   QZSS is based on 3 satellites in  highly elliptical, inclined orbits guaranteeing one satellite always in visibility of Japan with a minimum elevation angle of 60 degrees. Each satellite will transmit 6 signals in the L-band: 3 in L1, one in E6, one in L2 and one in L5.

One of the signals aims to provide sub-metre accuracy and integrity while maintaining compatibility with SBAS. The signal in E6 aims to support a commercial service with high data rate (2 kbps). The other signals are GPS-like signals, including L2C and L1C standards.

#### IRNSS

The Indian Regional Navigational Satellite System (IRNSS) will be an independent navigation system covering the Indian region through a space segment of 3 GEO satellites and 4 IGSO satellites. The inclination of the orbital plane of the IGSO satellites is low, so that all the satellites can be seen simultaneously over India. It is planned to launch the first IRNSS satellite in 2009, with system completion by 2011.

Three IRNSS services are anticipated:

• Open Service using signals in the L5 and S bands.

• Precise Positioning Service using signals in the L5 and S bands.

• Restricted Access Service using signals in the L5 band only.

The Open and Precise services target dual frequency users but it is also intended to compute and broadcast ionosphere-corrections to support single frequency users. Owing to the limited coverage of the IRNSS network of reference stations the satellites will, apart from the navigation payload, also include a dedicated C-band uplink/down-link ranging payload to support precise satellite orbit determination.

## Differential GNSS

Differential GNSS is an augmentation system for providing integrity and reducing the errors in the GNSS signals within a given area.  The process involves comparing the accurately surveyed position of the DGNSS Reference Station with positions determined from the GNSS satellites in view.  Messages containing positional errors and satellite integrity (health) information are broadcast for users equipped with appropriate receivers.  The result for the user is:

- enhanced positional accuracy within the service area, and;

- almost immediate notification of faulty satellites (compared with up to two hours for GPS).

In addition to GPS and GLONASS, the DGNSS augmentation system will be able to provide similar advantages to future GNSS signals (e.g. Galileo OS signals).

DGNSS services can be provided from Ground based augmentation systems (GBAS) using terrestrial transmitters such as the IALA beacon system and AIS or from Satellite Based Augmentation Systems (SBAS).

### IALA Beacon DGNSS

The internationally accepted method of providing DGNSS corrections to maritime users is by local broadcast stations transmitting non-encrypted corrections on frequencies within the maritime radionavigation band (283.5 to 325 kHz).

The radio link of this system is internationally defined at ITU (Recommendation ITU-R M.823-3). Type-approved DGNSS radiobeacon receivers meeting IEC 61108-4 test and performance standards are available. The IALA system provides DGNSS correction transmissions as well as a data channel for text messages (ASCII) and is available in most coastal waters, especially in areas of high traffic density.

IALA has recommended that members operating the beacon DGNSS service should consider modernization, to ensure that levels of service can be maintained and future requirements can be met (IALA Recommendation R-135). This recommendation takes into account the potential alternatives, in particular Satellite Based Augmentation Services (SBAS) and the Automatic Identification System (AIS). The simplest option may be replacement of existing hardware with similar dedicated Reference Stations and Integrity Monitors (RSIM), but this would limit the potential for development.  There are other possibilities, including software RSIM, Virtual Reference Station Networks and integration with SBAS.

The full list of about 400 maritime radiobeacon based DGNSS stations (as notified to IALA by authorities) can be accessed via the IALA website  http://www.iala-aism.org.

Further information on DGPS can be found on the USCG Navigation Centre website http://www.navcen.uscg.gov

#### DGNSS using AIS Message No. 17

Automatic Identification System (AIS) is a ship to ship and ship to shore data broadcast system, operating in the VHF maritime band. It is described in more detail in section XX

AIS has the capability of providing DGNSS corrections to onboard equipment using standardised transmissions (Message No 17) as described in IALA Recommendation A-124.

The use of AIS Message No 17 increases the number of vessels, which benefit from DGNSS transmissions with respect to better accuracy and integrity.

#### SBAS

Satellite Based Augmentation Systems support wide-area or regional augmentation through the use of additional satellite-broadcast messages. Such systems are commonly composed of multiple ground stations, located at accurately-surveyed points. The ground stations take measurements of the GNSS satellite signals and environmental factors which may impact the signal received by the users. Using these measurements, information messages are created and sent to one or more GEO satellites for broadcast to the end users.

#### WAAS

The Wide Area Augmentation System (WAAS) has been implemented by the US FAA to support the use of GPS for general and commercial aviation over continental United States. It was recently extended to cover parts of Mexico and Canada. At present, the WAAS architecture includes 38 reference stations, 3 master stations, 4 up-link stations, 2 geostationary satellite links and 2 operational control centres.

Further information on WAAS can be found on the USCG Navigation Centre website http://www.navcen.uscg.gov

#### EGNOS

The European Geo-stationary Navigation Overlay Service (EGNOS) is a joint project of the European Space Agency (ESA), the European Commission (EC) and Eurocontrol.  It consists of three GEOs and a network of ground stations and transmits correction and integrity information for GPS and potentially other GNSS, designed for safety-of-life applications (i.e. civil aviation). EGNOS is currently in its initial operational phase with discussions on-going to secure the long-term operation of the system. The system currently provides an SBAS service in the L1 band through two GEO transponders on INMARSAT satellites. A third GEO transponder on ARTEMIS is used to support system upgrades. EGNOS is presently fully interoperable with the current generation of WAAS and MSAS. However, these systems are already planning an evolution towards a dual-frequency capability in the L1/L5 bands.

Further information on EGNOS can be found via website: http://www.egnos-pro.esa.int

#### MSAS

In Japan, the Multi-Satellite Augmentation System (MSAS) is an SBAS similar to EGNOS and WAAS. MSAS has been commissioned for aviation use, with two GEO-links using the L1 band via dedicated satellites shared with communications and meteorological missions.

Further information on MSAS can be found via the website: http://www.kasc.go.jp/_english/msas_01.htm

#### GAGAN

India is developing a GPS-Aided Geo Augmented Navigation system (GAGAN), which is an SBAS similar to WAAS and EGNOS. It is currently at the end of its technology demonstration phase. GAGAN includes 8 reference stations, 1 mission control centre, 1 up-link station and 1 Geo link through the L1/L5 transponder on the INMARSAT 4-F1 satellite. Full operational capability has been announced for 2010, when the system would have been extended with an additional master control centre, an additional up-link station, 2 additional GEO links (L1/L5) plus one in-orbit spare and more reference stations.

#### SDCM

Russia is also considering an augmentation to GLONASS called SDCM (System for Differential Corrections and Monitoring), which appears to be in a preliminary phase. The concept originally diverged substantially from SBAS standards but it seems that there is an interest in converging towards a more interoperable system.

## RAIM

Receiver Autonomous Integrity Monitoring (RAIM) is a technology developed to assess the integrity of GNSS signals in a receiver. It is of special importance in safety-critical applications, such as aviation and marine. RAIM uses redundant measurements in a navigation receiver to confirm the integrity of the derived position solution. Usually RAIM uses only “Fault Detection” (FD), but newer receivers incorporate “Fault Detection and Exclusion” (FDE) which enables them to continue to operate in the presence of a single GNSS satellite failure. Because RAIM operates autonomously it requires a number of redundant pseudorange measurements. To detect a faulty satellite, at least five measurements are required, and to isolate and exclude a faulty space vehicle, at least six measurements are required. The need for redundant measurements reduces RAIM availability and continuity, especially in shadowed areas.  Depending on the satellite geometry more measurements may be needed to perform the RAIM process with an appropriate confidence level.

## Terrestrial – eLoran

### Introduction

Enhanced Loran (eLoran) is a development from the long-standing, low-frequency, Loran-C (LOng-RAnge Navigation) system.  It is intended to be an internationally-standardised Positioning, Navigation, and Timing (PNT) service for use by land, sea and air navigation as well as other applications, including timing. eLoran is an independent, dissimilar, complement to Global Navigation Satellite Systems like GPS and Galileo. It will allow GNSS users to retain the safety, security, and economic benefits of GNSS, even when their satellite services are disrupted.

### Background

The present Loran–C is a hyperbolic radionavigation system developed during the 1960s to meet U.S. Department of Defense requirements. The Russian Federation operates a similar radionavigation system called CHAYKA. There are currently about 24 Loran–C and CHAYKA chains operating around the world. The principal coverage areas include the USA, Canada, Saudi Arabia, China Sea, Korea, North West Pacific, Russian Federation and North West Europe.

Present Loran–C chains comprise between three to five stations that have a spacing of 600 to 1000 nautical miles. The signal format is a structured sequence of brief radio pulses on a carrier wave frequency centred on 100kHz. One of the stations is designated as the ‘master’ and transmits groups of 9 pulses. The other stations are called ‘secondaries’ and these transmit groups of 8 pulses.

The spacing between groups of pulses is a characteristic unique to each chain and is referred to as the Group Repetition Interval (GRI).

The carrier wave frequency favours the propagation of a stable ground wave over long distances. Loran receivers are designed to determine positions using the ground wave and reject the delayed sky wave that would potentially distort the received signal.

The transmissions from each chain are monitored and controlled continuously. System abnormality indicators are built into the signal format and can be identified by the receiver providing inherent integrity warnings.

### eLoran Performance

eLoran will meet the accuracy, availability, integrity and continuity performance requirements for aviation non-precision instrument approaches, maritime harbour entrance and approach manoeuvres, land-mobile vehicle navigation, and location-based services.  It is also a precise source of time and frequency for applications such as telecommunications. eLoran is predicted to provide accuracy (8 - 20 metres) and timing (stratum-1) performance equivalent to current GPS.

| eLoran Predicted Performance |  |  |  |
| --- | --- | --- | --- |
| Accuracy | Availability | Integrity | Continuity |
| 8 – 20 metres | 99.9% - 99.99% | 1 x 10-7 per hour | 99.9% to 99.99% over 150 seconds |

Source: International Loran Association: eLoran Definition document

### Core eLoran elements

eLoran comprises a core system (modernised control centres, transmitting stations and monitoring sites) and application-specific augmentations (e.g. differential reference stations) that will use a data channel on the eLoran signal to communicate with users. eLoran transmissions are synchronised to an identifiable, publicly-certified, source of Coordinated Universal Time (UTC) by a method wholly independent of GNSS. This allows the eLoran Service Provider to operate on a time scale that is synchronised with, but operates independently of, GNSS time scales. Synchronising to a common time source also allows receivers to employ a mixture of eLoran and satellite signals.

eLoran transmitters use modern solid-state components which are efficient, reliable and stable. This produces a highly stable eLoran signal. The lower power usage of these modern transmitters allows Uninterruptible Power Supplies (UPS) to be installed to overcome short periods of utility power outage, again ensuring a more stable and reliable signal. The use of reliable modern components increases the Mean Time between Failures and with in-built redundancy, allows eLoran stations to be un-manned. This ensures the system is a cost effective solution as backup to GNSS.

Absolute position offsets are caused by ASF (Additional Secondary Factors). These result from differing speeds of signal propagation over different terrains as well as the interfaces between those terrains e.g. land/sea path, mountains/wetlands. These terrains delay signal reception from that expected of a perfect sea path and is the main component of ASF. However, seasonal (summer/winter) and diurnal (day/night) changes to the ASF value will also occur. Modern eLoran receivers will include a built-in ASF table to remove the main ASF offset. One of the inherent features of eLoran is the inclusion of the data channel which will be used to transmit, amongst other messages, differential eLoran data to negate the varying ASF value. This differential eLoran data will be optimised for areas of importance such as ports and harbour approaches.

### Compatibility between eLoran and Loran-C

Legacy receivers will be able to use both eLoran and Loran-C signals at present as they are compatible, with eLoran stations forming part of the presently organised chains. eLoran retains the basic signal format. However, future eLoran stations may be independent of chains. The concept of hyperbolic position fixing from chains will be replaced with an all-in-view concept of operation. Therefore any eLoran station within reception can be used in the position solution.

### eLoran as a viable backup to GNSS

The need for a robust PNT system has been identified as a key component of the emerging e-Navigation concept. eLoran is a dissimilar system to GNSS in terms of frequency, signal power, infrastructure and signal penetration and with performance comparable to GNSS is considered to be the only contender as a viable back-up to GNSS. This has been recognised in the US where eLoran has been adopted as the national PNT system that complements the Global Positioning System (GPS) in the event of an outage or disruption in service.

It needs to be recognised that any future eLoran systems are likely to be regional, although Loran systems presently cover a large percentage of the major shipping ports.  eLoran is likely to be a component of the future World Wide Radio Navigation System.

## 7.7	Radar Aids to Navigation

Radar aids to navigation are devices that provide returns to a ship’s radar that help to locate and/or identify a navigation mark.

The IMO carriage requirements contained in (Chapter V, Regulation 19) of the SOLAS Convention 1974 (as amended), states:

- all ships of 300 gross tonnage and upwards to carry a 9 GHz radar, and;

- all ships of 3,000 gross tonnage and upwards to be fitted with a 3 GHz radar or, where considered appropriate by the Administration, a second 9 GHz radar.

Some administrations may impose other carriage requirements.

IMO Resolution MSC.192(79) Adoption of the Revised Performance Standards for Radar Equipment 06 December 2004 states that 9 GHz radars should be capable of detecting radar beacons and should be capable of detecting SARTs and radar target enhancers.

9 GHz radars are also extensively carried by vessels not covered by SOLAS or local regulation. Because of this high rate of carriage, radar aids to navigation in the 9 GHz band are especially useful.

### 7.7.1	New Technology Radars

Previous to IMO Resolution MSC 192(79) IMO radar performance standards stated that 3 GHz radars should be capable of detecting radar beacons. Resolution MSC 192(79) removed this requirement. A number of factors were considered in this decision. Among them were:

- Pressure from ITU and other organizations to reduce the bandwidth needed for 3 GHz radars and to reduce the amount of spurious out of band emissions.

- Demand by the mariner for higher performance radars, especially with small object detection and reduction of sea clutter. Higher performance 3 GHz radars have been available for many years. But, because of the techniques used by these radars, they would not detect racons, could not be used to satisfy carriage requirements and were rarely installed.

- Reduction of cost for sophisticated radars because of technology improvements in signal processing and solid-state transmitters.

With the removal of the racon detection requirement, ship-owners are free to use higher performing radars, often referred to as New Technology (NT) radars.

### 7.7.2	Radar Reflectors

A radar reflector is a passive device designed to return the incident radar pulses of electromagnetic energy back towards the source and thereby enhance the response on the radar display. By design, a radar reflector attempts to minimise the absorption and random scattering effects.

A radar reflector is generally installed as a supplementary device at sites that would also be marked with a light. The main objectives of its use are to enhance:

- target detection at long ranges (for example, for landfall navigation);

- target detection in areas of sea or rain clutter; and

- radar conspicuity of aids to navigation to reduce the risk of collision damage.

The performance of a radar reflector can be defined in terms of its effective radar cross section (RCS). This is a value determined by comparing the strength of radar signals returned by the radar reflector with the equivalent return from a radar reflective sphere.

The range at which a radar reflector target can be detected is dependent on the heights of the radar antenna and the reflector and the output power of the radar. There are analogies to the geographical range of visual marks. The radar performance of corner cluster reflectors may vary considerably from one make to another, despite being of similar physical size. This arises from differing design philosophies; some that favour the fabrication process and others that try to optimise the polar distribution of radar reflections.

Use of small radar reflectors can also be subject to multipath fading effects. Please see IALA Guideline No. 1010 on racon range performance for a discussion on multipath fading.

Most radar reflectors are designed for use by 9 GHz radars. The reflectors are also usable with 3 GHz radars; however, the effective radar cross section is about an order of magnitude less.

### 7.7.3	Radar Target Enhancers

A Radar Target Enhancer (RTE) is a device that amplifies and returns the pulse from a ship’s radar to give an enhanced image on the radar screen. The returned signal from an RTE is not coded. The RTE was designed primarily for buoys and small vessels that might normally carry a passive radar reflector. A paper on RTE trials presented at the 1998 IALA Conference noted that the RTE tested had an effective radar cross section (RCS) of about 100 square metres, compared with an RCS of 20 to 30 square metres for passive radar reflectors typically fitted to buoys.

To date, commercially available RTEs only operate in the 9 GHz band.

RTE use is subject to multipath fading effects. Please refer to IALA Guideline No. 1010 on racon range performance for a discussion on multipath fading.

### 7.7.4	Radar Beacon (Racon)

Racons are receiver/transmitter devices operating in the maritime radar frequency bands (9 and 3 GHz) that enhance the detection and identification of certain radar targets.

A racon responds to the presence of a ship’s radar by sending a characteristic pulse train. The response appears as a coded mark (or “paint”) on the ship’s radar display that highlights the range and bearing of the racon. The display paint can be fixed to a specified length or can be dependent on the radar range setting and uses a Morse character for identification.

Figure 1:  Example of a racon and a radar display with and without the racon character.

Applications

A racon is generally considered to be a supplementary aid to navigation installed at sites that would also be marked with a light. The number of vessels capable of making use of a racon is effectively unlimited.

A racon can be used for:

- ranging and identification of positions in ice conditions or on inconspicuous coastlines;

- identification of aids to navigation, both seaborne and land based;

- landfall identification;

- indicating centre and turning point in precautionary areas or Traffic Separation Scheme (TSS)

- marking hazards;

- indicating navigable spans under bridges; and

- identifying leading lines.

### 7.7.5	Frequency-Agile Racon

A frequency-agile racon responds on the frequency on which it is interrogated and the response can be re-painted on each radar sweep. The purpose of frequency agility is to provide a signal to the radar that is within the receiver bandwidth of the radar.

However, to avoid masking other features on the radar screen, the racon response is usually switched on and off on a preset cycle.

Signal Characteristics

Racons operate in the 9 GHz band with horizontal polarisation, and/or in the 3 GHz band with horizontal and optionally vertical polarisation.

.

| Preferred Terminology | Alternatives |  |  |
| --- | --- | --- | --- |
| 9 GHz | 9300 -9500 MHz | X - band | 3 cm |
| 3 GHz | 2900 -3100 MHz | S - band | 10 cm |

Table 1 – Preferred terminology for the description of racon operating frequencies

### 7.7.6   Performance Criteria

The availability of a racon is the principal measure of performance determined by IALA. In the absence of any specific considerations, IALA recommends that the availability of a racon should be at least 99.6%

Refer to IALA publications:

- IALA Guideline 1010 on racon range performance;

- IALA Recommendation R-101 on maritime radar beacons (Racons)

- IALA Recommendation O-113 for the marking of fixed bridges over navigable

- waters.

### 7.7.7	Technical Considerations

There are a number of technical considerations:

- The angular accuracy of the bearing between the ship and racon depends entirely on the interrogating radar, while the accuracy of the range measurement depends on both the radar and racon;

- When racons are used in leading line applications, an alignment accuracy of about 0.3 degrees can be expected;

- When the ship is very close to the racon, side-lobes from the radar antenna can trigger the racon. The resulting multiple responses on the radar display may be a distraction and can mask other targets. Side-lobe suppression techniques are standard features of frequency agile racons.

### 7.7.8	  Use with New Technology Radars

All currently available and installed racons are designed for use with high power pulse radars. In comparison, NT radars use low power transmissions with long pulses. Because of the low received signal strength and long pulse at the racon, current racons may not detect NT radars and may not transmit a response usable by NT radars.

Currently installed 3 GHz high power pulse radars are not expected to be replaced by NT radars in the near future and racons with 3 GHz capability will continue to be useful. At some point there may be a requirement to replace all high power pulse radars, at which time present racons with 3 GHz pulse capability may need replacement.

## 7.8	Non-radio positioning (Inertial)

Many studies have been carried out on the integration of GNSS with Inertial Measurement Units (IMU) for marine navigation.

There are different grades of IMU, ranging from the very expensive navigation grade, costing many tens of thousands of dollars; through tactical grade to low cost units based on MEMS (Micro Electro Mechanics System) technology. They give different rates of drift and are therefore capable of covering various lengths of GNSS outage. None of the systems available at present is capable of maintaining all levels of navigation accuracy during a lengthy outage of GNSS. For ocean areas, both navigation and tactical-grade IMUs will give protection for appreciable outages (over 1 hour and 15 minutes respectively). For coastal areas, 15 minutes of good accuracy would be maintained using a navigation-grade IMU. For the harbour approach phase, accuracy to 10 metres could be obtained for 3.5 minutes with a navigation-grade IMU-based system. For docking and where sub-10 metre accuracy is required, coupling inertial systems with GNSS does not appear to offer any benefit in the event of a GNSS failure. In the future low cost MEMS based IMUs could be a useful addition to a ship’s navigation system.

MEMS is an enabling technology with a global market. The major growth opportunities will come from automotive and consumer applications. The idea of an IMU on a chip with unit cost as low as a GPS module is anticipated in the near future.

Nearly all MEMS accelerometers work by measuring the motion of a proof mass against a fixed frame. The main sensing approaches are capacitive or piezoresistive. The power requirement of such sensors is in the range 1-10 mW.

The navigation performance of a MEMS based inertial system depends mostly on the gyro error behaviour, the position error can grow up to more than 80 m during a GNSS outage of 60 sec. Nevertheless MEMS based INS could make a useful contribution by providing a stable heading output during GNSS outages and could help to stabilise position even when GNSS is available, therefore the development of MEMS based IMUs should be kept under observation.

# Communications

## IALA Plan

IALA has prepared a plan for the communications required to support e-Navigation. This Maritime Radio Communications Plan (MRCP) is intended to meet the key strategy element of identifying communications technology and information systems to meet user needs. This may involve the enhancement of existing systems or the development of new systems. The IALA work starts by identifying existing and future systems, then drawing on the user requirements already identified to assess the information flows and the data channels needed.

## Long Range Identification and Tracking

### Introduction

Competent authorities with responsibility for aids to navigation, port security and other shoreside activities are often faced with the requirement to maintain surveillance of maritime approaches to their ports and port facilities for safety, security, and environmental protection.

These authorities are pursuing vessel tracking technologies to assist in the detection, classification, identification, and tracking of vessels. Among these technologies, Long Range Identification and Tracking (LRIT) is being implemented internationally for tracking ships globally.

### LRIT Concept

Long range identification and tracking (LRIT) is a cooperative surveillance capability. In the simplified LRIT concept (Figure 1, courtesy of INMARSAT), a ship carries radio communications equipment that reports identification, position and time to authorities tracking that ship.  However, the final implementation of LRIT is more complicated as explained below.

Figure :  Simplified LRIT Concept

### LRIT Performance Standards and Functional Requirements

The approved performance standards and functional requirements for long-range identification and tracking lay out the LRIT system architecture (Figure 2) and describe how the long-range identification and tracking system works.

Figure 2:  LRIT System Architecture

In this architecture, the Administration determines whether its ships will report to a national, regional/cooperative, or the international LRIT data centre. Each of these types of centres may use multiple communications service providers. The architecture is also designed to accommodate multiple application service providers.  Building on the basic concept noted above, a ship carries radio communications equipment that reports identification, position and time to the national, regional/cooperative, or international LRIT data centre tracking that ship. The Administration of the ship can access the LRIT information directly from the data centre.  Other Contracting Governments that are entitled to that information (i.e., port and coastal states) can request the information through their data centre and thence through the international LRIT Data Exchange.  The LRIT information is routed to the requesting data centre through the data exchange.

# AIS

## Overview

Automatic Identification System (AIS) is a ship and shore-based data broadcast system, operating in the VHF maritime band.  Its characteristics and capability make it a powerful tool for enhancing situational awareness, thereby contributing to the safety of navigation and efficiency of shipping traffic management.

An AIS unit is a VHF radio transceiver capable of exchanging information such as station identity, position, course, speed, length, ship type and cargo information etc., with other ships and suitable receivers ashore.  Figure 1 gives an overview of the system.

Once set up correctly, information from an operational shipboard AIS unit is transmitted continuously and automatically, without any intervention by the ship’s staff.

Shipboard AIS enables the provision of fast, automatic and accurate information regarding risk of collision allowing the Closest Point of Approach (CPA) & Time to Closest Point of Approach (TCPA) to be calculated from the positional information transmitted by target vessels.

Therefore, AIS is an important supplement to existing systems, including radar.  In general, data received via AIS will enhance the information available to the Officer of the Watch and the Vessel Traffic Service Operator (VTSO).

Note:  This section aims to provide a broad overview of AIS.  The reference list in the last section assists the reader seeking amplifying information on various aspects of AIS.

## Purpose & Function

The purpose of AIS is to positively identify vessels, provide additional information in order to assist in collision avoidance and assist in vessel tracking.  It also aims to simplify and promote the exchange of information automatically, thereby reducing the need for doing so verbally (e.g., mandatory ship reporting by radiotelephony).

AIS satisfies the following functional requirements, as laid down by IMO.

- in a ship-to-ship mode for collision avoidance;

- as a means for littoral States to obtain information about a ship and its cargo; and

- as a VTS tool, i.e. ship-to-shore (traffic management).

AIS automatically exchanges shipboard information (provided by shipboard sensors), between vessels and between a vessel and a shore station(s).

## System Characteristics

### Frequencies and capacity

AIS operates on two dedicated VHF FM radio frequencies AIS1 (channel 87B – 161.975 MHz) and AIS2 (channel 88B– 162.025 MHz) in the maritime mobile band.

Transmissions consist of bursts of ‘data packets’ from individual stations, according to an automatically determined time-ordered sequence.  Stations organize themselves on the common frequencies (AIS 1 and AIS 2) based on the knowledge of their own transmissions and that of other stations.  This method of operation is known as Self Organizing Time Division Multiple Access (SOTDMA).  The time slots for AIS transmissions are all precisely aligned to Coordinated Universal Time (UTC), provided for by a Global Navigation Satellite System (GNSS) receiver.  This avoids the possibility of two stations transmitting at the same time, in the same slot.

There are 2250 time slots available on each frequency per minute, making the total number of slots equal to 4500.

Figure 1 – Overview of the AIS System

### Shipboard AIS

A shipboard AIS unit transmits its own data to other vessels and to AIS equipped stations continuously and autonomously.  It also receives AIS data of other stations (ship and shore) and can display this data textually and graphically, as required.

Each AIS station consists of a VHF transmitter, two VHF SOTDMA receivers, a VHF DSC receiver, a GNSS receiver (to provide timing for slot synchronisation), and a marine electronic communications link to shipboard display and sensor systems.

Positional information can be derived form the internal GNSS or an external electronic position fixing system.

The display panel with the unit is often the only means of showing received AIS data. Together with a keypad, this basic configuration is known as a Minimum Keyboard and Display (MKD).

The display part of a MKD, as a minimum, consists of three lines of data, each showing bearing, range and identity of the target.  In practice, most MKDs display more lines of data and may also have a simple graphical display, showing the relative location of targets, rather like the Plan Position Indicator of a radar.

Ideally though, AIS information ought to be display graphically on a radar, ECDIS or on its own dedicated display.

### Information available

The AIS information transmitted by a ship station includes four different data sets:

- fixed or static information is entered into the AIS unit on installation and need only be changed if the ship changes its name, call sign etc.  This information is broadcast every six minutes or on request by a shore authority.

- voyage related information (destination, ETA etc) is manually entered and updated during the voyage.  This information is also broadcast every six minutes.   In order that correct AIS information is broadcast to other vessels and shore authorities, mariners are reminded to enter current voyage related data such as draught, type of hazardous cargo, destination and ETA properly at the beginning of each voyage and whenever changes occur.

- dynamic information is automatically updated from the ship sensors connected to the AIS.  This includes COG, SOG, position (with accuracy and integrity flag), time and navigation status (e.g., underway).

- Broadcast or addressed short safety related messages, as required

Refer to IALA publications:

IALA Guideline 1028 on the Universal Automatic Identification (AIS) – Volume 1, Part 1 – Operational Issues;

IALA Guideline 1029 on the Universal Automatic Identification System (AIS) – Volume 1, Part 2 – Technical Issues.

### Shore based AIS

SOLAS Chapter V, Regulation 19, 2.4 refers to the carriage requirements for AIS.  The regulation states that AIS shall provide and receive information from appropriately equipped shore stations. The provision of shore based AIS will be necessary to attain the full benefit of the 1974 SOLAS Convention (as amended).

National Members and other appropriate authorities should therefore consider the provision of an AIS shore infrastructure so that the full benefit of the system can be realised in terms of navigation safety and protection of the environment.

As AIS can be seen as a tool related to Vessel Traffic Services (VTS), Competent Authorities should consider implementing AIS into existing VTS Centres.

Refer to IALA publications:

IALA Recommendation A-123 on the provision of shore based automatic identification systems.

IALA Recommendation A-124 on the Automatic Identification System (AIS) Shore Station and Networking Aspects relating to the AIS Service

## AIS as an aid to navigation

A special type of an AIS station fitted to an aid to navigation provides positive identification of the aid without the need for a special ship-borne display.  In addition, AIS as an AtoN can provide information and data that will:

- complement or replace an existing aid to navigation, providing identity, state of ‘health’ and other information such as real time tidal height and local weather to surrounding ships or back to a shore authority;

- provide the position of floating aids (mainly buoys) by transmitting an accurate position (corrected by DGNSS) to monitor if they are on station;

- provide real-time information for performance monitoring, with the connecting data link serving to remotely control changes in AtoN parameters or switching on back-up equipment;

- provide local hydrological (hydrographical) and meteorological information;

- possibly replace radar beacons (racons) in the future, providing longer range detection and identification in all weather conditions; and

- gather shipping traffic data on AIS fitted ships for future aid to navigation planning purposes.

Refer to IALA publications:

IALA Recommendation A-126 on the use of the automatic identification system (AIS) in marine aids to navigation.

IALA Guideline 1050 on the Management and Monitoring of AIS Information.

## Carriage Requirements

There are two ‘types’ of AIS units for ship stations.  These are termed Class A and Class B units.

Class A ship-borne mobile units must comply with ITU-R M. 1371-1, and are required to be carried on board those vessels to which Regulation 19 of SOLAS Chapter V applies and meet the IMO performance standard.  All these ships were to have AIS fitted by 31 December 2004.

Class B ship-borne mobile equipment, while also complying with ITU-R M.1371-1, is designed for vessels such as pleasure craft and fishing vessels.  These units are provides with less functionality than Class A units, and do not necessarily meet all the IMO performance requirements.  They are designed to operate co-operatively with Class A units.

Administrations can require the carriage of Class B units as part of their domestic requirements.

## Cautions when using AIS

The Officer of the Watch (OOW) should always be aware that other ships, in particular pleasure craft, fishing vessels, warships and some shore stations including VTS centres, may not be fitted with AIS.

The OOW should always be aware that AIS fitted on other ships as a mandatory carriage requirement, might, under certain circumstances, be switched off, particularly where international agreements, rules or standards provide for the protection of navigational information. AIS can also provide incorrect information if the input data is wrong.

Navigators should be aware of the limitations of AIS.  In particular, government agencies and owners should ensure that watch-keeping officers are trained in the use of AIS. Because of these limitations navigators are advised that AIS should not be used as the primary means of collision avoidance.

## Strategic Applications of AIS

From a number of maritime perspectives (such as VTS and regulatory compliance), the availability of comprehensive ship information, offers a mechanism for:

- better monitoring of compliance with national and international regulations for mandatory routeing and reporting systems, Particularly Sensitive Sea Areas, discharging of oil, garbage disposal etc;

- maritime logistics applications such as fleet management, cargo tracking and port facilities management (movement of pilot boats, tugs etc);

- better control, co-ordination and response in the event of marine incidents, such as SAR and pollution;

- shore -based navigational assistance;

- shipping information gathered from AIS can be channelled into a central repository of a local, national or regional network serving maritime administrations, port authorities, shipping agents, freight handlers, customs, immigration, etc.

## International Standards, Recommendations and Guidelines

Further information on AIS can be found within IMO, IALA, ITU and IEC documentation

### IMO Documentation

- Recommendation on Performance Standards for an Universal Shipborne Automatic Identification System (AIS) (MSC 74(69) Annex 3).

- Guidelines for the onboard operational use of shipborne Automatic Identification Systems (AIS) (Resolution A.917 (22), as amended by Resolution A.956 (23)).

- Performance Standards for the presentation of navigation-related information on shipborne navigational displays (Resolution MSC. 191(79).

- SN/Circ. 227 Guidelines for the installation of a shipborne Automatic Identification System (amended by SN/Circ 245).

- SN/Circ. 236 Guidance on the application of AIS Binary Messages

- SN/Circ. 243 Guidelines for the presentation of navigation-related symbols, terms and abbreviations

- SN/Circ. 244 Guidance on the use of UN/LOCODE in the destination field of AIS messages.

### IALA Documentation

- Recommendation A-123 on the Provision Of Shore Based Automatic Identification Systems (AIS)

- Recommendation A-124 On AIS Shore Stations And Networking Aspects Related To The AIS Service .

- Recommendation A-126 on AIS for Aids to Navigation

- Guideline 1026 on AIS as a VTS Tool

- IALA Guidelines on the Automatic Identification System (AIS)

- Guideline 1028 - Volume 1, Part I – Operational Issues,

- Guideline 1029 - Volume 1 Part II – Technical Issues, Edition 1.1

- Guideline 1050 on the Management and Monitoring of AIS Information

- Technical Clarifications on ITU Recommendation ITU-R M.1371-1 Edition 1.5

### ITU Documentation

- ITU-R M.1371-1Recommendation on the Technical Characteristics for a Ship-borne Universal Automatic Identification System (AIS) Using Time Division Multiple Access in the Maritime Mobile Band.

- Radio Regulations, Appendix S18, Table of Transmitting Frequencies in the VHF Maritime Mobile Band.

- ITU-R M.823-2 Recommendation on the technical characteristics of differential transmissions for global navigation satellite systems from maritime radio beacons in the frequency band 283.5-315 kHz in rgion 1 and 285-325 kHz in regions 2 and 3.

### IEC Standards

- 61993 Part 2:  Class A Ship-borne equipment of the Automatic Identification System (AIS)  - Operational and Performance requirements, methods of testing and required test results.

- 61108-1 (2nd edition): navigation and radiocommunication equipment and systems – Global navigation satellite systems (GNSS).

- 61162-1 (2nd edition) Maritime navigation and radiocommunication equipment and systems – Digital interfaces – Part 1: Single talker and multiple listeners.

- 62320-1: Maritime Navigation and Radiocommunication equipment and systems – Automatic Identification System. AIS base stations - Minimum operational and performance requirements - methods of test and required test results

- 62320-2 Maritime Navigation and Radiocommunication equipment and systems – Automatic Identification System. AIS aids to navigation - Minimum operational and performance requirements - methods of test and required test result

- 62287-2 (Part A and B) Class B AIS (Part A – CSTDMA; Part B – SOTDMA)

- 61097-14 (pending) Global Maritime Distress and Safety System (GMDSS).  AIS search and rescue transmitter (AIS-SART) – Operational and performance requirements: methods of testing and required test results.

# ECDIS

## Electronic Chart Display and Information System (ECDIS)

Although ECDIS, as ship borne equipment, is not an “aid to navigation” as defined by IALA, it deserves to be mentioned because it brings major changes to the manner in which vessels are navigated.  ECDIS uses digital vector data in a way that replaces the traditional paper charts with a more versatile electronic product that can draw on a variety of positioning and data inputs, such as GNSS, DGNSS, AIS, radar, echo sounder, compass, an electronic chart, navigational publications, the chart amendments and tidal and meteorological information.

## Performance Standards

The performance standards for ECDIS have been defined by the International Maritime Organization (IMO), in conjunction with the International Hydrographic Organization (IHO).

IMO Resolution A.817(19) as amended by Resolution MSC.64(67) and by Resolution MSC.86(70) enables marine administrations to accept ECDIS as a legal equivalent to paper charts that are required to be carried for compliance with SOLAS Chapter V.

## Performance Elements

There are two key performance elements to ECDIS:

- An approved processing system (or ‘box’), and;

- Electronic Navigational Charts (ENCs) that have been prescribed by a national hydrographic office and meet the standards set down in the 3rd Edition of the IHO Special publication No.57 (S 57)

While an ECDIS ‘box’ may be capable of reading other forms of electronic charts, it ceases to be a compliant system without the official ENCs.

Non-S57 electronic charts include:

- Raster Navigation Charts (RNC) that are effectively electronic copies of paper charts, and;

- Electronic charts that are not issued by a national hydrographic authority or differ from the S57 standard.

When the relevant chart information is not available in the appropriate form (issued by a national hydrographic office and meeting the S57 standards) some ECDIS equipment may operate in the Raster Chart Display System mode, but in that case the ECDIS equipment should be used together with an appropriate portfolio of up-to-date paper charts.  Additional information on ECDIS is available on the IMO and IHO websites.

# Maritime Information Systems

## 11.1	Maritime Information

The timely provision and display of maritime information will be an essential component of e-Navigation. Generically called Marine Information Overlays (MIOs), this includes both static and dynamic information capable of being used ashore (e.g., at a VTS Centre) and onboard ships at sea.  Static information could pertain to marine protected areas, sea ice coverage, emergency management/response areas, and seafloor bathymetry.  Dynamic operational information would be broadcast via AIS binary messages as time-critical information regarding ship/voyage data, marine traffic signals, area notices, dangerous cargo, environmental, meteorological, hydrographic, and status of AtoN.  In particular, mariners require this type information pertaining to the planning and execution of voyages, the assessment of navigation risk and compliance with regulation. The provision and use of MIOs will depend on the current situation and task-at-hand.

At the 54th session of the IMO Safety of Navigation Subcommittee (July 2008) it was recommended that there be Common Maritime Information/Data Structure that would be accessible from a single integrated system.  Shore users require information pertaining to their maritime domain, including static and dynamic information on vessels and their voyages.  Ideally, this information should be provided in “an internationally agreed common data structure.  Such a data structure is essential for the sharing of information amongst shore authorities on a regional and international basis.”  At present, there is no specific guidance or standards related to the presentation/display of MIOs on shore-based equipment or systems.  However, there are a number of general and equipment-specific international standards that have been adopted by IMO, IHO, and IEC that contain “guidance” related to the presentation/display of various types of shipborne navigation-related information.  This is something that will need to be part of e-Navigation development and implementation.

## 11.2	AtoN Attribute Information

The exchange of information about AtoN between any parties in a digital environment will require an internationally agreed standard so that information can be automatically compiled for sending and automatically understood by systems that receive it.

Such a standard will enable harmonisation of the management of information about AtoN, and in particular information that is relevant to mariners: “situation normal” data (position, colour, shapes, light etc) and also “situation abnormal” (lost top mark, light on reduced range, unlit etc).  In GIS terms this sort of information can be described as attribute data (information particular to a GIS object, such as an AtoN) and metadata (data about the attribute data).

## 11.3	Meteorological & Hydrological Information

IMO is responsible for the AIS Binary Broadcast Messages (AIS Message 8) and a message structure has been defined for meteorological and hydrological information.

A number of countries operate tide gauges and current meters to assist the prediction of tidal heights and streams or for the broadcast of real-time information to shipping.  The latter is generally used to overcome the sometimes considerable differences between actual tide heights and predicted values due to meteorological and mean sea level fluctuations. Providing real-time information of this type, for example dynamic under-keel clearance, wave heights or sea state can be seen as applications of e-Navigation, requiring integration between shore-side and ship-borne systems.

## 11.4	Intergovernmental Oceanographic Commission

The Intergovernmental Oceanographic Commission (IOC) is responsible for co-ordinating the Global Sea Level Observing System (GLOSS) program to establish global and regional networks of sea level stations for providing essential information for international oceanographic research programmes, including those dedicated to the study of aspects of climate change, and operational oceanography.  The main component of GLOSS is a network of 287 sea level stations worldwide (referred to as the 'Global Core Network').

IALA supports and encourages participation in the GLOSS program.

Authorities that are procuring or upgrading Sea Level Measurement devices are encouraged to consider using equipment that can support the requirements of the GLOSS program.  Typically this calls for gauges capable of measuring to centimetre (1 cm.) accuracy in all weather (especially wave) conditions and for the free exchange of hourly sea level data with an International Sea Level Centre.

Information on the GLOSS Programme can be found at: http://www.pol.ac.uk/psmsl/programmes/gloss.info.html.

*********************************

The Standard Positioning Service is the one available to civilian and commercial users.  A Precise Positioning Service (PPS) is also provided for the US Military Services.

2 GPS SPS Performance Standard, 2008

A 1kW transmitter will generally allow position fixing to better than 10 metres over a radius of about 200 nautical miles.

“Active Radar Target Enhancers For Buoy Signature Enhancement”; Commissioners of Irish Lights.

IMO Resolution MSC.74 (69) Annex 3 refers.

Section 12 of the IALA Guideline 1028 on AIS Volume 1 Part 1 – Operational Issues