AISM Association Internationale de Signalisation Maritime      IALA International Association of Marine Aids to Navigation and Lighthouse Authorities

ANM2/WG2/WP1

Availability Objectives

# Recommendation on

Availability Objectives

of Aids to Navigation Services

## IALA Recommendation ?-???

DRAFT

DRAFT

DRAFT

20ter, rue Schnapper – 78100 Saint Germain en Laye – France

Telephone : +33 1 34 51 70 01  Telefax : +33 1 34 51 82 05

E-mail : aismiala@easynet.fr              Internet : http://www.iala-aism.org

Recommendation on

Availability Objectives

of Aids to Navigation Services

THE COUNCIL,

RECOGNISING that it is important for member authorities to state management objectives for operational performance levels of Aids to Navigation services provided to mariners;

RECOGNISING FURTHER the need to provide guidance on suitable and realistic levels of operational performance for members to adopt;

RECALLING that IALA recommends (E-105) that “purchasing authorities include reliability and quality requirements in their specifications” when procuring Aids to Navigation equipment;

NOTING that it is possible to ensure the required level of availability during the design phase of Aids to Navigation services since the theoretical relationship between individual component reliability and system availability is known;

NOTING ALSO that many Aids to Navigation Authorities already have adopted the suggested availability objectives in their management of Aids to Navigation systems;

HAVING CONSIDERED the proposal of the EEP, RNAV and ANM Committees;

RECOMMENDS that all members providing Aids to Navigation should:

- Categorise each individual or group of aids to navigation as per the categories defined in the Annex to this Recommendation, utilising an appropriate risk assessment methodology (IALA Guidelines on Risk Management, December 2000);

- Adopt the availability objectives set out in the annex to this recommendation in accordance to the level of service factors (IALA Guidelines on Level of Service, October 1997);

- Define the failure state(s) for each type of Aids to Navigation service provided;

- Establish formal procedures for collecting, processing and recording availability data.

# ANNEX

RECOMMENDED AVAILABILITY OBJECTIVES OF

AIDS TO NAVIGATION SERVICES

[Introduction – to be drafted, include ‘aim’ of document…GP to draft an intro on the risk assessment process]

[Background – to be drafted, include focus for the document – service providers/users…]

Definitions / Acronyms

Availability:	The probability that an Aid to Navigation or system is performing its specified function at any randomly chosen time. [this is the definition from the IALA Navguide] [Possible change suggested: The amount of time, expressed as a percentage of total time, that an AtoN or system is performing a specified function] [ note – this will need to be reflected in the Navguide as well]

Considerations

- The Availability Objectives will depend on a Risk Assessment methodology [reference IALA Risk Management Guidelines and Generic Risk Model ] taking into consideration factors such as:

| - Significance of the waterway - Environmentally sensitive areas - Nature and type of cargo - Traffic density | - Aids to Navigation coverage - Climatology - Political considerations |
| --- | --- |

- The Availability Objectives may also depend on aspects such as:

| - Existing technology - Logistics - Redundancy | - Accessibility - Other navigational services (i.e. Pilotage, VTS, etc) |
| --- | --- |

Categories

| CATEGORY | AVAILABILITY OBJECTIVE | CALCULATION |
| --- | --- | --- |
| 1 | 99.8% | Availability Objectives are calculated over a three-year continuous period, unless otherwise specified |
| 2 | 99.0% |  |
| 3 | 97.0% |  |

See Note 2

Category 1: 	An Aid to Navigation (AtoN) or mix of AtoN that is considered by the Competent Authority to be of primary navigational significance. For example, it includes DGNSS service or any lighted aids to navigation and racons that are considered essential for marking landfalls, primary routes, channels, waterways or new dangers.

Category 2: 	An AtoN or mix of AtoN that is considered by the Competent Authority to be of navigational significance. For example, it may include any lighted aids to navigation and racons that mark secondary routes and those used to supplement the marking of primary routes.

Category 3: 	An AtoN or mix of AtoN that is considered by the Competent Authority to be of less navigational significance than Category 1 and 2.

Notes

- The absolute minimum level of availability for individual Aids to Navigation is 95%, calculated over a three-year period;

- IALA Recommendations related to specific types of AtoN should be consulted;

- The availability of an Aid to Navigation may be calculated using the following equation, using the most accurate time available:

Availability= Total Time−Down TimeTotal Time

The IALA Navguide should be consulted for further information on the method of calculating the availability.