Draft

Emergency Wreck Marking Response Plan

# Introduction

Following an incident as a result of which a new (dangerous) wreck or obstruction is formed, certain measures will have to be taken by the responsible authorities in order to avoid further incidents and to prevent loss of life and/or pollution.

The aim of the EMERGENCY WRECK MARKING RESPONSE PLAN (EWMRP) is to ensure that an immediate, effective and well co-ordinated response can be given in such a situation.

The EWMRP points out procedures to be observed, as well as considerations to be taken into account with respect to all necessary measures that should be taken by the authorities when confronted with a new (dangerous) wreck or an obstruction as a result of an incident within their area of responsibility.

As such, the EWMRP gives guidance and provides recommendations with regard to emergency wreck marking to the authorities involved.

Considerations for Authorities.

Administrations need to assess their areas of responsibility and response capability. The wreck of the Tricolour in the Dover Straits has brought into sharp focus the effective responses required to adequately mark such dangerous wrecks and to prevent further collisions with the wreck. Administrations need to carry out risk assessments, assess their response capabilities and consider plans to deal with such situations.  Aspects of such a risk assessment should include:

- analysis of responses capabilities

- Areas of Responsibility

- Assessment of response required in specific areas

- Response times

- Intervention times

- Assess mobile resources eg  pollution combating vessels, buoy tenders, Emergency Towing Vessels, guardships, buoys, Temporary VTS capability,

*** A helpful tool for decision making with respect to the marking of wrecks and obstructions should be  developed beforehand a ‘marking requirements map’ for the responsibly area indicating the different marking requirements in specific sea areas.  For each of these areas the marking requirements should be based on information and knowledge of the sort of shipping that use the areas (see example)

Definitions and Acronyms

- Emergency Wreck marking Response Plan (EWMRP)

- Vessel Traffic System (VTS)

- Emergency Towing Vessel (ETV)

# Emergency Wreck Marking Response Plan – Decisions and Actions

## New (dangerous) wreck or obstruction

- Priority of Decisions to make and / or Actions to take (Also see flow diagram)

- Immediate broadcast / transmission of initial Safety message (navigational warning) concerning the new (dangerous) wreck / obstruction, Vital Navigational Warning?

- Obtain as much information as possible about new wreck / obstruction

- Guard ship necessary on a new wreck/obstruction locations?

- Temporarily VTS necessary on wreck/obstruction location

- Initial marking wreck/obstruction position

- Survey wreck/obstruction

- Assess pollution & environmental aspects and responses required.

- Permanent marking wreck / obstruction position

- Broadcast / transmission / issue Safety message(s) (navigational warning(s)), NAVTEC warning(s), Notice(s) to Mariners, NAVAREA information, etc. based on survey information details

- Continuation of VTS necessary?

Possible followed by:

- Salvage necessary?

- Salvage plan

- Marking and/or VTS during salvage operation

- Possibility of future wreck/obstruction marking developments:

- The use of  “impact” lights, possibly combining light colours.

- The use of other conspicuous marks, signal, or danger indicating tools

- AIS and AtoN such as ANIS.

- Virtual AIS

# Definition – Decision to make and / or Actions to take

## Priority

### Immediate broadcast of an initial Safety message concerning the new (dangerous) wreck/obstruction

Especially in busy waterways a new (dangerous) wreck or obstruction has the capability to cause tremendous environmental damage and loss of life.  It is therefore very important that shipping is informed as soon as possible about the new danger.  Although in most incidents detailed information is not directly available it is very important that initial information of a new wreck or an obstruction, regardless of how scant it may be, is made known to shipping without delay.

Initial Safety message(s) (navigational warning(s)), concerning the (approximate) position of the new wreck/obstruction and possible other relevant information, should be broadcast/transmitted/sent without delay on:

- VHF* (announced on DSC)

- MF*(announced on DSC)

- HF*(announced on DSC)

- NAVTEX*

- INMARSAT (EGC)

*In certain circumstances it might be necessary to issue a vital navigational warning.

### Obtain as much information as possible about wreck/obstruction

Information about a new wreck/obstruction should be gathered as soon as possible.  In certain situations this process might well start even before a wreck/obstruction is actually a fact.  After an incident, for example, a collision, as a result of which a ship is slowly sinking information about, amongst other, actual position of the foundering ship should be monitored if possible by other ships on scene (either assisting merchant vessels, fishing vessels, or governmental work vessels).  The sooner the actual location of a wreck is known the better. This will save valuable time to be able to initially mark the wreck and/or issue navigational warnings.  This will reduce the risk of undesired situations such as other ships hitting the w4reck whilst still unmarked.

In circumstances where a ship sinks with no other ship(s) in the vicinity able to report, no immediate information on the actual wreck location is available, the wreck location should be established as soon as possible so that navigational warnings can be issued and initial marking of the wreck location can be carried out.

### Guard ship necessary on a new wreck/obstruction location?

A new wreck/obstruction can cause many and immediate problems and could be very dangerous for shipping, not only when new, position not exactly known and/or still unmarked, but even when the position is known and the wreck is properly marked.  In the past many wrecks have caused numerous problems resulting in damage, pollution and even loss of lives.

When confronted with a new, possibly dangerous wreck/obstruction, authorities should decide as soon as possible if it is necessary to send a ‘guard ship’ to the (approximate) location of the wreck in order to ‘guard’ the location and inform ships navigating near the wreck position of the new danger.  The guard ship should be well equipped for her task and should be able to stay on location in all weather conditions and sea states.

### Temporary VTS necessary on wreck/obstruction location?

In busy shipping areas traffic lanes of separation schemes, precautionary areas, channels, harbour approaches, etc. action taken as described in section 3 might not be sufficient.  Further action in the way of establishing temporary VTS on or near the wreck/obstruction location should in such a case be contemplated.

In case temporary VTS on/near the wreck location is established it should be performed by certified VTS operators and around the clock until additional actions have been taken which can guarantee a safe situation.

### Initial Marking wreck/obstruction position

A new wreck or obstruction which can cause danger for shipping has to be marked.  Weather conditions, sea state and / or unknown facts about the new wreck can all hamper timely marking.  Regardless these circumstances it is of great importance that the wreck location is marked as soon as possible and can be recognised by ships as a new wreck location.

Taking into account shipping route structure on/near the wreck location effective initial marking should be carried out in the way of temporarily establishing 2 or more cardinal light buoys on location.  In addition, a RACON beacon – mores code ‘D’ (Danger) can be fitted.  This initial marking situation should be maintained until the wreck is well known and has been mentioned in the current nautical publications and/or until the wreck/obstruction has been surveyed and exact details such as position, swept clearance above the wreck/obstruction, etc. are known and permanent markings of the wreck can be carried out.

### Survey wreck/obstruction

A proper survey of the new wreck should be performed as soon as possible.  Survey information and details should at least include;

- Exact position of the wreck/obstruction

- Stability of the wreck

- Wreck heading

- Sub sea clearance above the wreck

### Permanent Marking wreck/obstruction position

As soon as the wreck survey information/details (see 6.) are available, but also based on the shipping route structure and density on / near the wreck location, the marking of the wreck should be reconsidered.

- Is the initial marking sufficient/no changes needed?

- Has the initial marking to be moved (re-positioned)?

- Is adjustment of initial marking necessary?

- Is additional marking needed?

- Configuration?

- Extra conspicuous AtoN needed?

- Other solution?

### Broadcast / transmission / issue of safety message(s) (navigational warning(s) – NAVTEX wa4rning(s), Notice(s) to Mariners, NAVAREA information, etc. based on survey details and information of the wreck

As soon as further (specific) survey details and /or information concerning the wreck/ obstruction becomes available, shipping and relevant authorities should be informed immediately.

### Continuation of VTS necessary?

Incidentally, regardless all measure taken earlier (navigational warnings, marking, etc.) it might be necessary to decide to continue VTS on the wreck/obstruction location.  In case of an extremely dangerous wreck, for instance in the middle of a busy shipping route or shipping lane, hampering passing ships to navigate and manoeuvre, VTS on location might be essential to avoid collision(s), ether with the wreck or between ships themselves whilst manoeuvring past the dangerous wreck.