Chapter 1	AISM – IALA	4

1.1	4

1.2	4

1.3	4

Chapter 2	CONCEPTS AND ACCURACY OF NAVIGATION	5

2.1 Navigation	5

2.1.1 Navigational Methods	5

2.1.2 Accuracy Standards for Navigation	5

2.1.3 Phases of Navigation	6

2.1.4 Measurement Errors and Accuracy	10

2.1.5 Hydrographic Considerations	12

Chapter 3	AIDS TO NAVIGATION	18

3.1 Definition of Aids to Navigation	18

3.2 Scope	18

3.3 Visual Aids to Navigation	18

3.3.1 Visual Marks	18

3.3.2 Aids to Navigation Lights	26

3.3.3 Fixed Aids to Navigation	62

3.3.4 Floating Aids to Navigation	64

3.3.5 Sector Lights and Leading (Range) Lines	70

3.3.6 Transits	77

3.4 Radio Aids to Navigation	92

3.4.1 Policy on Radio Aids to Navigation	92

3.4.2 Marine Radio Beacons	95

3.4.3 Radar Reflectors	95

3.4.4 Radar Target Enhancers	98

3.4.5 Radar Transponders	98

3.4.6 Radar Beacon (Racon)	99

3.4.7 LORAN / CHAYKA	102

3.5 Satellite Radionavigation Systems	104

3.5.1 IALA Policy	104

3.5.2 GNSS	104

3.5.3 Global Navigation Satellite System (GLONASS)	105

3.5.4 Differential Global Positioning navigation System (DGNSS)	106

3.5.5 World-Wide Radionavigation System (WWRNS)	110

3.6 Automatic Identification System (AIS)	113

3.6.1 Description	113

3.6.2 Components	115

3.6.3 AIS Information	117

3.6.4 AIS Services and Applications	125

3.6.5 AIS Applications	126

3.6.6 AIS as an Aid to Navigation	132

3.6.7 AIS Station Aids to Navigation Report Message	133

3.7 Vessel Traffic Services (VTS)	140

3.7.1 Definition	140

3.7.2 Services	140

3.7.3 VTS System	141

Chapter 4	OTHER FACILITIES	142

4.3 Pilotage	142

4.3.1 Pilotage as an Aid to Navigation	142

4.3.2 Types of Pilotage	143

4.3.3 Other Pilotage Considerations	143

4.3.4 Routeing	143

4.4 Sound Signals	147

4.4.1 Considerations on Sound Signals and their use	147

4.4.2 Considerations on Fog Detectors:	148

4.4.3 Sound Signals in the World:	148

4.4.4 Range of a sound signal	149

4.5 Nautical Publications	150

4.5.1 Navigational Warnings	150

4.5.2 World-Wide Navigational Warning Service	151

4.5.3 Lists of Aids to Navigation	151

4.5.4 Maritime Safety Information	155

4.6 Electronic Chart Display and Information System (ECDIS)	156

4.6.1 Description	156

4.6.2 Performance Standards	156

4.7 Tide Gauges and Current Meters	157

4.7.1 Purpose	157

4.7.2 Intergovernmental Oceanographic Commission	157

Chapter 5	Power Supplies	159

5.3 Types	159

5.3.1 IALA Survey Data on Power Supplies	159

5.4 Non-Electric	159

5.4.1 Acetylene	159

5.4.2 Propane	160

5.5 Electric - Non-Renewable Sources	160

5.5.1 Primary Cells	160

5.5.2 Internal Combustion Engine/Generators	162

5.5.3 Other	162

5.6 Electric - Renewable Energy Sources	163

5.6.1 Solar Power (Photovoltaic cell)	163

5.6.2 Wind Energy	166

5.6.3 Wave Energy	167

5.7 Rechargable Batteries	167

5.7.1 IALABATT 1 to 4	167

5.7.2 Principal types	167

5.7.3 Technical Considerations	169

5.7.4 Battery Disposal	170

5.8 Electrical Loads and Lightning Protection	170

5.8.1 Electrical Loads	170

5.8.2 Lightning Protection	171

Chapter 6	MANAGEMENT ISSUES	172

6.1 Provision, Design and Management of Aids to Navigation	172

6.1.1 International Criteria	172

6.1.2 LEVEL OF SERVICE (LOS)	173

6.1.3 Risk Management	180

6.1.4 Availability Objectives	183

6.1.5 Reviews and Planning	191

6.1.6 Performance Measurement	193

6.1.7 Quality Management	194

6.1.8 Maintenance	197

6.1.9 Service Delivery	199

6.2 Environment	201

6.2.1 Hazardous Materials	201

6.2.2 IALA “PHL” Advisory Panel	205

6.2.3 Lens Size and Terminology	206

6.2.4 Third Party Access To Aids To Navigation Sites	206

6.3 Human Resources Issues	208

6.3.1 Employer Responsibilities	208

# AISM – IALA

# CONCEPTS AND ACCURACY OF NAVIGATION

## Navigation

National aids to navigation authorities are generally established to provide a navigational safety regime that facilitates trade and economic development.  The primary services are therefore directed towards the needs of commercial trading vessels.  In some areas, authorities may provide additional services for ferries, fishing and recreational vessels and specialised maritime activities.

This section looks at the methods of navigation and accuracy requirements from the perspective of commercial trading vessels.

### Navigational Methods

IMO defines navigation as; “The process of planning, recording and controlling the movement of a craft from one place to another”.

The principal methods of marine navigation are briefly described as follows:

- Dead Reckoning: navigation based on speed, elapsed time and direction from a known position.  The term was originally based on the course steered and the speed through the water, however, the expression may also refer positions determined by use of the course and speed expected to be made good over the ground, thus making an estimated allowance for disturbing elements such as current and wind. A position that is determined by this method is generally called an estimated position.

- Terrestrial Navigation: navigation by means of information obtained by visual or radar observation of earth conspicuous objects / marks.

- Celestial or Astronomical Navigation: navigation using information obtained from celestial bodies (ie. sun, moon, planets and stars).

- Radionavigation: navigation using radio signals to determine a position or a line of position (eg. LORAN C, GPS, etc).

    - Accuracy Standards for Navigation

IMO Resolution A.915(22), adopted in January 2002, established accuracy standards for maritime navigation.

The following Table presents the relevant standards adopted in Appendixes 2 and 3 of that Resolution.

Table 2.1 Minimum Maritime User Requirements

| Application | Absolute horizontal accuracy (95%) (m) |
| --- | --- |
| General Navigation: Ocean Coastal Restricted waters Port Inland waterways | 10-100 10 10 1 10 |
| Hydrography | 1-2 |
| Oceanography | 10 |
| Aids to navigation management | 1 |
| Port operations: Local VTS Container/cargo management Law enforcement Cargo handling | 1 1 1 0.1 |

    - Phases of Navigation

Typically, navigation has been divided into three phases: ocean navigation, coastal navigation and restricted waters navigation. More recently, some documents have introduced other phases, namely harbour approaches, port and inland waterways navigation.

The harbour approach phase is a kind of a particularisation of the restricted waters phase, but will be treated separately in this guide.

Port and inland waterways navigation are two other particularisations of restricted waters navigation and will not be dealt in separately in this guide, because the cares and measures required for restricted waters navigation also apply on those waters.

#### Ocean Navigation

In this phase, the ship is typically:

- beyond the continental shelf (200 metres in depth) and more than 50 nm from land;

- in waters where position fixing by visual reference to land or to fixed or floating aids to navigation is not practical;

- sufficiently far from land masses and traffic areas that the hazards of shallow water and of collision are comparatively small.

- The requirements for accuracy in the Ocean Phase are not very strict and are based on providing the ship with a capability to correctly plan the approach to land or restricted waters.

The economic efficiency aspects of shipping (eg. transit time and fuel consumption) are enhanced by the availability of a continuous and accurate position fixing system that enables a vessel to follow the shortest safe route with precision.

Although the IMO adopted more strict accuracy requirements (see Table 2.1), the minimum navigation requirements for the Ocean Phase are considered to be a predictable accuracy of 2 to 4 nm, combined with a desired fix interval of 15 minutes or less (maximum 2 hours fix interval).

Note:

While the description of the Ocean Phase is valid for larger, commercial vessels, the 50 nm distance from land may not be realistic for small craft, notably leisure craft and some fishing vessels, and in some geographical areas:

- For small craft, the ocean phase of navigation could reasonably be considered to have commenced when the distance precludes position fixing by visual references to land or fixed or floating aids.

- Similarly, there are numerous areas of the world where deep water exists out of sight of land, but within 50 nm of land and where there are no natural hazards or aids to navigation.

#### Coastal Navigation

In this phase, the ship is typically:

- within 50 nm from shore or the limit of the continental shelf (200 meters in depth);

- in waters contiguous to major land masses or island groups where transoceanic routes tend to converge towards destination areas and where inter-port traffic exists in patterns that are essentially parallel to coastlines.

The ship may encounter:

- ship reporting systems (SRS) and coastal vessel traffic systems (VTS);

- offshore exploitation and scientific activity on the continental shelf;

- some fishing and recreational boating activity, although this tends to be focused in the coastal zone within 20 nm from the shoreline.

The Coastal Phase is considered to exist when the distance from shore makes it feasible to navigate by means of visual observations, radar and if appropriate, by depth (echo) sounder.  As with the Ocean Phase the distances from land can be varied to take account of the smaller vessels and local geographical characteristics.

Although the IMO adopted more strict accuracy requirements (see Table 2.1), international studies have established that the minimum navigation requirements for commercial trading vessels operating in the Coastal Phase is a navigation system capable of providing fix positioning to an accuracy of 0.25 nm, combined with a desired fix interval of 2 minutes (maximum 15 minutes).

More specialised maritime operations within the Coastal Phase may require navigational systems capable of a higher repeatable accuracy, either permanently or on an occasional basis.  These operations can include marine scientific research, hydrographic surveying, commercial fishing, petroleum or mineral exploration and Search and Rescue (SAR).

#### Harbour Approach

This phase represents the transition from coastal to harbour navigation.

- The ship moves from the relatively unrestricted waters of the coastal phase into more restricted and more heavily trafficked waters near and/or within the entrance to a bay, river, or harbour;

- The navigator is confronted with a requirement for more frequent position fixing and manoeuvring the vessel to avoid collision with other traffic and grounding dangers;

- The ship will generally be within:

  - the coverage areas of aids to navigation of varying complexity (including lights, racons, leading lights and sector lights);

  - pilotage areas, and;

  - the boundaries of SRS and VTS.

Safety of navigation issues that arise during the Harbour Approach Phase impose more stringent requirements on positional accuracy, fix rates and other real-time navigational information than during the Coastal Phase.

The advent of GPS and of DGPS has provided a means of achieving the Harbour Approach requirements for high positional accuracy and fix rates at better than 10-second intervals.  However, it is not practical to plot these fixes on a chart in the traditional way.  To utilise this information effectively some form of an automatic display is required that may take the form of chart plotters, Electronic Chart Systems (ECS) and the emerging ECDIS technology.

#### Restricted Waters

While similar to the Harbour Approach Phase, in the proximity to dangers and the limitations on freedom of manoeuvre, a Restricted Waters Phase can also develop during a coastal navigation phase, such as in various Straits around the world.

The Pilot or Master of a large vessel in restricted waters must direct its movement with great accuracy and precision to avoid grounding in shallow water, striking submerged dangers or colliding with other craft in a congested channel.  If a large vessel finds itself in an emerging navigational situation with no options to turn away or stop, it may be forced to navigate to limits measured within a few metres in order to avoid an accident.

Requirements for safety of navigation in the Restricted Waters Phase make it desirable for navigation systems to provide:

- accurate verification of position almost continuously;

- information depicting any tendency for the vessel to deviate from its intended track;

- instantaneous indication of the direction in which the ship should be steered to maintain the intended course.

These requirements are not currently achievable through the use of visual aids and ships’ radar alone, but as with Harbour Approach navigation, they can be achieved with a combination of DGPS and Electronic Charts Systems.

    - Measurement Errors and Accuracy

Good practice in both navigation and aids to navigation design dictates that an indication of the error or uncertainty in measuring a parameter or in obtaining a position fix should be reported along with the derived result.

#### Measurement Error

The Measurement error is defined as the difference between the true value and the measured value.

In general, three types of errors are recognised:

- Systematic errors: (or fixed or bias errors) are errors that persist and are related to the inherent accuracy of the equipment or result from incorrectly calibrated equipment.  This type of error can to some extent be foreseen and compensation applied.

- Random errors: cause readings to take random values either side of some mean value.  They may be due to the observer/operator or the equipment and are revealed by taking repeated readings.  This type of error can neither be foreseen nor totally compensated.

- Faults and mistakes: errors of this type can be reduced by appropriate training and by following defined procedures.

#### Accuracy

In a process where a number of measurements are taken, the term accuracy refers to the degree of conformity between the measured parameter at a given time and its true parameter at that time. (The term parameter includes: position, coordinates, velocity, time, angle, etc.)

For navigational purposes, four types of accuracy can be defined:

- Absolute accuracy (Geodetic or Geographic accuracy): the accuracy of a position with respect to the geographic or geodetic coordinates of the Earth.

- Predictable Accuracy: the accuracy with which a position can be defined when the predicted errors have been taken into account. It therefore depends on the state of knowledge of the error sources.

- Relative accuracy or Relational Accuracy: the accuracy with which a user can determine position relative to that of another user of the same navigation system at the same time.

- Repeatable Accuracy: the accuracy with which a user can return to a position whose coordinates have been measured at a previous time using uncorrelated measurements from the same navigation system.

For general navigation, the Absolute and Predictable accuracy are the principal concerns.  Repeatable Accuracy is of more interest to fishermen, the offshore oil and gas industry, ships making regular trips into an area of restricted waters and lighthouse authorities when positioning floating aids to navigation.

#### Accuracy of a Position Fix

A minimum of two lines of position (LOP) are necessary to determine a position at sea.  Since there is an error associated with each LOP, the position fix has a two dimensional error.  There are a number of ways of analysing the error boundary, however the radial position error relative to the true position, taken at the 95% probability level, has been adopted as the preferred method.

#### Navigational Position Fixing Measurements

Table 2.2 shows the typical accuracy (95% probability) achieved using common navigational instruments or techniques.

Table 2.2 Accuracy of some position-fixing processes and systems.

| Process | Typical accuracy (95%probability) | Accuracy at 1 nm (metres) |
| --- | --- | --- |
| Magnetic compass bearing on a light or landmark | +3º The accuracy may deteriorate in high latitudes | 93 |
| Gyro-compass bearing on a light or landmark | 0.75º X secant latitude (below 60º of latitude) | < 62 |
| Radio direction finder | +3º to +10º | 93 - 310 |
| Radar bearing | +1º Assuming a stabilized presentation and a reasonably steady craft. | 31 |
| Radar distance measurement | 1 % of the maximum range of the scale in use or 30 metres, whichever is the greater |  |
| LORAN-C / CHAYKA | 0.25 nm |  |
| GPS | 13 – 36 metres |  |
| DGPS (GNSS) (ITU-R M.823/1 Format) | 1 – 3 metres |  |
| Dead Reckoning (DR) | Approximately 1 nautical mile for each hour of sailing |  |

    - Hydrographic Considerations

#### Charts

The IMO definition of a nautical chart or nautical publication is a special-purpose map or book, or a specially compiled database from which such a map or book is derived, that is issued officially by or on the authority of a Government, authorised Hydrographic Office or other relevant government institution and is designed to meet the requirements of marine navigation.

The principal international organisation on charting matters is the International Hydrographic Organisation (IHO).

The IHO is an intergovernmental consultative and technical organization that was established in 1921 to support the safety in navigation and the protection of the marine environment. The object of the Organization is to bring about: - The coordination of the activities of national hydrographic offices; - The greatest possible uniformity in nautical charts and documents; - The adoption of reliable and efficient methods of carrying out and exploiting hydrographic surveys; - The development of the sciences in the field of hydrography and the techniques employed in descriptive oceanography. body responsible for determining international standards for the quality of hydrographic surveys and chart production.

#### Datum

In its simplest form, a datum is an assumed or defined starting point from which measurements are taken.

A more complex example of a datum is a Geodetic Datum used in the mathematical representation of the earth’s surface.  Many different data (plural of datum) have been devised over time to define the size and shape of the earth and the origin and orientation of coordinate systems for chart and mapping applications.  These have evolved from the consideration of a spherical earth, through to the geoid and ellipsoidal models, and also the planar projections used for charts and maps.

The geoid model considers the earth’s surface to be defined as the equipotential surface that would be assumed by the sea level in the absence of tides, currents, water density variations and atmospheric effects.

A further approximation uses an ellipsoid, which is a smooth mathematical surface, to give a best-fit match of the geoid.  Early ellipsoid models were developed to suit the mapping and charting of local regions or countries.  However they would not necessarily provide a satisfactory solution in other parts of the world.  Some nautical chart still carry a legend referring to a local datum, for instance, Ellipsoid Hayford or International – Datum Potsdam, Paris or Lisbon.

#### Chart Datum

Chart datum is defined as the datum or plane of reference to which all charted depths and drying heights are related.  It is relevant to a localised area and is a level that the tide will not frequently fall below.  It is usually defined in terms of Lowest Astronomical Tide (and in some cases by Indian Spring Low Water).

#### Levelling Datum or Vertical Control Datum

These are generic terms for levelling surfaces that are used to determine levels or elevations.  Using nautical charts as an example:

- water depths are measured from Chart Datum to the seabed;

- elevations of land masses and man-made features are referenced to either Mean High Water Springs (where there are predominantly semi-diurnal tides) or Mean High High Water (where there are predominantly diurnal tides) ;

- clearance heights for bridges are generally referenced to Highest Astronomical Tide.

The definition of these levels and other related levels are provided in Table 2.3.

Table 2.3 Description of some common levels

relevant to navigation in coastal and restricted waters.

| Levels and Description | Abbreviation |
| --- | --- |
| highest astronomical tide:- the highest tidal level which can be predicted to occur under average meteorological conditions and under any combination of astronomical conditions (IHO Dictionary, S-32, 5th Edition, 2244) | HAT |
| mean higher high water:- the average height of higher high waters at a place over a 19-year period. (IHO Dictionary, S-32, 5th Edition, 3140) | MHHW |
| mean high water springs:- the average height of the high waters of spring tides. Also called spring high water. (IHO Dictionary, S-32, 5th Edition, 3144) | MHWS |
| mean sea level:- the average height of the surface of the sea at a tide station for all stages of the tide over a 19-year period, usually determined from hourly height readings measured from a fixed predetermined reference level. (IHO Dictionary, S-32, 5th Edition, 3156) | MSL |
| mean low water springs:- the average height of the low waters of spring tides. Also called spring low water. (IHO Dictionary, S-32, 5th Edition, 3150) | MLWS |
| mean lower low water:- the average height of the lower low waters at a place over a 19-year period. (IHO Dictionary, S-32, 5th Edition, 3145) | MLLW |
| Indian spring low water:- a tidal datum approximating the level of the mean of the lower low water at spring tides. Also called Indian tidal plane. (IHO Dictionary, S-32, 5th Edition, 2427) ISLW was defines by G.H. Darwin for the tides of India at a level below MSL and is found by subtracting the sum of the harmonic constituents M2, S2, K1 and O1 from Mean Sea Level | ISLW |
| lowest astronomical tide:- the lowest tide level which can be predicted to occur under average meteorological conditions and under any combination of astronomical conditions. (IHO Dictionary, S-32, 5th Edition, 2936) | LAT |

#### Chart Datum Issues

Until the advent of satellite navigation, nautical charts were generally produced to local and national data.  The now widely used GPS positioning system uses an earth centred datum referred to as World Geodetic System 1984 (WGS-84) is considered to be the best compromise for representing the whole of earth’s surface.

Generally, WGS-84 is the geodetic system associated with the differential correction information broadcast by maritime DGPS stations using the ITU-R M.823/1 signal format.

The IHO Technical Resolution B1.1 recommends that all countries that issue national navigational charts should base these on the WGS 84 geodetic system.  For many countries this simple objective represents a formidable workload and will take a number of years to achieve.  Consequently, many nautical charts will continue to refer to data other than WGS-84 and discrepancies of several hundred metres can exist between a GPS derived position and the charted position.  During this transitional period, it is important for navigators and other persons using charts to:

- be aware of the datum applicable to the chart in use;

- include the applicable reference datum when communicating a measured position;

- determine whether or not a satellite derived position can be directly plotted onto a chart.  In some cases a chart will include information for adjusting a satellite derived position to align to the chart datum;

- be aware that some GPS receivers have the facility to automatically convert (and display) WGS-84 positions into other geodetic coordinate systems.  The user should be aware of the settings that have been applied to the receiver.

Examples of the style of note found on some charts is as follows:

SATELLITE-DERIVED POSITIONS Positions obtained from the Global Positioning System (GPS) in the WGS 1984 Datum must be moved 0.09 minutes SOUTHWARD and 0.06 minutes WESTWARD to agree with this chart.

SATELLITE-DERIVED POSITIONS Positions obtained from the Global Positioning System (GPS) in the WGS 1984 Datum can be plotted directly onto this chart.

SATELLITE-DERIVED POSITIONS Positions obtained from the Global Positioning System (GPS) in the WGS 1984 Datum cannot be plotted directly onto this chart. The difference between GPS positions and positions on this chart cannot be determined; mariners are warned that these differences may be significant and are therefore advised to use alternative sources of positional information, particularly when closing the shore or navigating in the vicinity of dangers.

#### Accuracy of Charts

At a national level, it is important that the Authorities responsible for aids to navigation and hydrographic services work together to ensure that both the network or mix of aids to navigation provided, and the available charts are appropriate for mariners to navigate safely.

The accuracy requirements for general navigation can be related to the scale of the chart necessary for each part of the passage.

Table 2.4 shows chart scales with the corresponding accuracy requirements recommended by IHO and the equivalent dimension of a 0.5 mm dot on a chart:

Table 2.4 Chart scales, applications and related accuracy considerations.

| Chart scale | Corresponding need for accuracy (metres). | Approximate pencil width (0.5 mm) equivalence (metres). | Application |
| --- | --- | --- | --- |
| 1:10,000,000 1:2,500,000 1:750,000 1:300,000 1:100,000 1:50,000 1:15,000 1:10,000 1:5,000 | 10,000 2,500 750 300 100 50 15 10 5 | 5000 1250 375 150 50 25 7.5 5 2.5 | Ocean navigation ------ Coastal navigation ------ Approach ------ Restricted waters Harbour plans |

#### Charted Buoy Positions

No reliance can be placed on floating aids always maintaining their exact positions. Buoys should, therefore be regarded with caution and not as infallible navigating marks, especially when in exposed positions; and a ship should always, when possible, be navigated by bearings of fixed objects or angles between them, and not by buoys.

# AIDS TO NAVIGATION

## Definition of Aids to Navigation

A marine Aid to Navigation is a device or system external to vessels that is designed and operated to enhance the safe and efficient navigation of vessels and/or vessel traffic.

## Scope

This chapter describes the major types of aids to navigation in current use and provides comments on the application and performance of the technology.  AIS and Vessel Traffic Services (VTS) are covered in the chapter, since these services can also satisfy the definition of an aid to navigation.

## Visual Aids to Navigation

### Visual Marks

#### Types

Visual marks for navigation can be either natural or man-made objects. They include structures specifically designed to assist navigation, as well as conspicuous features such as headlands, mountain-tops, rocks, trees, church-towers, minarets, monuments, chimneys, etc.

Visual marks can be provided with a light if navigation at night is required, or left unlit if daytime navigation is sufficient.

Navigation at night is possible, to a limited extent, if the unlit aids are provided with:

- a radar reflector and the vessel has a radar, or;

- retro-reflecting material and the vessel has a searchlight.  This approach would generally only be acceptable for small boats operating in safe waterways and with the advantage of local knowledge.

#### Visual Aids to Navigation

##### Description

Visual aids to navigation are purpose-built facilities that communicate information to a trained observer on a vessel for the purpose of assisting the task of navigation. The communication process is referred to as Marine Signalling.

Common examples of visual aids to navigation include lighthouses, beacons, leading (range) lines, lightvessels, buoys (lit or unlit), daymarks (dayboards) and traffic signals.

The effectiveness of a visual aid to navigation is determined by factors such as:

- type and characteristics of the aid provided;

- location of the aid relative to typical routes taken by vessels;

- distance (range) of the aid from the observer;

- atmospheric conditions;

- contrast relative to background conditions (conspicuity);

- the reliability and availability of the aid.

Refer to IALA publications: - The IALA Recommendation on Availability Objectives of AtoN Services; - The IALA Guideline on the methodology of Collecting Information on Reliability and Failures of AtoN Equipment; - Chapter 6 of this Guide.

##### Distinguishing Features

Visual aids to navigation are distinguished by:

- type;

  - fixed structure;

  - floating platform;

- location;

  - inclusion of subsidiary aids

  - relationship to other aids to navigation and observable features

- characteristics;

  - shape;

  - size;

  - elevation;

  - colour;

  - lit/unlit;

  - signal character;

  - light intensity;

  - sectors;

  - construction materials;

  - retro-reflective features;

  - names, letters and numbers.

#### Signal Colours

IALA has made recommendations on colours for lit (lighted) aids to navigation and for surface colours for visual signals on aids to navigation.

- Lights use a four-colour system comprised of white, red, green and yellow that conforms to the International Commission on Illumination (CIE) publication No.2.2 (1975) “Colours of Signal Lights”;

- Recommended surface colours for visual signals on aids to navigation are as follows:

  - Ordinary colours should be limited to white, black, red, green, yellow and blue.

  - Orange and fluorescent red, yellow, green and orange may be used for special purposes requiring high conspicuity.

Refer to IALA publications: - Recommendation for the colours for light signals on aids to navigation, December 1977 ; - Recommendation for the use of retro-reflecting material on aids to navigation marks within the IALA Maritime Buoyage System (E106), May 1998; - Recommendation for the surface colours used as visual signals on aids to navigation (E108), May 2004.

The CIE standard on the measurement of colours (colorimetry) is based on three reference colours (ie. a tri-stimulus system) that in varying combination can generate the spectrum of colours.  A particular colour is described by the symbols; x, y and z that represent the proportions of the reference colours.

Using ratios of the tri-stimulus values, such that: x + y + z = 1, colours can be defined in terms of chromaticity using just the x and y values.  The advantage of this arrangement is that colours can be mapped on a two-dimensional chromaticity diagram.

CIE colour standards for marine signalling can be depicted as areas on the chromaticity diagram.  These areas are defined by boundaries expressed as functions of x and y (equations).

If the chromaticity co-ordinates of a coloured light, filter material or a paint product are known, its acceptability for marine signalling applications can easily be determined.

Fig 3.1 Illustrates the colour zones on the 1931 CIE chromaticity diagram.

(Note that the colour rendering is indicative only).

The CIE standard for marine signalling colours has recently been revised, with some adjustments to the boundaries of signal colours.  An extract from the CIE illustration of the colour boundaries is shown in Fig 3.2.  For further details on the chromaticity coordinates and equations of the boundaries, please refer to

CIE S 004/E-2001 Colours of Light Signals

Fig 3.2 IALA allowed chromaticity areas for red, orange, yellow, green, blue, white and black of Ordinary Surface Colours plotted on the 1983 CIE chromaticity diagram (Courtesy of CIE)

#### Visibility of a Mark

The visibility of a mark is affected by one or more of the following factors:

- observing distance (range);

- curvature of the Earth;

- atmospheric refraction;

- atmospheric transmissivity (meteorological visibility);

- height of the aid above sea level;

- observer's visual perception;

- observer's height of eye;

- observing conditions (day or night);

- conspicuity of the mark (shape, size, colour, reflectance, and  the properties of any retro-reflecting material);

- contrast (background lighting);

- mark lit or unlit;

- intensity and character;

#### Observational Factors

##### Meteorological Visibility

Meteorological visibility (V) is defined as the greatest distance at which a black object of suitable dimensions can be seen and recognised by day against the horizon sky, or, in the case of night observations, could be seen if the general illumination were raised to the normal daytime level.  It is usually expressed in kilometres or nautical miles.

##### Atmospheric Transmissivity

The atmospheric transmissivity (T) is defined as the transmittance or proportion of light from a source that remains after passing through a specified distance through the atmosphere, at sea level.

Since the atmosphere is not uniform over the observing distances of most visual aids, a representative value is used:

- typically, the atmospheric transmissivity is taken as T = 0.74 over one nautical mile;

- a figure of T = 0.84 is occasionally used in regions where the atmosphere is very clear.

A number of countries collect data on atmospheric transmissivity for different parts of their coastline.  This enables the luminous range of lights to be:

- calculated more precisely, and;

- better matched to local conditions and user requirements.

##### Atmospheric Refraction

This phenomenon results from the normal decrease in atmospheric density from the Earths’ surface to the stratosphere.  This causes light rays that are directed obliquely through the atmosphere to be refracted (or bent) towards the Earth.

##### Contrast

The ability to detect differences in luminance between an object and an otherwise uniform background is a basic visual requirement and is used to define the term contrast.  It is represented by the equation:

C = (Lo −LB)LB

where:

C   =       contrast

| LB = | luminance of background (cd/m2) |
| --- | --- |
| Lo = | luminance of object (cd/m2) |

The contrast at which an object can be detected against a given background 50% of the time is called the threshold contrast.  For meteorological observations, a higher threshold must be used to ensure that the object is recognised.

A contrast value of 0.05 has been adopted as the basis for the measurement of meteorological optical range.

##### Use of Binoculars

While it is generally assumed that observations will be made with the naked eye, mariners will quite often use binoculars.  This can allow:

- a light being observed, or the characteristics resolved, at a greater luminous range than with the naked eye;

- a limited improvement in the sensitivity of leading lights;

  - about a 30% improvement in the detectable difference from a given bearing;

- the identification of a light operating against background lighting conditions.

Generally, the most suitable binoculars for use at sea are considered to be:

- 7 x 50 type at night, and;

- 10 x 50 binoculars by day.

#### Range of a Visual Mark

The range of an aid to navigation can broadly be defined as the distance at which the observer’s receiver can detect and resolve the signal.  In the case of visual marks the observer’s receivers are his/her eyes.  This broad definition of range leads to a number of more specific definitions that are described below.

##### Geographical Range

This is the greatest distance at which an object or a light source could be seen under conditions of perfect visibility, as limited only by the curvature of the earth, by refraction of the atmosphere, and by the elevation of the observer and the object or light. (IALA International Dictionary of Aids to Marine Navigation).  A Geographical Range table is shown in Table 3.1.

Table 3.1 IALA Geographical Range Table.

| Geographical Range in Nautical Miles |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Observer eye height metres | Elevation of Mark metres |  |  |  |  |  |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 10 | 50 | 100 | 200 | 300 |
| 1 | 2.0 | 4.1 | 4.9 | 5.5 | 6.1 | 6.6 | 8.5 | 16.4 | 22.3 | 30.8 | 37.2 |
| 2 | 2.9 | 4.9 | 5.7 | 6.4 | 6.9 | 7.4 | 9.3 | 17.2 | 23.2 | 31.6 | 38.1 |
| 5 | 4.5 | 6.6 | 7.4 | 8.1 | 8.6 | 9.1 | 11.0 | 18.9 | 26.9 | 33.3 | 39.7 |
| 10 | 6.4 | 8.5 | 9.3 | 9.9 | 10.5 | 11.0 | 12.8 | 20.8 | 26.7 | 35.1 | 41.6 |
| 20 | 9.1 | 11.1 | 12.0 | 12.6 | 13.1 | 13.6 | 15.5 | 23.4 | 29.4 | 37.8 | 44.2 |
| 30 | 11.1 | 13.2 | 14.0 | 14.6 | 15.2 | 15.7 | 17.5 | 25.5 | 31.4 | 39.8 | 46.3 |

The values in Table 3.1 are derived from the formula:

Rg=2.03×(ho+ Hm)

where:

| Rg = | geographical range (nautical miles) |
| --- | --- |
| ho = | elevation of observer’s eye (metres) |
| Hm = | elevation of the mark (metres) |

Note

The factor 2.03 accounts for refraction in the atmosphere.

Climatic variations around the world may lead to different factors being recommended.   Typical range of factors is 2.03 to 2.12.

##### Meteorological Optical Range

This is the distance through the atmosphere that is required for 95% attenuation in the luminous flux of a collimated beam of light using a source colour temperature of 2700ºK.

The meteorological optical range is related to the atmospheric transmissivity by the formula:

V=dlog0.05logT		or		T=0.05dV

Where:

V  =	meteorological optical range (nautical miles)

d  =	distance (nautical miles)

T  =	atmospheric transmissivity

It is often convenient to simplify the above expression by giving the distance term a  value of one, such that:

T=0.051V		or		TV=0.05

##### Visual Range

This is the maximum distance at which the contrast of the object against its background is reduced by the atmosphere to the contrast threshold of the observer.  The visual range can be enhanced if the observer uses binoculars, although the effectiveness depends on the stability of the observer’s platform.

##### Luminous Range

This is the maximum distance at which a given light signal can be seen by the eye of the observer at a given time, as determined by the meteorological visibility prevailing at that time. It takes no account of elevation, observer's height of eye or the curvature of the Earth.

##### Nominal Range

Nominal range is the luminous range when the meteorological visibility is 10 nautical miles, which is equivalent to a transmission factor of T = 0.74.  It is generally the figure used in official documentation such as charts, Lists of Lights, etc.

Nominal range assumes that the light is observed against a dark background, free of background lighting.

    - Aids to Navigation Lights

#### Light Sources

##### Brief History

Until the first application of electricity to lighting late in the nineteenth century, all artificial light was produced by fire.  Illuminants progressed from pyres of wood (used up until the 1800’s), to oil wick lamps, vaporised oil and gas burners and then electric arc and tungsten filament lamps.  Optical devices matched these developments, first with reflector systems and later with lenses.

It is interesting to note that the efforts to understand human perception of light and to improve the efficiency and effectiveness of aids to navigation illuminants and optical apparatus were for many years at the forefront of scientific endeavours.

The glass lens design pioneered by Fresnel around 1820 remains a principal element of the modern aid to navigation light.  Present day lenses are often made of plastic rather than glass.

A few countries still have gas lights that burn acetylene or propane. However, the majority of aids to navigation lights use electric lamps of various types.  Increasingly, these lamps draw their power from renewable energy sources such as solar, wind or wave power.

Electric lamps have been specifically designed for aids to navigation applications.  However, lamps selected from the enormous range of commercial products have also been used or adapted for aids to navigation.

Light emitting diode (LED) technology is rapidly emerging as an alternative to filament lamps.

##### Gas Lights

Acetylene

The acetylene light has a special place in the history of aids to navigation, primarily for being the first reliable means of automating lighthouses, buoys and beacons during the earlier part of the 20th century.  Acetylene lighting systems originated from the inventions of Gustaf Dalen and were made by a number of suppliers. Acetylene gas has the unusual property of burning with a white flame when correctly mixed with air. This enabled the development of exceptionally reliable open flame lanterns.

Acetylene lighting technology was further enhanced by the development of the Dalen “mixer” that allowed gas and air to be drawn into a chamber and then consumed in an incandescent mantle to produce a brighter light source than the open flame type.  The incandescent mantle can be operated as a flashing source inside a fixed lens or as a continuous source inside a rotating lens.  Related developments included a gas-operated mechanism for rotating a lens and a clockwork powered automatic mantle changing device.

Refer to IALA publication: - Practical Note for the Safe Handling of Gases, October 1993.

Propane and Butane

Propane and butane gases have been used as fuels for gas lighting systems.  The lighting equipment has to use an incandescent mantle burner as both gasses burn with yellow/orange flames when an open flame burner is used.

Gas lighting installations continue to be used in some countries, where they are chosen for their robustness and simplicity of operation. They have been replaced by solar/electric installations in many countries generally resulting in considerable savings in operating costs.

##### Electric Lights

Incandescent filament

Tungsten filament

Description of operation:

Can be operated directly from an appropriate electrical supply.

Nominal voltage 6 to 240 V both AC and DC

These light sources have been used in most countries from at least the early 1900s. Many special lamp designs have been used over the years, as filament size, shape, and location must be well matched to the lens system.

Typical use:

All types of lighted beacons  (leading lights, sector lights, 360° lights, lanterns on light buoys...)

Some countries and manufacturers have adopted standard designs, with reference codes, for lamps designed specially for lighthouse applications. These designs typically include filament supports to maintain filament shape and ensure an even output over 360 degrees azimuth.

Technical data:

- Power:	From 2  to 1000 Watt (3 500 W lamps have been used in the past)

- Efficiency:	From 9 to 19 lumens/ W

- Operating life:	From 300 to1500 hours

Advantages:

Coding can be easily achieved by interrupting the electrical supply.

Filament geometry can be designed to match the optic.

Diffusing envelope (e.g. pearl or etched) can improve lens illumination when used in older optics, but at the expense of reduced intensity.

Broad colour spectrum provides good performance with most coloured filters.

Output reasonably stable over lifetime, but lamp envelope may start to blacken before lamp failure.

Remote monitoring by current sensing is simple.

Disadvantages:

Relatively short lifetime.

Specialist AtoN lamps are expensive.

Colour rendering is not in the preferred white region (tends towards yellow).

Efficiency poor.

Safety:

High envelope temperature when in use.

Operating voltage may be hazardous.

Possible risk of arcing

General glass hazard

Disposal:

Safe disposal appropriate for metal and glass waste.

Tungsten halogen

Description of operation:

The tungsten halogen lamp encloses a small amount of halide with inert gas, and the tungsten evaporating from the hot filament combines with the halogen to diffuse around the envelope wall. Due to careful design the envelope operates at high temperature, and this prevents deposition of tungsten on the glass. The tungsten-halide is then carried by convection towards the filament where it is decomposed and the tungsten metal deposited back onto the filament.

Can be operated directly from an appropriate electrical supply.

Nominal voltage 12 to 240 V both AC and DC.

Typical use:

All types of lighted beacons, but several lamps could be used in a cluster in large rotating optics, to produce a large light source similar to the original non-electric light source.

Technical data:

- Power:	From 5 to 1000 W, exceptionally 1500 and 2000 W

- Efficiency:	20 to 25 lumen/W

- Operating life:	300 to 4000 hours.         Note- some very bright lamps have short working life.

Advantages:

Coding can be easily achieved by interrupting the electrical supply, but see below.

Higher luminance. than tungsten lamps.

Output very stable over lifetime.

Colour rendering good in white preferred region.

High performance general-purpose lamps available at low cost.

Small lamp sizes (10 to 100W) are very mechanically robust.

Envelope sizes are typically smaller than tungsten lamps, and may allow smaller optic system sizes.

Disadvantages:

The filament size is usually small hence the geometry is poor when modernising old lens systems.

Generally low operating voltage results in high current requiring careful design of lamp holder and associated wiring.

A cluster of several lamps will be required to match these small lamps to existing large optics.

Lamps are not made specifically for AtoN use and specifications may be changed without notice.

The flashing of tungsten halogen lamps may lead to the interruption of the halogen cycle with consequent blackening of the envelope and premature failure. Practical trials are recommended with the proposed operating voltage and duty cycle or consultation with other IALA members.

Lamps should not be touched with bare hands due to consequential reduction in lamp life.

Safety:

Operating voltage may be hazardous.

General glass hazards.

Very high surface temperatures because of the small envelope size.

Possible high UV radiation risk (dependant on lamp type).

Risk of explosion with high-pressure lamp types.

Disposal:

Local and national waste disposal regulations should be consulted regarding tungsten halogen lamps.

###### Discharge lamps

Fluorescent

Description of operation:

110 to 240 V system voltage with control circuitry to provide a high starting voltage.

Typical use:

Direction arrows, signs and light tubes or bars used for leading lights. Applications where large areas of illumination are an advantage.

Technical data:

- Power:	8 to 100 W

- Efficiency:	80 to 100 lumen/W

- Operating life:	Up to 20000 hours

Advantages:

High luminous efficiency.

Large illuminated area. In suitable applications no optical elements are needed thus providing a very low cost AtoN.

Very wide range of commercially available products at low prices.

Many colours available (no additional colour filters needed).

Disadvantages:

Low luminance.

Difficult to use with lens systems due to source size.

Light output falls considerably over lifetime.

Requires control circuitry matching lamp and supply voltage.

Not normally coded as complex circuitry needed to flash.

Possible EMC problems.

Safety:

Mains voltage.

General glass hazard.

Internal tube coatings may be hazardous if exposed and they contain traces of gaseous mercury.

High voltage due to starting equipment.

Disposal:

Tube coatings may be hazardous and contain traces of mercury. Consult local and national disposal regulations.

Low-pressure sodium vapour lamps

Description of operation:

110 and 240 V AC with associated control circuitry.

NOTE: Only available as yellow colour

Typical use:

Flood lighting and external illumination of structures, towers, locks etc.

Technical data:

- Power:	20 to 180 W

- Efficiency:	180 lumen/W

- Operating life:	10 000 hour

Advantages:

Long life.

High luminous efficiency.

Mercury free.

Low envelope surface temperature.

Can be used to provide yellow signal colour.

Minimum attraction for insects.

Disadvantages:

Only produces yellow light.

Low luminance.

Not practical to flash (code).

Limited operating positions.

Safety:

General glass hazards.

High AC voltage.

Chemical hazard due to sodium content.

Disposal:

Consult local and national regulations regarding sodium disposal.

                - High Pressure Sodium Vapour Lamps

Description of operation:

110 or 240 V AC with associated control circuitry.

Typical Use:

White lamps may be used as AtoN light source.

Technical Data:

- Power	50 to 400W

- Efficiency	90 lumens/W

- Operating Life	10000 hour

Advantages:

Long life.

Mercury free.

High efficiency.

Available in white.

Disadvantages:

Cannot be coded (flashed).

Only practical as white Low red content makes colour filtering impractical.

High strike voltage to start.

Complex lamp changer required due to long warm-up period and  cool-down necessary before restart.

Arc tube geometry is poor for most optics.

Light output falls over life and white colour degrades to yellow.

Safety:

General glass hazards.

High voltages.

Chemical hazards giving rise to disposal or health problems.

Disposal:

Consult national and local regulations regarding sodium disposal.

Metal halide

Description of operation:

The metal halide lamp is one of a family of HID [High Intensity Discharge] lamps, and its arc tube is made of silica glass. The principle of emission is as follows. 1) High voltage from the ballast initiates current flow between the electrodes. 2) As the lamp temperature rises, metals in the lamp evaporate, and light emission occurs.

110 and 240 V    Working with control circuitry allows input voltages 12V to 240V

Typical use:

Used as a fixed light source in rotating optics, fixed lenses with rotating screens and general floodlighting.

Technical data:

- Power:	10 to 2000 W

- Efficiency:	80 to110 lumen/W

- Operating life:	6 000  to 20 000 h

Note life is very dependant on number of times that lamp is switched on

Advantages:

High luminous efficiency.

Clear envelope types have high illuminance.

Coated envelope types have good geometry for traditional lenses.

Long life.

Many commercial lamp types available.

Colour rendering is good, within IALA white preferred region.

Absence of a filament means good resistance to vibration and shock.

Disadvantages:

Not practical to code (flash).

Initial warm-up is slow.

Cooling time needed before re-strike hence complex lamp changer design.

Difficult to remote monitor by simple current sensing.

Light output falls with life.

Red spectrum is limited so red filtering is possible but poor, green is good.

Safety:

High voltage hazards.

High UV radiation.

Possible EMC problems.

Possible explosion hazard.

General glass hazard.

May contain hazardous metal.

Disposal:

Consult local and national regulations for these lamps, as there is some mercury content.

Xenon Lamps

Description of operation:

Xenon lamps are discharge lamps, with the xenon gas enclosed in a silica tube at high pressure. An electrical discharge through the xenon gas generates a high intensity white light. [Xenon discharge is commonly used in camera flash guns.]

110 and 240V typically.

Charging DC supply requires complex control circuitry.

Available as pulsed or continuous discharge types.

Typical use:

A specialised light source used where high intensity is of paramount importance. Can be used in fixed or rotating optics.

Technical data:

- Power:	150 to 2000 W

- Efficiency:	35 lumens/W

- Operating life:	2000 h

Advantages:

High luminance enabling high intensities to be achieved with suitable optics.

Broad white colour spectrum allowing good colour filtering.

Disadvantages:

Electrical control system is complicated.

Lamp changer design is very complex.

Short lamp life.

Electronic control components have short life.

Relatively expensive.

Power consumption is variable due to the charging/discharging cycle of the system, resulting in varying loads on the power supply system.

Safety:

High voltages.

Possible danger of explosion as the pressure in the lamp is high.

High UV radiation

High surface temperature

General glass hazards

Disposal:

Check local and national regulations for the lamp type. There may be some mercury content.

###### Light Emitting Diode (LED)

Description of operation:

Coloured LEDs

LEDs are electronic semiconductor devices that produce near monochromatic light. The semiconductor junction is encapsulated in a clear plastic housing that usually incorporates a lens. Several LEDs may be grouped together in a cluster or an array to provide a light source of the required size and intensity with lamp redundancy. New high power LEDs allow short range lanterns using a single LED.

LEDs operate from a low voltage DC supply. Correct operation depends on accurate control of the supply current.

White LEDsA semiconductor junction emitting blue/violet light is encapsulated with an integral phosphor such that both blue and broad band yellow light are emitted together to form a near white light.  Research is in progress on combining red and green LED lights to produce a white lights within the IALA chromaticity specification.

LED marine lanterns are sometimes reported as having intense colours and ranges longer than the current IALA calculation method would suggest. Current work by IALA is investigating this.

Typical use:

Lighted beacons on buoys and other short range AtoN, but longer range LED lanterns are appearing on the market.

Range lights consisting of flat arrays of LEDs or single high power LEDs

Signs and signals formed by arrays of LEDs in the shape of letters, numerals, signs etc.

Technical data:

- Power

Single LED  1mW to 5W

LED cluster in lantern 1W to 60W or even higher.

- Efficiency:

This depends on colour:

  - Red and green – 25 to 30 lumen/W

  - Yellow	-20 to 30 lumen/W

  - White –20 to 30 lumen/W

  - Blue – 10 lumen/W

NOTE- Extensive research in LED technology is continually providing more efficient products

- Operating life:

Depends on the LED junction operating temperature and operating    environment

Possible values in excess of 100,000 hours for a single diode

Advantages:

Very long life (so long as input power and temperature is carefully controlled) and hence low whole life costs.

Life is so long that lampchangers are not considered necessary.

High luminous efficiency in red and green

Light produced in saturated signal colours therefore coloured filters not needed

Mechanically robust when compared with conventional lamps

Rise and fall times are very fast.

Relatively cool operation.

Disadvantages:

Complex electronic control needed to achieve long life and high performance.

Generally difficult to match to existing optics.

Luminous efficiency decreases slowly with life.

White LEDs will be outside the new CIE (2001) white colour region.

White LEDs will be very inefficient with red and green filters.

Lamp life can be severely reduced if input power and temperature are not carefully controlled.

LEDs are not yet suitable for long range lights.

Safety:

No special hazard.

Disposal:

Consult National and local regulations regarding disposal of semiconductors.

###### Lasers

A laser is a device that produces a coherent collimated beam of monochromatic light.

Their use has not been established in AtoN light systems, despite the work over some decades. However there are two areas of development.

(1) High power lasers can be used to provide a line of light in the sky where particles of dust, water etc. are illuminated by the laser beam to provide a leading line. These devices require considerable electrical power (several kW). Detailed safety procedures are required for operation and servicing.

(2) Low power lasers are being trialled by the Canadian Coastguard where the laser is aimed directly at the mariner. Different coloured lasers are used to identify areas of navigational importance. The laser light is visible at useful range in daylight.

Power requirements are low (tens of Watts). The laser projectors are expensive and require complex control systems. Detailed procedures are required for safe servicing.

Laser trials are also taking place in Russia and Japan.

Advantages

Single wavelength

Strong directivity

Simple optic design

Disadvantages

High power requirements for high power lasers

System complexity may be a problem at some locations

Safety

Possible eye damage under certain conditions, if appropriate ASTM safety standards are not applied.

Refer to IALA publication: - Guidelines on new light sources and associated power supplies, December 2001.

#### Photometry of Marine Aids to Navigation Signal Lights

##### Measurement of Light

The behavior of light, in physics, is normally seen in the context of either a form of electromagnetic radiation or particle motion.  The latter includes the concept of “rays” of light that are used in analyzing the interactions of light and lenses.

The units of interest for  electromagnetic applications of light are generally meters (wavelength) and watts (power).

The study of photometry and the use of lights for signal application has necessitated a parallel set of units to be developed to  account for the physiological aspects of how the human eye evaluates a light source.

The spectral sensitivity of the human eye (or the response of the eye to different coloured light) has been evaluated in tests of large numbers of people.  The results have been presented as a standard spectral sensitivity distribution or V-lambda curve.

Fig 3.6 Spectral sensitivity distributions or V-lambda curves

for the human eye also showing the difference between day and night vision.

##### Units of Measurement

Table 3.2 Photometric units of measurement.

| Term | Description | Unit | Abbreviation |
| --- | --- | --- | --- |
| Luminous flux | This is the total light emitted from the source (ie. lamp) The peak sensitivity of the human eye occurs at about 555 nanometres, a wavelength that corresponds to green. At this wavelength, the photometric equivalent of one watt is defined as 680 lumens. | lumens | lm |
| Luminous intensity | This is the part of the luminous flux in a particular direction. Also expressed as the luminous flux per solid angle (or steradian) | candela | cd |
| Luminance (Brilliance) | This is the portion of the luminous flux emitted in a specific direction by a surface element of the luminous body. This variable is an important term for rating the brightness impression of light sources and illuminated objects. Luminance is the parameter that causes the response in the eye. | candelas per square meter and also as candelas per square centimetre | cd/m2 cd/cm2 |
| Illuminance | This is the density of the luminous flux incident on a surface. It is the quotient of the luminous flux by the area of the surface when the surface is uniformly illuminated | lux (lumens/square metre) | lx |
| Luminous efficacy | This is the efficacy with which electrical power is converted to visible radiation. | lumens per watt of electrical power consumed |  |
| Colour temperature | This is a measure of how yellow or blue-white the light appears to the eye. It is related to the temperature of a tungsten filament. | Kelvin | ºK |
| Colour rendering index CRI | characterises the colour rendering quality of the light from a lamp. It is the same for all incandescent lamps by definition and equal to the maximum value of 100. |  |  |

##### Threshold of Illuminance

In physical terms, the threshold of illuminance is the lowest level of illuminance from a point source of light, against a given background level of luminance, that causes a visual response at the eye.

For visual signalling applications, the threshold of illuminance (E) is taken to be 0.2 micro lux.

In the case of leading lights of limited range and with a high level of shore illumination, the above figures may be found too low.

It is recommended that to observe the relative position of the lights easily and to derive the maximum possible accuracy from leading lights, it is generally necessary to have a minimum illuminance of 1 micro lux at the eye of the observer. Refer to IALA publication: - Recommendation for Leading Lights (E112), May 1998.

This condition is to be met at the outer limits of the useful segment for the minimum meteorological visibility under which the leading lights are to be used.

##### Luminous intensity

The luminous intensity of a navigation light is directly proportional to the luminance of the light.  The dimension of the light source is also important because the luminous intensity of any given fixed lens is proportional to the area of the light source.

##### Inverse Square Law

Light emitted from a source radiates out in all directions. For a point source, the wave fronts of light can be imagined to generate a series of spherical surfaces.  As shown in Fig 3.7, the further the light travels from the source, the greater is the surface area of the sphere and consequently, the lower the illuminance.  Since illuminance is measured in lumens/metre2 and the surface area of a sphere increases in proportion to the square of the radius, the illuminance decreases in proportion to the square of the distance from the source.  The decline in illuminance with distance is described as an inverse-square law.

.

Fig 3.7 Illustration of the Inverse-Square-Law concept.

##### Allards Law

The illuminance of a light source reaching an observer’s eye determines whether the light is seen.  The relationship between the illuminance produced at the observer’s eye, the luminous intensity of the light source and the atmospheric transmissivity is known as Allards Law:

E = I×Tdd2

where:

| E = | illuminance at the observer’s eye (lm/m2) |
| --- | --- |
| I = | effective intensity of the light source (cd) |
| T = | atmospheric transmissivity |
| d = | distance between the light source and the observer. [Because T is measured per nautical mile, d in the numerator must also be in nautical miles. In the denominator, d is in metres] |

Note

Allards law applies only when the luminance of the background is small compared to the average illuminance of the light.

Where average background luminance is large, as typically happens during the daytime, the equation becomes:

E = [I − (L − L')A] × Tdd2

where:

| L = | luminance of the background of the light in candelas per unit and is measured in the direction of the line of sight from a position near the light (cd/m2) |
| --- | --- |
| L' = | average luminance of the unlighted projector in candelas per unit and is measured in the direction of the line of sight from a position near the light (cd/m2) |
| A = | area of the entire projector projected on a plane normal to the line of sight (m2) |
| (L - L' )A= | intensity required of the light to make its average luminance equal to that of the background (cd) |

#### Rhythms / Character

IALA has produced a recommendation on the characters for light on aids to navigation

Refer to IALA publication: - Recommendation for the rhythmic characters of lights on aids to navigation (E110), May 1998.

The tables of classifications and specifications of aid to navigation characters from Recommendation E110 are reproduced below.

(EDITORIAL NOTE: WG 4 HAS BEEN REQUESTED TO REVIEW THE NEED FOR GRAPHICAL REPRESENTATIONS OF THE CHARACTERS TO ASSIST WITH INTERPRITATION OF THE REQUIREMENTS)

| CLASSIFICATION OF THE RHYTHMIC CHARACTERS OF LIGHTS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Class | Abbreviation | General description | IALA Specification | Particular use in the IALA Maritime Buoyage System |
| 1 | FIXED LIGHT | F | A light showing continuously and steadily. | A single fixed light should be used with care because it may not be recognized as an aid to navigation light. | A single fixed light shall not be used. |
| 2 | OCCULTING LIGHT |  | A light in which the total duration of light in a period is longer than the total duration of darkness and the intervals of darkness (eclipses) are usually of equal duration. | A light in which the total duration of light in a period is clearly longer than the total duration of darkness and all the eclipses are of equal duration. |  |
| 2.1 | Single-occulting light | Oc | An occulting light in which an eclipse is regularly repeated | The duration of an appearance of light should not be less than three times the duration of an eclipse. The period should not be less than 2 s | A single-occulting White light indicates a safe-water mark. |
| 2.2 | Group-occulting light | Oc(#) eg. Oc(2) | An occulting light in which a group of eclipses, specified in number, is regularly repeated. | The appearances of light between the eclipses in a group are of equal duration, and this duration is clearly shorter than the duration of the appearance of light between successive groups. The number of eclipses in a group should not be greater than four in general, and should be five only as an exception. The duration of an appearance of light within a group should not be less than the duration of an eclipse. The duration of an appearance of light between groups should not be less than three times the duration of an appearance of light within a group. In a group of two eclipses, the duration of an eclipse together with the duration of the appearance of light within a, group should not be less than 1 s. In a group of three or more eclipses, the duration of an eclipse together with the duration of an appearance of light within the group should not be less than 2 s. | A group-occulting Yellow light indicates a special mark. |
| 2.3 | Composite group-occulting light | Oc(2 + 1) | A light similar to a group-occulting light except that successive groups in a period have different numbers of eclipses. | This class of light character is not recommended because it is difficult to recognize. |  |
| 3 | ISOPHASE LIGHT | Iso | A light in which all the durations of light and darkness are clearly equal. | The period should never be less than 2 s, but preferably it should not be less than 4 s in order to reduce the risk of confusion with occulting or flashing lights of similar periods. | An isophase White light indicates a safe-water mark. |
| 4 | FLASHING LIGHT |  | A light in which the total duration of light in a period is shorter than the total duration of darkness and the appearances of light (flashes) are usually of equal duration. | A light in which the total duration of light in a period is clearly shorter than the total duration of darkness and all the flashes are of equal duration. |  |
| 4.1 | Single-flashing light | Fl | A flashing light in which a flash is regularly repeated (at a rate of less than 50 flashes per minute). | The duration of the interval of darkness (eclipse) between two successive flashes should not be less than three times the duration of a flash. The period should not be less than 2 s (or not less than 2.5 s in those countries where a quick rate of 50 flashes per minute is used). | A single-flashing Yellow light indicates a special mark. |
| 4.2 | Long-flashing light | LFI | A single-flashing light in which an appearance of light of not less than 2 s duration (long flash) is regularly repeated. |  | A long-flashing White light with a period of 10 s indicates a safe-water mark. |
| 4.3 | Group-flashing light | Fl(#) eg. Fl(3) | A flashing light in which a group of flashes, specified in number, is regularly repeated. | The eclipses between the flashes in a group are of equal duration, and this duration is clearly shorter than the duration of the eclipse between successive groups. The number of flashes in a group should not be greater than five in general, and should be six only as an exception. The duration of an eclipse within a group should not be less than the duration of a flash. The duration of an eclipse between groups should not be less than three times the duration of an eclipse within a group. In a group of two flashes, the duration of a flash together with the duration of the eclipse within the group should not be less than 1 s. In a group of three or more flashes, the duration of a flash together with the duration of an eclipse within a group should not be less than 2 s (or not less than 2.5 s in those countries where a quick rate of 50 flashes per minute is used). | A group-flashing White light with a group of two flashes, in a period of 5 s or 10 s, indicates an isolated-danger mark. A group-flashing Yellow light with a group of four, five or (exceptionally) six flashes indicates a special mark |
| 4.4 | Composite group-flashing light | eg Fl(2 + 1) | A light similar to a group-flashing light except that successive groups in a period have different numbers of flashes. | Light characters should be restricted to (2 + 1) flashes in general, and should be (3 + 1) flashes only as an exception. | A composite group-flashing Red or Green light with a group of (2 + 1) flashes indicates a modified lateral (preferred-channel) mark. A composite group-flashing Yellow light indicates a special mark. |
| 5 | QUICK LIGHT |  | A light in which flashes are repeated at a rate of not less than 50 flashes per minute but less than 80 flashes per minute. | A light in which identical flashes are repeated at the rate of 60 (or 50) flashes per minute. The higher rate of flashing is preferred. |  |
| 5.1 | Continuous quick light | Q | A quick light in which a flash is regularly repeated. |  | A continuous quick White light indicates a north cardinal mark. |
| 5.2 | Group quick light | Q(3) Q(9) IQ(6) + LFl | A quick light in which a specified group of flashes is regularly repeated. | The number of flashes in a group should be three or nine. An exceptional light character is reserved for use in the IALA Maritime Buoyage System to indicate a south cardinal mark. | A group quick While light with a group of three flashes, in a period of 10 s, indicates an east cardinal mark. A group quick White light with a group of nine flashes, in a period of 15 s, indicates a west cardinal mark. A group quick White light with a group of six flashes followed by a long flash of not less than 2 s duration, in a period of 15 s, indicates a south cardinal mark. |
| 5.3 | Interrupted quick light | IQ | A quick light in which the sequence of flashes is interrupted by regularly repeated eclipses of constant and long duration. | The number of flashes in a period should be at least eight. The duration of the long eclipse should not be less than 3 s. | An interrupted quick light should not be used. |
| 6 | VERY QUICK LIGHT |  | A light in which flashes are repeated at a rate of not less than 80 flashes per minute but less than 160 flashes per minute. | A light in which identical flashes are repeated at the rate of 120 (or 100) flashes per minute. The higher rate of flashing is preferred. |  |
| 6.1 | Continuous very quick light | VQ | A very quick light in which a flash is regularly repeated. |  | A continuous very quick White light indicates a north cardinal mark. |
| 6.2 | Group very quick light | VQ(3) VQ(9) VQ(6) + LFl | A very quick light in which a specified group of flashes is regularly repeated. | The number of flashes in a group should be three or nine. An exceptional light character is reserved for use in the IALA Maritime Buoyage System to indicate a south cardinal mark. | A group very quick White light with a group of three flashes, in a period of 5 s, indicates an east cardinal mark. A group very quick White light with a group of nine flashes, in a period of 10 s, indicates a west cardinal mark. A group very quick White light with a group of six flashes followed by a long flash of not less than 2 s duration, in a period of 10 s, indicates a south cardinal mark. |
| 6.3 | Interrupted very quick light | IVQ | A very quick light in which the sequence of flashes is interrupted by regularly repeated eclipses of constant and long duration. | The number of flashes in a period should be at least eight. The duration of the long eclipse should not be less than 3 s. | An interrupted very quick light should not be used. |
| 7 | ULTRA QUICK LIGHT |  | A light in which flashes are repeated at a rate of not less than 160 flashes per minute. | A light in which flashes are repeated at a rate of not less than 240 flashes per minute and not more than 300 flashes per minute. |  |
| 7.1 | Continuous ultra quick light | UQ | An ultra quick light in which a flash is regularly repeated. |  |  |
| 7.2 | Interrupted ultra quick light | IUQ | An ultra quick light in which the sequence of flashes is interrupted by eclipses of long duration. | An ultra quick light in which the sequence of flashes is interrupted by regularly repeated eclipses of constant and long duration. The approximate duration of the sequence of flashes may be specified. number of flashes in a period should be at least 25. The duration of the long eclipse should not be less than 3 s. |  |
| 8 | MORSE CODE LIGHT | Mo(#) eg. Mo(L) | A light in which appearances of light of two clearly different durations are grouped to represent a character or characters in the Morse Code. | Light characters should be restricted to a single letter in the Morse Code in general, and should be two letters only as an exception. The duration of a "dot" should be about 0.5 s, and the duration of a "dash" should not be less than three times the duration of a "dot". | A Morse Code White light with the single character "A" indicates a safe-water mark. A Morse Code Yellow light, but not with either of the single characters "A" or "U"*, indicates a special mark. |
| 9 | FIXED AND FLASHING LIGHT | FFl | A light in which a fixed light is combined with a flashing light of' higher luminous intensity. | This class of light character should be used with care because the fixed component of the light may not be visible at all times over the same distance as the rhythmic component. |  |
| 10 | ALTERNATING LIGHT | AlWR | A light showing different colours alternately. | This class of light character should be used with care, and efforts should be made to ensure that the different colours appear equally visible to an observer. |  |

| APPENDIX RHYTHMIC CHARACTERS OF THE LIGHTS IN THE IALA MARITIME BUOYAGE SYSTEM |  |  |  |
| --- | --- | --- | --- |
|  | Mark | Rhythmic character of the light | Remarks and further recommendations. |
|  | LATERAL | All recommended classes of rhythmic character , but a composite group flashing light with a group of (2 + 1) flashes is solely assigned to modified lateral marks that indicate preferred channels. | Only the colours Red and Green are used. |
|  | Modified lateral (preferred channel) | Composite group flashing light with a group of (2 + 1) flashes, in a period of not more than 16 s | The duration of the eclipse after the single flash should not be less than three times the duration of the eclipse after the group of two flashes. |
|  | CARDINAL |  | Only the colour White is used. |
|  | North cardinal | (a) Continuous very quick light. (b) Continuous quick light. |  |
|  | East cardinal | (a) Group very quick light with a group of three flashes, in a period of 5 s. (b) Group quick light with a group of three flashes, in a period of 10 s. |  |
|  | South cardinal | (a) Group very quick light with a group of six flashes followed by a long flash of not less than 2 s duration, in a period of 10 s. (b) Group quick light with a group of six flashes followed by a long flash of not less than 2 s duration, in a period of 15 s. | The duration of the eclipse immediately preceding a long flash should be equal to the duration of the eclipses between the flashes at the very quick rate. The duration of a long flash should not be greater than the duration of the eclipse immediately following the long flash. The duration of the eclipse immediately preceding a long flash should be equal to the duration of the eclipses between the flashes at the quick rate. The duration of a long flash should not be greater than the duration of the eclipse immediately following the long flash. |
|  | West cardinal | (a) Group very quick light with a group of nine flashes, in a period of 10 s. (b) Group quick light with a group of nine flashes, in a period of 15 s. |  |
|  | ISOLATED DANGER | (a) Group-flashing light with a group of two flashes, in a period of 5 s. (b) Group-flashing light with a group of two flashes, in a period of 10 s. | Only the colour White is used. The duration of a flash together with the duration of the eclipse within the group should be not less than 1 s and not more than 1.5 s. The duration of a flash together with the duration of the eclipse within the group should be not less than 2 s and not more than 3 s. |
|  | SAFE-WATER | (a) Long-flashing light with a period of 10 s. (b) Isophase light. (c) Single-occulting light. (d) Morse Code light with the single character "A". | Only the colour White is used. |
|  | SPECIAL | (a) Group-occulting light. (b) Single-flashing light, but not a long-flashing light with a period of 10 s. (c) Group-flashing light with a group of four, five or (exceptionally) six flashes. (d) Composite group-flashing light. (e) Morse Code light, but not with either of the single characters "A" or “U”`. | Only the colour Yellow is used. A group-flashing light with a group of five flashes at a rate of 30 flashes per minute, in a period of 20 s, is assigned to Ocean Data Acquisition Systems (ODAS) buoys. |

##### Maximum Periods for Light Characters

Refer to IALA publications: - Recommendation for the rhythmic characters of lights on aids to navigation (E110), May 1998; - Recommendation for the calculation of the effective intensity of a rhythmic light, November 1980.

Table 3.3 is an extract of the recommended maximum periods for rhythmic characters of lights.

Table 3.3 Maximum period for rhythmic characters

of aids to navigation lights.

| Character Class | Maximum period (seconds) |
| --- | --- |
| Isophase light | 12 |
| Single-occulting light | 15 |
| Single-flashing light | 15 |
| Group very quick light | 15 |
| Interrupted very quick light | 15 |
| Interrupted ultra quick light | 15 |
| Group-occulting light | 20 |
| Long-flashing light | 20 |
| Group-flashing lights of two flashes | 20 |
| Group-quick light | 20 |
| Interrupted quick light | 20 |
| Group-occulting light of three or more eclipses | 30 |
| Group-flashing light of three or more flashes | 30 |
| Composite group-flashing light | 30 |
| Morse code light | 30 |

#### Timing of Astronomical Events

The description of a lighted Aid to Navigation emphasises the night time operations but daytime role is often as important.  The astronomical events that define the transitions from day to night are shown in Table 3.4..

Table 3.4 Timing of Astronomical Events.

| Event | Condition | Typical Illumination Lux | Comment (Assuming the absence of moonlight, artificial lighting or adverse atmospheric conditions) |
| --- | --- | --- | --- |
| Sunset/Sunrise | Upper edge of the sun’s disc is coincident with the horizon. | 600 |  |
| Civil Twilight (beginning / ending) | Centre of the sun is at a depression angle of six (6) degrees below the horizon. | 6 | Illumination is sufficient for large objects to be seen but no detail is discernible. The brightest stars and planets can be seen For navigation at sea, the sea horizon is clearly defined. |
| Nautical Twilight (beginning / ending) | Centre of the sun is at a depression angle of twelve (12) degrees below the horizon. | 0.06 | It is dark for normal practical purposes. For navigation at sea, the sea horizon is not normally visible. |
| Astronomical Twilight (beginning / ending) | Centre of the sun is at a depression angle of eighteen (18) degrees below the horizon. | 0.0006 | Illumination due to scattered light from the sun is less than that from starlight and other natural light sources in the sky. |

#### Night Operations

Refer to IALA publication: - Recommendation for the notation of luminous intensity and range of lights, November 1966.

##### Nominal Range and Luminous Intensity

Table 3.5 is an extract of the IALA recommendation for the notation of luminous intensity and range of lights and provides a conversion between nominal range and luminous intensity.

Table 3.5 IALA conversion table for Luminous Intensity and Nominal Range for night observations .  This assumes an atmospheric transmissivity of T=0.74 and a threshold of illumination of 0.2 microlux.

| Nominal Range (nautical miles) | Luminous Intensity (candela) | Nominal Range (nautical miles) | Luminous Intensity (candela) |
| --- | --- | --- | --- |
| 1 | 0.9 | 12 | 3600 |
| 1.5 | 2.4 | 13 | 5700 |
| 2 | 5 | 14 | 8900 |
| 2.5 | 9 | 15 | 14000 |
| 3 | 15 | 16 | 21000 |
| 3.5 | 24 | 17 | 32000 |
| 4 | 36 | 18 | 49000 |
| 4.5 | 53 | 19 | 73000 |
| 5 | 77 | 20 | 110000 |
| 6 | 150 | 21 | 160000 |
| 7 | 270 | 22 | 240000 |
| 8 | 480 | 23 | 360000 |
| 9 | 820 | 24 | 520000 |
| 10 | 1400 | 25 | 770000 |
| 11 | 2200 | 26 | 1100000 |

##### Switch-on / switch off times

For lit (lighted) aids to navigation that only operate at night, the switch-on / switch-off times can be regulated by either time switches or photo-sensitive devices that are calibrated to correspond to a nominated light intensity.

IALA is reviewing switch-on/switch-off times with a view to issuing a Recommendation on the topic. (EDITORIAL NOTE: DETAILS OF THE RECOMMENDATION WILL BE AVAILABLE BEFORE THE NAVGUIDE IS RE-ISSUED)

##### Background Lighting

Nominal range at night is calculated with no allowance for glare from background lighting.  Excessive background lighting, from street lights, neon signs etc., frequently makes an aid to navigation light less effective and, in some cases, it becomes completely lost in the general background clutter.

The light can be made more conspicuous by increasing its intensity, changing its colour or by varying its rhythm.

##### Glare

Glare can be caused by bright lights emitted from the shore, such as car headlights, or from another vessel indiscreetly using a search-light.  An aid to navigation light can also cause glare if it is too bright for the shortest viewing distance, especially when the focal plane of the light and the observer's eye are at the same height.  This situation can arise with two station leading lines.

For aids to navigation lights it is generally accepted that the illuminance at the eye of the navigator from the light: - should not exceed 0.1 lux, and; - should be reduced to 0.01 lux if the background is very dark . Refer to IALA publications: - Recommendation for leading lights (E112), May 1998; - Guidelines for the design of leading lines, December 2001.

In situations where glare is a problem, one or more of the following alterations may lead to a satisfactory result:

- raise the focal plane of the light so that the mariner uses the loom of the light or a less intense part of the vertical distribution of the light;

- reduce the intensity of the light by;

  - reducing the illuminance of the light source;

  - reducing the size of the optic;

  - masking the optic with, for example, perforated metal sheet;

- screen unnecessary arcs of the light;

- use two or more lower intensity lights instead of one higher intensity light.

Whatever methods are used, it will be necessary to measure or calculate the intensity and distribution of the modified light or lighting system.

##### Intensity Losses through Glazing

Some lighting equipment has to be installed inside a protective lantern housing.  Unless it is practicable to measure the luminous intensity of the complete installation, it is normal practice to apply a de-rating factor to the intensity of the lighting equipment to allow for the reflection and transmission losses at the lantern glazing.  Generally referred to as the glazing loss factor.

IALA recommends that, in the absence of more definitive information, the glazing loss factor be taken as 0.85 for a system in clean condition.

##### Service Conditions Factor

Under normal operating conditions the luminous intensity of a light is likely to degrade between service (maintenance) intervals.  There are several components to this degradation:

- meteorological conditions (which may only be temporary);

- dirt and salt deposition (which can be minimised by an efficient regular programme of cleaning of the optical system and housing), and;

- progressive deterioration of the light source over the service interval.

It is clearly impossible to represent such a complex array of factors in any simple way, and a proper assessment of the various effects could only be made by measurements on site at regular intervals.  However, in order to give a more realistic figure for the performance of the light under normal operating conditions than when the luminous intensity is measured in a laboratory or on a photometric range, it may be appropriate to apply a service conditions factor to the measured intensity.

IALA recommends that, in the absence of more definitive information, the service condition factor be taken as 0.75.10

#### Day Operations

##### Trends

A number of authorities have established daytime lighted leading  lines in major ports and waterways to achieve a more consistent performance than is possible with dayboards.

##### Nominal Daytime Range and Luminous Intensity

IALA has published a Recommendation for a definition of the nominal daytime range of maritime signal lights intended for the guidance of shipping by day, April 1974.

Table 3.6 is an extract of this  Recommendation and provides a conversion between nominal daytime range and luminous intensity.

Table 3.6 IALA conversion table for Luminous Intensity and Nominal Daytime Range.

| Nominal Daytime Range (nautical miles) | Luminous Intensity (candela) | Nominal Daytime Range (nautical miles) | Luminous Intensity (candela) |
| --- | --- | --- | --- |
| 1 | 4600 | 11 | 11000000 |
| 2 | 25000 | 12 | 18000000 |
| 3 | 75000 | 13 | 28000000 |
| 4 | 182000 | 14 | 45000000 |
| 5 | 383000 | 15 | 69000000 |
| 6 | 745000 | 16 | 105000000 |
| 7 | 1400000 | 17 | 161000000 |
| 8 | 2400000 | 18 | 244000000 |
| 9 | 4100000 | 19 | 367000000 |
| 10 | 6900000 | 20 | 549000000 |

##### Daymarks (Dayboards)

The size of a dayboard should be determined for the maximum useful viewing distance and minimum visibility conditions.  Daymarks used on leading lines are typically rectangular with the long side vertical.  The aspect ratio for the rectangle is commonly 2:1.

The typical operational range of daymarks under different visibility conditions is shown in Table 3.7.

Table 3.7 Typical operational range of daymarks.

| Operational Range of Daymarks (Nautical Miles) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Daymark height (metres). Aspect ratio h=2w |  |  |  |  |
| Minimum visibility (Nautical Miles) | 1.8 | 2.4 | 3.7 | 4.9 | 7.3 |
| 1 | 0.5 | 0.7 | 0.9 | 1.0 | 1.1 |
| 2 | 0.6 | 0.9 | 1.2 | 1.4 | 1.5 |
| 3 | 0.6 | 1.1 | 1.5 | 1.9 | 2.1 |
| 4 | 0.7 | 1.3 | 1.8 | 2.3 | 2.7 |
| 5 | 0.8 | 1.5 | 2.1 | 2.7 | 3.3 |
| 6 | 0.8 | 1.6 | 2.3 | 2.9 | 3.6 |
| 7 | 0.9 | 1.7 | 2.4 | 3.3 | 4.0 |
| 8 | 0.9 | 1.7 | 2.6 | 3.5 | 4.2 |
| 9 | 0.9 | 1.9 | 2.8 | 3.8 | 4.5 |
| 10 | 1.0 | 2.0 | 3.0 | 4.0 | 5.0 |

#### Luminous Range Diagram

The Luminous Range Diagram, shown in Fig. 3.8, enables the mariner to determine the approximate range at which a light may be sighted, by night or by day, in the meteorological conditions prevailing at the time, and for various levels of background lighting or sky luminance, respectively.

This Luminous Range Diagram draws on information contained in:

- IALA Recommendation for the notation of luminous intensity and range of lights, November 1966;

- IALA Recommendation for a definition of nominal daytime range of maritime signal lights Intended for the guidance of shipping by day, April 1974.

Fig 3.8 Luminous Range Diagram The top scale on the graph gives the equivalent intensity values in candelas for both day and night operation, and should be used when only the intensity of a light is known. It must be remembered that : - The atmosphere transmissivity is not necessarily consistent between the observer and the light. - No allowance is made for range limitations imposed by the height-of-eye of the observer or the elevation of the light. A separate check should be made using the geographic range table to confirm that the light will be visible at the estimated luminous range. - Glare from background lighting will reduce considerably the range at which lights are sighted at night.

#### Using the Luminous Range Diagram

Example 1

There are some countries that publish the range of lights to a different meteorological visibility than the 10 nautical miles meteorological visibility used in the definition of nominal range.  If the meteorological visibility value is known, the quoted range of a light can be converted to a nominal range using the graph as follows:

- Locate the quoted range on the left hand axis;

- Run a horizontal line across to the intercept with the curve for the stated meteorological visibility;

- Then move vertically down to the bottom scale to read the equivalent nominal range;

- For example:  A light with a published range of 24 nautical miles with a meteorological visibility of 20 nautical miles has a nominal range of 15 nautical miles.

Example 2

To determine the luminous range of a light after making an estimate of the prevailing meteorological conditions:

- Locate the nominal range on the bottom axis;

- Then move vertically to the intercept with the curve for the estimated meteorological visibility;

- Read off the luminous range on the left hand scale;

- For example:  A light with a nominal range of 9 nautical miles will have a range of 6 nautical miles when the visibility is 5 nautical miles.

Example 3

To determine the luminous range at night when the light is affected by glare from background lighting.

- Estimate the level of background lighting between none and substantial;

- Select the adjustment interval from the right hand auxiliary scale on the graph;

- Subtract this interval from the nominal range of the light;

- Then move vertically to the intercept with the curve for the estimated meteorological visibility;

- Read off the luminous range on the left hand scale;

- For example: If there is substantial background lighting from a port facility or large urban development, an 18 mile light must first be corrected to 8 miles before the luminous range can be determined for the prevailing meteorological conditions.

Example 4

To determine the luminous range of a light intended for day-time use under prevailing day-time luminance conditions.

- Estimate the day sky luminance between glaring cloud and dark overcast. The nominal condition for day-time use is bright cloud or a clear sky (looking away from the sun);

- Select the adjustment interval from the nominal condition from the left hand auxiliary scale on the graph;

- Add or Subtract this interval from the nominal range of the light;

- Then move vertically to the intercept with the curve for the estimated meteorological visibility;

- Read off the luminous range on the left hand scale;

- For example: When a light with a 5 nautical mile nominal range by day is viewed against an overcast sky, the nominal range can be increased to around 8.8 nautical miles before determining the luminous range for the prevailing meteorological conditions;

- If a light with a 5 nautical mile nominal range by day is viewed against very bright glaring cloud the nominal range would be reduced to 3 nautical miles before determining the luminous range for the prevailing meteorological conditions.

Example 5

The graph can also be used to show:

- Using the top and bottom scales that a light with an intensity of 100,000 candelas has a nominal range of a little over 20 nautical miles;

- If the same light was sighted at 12 nautical miles it would imply that the meteorological visibility was about 5 nautical miles;

- The same light would have a nominal day-time range of a little over 3 nautical miles.

    - Fixed Aids to Navigation

#### Description

The IALA  International Dictionary of Aids to Marine Navigation defines a beacon as “a fixed artificial navigation mark” that can be recognised by its shape, colour, pattern, topmark or light character, or a combination of these. While this functional definition includes lighthouses and other fixed aids to navigation, the terms lighthouse and beacon are used more specifically to indicate importance and size.

#### Lighthouses

A lighthouse is generally considered to be:

- A large conspicuous structure (visual mark) on land, close to the shoreline or in the water;

  - that acts as a daymark, and;

  - provides a platform generally for higher range marine signalling lights.

  - other aids to navigation or audible signals on or near the lighthouse;

- A manned or automated facility.

  - the former is becoming less common;

  - an automated lighthouse will often be remotely monitored and in some cases remotely controlled.

#### Beacons

A beacon is usually considered to be a small fixed visual mark on land or in the water.  Visual characteristics are often defined by daymarks, topmarks, and by numbers.  A marine signalling light, if fitted, would generally  be of a lower range than lighthouses.

In navigable channels a pile beacon may be used as an alternative to a buoy.

#### Purpose of Lighthouses and Beacons

A lighthouse or beacon may perform one or more of the following navigational functions:

- mark a landfall position;

- mark an obstruction or a danger;

- indicate the lateral limits of a channel or navigable waterway;

- indicate a turning point or a junction in a waterway;

- mark the entrance of a Traffic Separation Scheme (TSS);

- form part of a leading (range) line;

- mark an area;

- provide a reference for mariners to take a bearing or line of position (LOP).

However it is not uncommon for lighthouses, in particular, to be used for other purposes that can include:

- base for AIS equipment;

- coastwatch or coastguard functions;

- VTS functions;

- base for audible (fog) signals;

- collection of meteorological and oceanographic data;

- radio and telecommunication facilities;

- tourist facilities.

#### Performance Criteria for Lighthouses and Beacons

The availability of a light is the principal measure of performance determined by IALA.  For lighthouses and beacons IALA recommends the following availability targets:

| Type of Aid | Availability Target |  |
| --- | --- | --- |
| Lighthouses and beacons considered to be of primary navigational significance | Category 1 | at least 99.8% |
| Lighthouses and beacons considered to be of navigational significance | Category 2 | at least 99% |
| Lighthouses and beacons considered to have less navigational significance than either Categories 1 or 2 | Category 3 | at least 97% |
| Daymarks and Topmarks (where fitted) |  | at least 97% |

Note

- The importance of a visual aid to navigation may well change over time.

    - Floating Aids to Navigation

#### Description

A floating aid to navigation serves a similar purpose to a beacon or lighthouse.  However the floating aid to navigation is normally associated with locations where:

- it would be impractical due to water depth, seabed conditions or cost to establish a fixed aid;

- the hazard shifts over time (eg. sand banks, an unstable wreck, etc.);

- where the aid is at high risk of damage or loss from ice flows or ship impacts and as a consequence is treated as expendable;

- a temporary mark is required.

##### Buoys

Buoys are defined as minor floating aids and whilst it is normal that they are lit there are instances where no light is installed.

These types of aids to navigation:

- are specifically covered by the IALA Maritime Buoyage System;

- tend to have circular hull forms in range of 1 to 3 m diameter;

- where lights are exhibited they are usually solar or single cycle battery powered, however gas buoys are still in operation;

- due to limited power on the buoy light ranges are restricted, to typically 2 to 5 nautical miles although much higher ranges are used in some applications;

- due to limited power on the buoy additional services are restricted but Racon and AIS units are sometimes deployed in additional to a light.

##### Lightvesels, Lightfloats and Large Navigational Buoys (LANBYS)

Lightvessels, lightfloats and lanbys (or LNB) are defined as major floating aids and may carry a racon, sound signal, and in some cases, a radio beacon in addition to the aid to navigation light. A lightvessel may also display a white riding light to signify a vessel at anchor.

These types of aids to navigation:

- generally have high operating costs;

- are only deployed at critical locations;

- are often assigned an availability target that is higher than for a buoy;

- are not specifically covered by the IALA Maritime Buoyage System.

Some lightvessels continue to be manned, but the trend is towards making them operate automatically, often with remote monitoring and control.

Refer also to IALA Recommendation for off-station signals for major floating aids (O104), November 1989.

#### IALA Maritime Buoyage System (MBS)

The Maritime Buoyage System represents one of IALA’s major contributions to enhancing the safety of navigation.  As recently as 1976 there were more than thirty buoyage systems in use world wide and conflicting sets of rules applied.  In 1980 Lighthouse Authorities from fifty countries and representatives from nine international organisations reached agreement on the rules for a single system.

##### General Principles and Rules of the MBS System

The content of the General Principles and Rules of the IALA Maritime Buoyage System can be found at Annex A.

#### Performance Criteria for Floating Aids

The availability of a floating aid is the principal measure of performance determined by IALA.  The recommended availability targets are as follows:

| Type of Aid (examples only) | Availability Target |  |
| --- | --- | --- |
| Floating aids to navigation that are considered to be of primary navigational significance | Category 1 | at least 99.8% |
| Floating aids to navigation that are considered to be of navigational significance | Category 2 | at least 99% |
| Floating aids to navigation that are considered to be of less navigational significance than Category 1 or 2. | Category 3 | at least 97% |

Note

The availability objective assigned to floating aids to navigation conforming to the IALA Maritime Buoyage System should also apply to the topmark.

#### Technical Considerations for Floating Aids to Navigation

##### Cost:

The cost of establishing a floating aid at a given location will generally be less than for a fixed structure.  The cost difference increases with increasing water depth and exposure to wind and wave loads.

In contrast, the maintenance cost of floating aids to navigation tends to be high relative to the capital value.  This has caused many authorities to critically examine the potential for savings through design changes, use of alternative materials and amending maintenance practices, generally with the aim of extending maintenance intervals.

Where an authority operates large numbers of floating aids, it may become practicable to operate a dedicated buoy tender vessel with specialised equipment to minimise buoy change-out times and to improve occupational safety.

##### Floating Aid Design

The process of designing a buoy to meet specific requirements is a specialised task.  It involves, but is not limited to:

- defining the operational performance characteristics;

- defining the equipment, power requirements and power source(s);

- defining the type and capabilities of the vessels that will be used to service the buoy;

- selecting the initial type proportions and mooring for the buoy  (See section 3.6.4.3);

- integration of equipment and power supply;

- consideration of the maintenance requirements;

  - deployment and recovery techniques;

  - protecting equipment from damage;

  - ability to rectify faults without having to lift the buoy;

- determining the buoy response to the wave, wind and current conditions at the site(s);

- design optimisation.

Refer also to: - IALA Maritime Buoyage System; - IALA Maritime Buoyage System Guidelines; - IALA Guidelines on Plastic Buoys; - Report on the IALA Workshop on Large Navigation Buoys and Automatic Lightvessels Floataid 84.

##### Mooring Design

###### General

The mooring system for a floating aid to navigation is the sum of the components that keep the aid within a nominated area.  These components have to withstand the forces of wind, wave and current on the floating aid and drag on the mooring line.  Methods for determining the forces are covered in the IALA Recommendation E107.  The basic assumptions made are:

- the chain is tangential to the sea bed under all conditions of current and wind at the site;

- the buoy axis is vertical under the most common conditions of current and wind;

- the ratio of the breaking stress of the chain to the calculated stress is not less than 5 under the most unfavourable conditions of current and wind;

- the reserve buoyancy of the fully equipped floating aid is greater than the combined loads of current and wind under the most unfavourable conditions.

###### Swing Radius

The IALA Recommendation on the design of normal moorings (E-107), May 1998 indicates that the maximum swing radius (watch circle radius) of a floating aid is:

rm  = L2 − H2

where:

| rm = | Maximum swing radius (m) |
| --- | --- |
| L = | Length of mooring line (m) |
| H= | Depth of water (m) (This is defined as the maximum depth of water and includes the highest tide level and half the maximum wave height at the particular site.) |

The recommended minimum length of mooring line is:

- Lmin = 2H for depths less than 50 metres;

- Lmin = 1.5H for depths greater than 50 metres;

#### IALA References on Floating Aids AND Moorings

##### IALA Publications

- IALA Recommendation on the design of normal moorings (E-107), May 1998; - IALA Practical Notes on the use of mooring chains for floating aids to navigation, June 1989.

##### Conference References

- Development of a 5 year buoy, Canadian Coast Guard, IALA Conference 1998;

- Computer aided buoy and mooring design, US Coast Guard, IALA Conference 1994;

- A large solar buoy to replace light vessels and lanby buoys, Trinity House UK, IALA Conference 1994;

- Conversion of acetylene powered light float to solar operation, Trinity House UK, IALA Conference 1994;

- Evaluation of the visibility of buoys and topmarks, US Coast Guard, IALA Conference 1994;

- Environmental aspects relating to buoys, Seezeichenversuchsfeld, Germany, IALA Conference 1994.

#### Positioning of Floating Aids

The charted position of a floating aid defines the nominal (or true) position for the anchor.

With most floating aids there is potential for the mooring anchor to be moved off-station during storms or for positional errors to occur while laying the anchors.

Anchors have traditionally been laid while taking cross bearings and/or horizontal sextant angles from fixed visual marks.  When out of sight of land the process may have relied on radionavigation or radio-positioning aids.  While some authorities may still use these procedures, the use of DGPS position fixing is increasingly seen as the preferred method because of its convenience, accuracy and repeatability.

A buoy tender using DGPS can generally be brought to within 10 metres of the nominal buoy position at the time of releasing the anchor.  If the anchor is allowed to free-fall, its final resting position will depend on the prevailing current, water depth, shape of the anchor and the nature of the seabed.  Controlling the decent of the anchor may serve to improve the positional accuracy of the buoy.

#### Markings and Topmarks

##### Markings

Floating aids to navigation are often identified by names, abbreviations of names, letters and/or numbers.  Authorities should ensure that the actual marking is identical to the List of Lights reference and the charted marking.

##### Topmarks

The type, colour and arrangement of topmarks on a buoy are shown in the IALA Maritime Buoyage System, extracts of which are shown in Annex A.

###### Dimensions of Topmarks

- Conical topmarks (for lateral and cardinal marks):

  - The vertical height of a cone from base to apex should be about 90% of the base diameter;

  - For cardinal marks, the separation distance between cones should be about 50% of the base diameter of the cone;

  - The vertical clear space between the lowest point of the topmark and all other parts of the mark should be at least 35% of the base diameter of the cone;

  - In the case of a buoy, the base diameter should be 25%-30% of the diameter of the buoy at waterline.

- Cylindrical (CAN) topmarks (for lateral marks):

  - The vertical height of a cylinder should be 1 to 1.5 times the base diameter;

  - The vertical clear space between the lowest part of the cylinder and all other parts of the mark should be at least 35% of the diameter of the cylinder;

  - In the case of a buoy, the base diameter of the cylinder should be 25%-30% of the diameter of the buoy at the waterline.

- Spherical topmarks (for isolated danger and safe water marks):

  - In the case of a buoy, the diameter of the sphere(s) should be at least 20% of the diameter of the buoy at the waterline;

  - For isolated danger marks the separation distance between spheres should be about 50% of their diameter;

  - The vertical space between the lowest part of the sphere(s) and all other parts of the mark should be at least 35% of the diameter of the sphere(s).

- 'X' (Diagonal Cross) topmarks (for special marks):

  - In the case of a buoy, the arms of the 'X' should be diagonally contained within a square with length of side of about 33% of the buoy diameter at the waterline. The width of the arms of the 'X' should be about 15% of the length of side of the square.

    - Sector Lights and Leading (Range) Lines

Note: Bearings, directions of leading (range) lines and limits of sectors should always be stated in terms of the bearings that would be seen by the mariner. Bearings may carry a suffix ‘TBS” or True Bearing from Seaward as confirmation.

#### Sector Lights

A sector light is an aid to navigation that displays different colours and/or rhythms over designated arcs.

A Precision Direction Light  (PDL)is a specialised form of sector light that can generate sharply defined sector boundaries.  This feature is particularly useful for applications that require one or several narrow sectors. The PDL may use a white light source with coloured filter, but newer designs are utilising LED or laser lights.

A common means of creating a sector is to fit a coloured filter in front of the main light.  A sector can also be produced by a subsidiary light on the same structure.  The subsidiary light can take any of the following forms:

- Range (directional) light;

- Beacon with a coloured lens, masked to achieve the sector angle;

- Beacon fitted with internal or external filter panels;

- Precision Direction Light.

The limits or boundaries of a sector are not always precisely cut off due to the characteristics of the light source and fading of colours or changing rhythms between adjacent sectors.

For a beacon fitted with coloured filter panels, the reason for the lack of a precise transition at the sector boundary is readily apparent from Fig 3.10 which shows the light source, lens and filter geometry.  The transition zone is defined by an "angle of uncertainty" (Refer IALA Dictionary 2-6-305).

Fig 3.10 Angle of Uncertainty

It can also be noted that:

- the observed angle of uncertainty is generally less than the geometric angle due to the relative intensities of sector colours (ie. colour mixing) as the observer passes through the transition zone;

- if space on the aid to navigation structure is not a limiting factor, it is usually possible to achieve an angle of uncertainty of around 0.25 with this type of sector arrangement;

- the angle of uncertainty can be reduced by decreasing the physical size of the light source or by increasing the radial distance to the coloured filter;

- in situations where the main light has a large projected area, such as a rotating lens or reflector array, it is generally preferable to use a separate sector light rather than installing a coloured filter in front of the main light.

From time to time specialised sector lights have been developed to exhibit different rhythms over different sector bearings.  This capability is found in some Precision Direction Lights.

PDL Sector Lights are so precise that a complete colour change at a sector boundary occurs over an angle of less than 1 minute of arc in most models.

##### Applications

The design of sector lights can be a complex task.  The process should be carried out with reference to a good quality chart of the area.  In some cases good local knowledge is also required.

A sector light may indicate one or more of the following:

- boundaries of a navigable waterway;

- change of course position;

- shoals, banks, etc.;

- an area or position (eg. an anchorage);

- the deepest part of a waterway;

- position checks for floating aids.

A Precision Direction Light (PDL) allows for further applications that include the ability to:

- produce narrow sectors with an angles of uncertainty down to around one minute of arc;

- define the central zone of a channel;

- accurately mark one side of a straight channel.  (A pair of PDLs can cover the permutations of converging, diverging and parallel channels);

- define different rhythms over adjacent sectors.

##### Examples

Some examples of sector lights applications are illustrated in Figs 3.11 and 3.12.

Figure 3.11

This illustration follows the IALA Maritime Buoyage System colour convention for Region A (‘red to port when approaching the aid from seaward’).  The white sector should, if possible, be wide enough to provide a margin of safety for a vessel that inadvertently leaves the white sector. Curves C and D indicate depth contours or limiting dangers that dictate the boundaries of sectors.

Figure 3.12  Shows several applications for sector lights.

The function of each light is described below:

Light 1 is a coastal white light with a red sector indicating a danger.

Light 2 is a sector light obscured over the shore, with two white sectors indicating a safe channel.  When sailing towards the sector light it shows red to port and green to starboard following the IALA Maritime Buoyage System colour convention for Region A.  The boundary between the red and the green sector also indicates the position of a buoy.

Light 3 is a sector light with a red light and 4 white sectors indicating four anchorage positions.  It is obscured over the shore.

Light 4 is a sector light with a white sector indicating a safe channel.

##### Design Considerations for Sector Lights

Where a single sector light defines a navigable channel:

- There is no reference of the vessel’s lateral position within the channel until a sector boundary is reached.  This may cause a problem in channels subject to a strong cross current.  For vessels with local knowledge, the zones defined by the angle of uncertainty can sometimes provide a useful guide to the vessel’s proximity to a sector boundary;

- Where practicable, there should be a margin of safety between the sector boundary and adjacent hazards.  If an appropriate safety margin cannot be achieved within the sector boundary, the hazards could be marked separately.

- Zones defined by the angle of uncertainty should be considered an additional margin of safety over the actual sector boundary;

- The design process for a sector light needs to consider the draught and manoeuvrability of the largest vessels likely to utilise the sector, how quickly they can respond once they cross a sector boundary and the situations that may develop when other vessels are in the vicinity;

- A sector design should take account of the spectral distribution of the light source and the proportion of this light transmitted through the filter material.  The process should also check for potential for glare problems;

- The period of the light flash should be selected to provide ample time for a mariner to recognise the transitional phases that occur at the sector boundary;

- A white light is normally the first preference for a lighthouse or beacon.  If a single coloured sector is added, the preferred colour for the sector is red.

- If a white sector light is used to mark a navigation channel, coloured sectors may be used either side of the white to indicate the lateral limits.  In such cases it is common practice to use red and green sectors that follow the convention of the IALA Maritime Buoyage System;

- Multiple sectors can be used to provide a better indication of a vessel’s lateral position within the channel but at the expense of complexity for both the system designer and navigators.

Refer to IALA publication: - Guideline on sector lights, to be issued in 2004/2005.

#### Leading (Range) Lines

A leading line is an aid to navigation system that comprises two separated structures with marks or lights that are aligned when viewed from the centreline or deepest route along a straight section of channel.

In a two station leading line, the structures lie along an extension of the centreline of the nominated channel.  The rear structure must have a greater elevation than the front structure to enable both marks or lights to be viewed simultaneously.

A leading line provides a vessel with a heading reference and a visual indication of the size and direction of any cross track error.

##### Purposes of Leading Lines

A leading line may be used to:

- indicate the centreline of a straight section of a navigable channel;

- indicate to deep draught vessels the deepest part of the waterway;

- indicate the navigable channel where fixed and floating aids to navigation are not available or do not satisfy the accuracy requirements for safe navigation;

- define a safe approach bearing to a harbour or river entrance, particularly where there are cross currents;

- separate two-way traffic (ie. when passing a bridge)

##### Design Considerations for Leading Lines

A well-designed leading line will enable vessels of the type and size that commonly use the channel to :

- Identify the marks or lights when the ship is at the inner and outer sections of the channel and readily detect cross track position errors from the centreline of the channel;

- Detect cross track position errors with sufficient sensitivity that the channel can be utilised without abrupt changes to the vessels heading and speed;

- Observe both lights together, by selection of leading light character rhythms that appropriately overlap in their free running condition. In some situations it may be preferable to provide additional equipment to synchronise the light characters.

- Observe the lights in all ambient conditions for which they are designed to be used without glare, if lights are to be used for both day and night operations light intensities will need to be varied.

- The characters of rhythmic leading lights should be selected so that the front and rear lights, in their free running states, can generally be observed together.  In some situations it may be preferable to provide additional equipment to synchronise the light characters;

- If lights are to be used both day and night, the light intensities should be adapted for each situation to avoid glare at night.

Leading Lines (or lights) should conform to the following IALA Recommendations: - Recommendation for leading lights (E112), May 1998; - Guidelines for the design of leading lines, December 2001; - Recommendation for a definition of nominal daytime range of maritime signal lights intended for guidance of shipping by day, April 1974.

    - Transits

A transit is defined in the IALA Dictionary (2-6-015) as the alignment of two or more marks

A Leading(or Range) light is a specialised application of a transit.

A simple transit can be used to:

- Provide a turning reference;

- Define a clearing line for the limits of safe navigation;

- Provide a distance mark along a waterway.

ANNEX A – IALA MARITIME BUOYAGE SYSTEM

Within the IALA Buoyage System there are 5 types of marks that may be used in combination.  The mariner can easily distinguish between these marks by readily identifiable characteristics.

Lateral marks differ between Buoyage Regions A and B, as described below, whereas the other four (4) types of marks are common to both regions.

LATERAL MARKS

Following the sense of a “conventional direction of buoyage”, Lateral marks in Region A utilize red and green colours by day and night to denote the port and starboard sides of channels respectively.  However, in Region B these colours are reversed with red to starboard and green to port.

A modified Lateral mark may be used at the point where a channel divides to distinguish the preferred channel, that is to say the primary route or channel that is so designated by an Authority.

CARDINAL MARKS

Cardinal marks indicate that the deepest water in the area lies to the named side of the mark.  This convention is necessary even though for example, a North mark may have navigable water not only to the North but also East and West of it.  The mariner will know he is safe to the North, but must consult his chart for further guidance.

Cardinal marks do not have a distinctive shape but are normally pillar or spar.  They are always painted in yellow and black horizontal bands and their distinctive double cone topmarks are always black.

An aide-memoire to their colouring is provided by regarding the topmarks as pointers to the positions of the black band(s):

- Topmarks pointing upward: black band above yellow band

- Topmarks pointing downward: black band below yellow band

- Topmarks pointing away from each other: black bands above and below a yellow band

- Topmarks pointing towards each other: black band with yellow bands above and below.

Cardinal marks also have a special system of flashing white lights.  The rhythms are basically all “very quick” (VQ) or “quick” (Q) flashing but broken into varying lengths of the flashing phase.

- “very quick flashing” is defined as a light flashing at a rate of either 120 or 100 flashes per minute,

- “quick flashing” is a light flashing at either 60 or 50 flashes per minute.

The characters used for Cardinal marks will be seen to be as follows:

- North: Continuous very quick flashing or quick flashing;

- East: Three “very quick” or “quick” flashes followed by darkness;

- South: Six “very quick” or “quick” flashes followed immediately by a long flash, then darkness;

- West: Nine “very quick” or “quick” flashes followed by darkness.

The concept of three, six, nine is easily remembered when one associates it with a clock face.  The long flash, defined as a light appearance of not less than 2 seconds, is merely a device to ensure that three or nine “very quick” or “quick” flashes cannot be mistaken for six.

It will be observed that two other marks use white lights.  Each has a distinctive light rhythm that cannot be confused with the very quick or quick flashing light of the Cardinal marks.

ISOLATED DANGER MARK

The Isolated Danger mark is placed on a danger of small area that has navigable water all around it.  Distinctive double black spherical topmarks and Group flashing (2) white lights, serve to associate Isolated Danger marks with Cardinal marks.

SAFE WATER MARKS

The Safe Water mark has navigable water all around it, but does not mark a danger.  Safe Water marks can be used, for example, as mid-channel or landfall marks.

Safe Water marks have an appearance quite different from danger marking buoys. They are spherical, or alternatively pillar or spar with a single red spherical topmark.  They are the only type of mark to have vertical stripes (red and white).  Their lights, if any, are white using isophase, occulting, one long flash or Morse “A” rhythms.

SPECIAL MARKS

Special marks are not primarily intended to assist navigation but are used to indicate a special area or feature whose nature may be apparent from reference to a chart or other nautical document.

Special marks are yellow.  They may carry a yellow “X” topmark, and any light used is also yellow.  To avoid the possibility of confusion between yellow and white in poor visibility, the yellow lights of Special marks do not have any of the rhythms used for white lights.  Their shape will not conflict with that of navigational marks, this means, for example, that a special buoy located on the port hand side of a channel may be cylindrical, but will not be conical.  Special marks may also be lettered or numbered to indicate their purpose.

NEW DANGERS

It should be specially noted that a “new danger” which is one not yet shown in nautical documents may be indicated by exactly duplicating the normal mark until the information is sufficiently promulgated.

A “new danger” mark may carry a Racon coded Morse “D”.

RULES OF THE MARITIME BUOYAGE SYSTEM (MBS)

- General

  - Scope

The Maritime Buoyage System provides rules that apply to all fixed and floating marks (other than lighthouses, sector lights, leading lights and marks, lightships and large navigational buoys) serving to indicate:

    - The lateral limits of navigable channels.

    - Natural dangers and other obstructions such as wrecks.

    - Other areas or features of importance to the mariner.

    - New dangers.

  - Types of marks

The Maritime Buoyage System provides five types of marks that may be used in combination:

    - Lateral marks, used in conjunction with a “conventional direction of buoyage”, generally used for well defined channels.  These marks indicate the port and starboard sides of the route to be followed.  Where a channel divides, a modified lateral mark may be used to indicate the preferred route.  Lateral marks differ between Buoyage Regions A and B as described in MBS Sections 2 and 8.

    - Cardinal marks, used in conjunction with the mariner's compass, to indicate where the mariner may find navigable water.

    - Isolated Danger marks to indicate isolated dangers of limited size that have navigable water all around them.

    - Safe Water marks to indicate that there is navigable water all around their position, e.g. mid-channel marks.

    - Special marks, not primarily intended to assist navigation but to indicate an area or feature referred to in nautical documents.

  - Method of characterising marks

The significance of the mark depends upon one or more of the following features:

    - By night, colour and rhythm of light.

    - By day, colour, shape, topmark.

- LATERAL MARKS

  - Definition of “conventional direction of buoyage”

The “conventional direction of buoyage”, which must be indicated in appropriate nautical documents, may be either:

    - The general direction taken by the mariner when approaching a harbour, river, estuary or other waterway from seaward, or

    - The direction determined by the proper authority in consultation, where appropriate, with neighbouring countries.  In principle it should follow a clockwise direction around land masses.

  - Buoyage Regions

There are two international Buoyage Regions A and B where lateral marks differ. These buoyage regions are indicated in Section 8.

  - Description of Lateral Marks used in Region A

|  | 2.3.1. Port hand Marks | 2.3.2. Starboard hand Marks |
| --- | --- | --- |
| Colour | Red | Green |
| Shape (Buoys) | Cylindrical (can), pillar or spar | Conical, pillar or spar |
| Topmark (if any) | Single red cylinder (can) | Single green cone, point upward |
| Light (when fitted) |  |  |
| Colour | Red | Green |
| Rhythm | Any, other than that described in section 2.3.3. | Any, other than that described in section 2.3.3. |

    - At the point where a channel divides, when proceeding in the “conventional direction of buoyage”, a preferred channel may be indicated by a modified Port or Starboard lateral mark as follows:

|  | 2.3.3.1. Preferred channel to Starboard | 2.3.3.2. Preferred channel to Port |
| --- | --- | --- |
| Colour | Red with one broad green horizontal | Green with one broad red horizontal |
| Shape (Buoys) | Cylindrical (can), pillar or spar | Conical, pillar or spar |
| Topmark (if any) | Single red cylinder (can) | Single green cone, point upward |
| Light (when fitted) |  |  |
| Colour | Red | Green |
| Rhythm | Composite group flashing (2 + 1) | Composite group flashing (2 + 1) |

  - Description of Lateral Marks used in Region B

|  | 2.4.1. Port hand Marks | 2.4.2. Starboard hand Marks |
| --- | --- | --- |
| Colour | Green | Red |
| Shape (Buoys) | Cylindrical (can), pillar or spar | Conical, pillar or spar |
| Topmark (if any) | Single green cylinder (can) | Single red cone, point upward |
| Light (when fitted) |  |  |
| Colour | Green | Red |
| Rhythm | Any, other than that described in section 2.4.3. | Any, other than that described in section 2.4.3. |

    - At the point where a channel divides, when proceeding in the “conventional direction of buoyage”, a preferred channel may be indicated by a modified Port or Starboard lateral mark as follows:

|  | 2.4.3.1. Preferred channel to Starboard | 2.4.3.2. Preferred channel to Port |
| --- | --- | --- |
| Colour | Green with one broad red horizontal | Red with one broad green horizontal |
| Shape (Buoys) | Cylindrical (can), pillar or spar | Conical, pillar or spar |
| Topmark (if any) | Single green cylinder (can) | Single red cone, point upward |
| Light (when fitted) |  |  |
| Colour | Green | Red |
| Rhythm | Composite group flashing (2 + 1) | Composite group flashing (2 + 1) |

  - General Rules for Lateral Marks

    - Shapes

Where lateral marks do not rely upon cylindrical (can) or conical buoy shapes for identification they should, where practicable, carry the appropriate topmark.

    - Numbering or lettering

If marks at the sides of a channel are numbered or lettered, the numbering or lettering shall follow the “conventional direction of buoyage”.

- CARDINAL MARKS

  - Definition of Cardinal quadrants and marks

    - The four quadrants (North, East, South and West) are bounded by the true bearings NW-NE, NE-SE, SE-SW, SW-NW, taken from the point of interest.

    - A Cardinal mark is named after the quadrant in which it is placed.

    - The name of a Cardinal mark indicates that it should be passed to the named side of the mark.

  - Use of Cardinal Marks

A Cardinal mark may be used, for example:

    - To indicate that the deepest water in that area is on the named side of the mark.

    - To indicate the safe side on which to pass a danger.

    - To draw attention to a feature in a channel such as a bend, a junction, a bifurcation or the end of a shoal.

  - Description of Cardinal Marks

|  | 3.3.1. North Cardinal Mark | 3.3.2. East Cardinal Mark |
| --- | --- | --- |
| Topmark (a) | 2 black cones, one above the other, points upward | 2 black cones, one above the other, base to base |
| Colour | Black above yellow | Black with a single broad horizontal yellow band |
| Shape (Buoys) | Pillar or spar | Pillar or spar |
| Light (when fitted) |  |  |
| Colour | White | White |
| Rhythm | VQ or Q | VQ(3) every 5s or Q(3) every 10s |

|  | 3.3.3. South Cardinal Mark | 3.3.4. West Cardinal Mark |
| --- | --- | --- |
| Topmark (a) | 2 black cones, one above the other, points downward | 2 black cones, one above the other, point to point |
| Colour | Yellow above black | Yellow with a single broad horizontal black band |
| Shape (Buoys) | Pillar or spar | Pillar or spar |
| Light (when fitted) |  |  |
| Colour | White | White |
| Rhythm | VQ(6) + Long flash every 10s or Q(6) + Long flash every 15s | VQ(9) every 10s or Q(9) every 15s |

Note: (a) The double cone topmark is a very important feature of every Cardinal mark by day, and should be used wherever practicable and be as large as possible with a clear separation between the cones.

- ISOLATED DANGER MARKS

  - Definition of Isolated Danger Marks

An Isolated Danger mark is a mark erected on, or moored on or above, an isolated danger that has navigable water all around it.

  - Description of Isolated Danger Marks

| Topmark (b) | 2 black spheres, one above the other |
| --- | --- |
| Colour | Black with one or more broad horizontal red bands |
| Shape (Buoys) | Optional, but not conflicting with lateral marks; pillar or spar preferred |
| Light (when fitted) |  |
| Colour | White |
| Rhythm | Group flashing (2) |

Note: (b) The double sphere topmark is a very important feature of every Isolated Danger mark by day, and should be used wherever practicable and be as large as possible with a clear separation between the spheres.

- SAFE WATER MARKS

  - Definition of Safe Water Marks

Safe Water marks serve to indicate that there is navigable water all round the mark; these include centre line marks and mid-channel marks. Such a mark may also be used as an alternative to a Cardinal or a Lateral mark to indicate a landfall.

  - Description of Safe Water Marks

| Colour | Red and white vertical stripes |
| --- | --- |
| Shape (Buoys) | Spherical; pillar or spar with spherical topmark |
| Topmark (if any) | Single red sphere |
| Light (when fitted) |  |
| Colour | White |
| Rhythm | Isophase, occulting, one long flash every 10s or Morse “A” |

- SPECIAL MARKS

  - Definition of Special Marks

Marks not primarily intended to assist navigation but which indicate a special area or feature referred to in appropriate nautical documents, for example:

    - Ocean Data Acquisition Systems (ODAS) marks.

    - Traffic separation marks where use of conventional channel marking may cause confusion.

    - Spoil Ground marks.

    - Military exercise zone marks.

    - Cable or pipeline marks.

    - Recreation zone marks.

  - Description of Special Marks

| Colour | Yellow |
| --- | --- |
| Shape (Buoys) | Optional, but not conflicting with navigational marks |
| Topmark (if any) | Single yellow “X” shape |
| Light (when fitted) |  |
| Colour | Yellow |
| Rhythm | Any, other than those described in sections 3, 4 or 5 |

  - Additional Special Marks

Special marks other than those listed in paragraph 6.1 and described in paragraph 6.2 may be established by the responsible administration to meet exceptional circumstances. These additional marks shall not conflict with navigational marks and shall be promulgated in appropriate nautical documents and the International Association of Lighthouse Authorities notified as soon as practicable.

- NEW DANGERS

  - Definition of New Dangers

The term “New Danger” is used to describe newly discovered hazards not yet indicated in nautical documents. “New Dangers” include naturally occurring obstructions such as sandbanks or rocks or man made dangers such as wrecks.

  - Marking of New Dangers

    - “New Dangers” shall be marked in accordance with these rules. If the appropriate Authority considers the danger to be especially grave at least one of the marks shall be duplicated as soon as practicable.

    - Any lighted mark used for this purpose shall have an appropriate Cardinal or Lateral VQ or Q light character.

    - Any duplicate mark shall be identical to its partner in all respects.

    - A “New Danger” may be marked by a racon, coded Morse “D” showing a signal length of 1 nautical mile on the radar display.

    - The duplicate mark may be removed when the appropriate Authority is satisfied that information concerning the “New Danger” has been sufficiently promulgated.

- INTERNATIONAL BUOYAGE REGIONS A AND B

There are two international Buoyage Regions A and B where Lateral marks differ as described in Section 2. The current geographical divisions of these two Regions are shown on the following world map.

## Radio Aids to Navigation

Radio aids to navigation are generally regarded as the “new technologies” of marine navigation.  When compared with visual aids, radio aids typically have a greater coverage area, and can be more cost effective if sufficient numbers of vessels carry the appropriate receivers or equipment.  However specific radio aids to navigation systems have typically become obsolete within a relatively short space of time.  Some recent examples include Decca, Omega, Rana and Toran

Radio aids to navigation systems can be categorised into two main groups:

- Positioning systems: - such as Global Navigation Satellite Systems (GNSS), Differential Global Navigation Satellite Systems (DGNSS), LORAN-C/CHAYKA

- Information services: - that reduce the risk of collisions and groundings and thereby contribute to the protection of the marine environment.  These take the form of Vessel Traffic Services (VTS), Ship Reporting Systems (SRS) and Automatic Identification Systems (AIS).

Of all the current radionavigation technologies, the availability of Global Navigation Satellite Systems (GNSS) for civilian usage has brought fundamental changes to the practice of marine navigation.  These include:

- an alternative to traditional position fixing methods when navigating outside restricted waters ;

- opening the way to the development of Electronic Chart Systems (ECS) and Automatic Identification Systems (AIS) for ships, and;

- the opportunity for systems such as ECDIS that merge the positioning reference and information functions;

- multi-modal concepts where aspects of marine navigation merge with the management and tracking of cargo from source to destination.

DGNSS, Electronic Chart Systems and AIS are being used throughout the world.    National authorities should take this into consideration when determining the appropriate mix of aids to navigation for their jurisdiction.

### Policy on Radio Aids to Navigation

#### Role of IALA

IALA has played and is continuing to play an important role in the development of maritime radionavigation throughout the world.

Over the past few years many significant changes have taken place including the following:

- Provision of new and improved radionavigation systems with more (improvements of) systems under development or consideration;

- Recognition by IMO of GPS and GLONASS as components of the present World Wide Radionavigation System;

- Adoption by IMO of the maritime requirements for future radionavigation systems (A.915(22));

- Adoption by IMO of a revised SOLAS Chapter V with relevant change for radionavigation, in particular introduction of carriage requirements for electronic radionavigation receivers, AIS and VDR;

- Adoption of new and amended Performance Standards relevant for radionavigation, in particular for radionavigation receivers, AIS and ECDIS.

#### Scope of the policy

The Policy includes, but is not limited to:

- Satellite systems: - Space-based means which can be used to derive a position fix, such as GPS and GLONASS;

- Terrestrial systems: - Ground-based means which can be used to derive a position fix, such as LORAN-C;

- Augmentation systems: - Space and/or ground-based supplementary means which can be used to improve the position fix, such as WAAS/EGNOS, DGNSS and RAIM receivers;

- Racons: - Aids to navigation based means to derive the identity and position of the aids to navigation;

- Hybrid/Integrated systems: - Any combination of the above mentioned means which can be used to improve the position fix, such as Eurofix and hybrid/integrated receivers;

- Associated integrated bridge systems: - Effect of radio navigation systems on any bridge system on the vessel which uses the position fix, such as ECDIS and VDR;

- Associated communication systems: - Effect of radio navigation systems on any ship borne radio communication means used to transfer a position fix to other vessels and/or shore, such as AIS and GMDSS.

#### IALA Policy

The following statement revokes the IALA Policy on Radio Aids to Navigation adopted in 1992.

Considering

- The use of radio aids to navigation can foster safe, economic and efficient movement of vessels, which is beneficial to the maritime community and the protection of the environment;

- The IALA members can improve the contribution of radio navigation to these objectives by improving the performance of radio aids to navigation;

- IALA offers a platform for IALA member to harmonise and co-ordinate their intentions and activities with respect to radio aids to navigation.

Taking into account

- The increasing convergence of interests on radio navigation with other international organisations, such as IEC, CIRM, PIANC, as well as the UN Specialised Agencies, IMO, ITU, IHO;

- The accelerating technological development with respect to radio aids to navigation;

- The accelerating technological development of shipborne radio navigational aids, including the integration with other shipborne navigational/bridge systems, such as ECDIS, VDR, AIS and GMDSS;

- The increasing reporting of position fixes from the radio aids to navigation/navigational aids to other vessels and shore through AIS, VDR and other means;

- The increasing use of radio aids to navigation by other maritime users, such as fishing vessels and recreational craft, by inland waterways, land and non-transport users;

- The increasing introduction of radio aids to navigation which are not developed for maritime use only, but are multi-modal;

- The potential for unintentional interference and deliberate jamming of radio navigation systems.

Decides

That IALA should continue to support and develop radio aids to navigation in the maritime sector where applicable and appropriate by encouraging and/or enabling:

- Enhanced co-operation with other international organisations on development and standardisation of radio navigation systems.

- Harmonisation and co-ordination of the introduction, operation, maintenance and use of radio aids to navigation in the maritime sector, including aids not developed for maritime use only.

- Incorporation of all user interests in the maritime sector, including fishery, recreation etc. and consultation with users, especially in the event of discontinuation of radio navigation systems (see IALA Recommendation on the Discontinuation of Radiobeacons, R115).

- Integration of (the output of) different radio aids to navigation and radio navigational aids in the maritime sector.

- Awareness of the integration of shipborne radio navigational aids with other shipborne navigational /bridge systems, such as ECDIS and VDR and radio communication systems, such as AIS and GMDSS.

### Marine Radio Beacons

The International Convention on the Safety of Life at Sea, 1974, (SOLAS) [include revision date here], Chapter V, Regulation 12 (p) currently requires ships of 1600 grt and upwards that are engaged on international voyages to be fitted with radio direction-finding apparatus, unless the ship is provided with other radionavigation equipment suitable for use throughout its intended voyage.

When the direction-finding apparatus is used in conjunction with shore based radio beacons transmitting in the maritime radionavigation band (283.5 to 325 kHz), ships have a means of positioning fixing and homing at ranges of up to 200 nautical miles.  Since the accuracy or the bearing measurement is typically 5º (95% probability), the accuracy of the position fix is significantly inferior to GNSS derived positions.

Under current International Telecommunications Union (ITU) regulations, the marine radionavigation band can be used to transmit differential corrections for GNSS, on condition that no harmful interference is caused to radio beacon stations operating in the radionavigation service.

An increasing number of marine radio beacons  are now used solely for transmitting DGNSS corrections and the number of direction finding beacons is declining.

### Radar Reflectors

#### Description

A radar reflector is a passive device designed to return the incident radar pulses of electromagnetic energy back towards the source and thereby to enhance the response on the radar display.

By design, a radar reflector attempts to minimise the absorption and random scattering effects.

#### Applications

A radar reflector is generally installed as a supplementary device at sites that would also be marked with a light. Chapter V of the SOLAS Convention requires all ships over 300 grt  to be fitted with a 9GHz radar.  As a consequence, a large number of vessels  can make use of radar reflectors.

The main objectives of its use are to enhance:

- target detection at long ranges (for example, for landfall navigation);

- target detection in areas of sea or rain clutter;

- radar conspicuity of aids to navigation to reduce the risk of collision damage.

#### Radar Cross Section (RCS)

The performance of a radar reflector can be defined in terms of its effective radar cross section (RCS).  This is a value determined by comparing the strength of radar signals returned by the radar reflector with the equivalent return from a radar reflective sphere.

Example:  A radar reflector quoted as having an RCS of 30 m2 would be equivalent to a radar reflecting sphere of diameter:

D = 30×4π = 6.2 metres

#### Types of Radar Reflectors

The most common types of radar reflectors use two or more planes joined together at right angles so that the incident radar pulse is turned back towards the source after multiple reflections.  Examples are the dihedral and trihedral (or corner) reflectors.

##### Dihedral Reflector

The dihedral reflector with the plates mounted vertically only responds well to radar signals that are closely aligned to the horizontal plane (ie. normal to the axis between the plates).  Applications for dihedral radar reflectors are limited to fixed aids to navigation structures and then only when the installed elevation is close to the radar antenna height of typical user vessels.

##### Trihedral Reflector

The trihedral radar reflector is better suited to applications on floating aids and general situations where there is considerable variation in the radar antenna height of passing vessels.

A single trihedral reflector has a beam width of about 40º (to the 3dB limits).  For an omnidirectional response it is usual to combine a number of trihedrals into what is commonly called a “corner cluster” reflector.

While nine reflector elements of 40º beam widths (ie. 360º/40º) are theoretically needed to give an omnidirectional coverage, it is not uncommon for manufacturers to offer products with fewer reflector elements, for example octagonal (8) and pentagonal (5) clusters.

##### Luneberg Lens

Another type of radar reflector is the Luneberg Lens.  This is a microwave lens consisting of a series of concentric shells of differing dielectric constants. The highest dielectric constant, or refractive index, resides at the core and the lowest, at the outer shell.  Microwaves passing through this arrangement of shells are focused in the same manner as light passing through a glass lens.  In radar reflector applications, the radar pulse is focused onto a metal plate that reflects it back along the same path.

A Luneberg Lens can be fabricated from materials such as polystyrene foam, foamed glass, and other cellular materials.  The bulk density of the foam can varied to alter the dielectric constant.

For the same RCS, a Luneberg Lens tends to be more expensive and less durable than a corner cluster radar reflector.

#### Effectiveness

The effectiveness of a radar reflector is determined by:

- type and design of reflector;

- physical size;

- the type of platform (fixed or floating);

- the installed height (elevation).

Table 6.1 indicates the typical level of improvement that can be obtained from using a radar reflector.

Table 6.1 Indicative effectiveness of radar reflectors.

| Target | Range (nautical miles) |  |
| --- | --- | --- |
|  | Without Radar Reflector | With Radar Reflector |
| Coastlines rising to 60 m | 20 | 25 |
| Coastlines rising to 6 m | 7 | 14 |
| Navigation buoy with effective echoing area of approximately 10m2 | 2 | 10 (reflector 3m above waterline) |

#### Performance Criteria

IALA has not established any recommendations or guidelines on the use of radar reflectors.

#### Technical Considerations

- The range at which a radar reflector target can be detected is dependent on the heights of the radar antenna and the reflector and the output power of the radar.  There are analogies to the geographical range of visual marks.

- The radar performance of corner cluster reflectors may vary considerably from one make to another, despite being of similar physical size.  This arises from differing design philosophies; some that favour the fabrication process and others that try to optimise the polar distribution of radar reflections.

### Radar Target Enhancers

#### Description

A Radar Target Enhancer (RTE) is a device that amplifies and returns the pulse from a ship’s radar to give an enhanced image on the radar screen.  Unlike a racon, the returned signal from an RTE is not coded.

The RTE was designed primarily for buoy applications, but could also be used on small vessels that might normally carry a passive radar reflector.

A paper on RTE trials presented at the 1998 IALA Conference noted that the RTE tested had an effective radar cross section (RCS) of about 100 square metres, compared with an RCS of 20 to 30 square metres for passive radar reflectors typically fitted to buoys.

To date, commercially available RTEs only operate in the 9 GHz band.

### Radar Transponders

A radar transponder as defined by IMO Resolution A.615(15) is not an aid to navigation and should not be confused with a racon.  The definition of these radar transponders is stated as:

Transponders in the maritime radiodetermination service are receiver/transmitter devices that automatically transmit when being interrogated by a characteristic signal.  Transmission can also be initiated by a local command.  The transmission may include a coded or non-coded identification signal and/or data.  The response may be displayed on a radar PPI, or on a display separate from any radar, or both, depending upon the application and content of the signal.

- Radar transponders have not been adopted for general maritime use.

### Radar Beacon (Racon)

#### Description

Racons are receiver/transmitter devices operating in the maritime radar frequency bands (9 and 3 GHz) that enhance the detection and identification of certain radar targets.

A racon responds to the presence of a ship’s radar by sending a characteristic pulse train. The response appears as a coded mark (or “paint”) on the ship’s radar display that highlights the range and bearing of the racon. The display paint can be fixed to a specified length or can be dependant on the radar range setting  and uses a Morse character for identification.

| Fig 3.13 Example of a racon and a radar display with and without the racon character. |  |
| --- | --- |

#### Applications

A racon is generally considered to be a supplementary aid to navigation installed at sites that would also be marked with a light.  As noted in the IMO carriage requirements contained in Chapter V of the SOLAS Convention, 1974, [amended date ] a large number of vessels are capable of making use of a racon.

SOLAS Chapter V, Regulation 20, requires: [confirm carriage requirements with recent amendments to SOLAS] - all ships over 300 grt to carry a 9GHz radar, and; - all ships over 10,000 grt to be fitted with a second radar, preferably 3GHz.

A racon can be used for:

- ranging and identification of positions in ice conditions or on inconspicuous coastlines;

- identification of aids to navigation, both seaborne and land based;

- landfall identification;

- indicating centre and turning point in precautionary areas or TSS [confirm expansion of TSS included in document – Traffic Separation Scheme];

- marking hazards;

- indicating navigable spans under bridges;

- leading lines.

#### Racon Types

There are two different operating modes:

- The frequency-agile racon which represents current technology, and;

- The earlier and obsolete swept-frequency racon.

Frequency-Agile Racon: - responds on the frequency on which  it is interrogated  and hence the response could be re-painted on each radar sweep. However, to avoid masking other features on the radar screen the racon response is usually switched on and off on a preset cycle.

Swept-Frequency Racon: - has a receiver tuned to the whole 9.3 to 9.5 GHz band width and a transmitter (oscillator) that changes frequency from the lower band limit to the higher limit over the sweep cycle, generally 1 to 2 minutes.  When the racon receiver detects a ship’s radar, the racon transmitter is activated but the ship’s radar will not display the racon response until there is a frequency match between the racon and radar.  This will occur for several turns of the radar antenna over the sweep period. As noted above, this type of Racon is obsolete.

#### Signal Characteristics

Racons operate in the 9 GHz band with horizontal polarisation, and/or in the 3 GHz band with horizontal and optionally vertical polarisation.

Table 3.9 Preferred terminology for the description of racon operating frequencies.

| Preferred Terminology | Alternatives |  |  |
| --- | --- | --- | --- |
| 9 GHz | 9300 -9500 MHz | X - band | 3 cm |
| 3 GHz | 2900 -3100 MHz | S - band | 10 cm |

#### Performance Criteria

The availability of a racon is the principal measure of performance determined by IALA.

In the absence of any specific considerations IALA recommends that the availability of a Racon should be at least 99.6%

Refer to IALA documents: - Guidelines on Racon Range Performance, December 1999; [to be updated – include date here] - Recommendations on Maritime Radar Beacons (Racons) (R-101), January 1995. [updated – include date here] - Recommendations for the marking of fixed bridges over navigable waters (O-113), May 1998.

#### Technical Considerations

- The angular accuracy of the bearing between the ship and racon depends entirely on the interrogating radar while the accuracy of the range measurement depends on both the radar and racon;

- When racons are used in leading line applications, an alignment accuracy of about 0.3 degrees can be expected;

- When the ship is very close to the racon, side-lobes from the radar antenna can trigger the racon.  The resulting multiple responses on the radar display may be a distraction and can mask other targets.  Side-lobe suppression techniques are now standard features of frequency agile racons.

### LORAN / CHAYKA

#### Description

These are long range terrestrial radionavigation systems that can be used for land, sea and air navigation as well as timing.

LORAN–C is a hyperbolic radionavigation system developed during the 1960s to meet U.S. Department of Defence requirements.  The Russian Federation operates a similar radionavigation system called CHAYKA.  There are currently about 24 LORAN–C and CHAYKA chains operating around the world.  The principal coverage areas include the USA, Canada, Saudi-Arabia, India, China Sea, Korea, North-West Pacific, Russian Federation, and North West Europe.  The two systems can be accessed using commercially available receivers.

A LORAN–C chain comprises between 3 to 5 stations that have a spacing of 600 to 1000 nm.  The signal format is a structured sequence of brief radio pulses on a carrier wave frequency centred on 100kHz.  One of the stations is designated as the ‘master’ and transmits bursts of 9 pulses.  The other stations are called ‘secondaries’ and these transmit bursts of 8 pulses.

The pulse spacing is a characteristic unique to each chain and is referred to as the Group Repetition Interval (GRI). It is recommended that the choice of GRI for any new LORAN–C or CHAYKA should be coordinated through IALA to avoid interference with other chains.

The selected carrier wave frequency favours the propagation of a stable ground wave over long distances.  LORAN–C receivers are designed to determine positions using the ground wave and reject the delayed sky wave that would potentially distort the received signal.

The transmissions from each chain are monitored and controlled continuously.  System abnormality indicators are built into the signal format and can be identified by the receiver providing inherent integrity warnings.

#### Performance Criteria

LORAN–C / CHAYKA can deliver:

- a ground wave propagation range of 800 to 1200 NM, depending on transmitter power, receiver sensitivity and attenuation over the signal path;

- positional accuracy of 0.25 NM (2 drms) or better;

- repeatable and relative accuracies of between 18 and 90 m.

In the absence of any specific considerations IALA recommends that the availability of a radio aid to navigation should be at least 99.6%.

#### Future Development

- The USA is currently re-capitalising their stations and control systems for an improved Loran service. This may include a method for transmitting DGNSS corrections by modulating LORAN-C pulses.

- European operators of LORAN–C, (NELS ) are using LORAN–C as a means of distributing Differential GNSS data (Eurofix). Eurofix is also to be installed on the Saudi Arabia system.

- The Russian Federation is developing a modification of the Eurofix communications capability.

- Both Europe and the USA are developing methods of disseminating UTC time via the LORAN-C communications capability.

- The extent of the present LORAN–C / CHAYKA coverage and possible expansion of coverage being considered for Europe and Asia makes this system a potential component of the IMO World-Wide Radionavigation System;

- The value of LORAN–C / CHAYKA is largely dependent on coverage, the availability of low cost receivers and the extent to which ship owners and operators are prepared to install the receivers.

Further information on Eurofix and future developments can be found on the Delft Technical University website [www.eurofix.tudelft.nl]

## Satellite Radionavigation Systems

### IALA Policy

The IALA Statement on radio aids to navigation, (at Section 3.2.1), supports and encourages Authorities providing satellite radionavigation systems to make their systems available to users and to ensure that the accuracy and availability of the navigational information provided is to the highest standard possible.

### GNSS

Global Navigation Satellite System (GNSS) is a generic term for a satellite system that provides a world-wide position determination, time and velocity capability for multi-modal use.

GNSS is based on a constellation of active satellites which continuously transmit coded signals in two or more frequency bands. These signals can be received by users anywhere on the earths surface to determine their position and velocity in real time based on ranging measurements.

If a GNSS conforms to IMO resolution  A.953(23), February 2004 for a World-Wide Radionavigation System (WWRNS), the receivers of that GNSS will satisfy the IMO carriage requirements for position fixing equipment referred to in Chapter V of the SOLAS Convention, 2002.

Since 1996, the US “Navstar” Global Positioning System (GPS) and the Russian Global Orbiting Navigation Satellite System (GLONASS) have been recognised as components of the WWRNS.  In the future, GNSS  will include other systems such as “Galileo”, a system  currently under construction by the European Union.

#### Global Positioning System (GPS)

The Global Positioning System, Standard Positioning Service (GPS SPS) is a three-dimensional positioning, three-dimensional velocity and time system that became fully operational in 1995.  The system is operated by the United States Air Force on the behalf of the United States Government.

The GPS SPS is available on a non-discriminatory basis and free of direct user fees to all users with an appropriate receiver.  The service satisfies the requirements for general navigation with a horizontal position accuracy of 15 to 25 metres (95% probability).

GPS receivers, in combination with other equipment are able to provide:

- absolute positioning (ie. where am I?);

- relative positioning (ie. where am I with respect to some other thing? This can be further processed to derive speed over ground (SOG) , course over ground (COG), etc.);

- timing.

This information may refer to a stationary observer (static positioning) or while the observer is moving (kinematic positioning).

Further information on GPS can be found on the USCG NAVCEN website [www.navcen.uscg.gov]. The Internet site also has a link to the United States Federal Radionavigation Plan 2001 (FRP 2001) that provides a comprehensive account of current and future developments for GPS.

### Global Navigation Satellite System (GLONASS)

Global Navigation Satellite System (GLONASS) is a three-dimensional positioning, three-dimensional velocity and time system managed by the Russian Space Agency for the Russian Federation.

GLONASS has a similar potential user community to GPS SPS.  It is available on a non-discriminatory basis and free of direct user fees to all users with an appropriate receiver.  With a full complement of 24 satellites, the service would satisfy the requirements for general navigation and give a horizontal position accuracy of 45 metres (95% probability). However, the service does not have a full complement of satellites at present.

Further information on GLONASS and future developments can be found on the Ministry of Defence of the Russian Federation Coordination and Information Centre website [http: www.glonass-center.ru]

#### Galileo

Galileo, the future European satellite navigation system,  will be under civilian control,

Galileo is expected to have a constellation of 30 satellites, of which 27 will be

Operational with three active spares. These will be positioned in a Medium Earth

Orbit (MEO) with an inclination of 56o to the equator. These satellites will provide

dual frequencies as standard, enabling users to calculate their positions with metre level accuracy.

Galileo will provide five levels of service, each with their own performance

standards: Open Service (OS), Safety-of-Life (SoL), Commercial Service (CS),

Public Regulated Service (PRS) and Search and Rescue service (SAR). Galileo is

expected to provide timely warnings of integrity failure, within a few seconds.  Among its novel features will be a system to relay distress messages to COSPAS-SARSAT service centres while keeping the user informed.

Galileo full operational capacity is scheduled for 2008 and recent developments have included the draft agreement over interoperability and cooperation with GPS.

Further information on Galileo can be found on the Internet Home Page [http://europa.eu.int/comm./dgs/energy_transport/Galileo_en.htm]

### Differential Global Positioning navigation System (DGNSS)

Differential GNSS is an augmentation system for reducing the errors in the GNSS signals within a localised area.  The process involves comparing the accurately surveyed position of the DGNSS (or Reference) station against positions determined from the GNSS satellites in view.  Messages containing positional errors and satellite integrity (health) information are broadcast for users who have the appropriate receivers.  The result for the user is:

- enhanced positional accuracy within a localised area, and;

- almost immediate notification of faulty satellites (compared with up to two hours with GPS).

#### DGNSS Broadcast Stations

The internationally accepted method of providing DGNSS corrections to maritime users is by local broadcast stations transmitting  non-encrypted corrections on frequencies within the maritime radionavigation band (285 to 325 kHz).  IALA maintains a list of radiobeacon-based DGNSS stations which is accessible via the Internet.

The full list of maritime radiobeacon based DGNSS stations (as notified to IALA by authorities) can be accessed via the IALA website [www.iala-aism.org]

#### “Wide Area” DGPS systems

The Wide Area Augmentation System (WAAS) developed by the US Federal Aviation Administration for the commercial aviation industry service uses geostationary satellites instead of the terrestrial radio broadcast station to deliver the GPS correction and integrity data.  This enables a much larger area to be covered although there is more complexity involved in message processing.  Similar systems have been developed in Europe (EGNOS) and Japan (MSAS). EGNOS includes corrections and integrity information on GPS and GLONASS.

Further information on DGPS and WAAS can be found in the United States Federal Radionavigation Plan 2001 (FRP 2001) that can be accessed via the USCG NAVCEN website [www.navcen.uscg.gov]. Further information on EGNOS can be found via www.esa.int/export/esana/

#### Maritime Applications of DGNSS

In the maritime area, DGNSShas a number of applications.  Some of these are indicated in Table 5.1.

Table 5.1 Maritime Applications ofDGNSS.

| - Navigation on the high seas. | - Location of commercial fishing traps and gear. |
| --- | --- |
| - Search and rescue. | - Offshore drilling research. |
| - All weather harbour approach navigation. | - Ice breaking and monitoring icebergs and flows. |
| - Vessel Traffic Services. | - Observing tides and currents. |
| - Dredging of harbours and waterways. | - Harbour facility management. |
| - Positioning of buoys and marine navigation aids. | - Location of containers in marine terminals. |
| - Navigation for recreational vessels |  |

The design of most maritime DGNSS stations does not exclude the correction signals from being received inland from the coast.  The general practice of using a  non-encrypted signal allows non-maritime users to take advantage of the DGNSS service.  Some of the many non-maritime applications are included in Table 5.2.

Where Authorities apply a levy on shipping to fund the aids to navigation network, including any DGNSS stations, there may be an issue of equity to consider as a result the access that non-paying users have to the DGNSS corrections.

#### Other Applications for DGNSS

Table 5.2 Other applications forDGNSS.

| AGRICULTURE and FORESTRY |  |
| --- | --- |
| - Forest area and timber estimates. | - Water resources. |
| - Identifying species habitats. | - Locating property boundaries. |
| - Fire perimeters. | - Precision ploughing, planting and fertilizing (with & without operators). |
| AVIATION |  |
| - Oceanic and en route navigation. | - Precise airfield and landing aid locations. |
| - Non-precision and precision all-weather approaches. | - Seamless (global) air space management. |
| - Direct routing of aircraft for fuel savings. | - Less expensive avionics equipment. |
| - Aircraft traffic management. | - Precision departures. |
| - Airport surface traffic management. | - Missed approach applications |
| - Monitor wing deflections in flight. | - Enhanced ground proximity warning system. |
| - Wind shear detection | - Automatic dependent surveillance. |
| ELECTRIC POWER |  |
| - Synchronization of power levels. | - Event location. |
| EMERGENCY RESPONSE |  |
| - Ambulance, police, and fire department dispatch. | - Road service locating disabled vehicles. |
| ENVIRONMENTAL PROTECTION |  |
| - Hazardous waste site investigation. | - Oil spill tracking and cleanup. |
| - Ground mapping of ecosystems. | - Precise location of stored hazardous materials. |
| HIGHWAY and CONSTRUCTION |  |
| - Intelligent vehicle-highway system operation. | - Navigation for motor vehicle drivers. |
| - Highway facility inventory and maintenance. | - Truck fleet on-the-road management |
| - Accident location studies. | - Monitoring status of bridges. |
| - Highway construction |  |
| LAW ENFORCEMENT and LEGAL SERVICES |  |
| - Tracking and recovering stolen vehicles | - Border surveillance. |
| - Tracking narcotics and contraband movements. | - Measuring and recording property boundaries. |
| - Maintaining security of high government officials and dignitaries while travelling. | - Tort claim evidence in aviation and maritime accidents. |

| PUBLIC TRANSPORTATION |  |
| --- | --- |
| - Bus fleet on-the-road management. | - Passenger and operator security monitoring |
| RAILROAD |  |
| - Railroad fleet monitoring. | - Facility inventory control and management. |
| - Train control and collision avoidance. |  |
| RECREATION |  |
| - Hiking and mountain climbing. | - Setting lines on sports fields. |
| - Measuring at sports events. |  |
| SURVEYING |  |
| - Electronic bench marker providing absolute reference of | - Efficient and accurate photo surveys. |
| - latitude, longitude and altitude. | - Measuring areas without triangulation. |
| - Precision surveys. | - Oil and mineral prospecting. |
| - Real-time dam deformation monitoring. | - National spatial data infrastructure. |
| - Hydrographic surveying |  |
| TELECOMMUNICATIONS |  |
| - Precise timing for interlacing messages/network |  |
| WEATHER, SCIENTIFIC and SPACE |  |
| - Use as weather balloon position radiosonde. | - Monitoring earthquakes and tectonic plates. |
| - Measurement of sea level from satellites. | - Measuring ground subsidence (sinking). |
| - Navigating and controlling space shuttles. | - Measuring atmospheric humidity from ground. |
| - Placing satellites into orbit. | - Precise global mapping of ionosphere. |

#### System Characteristics

The characteristics of DGPS correction signals for LF/MF radiobeacon transmissions are defined in ITU-R M Recommendation 823. This incorporates the format recommended by the Radio Technical Commission for Maritime services (RTCM), Special Committee 104 (revised in 1997).

#### Principal Elements

The principal elements of the system characteristics include:

- radio transmissions are in the band allocated for maritime radionavigation (“radio beacons”);

- the carrier frequency of the differential correction signal is an integer multiple of 500 Hz  (ITU-R M.823/1.1);

- the frequency tolerance of the carrier is ± 2 Hz  (ITU-R M.823/1.2);

- the data transmission rate is selectable from 25 (GLONASS only), 50,100 and 200 bits/second  (ITU-R M.823/1.6).

#### DGNSS Message Types

Table 5.3 DGNSS and DGLONASS message types

| DGPS Message Type Number | Title | DGLONASS Message Type Number |
| --- | --- | --- |
| 1 | Differential GNSS corrections (full set of satellites) | 31 |
| 3 | Reference station parameters | 32 |
| 4 | Reference station datum | 4 |
| 5 | Constellation health | 33 |
| 6 | Null frame | 34 (N=0 or N=1) |
| 7 | Radio beacon almanacs | 35 |
| 9 | Subset differential GNSS corrections (this may replace message Types 1 or 31) | 34 (N>1) |
| 16 | Special message | 36 |
| 27 | Extended Radio beacon almanacs message | N/A |

The ITU-Recommendation M.823-2 also includes the receiver characteristics:

- With the appropriate receiver capability (IEC 61108-4) the type 27 message replaces type 7.

#### Performance Criteria

IALA Recommendation R-121, Guidelines for the Performance and Monitoring of a DGNSS Service in the Band 283.5 - 325 kHz (March 1999)] addresses performance criteria. This recommendation is currently being revised (September 2004)

### World-Wide Radionavigation System (WWRNS)

IMO Resolution  A.953(23), February 2004 – World-Wide Radionavigation System (WWRNS) lays down the procedures and responsibilities concerning the recognition of systems and establishes the operational requirements for a world-wide radio navigation system.  These are applicable to DGNSS services and are provided in the following section for ease of reference.

#### Operational requirements

The operational requirements for a world-wide radionavigation system should be general in nature and capable of being met by a number of systems.  All systems should be capable of being used by an unlimited number of ships:

- the requirements may be met by individual radionavigation systems or by a combination of such systems;

- for ships with operating speeds above 30 knots more stringent requirements may be necessary.

#### Operational Requirements for High Volume / Significant Risk

The operational requirements for navigation in those harbour entrances, harbour approaches and coastal waters with a high volume of traffic and/or a significant degree of risk are defined as follows:

- where a radionavigation system is used to assist in the navigation of ships in all such waters the system, including any augmentation, should provide positional information with an error not greater than 10 metres with a probability of 95%;

- taking into account the radio frequency environment, the coverage of the system should be adequate to provide position-fixing throughout this phase of navigation;

- update rate of the computed and displayed position data should be greater than once every 10 seconds.  If the computed position data is used for AIS graphical display or for direct control of the ship, then the update rate should be greater than once every 2 seconds;

- signal availability should exceed 99.8%, calculated over a 2-year period;

- when the system is available, the service continuity should be >99.97% over 3 hours;

- a warning of system non-availability or discontinuity should be provided to users within 10 seconds.

#### Operational Requirements for Low Volume / Less Significant Risk

The operational requirements for navigation in those harbour entrances, harbour approaches and coastal waters with low volume of traffic and/or a less significant degree of risk are defined as follows:

- where a radionavigation system is used to assist in the navigation of ships in such waters the system, including any augmentation, should provide positional information with an error not greater than 10 metres with a probability of 95%;

- taking into account the radio frequency environment, the coverage of the system should be adequate to provide position-fixing throughout this phase of navigation;

- update rate of the computed and displayed position data should be greater than once every 10 seconds.  If the computed position data is used for AIS graphical display or for direct control of the ship, then the update rate should be greater than once every 2 seconds;

- signal availability should exceed 99.5%, calculated over a 2-year period;

- when the system is available, the service continuity should be >99.85% over 3 hours;

- a warning of system non-availability or discontinuity should be provided to users within 10 seconds.

#### Operational Requirements for Navigation in Ocean Waters

Where a radionavigation system is used to assist in the navigation of ships in ocean waters the system should provide positional information with an error not greater than 100 metres with a probability of 95%.  This degree of accuracy is suitable for purposes of general navigation and provision of position information in the GMDSS.

In view of the fact that merchant fleets operate world-wide, the information provided by a radionavigation system must be suitable for use for general navigation by ships engaged on international voyages in any ocean waters:

- Taking into account the radio frequency environment, the coverage of the system should be adequate to provide position-fixing throughout this phase of navigation;

- Update rate of the computed and displayed position data should be greater than once every 10 seconds.  If the computed position data is used for AIS graphical display or for direct control of the ship, then the update rate should be greater than once every 2 seconds;

- Signal availability should exceed 99.8%, calculated over a 30-day period;

- A warning of non-availability or discontinuity of the service should be provided to users as soon as practicable by Maritime Safety Information (MSI).

## Automatic Identification System (AIS)

    - Description

Automatic Identification System (AIS) is a ship and shore-based broadcast system, operating in the VHF maritime band.  Its characteristics and capability make it an outstanding new tool for enhancing the safety of navigation and efficiency of shipping traffic management.

An AIS station is a VHF radio transceiver capable of sending ship information such as identity, position, course, speed, length, ship type and cargo information etc., to other ships and to suitable receivers ashore.  See Fig 4.1 for an overview of the system.

Information from an operational shipboard AIS unit is transmitted continuously and automatically without any intervention of the ship’s staff.

When used with an appropriate graphical display, shipboard AIS enables provision of fast, automatic and accurate information regarding risk of collision by calculating Closest Point of Approach (CPA) & Time to Closest Point of Approach (TCPA) from the positional information transmitted by target vessels.

Therefore, AIS is an important supplement to existing navigational systems, including radar. In general, data received via AIS will enhance the quality of information available to the ships staff.  AIS is an important tool to enhance the awareness of the traffic situation for all users.

#### Purpose

The purpose of AIS is to:

- identify vessels;

- provide additional information in order to assist in collision avoidance;

- assist in target tracking;

- simplify and promote information exchange; and

- reduce verbal mandatory ship reporting.

#### Principal Applications of AIS

The International Maritime Organisation (IMO) specifies the following functional requirements for AIS:

- For ship to ship information exchange to assist in collision avoidance.

- For littoral states to obtain information about ships and their cargoes.

- As a VTS tool, for traffic management.

#### Capabilities

AIS is an additional source of navigational information.  AIS supports, but does not replace navigational systems such as radar target tracking and VTS.

In general, AIS tracking offers the following significant benefits:

- highly accurate information;

- provided in near real-time;

- capable of instantaneously presenting target course alterations;

- not subject to target swap;

- not subject to target loss in clutter; and

- not subject to target loss due to fast manoeuvres.

In addition, AIS can:

- ‘look’ behind the bend in a channel or behind an island in an archipelago, to detect the presence of other ships and identify them;

- predict the exact position of a meeting with other ships in a river or in an archipelago;

- know which port and which harbour a ship is bound for;

- know the size and the draft of ships in the vicinity;

- detect a change in a ship’s heading almost in real time; and

- identify a ferry leaving the shore bank in a river.

CAUTION

NOT ALL SHIPS CARRY AIS

The Officer of the Watch (OOW) should always be aware that other ships, in particular pleasure craft, fishing vessels, warships and some shore stations including Vessel Traffic Service (VTS) centres, may not be fitted with AIS.

The OOW should always be aware that AIS fitted on other ships as a mandatory carriage requirement, might, under certain circumstances, be switched off, particularly where international agreements, rules or standards provide for the protection of navigational information.

Navigators should be aware of the limitations of AIS.  In particular, government agencies and owners should ensure that watch-keeping officers are trained in the use of AIS and are aware of its limitations.

Fig 4.1 Overview of the AIS System

    - Components

#### AIS Station

Each AIS station consists of:

- one VHF transmitter;

- two VHF SOTDMA receivers;

- one VHF DSC receiver;

- a GNSS receiver providing timing for slot synchronisation, and

- a marine electronic communications link to shipboard display and sensor systems.

Positional and timing information is normally derived from an external global navigation satellite system (e.g. GNSS), including an MF Differential GNSS receiver for precise positioning in coastal and inland waters.

The AIS system operates primarily on two dedicated VHF channels in the maritime mobile band. Where these channels are not available regionally, the system is capable of automatically switching to designated alternate channels.

In practice, the capacity of the system is unlimited, allowing for a great number of ships to be accommodated at the same time.

#### Shipborne AIS Component

This part of the system continuously and autonomously:

- transmits the ship's own data to other vessels and to AIS equipped stations;

- receives data of other vessels and AIS equipped stations, and can display this data textually and graphically, as required.

Fig 4.2 Schematic diagram of AIS station.

The AIS is able to “see” around bends and behind islands if the landmasses are not too high. A typical range to be expected at sea is 20 to 30 nautical miles, depending on antenna height.

Typically, the components of the ship system are the AIS station, antennas, a small personal computer (PC) and an ECS/ECDIS application. Other devices to be fitted include an (external) GNSS receiver, a compass (for heading input) and, optionally, a differential GNSS (DGNSS) receiver.

The functionality and benefits provided to ships’ operators include:

- Real time tracking of all AIS fitted ships on the ECS/ECDIS display;

- Near instantaneous presentation of positions (at DGNSS accuracies), along with SOG and COG;

- Presentation of predicted track when turning or manoeuvring;

- ETA (Estimated Time of Arrival) functionality for all AIS fitted ships;

- Recording of track history;

- Availability of DGNSS corrections from base stations over the SOTDMA data link;

- Broadcast of own ships dynamic, static and voyage related data to other ships and to VTS centre, and;

- Ability to send or receive short safety related messages to/from VTS centre or other ships.

    - AIS Information

#### Ship’s Data Content

#### Information Types

The AIS information transmitted by a ship includes four different types of information:

- fixed or static information that is entered into the AIS unit on installation and need only be changed if the ship changes its name, call sign or undergoes a major conversion from one ship type to another. This information is broadcast every six minutes.

- dynamic information that, in general, is automatically updated from the ship sensors connected to the AIS. This information is updated as per the table in the section ‘Reporting Rates’.

- voyage related information that might need to be manually entered and updated during the voyage. This information is also broadcast every six minutes.

- If required, short safety related messages.

These are explained and details of data contents are given in the following table.

Table 4.1 Data sent by ship.

| Information item | Information Generation - Type and Quality |
| --- | --- |
| Static |  |
| MMSI | Maritime Mobile Service Identity (set on installation) This might need amending if the ship changesownership. |
| Call sign and name | Set on installation. This might need amending if the ship changes ownership. |
| IMO Number | Set on installation. |
| Length and beam | Set on installation, or if changed. |
| Type of ship | Select from pre-programmed list. |
| Location of position fixing antenna | Set on installation or may be changed for bi-directional vessels or those fitted with multiple antennae. |
| Dynamic |  |
| Ship’s position with accuracy indication and integrity status | Automatically updated from the position sensor connected to AIS. The accuracy indication is for better or worse than 10 m. |
| Position Time stamp in UTC | Automatically updated from ship’s main position sensor connected to AIS. |
| Course over ground (COG) | Automatically updated from ship’s main position sensor connected to AIS, if that sensor calculates COG. |
| Speed over ground (SOG) | Automatically updated from the position sensor connected to AIS. |
| Heading | Automatically updated from the ship’s heading sensor connected to AIS. |
| Navigational status | Navigational status information has to be manually entered by the Officer of the Watch and changed, as necessary, for example: – underway by engines; – at anchor; – not under command (NUC); – restricted in ability to maneuver (RIATM); – moored; – constrained by draught; – aground; – engaged in fishing; – underway by sail. In practice, since all these relate to the COLREGS, any change that is needed could be undertaken at the same time that the lights or shapes were changed. |
| Rate of turn (ROT) | Automatically updated from the ship’s ROT sensor or derived from the gyro. |
| Voyage related |  |
| Ship’s draught | To be manually entered at the start of the voyage using the maximum draft for the voyage and amended as required. e.g. – Result of de-ballasting prior to port entry. |
| Hazardous cargo (type) | To be manually entered at the start of the voyage confirming whether or not hazardous cargo is being carried, namely: DG Dangerous goods HS Harmful substances MP Marine pollutants Indications of quantities are not required. |
| Destination and ETA | To be manually entered at the start of the voyage and kept up to date as necessary. |
| Text |  |
| Short safety-related messages | Free format short text messages, to be manually entered, addressed either a specific addressee or broadcast to all ships and shore stations. |

#### Short Safety Related Messages

Short safety related messages are fixed or free format text messages addressed either to a specified destination (MMSI) or to all ships in the radio coverage area.

Their content should be relevant to the safety of navigation, e.g. an iceberg sighted or a buoy not on station.

Messages should be kept as short as possible. The system allows up to 158 characters per message but it will be easy for shorter messages to find free space for transmission.  At the moment, these messages are not further regulated, in order to keep the arrangement as flexible as possible.

Short safety related messages are only an additional means to broadcast Maritime Safety Information (MSI).  Whilst their importance should not be underestimated, the usage of such short safety related message does not remove any of the obligations or requirements under existing communication protocols.

The operator should ensure that he/she displays and considers incoming safety related messages and should send safety related messages as required.

Note:

- The system can handle over 2,000 reports per minute and updates as often as every two seconds.  Self Organising Time Division Multiple Access (SOTDMA) technology ensures that this high broadcast rate is met and that stable and reliable ship-to-ship operation is achieved.

- The system has limited DSC  capability, allowing shore-based GMDSS systems to establish AIS operating frequency channels and to identify and track AIS-equipped vessels.

#### Technical Information

##### Signal Characteristics

AIS operates primarily on two dedicated VHF radio frequencies AIS1 (channel 87B) – 161.975 MHz and AIS2 (channel 88B)– 162.025 MHz in the maritime mobile band.

##### System Characteristics

According to the IMO performance standard, the required reporting capacity of the system is a minimum of 2000 reports per minute.  Further, the ITU Technical Recommendation for the AIS provides for 4500 time slots per minute.  A position report message from one AIS station fits into one of the 2250 time slots, established every 60 seconds (other reports may occupy more slots).  As two VHF channels are available for use, the number of time slots available are doubled to 4500.

The broadcast mode (based on SOTDMA) allows the system to be locally overloaded by 400 to 500% and still provides nearly 100% throughput for ships closer than 8 to 10 NM to each other in a ship-to-ship mode.

In the event of a system overload, only targets far away will be subject to drop out in order to give preference to targets close by, which is a primary concern for ship-to-ship operation of AIS.  In practice, the capacity of the system is unlimited allowing for a great number of ships to be accommodated at the same time.

##### Data Transfer Using SOTDMA

A shipborne mobile AIS station will normally operate in an autonomous and continuous mode, regardless of whether the fitted vessel is operating in the open seas, coastal waters or inland areas.

As VHF coverage is characteristically short-ranged, a substantial data rate is required.  The AIS station communicates using Time Division Multiple Access (TDMA) on two parallel VHF channels.  Each minute of time is divided into 2250 slots of equal length and these are accurately synchronised using UTC time information as a first phase timing mechanism.  The system is able to operate using a secondary independent timing mechanism if required, which provides timing accuracy of better than 10 s. These 2250 slots constitute a frame, and each frame is repeated every minute.

Each AIS station determines its own transmission schedule (slot allocation), based upon data link traffic history and knowledge of future actions by other stations.  A position report message from one AIS station fits into one of 2250 slots, established every 60 seconds.

AIS stations continuously synchronise themselves with each other to avoid overlap of slot transmissions.  The slot selection protocol by stations ensures that vessels will always receive new stations, including those stations that suddenly come within radio range of other vessels.

As the system operates in the VHF radio band, it is capable of communicating within ‘line of sight’.  Should the number of AIS stations within line of sight range of a receiving AIS station exceed the frame capacity (in terms of reports per minute), the SOTDMA algorithms ensure that the effective radio cell for each AIS station slowly decreases.

The overall effect is that, as a channel approaches an overloaded state, the SOTDMA algorithms cause a ‘graceful degradation’ of radio cell size, dropping off reports from AIS stations at long range while maintaining the integrity of the (more important) closer range reports.

While the basic SOTDMA VHF AIS network is exciting in itself, the further extension of this system employing advanced technological applications, promises an exciting future.  That future will see the inclusion of additional functionality, such as DGNSS correction services, portable Pilot Packs, radar target broadcasting, a long range AIS mode (to facilitate EEZ monitoring and SAR), all presented to the VTS operator and ship navigator on an ECDIS, radar or AIS dedicated display.

Refer to the IALA Guidelines on the Automatic Identification System (AIS) Parts 1 and 2 for more information.

Fig 4.3 Principles of SOTDMA.

##### AIS Reporting Rates

The IMO Performance Standards provide the type of data to be exchanged but not the required reporting rate. In taking into account potential VTS/Ship Reporting System requirements, considerations were based on current radar techniques, timings of consecutive DGNSS position fixes and finally - as a worst case scenario – peak traffic scenarios in the Singapore and Dover Straits.

Using a theoretical maximum VHF radio range of 40 NM, an estimate of about 3000 reports per minute was calculated for the Singapore Straits. A similar calculation for Dover Strait gave a requirement for about 2500 reports per minute.

Consecutive DGNSS position fixes:- In restricted waters, positions of other ships are required to accuracies of the order of ‘better than 15 metres’ for reliable tracking, collision avoidance and remote pilotage purposes. Navigating using DGNSS provides positional accuracy of about 10 metres. Allowing for the application of movement prediction algorithms means that the added position error will be in the order of less than 10 metres.

[add – integrity monitoring – vital part of broadcast]

For ships not changing course the update rates needed to achieve this level of positional accuracy are dependent on the speed of the ship and result in the following reporting intervals:

Table 4.2 AIS reporting intervals.

| Ship’s manoeuvring condition | Nominal reporting interval |
| --- | --- |
| Ship at anchor or moored and ships not moving faster than 3 knots | 3 min |
| Ships at anchor or moored and ships moving faster than 3 knots | 10 sec |
| Ships 0-14 knots | 10 sec |
| Ships 0-14 knots and changing course | 31/3 sec |
| Ships 14-23 knots | 6 sec |
| Ships 14-23 knots and changing course | 2 sec |
| Ships > 23 knots | 2 sec |
| Ships > 23 knots and changing course | 2 sec |

Note:

- Some Administrations can be expected to apply the AIS requirements to a wider range of smaller vessel categories including pleasure craft and fishing vessels.

- In recognition of this, allowance has been made in the AIS Technical Recommendation (ITU-R M.1371-1) for a Class A and Class B Shipborne Mobile AIS Station.

- Class A equipment complies with the IMO AIS carriage requirements, while Class B provides facilities that are not necessarily fully compliant with IMO carriage requirements.

#### Display Requirements

##### Considerations

There is a need to consider display requirements for the data, if it is to be useful to the mariner. The IMO Performance Standard for the Presentation of Navigation-Related Information on Shipborne Navigational Displays addresses this issue, although it is possible to display AIS information on the ship’s radar or on an electronic chart display and information system (ECDIS).

The danger of overloading the radar screen would need to be considered and correlation between primary radar targets and AIS targets is likely to be required.

- The most cost-effective way to display the AIS data is to have a radar-like display on a PC with only the AIS targets and information on them presented, so that bearing and distances to the targets could be compared and identified on the radar. Software programs are freely available and there is only the cost of a PC (which often is normal inventory on a ship today).

Recently, ship borne radars have received the type approval to display AIS information, although ECDIS does not have similar type approval. Consequently, many AIS displays employ a computer based, dedicated graphical display.

##### IEC Test Standard

In developing the Test Standard IEC 61993-2, the IEC Technical Committee 80 have founded it necessary to specify a “minimum display requirement for AIS” in order to validate the proposed test functions.

It is emphasised that this is a minimum display requirement for AIS, primarily for testing purposes, although it could be used at sea in a low shipping activity environment. To obtain the full benefit of the AIS capability, the system should be integrated into one of the existing graphical displays on the bridge, or a dedicated graphical display.  Greater functionality would be provided by a more capable graphical display but selection of the type of display is dependent on the user requirement and manufacturer’s offered options.

##### Integration with Electronic Chart Systems

- Some ships already have an Electronic Chart System (ECS) or a full SOLAS compliant Electronic Chart Display and Information System (ECDIS) where the AIS could be presented. For manufacturers of other chart systems, it is a matter of implementing the ability to present the AIS into their software. For ECS, it is important that it works with the geodetic datum WGS 84, as with the charts.

- To present the AIS on the display of an old radar is not always possible, but when AIS is mandatory on new ships, radar manufacturers would be able to include this facility. AIS targets, superimposed on the radar display, will give the operator information of which targets have AIS and which do not.

- IMO NAV 50 (July 2004) agreed on the following:

- A) Draft Recommendation on Performance Standards for the Presentation of Navigation-Related information on Ship-borne Navigational Displays

- B) SN/Circular titled Guidelines for the Presentation of Navigation-Related Symbols, Terms and Abbreviations.

#### High Update Rate

- Today, with a modern ARPA radar, 20 targets could be selected and tracked with an update rate of 3 seconds. The symbol on the PPI will, in most cases, follow the target, except in the case of a target swap. The limitations of the ARPA make a course alteration on another ship undetectable, until one or two minutes after the course change has started and for a large tanker it could take up to five minutes before a change is detected. Furthermore, the ARPA also needs one or two minutes before it can present a vector with course and speed through water of a target.

- For AIS targets superimposed on a radar display, the need for a high update rate becomes obvious. If the update rate is too slow there will be situations on smaller ranges where the AIS symbol will not catch up with the radar target. The optimal solution would be, that the update rate would be the same as the radar, i.e. every 3 seconds.

    - AIS Services and Applications

Other services and applications have been recognised for AIS, and these are discussed below:

#### Applications

- Radar applications;

- Radar target broadcasting;

- Vessel traffic management application;

- Long range application;

- AIS in search and rescue (SAR) operations;

- VTS and radar;

- AIS as an aid to navigation.

#### Services

- Broadcast of DGNSS corrections;

- Polling.

#### Stations

- Portable Pilot Units;

- Repeater stations.

    - AIS Applications

#### Radar Applications

Where radar is already available, AIS serves to provide positive vessel identification. This is something that is otherwise not achievable without exchanging voice message over VHF radio. Some ports and VTS areas will therefore look to AIS as a VTS tool to further augment radar-covered areas. However, because of the different radio propagation features of VHF and its capability to provide effective and accurate traffic management coverage in non-radar covered areas, some will see it as a tool to enhance shipboard navigation in narrow, radar constricted areas or heavily radar video cluttered river or canal routes.

Such areas exist on the Canadian West Coast, United States (Mississippi), Norway, Sweden, Finland and Germany.  It is therefore not surprising that these nations are amongst the leaders in the trials and development of the AIS.

Fig 4.4 A comparison of AIS and radar displays in a constricted river or canal.

#### Broadcast of Differential GNSS Corrections

Broadcasting differential GNSS corrections on the SOTDMA data link to all mobile AIS stations enables those recipients to navigate with DGNSS accuracy. The positional information broadcast from vessels will have differential accuracy, using the best available correction at that instant.

This type of system could serve as the primary system in a port or VTS area or as a back up for the IALA DGNSS MF Beacon System. For full compatibility with the IALA DGNSS MF Beacon System, it should be provided with capabilities for integrity monitoring and to transfer that information to the user.

#### Radar Target Broadcasting

Another proven application is the process of converting radar targets to AIS targets in the radar target processor and transmitting these from the VTS to ships in the area.  This allows all AIS fitted units in the vicinity to view radar targets held by the shore based radar system of the VTS as well as those tracks sourced from their own radar(s).

This function would also enable small, local vessels that might only be equipped with an AIS station and ECS, to gain the benefits of the radar picture acquired by the VTS.

#### Vessel Traffic Management Application

In the VTS/VTMIS environment, AIS based applications provide certain benefits for the VTS operator including:

- Vessel identity and other static and voyage related information are automatically presented on the VTS operator’s display;

- Radar-based automatic tracking problems, such as radar target swapping and degradation due to sea clutter and weather, are non-existent;

- Higher update rate of tracks compared to tracks provided by radar;

- Additional information such as rate of turn, heading, course and speed over ground and ship dimensions is available to the VTS operator in near real time;

- Transmissions from AIS fitted targets can generally be received from positions where radar signals can not normally reach, such as behind headlands or around bends in rivers or canals;

- Text Messages can be sent from the VTS:

  - to a specific mobile AIS station, identified by its MMSI number;.

  - to all mobile AIS stations within a geographic area, or

  - in connection with a specific AIS base station; or

  - to a number of specified AIS stations identified by their MMSI number;

  - used for weather data transmissions, system status, berthing plan, pilot boarding.

The ability to transmit the following additional information to a ship might also be of benefit:

- Local navigation warnings;

- Local (temporary) corrections to Electronic Navigation Charts (ENC);

- Traffic image data, not already received via ship–ship AIS means;

- Port VTS status reports;

- Local aids to navigation status reports;

- Pilot / tugs / entry plan / berthing plan information;

- Incidents requiring adjustment to navigation plans.

Some potential benefits provided by AIS to VTS operations include:

- Use of AIS functionality and additional base stations to extend coverage of VTS data transfer;

- Enhanced navigation assistance capability through;

  - Precise navigation monitoring, and;

  - Path prediction;

- Ability to advise manoeuvring ships (as opposed to ships navigating);

- Monitoring of traffic data and the ability to archive and retrieve such data;

- Enhancing of capability of conventional aids to navigation and the ability to monitor them;

- Acting as a channel for the automatic input and relay of information;

- A real-time data source for emerging concepts, such as computer assisted decision processes;

- Potentially reduce operator workload in voice communications;

- Provision of improved track integrity.

#### Portable Pilot Unit (PPU)

Portable Pilot Units (PPU), comprising of a personal computer with ECS software and an AIS module, are being developed to enable a Pilot to set up an AIS station on board.  This will enable the pilot to access full details and movements of all traffic in the vicinity and would provide an additional communication link with the VTS operator, if required.

The major problem at present is the weight of the complete package.

#### Repeater Stations

An AIS repeater station can be placed on a site without any connection to ground infrastructure and serve to increase the coverage area.  As the name implies, these stations repeat (send out) all received messages received from a specific area, on the SOTDMA data link.  The repeater station is linked by the VHF data link (VDL) into the nearest base station, or a string of repeater stations that culminates into base station.  For example, repeaters be placed on an island or off shore structure to extend the coverage of the shore based VTS/AIS base station or set up along the coastline to achieve long distance relaying.

#### Long Range Application

The IMO performance standard for AIS requires that the equipment should function  “as a means for littoral States to obtain information about a ship or its cargo” when a vessel is operating in that State’s area of maritime responsibility.  An AIS long-range reporting mode is required to satisfy this function and to assist administrations in meeting their responsibilities for wide area or offshore monitoring of shipping traffic.

These responsibilities include safety of navigation, search and rescue (SAR), resource exploration and exploitation and environmental protection in offshore areas including the continental shelf and economic exclusion zones (EEZ).

#### Polling and Assigned Mode

Polling (or interrogation) techniques allow the VTS or competent authority to remotely access a mobile AIS and require it to transmit a report immediately or cause the ship’s station into a fixed (assigned) access communication scheme.  This effectively means that the AIS base station (or perhaps another ship, if authorised) can direct the mobile AIS station to report at pre-set intervals or times.

As all shipborne mobile AIS stations on ships transmit position reports almost continuously, it is likely that polling will occur for other type of information, not broadcast frequently.  For example, extended ship static and voyage related data, persons on board etc.  Another use of polling is in the “Long Range” mode where ships offshore may only be reporting their position and other details once in 12 or 24 hours.  Polling would normally be utilised if an authority required information on specific areas or for specific ships, such as during a SAR incident.

#### AIS in Search And Rescue (SAR) Operations

- Maritime Rescue and Co-ordination Centres’ (MRCC) SAR operations would be much more efficient if they had all rescue craft fitted with AIS, to quickly determine which one is closest to a distress situation. During a search, all craft could be tracked and plotted, enabling the MRCC to monitor the progress, to direct the available resources efficiently and to ascertain that search coverage is without gaps. Furthermore, if a ship in distress has AIS operational, it could be seen on displays of all the surrounding ships and also at the MRCC.

#### VTS, Radar and Voice Communication

##### VTS and Radar

- Currently, the main sensor of the VTS to detect a ship is radar.

- A VTS radar has similar errors in range and bearing resolution as any other radar, although it has it has the advantage of:

- a known position;

- being North oriented and ground stabilised;

- no compass errors involved.

- The limitations of the ARPA radar to track ships due to target swap, land, beacons, bridges and other ships, makes the tracking facility in the ARPA rather limited.

- There is a requirement for improving the VTS, in order to be able to:

- cover areas where radar coverage is almost impossible to achieve, like river bends and archipelagos;

- identify radar targets on the VTS operator’s display automatically;

- interrogate ships for information regarding type of cargo;

- track a ferry on a regular run between two ports in the bay or a river continuously, without needing to reacquire it every time the radar tracking has swapped to another target, when the ferry has been moored to a pier or passed too close to a beacon or a passing ship;

- know which port and which harbour a ship is bound for;

- know the size and the draft of ships in the vicinity;

- detect a change in a ship’s heading in near real time.

A high update rate AIS, with interrogating capabilities has the potential to solve all of the above requirements of the VTS.

The establishment of a global AIS in general, or a land based AIS reception network in particular, may actually mitigate the need for VTS in many areas.  The cost of installing and maintaining an AIS network is minute compared to a VTS radar network.

AIS is unlikely to render either radar or voice communications obsolete.  Initially at least, non-SOLAS vessels are unlikely to be fitted with AIS.  It can also be expected that many older SOLAS vessels will delay fitting the equipment .  Radar will remain, therefore, the only detection and tracking system capable of handling all targets.  It will also provide a tool to monitor the correct position of floating aids to navigation and an important check of electronic position fixing integrity.

However there may be a requirement to retain VTS radar in some areas to pass on VTS tracked radar targets to be broadcasted as AIS targets (see below).  The retention of radar within a VTS, after the implementation of AIS, will depend upon the operational characteristics of that VTS area and, in particular, the density of non-AIS fitted vessels operating in the area.

##### Voice Communications.

The use of AIS will aid efficiency by reducing the need for voice communications and for VTS operator manual input functions.  Voice communication will remain an essential method for passing VTS information to vessels not fitted with AIS.  It will also be required in emergency situations and other cases where immediate confirmation or an acknowledgement is required, such as when providing navigational assistance.

    - AIS as an Aid to Navigation

#### Potential

A special type of AIS station (AtoN AIS) fitted to an Aid to Navigation will provide a positive identification of the aid without the need for a specialised shipborne display.  In addition, this equipment can provide information and data that would:

- complement or replace an existing aid to navigation, providing identity, state of ‘health’ and other information such as real time tidal height and local weather to surrounding ships or back to a shore authority;

- provide the position of floating aids (mainly buoys) by transmitting an accurate position (corrected by DGNSS) to monitor if they are on station;

- provide real-time information for performance monitoring, with the connecting data link serving to remotely control changes in AtoN parameters or switching on back-up equipment;

- possibly replace radar beacons (Racons) in the future, providing longer range detection and identification in all weather conditions;

- gather data from AIS fitted shipping traffic for future aid to navigation planning purposes.

#### Virtual Aids to Navigation

Where the authority is responsible for providing aid to navigations and also provides VTS services, it is possible to use the AIS base station to retransmit, as AIS targets, the position, status, and supplementary data from aid to navigations within VHF coverage of the base station.  Some of the advantages and disadvantages of this application are shown in Table 4.3.

Table 4.3 Advantages and disadvantages of virtual aids to navigation using AIS.

| Advantages | Disadvantages |
| --- | --- |
| - Greater range than for the majority of the aid to navigations - Integrity check prior transmission - Cost reduction, as fewer stations are required - Better control over slot occupancy - Considerable reduction in power requirements to the aid to navigation | - Not suitable for aid to navigations near the edge or outside the VHF service area of the base station - No significant reduction in actual slot usage |

##### Radar Reference Target

With the ever-increasing requirements for integrated display systems, the problems of synchronising two or more systems on one display surface are increased.  If two (or preferably three) AIS stations can be installed on prominent fixed radar targets within an area of special interest, such as a harbour or harbour approach, then the AIS GNSS positions, radar echoes and chart symbols of each target can be synchronised.  This will result in reduced ambiguity and clutter on the display.

##### Other Functionality

An AtoN AIS station on a fixed aid to navigation such as a lighthouse could, in addition to transmitting its own identification, act as a relay for other aid to navigations.  It can also provide local hydrographic and meteorological data, using application specific messages.  These messages are currently undergoing development.  When available, details will be provided in the IALA Guidelines.

    - AIS Station Aids to Navigation Report Message

#### Message Content

This message is to be used by an AtoN AIS station.  The message should be transmitted autonomously at a reporting rate of once every three minutes or it may be assigned another rate by an Assigned Mode Command via the VHF data link or by an external command.  The message should also be transmitted immediately after any parameter value changes.

The main contents of the message are:

- Type of aid to navigation;

- Name of the aid to navigation;

- Position;

- Position accuracy indicator;

- RAIM indicator;

- Off position indicator;

- Type of position fixing device;

- Time stamp;

- Dimension of the aid to navigation and reference for position;

- 8 bits reserved for use by the regional/local authority (can include the technical status of the aid to navigation);

- Pseudo aid to navigation target flag.

##### Message Descriptions

Table 4.4 Aid-to-Navigation Report Message.

| Parameter | Description |
| --- | --- |
| Message ID | Identifier for this message (21) |
| ID | MMSI number |
| Type of Aids-to-Navigation | 0 = not available = default; 1 - 15 = Fixed Aid-to-Navigation; 16 – 31 = Floating Aid-to-Navigation; refer to appropriate definition set up by IALA |
| Name of Aids-to-Navigation | Maximum 20 characters 6 bit ASCII, "@@@@@@@@@@@@@@@@@@@@" = not available = default. |
| Position accuracy | 1 = high (< 10 m; Differential Mode of e.g. DGNSS receiver) 0 = low (> 10 m; Autonomous Mode of e. g. GNSS receiver or of other Electronic Position Fixing Device) ; Default = 0 |
| Longitude | Longitude in 1/10 000 min of position of Aids-to-Navigation (180 degrees, East = positive, West = negative. 181 degrees = not available = default) |
| Latitude | Latitude in 1/10 000 min of Aids-to-Navigation (90 degrees, North = positive, South = negative, 91 degrees = not available = default) |
| Dimension/ Reference for Position | Reference point for reported position; also indicates the dimension of Aids-to-Navigation in metres, if relevant. |
| Type of Electronic Position Fixing Device | 0 = Undefined (default); 1 = GPS, 2 = GLONASS, 3 = Combined GPS/GLONASS, 4 = Loran-C, 5 = Chayka, 6 = Integrated Navigation System, 7 = surveyed 8 - 15 = not used; |
| Time Stamp | UTC second when the report was generated (0 –59),or 60 if time stamp is not available, which should also be the default value, or 61 if positioning system is in manual input mode, or 62 if Electronic Position Fixing System operates in estimated (dead reckoning) mode, or 63 if the positioning system is inoperative. |
| Off-Position Indicator | For floating Aids-to-Navigation, only; 0 = on position; 1 = off position; Note: This flag should only be considered valid by receiving station, if the Aid-to-Navigation is a floating aid, and if Time Stamp is equal to or less than 59. |
| Reserved for regional or local application | Reserved for definition by a competent regional or local authority. Should be set to zero, if not used for any regional or local application. Regional applications should not use zero. |
| RAIM-Flag | RAIM (Receiver Autonomous Integrity Monitoring) flag of Electronic Position Fixing Device; 0 = RAIM not in use = default; 1 = RAIM in use) |
| Pseudo A to N target flag | 0 = default = real A to N at indicated position; 1 = Pseudo/virtual A to N, does not physically exist; may only be transmitted from an AIS-station nearby, under the direction of a competent authority. This flag is not yet included in M.1371-1, but will probably be included in the IALA Recommendation on Technical Clarifications on Rec. ITU-R M.1371-1 which is currently under development within the IALA AIS Committee. |

Note:

The position derived from the internal GNSS receiver can be used in conjunction with a reference position and a ‘guard ring’ to monitor the position of the floating aids to navigation and to generate an ‘off station’ condition.  This would be indicated in the AtoN AIS message and also be transmitted as a navigational warning message in the form of a Safety Related text message.

##### Aids to Navigation Descriptions for AIS Messages

The nature and type of aids to navigation can be indicated with 32 different codes, as shown in Table 4.5.

Table 4.5 Codes proposed for describing the types aids to navigation in AIS messages.

| Aid Type | Code | Definition |
| --- | --- | --- |
|  | 0 | Default, Type of A to N not specified |
|  | 1 | Reference point |
|  | 2 | RACON |
|  | 3 | Off Shore Structure |
|  | 4 | Spare |
| Fixed Aids to Navigation | 5 | Light, without sectors |
|  | 6 | Light, with sectors |
|  | 7 | Leading Light Front |
|  | 8 | Leading Light Rear |
|  | 9 | Beacon, Cardinal N |
|  | 10 | Beacon, Cardinal E |
|  | 11 | Beacon, Cardinal S |
|  | 12 | Beacon, Cardinal W |
|  | 13 | Beacon, Port hand |
|  | 14 | Beacon, Starboard hand |
|  | 15 | Beacon, Preferred channel port hand |
|  | 16 | Beacon, Preferred channel starboard hand |
|  | 17 | Beacon, Isolated danger |
|  | 18 | Beacon, Safe water |
|  | 19 | Beacon, Special mark |
| Floating Aids to Navigation | 20 | Cardinal Mark N |
|  | 21 | Cardinal Mark E |
|  | 22 | Cardinal Mark S |
|  | 23 | Cardinal Mark W |
|  | 24 | Port hand Mark |
|  | 25 | Starboard hand Mark |
|  | 26 | Preferred Channel Port hand |
|  | 27 | Preferred Channel Starboard hand |
|  | 28 | Isolated danger |
|  | 29 | Safe water |
|  | 30 | Special Mark |
|  | 31 | Light Vessel/LANBY |

Note:

- There is potential for confusion when deciding whether an aid is lighted or unlighted. Competent Authorities may wish to use the regional/local section of the message to indicate this.

#### Use of Other Data Items

##### Identification of Aids to Navigation

- Charted name;

- National List of Lights number (if applicable);

- Description of special exhibits.

##### Off-position indicator

This flag should only be considered valid by the receiving station if the aid is a floating aid and the time stamp is less than or equals 59.  A message from an external source is required to set the Off position indicator on the AtoN AIS equipment.

##### Type of position fixing device

For fixed aids, the surveyed position (in WGS 84) is preferred.  In this case, when using a surveyed position, the type of position fixing device shall be set to 7.

All positions shall be given in WGS 84 format.

##### Aids to Navigation Dimensional Properties

- For large AtoN and off shore structures the minimum definable dimension is 1 metre;

- For objects smaller than 1x1 m the values of the fields should be set to:  a = b = 0 and c = d = 1.

##### 8 BITS for Regional/Local Use

The definition of the status of the aids to navigation is left to local or regional authorities.  As an example, they may be indicated as:

- Light (2 bits);

- Normal

- Unreliable;

- Failed;

- Racon (1 bit);

- Normal;

- Failed.

##### Proposed ‘Pseudo AtoN target flag’

In some situations it might be useful to transmit information on a physically non-existing aids to navigation. A method is required to differentiate between a pseudo aids to navigation targets and real aids to navigation objects.  It is proposed that one of the spare bits in Message #21 be used as a ”Pseudo A to N target flag” defined as: [note – bring in line with Recommendation re Virtual and Synthetic]

- 0 = default = real A to N at indicated position;

- 1 = Pseudo/virtual A to N, does not physically exist; may only be transmitted from an AIS station nearby, under the direction of a competent authority.

#### Strategic Applications and Benefits of AIS Technology

From a number of different maritime perspectives (for example VTS, traffic monitoring, regulatory & administrative), the availability of very complete ship voyage information, including cargo details, offers a framework and mechanism for:

- better monitoring of compliance with international regulations including traffic and environment regulations (mandatory routeing and reporting systems, Particularly Sensitive Sea Areas, illegal oil discharges and garbage disposal);

- maritime logistics applications such as fleet management (where are my vessels?); cargo flow management (where is my cargo?) and resource management (port facilities, pilots, tugs etc);

- better control, co-ordination and responsiveness in the event of marine incidents, such as search and rescue (SAR) and pollution. With all ships (and SAR assets, including aircraft) fitted with AIS, local area co-ordination should be simpler and more efficient;

- more accurate and efficient monitoring of the movements of ships, dangerous goods and polluting cargoes;

- greater knowledge in order to facilitate the protection of the marine environment, both coastal and EEZ;

- shore based pilotage;

- shipping information gathered from AIS sources could be channelled into a central hub of what might be a local, national or regional network serving maritime administrations, port authorities, shipping agents, freight handlers, Customs, Immigration, etc.  The dissemination of such gathered information could be a commercially viable service;

- AIS offers a very useful complement to the marine radar because of the enhanced situational awareness offered by the system and the improved detection performance in all weathers and areas of radar shadow and blind sectors. This will also benefit VTS radars, improving automatic identification and tagging, avoiding track swaps and providing course and speed data in real time;

- an AtoN AIS station positioned at a significant geographic point or at a danger to navigation could provide information and data that would serve to complement or replace an aids to navigation, providing identity, state of “health” and other information such as real time tidal height, tidal stream and local weather to surrounding ships or back to the shore authority;

- provide the position of floating aids (primarily buoys) by transmitting an accurate position (perhaps based on the DGNSS corrections) to monitor that they are “on station”;

- provide information for performance monitoring, with the connecting data link serving to remotely control changes of aids to navigation parameters or switching in back-up equipment;

- possible replacement of radar beacons (racons) in the future, providing longer range detection and identification in all weather conditions;

- a data gathering tool, provide very complete information on all AIS fitted shipping traffic passing within VHF range of the site. This data, when manipulated statistically, could be used for detecting the "hot spots" of crossing traffic, even by ship type;

- the early and reliable detection of small vessels in sea clutter and heavy rain is now a reality, if only those small, vulnerable vessels are fitted with the AIS technology.  It is reasonable to assume that, as with an EPIRB, safety regulators and small vessel owners will eventually see the benefits of carrying an AIS station.

#### IALA Role in AIS Standards Development

IALA was the primary organisation to sponsor and co-ordinate the development of the AIS stations.  In 1996, the VTS and Radionavigation Committees of IALA prepared the draft recommendation that, with further refinement within the IMO Sub-Committee on Safety of Navigation (NAV), became the basis for the IMO Performance Standard for AIS.

In October 1997, at the request of several emerging AIS equipment manufacturers, IALA hosted a working group of manufacturers and maritime administrations to agree on a standard technology for AIS.  This group was initially designated the IALA AIS Working Group but has since become the IALA AIS Committee.

The AIS Working Group completed a draft recommendation, that had been prepared by Sweden, on behalf of Finland, Germany, Canada, South Africa, and the United States that was formed the basis of the International Telecommunications Union – Radiocommunications Study Group Recommendation ITU-R M.1371.

#### AIS Standards, RECOMMENDATIONS AND GUIDELINES

- IMO Recommendation on Performance Standards for an Universal Shipborne Automatic Identification System (AIS), (MSC 74(69) Annex 3).

- IMO Guidelines for the onboard operational use of Shipborne Automatic Identification Systems (AIS) (Resolution A.917 (22), amended by Resolution A.956(23)).

- IMO SN/Circ.217 - Interim Guidelines for the presentation and display of AIS target information.

- IMO SN Circ. 227 - Guidelines for Installation of Ship borne AIS.

- IMO Performance Standards for the presentation of navigation-related information on ship-borne navigational displays

- IMO SN/Circ on guidelines for the presentation of navigation-related symbols, terms and abbreviations

- ITU Radio Regulations, Appendix S18, Table of Transmitting Frequencies in the VHF Maritime Mobile Band.

- ITU Recommendation on the Technical Characteristics for a Ship-borne Universal Automatic Identification System (AIS) Using Time Division Multiple Access in the Maritime Mobile Band (ITU-R M.1371-1).

- IEC Standard 61993 Part 2:  Class A Ship-borne equipment of the Automatic Identification System (AIS)  - Operational and Performance requirements, methods of testing and required test results.

- IALA Guidelines on the Automatic Identification System (AIS)

- Volume 1, Part I – Operational Issues

- Volume 1 Part 2 – Technical Issues

- IALA Technical Clarifications on ITU Recommendation ITU-R M.1371-1 Edition 1.4

- IALA Recommendation A-123 on the Provision Of Shore Based Automatic Identification Systems (AIS)

- IALA Recommendation A-124 On AIS Shore Stations And Networking Aspects Related To The AIS Service

- IALA Recommendation A-126 on AIS for Aids to Navigation

#### AIS References

- IMO SOLAS Convention, Chapter V [2002].

- IMO Resolution MSC.140 (76) – Recommendation for the protection of the AIS VHF Data Link

- International Radio Regulations, Appendix S18 (allocation of frequencies in the VHF maritime band for use of AIS) and Recommendation ITU-R M.1084 Annex 4.

## Vessel Traffic Services (VTS)

    - Definition

A VTS, as defined by IMO Resolution A857(20), Guidelines for Vessel Traffic Services, is : “A service implemented by a competent authority, designed to improve safety and efficiency of vessel traffic and to protect the environment. The service should have the capability to interact with the traffic and respond to traffic situations developing in the VTS area.”

Associated operational objectives include:

- minimising the probability of incidents such as collisions and groundings,

- minimising risks to life, the environment and surrounding infrastructure, including the identification and monitoring of vessels carrying hazardous cargoes, and;

optimising the efficient use of vessels, waterways, allied and other related services.

Note: - A Traffic Separation Scheme (TSS), or a Ship Reporting System (SRS) can be an important element of a VTS, but should not be regarded as a VTS in its own right. - The IALA VTS Manual provides a comprehensive reference for the planning, structure and operation of a VTS.

    - Services

In order to achieve these operational objectives the following services may be provided:

- Information Service: provides ships with essential information in time to contribute to on-board navigational decision-making.

- Navigational Assistance Service: is provided to an individual ship when requested, or when deemed necessary.  The VTS provides appropriate advice to assist with on-board navigational decision-making and monitors the navigation of the vessel.

- Traffic Organisation Service: prevents the development of dangerous maritime situations and provides for the safe and efficient navigation of traffic within the VTS area by managing vessel movements.

- Support of Allied Services: a service which disseminates co-ordinated information and data to allied services and adjacent VTS to meet specific needs.

    - VTS System

A VTS collects, evaluates and disseminates selected data.  The process requires:

- VTS operators trained and qualified for their allotted tasks.

- Equipment suitable for the services that are to be provided;

- Operational procedures governing both internal and external interactions and data handling;

# OTHER FACILITIES

## Pilotage

    - Pilotage as an Aid to Navigation

Pilotage is a specialised, and usually, licensed service that may be applied to navigation in “restricted” waters.  The skill draws on a local knowledge of the relative positions of geographic points or aids to navigation, submerged features, tides, currents and climatic conditions.

Pilotage may be required in coastal waters, estuarial waters, rivers, ports, harbours, lakes, canals, or enclosed dock systems or any combination of these areas.

When a pilot boards a vessel, it is customary for the pilot to be given "conduct of the vessel", but not “command”.  The role of the pilot is to:

- act as an adviser to the master, but this often includes:

  - giving the necessary instructions to the ships personnel operating equipment essential to the safe navigation and manoeuvring of the vessel;

  - assisting with local communication to a VTS centre, port control and other vessels;

  - communicating instructions to tugs and linesmen if berthing or sailing;

- provide current and specialist knowledge of;

  - local conditions and traffic;

  - operational status of aids to navigation;

  - sailing directions;

  - restrictions applicable to the piloted vessel;

In addition the pilot needs to be able to adapt quickly to:

- operational culture aboard the vessel;

- the handling characteristics of the vessel and;

- state of navigation equipment aboard (and able to compensate for deficiencies).

    - Types of Pilotage

Pilotage is commonly applied within declared ports but may also be applied to some coastal areas, lakes and inland waterways.  These areas would normally fall within the definition of “restricted waters” (see Section xxxxx).

Where pilotage services are licensed, it is usual for the applicable pilotage area to be stated on the licence.  The service provider may then be described as a port pilot or a coastal pilot etc.

Various levels of enforcement can be applied to a pilotage area:

- Compulsory (Mandatory) pilotage:- Applicable vessels must take a pilot when entering a declared area.

  - A country may seek IMO approval for an area to be declared a Particularly Sensitive Sea Area (PSSA).  If approved, the declaration allows special marine environmental protection measures to be applied to shipping. This can include compulsory pilotage arrangements. (IMO Resolution A710 (17) is an example)

- Recommended pilotage:- An authority can promulgate notices recommending that masters of applicable vessels, who are unfamiliar with a particular area, should engage a licensed pilot.

    - Other Pilotage Considerations

- Pilot Services can be provided by public or private operators.  However generally the pilot licensing authority would be a government agency.

- The IMO has set the minimum standards for pilots and includes recommendations on the qualification and training of pilots.  However individual countries may impose more stringent standards.

- When developing proposals for marking restricted waterways, the requirements for pilotage services should be considered concurrently with the selection of the aids to navigation.

    - Routeing

#### IMO Ship’s Routeing

The General Provisions on Ships' Routeing are established by the SOLAS Convention Chapter V, Regulation 8.  The objectives and definitions of the General Provisions are reproduced below:

##### Objectives

The purpose of ships' routeing is to improve the safety of navigation in converging areas and in areas where the density of traffic is great or where freedom of movement of shipping is inhibited by restricted sea-room, the existence of obstructions to navigation, limited depths or unfavourable meteorological conditions.  Ships' routeing may also be used for the purpose of preventing or reducing the risk of pollution or other damage to the marine environment caused by ships colliding or grounding in or near environmentally sensitive areas.

The precise objectives of any routeing system will depend upon the particular hazardous circumstances which it is intended to alleviate, but may include some or all of the following:

- the separation of opposing streams of traffic so as to reduce the incidence of head-on encounters;

- the reduction of dangers of collision between crossing traffic and shipping in established traffic lanes;

- the simplification of the patterns of traffic flow in converging areas;

- the organization of safe traffic flow in areas of concentrated offshore exploration or exploitation;

- the organization of traffic flow in or around areas where navigation by all ships or by certain classes of ship is dangerous or undesirable;

- the organization of safe traffic flow in or around or at a safe distance from environmentally sensitive areas;

- the reduction of risk of grounding by providing special guidance to vessels in areas where water depths are uncertain or critical, and;

- the guidance of traffic clear of fishing grounds or the organization of traffic through fishing grounds.

#### Definitions

The following terms are used in connection with matters related to ships' routeing:

Routeing system:- Any system of one or more routes or routeing measures aimed at reducing the risk of casualties, it includes traffic separation schemes, two-way routes, recommended tracks, areas to be avoided, inshore traffic zones, roundabouts, precautionary areas and deep-water routes.

Mandatory routeing system:- A routeing system adopted by the Organization, in accordance with the requirements of regulation V/8 of the International Convention for the Safety of Life at Sea 1974, for mandatory use by all ships, certain categories of ships or ship carrying certain cargoes.

Traffic separation scheme:- A routeing measure aimed at the separation of opposing streams of traffic by appropriate means and by the establishment of traffic lanes.

Separation zone or line:- A zone or line separating the traffic lanes in which ships are proceeding in opposite or nearly opposite direction or separating a traffic lane from the adjacent sea area; or separating traffic lanes designated for particular classes of ship proceeding in the same direction.

Traffic lane:- An area within defined limits in which one-way traffic is established. Natural obstacles, including those forming separation zones, may constitute a boundary.

Roundabout:- A routeing measure comprising a separation point or circular separation zone and a circular traffic lane within defined limits. Traffic within the roundabout is separated by moving in a counter clockwise direction around the separation point or zone.

Inshore traffic zone:- A routeing measure comprising a designated area between the landward boundary of a traffic separation scheme and the adjacent coast, to be used in accordance with the provisions of rule 10(d), as amended, of the International Regulations for Preventing Collisions at Sea, 1972 (Collision Regulations).

Two-way route:- A route within defined limits inside which two-way traffic is established, aimed at providing safe passage of ships through waters where navigation is difficult or dangerous.

Recommended route:- A route of undefined width, for the convenience of ships in transit, which is often marked by centreline buoys.

Recommended track:- A route which has been specially examined to ensure so far as possible that it is free of dangers and along which ships are advised to navigate.

Deep-water route:- A route within defined limits which have been accurately surveyed for clearance of sea bottom and submerged obstacles as indicated on the chart.

Precautionary area:- A routeing measure comprising an area within defined limits where ships must navigate with particular caution and within which the direction of traffic flow may be recommended.

Area to be avoided:- A routeing measure comprising an area within defined limits in which either navigation is particularly hazardous or it is exceptionally important to avoid casualties and which should be avoided by all ships, or certain classes of ship.

Established direction of traffic flow:- A traffic flow pattern indicating the directional movement of traffic as established within a traffic separation scheme.

Recommended direction of traffic flow:- A traffic flow pattern indicating a recommended directional movement of traffic where it is impractical or unnecessary to adopt an established direction of traffic flow.

#### Approach Channels

An approach channel is defined as any stretch of waterway linking the open sea to the berths within a port.  It is convenient to analyse the functional requirements of the design in a number of parts.  For example:

- the open water component or outer channel, and;

- the inner channel component which lies in relatively sheltered waters.

The design process requires inputs from a number of disciplines, including:

- ship handling;

- vessel size and behaviour;

- human factors in ship handling;

- maritime engineering;

- aids to navigation;

- the physical environment.

The joint PIANC-IAPH Working Group II-30 in cooperation with IMPA and IALA published a document “Approach Channels - A Guide for Design” June 1997

#### Vessel Manoeuvring Considerations

If a waterway is defined as a series of straights and turns sections, the passage of a vessel along the waterway can be described by a number of navigational phases that are illustrated in Fig 9.1.  These comprise:

- turning;

- recovery, and;

- track keeping.

The type of manoeuvre within a section determines the information that the navigator requires from the aids to navigation.

Fig 9.1 Vessel manoeuvring phases.

#### Real-Time Simulation

##### (Ship’s) Bridge Simulators

A number of countries have bridge simulators for training deck officers.  These facilities may be available for hire to an aids to navigation authority to evaluate proposed changes to routes and marking arrangements.

A computer program can be generated for the simulator to model the shoreline features and depth soundings for a waterway.  Additional program layers can be added to:

- show the proposed marking arrangements;

- simulate local tide and currents;

- simulate the characteristics of the vessel “under command”;

- introduce traffic and navigational situations.

The use of a simulator can be of real benefit in confirming the effectiveness of marking proposals that will have a high cost or that are intended to meet the needs of a complex navigational situation.

## Sound Signals

Although Sound Signals still exist, it has been IALA policy since 1985 that these devices should only be used as a hazard warning.

It is a matter for the appropriate authority to determine whether a hazard requires a sound signal and the amount of reduced visibility per year that justifies its installation (ex: 10 days of visibility under 1NM/year). Certain man-made obstructions such as offshore structures, bridges and breakwaters are examples of applications for sound signals but they can also be installed on specially configured landfall areas, islands or groups of islands close to navigation routes.

Refer to IALA documents: - Recommendations for the calculation of the range of a sound signal (E-109), May 1998; - Recommendations for the marking of fixed bridges over navigable waters (O-113), May 1998; - Recommendations for the marking of offshore structures (O-114), May 1998.

    - Considerations on Sound Signals and their use

- IALA recommends on O-114 that, on offshore structures, the sound signals shall be operated when the meteorological visibility is two miles or less.

- Sound propagates in the atmosphere in a very capricious way, varying the perception of direction and distance to the emitter and making it only useful for the indication of the existence of a danger but never its location.

- A linear increase in the perception of a sound corresponds to a exponential increase on the sound source.

- Background noise level on board vessels may preclude the listening of a sound signal.

- Occasionally there are places close to a sound signal where it is almost not audible

- The identification of the rhythm of a sound signal is not reliable because fluctuations in the propagation may cause interruption of reception. To mitigate this problem, IMO defines, in the International Regulations for Preventing Collisions at Sea, a short blast as a blast of about one second duration and a prolonged flash as a blast of from four to six seconds duration. IALA recommends on O-114 that the minimum duration of the short blast of the Morse Code letter signalling Offshore Structures shall be 0.75 seconds.

- A sound signal provokes incommodity to populations leaving close to them.

- In some situations, there is the need to combine two or more sound sources or to install a device to avoid the propagation of sound in a certain direction. In both cases, care must be taken to the risk of annulation of the sound of one source by the sound of the other or by the reflected sound on the referred device which should be sound absorbent.

- Sound signals are normally operated automatically using a fog detector.

    - Considerations on Fog Detectors:

Until 30 years ago, SOUND SIGNAL were operated by lighthouse keepers watching the lights that on sight of fog, turned on the sound signal. Actually, automatic fog detectors that emit an infrared beam to the atmosphere and measure the reflection on the meteors existing on the air, being calibrated to activate the sound signal on a certain visibility, are commonly used.

Fog detectors existing on the market are expensive, with low resistance to the marine environment and not redundant. They are easily deceived by the salt and spray of swell, being unreliable. It is common to have situations when a fog detector activates a sound signal on clear visibility or does not activate it on poor visibility.

    - Sound Signals in the World:

Some countries, including Finland, Iceland and Norway have abolished the use sound signal.

The study of  the nautical charts and lists of lights of the Portuguese, Spanish, French and English coasts of the Atlantic shows that, apart from bridges and offshore structures, sound signals are normally only used on natural dangers and some breakwaters projecting into the sea.

    - Range of a sound signal

In accordance with IALA recommendation E-109 on the calculation of the range of a sound signal, May 1998, there are practical reasons that justify the use of two range definitions:

Nominal Range: the distance at which, in foggy weather, a lookout positioned in the wing of the bridge has a probability of 90% of hearing the signal when subjected to a noise as defined by IALA as being equal to or in excess of that found in 84% of large merchant vessels, the propagation between the sound signal emitter and the listener occurring during relatively calm weather and with no intervening obstacles.

| Sound Pressure Level |  |  |  |  |
| --- | --- | --- | --- | --- |
| Pn(nm) f(Hz) | 0.5 | 1 | 1,5 | 2 |
| 25 | 162 | 172 | 176 | 178 |
| 50 | 149 | 161 | 165 | 168 |
| 100 | 138 | 150 | 154 | 157 |
| 200 | 130 | 142 | 147 | 150 |
| 400 | 122 | 135 | 140 | 144 |
| 800 | 115 | 130 | 137 | 142 |
| 1000 | 113 | 129 | 137 | 144 |
| 1250 | 112 | 129 | 138 | 146 |
| 1600 | 110 | 130 | 140 | 150 |
| 2000 | 109 | 132 | 145 | 156 |
| 2500 | 108 | 136 | 151 | 166 |
| 3150 | 107 | 141 | 160 | 179 |
| 4000 | 109 | 150 | 177 | 199 |

              - Pn	– Nominal Range in sea miles

              - f	– Frequency of the sound in Hz

              - Nr 	– Sound Pressure Level, in decibels, of the sound emitted by the SOUND SIGNAL at the reference distance of 1 meter in the direction concerned.

Usual Range: the distance at which, in foggy weather, a lookout positioned in the wing of the bridge has a probability of 50% of hearing the signal when subjected to a noise as defined by IALA as being equal to or in excess of that found in 50% of large merchant vessels, the propagation between the sound signal emitter and the listener occurring during relatively calm weather and with no intervening obstacles.

| Sound Pressure Level |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pu(nm) f(Hz) |  | 0.5 |  | 1 |  | 1,5 |  | 2 |  |
| 25 | 155 |  | 162 |  | 165 |  | 168 |  |  |
| 50 | 144 |  | 150 |  | 154 |  | 157 |  |  |
| 100 | 132 |  | 139 |  | 143 |  | 146 |  |  |
| 200 | 125 |  | 132 |  | 136 |  | 140 |  |  |
| 400 | 117 |  | 125 |  | 130 |  | 135 |  |  |
| 800 | 112 |  | 121 |  | 128 |  | 134 |  |  |
| 1000 | 110 |  | 121 |  | 128 |  | 135 |  |  |
| 1250 | 109 |  | 121 |  | 129 |  | 137 |  |  |
| 1600 | 109 |  | 122 |  | 132 |  | 141 |  |  |
| 2000 | 108 |  | 123 |  | 136 |  | 148 |  |  |
| 2500 | 109 |  | 127 |  | 142 |  | 157 |  |  |
| 3150 | 110 |  | 132 |  | 152 |  | 170 |  |  |
| 4000 | 112 |  | 142 |  | 168 |  | 193 |  |  |

              - Pu	– Usual Range in sea miles

              - f	– Frequency of the sound in Hz

              - Nr 	– Sound Pressure Level, in decibels, of the sound emitted by the SOUND SIGNAL at the reference distance of 1 meter in the direction concerned.

## Nautical Publications

    - Navigational Warnings

SOLAS Chapter V Regulation 13 requires for contracting governments to provide navigational information to mariners.  The Regulation 13 states:

Contracting Governments undertake to arrange for information relating to aids to navigation to be made available to all concerned. Changes in the transmissions of position-fixing systems which could adversely affect the performance of receivers fitted in ships shall be avoided as far as possible and only be effected after timely and adequate notice has been promulgated.

This information falls into three basic categories:

- information about planned changes, such as:

  - dredging, surveying, pipe and cable laying;

  - changes to an existing aid or the establishment of new aids to navigation;

  - changes to traffic arrangements;

  - commercial maritime activities;

  - short term events (naval exercises, yacht races, etc.).

- information about navigational un-planned events, such as:

  - the failure to aids to navigation;

  - marine incidents (groundings, collisions, wrecks etc.) ;

  - search and rescue activities.

- new information arising from survey work or previously undiscovered hazards.

    - World-Wide Navigational Warning Service

The promulgation of information on navigational safety is coordinated by means of the World-Wide Navigational Warning Service that was established jointly by the IMO and the IHO in 1977.

The World-Wide Navigational Warning Service is administered through 16 NAVAREAS, as is shown in Fig 6.5.  Each NAVAREA has an Area Coordinator who is responsible for collecting information, analysing it, and transmitting NAVAREA Warnings.

Figure 6.5

    - Lists of Aids to Navigation

#### List of Lights and List of Radio Signals

Lists of aids to navigation are produced by (or for) most national authorities as part of the navigational information made available to mariners in support of SOLAS Chapter V Regulation 13.  They provide details of:

- name;

- location;

- the characteristics of the aids;

- operating schedule.

These lists will not always include buoys and unlit aids to navigation.

#### Standard Descriptions

Joint IMO/IHO/WMO Manual on Maritime Safety Information (IMO COMSAR/Circ 15).

This document provides definitions of standard terms to describe particular events that should be used when composing Navigational Warnings.  Some of the terms that are relevant to the condition of aids to navigation have been defined as follows:

Sample of the COMSAR/Circ 15 standard terms.

| Descriptors for Lighthouses, Beacons Buoys and Lightvessels | Comments |
| --- | --- |
| UNLIT | Incorrect Terms include: Out, Extinguished, Not Burning, Not Working |
| LIGHT UNRELIABLE | Incorrect Terms include: Weak, Dim, Low Power, Fixed, Flashing incorrectly, Out of Character |
| DAMAGED | Use only for major damage where there is a significant loss of functionality |
| DESTROYED | Incorrect Terms include: Temporarily destroyed |
| ESTABLISHED (+ location) | New light |
| OFF STATION | Buoy (lightvessel) not in the charted position |
| MISSING | Buoy (lightvessel) completely absent from position |
| RE-ESTABLISHED | Only appropriate for lights that have previously been Charted or listed as Destroyed |

The above list of terms and definitions do not adequately cover all of the situations that an Authority might want to use when issuing a Navigation Warning.  An expanded set of definitions of terms for use in Navigation Warnings is provided for consideration in the following table.

A suggested expanded list of terms and definitions for use in Navigation Warnings

| Term | Definition |
| --- | --- |
| Station | The authorised and exact location of an aid to navigation. |
| Established in position | Any type of aid placed in operation for the first time at a given station. |
| Re-established in position | Any type of aid placed in operation at a station at which a similar type of aid with identical characteristics had been previously established, but subsequently destroyed, withdrawn or discontinued. |
| Unlit | When a light is out because of defective equipment, or any unintentional or deliberate occurrence and it is intended to restore it to normal as soon as practicable. |
| Unreliable | When an aid of any type is not exhibiting its correct characteristics and it is intended to restore it to normal as soon as practicable. |
| Reduced power | When an aid of any type is not operating at its correct power, but is exhibiting the correct characteristics and it is intended to restore it to normal replace it as soon as practicable. |
| Off station | When a floating aid is adrift, missing or out of position and it is intended to replace it as soon as practicable. |
| Altered | When the characteristics or structure of any aid have been altered, without changing the type of aid or its station. |
| Altered in position | When a change is made to the station of an aid (ie its location) without changing the type of aid, character or type of structure. |
| Destroyed | Any type of aid that has been damaged to the extent that it is no longer of use as an aid to navigation, but remnants of the structure may remain. |
| Restored to normal | Any type of aid that has been previously described as unlit, unreliable, reduced power or temporarily discontinued and has now been serviced so as to exhibit its correct characteristics and power. |
| Replaced in position | When a floating aid previously described as off station or temporarily discontinued is returned to station or replaced by another with the same characteristics. |
| Temporarily replaced by | When any aid is discontinued, temporarily withdrawn or off station and another aid of different type or characteristics is immediately established at the same station. |
| Temporarily withdrawn | When a floating aid has been entirely removed from its station and no similar aid is left in its place, but it is intended to re-establish the aid in the near future. |
| Temporarily discontinued | When a sound signal or radionavigation service is silent because of maintenance requirements, or any unintentional or deliberate occurrence, and it is intended to restore it to normal as soon as practicable. |
| Permanently withdrawn | When a floating aid has been entirely removed from its station with no similar aid is left in its place and it is not intended to re-establish that aid in the near future. |
| Permanently discontinued | When any aid, other than a floating aid, is removed from a station or the service is terminated or silenced because it is no longer required. |

#### Positions and Bearings

##### Positions

The Joint IMO/IHO/WMO Manual on Maritime Safety Information (IMO COMSAR/Circ 15) states that positions should always be given in Degrees, Minutes and decimal minutes in the form:

- DD-MM.mmm N or S;

- DDD-MM.mmm E or W;

- leading zeros should always be included;

- the same level of accuracy should be quoted for both Latitude and Longitude.

##### Recording of Aids to Navigation Positions

IALA has produced a Recommendation for the recording of aids to navigation positions (O118), June 2000.

The Recommendation states that:

- where an Authority has operational DGPS stations, a program should be implemented to determine the WGS84 positions of each aid to navigation (fixed and floating) within the coverage area, and for this information to be passed to the national hydrographic authority for future use.  It is anticipated that the information would assist the hydrographic authority in checking the accuracy of charts, planning future survey requirements and for updating List of Lights.

- in the case of lighted fixed aids to navigation the WGS84 position should be measured close to the focal centre of the light so that the WGS84 elevation is also determined.  Alternatively, several positions around the optic or lantern house could be measured and a central position computed.

- in the case of unlighted fixed aids to navigation the WGS84 position should be the base of the structure.

- in the case of floating aids to navigation the WGS84 position should be the position of the anchor.

- each position should be recorded to three decimal places of a minute and include the time, date and details of the measuring equipment.

  - where an Authority has to refer to charts of different datums, positions are communicated with the appropriate datum reference.

  - for example 51 04.372’N, 100 26.794’E (WGS 84).

##### Bearings

Bearings, directions of leading (range) lines and limits of sectors should always be stated in terms of the bearings that would be seen by the mariner.  Observing a practice of communicating bearings with the suffix  ‘TBS” or True Bearing from Seaward will minimise the risk of confusion.

    - Maritime Safety Information

Within a NAVAREA, there can be a hierarchy of Warnings promulgated by the National co-ordinator.  Collectively referred to as Maritime Safety Information (MSI).  The Warning hierarchy covers:

- NAVAREA Warnings:- that are concerned with information that ocean-going vessels require for safe navigation;

  - are transmitted in English and, where appropriate, in other languages;

  - are promulgated by;

  - radiotelephony;

  - Digital Selective Calling (DSC);

  - Enhanced Group Calling (EGC);

- NAVTEX  (used for the automatic broadcast of localised Maritime Safety Information (MSI) using radio telex);

- covers the specific NAVAREA and portions of adjacent areas;

- details of the broadcast schedules are shown in the List of Radio Signals published by Hydrographic Offices and in the publications of the ITU;

- are generally promulgated for a sufficient period of time to ensure its safe reception after which it is cancelled or published in a Notice to Mariners;

- Coastal Warnings:- that are concerned with information relating to a regional area covering 100-200 nautical miles from the coast.  These are:

  - transmitted from a national network of coastal radio stations;

  - broadcast at scheduled times;

  - use English and the national language;

- Local Warnings:- that cover the area within the limits of a harbour or port authority:

  - to supplement Coastal Warnings, and;

  - may be limited to the national language.

Information concerning navigational warnings can be obtained from the Joint IMO/IHO/WMO Manual on Maritime Safety Information (MSI), February, 1998.

- Off-Station Warnings for Major Floating Aids

IALA Recommendation for off-station signals for major floating aids (O -104), November 1989 states that: - When any Lightvessel, Lightfloat or Lanby (LNB) manned or unmanned is out of position such that it could be misleading to navigation, all its aids to navigation (lights, sound signals, racon, radio beacon) should be discontinued. - To avoid the risk of collision with passing vessels, warning lights should be continuously displayed as follows: - Two all-round red lights in a vertical line similar to those prescribed by Rule 27 of the COLREGS for a vessel "Not under command". - If the appropriate Administration requires a sound signal to be operated, it should be coded MORSE "D" as prescribed by Rule 35 of the COLREGS for a vessel "Not under Command" - If a Racon is deployed, it should be coded MORSE "D".

## Electronic Chart Display and Information System (ECDIS)

    - Description

Although ECDIS is not an “aid to navigation” as defined by IALA, it deserves to be mentioned because this piece of ship borne equipment is likely to bring major changes to the ways that vessels are navigated.  ECDIS uses digital vector data in a way that will allow it replacing the traditional paper charts with a more versatile electronic product that can draw on a variety of positioning and data inputs, such as GPS, DGPS, AIS, radar, echo sounder, compass, an electronic chart, navigational publications, the chart amendments and tidal and meteorological information.

    - Performance Standards

The performance standards for ECDIS have been defined by the International Maritime Organization (IMO), in conjunction with the International Hydrographic Organization (IHO).

IMO Resolution A.817(19) [as amended  by Resolution MSC.64(67) and by Resolution MSC.86(70)] enables marine administrations to accept ECDIS as a legal equivalent to the paper charts that are required to be carried for compliance with SOLAS regulation V/27.

There are two key performance elements to ECDIS:

- An approved processing system (or ‘box’), and;

- Electronic Navigational Charts (ENCs) that have been prescribed by a national hydrographic office and meet the standards set down in the 3rd Edition of the IHO Special publication No.57 (S57)

While an ECDIS ‘box’ may be capable of reading other forms of electronic charts, it ceases to be a compliant system without the official ENCs.

Non-S57 electronic charts include:

- Raster Navigation Charts (RNC) that are effectively electronic copies of paper charts, and;

- Electronic charts that are not issued by a national hydrographic authority or differ from the S57 standard.

When the relevant chart information is not available in the appropriate form (issued by a national hydrographic office and meeting the S57 standards) some ECDIS equipment may operate in the Raster Chart Display System mode, but in that case the ECDIS equipment should be used together with an appropriate portfolio of up-to-date paper charts [Resolution MSC.86(70)].

## Tide Gauges and Current Meters

    - Purpose

A number of countries operate tide gauges and current meters to assist the prediction of tidal heights and streams or for the broadcast of real-time information to shipping.  The latter is generally used to overcome the sometimes considerable differences between actual tide heights and predicted values due to meteorological and mean sea level fluctuations.

    - Intergovernmental Oceanographic Commission

The Intergovernmental Oceanographic Commission (IOC) is responsible for co-ordinating the Global Sea Level Observing System (GLOSS) program to establish global and regional networks of sea level stations for providing essential information for international oceanographic research programmes, including those dedicated to the study of aspects of climate change, and operational oceanography.  The main component of GLOSS is a network of 287 sea level stations worldwide (referred to as the 'Global Core Network').

IALA supports and encourages participation in the GLOSS program.

Authorities that are procurement or upgrading of Sea Level Measurement devices, are encouraged to consider using equipment that can support the requirements of the GLOSS program.  Typically this calls for gauges capable of measuring to centimetre (1 cm.) accuracy in all weather (especially wave) conditions and for the free exchange of hourly sea level data to an International Sea Level Centre.

Information on the GLOSS Programme can be found at: http://www.pol.ac.uk/psmsl/programmes/gloss.info.html.

Technical recommendations on sea level observations can be found at: http://www.pol.ac.uk/psmsl/manuals/.

# Power Supplies

## Types

A wide range of power systems and energy sources have been used or contemplated for operating lighthouses and floating aids.  Everything from clockwork to radio-active isotopes have been used.  Some of the more common types are listed in Table 7.1.

Table 7.1 Power sources for operating lighted aids to navigation.

| Electric Power Systems | Non-Electric Energy Sources |
| --- | --- |
| - Primary cells | - Acetylene |
| - Mains (utility) electricity supply | - Propane |
| - Diesel and Petrol engine driven generators | - Butane |
| - Photovoltaic Solar modules |  |
| - Wind generators |  |
| - Wave activated generators |  |

    - IALA Survey Data on Power Supplies

#### IALA Annual Questionnaire

IALA will carry out a questionnaire survey of all IALA Members annually

There is a general trend away from gas, using mains utility electricity where available and photovoltaic solar power where mains is not available.

## Non-Electric

    - Acetylene

Acetylene (C2H2) has been used to operate lights on buoys and unattended aids to navigation for many years.  Acetylene can explode if compressed directly, but can be safely contained under low pressure in special cylinders when dissolved in acetone.

The manufacture of acetylene, standards for the cylinders and the filling process are usually controlled by government regulations.

Acetylene has been a convenient and reliable energy source for aids to navigation. However appropriate attention should be given to:

- safe handling of cylinders;

- the broad range of explosive mixtures with air (between 3 and 82% acetylene).

- the purity off the gas, and;

- minimizing leaks in pipe work and fittings;

    - Propane

Propane gas (CH3 CH2 CH3) has been used as an alternative fuel to acetylene, particularly in buoys.  Although propane has to be consumed in an incandescent mantle burner to provide a white light, it has several advantages over acetylene:

- it is a by-product in oil refining processes;

- its abundance and low cost;

- propane liquefies at a pressure of 6 atmospheres at 17ºC., and can be transported in quite light and low cost gas bottles;

- propane will maintain a positive pressure down to -40ºC and will not freeze in conditions likely to be encountered at sea;

- placing the bottles in pockets in the buoy or by filling it directly into the body of an buoy, or pressure vessel;

- the comparable containers are the 20 kg propane bottle with gross weight of 48 kg and the 7,000 litre acetylene cylinder, weighing 105 kg;

- furthermore the cost of the propane bottle is only about one-third of that of a acetylene cylinder;

- propane is a particularly safe gas, as only some 6 % of all its possible mixtures with air are explosive against a figure of 80% for acetylene;

- burns cleanly without the risk of sooting that can occur with a poorly adjusted acetylene burner.

Refer to IALA publication: - Practical Notes for the safe handling of gases, October 1993. Refer also to national standards for the safe handling of gas.

## Electric - Non-Renewable Sources

    - Primary Cells

Primary cells provide electrical energy by a non-reversible chemical process.  They were used in large numbers up until the 1980s to operate buoys and automatic beacon lights.

#### Zinc-Air Cell

The zinc-air primary cell was a common energy source for operating buoy and beacon applications.  The cell uses a porous carbon block to supply oxygen from the air through an alkaline electrolyte to oxidize a zinc anode.  Individual primary cells have an open circuit voltage of about 1.2 volts and can supply 1000 to 2000 Ah at a maximum rate of about 1 ampere.

#### Lithium-Thionyl Chloride Cell

Another type of primary cell in use in buoy applications is the lithium-thionyl chloride cell.  This has a higher energy density and a longer shelf life than the zinc-air cell.

The usage of primary cells has declined sharply since commercial solar power (photovoltaic) modules have become available.  A related issue that hastened the decline of primary cells was the tightening environmental standards in a number of countries that required cells to be recovered from site for disposal in an approved manner.  Disposal compliance costs, and occupational health and safety aspects of the frequent change-out of primary cells have worked in favour of converting to renewable energy sources (eg. solar, wind and wave generators).

#### Sea-Water Cells

The sea-water cell developed for buoy applications in Norway is a primary cell that uses a magnesium anode and a largely inert copper cathode.  The sea water acts both as an electrolyte and the provider of dissolved oxygen for the cathode.

A single cell is installed as part of the buoy tail-tube.  The motion of the buoy has a beneficial effect in agitating the water to provide an oxygen-rich flow through the cell and carry away the reaction products.

Copper was selected for the cathode material because of its inherent antifouling properties.  A magnesium anode was considered environmentally acceptable because it is a naturally occurring element of sea water.  The cell produces a voltage of 0.8 to 1 volt under load.  Components of the cells under evaluated have been sized to provide around 35,000 watt hours of energy.

Since it is impractical to use more than one cell due to the current leakage that would occur, a dc-dc converter is used to raise the voltage to the level required by the load.

    - Internal Combustion Engine/Generators

#### Diesel Generators

Diesel engine driven generators are often used as the primary source of electrical power where the location of a lighthouse is too remote to be supplied from a mains electricity grid.  Diesel generators are also used to provide emergency or backup power.

The generator capacity to support the operational and domestic loads of a lighthouse would be in the range of 10 to 30 kW.  Diesel generators of this size could be expected to consume around 0.4 litres/kWh.

The requirement for diesel generators in lighthouses is decreasing as a result of:

- lighthouse automation (destaffing), and because;

- new beacon and lamp technology that enables a light with a nominal range 18 to 20 nautical mile to be operated from a renewable energy source.

#### Petrol-engine Generators

Petrol-engine generators are a useful source of power for maintenance work, but are less common in permanent installations due to:

- fuel and fuel storage safety issues;

- maintenance requirements on the spark-ignition system, and;

- the petrol engine generally being regarded as less durable than a diesel.

    - Other

Some of the less common power generating equipment considered for aids to navigation applications are mentioned below.

#### Thermo-electric generator

- This is a solid-state generator in which a heat source, commonly from a propane burner, is directed onto a thermopile (ie. an array of thermocouple type elements).  Since each thermocouple only produces a voltage of around 0.5 volts, a number are connected in series;

- This type of generator has a low thermal efficiency (around 5%) and is rarely used.

#### Stirling engine driven generator

- The Stirling engine is an external combustion engine that can be operated on gas or diesel fuel.  Packaged generator sets are available that could operate a lighthouse.  Typical generators produce 1kW electrical output and 5kW of heat.  The heat output could be a useful by-product for building conditioning in a lighthouse.

#### Fuel cell

- This is a solid-state device that uses a catalytic process to oxidise fuel to generate an electrical current. A common fuel is hydrogen, or hydrogen rich fuels.  It can be thought of as a continuously fed battery.

- The commercial fuel cell is still an emerging technology and at this stage is an expensive power source.  Aids to navigation applications are likely to be limited to situations where solar energy (photovoltaic) is impractical due to limited insolation or icing conditions.

## Electric - Renewable Energy Sources

    - Solar Power (Photovoltaic cell)

#### Aids to Navigation Applications

Solar power is an ideal power source for many aids to navigation applications.  It offers:

- a power source with no moving parts;

- no maintenance requirements other than being cleaned;

- negligible deterioration in power output over its life;

- low life-cycle costs;

When used to power a light, the battery recharging process is separated from the operation of the light so that the recharge voltage can be optimized without detriment to lamp life.

The potential difficulties associated with solar power are:

- finding ways to minimise bird fouling;

  - mounting solar modules vertically is probably the best long-term solution for buoys;

- sizing arrays to operate at high latitudes;

- protecting solar modules from;

  - wave damage on buoys;

  - vandalism and theft

  - lightning.

Aids to navigation exposed to icing conditions are perhaps the only applications unsuited to the use of solar modules.

Refer IALA publications: - Specification for solar voltaic systems, June 1988; - Practical note on the application of solar power systems for aids to navigation, September 1993; - Guidelines for renewable energy sources for marine aids to navigation – solar photovoltaic systems, October 1997; IALA is studying the feasibility of developing an IALA Computer program for sizing solar power supplies.

#### Types

There are three common technologies employed in the manufacture of silicon based, solar modules and are listed in Table 7.3 :

Table 7.3 Silicon solar cell technology.

| Type (Technology) | Comments |
| --- | --- |
| Monocrystaline cells | Are made from a thin slice cut from a single large crystal of silicon, usually produced as a circular section rod. Generally have the highest efficiency of the three technologies, If circular wafers of silicon are used the module fill factor is significantly less than with polycrystalline cells. It is now usual for the cells to be trimmed to approximate a square. |
| Polycrystaline cells | Are made from a thin slice cut from a large cast billet of silicon comprising many crystals Are slightly less efficient than the mono crystalline cell but they can be shaped to completely fill the module. |
| Amorphous module | Are made by depositing thin films of silicon directly onto a glass or stainless steel substrate a thin slice cut from a single large crystal of silicon. The cell has a lower efficiency than either of other technologies but can be multi-layered for enhanced performance. Problems have been found with lifetime of these cells. |

In addition to the silicon cell technologies, there are two optional module configurations based on the numbers of series connected cells.  The standard module normally has 36 cells in series to give an open circuit voltage of around 20 volts.  For 12 volt battery charging applications, a voltage (charge) regulator is considered essential.

A self-regulating solar module was promoted as a means of eliminating the voltage regulator that had frequently been found to be the least reliable component in a solar power supply.  The self-regulating module typically has 32 cells in series to give an open circuit voltage of around 18 volts (and a maximum on load voltage of around 15 volts). In a self-regulating solar module, the charging rate is determined by the interaction between the electrical characteristics of the battery and solar module.

- Both concepts can be made to work well.

#### Module or Array Orientation

In the northern hemisphere, solar modules are normally installed facing south and inclined at an angle to the horizontal that is related to the latitude of the site.

The inclination angle for solar modules is often optimised for the particular site as part of the sizing calculations

One of the main problems experienced with solar powered aids to navigation has been bird fouling.  Numerous ingenious solutions have been trialed, generally with mixed results.  Generally solar modules mounted at an angle or vertically benefit from self washing from rain.

The cost of additional solar modules needed for a vertical installation may be largely off-set by the savings that result from simplifying the mounting arrangements or framework.

    - Wind Energy

#### Aids to Navigation Applications

Wind generators (or wind turbines) have been used by a number of IALA Members to power aids to navigation.  The most popular type were horizontal axis machines with a two bladed (propeller type) turbine. The maintenance requirements arising from the moving parts design of wind generators and susceptibility to storm damage has limited the use of wind generators.

Refer IALA publication: - Specification for medium size wind generator systems, June 1988.

#### Installations

Wind generator installations at aids to navigation sites pose a number of problems:

- wind generators tend to require a lot of maintenance if operated in turbulent air flows:

- if the wind generator is installed on a separate mast some distance from the lighthouse, consideration has to be given to cable voltage drop:

- the size of wind generator likely to be considered for operating aids to navigation is at considerable risk of damage if there are populations of birds at the site.

#### Wind Generator Types

A comparison of the typical performance of different types of wind generators is shown in Fig 7.2.

Fig 7.2

    - Wave Energy

#### Wave Activated Generator

The wave activated generator (WAG) was developed in Japan and has been successfully used to power lighted buoys.  The interaction between the buoy and wave motions acts as a simple air pump that is used to drive an air turbine and generator.  The WAG is mounted on an extension of a hollow tail tube that passes through the buoy hull.  With wave heights of 0.5 metres, the power output is almost 100 watts.  WAG’s have limited life and current systems suffer from excessive wear.

Site conditions will determine the rate at which the tail tube of the buoy accumulates weed and other forms of fouling, and these aspects need to be taken into consideration when developing the maintenance regime for the WAG. WAG’s can also be very susceptible to storm damage.

## Rechargable Batteries

    - IALABATT 1 to 4

In recognition of the importance of battery technology in the operation of the modern aids to navigation, IALA has held four workshops on battery related topics for the benefit of members and manufacturers.  The first was held in Bulgaria in 1987 and subsequently in France in 1993 &1997, and combined with IALALITE in Germany, in 2001.  Reports on the proceedings are available as IALA Publications.

    - Principal types

There are two main types of storage battery technologies applied to aids to navigation:

- lead acid;

- nickel cadmium.

- Are there other types ???

The lead acid type is generally preferred because of its lower cost and higher energy exchange efficiency (95% vs. 80%) than the nickel cadmium battery.  However, the nickel cadmium battery can operate in lower temperatures and for a greater number of deep discharge cycles.

#### Lead Acid

The basic form of this battery uses a lead dioxide positive plate and a lead immersed in an electrolyte of dilute sulphuric acid.  These were originally wet or flooded cells.  However in recent years various forms of “sealed” cell batteries have become available and are quite common in aids to navigation applications.

##### Flooded Lead-Acid Cells

There are three main types of flooded lead acid cell in general use:

- Plate Cells:- that have a pure lead positive plate with a large surface area, in which active material is formed from the lead of the plate itself.

- Tubular Plate Cells:-that use a tubular plate for the positive electrode.  The active material is contained in permeable insulating tubes through which the electrolyte can diffuse.

- Pasted Plate Cells:- in which the positive electrode is a pasted plate consisting of active material formed from oxides or salts of lead pressed into a lead alloy plate of grid form.

##### Valve Regulated Lead-Acid Cell (VRLA)

The VRLA comes in two types:

- AGM:- (absorbed glass-mat) that use a micro glass separator system to absorb the electrolyte.

- Gel Batteries:- that use a gelified electrolyte and polymeric separators to prevent short circuits between the positive and negative plates.

Both types have no “free electrolyte” and the batteries theoretically can be used in any position without acid leakage.  It is recommended that feature be confirmed before system designs are finalised.

The batteries use an oxygen recombination process so that normally there is no gassing. However, a safety valve is provided to release any excess pressure.

Operational characteristics of the battery include:

- a 5-10 year life on float-charge applications;

- tolerance to deep discharge cycles (ie. 80%);

- gradual loss of capacity in cycling applications compared with a flooded cell.

#### Nickel Alkaline Battery

These batteries use compounds of nickel and, generally, cadmium with a solution of potassium hydroxide as the electrolyte.

Nickel-cadmium cells use perforated steel plates that hold the active material, mainly a nickel hydroxide in the positive plate and a cadmium compound in the negative plate.  The construction is generally referred to as a "pocket plate” cell.

A range of valve regulated nickel-cadmium batteries that use a recombination process now complement the traditional flooded cell design.  Under normal float charging conditions any gas produced is recombined within the battery and water loss is negligible.  However if the battery is overcharged it will vent but water can be added if necessary.

    - Technical Considerations

#### Flooded cell or “Sealed”

The traditional lead acid and nickel cadmium flooded cell technologies have provided good service in many lighthouses and other fixed aids to navigation.  Large capacity flooded cell batteries (or “wet cells”) have a considerable cost advantage for large installations.  However sealed batteries offers some important advantages that include:

- suitability for both floating and fixed aids;

  - facilitates standardisation;

  - reduces on-site maintenance (work, time and cost);

- safer to transport and handle;

  - reduced risk of occupational injuries from electrolyte spills;

  - encourages recovery for appropriate disposal;

  - a less hazardous cargo for helicopters and small boats.

#### Technical Comparison

Table 7.4 shows some of the technical differences between the lead-acid and nickel cadmium batteries.

Table 7.4 A comparison of the performance characteristics of lead-acid and nickel cadmium batteries.

| Type (Technology) | Operating Temperature | Self-Discharge Rate Per Month | Cycle Life to 80% DoD at 40ºC | Ah Charge Efficiency from 20% Discharged to 100% at 25ºC | Relative Cost (1990) |
| --- | --- | --- | --- | --- | --- |
| Tubular-plate flooded | -15 to 55 | 2 | 1800 | 83 | 1 |
| Flat plate flooded | -15 to 55 | 2 | 1200 | 80 | 1 |
| Tubular-plate VRLA | -30 to 50 | 3 | 1000 | >90 | 1.6 |
| Flat plate VRLA | 5 to 50 | 1 | 500 | >90 | 2 |
| Pocket plate VR | -40 to 50 | 2 | 2000 | 71 | 3.9 |
| Pocket plate flooded | -40 to 50 | 4 | 2500 | 71 | 3.9 |

    - Battery Disposal

A number of countries now have standards and regulations relating to the safe and environmentally acceptable methods of disposing or recycling of batteries.  The topic was also addressed during IALABATT 3

Refer to IALA publication: - Guidelines on Maintenance and operation of batteries, December 2001.

## Electrical Loads and Lightning Protection

    - Electrical Loads

The requirement for IALA to prepare standard methodology for calculating and defining the load profile of electric aids to navigation arose at IALABATT 2 and reconfirmed at IALABATT 3.  IALA has produced a comprehensive Guideline covering the loads for:

- lights;

- racons

- electric sound signals

- fog detectors

- monitoring and telemetry systems

- charge controllers

- signal control equipment

Refer to the IALA publication: - Guidelines on a standard method for calculating and defining the load profile of aids to navigation, December 1999.

    - Lightning Protection

To assist those engaged in the design of aids to navigation, IALA has produced Guidelines to describe practical methods for the design, installation, inspection and testing of lightning protection systems.  The information covers lightning protection for aids to navigation structures, equipment and systems.

Refer to IALA publication: - Guidelines for the protection of lighthouses and aids to navigation against damage from lightning, December 2000.

# MANAGEMENT ISSUES

## Provision, Design and Management of Aids to Navigation

### International Criteria

#### International Convention for the Safety of Life at Sea, 1974 (SOLAS)

SOLAS is one of the oldest international conventions and originates from a conference held in London in 1914 to address aspects of safety at sea following the sinking of the White Star liner Titanic in 1912.  Since then there have been four other SOLAS Conventions, the latest being the 1974 version that came into force in 1980.

The SOLAS Convention is administered by United Nations through the International Maritime Organisation (IMO).

The 1974 Convention is divided into eleven chapters and within these a series of regulations.  The contents cover:

| Chapter I | General provisions |
| --- | --- |
| Chapter II-1 | Construction – Subdivision and stability, machinery and electrical installations |
| Chapter II-2 | Construction – Fire protection, fire detection and fire extinction |
| Chapter III | Life-saving appliances and arrangements |
| Chapter IV | Radiocommunications |
| Chapter V | Safety of navigation |
| Chapter VI | Carriage of cargoes |
| Chapter VII | Carriage of dangerous goods |
| Chapter VIII | Nuclear ships |
| Chapter IX | Management for the safe operation of ships |
| Chapter X | Safety measures for high-speed craft |
| Chapter XI | Special measures to enhance maritime safety |
| Chapter XII | Additional safety measures for bulk carriers |
| Appendix | Certificates |

#### SOLAS Chapter V

SOLAS Chapter V, and Regulation 13 in particular, defines the obligations on Contracting Governments to provide aids to navigation and related information.  It therefore defines one of the primary roles of IALA National Members.

In December 2000, the 73rd session of the IMO Maritime Safety Committee (MSC) adopted a completely revised SOLAS Chapter V on Safety of Navigation that came into force on 1 July 2002.

#### Regulation 13 - Establishment and operation of aids to navigation

The Regulation states:

1. Each Contracting Government undertakes to provide, as it deems practical and necessary either individually or in co-operation with other Contracting Governments, such aids to navigation as the volume of traffic justifies and the degree of risk requires.

2. In order to obtain the greatest possible uniformity in aids to navigation, Contracting Governments undertake to take into account the international recommendations and guidelines (Reference is made to IALA) when establishing such aids.

3. Contracting Governments undertake to arrange for information relating to aids to navigation to be made available to all concerned. Changes in the transmissions of position-fixing systems which could adversely affect the performance of receivers fitted in ships shall be avoided as far as possible and only be effected after timely and adequate notice has been promulgated.

#### Comments

To satisfy the obligations of Regulation 13, the contracting government has to make assessments on:

- whether or not to provide particular types of aids to navigation;

- the type, number and location of aids to navigation;

- what information services are necessary to adequately inform the mariner.

    - LEVEL OF SERVICE (LOS)

In general, there are three levels of service components for aids to navigation services:

- extent which addresses whether a service will be provided by the contracting government or other agencies;

- quantity which addresses the type, size, number and mix of aids to navigation required; and

- quality which addresses the operational reliability of the service.

The SOLAS regulation is an excellent, general level of service statement in terms of the “extent” (where will National Authorities provide the service) of service.  Each contracting government is expected to establish its own service level standards to determine where a service will be provided, based on a measure of benefits, costs, and risks.

The following guidelines address the levels of service provided in terms of “quantity” and “quality” of service wherever a contracting government has decided to provide an AtoN service.

#### National Obligations

To fulfill the obligations imposed by the requirements of the SOLAS Convention, it is recommended that National Authorities prepare Level of Service (LOS) statements as outlined below:

- Review both the quantity and quality of the service and the related LOS standards and Operational Performance Statements (OPS) on a regular basis and update them as necessary to reflect changing patterns of use and reassess risk factors, as well as changing technology and resource levels provided by contracting governments.

- Establish a formal mechanism for promulgating operational parameters and changes which can be accessed by all potential users.  This should include a means of immediate notification, within practical limits, when the failure of an AtoN is identified.

- Where responsibility for the provision of an AtoN service is delegated, for example, to state, territory, or local government organizations, or to port, harbour, or waterway authorities, or to local private groups, responsibility to ensure and/or enforce national obligations shall remain with the National Authority concerned.  (some Authorities may not have sufficient resources to approve and monitor all private and other authorities AtoNs, but will enforce national standards if users report a problem).

#### Level of Service Statement for Quantity

- The Level of Service (LOS) standards and Operational Performance Statements (OPS) for quantity will vary both from country to country and for individual areas depending on usage patterns, degree of risk and the mix of aids to navigation provided.  The LOS/OPS statements represent a commitment from the National Authority to mariners in general who are navigating or operating in the area, and to the government and/or other client groups responsible for funding the provision of the AtoN service.  It is important that the LOS/OPS statements are clearly understood and available to all concerned. The following outlines the stages that must be followed in the preparations of LOS/OPS statements:

- Level of Service/Operational Performance Statements

- It is recommended that LOS/OPS statements be prepared in three stages as follows:

- Step 1: 	Determine the maritime usage patterns for the area concerned and identify risk factors for both vessels and the environment in general, taking into account at least the following:

- Nature and character of the area:

- depth

- siltation

- hazards

- tides and currents

- visibility, weather patterns, and sea-ice conditions

- distinctive shoreline features for both visual and radar navigation.

- background lighting/background environment and relative position of the sun to the traffic routes.

- Traffic Analysis including:

- type of traffic and vessel characteristics by user group

- routes and speed, including route marking requirements

- volume of traffic by user group

- type of cargo, particularly when hazardous

- Risk assessment including:

- basic navigational risks, particularly in high traffic situations

- risk to vessels caused by conflict between user groups

- risk to the environment resulting from a maritime incident

Step 2:	Develop an overall Navigation Plan (NAVPLAN) for the area concerned indicating the mix of AtoN considered necessary to provide the required level of service in the most cost effective manner.  This plan may also include appropriate vessel reporting and control requirements and separation schemes to further reduce the risk of an incident.  As far as possible the NAVPLAN should ensure that failure of a single AtoN does not result in a major penalty to the overall AtoN service, or a substantial increase in risk.

In addition, the NAVPLAN should take into account the likely navigational aids carried by user vessels, including the probability that elements of these onboard navigational aids may themselves suffer failure.  For this reason, the number, type and mix of AtoN types provided must be considered in terms of short-term, navigational aid failure during a critical manoeuvre, or extended period failure.

Step 3:	For each proposed AtoN, or system of AtoNs, prepare an Operational Performance Statement (OPS).  The format for the OPS will vary depending on the type of AtoNs and mix of user groups.

- For a light signal, the OPS should typically comprise information regarding   the probability that the light can be seen at the required range when approached by a vessel at any random time while the AtoN is required for service.  Inputs to this type of statement include the visibility  in the area expressed as a cumulative probability by number of days and the AtoN equipment availability based on meantime between failures and meantime to repair; or

The statement may also be expressed in terms of the maximum operational capability of the light, or totality of lights in an area.  Such an OPS would identify the minimum visibility level at which the light(s) can be seen at the required range when approached by a vessel at night.  The statement would be similar for an unlighted system of aids - eg. AtoN system supports visual navigation until visibility is reduced below 1 nautical mile.

The type of OPS will depend on the method used to measure or establish the LOS standards for the quantity of service.

There are a number of analytical methods available for the establishment of LOS standards and OPS, including:

Relative Risk:

- The level of service standard is based on a relative risk scale using quantified threats or risk factors to determine acceptable risk levels.  Statistical probability ratios or percentages may be the basis for both the LOS and OPS.

Visual Design:

- The level of service standard is based on a standard percentage of time that operational aids will be seen, using predetermined, acceptable risk levels based on professional judgment for each individual, quantified threat or risk factor for each type and size of vessel. The OPS may be expressed in terms of percentage of time that the aids will be visible due to known, local atmospheric conditions, or as the minimum visibility level at which the AtoN system will support visual navigation and below which radar or other electronic systems will be required.

- For a radio AtoN such as a broadcast station providing differential GPS corrections, the LOS should take into account both the expected propagation conditions between the transmitting site and the user and the equipment availability of the AtoN itself.

#### Level of Service Statements for Quality

- Equipment availability

- The equipment availability of an AtoN is governed by the equipment operational reliability measured in terms of meantime- between- failure (MTBF) and the time to complete repairs in the event of a failure, measured in downtime or meantime-to-repair (MTTR).  Where routine maintenance cannot be carried out during non-operational periods, an additional factor for planned downtime must be included.

- Equipment with very high reliability may achieve  required availability standards, even with a long MTTR.  National Authorities should, therefore, consider including a maximum targeted downtime specified in the OPS, where usage and risk factors so dictate.  MTTR depends not only on the ease with which a fault may be diagnosed and repaired on site, but also on access limitations (including weather and sea state conditions), availability of trained staff, and spares holdings, etc.

- Key elements in achieving high equipment availability are:

- choice of equipment which is inherently reliable;

- inclusion of active or passive redundancy features wherever availability targets or maximum downtime targets are not achievable due to remoteness of the site or limited access due to weather or sea state conditions.

- provision of performance measuring facilities, whenever availability or downtime targets are not achievable, will allow potential failures to be identified and corrected before actual failure occurs.  For unwatched automatic AtoN this will normally require provision of a remote monitoring system.

- availability of adequately trained staff and provision of suitable spares.  In this context, National Authorities should address the option of contracting out maintenance and repair tasks with care.  In particular:

- Responsibility for meeting SOLAS Convention requirements cannot be delegated to the contracting organization.

- When contracting out is contemplated, the National Authority concerned should retain sufficient design and performance assessment expertise to be able to audit the performance of the contracting organization in an efficient and effective manner.

- The contracting organization should meet appropriate quality management criteria, including staff training requirements.

Level of service statements for quality of service should be expressed as one, or a combination of the following:

- an overall percentage availability statement reflecting the percentage of time that any AtoN can be expected to be operational.

- percentage availability targets based on importance or type of AtoN.

- maximum downtime targets for AtoN overall, or within a specific area.

#### Consultation and Review of LOS

Development of the LOS targets, the NAVPLAN, and the OPS for each AtoN or AtoN area, should be an iterative process in consultation with all user groups and funding organizations.  It is recommended that the consultation process include:

- meetings with all user representatives conducted during the development of the LOS standards, OPS and NAVPLAN to review proposals and arrive at an equitable balance between service, risk and cost;

- Regular review meetings facilitated by the creation of an Advisory Board, where appropriate, and by feedback from both commercial and pleasure craft operators.

It is recommended that National Authorities put in place procedures to monitor the performance of individual AtoN and that regular reports of achieved equipment availability based on MTBF and MTTR are provided.  As far as is practical, equipment performance monitoring is required in addition to reports of equipment failure received from mariners.

#### Mix of Aids to Navigation (Layers of Service)

#### Options

Tables 6.1 and 6.2 provides a summary of available aids to navigation systems and obtainable accuracies.  It is assumed that radar and visual bearings have an accuracy of 0.5º and radio bearings 2º respectively.

Table 6.1 Indicative accuracies of aids to navigational systems.

| Distance off shore in nautical miles | Obtainable accuracies |  |  |
| --- | --- | --- | --- |
|  | > 500 m | 100 - 500 m | < 100 m |
| Unlimited | Astronomical Fix |  | GPS GLONASS |
| 150 - 800 | Astronomical Fix | LORAN-C | GPS GLONASS |
| 30 - 150 | Astronomical Fix Radio Beacons |  | GPS GLONASS LORAN-C Precision Systems |
| 6 - 30 | Astronomical Fix Radio Beacons Visual Bearings Radio Bearings | Radar Bearings | GPS GLONASS LORAN-C Precision Systems |
| 6 or less |  | Radio Beacons Visual Bearings Radar Bearings | GPS GLONASS LORAN-C Precision Systems |

The various type of aids to navigation have advantages and disadvantages for the user as well as for the provider as indicated in Table 9.3.

Table 6.2 Comparison of the advantages and disadvantages of different types of aids to navigation.

| System |  | Users |  | Providers |
| --- | --- | --- | --- | --- |
|  | Advantages | Disadvantages | Advantages | Disadvantages |
| Visual | Can be used to position Convey immediate information Can be used without a chart if user has a good local knowledge | Range depends on site, height, colour, background Limited by visibility Position of floating aids not always accurate | For hazard warning, traffic regulation, guidance, etc. Placement flexible Maintenance requires little training | Maintenance expensive Planning for maintenance depends on weather conditions Logistic system required Training maintenance personnel |
| Radar | Identification with racon possible in reduced visibility conditions With a racon identification of low coastline Only one aid is required Rapid deployment | Onboard equipment needed Racons may interfere if not placed in an appropriate configuration, aids equipped with radar reflector are difficult to identify | Can replace visual aids Warnings of dangers (New dangers) | Radar reflectors needed Some vessels do not have radar Racon investment expensive Training for maintenance of racons |
| Radionavigation | Wide scale coverage All weather use Automatic navigation Precision possible | On board equipment needed | Reduced maintenance-Automatic monitoring Reduction of visual aids possible | May not be under Lighthouse Authority control Monitoring requirement Training maintenance personnel Large investment |

    - Risk Management

This section needs to amended based on the IALA Risk Model. Proposing G.Prosser

Dealing with “risk” is a basic issue of human existence.  The establishment of the early lighthouses represent a tangible way of addressing some of the problems that arose when humans decided to venture out to sea, and then into global trade and the mass transport of people by ships.

The traditional definition of risk is the probability of an unwanted event occurring, multiplied by the impact or consequence of that event.

R = P X C

However no event is completely “Risk Free” and therefore its acceptability by stakeholders will vary in accordance of what they perceive to be in jeopardy. To compensate for this, acceptability is now included in the risk formula.

R = P X C X A

Unwanted events include deprival, loss or injury to persons, property or the environment.

#### Risk Management

Risk management is a term applied to a structured (logical and systematic) process for

- identifying, analysing, assessing, treating, monitoring and communicating risks for any activity, and;

- achieving an acceptable balance between the costs of an incident, and the costs of implementing measures to reduce the risk of the incident happening.

The risk management approach works equally well for identifying the risks at a detailed or broad level.  It can also address the risks from different perspectives.

For example, if the issue is the automation and destaffing of a lighthouse, there are likely to be different sets of risk for:

- service providers (aids to navigation authority, the lightkeepers);

- service users (mariners);

- external groups (politicians, local community, conservation groups).

IALA held a Workshop in Quebec in October 2000 to assist in preparing the publication “IALA Guidelines on Risk Management, December 2000”

A number of countries also have national standards on risk assessment and risk management.

#### Risk Management Decision Process

The Risk Management process described in the IALA Guidelines comprises five steps that follow a standardized management or systems analysis approach:

a. Identify risks/hazards;

b. Assess risks

c. Specify risk control options

d. Make a decision; and

e. Take action.

The diagram in Fig 6.1 provides a guide to the steps involved in the IALA Risk Assessment and Risk Management process.

Fig 6.1 The IALA Risk Assessment and Risk Management process.

#### Levels of Risk

Once the risks have been identified it is generally useful to rank them in order of consequence.  Resources can then be assigned to treating the most serious risks first.  The matrix at Fig 6.2 provides a basis for prioritising risks.

| Impact | High |  |  |  |
| --- | --- | --- | --- | --- |
|  | Medium |  |  |  |
|  | Low |  |  |  |
|  |  | Low | Medium | High |
|  |  | Likelihood |  |  |

|  |  | High Level of Risk |
| --- | --- | --- |
|  |  | Medium Level of Risk with Caution |
|  |  | Low Level of Risk |

Fig 6.2 Risk matrix.

#### Coastal Landfall and Waterway Risk Factors

A risk analysis associated with a coastal landfall, waterway or port approach might consider a range of factors that contribute to the overall risk exposure.  Table 6.3 provides an indication of the factors that could be taken into consideration.

Table 6.3 Indicative risk factors relating to marine navigation.

| Ship traffic consideration | Traffic volume | Navigational conditions | Waterway configuration | Short-term consequence | Long-term consequence |
| --- | --- | --- | --- | --- | --- |
| Quality of vessels | Deep draught | Night/Day operations | Depth | Injuries to people | Health and safety impacts |
| Crew competency | Shallow draught | Sea state | Channel width | Oil spill | Lifestyle disruptions |
| Traffic mix | Commercial fishing vessels | Wind conditions | Visibility obstructions | Hazardous material release | Fisheries impacts |
| Traffic density | Recreational boats | Currents (river, tidal, ocean) | Waterway complexity | Property damage | Endangered species |
| Nature of cargo | High speed craft | Visibility restrictions | Bottom type | Denial of use of waterway | Shoreline damage |
|  |  | Ice conditions | Stability (siltation) |  | Reef damage |
|  |  | Background lighting |  |  | Economic impacts |
|  |  | Debris |  |  |  |

    - Availability Objectives

#### History

Members of IALA became interested in the concept of availability around 1975 when significant numbers of lighthouses were being automated and destaffed.  The measurement of ‘Availability’ provided a quantitative measure of performance (or service to the mariner) that was independent of whether an aid to navigation was manned or not.

‘Availability’ is a useful indicator of the level of service provided by individual or defined groups of aids to navigation because it is representative of all the considerations, within the control of the Authority, that have gone into providing and maintaining the facility.  These include:

- quality assurance procedures;

- design and systems engineering;

- procurement;

- installation;

- maintenance procedures;

- failure response;

- logistic arrangements.

In developing the concept of Availability, IALA considered it necessary to measure the long-term performance of an aid to navigation.  To achieve this it was recommended that the calculations should use a time interval greater than 2 years.  The original examples developed for the three availability Categories of lights were based on a 1000 day time interval (most likely to simplify the conceptual calculations).

#### Calculation of Availability

The availability of an aid to navigation may be calculated using one of the following equations, and is usually expressed as a percentage:

Availability =MTBFMTBF+MTTR or Up timeTotal Time or Total time−Down TimeTotal Time

#### Definition and Comments on Terms

Reliability

This is the probability that an aid to navigation, when it is available, performs a specified function without failure under given conditions for a specified time.

Availability

This is the probability that an aid to navigation or system is performing its specified function at any randomly chosen time.

  - IALA generally uses the term as a historical measure of the percentage of time that an aid to navigation was performing its specified function.

- The non-availability can be caused by scheduled and/or unscheduled interruptions.

Continuity

This is the probability that an aid to navigation or system will perform its specified function without interruption during a specified time.

- for example, if a DGPS station is functioning correctly when a vessel is about to make its approach into a port, the continuity term states the probability that the DGPS service will not be interrupted in the time it takes the vessel to reach its berth;

- for GNSS systems, IALA has proposed that the time interval for continuity calculations be based on a three-hour time frame.

Redundancy

This is the existence of more than one means, identical or otherwise for accomplishing a task or mission.

Integrity

This is the ability to provide users with warnings within a specified time when the system should not be used for navigation.

Failure

This is the unintentional termination of the ability of a system or part of a system to perform its required function.

Mean Time Between Failures (MTBF)

This is the average time between successive failures of a system or part of a system.  It is a measure of reliability.

- for components, such as lamps, it is usual to determine the MTBF (or life) statistically by testing a representative sample of components to destruction;

- for a system such as a DGPS station the MTBF is determined from the number of failures that have occurred within a given interval.  For example; if four failures occur over a two year interval, the MTBF would be 4380 hours (ie. =24*365*2/4).

Mean Time to Repair (MTTR)

This is a measure of an Authority’s administrative arrangements, resources and technical capability to rectify a failure.

- for a small port, the MTTR times might only be several hours;

- an Authority with a more distributed network of aids to navigation may have MTTR times equivalent to several days because of the distances and transport mobilisation limitations.

Failure Response Time

This is a sub-set of the MTTR and relates to the time it takes to be notified of a failure, to confirm the details and mobilise personnel to depart for the aid to navigation.

#### IALA Categories for Traditional Aids to Navigation

In 2004 ???, IALA published a Recommendations on Availability Objectives of Aids to Navigation Services.  This document states management objectives for operational performance levels of aid to navigation services provided to mariners. It also provides guidance on suitable and realistic levels of operational performance for members to adopt. The 2004 approach for the categorisation of aids to navigation service is different to the earlier version of the recommendation.

| CATEGORY | AVAILABILITY OBJECTIVE | CALCULATION |
| --- | --- | --- |
| 1 | 99.8% | Availability Objectives are calculated over a three-year continuous period, unless otherwise specified |
| 2 | 99.0% |  |
| 3 | 97.0% |  |

Category 1: An Aid to Navigation (AtoN) or system of AtoN that is considered by the Competent Authority to be of primary navigational significance. For example, lighted aids to navigation and racons that are considered essential for marking landfalls, primary routes, channels, waterways or new dangers.

Category 2: An AtoN or system of AtoN that is considered by the Competent Authority to be of navigational significance. For example, it may include any lighted aids to navigation and Racons that mark secondary routes and those used to supplement the marking of primary routes.

Category 3: An AtoN or system of AtoN that is considered by the Competent Authority to be of less navigational significance than Category 1 and 2.

The Recommendation also states that the absolute minimum level of availability of an aid to navigation should be set at 95%.

More details about the availability objectives for conventional or traditional aids to navigation are contained in the publication: IALA Recommendations on availability objectives of aids to navigation, publication ####, published in 2004.

#### Availability and Continuity of Radionavigation Services

The availability objectives for DGNSS (DGPS) services have been handled somewhat differently from traditional aids to navigation. This reflects the broader policy formulation process that includes the IMO Resolution A.815(19) for a World Wide Radionavigation System.

The IALA recommendation on availability objectives for radionavigation aids to navigation are contained in the Recommendation on the Performance and Monitoring of DGNSS Services in the Frequency Band 283.5 – 325 kHz (R121)

Recommendation R121 retains the original definition of availability, but adds a statement about “non-availability”:

Non-availability is equivalent to “down time” but as proposed includes both scheduled and/or unscheduled interruptions (ie. preventative and corrective maintenance).  The revised equation becomes:

Availability =MTBOMTBO+MTSR

Where:

| MTBO = | Mean time between outages; based on a 2 year averaging period (30 days ocean phase) |
| --- | --- |
| MTSR = | Mean time to service restoration; based on a 2 year averaging period (30 days ocean phase) |

Example (1):

- assume a scheduled maintenance cycle of 6 months:

  - mean time between scheduled maintenance is 0.5 years;

    - ie. 4 scheduled maintenance breaks in 2 years;

- assume a MTBF of 2 years;

  - the average number of failures over 2 years is expected to be approximately 1;

- this gives a total of 5 outages over the two year period;

  - mean time between outages is 2/5 years or approximately 3500 hours;

- if the average out of service time for scheduled maintenance is 6 hours;

  - the total out of service time for scheduled maintenance over the two year period is 24 hours;

  - similarly, if the unscheduled maintenance period is 12 hours, the total time out of service over the two year period is 36 hours;

    - this covers 5 maintenance events and, therefore, the mean time to service restoration is 36/5 hours or approximately 7 hours;

- the overall availability over the two year period is (3500/(3500+7)) or 99.8%.

Example (2):

- assume a scheduled maintenance cycle of 6 months;

  - mean time between scheduled maintenance is 0.5 years;

  - ie. 4 scheduled maintenance breaks in 2 years;

- assume a MTBF of 2000 hours;

  - therefore the average number of failures over 2 years (17520 hours) is expected to be 8.76, rounded up to 9;

- this gives a total of 13 outages over the two year period (4 scheduled + 9 unscheduled);

  - mean time between outages is  17520 hours/13 or 1348 hours;

- if the average out of service time for scheduled maintenance is 6 hours;

  - the total out of service time for scheduled maintenance over the two year period is 24 hours;

  - similarly, if the unscheduled maintenance period is 67 hours, the total time out of service over the two year period is 91 hours.  This covers 13 maintenance events;

    - mean time to service restoration is 91/13 hours or approximately 7 hours;

- the overall availability over the two year period is 1348/(1348+7) or 99.5%.

#### Over and Under Achievement

Issues

As discussed at Section 6.1.2, the actual availability achieved by an individual aid to navigation is a reflection of the quality of the establishment process, the maintenance regime and the skills of personnel involved.

There is a cost penalty associated with prescribing a higher level of availability for a system such as an aid to navigation.  There is also a cost penalty associated with the maintenance of unreliable systems.  The interrelationship is complex, but the objective it to find the minimum cost solution as illustrated in Fig 6.3 .

Fig 6.3 The cost of reliability.

Over-Engineering vs. Unreliability

For a lighthouse in a remote location, the cost of time and transport to rectify equipment failures can be very high.  From this perspective:

- the one-off cost of over-engineering is generally not as expensive in the long term as the ongoing cost of attending to un-reliable equipment and/or poor system designs;

- a conservative design approach has its merits.

If the aid is not achieving its availability objective, the Authority should ascertain the reasons for this and implement actions that remedy the situation.  IALA has recommended that if a facility cannot achieve an availability of 95% (ie. 50 days out per 1000 days) after reasonable endeavours, that consideration should be given to withdrawing the facility (as an aid to navigation)

If a single aid within a group is performing above its availability objective, this could be due to either technical or environmental reasons.  If the performance difference occurs between sites using similar equipment, and this trend has been established for some time, it may be of benefit to investigate the reasons for the difference.

If a group of aids is found to be over performing for a relatively long period of time, there is an opportunity to review the maintenance practices with a view to determining the reasons, and possibly to consider extending the maintenance intervals or reducing the maintenance effort.  This could lead to:

- lower operating costs;

- issues relating to the consequential surplus maintenance capacity.

#### Continuity

IMO uses a more elaborate definition of Continuity than that given in Section 6.1.4.2.  It states that:

Continuity is the probability that, assuming a fault free receiver, a user will be able to determine position with specified accuracy and is able to monitor the integrity of the determined position over the (short) time interval applicable for a particular operation within a limited part of the coverage area.

If the service is available at the beginning of the operation, then the probability that it is still available at a time ‘t’ later is:

P = exp (-t/MTBF)

This is the standard expression for reliability and excludes scheduled outages

- it uses MTBF and assumes that planned outages will be notified.

The Continuity, or probability that the service will be available after a continuity time interval (CTI), is then:

C = exp (-CTI/MTBF)

If MTBF is much greater that CTI, the equation approximates to:

C = 1 – (CTI/MTBF)

Where:

| MTBF = | Mean time between failures based on a 2 year averaging period |
| --- | --- |
| CTI = | Continuity Time Interval; in the case of maritime continuity, is equal to 3 hours |

There is no need to include the availability at the beginning of the time period of the operation because if there is no service, then the operation will not commence.

Example (1):

Using the figures in the previous example for a system with a 2 year MTBF, the continuity over a three hour period is 1-(3/17520), or 99.98%

Example (2):

Using the figures in the previous example for a system with a 2000 hour MTBF, the continuity over a three hour period is 1-(3/2000), or 99.85%.

    - Reviews and Planning

#### Reviews

In many countries, the network of aids to navigation has been built up over a considerable time, in some cases, centuries.  It should be recognised that the nature of shipping is continually changing and this means that the aids to navigation infrastructure should be reviewed periodically.  The rate of change varies from place to place, but it would be reasonable to adopt a review process using one of the change management tools described LATER ON that provides:

- a Strategic Plan (Navigation Plan) with a suggested 10 year outlook, and;

- an Operational Plan with a suggested rolling 5 year work program.

#### Strategic Plans

A Strategic Plan is the result of an informed and consultative process that sets the long term goals and objectives for an organization.  For an Aids to Navigation Authority it would include decisions on:

- the role of the authority, for example:

  - to promote a high standard of maritime safety;

  - to provide infrastructure and information services to support the safety of navigation in a particular area;

- how the authority will go about discharging its responsibilities, for example:

  - outline of the corporate values of the authority;

  - corporate governance arrangements;

  - funding arrangements;

  - reviews of industry trends;

  - an understanding of the users and navigation requirements.

Because of its importance and its effect on the mariners, any strategic plan should be developed as much as possible in full consultation with the mariners and other stakeholders.

#### Operational Plans

The Operational Plan might cover:

- The implementation of the strategic plan, and may include statements on current policy issues such as:

  - maintenance;

  - current and new technology;

  - the design life of new infrastructure

  - remote monitoring and control;

  - historic lighthouses;

  - environmental culture and safety;

  - the program for aids to navigation reviews;

  - contract services;

  - transport services;

  - sources of revenue;

  - external relationships;

  - information, communication  and consultation management.

- a list of changes to individual aids to navigation, including any new facilities.  The list would reflect:

  - decisions resulting from user and stakeholder consultations;

  - reviews, including those that use:

- - risk analysis, risk management procedures (see section 6.3.3), or;

- - a level of service process, (see section 3.5.2), or;

- - the authority’s quality management procedures (see section 6.3.7)

  - the authority’s technical and maintenance policies etc.

- project schedules that reflect known priorities, such as:

  - government policies;

  - user requirements;

  - available resources;

  - budget (revenue) forecasts and constraints.

    - Performance Measurement

Performance Measurement is a process of identifying, collecting, analyzing, evaluating, monitoring and communicating information relative to an organization’s planned performance versus the results that it delivers.  Though it includes reporting, performance measurement is more than a reporting system; in fact, it is a discipline that is intended to assist leaders in making key decisions that maintain alignment with the priorities of the organisation.  This process encompasses several activities such as: aligning resources to results; setting performance targets; monitoring and measuring progress against the plan; and, reporting on the progress as the plan is delivered.  It is therefore a critical process in the effective delivery of the organisation’s business plans which align with strategic objectives.

Sound management elements need to be in place for effective Performance Measurement.  These include:

- Fully defined mission and objectives;

- Fully defined system and program requirements;

- Supporting acts and regulations;

- Service standards and levels of service agreements;

- Environmental scan;

- Data management (Information Management system);

- Reporting mechanism.

Senior management’s commitment, personel involvement at the specialist lines, partners, stakeholders and effective communication are critical success factors.

Performance measurement is a management tool that can be used to measure, analyse and monitor the performance of a network of aids to navigation and/or specific systems and equipment.  The information obtained can be used to:

	show accountability to government and stake holders;

	demonstrate the efficiency and effectiveness of the service being provided;

	compare the performance of:

similar systems or equipment in different locations;

contract and internally provided services ;

	amend:

system designs;

procurement decisions;

equipment choices;

maintenance procedures and practices;

	increase or reduce maintenance effort;

	extend maintenance intervals.

    - Quality Management

#### quality Systems

Quality Management Systems have been developed and introduced by numerous businesses, but increasingly are being based on the ISO9000 series of standards. These standards provide a broadly accepted framework for implementing a quality management system.

A generic quality management system is process focused and defines procedures for how things are to be done and what resources are necessary.  It addresses:

- who does what?

- what skills and qualifications are necessary?

- what processes have to be followed to get consistent outcomes?

- what resources are necessary to do the work efficiently?

Refer also to: - The IALA Quality Assurance Guideline for the procurement, maintenance and repair of aids to navigation equipment and systems. - IALA Guidelines for the establishing of Quality Management in aids to navigation administrations, April 1997.

#### PRODUCT QUALITY AND CERTIFICATION

To be completed by Juan F Rebollo based on the guideline to be developed by IMC

#### SERVICE QUALITY AND CERTIFICATION

To be completed by Juan F Rebollo based on the guideline to be developed in the “workshop on service quality management. First draft for ANM5

#### ISO 9000 Series

The 1994 quality standard series of ISO 9001, 9002 and 9003 have been jointly revised and amalgamated into ISO 9001-2000.

The new series of standards designated as ISO 9000 comprises:

- ISO 9000 Quality management systems - Fundamentals and vocabulary

- ISO 9001 Quality management systems - Requirements

- ISO 9004 Quality management systems - Guidance for Performance Improvement

#### ISO 9001 - 2000

ISO 9001 specifies requirements for a quality management system that can be used for internal application by organizations, or for certification, or for contractual purposes.  It focuses on the effectiveness of the quality management system in meeting customer requirements.  See Fig 6.4.

#### ISO 9004 - 2000

ISO 9004 gives guidance on a wider range of objectives of a quality management system than does ISO 9001, particularly for the continual improvement of an organization's overall performance and efficiency, as well as its effectiveness.  ISO 9004 is recommended as a guide for organizations whose top management wishes to move beyond the requirements of ISO 9001, in pursuit of continual improvement of performance.  However, it is not intended for certification or for contractual purposes.

Fig 6.4 Diagram from ISO 9001 showing the emphasis on

satisfying customer requirements.

#### ISO 14000 Series

        - ISO 14000

This is a collection of voluntary consensus standards that have been developed to assist organizations to achieve environmental and economic gains through the implementation of effective environmental management systems.

There are three standards that deal with Environmental Management Systems (EMS).  These are ISO 14001,14002 and 14004.  ISO14001 is the only standard intended for third party accreditation.  The other standards are for guidance.

        - ISO 14001

ISO 14001 specifies the requirements for an environmental management system, to enable an organization to:

- formulate a policy and objectives taking into account legislative requirements and information about significant environmental impacts;

- it applies to those environmental aspects that the organization can control and over which it can be expected to have an influence;

- it does not itself state specific environmental performance criteria;

- demonstrate to itself, and to other interested parties, conformance with its stated environmental policy;

- seek certification/registration of its environmental management system by an external organization;

- manage and measure a program of continual improvement.

#### ISO 19011 Environmental Quality

- To be developed by Pedro Rodriguez from Venezuela & JFRebollo

    - Maintenance

#### Trends

While few maintenance statistics have been collected in recent years, IALA Conference papers and Bulletin articles indicate a general interest in ways to extend maintenance intervals.  The automation and destaffing of major lighthouses has shifted the maintenance functions from a daily activity to less frequent events.

The optimal maintenance interval for aids to navigation is determined from a consideration of national priorities and the Authority’s administrative, technical and environmental constraints.  Where cost efficiency and effectiveness is the driving issue, Authorities are:

- using automation and renewable power supplies to contain or reduce costs;

- addressing the potential for new technology to:

  - reduce acquisition and operating costs;

  - extended maintenance intervals;

- reviewing transport service options.

IALA has published - Guidelines on Lighthouse Maintenance, and; - Quality Assurance Guideline for the Procurement, Maintenance and Repair of Aids to Navigation Equipment and Systems

#### Maintenance Intervals

The maintenance intervals for aids to navigation varies from daily in the case of a manned lighthouse to perhaps five years for a light buoy.  It is difficult to establish a clear view of typical maintenance intervals other than what is stated in conference and workshop papers.  Some examples include:

- major facilities are being inspected on a monthly basis;

- automated lights are being inspected less frequently (typically, one of the following: quarterly, semi-annually, annually).

Advances in self contained beacons, lamps, self-powered LED lanterns, and solar power supplies make it relatively easy for a well designed system on a fixed structure to achieve annual or biannual servicing intervals.  Systems that can be maintained in multiples of a year can be set up to take advantage of the times of the year that minimise the weather risk on work schedules and disturbance to flora and fauna.

However, a balance has to be found since longer maintenance intervals affect the authority’s knowledge of storm damage, general deterioration to aids to navigation, and control over vegetation growth that could increases the risk of obscuration and fire damage etc.  There may also be a detrimental affect on the detailed level of knowledge held by maintenance personnel.

#### Improving efficiency

Authorities have been able to achieve significant cost savings by:

- automating of lighthouses:

  - reduces the work load for lightkeepers;

  - facilitates destaffing;

- destaffing aids to navigation reduces:

  - staff costs (payroll)

  - power consumption;

  - the frequency of stores replenishment;

  - commitments on infrastructure such as houses or accommodation facilities, water and fuel storage and in some cases jetties and cargo handling equipment;

  - the requirements for station vehicles, plant and equipment;

- using more reliable equipment, better system designs, with "fail safe" or “fail by stages” features coupled with;

  - longer intervals between maintenance visits;

  - a review of maintenance management procedures;

- converting aids to navigation that operates on oil, gas or primary battery to solar power or self-powered LED lanters may provide:

  - greater flexibility in scheduling maintenance visits because of the renewable energy source;

  - opportunities for extending maintenance intervals;

- replacing floating aids with fixed structures in waterways of moderate depth;

  - particularly if it also allows a dedicated buoy tender to be replaced by some other means of transport such as smaller vessel or launch.

- introducing low maintenance materials such as high density polyethylene, GRP, stainless steel, etc to reduce maintenance time on site.  It may also:

  - decrease the number of ship-day requirements;

  - reduce the need for construction (or structural maintenance) skills;

- standardising equipment to simplify of spares management, and:

  - benefit the purchasing power of the organisation;

  - reduce the range of skills required by maintenance staff;

  - give more flexibility on the choice of basic qualification when recruiting maintenance staff;

  - provide more opportunity to understand the inherent deficiencies in particular pieces of equipment and for remedial actions to be implemented;

- remote monitoring (and control) of distant or isolated aids to navigation can save on the cost of responding to what is later found to be a false outage report;

- analysis of aids to navigation systems using risk analysis / risk management techniques may produces cost savings from a rearrangement and or reduction of the aids to navigation within a nominated area.

    - Service Delivery

#### Current Practice

Authorities with the responsibility for the provision of aids to navigation are generally government owned.  They are usually the sole national regulator of marine aids to navigation infrastructure and services, but are not necessarily the sole provider of these services.  In some countries there is a division of responsibility between the authority representing the national government and other organisations that include:

- state and territorial authorities;

- local government organisations;

- port, harbour or waterway authorities, or;

- local private groups.

#### Service Delivery Requirements

The SOLAS Convention applies to a range of vessels over 500 gross tonnage (and over 300 gross tonnage from July 2002) that are engaged in international voyages:

- where more than one local authority provide aids to navigation services, the Contracting government has ultimate responsibility for obligations under the SOLAS Convention.

The requirements for vessels operating on coastal and interstate voyages, and those that are less than 500 gross tonnage are determined by the national or state governments.

Aids to navigation may be provided to meet the specific needs of these vessels and it is these that are often operated by state, territories and local government organisations or private groups.

#### Contracting Out

In some parts of the world there have been moves by governments to sell off government business activities to the private sector.  The motivation for this varies, but includes:

- adding flexibility to how work is carried out;

- breaking down entrenched work practices that are perceived to be inefficient;

- accessing a wider range of skills and resources on demand;

- recognition that as aids to navigation become more reliable and maintenance intervals are increased, it becomes more difficult to:

  - justify having permanently staffed maintenance depots;

  - maintain the currency of work skills;

- using on-call contractors in regional locations to improve fault rectification times through reducing the travelling time to the aid.

The key elements to success when contracting out are:

- to retain sufficient skills within the Authority to understand the functional requirements of the aids to navigational network.  This includes:

  - good contract management skills to handle the day-to-day operational issues;

  - personnel to engage in user consultation and forward planning;

  - the knowledge to act as an “informed purchaser” of services;

- to retain control of intellectual property such as:

  - original drawings;

  - documentation covering the design and configuration of individual aids to navigation;

  - a register of assets and spares;

- defining a set of key performance indicators to measure the performance of the contractor.

## Environment

This section 6.2 needs to be revised by EEP

    - Hazardous Materials

#### General Issues

As an employer and in the wider context of an aids to navigation authority being a provider of safety services, there are obligations on each Authority to inform staff, temporary employees and contractors of known hazards when working on and around lighthouses and other aids to navigation.  This section provides information and guidance on some of the hazardous materials.

#### Mercury

A number of historic lighthouses are still operating rotating glass lenses and mercury float pedestals.  This was a clever method for providing a heavy lens with an almost frictionless bearing so that it could be turned by a clockwork mechanism.  However, given the toxic and corrosive properties of mercury, the following information may assist Authorities to implement appropriate safety procedures.

The mercury-bath pedestal for a first-order rotating lens contains about 13 litres of mercury.  Quantities of mercury can also be found in the electrical slip-ring units in rotating lamp array lighting equipment, some tilt-action switches, high current contact breakers, manometers and thermometers.

Physical Properties

General properties of Mercury.

| Appearance | Silvery metallic liquid |
| --- | --- |
| Chemical Symbol | Hg |
| Specific gravity | 13.546 |
| Boiling Point | 357ºC (630ºK) |
| Freezing Point | - 38.7ºC (234.3ºK) |

Spill Risk

The mercury in a lighthouse optic system does not present a significant hazard, unless personnel come into contact with "uncontained" mercury as a result of accidental spills.  Such events are usually the result of a mishap during maintenance work, or as a result of a natural disaster such as an earth tremor that displaces mercury from its containment bath.

If spilt, the mercury can enter cracks in floors, and is readily absorbed into porous surfaces such as concrete, masonry and timber.  When broken into small globules or droplets, the surface area and vaporisation rate rises rapidly.  Minute droplets will adhere readily to dust and can form respirable particles.

Mercury is a corrosive substance if it comes into contact with metals such as zinc and aluminium.

Occupational Hazard

The occupational hazard associated with mercury relates to:

- the threshold limit value (TLV) which is the concentration of the substance that most workers can be exposed to without adverse effects.

- the permissible exposure limit (PEL) expressed as a time-weighted average.  This is the concentration of the substance that most workers can be exposed to without adverse effects averaged over a normal 8 hour day or a 40 hour week.

- Vapour inhalation: Some vaporization from a free mercury surface will occur at normal room temperature and this is the most likely first contact that lighthouse personnel will have with mercury.  Unless the mercury vapour levels have been measured, personnel are unlikely to be aware of the hazard.  If the work-space around lighthouse equipment containing mercury is not well ventilated, the concentration levels can rise above recommended limits and there is potential for mercury poisoning:

  - the typical TLV and PEL concentrations for elemental mercury vapour is 0.05 mg per m3 ;

  - it is suggested that personnel should not knowingly be subjected to mercury vapour concentrations exceeding 40% of the TLV (ie. 0.02 mg per m3) without respiratory protection;

  - mercury vapour is heavier than air and in still air will tend to concentrate in low parts of the work-space.

- Ingestion:- is less common than vapour inhalation but can lead to acute mercurial poisoning;

- Absorption through the skin:- mercury is not easily absorbed through the skin and generally health authorities do not quote a TLV for skin contact.

Precautions

It is advisable that:

- personnel assigned to service equipment containing mercury should have a medical check prior to the commencement of work to establish a base-line for mercury levels in their bodies.  Further checks should take place periodically to monitor the mercury levels;

- personnel should receive training on mercury safety and spill cleanup, decontamination and waste disposal procedures;

- mercury vapour levels at the work site should be determined prior to the commencement of work to establish a base-line value;

- mercury vapour concentrations should be measured periodically during the course of the work, and action taken if values rise above safe levels;

- lighthouse towers and the equipment work space should be well ventilated when personnel are present;

- the work space should be kept clean and dust levels minimised.

Spill Cleanup

A typical approach for a mercury spill cleanup would include:

- keeping temperatures as low as practicable to limit the rate of vaporization.

- supplying personnel with disposable protective clothing and respirator. For example:

  - an appropriate disposable respirator;

  - disposable overshoes;

  - disposable coveralls;

  - eye protection;

  - disposable gloves;

- checking mercury vapour concentrations with a correctly calibrated mercury vapour meter;

- recovering all visible mercury from surfaces, using the special vacuum cleaner or an approved mercury vacuum pump;

- using sulphur powder to neutralise small spills and a solution of sulphur powder and calcium hydroxide for washing down surfaces.

Consignment

Whenever consigning mercury, the following details should be provided on the outside of the package using the approved label for the type of transport:

UN No.2809

Technical name: Mercury

Dangerous Goods Class: 8 (Corrosive)

HAZCHEIVI: 2Z

Note:

- Both IMO and the International Air Transport Association (IATA) have regulations covering the transportation of mercury.

#### Paints

Aids to Navigation authorities use a significant quantity and variety of paints and related surfacing materials.  There is potential for hazardous situations to arise and for environmental pollution.  For example:

- storage of inflammable paints and solvents;

- during surface preparation and removal of paint prior to repainting;

- contact with vapours and solvents during application;

- clean-up and waste disposal.

#### Lead

Lead based paints have been widely used in the past, but are now restricted or prohibited in some countries.  Authorities maintaining older lighthouses are likely to be faced, at some stage, with having to remove lead based paints and dispose of the waste.

Members are encouraged to assess the risks and to adopt appropriate measures to safeguard maintenance personnel and the environment.

#### Anti-Fouling Coatings

Antifouling paints contain biocides and are applied to vessels and floating aids to navigation to reduce the accumulation of marine organisms.

For service vessels the antifouling paint assists to minimise fuel consumption.

On buoys and lightvessels the build-up of marine growth is not particularly detrimental.  In view of the concentration of these types of aids to navigation in port approaches and internal waterways, less toxic paint systems may be preferred to minimise environmental pollution.

A particular group of antifouling paints using Tributyltin (TBT) has come under scrutiny in recent years due to its adverse affects on the genetics of marine shellfish and findings on TBT concentrations in waterway sediments.  The IMO Marine Environmental Protection Committee is studying a proposal for these antifouling paints to be phased out over a ten-year interval.

    - IALA “PHL” Advisory Panel

The IALA Advisory Panel on the Preservation of Lighthouses, Aids to Navigation, and Related Equipment of Historic Interest (PHL) was established by the IALA Council in 1996 in response to membership interest in the cultural value of lighthouses.

The objectives of the Panel are to:

- encourage deeper commitment by members to preserve historic values;

- encourage member countries to see the preservation of their own lighthouses in an international context;

- share information on the subject between both members and non-members, with particular attention being given to the alternative use of lighthouses.

The work of the Panel has produced:

- the format of an IALA database for recording details of historic lighthouses;

- a book, titled “Lighthouses of the World” was published in 1998 with English, French, German and Spanish versions.  It features over 180 historic lighthouses from around the world;

- a Workshop to gather information on the range of alternative uses of lighthouses.

Future work for the Panel will be directed at preparing two IALA Guidelines that will address:

- the technical aspects of operating and maintaining historic lighthouses, and;

- the Policy, Planning and Practical aspects of preserving historic lighthouses.

    - Lens Size and Terminology

The following table provides information on terminology for historical glass lens systems and the typical amount of mercury held in mercury bath pedestals (for rotating lens systems).

Terminology for historical glass lens systems and associated quantities of mercury used in rotating lens systems.

| Description | Focal distance | Typical quantity of mercury for mercury-bath pedestals |  |
| --- | --- | --- | --- |
|  | mm | kilograms | litres |
| Hyper-radial | 1330 |  |  |
| Meso-radial | 1125 |  |  |
| First Order | 920 | 175 | 12.9 |
| Second Order | 700 | 126 | 9.3 |
| Third Order | 500 | 105 | 7.7 |
| Small Third Order | 375 | 96 | 7.0 |
| Fourth Order | 250 |  |  |
| Fifth Order | 187.5 |  |  |
| Sixth Order | 150 |  |  |

    - Third Party Access To Aids To Navigation Sites

In 1998, IALA conducted a survey to investigate the extent to which Authorities were permitting aids to navigation sites to be used for collecting “non-aids to navigation” data.  This study was associated with investigations of the Advisory Panel on the Preservation of Historic Lighthouses into alternative uses of lighthouses and other aids to navigation.

The responses came from a wide range of IALA members and shared several common themes:

- the predominant applications were for the collection of meteorological data (i.e. weather, wind speed and direction), tidal/ current data and for telecommunication installations;

- data collected for or by other governmental agencies generally did not attract a fees, but fees were often charged for data obtained for commercial purposes;

- data acquisition equipment had to have its own separate power supply unless that aids to navigation site had mains power available.

IALA acknowledges that Authorities face an increased demand to share aids to navigation sites with  “third parties”.  While it is important to ensure that the integrity and security of aids to navigation are maintained, the presence of a third party may be beneficial:

- in reducing the risk of vandalism;

- as a source of revenue or sharing of operational costs (eg power, road maintenance etc);

- as a means of monitoring the operation of the aid.

If an Authority receives a request for a third party installation, it should first establish whether such involvement is permitted in the Authority’s legislation.  If there are no impediments the Authority may consider negotiating an agreement with potential third party to clearly establish the responsibilities and liabilities of each party.  The agreement may also address:

- conditions to apply to the third party installation and operation to ensure that the equipment does not compromise the integrity and security of the aids to navigation and other property owned by the Authority;

- access to electrical power.  At sites with mains power, it may advisable for the Authority to require separate metering of the third-party supply so that electricity costs can be recovered;

- if no mains power is available, it is reasonable to require that the third-party provide its own power supply;

- where practical, the installation of the third-party equipment should take into consideration and preserve the heritage value of the aid to navigation.

Authorities should reserve the right to cancel any third party agreement if continued use jeopardizes the performance or functionality of the aid to navigation.

## Human Resources Issues

    - Employer Responsibilities

Aids to navigation authorities should ensure that all employees have the knowledge, skills and training to perform their duties effectively, and with safety.  The term, ‘employees’ includes newly hired, part time and temporary employees.

The ISO 9001 Quality Management standard places considerable emphasis on competence, awareness and training.  (See Section 6.3.7.4)

#### Source of Skills

Skill development processes for aids to navigation work.

| Skill | Process |
| --- | --- |
| Education | - school - tertiary institution |
| Experience | - work experience - related work experience |
| Training | - induction training - on-the-job training - apprenticeships - specific training programs - refresher courses |

#### Training for Maintenance Personnel

A number of IALA surveys have indicated that, in some areas, maintenance personnel lack the training to perform their tasks effectively.  The situation can be addressed by:

- the authority developing a written maintenance philosophy;

- carrying out a skills audit to identify the gaps between available skills and skills required for the various maintenance tasks;

- arranging for training programs to fill the gaps, noting that:

  - modular training courses are available or can be adapted to the particular needs of each trainee or group of trainees;

  - computer based training and distance learning methods are a useful ways of achieving continuity of training when personnel are also engaged in field activities.  Some adjustment to work schedules would be necessary to allow for training time;

- using training courses are accredited with a recognised institution.  This has several benefits:

  - a trainee may be more highly motivated if he/she can see accredited courses leading on to a formal qualification (ie. career path prospects);

  - accredited courses are “portable” and are of benefit to those changing jobs;

  - recognised courses could be referenced in position descriptions to broadening the range of applicants for job vacancies.

#### Occupational Health & Safety

- To be developed by Pedro Rodriguez from Venezuela & JFRebollo

IMO Resolution A.915(22), Appendix 1.

IMO Resolution A.915(22) includes in its Appendix 2 the requirement for an accuracy of 10 m on ocean navigation, while IMO Resolution A.953(23) mentions that “where a radionavigation system is used to assist in the navigation of ships in ocean waters, the system should provide [an accuracy of] 100 m with a probability of 95%”.

SOLAS Chapter V Regulation 2

These have the same potential gravity at each point.

It should be noted that elevations of land features on maps are generally referenced to Mean Sea Level

The World geodetic system (WGS) is a consistent set of parameters for describing the size and shape of the earth, positions of a network of points with respect to the centre of mass of the Earth, transformations from major geodetic data and the potential of the Earth. (IMO Resolution A860(20)).

Examples taken from Australian Charts.

The chart scale is generally referenced to a particular latitude eg. 1:300,000 at lat 27º 15’ S.

This information may be helpful in assessing the practical accuracy requirements for laying buoy moorings.

Blue surface colours may be used in inland waterways, estuaries and harbours where the colours may be seen at close range.  See Recommendation E108.

The CIE web site address is: http://www.cie.co.at/cie

That is, a magnifying power of 7 and an objective lens of 50 mm diameter.

The steradian is the equivalent in solid geometry to the definition of radian in plane geometry.  The steradian is defined as the unit of measure of a solid angle with its vertex at the centre of a sphere and enclosing an area of the spherical surface equal to that of a square with sides equal in length to the radius.  There are 4π steradians in a sphere.

This condition is to be met at the outer limits of the useful segment for the minimum meteorological visibility under which the leading lights are to be used.

The term “long flash”, which is used in the descriptions of the long-flashing light and of the light characters reserved for south cardinal marks, means an appearance of light of not less than 2 seconds duration. The term “short flash” is not commonly used and does not appear in the Classification. If an Authority requires discrimination between two flashing lights that only differ in having flashes of different durations, then the longer flash should be described as “long flash” and be of not less than 2 seconds duration, and the shorter flash may be described as “short flash” and should be of not more rhythmic character of such a light is than one third of the duration of the longer flash.

A single fixed light shall not be used on a mark within the scope of the IALA Maritime Buoyage System because it may not be recognized as an aid to navigation light.

A Morse Code white light with the single character "U" is assigned to offshore structures.

The timing of astronomical events can also be applied to calculations (computer programs) for sizing solar power supplies.

Recommendation on the determination of the luminous intensity of a marine aid to navigation light December 1977

The most common of the alternatives is a meteorological visibility of 20 nautical miles.

In these situations the beacon will generally show a colour scheme and topmarks in accordance with the IALA Maritime Buoyage System.

Also known as a Port Entry Light (PEL).

See also IALA Recommendation E 110, The Rhythmic Character of Lights on Aids to Navigation, Paragraph 2.4.

For example, in waterways where the aid may be drifting or destroyed due to ice conditions.

The direction of buoyage is defined generally as inbound (from seaward), and clockwise around the land-mass. See MBS Section 2.1.

GNSS has rendered systems such as Decca, Rana and Toran obsolete and the information that was presented on these systems in earlier editions of the NAVGUIDE has been omitted from this edition.

Or more precisely, 283.5 - 315 kHz in Region 1 and 285 to 325 kHz in Regions 2 and 3.

The data is from “Radar reflectors, Radar Beacons and Transponders as Aids to Navigation”, McGeoch & Stawell, Journal of Navigation Vol. 40 No. 3 September 1987. Royal Institute of Navigation.

“Active Radar Target Enhancers For Buoy Signature Enhancement” ; Commissioners of Irish Lights.

The accuracy is dependent upon the Geometric Dilution of Precision (GDOP) at the user's location, the measurement error (signal-to-noise ratio) and chart or local area calibration. The application of ASF (additional Secondary Factors) corrections can improve positional accuracy.

See Annex, paragraph 2.

The Standard Positioning Service is the one available to civilian and commercial users.  A Precise Positioning Service (PPS) is also provided for the US Military Services.

A 1kW transmitter will generally allow position fixing to better than 10 metres over a radius of about 200 nautical miles.

At the time of printing, there are more than 400 stations listed worldwide either in service or planned.

SOLAS V/13 requires each contracting government to provide, as it deems practical and necessary either individually or in co-operation with other contracting governments, such aids to navigation as the volume of traffic justifies and the degree of risk requires.

This applies to the computed and displayed position data, but not to the update rate of correction data, which remains valid for approximately 30 s.

Calculated in accordance with IMO A.953(23) and current IALA Guidelines.

SOLAS V/13 requires each contracting government to provide, as it deems practical and necessary either individually or in co-operation with other contracting governments, such aids to navigation as the volume of traffic justifies and the degree of risk requires.

This applies to the computed and displayed position data, but not to the update rate of correction data, which remains valid for approximately 30 seconds.

IMO Resolution MSC.74 (69) Annex 3 refers.

Although the description of this manoeuvring condition may appear illogical, it caters for situations where a vessel is moored but is free to swing with the tide and wind.  Other circumstances are where the master of the vessel fails to manually change the NavStatus of the AIS after getting under way or where a vessel has broken away from its mooring and is drifting.

SOLAS V provides for exemptions to ships being taken out of service within two years of the AIS implementation.

Also see the list of standard descriptions for the status of aids to navigation at Section 10.3.3.

Verify the info Refer to IMO Resolution A.485(XII) and NAV 46.

Ships’ routeing is covered by Regulation 10 in the new Chapter V that comes into effect on 1 July 2002.

Also known as Narrow Band Direct Printing, or NBDP.

The chemistry of the sea-water cell and the prototype light buoys using this cell have been described in papers for the 1990 IALA Conference and IALABATT 2 and 3.

Refer to IALABatt 3  “Fuel cells for aids to navigation”.

Table adapted from IALABATT 2 Overview of PV Battery Charging, Hyperion Energy Systems Ltd, Ireland.

Example include Canada and Australia / New Zealand.

Or any nominated system or component.

IMO Resolution A.860(20) Appendix 1.

Irrespective of whether or not the increased availability is required by the mariner.

This is the same definition as “mission reliability”.

Some aids to navigation are in reality monuments to historical accidents and are of little value to modern shipping.

For example with national, state, territory, and local governmental bodies and international organisations.

The quantity of mercury used in higher order optics is shown in Section 6.4.3.

The US Environmental Protection Agency has a website reference to mercury:  www.epa.gov/mercury

The 3M respirator type 9008 has been used for this purpose.