20ter, rue Schnapper     -     78100     Saint Germain en Laye        -       France

Telephone +33 1 34 51 70 0 Telefax +33 1 34 51 82 05 Telex 695499 ialaism f

E-mail          :         iala-aism@wanadoo.fd    Internet      :      hjttp://iala-aism.org

IALA Guideline No. XXXX

On

[Contracting Aids to Navigation Services]

DRAFT Edition 1

[Date approved by Council]

AISM Association of Internationale de Signalisation Maritime  IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

IALA Guidelines on Contracting Aids to Navigation Services

# INTRODUCTION

For a number of decades, some national authorities have utilised private organizations for the delivery and maintenance of aids to navigation (AtoN) services. Contracting out is performed in different degrees and mainly in the field of conventional Aids to Navigation (or traditional Aids to Navigation). Although the majority of AtoN services are done by National Authorities that are governmental bodies, some of them have adopted fairly advanced forms of  contracting out.

IALA has issued documents on this subject in 1994 and 1998 named “IALA Guidelines on Contracting out”. This current document intends to revise and update these former documents.

The purpose of this document is not to propose any preferential form of operation for the Authority to undertake, but to identify the characteristics, advantages and disadvantages of different levels and types of contracting out.

In the process of contracting out, several steps should be distinguished:

    - Step 1: Assessing whether to outsource part of the service

    - Step 2: Selection of the contractor and the type of contract

    - Step 3: Contract management

Every organisation considering contracting out should have a clear understanding of what is to be achieved by delivering the service through optional means as opposed to the current delivery mechanism. These are factors that will first justify transferring the service from the current delivery mechanism to an optional one, and later contribute to determining if the contracting out experience was successful.

Several objectives can be taken into consideration in the analysis of contracting out, for example:

        - Quality of the service:

            - Service offered to mariners

            - Reliability and continuity of the service

            - Response to outages

            - Aids in place at the right time

        - Physical proximity of contractors with clients

        - Flexibility and innovation from the contractor

        - Implementation of new technologies

        - Risk Management

        - Costs of the service:

            - Reduction of costs

            - Cost-effectiveness

            - Financial risk

            - Avaibility of cost recovery exercise

# Definitions / Acronyms / Initialisms

  - National Competent Authorities are defined as the governmental entity authorized to regulate and administer the aids to navigation program.

  - Responsible Party is the entity with the responsibility to establish aids to navigation. This may be the National Competent Authority or other parties.

  - The Service Provider is the entity which provides the aids to navigation service.

  - Contracting out of aids to navigation (AtoN) services

Any contract between the Authority responsible for the Aids to Navigation system and the private sector for doing tasks on aids to navigation under the responsibility of the authority. Contracting out can be one option exercised by a responsible authority as a means of delivering all or a portion of their services, however there are variety of options ranging from the authority continuing to carry out all the work using its own resources to fully privatizing service delivery. There are other possible arrangements such as partnerships and alliances with private organizations, formation of wholly or partly owned companies within an agency and the possibility of contracting in additional work of similar nature to, or compatible with an agency core business.

  - Servicing

The word “servicing” is used in a broad sense and includes some or all of the following tasks:

    - Monitoring and*or checking of the aids for proper operation, characteristic and position

    - replacement of components (e.g bulbs, lenses, flashers, changers, etc)

    - positioning buoys on station and lifting or moving them as required

  - Maintenance

Maintenance means the repair and painting of the fixed and/or floating aids included in the contract

  - Design and plan of AtoN system

# CONSIDERATION FOR CONTRACTING OUT

It should be taken in account the responsibility, liability and accountability of the Authority with respect to the Aids to Navigation systems.

## Consideration on SOLAS

According to existing laws, the responsibility towards the international community always remains with the contracting Government as signatory of the SOLAS Convention, notwithstanding the type of organization. The accountability of government cannot be contracted out due to the fact that it provides aids to navigation as signatory of the SOLAS Convention

## Legal mandate of the Authority

## Risk Management

(It would be good to add a reference to the IALA Risk Management Guidelines)

# STEP 1:  TAKING THE DECISION OF CONTRACTING OUT

Each country and/or organization should have its own internal or external procedures for advance in the decision of contracting out. The decision is not only technical or economical but also has considerations from the administrative, social, legal and mainly service point of view. The following steps have been extracted from the Canadian Coast Guard procedure (Ref. 1) and can be used as a guide for fulfilling this step.

## Screen for Opportunities

To identify the services, systems, information, resources, processes, activities, tasks, etc (hereafter “services”) that may benefit from being delivered in an alternate way, i.e. contracted out. At the end of this step, a brief list of services and the reasons why they were chosen should be produced.

## Define the Performance Requirements

To define client expectations, service standards and required outputs for a refined set of activities that are under consideration. This step will produce a concise statement of processes and activities and a set of related service standards and required outputs. It should be clearly stated what terms and conditions are important to delivering this service, i.e. availability, level of service, response time, etc. It is advisable to use the current service standard as the starting point for the review. It should also be kept in mind that highly specialized requirements will have the effect of severely limiting the number of future contractors.

## Build the Team and Project Resources

To build a team that would be able to make an informed and well considered recommendation supported by a strong case for a preferred delivery alternative.

## Develop the Communication Plan

To develop a plan for communicating to employees, unions (if applicable), clients, management, central agencies and other stakeholders the intentions of, process, progress and results of the Alternative Service Delivery review.

## Choose the Alternatives

To identify a variety of service delivery alternatives or options that merit review and analysis.

## Construct the Assessment Framework

To provide a simple, complete framework for assessing the feasibility of the Alternative Service Delivery options arising from step 5. This steps should consider the impacts of contracting out in the whole organization, which includes the following:

  - Human resources/Work force adjustment considering the knowledge base within the Competent Authority. Whatever solution is considered, a procedure has to be implemented to ensure the National authority  expertise and contracting power. Adequate resources  need to be allocated to manage contracting out and keeping personnel motivated

  - Internal resources

    - Existing contractual agreements

    - Asset management

    - Revenues

  - Clients and stakeholders

    - Levels of service

    - Client expectations and satisfaction

    - User pay/user say

  - Services posture and operations

  - Public policies and governance

    - Public policy requirements

    - Legal and regulatory framework

    - Long term risks

## Do the Cost Analysis

To compare a chosen set of alternatives on the basis of revenues, costs, cost recovery and financial risks.

## Do the Qualitative Comparison

To compare chosen alternatives on the basis of qualitative factors: strategic fit, client satisfaction, operational performance and risks, and ease of implementation.

## Assess Potential Service Provider Capabilities

To compare alternatives on the basis of service provider capabilities, understanding that delivery options may be provided by a range of parties, including clients and private sector, public sector, etc. It should consider that, in some cases, the Government Authority may be the contractor.

## Develop a Recommendation

To develop a well considered, transparent recommendation of a preferred service delivery option

## Validate the Recommendation

To confirm the soundness and durability of the recommendation for a preferred service delivery

## Seek the “Go-No Go” Decision

To obtain the advice and approval of Senior Management to proceed on with implementation of the recommended delivery option or how to improve your recommendation

# STEP 2: SELECTION OF THE CONTRACTOR AND TYPE OF CONTRACT

## Selection of  a contractor

The criteria for selection of successful contractors must be carefully considered and cover such issues as technical and managerial competency of contractors, their likely financial viability over the term of the contract and the adequacy of their quality and safety management.

## Type of contract

A lump sum contract with a strict control from the Client avoids a lot of discussions. If the objectives are clear and well defined the Contractor has no excuses to fulfill the objectives.

It is very difficult to control all aspects related to the good maintenance of an AtoN. And it requires a lot of people from the side of the Client and this is just the contrary of the basic idea of contracting a third Company to do the job. Therefore the best way of having good results and not having permanent discussions is that all responsibility for the AtoN is on the side of the Contractor and the money paid in any way for the job is a fixed amount. In this case the control activities can be very specific and can be done with few very capable people. On the other side, the Client knows in advance how much money is going to spend.

The contract specifications could be performance based and have adequate performance measures that are objective and easily measured or observed. There should be rewards for good performance and penalties for inadequate performance

Due to the difficulties of establishing the initial conditions of a range of equipment from structures to batteries (particularly for remote aids to navigation) it is not simple to get contractors to accept performance based contracts

A balance should be achieved related to risks. If the contracting government agency retains all the risk, the contractor can be in a position of being rewarded for poor performance, however contracting out all the risk can lead contractors to build large risk premiums in their contract prices

## Clauses of the contract

### Object of the contract

The object of the contract is to be very well specified and should consist of operational targets and service level measures. All future daily work planning should remain in the Contractor side.

### Extension of the work

This is a very important point. The number of AtoN and the extension of the area to be attended by the Contractor should have a minimum size. This is related to the number of buoy tenders that should have the Company. Minimum number is one buoy tender but to cope with time for repairs, classification and other non-working time it is very convenient that the work could justify more than one buoy tender. When the work does not require one full buoy tender it is not easy to find additional work for this ship resulting in higher costs for the service. The same applies to the number of AtoN. There is a minimum number that justifies having enough spare parts of everything and competent people in the yard.

### Duration of the contract

There are many decisions that depend on the duration of the contract. It concerns people, ships and material. The duration of the contract should justify the investment in good quality elements for the AtoNs.. Also there is the influence of the environmental factors. For the evaluation of the maintenance costs it should be taken into consideration that meteorological conditions are not the same year after year. The evaluation for shorter periods implies that worse conditions are considered always implying higher costs. With longer periods the bad years are somewhat compensated with the good ones.

The period of the contract must be carefully considered; if too short, it will discourage some contractors from tendering due to high set up and tendering costs, which must be amortized over the life of the contract, but if too long, may not result in full advantage being obtained from the competitive tendering process: five years could be considered a reasonable term except where major plant e.g. ships are necessary.

### Buoy tenders

The provision of buoy tenders can be done by the contractor or by the Authority. It is very interesting and convenient to find a formula for letting the Contractor use the buoy tenders of the Authority if possible. The Contractor should have the obligation of keeping the buoy tenders in good condition and under class and the Authority should have a good control schedule of the conditions of the ships. If not used the buoy tenders should return to the Authority. The buoy tenders should not be used in other activities out of the AtoN service without the written permission of the Authority. This solution saves a lot of money from the Contractor and from the Authority. From the Contractor because is not easy to find appropriate ships for this type of job. For the Authority because it is possible that he has not any more work for the ship and in this way preserves the possibility of returning later to the previous system.

### Personnel

To solve adequately the problem of personnel is a condition for the successful result of the pass from the public administration to the private sector. One adequate way of doing this is to promote the incorporation of personnel from the administration in the contractor.

The contract should specify core skills and competencies required by the contractor

Personnel can be divided in four sectors: a) mariners and technical staff on buoy tenders, b) technical people at the yard, c) technical people at office and d) managerial sectors.  In general it is possible to include as condition in the TOR the inclusion of an important quantity of people from sectors a) and b) and a minimum quantity from sector c). People from sectors c) and d) are necessary for the control of the contract

### Technical specifications

In normal circumstances AtoN systems are already installed in reasonably good state of maintenance. It is natural for  people working in that system to consider it the best solution achievable. Nevertheless, there are always fields where new technologies are emerging that could be worth use, trying to improve the quality or the economic equation of the system.

The Client should specify clearly the objectives to be obtained with the AtoN system but leaving the implementation of the same, with the adequate discussions and approvals, to the Contractor. In a 5 to 10-year contract it is reasonable to leave open the possibility of introducing changes to the established way of doing things.

### Performance Incentatives

It should be established the required level of the system according to the real needs of the mariners. Higher levels of service are always obtainable but that involves always higher costs. Penalties should be applied in case of not fulfilling the established standards.

The contract should specify mandatory and minimum performance criteria.  Performance in excess, can be rewarded with incentives if thought appropriate by the contracting authority.

### Payments – Income

It is worth analyzing the possibility of putting part or total cost of the work on passing ships through a toll. It depends on each country legislation and uses.

When passing ships are involved in the cost of maintaining the AtoN there is an additional control on the quality of the system from the passing ships and pilots.

# STEP 3: CONTRACT MANAGEMENT

Navigation Authorities. Following are the main phases of contract management to consider when performing contracting out activities (Ref. 2)

## Transition phase

The first phase of contract management, the transition period or implementation, commences from the time the supplier is selected and runs through to a time agreed by contracting parties. A strategy for smooth transition from the old arrangements to the new arrangements should be developed. The objectives of the transition phase are to establish a strategy to manage the transition to contracted service delivery, minimizing the chances of a loss of service delivery and the impact on clients and other stakeholders, including internal employees.

## On-going management

Navigation authorities should implement actions items for the inspection, monitoring and auditing the delivery of the service by the contractor. Effective management requires sound planning and risk assessment; the allocation of adequate resources; a proactive approach supported by clear procedures; regular monitoring; and a comprehensive reporting and analysis to measure achievement against the stated objectives.

## Aid Availability

The contract should specify in details the reliability standards to be delivered to the mariners by each of the aids to navigation under its responsibility. On a similar note, the contractor should not determine the level of service to be provided to the mariner. The level of service should, in most cases, be agreed on between the National Authority and the mariners.

## Notices to Mariners Process

Since the Notice to mariners process should be the means employed by National Autorities to inform mariners of malfunctioning of aids to navigation, the contract should specify that the contractors must use this process to inform mariners of any outages of the aids to navigation under its responsibility.

## Flexibility of contracts ????

Raul, What do you  mean ? What do you want to talk about?

## Insurance requirements for the contractor, public liability

In the case where the aids to navigation covered by the contract are still owned by the National Authority, the National Authority could be held liable to any third party injured as a result of negligence attributable to the contractor in the execution of the contract.  Safeguards, such as the “save harmless” clause and the requirement to hold liability insurance, could be included in the contract. The decision not to demand these safeguards must be taken into consideration when conducting a risk management analysis. Another way to reduce the likelihood of the National Authority’s being held liable is to inspect the work performed by its contractor, in accordance with the description of the work as stated in the contract, by having the contractor submit maintenance reports to the National Authority. Lastly, the scope and frequency of inspection of the contractor’s work are criteria that should be considered in the risk management process. This inspection could be conducted through a statistically random control mechanism based on the knowledge and experience of the contractor.

## Contract succession – termination of contract and continuation of service

This phase of the contract management lifecycle looks at ensuring a smooth succession phase at the end of the contract period to ensure minimal impact on the business of the Authority. This phase may involve undoing all the arrangements previously in place to allow for a completely new service provider to take over, or to bring the service delivery back in house. The objectives of this phase is to ensure that there is a smooth, efficient and effective transition at the end of the contract for either an extension of the existing contract with any revised service levels, for selection of a new provider or for returning the services to in-house provision.

The technical conditions at which the AtoN system should be returned to the Authority should be clearly specified in the TOR and they should be reasonable. For example, the buoys of the system are taken out of the water to receive full clean up, reparation and painting each a number of years. If it is established in the TOR that at the time of returning the system  all buoys should have received such treatment in a period not exceeding one or two years of the date of ending of the contract it puts a very difficult to fulfill obligation on the Contractor.

# EVALUATION OF EFFECTIVENESS OF CONTRACTING EFFORT

I don’t know what to write here, for the moment at least. I will do some research on my side too. (AC)

# POTENTIAL ADVANTAGES AND DISADVANTAGES OF CONTRACTING OUT (FROM PREVIOUS GUIDELINE)

## Advantages

- May be more cost effective, with better financial control, and more flexibility in use of resources and staff motivation

        - May be politically more acceptable than having governmental agencies provide services directly

        - Relieves government of the burden of capital investment in plant and equipment

        - Reduces the number of civil servants, whenever it is a goal

        - Private companies may be more innovative in operational and practical matters

## Disadvantages (from PREVIOUS GUIDELINE)

- Less clear attribution of responsibility because of sharing  work between authority and contractor

- Difficult for National authority to retain sufficient expertise over time to adequately monitor contractor performance

- Private companies gain a strong position in price negotiation for emergency work or contract renewal once agencies loose the resources to do the work themselves

- The combined cost of contract provision of the service and adequate monitoring of contractor performance may not be as cost effective as retaining the function in house in some cases

- It may not be possible to reduce staff resources coincident with contracting out so as to immediately realise the potential savings

- In case of changing contractor (bankruptcy for instance) continuity of service can be affected and increase costs incurred

- Possible creation of a private monopoly position

- Possible demotivation of staff

# CONCLUSIONS

The change of system from a public administration to a private one in the subject of AtoN is not easy and involves technical, economic, social and political decisions. This process is usually long and should be done very carefully to avoid unnecessary drawbacks.

The subject is very much dependent on case by case due to local geographical conditions, applicable laws, general political strategy, and sometimes to a lot of non technical issues.

# References