IALA Recommendation  XXXX

On

Emergency Wreck Marking Buoy

Edition #

[Date approved by Council]

[Date of previous edition]

AISM Association of Internationale de Signalisation Maritime  IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone +33 1 34 51 70 0 Telefax +33 1 34 51 82 05

E-mail - iala-aism@wanadoo.fr      		Internet -  www.iala-aism.org

IALA Recommendation on Emergency Wreck Marking Buoy

Recommendation XXXX

THE COUNCIL:

NOTING the function of IALA with respect to the safety of marine navigation, the efficiency of maritime transport and the protection of the environment;

NOTING ALSO the provisions contained within the IALA Maritime Buoyage System (MBS), and related IALA Recommendations and IALA Guidelines;

RECOGNISING the significant hazard to shipping posed by new wrecks or obstructions;

RECOGNISING ALSO that it is a matter for a National Authority to assess the danger to shipping, navigational requirement, the risk involved, and to decide on emergency wreck marking;

RECOGNISING FURTHER that emergency marking of dangerous wrecks is intended to preserve the safety of life, safety of navigation and to protect the marine environment;

HAVING CONSIDERED the proposals by the IALA Aids to Navigation Management Committee and taking into account IALA Guideline No. 1046 : Response Plan for the Marking of New Wrecks,

ADOPTS the “Emergency Wreck Marking Buoy”, set out in the Annex to this Recommendation

RECOMMENDS that Responsible Authorities consider the deployment of Emergency Wreck Marking Buoys, as described in the Annex to this Recommendation, for initial marking of new dangers in conjunction with other measures.

- .

* * * * * *

ANNEX – Emergency Wreck Marking Buoy

# INTRODUCTION

The wreck of the ‘Tricolor’ in the Dover Straits in 2002 has brought into sharp focus the effective responses required to adequately and quickly mark such new dangers and prevent collisions. Responsible Authorities need to assess their areas of responsibility and rapid response capability as part of their contingency planning.

The IALA Guideline No.1046 Response Plan for the Marking of New Wrecks provides guidance to Authorities for an immediate, effective and well co-ordinated response that can be given in such a situation. The guidelines recommend procedures to be observed, as well as considerations to be taken into account with respect to all necessary measures when confronted with a new danger or an obstruction as a result of an incident within their area of responsibility.

Furthermore, there has been considerable unease as to whether the IALA Maritime Buoyage System is sufficient to adequately mark new dangers in the first instance. At present new dangers are generally marked by cardinal or lateral buoys although it is recognised that a number of Authorities also deploy isolated danger marks. Recent groundings and collisions have indicated a need for a revision of how new dangers are to be marked especially in an emergency situation and the issues to be considered. As such, the Plan gives guidance and provides recommendations with regard to emergency wreck marking to the authorities involved.

# Scope & Objectives

The Recommendation provides details of a new buoy configuration, in addition to that already found in the IALA Maritime Buoyage System that Authorities may consider initially deploying in the event of a new danger or obstruction.

# CONSIDERATIONS

A new wreck can be very dangerous for shipping. Not only when its position is not exactly known and is still unmarked, but even when the position is known and the wreck is properly marked.  In the past new wrecks have caused problems to other shipping resulting in damage, pollution and even loss of life. As detailed in the IALA Guideline No.1046 Response Plan for the Marking of New Wrecks, Authorities should consider a range of responses including the deployment of guardships, AIS, temporary VTS and buoyage amongst other risk mitigation measures.

Whatever additional risk mitigation measures are initiated, a new danger must be physically marked.  Weather conditions, sea state and unknown facts about the danger can all hamper timely marking.  Irrespective of these circumstances it is of great importance that the location of the danger is marked as soon as reasonably practicable and that this marking can be readily recognised by ships as a new hazard.

The volume of traffic, background lighting and proliferation of Aids to Navigation in the area may make the deployment of cardinal or lateral marks difficult for mariners to quickly identify a new danger in the initial stages of an incident. Authorities are invited to consider the deployment of an emergency wreck marking buoy that is specifically designed to mark new dangers.

# EMERGENCY WRECK MARKING BUOY

The emergency wreck-marking buoy is designed to provide high visual and radio aid to navigation recognition.  It should be placed as close to the wreck as possible, or in a pattern around the wreck, and within any other marks that may be subsequently deployed.

The emergency wreck marking buoy should be maintained in position until:

- The wreck is well known and has been promulgated in nautical publications

- The wreck has been fully surveyed and exact details such as position and least depth above the wreck are known.

- A permanent form of marking of the wreck has been carried out.

## Characteristics

The buoy is to be a pillar or spar buoy, the size of which will be dependant on location.

This buoy is to be coloured in equal number and dimension of blue and yellow vertical stripes (minimum 4 and maximum 8).

It is to be fitted with an alternating blue* and yellow flashing light with a nominal range of 4 nautical miles (authorities may wish to alter the range depending on local conditions) where the blue and yellow 1 second flashes are alternated with an interval of 0.5 seconds.

B1.0s + 0.5s + Y1.0s + 0.5s  = 3.0s

- If multiple buoys are deployed then the lights should be synchronised.

- Consideration should be given to the use of a racon Morse Code “D” and/or AIS transponder.

- The top mark, if fitted, is to be a standing/upright yellow cross.

*The light characteristic was chosen to eliminate confusion with blue lights to identify law enforcement, security and emergency services.

# Conclusion and recommendation

It is recommended that Authorities consider this distinctive and specific aid to navigation be deployed as the initial means of marking a new danger until a more permanent form of marking of the wreck has been carried out or the danger removed.