| From: Hendrik Eusterbarkey, Germany |  |
| --- | --- |
| To: ANM 8 | 2006 September 6th |

Input paper for point 6 of the provisional agenda

Develop guidance on channel design and a minimum comprehensive mix of AtoN

- Background

A work programme item for the ANM Committee will be to draft a Manual on channel design and a minimum comprehensive mix of AtoN.  Cooperation with the International Navigation Association (PIANC) should be useful, because the PIANC “Approach Channel – A Guide for Design” from June 1997 (Annex 1) is now under review by a PIANC Working Group.

- Preliminary Note

The role of Aids to Navigation and other Traffic Technology Services for the design of navigation channels might be underestimated. A big contribution to economical and ecological optimization of navigation channels by employing an adequate mix of Aids to Navigation and Traffic Technology can be expected.

This input paper deals with the additional widths for straight channel sections, which are given in table 5.2 of the PIANC “Approach Channel – A Guide for Design” from June 1997. I try to compare the additional widths caused by Aids to Navigation – if they are not “excellent”  - with the figures given in the IALA-paper  “SG 3 – 4  - 1, Navigation Accuracy by the use of buoys and leading lights in restricted waterways” (Annex 2).

The IALA paper  was provided by IALA Secretary- General to the IALA Council member of Germany in order to help us with background information for the implementation of IALA Risk Assessment Tools.

As far as I know,

- this paper  is one of the results of some very extensive testing carried out in Portsmouth harbour., and

- the principles of these paper are incorporated into IWRAP.

There is another paper resulting from these testing which might be considered in this context (Annex 3: The provision of buoys, leading lights in waterways and the resultant risk reduction values).

- Action Requested

An IALA manual on channel design and a minimum comprehensive mix of AtoN should also give guidance how to advice the channel planners if they consider to minimize major and maintenance dredging by reducing channel width of navigation channels by employing an adequate mix of Aids to Navigation and Traffic Technology.

In this case – assuming  the wording in table 5.2 of the PIANC “Approach Channel – A Guide for Design” will not change – the question will have to be answered what excellent, good and moderate AtoNs are. This could be estimated by the IALA paper “Navigation Accuracy by the use of buoys and leading lights in restricted waterways”.

In table 1 of this paper (Annex 2) we find the minimum navigational accuracies for different widths of fairways – by the way: this could help us to find the minimum comprehensive mix. But what is an excellent mix of AtoN?

Excellent in the sense of  table 5.2 of the PIANC “Approach Channel – A Guide for Design” is when no additional width is required for the inaccuracy of AtoN; is this a required accuracy of  0 (zero !) metres? I don’t think so. So the first thing we have to do – may be together with PIANC experts – is to find out if there are any hidden reserves in the calculations and tables of the PIANC Guide.

Assuming the excellent  mix of AtoN might be if the navigation accuracy is 10% of the width of fairway we could use table 5 of the in the IALA-paper “Navigation Accuracy by the use of buoys and leading lights in restricted waterways”. This shows us some “all included”-scenario with the “standard number” of buoys, leading lights and Radionavigation. I have to admit that I’m not sure how to deal with the VTS, which is also mentioned in the excellent mix of AtoN in table 5.2 of the PIANC “Approach Channel – A Guide for Design”. The “standard number” of buoys is calculated by simple calculations under 2.1 of the IALA paper “Navigation Accuracy by the use of buoys and leading lights in restricted waterways”.

So we might have already found the “excellent” mix of AtoN. Now for “good” AtoN: This is in the sense of table 5 of the PIANC Guide, when there is an additional space for the inaccuracy of AtoN of 10% of the ships beam B. This means 5% on each side. So we can accept only 5% of B more for the navigational accuracy in metres.

There are different scenarios how to realize the navigational accuracy. Some examples are shown in the table (annex 4), calculated by the formulas and tables of the IALA paper “Navigation Accuracy by the use of buoys and leading lights in restricted waterways”. Using these formulas, there are only small differences between the “excellent” and the “good” mix of A to N in the sense of the PIANC Guide.

ANM 8 should consider if

- this approach is appropriate to define the relevant mix of AtoN as a contribution of  AtoN- and Traffic Technology for the waterways planning experts based on PIANC “Approach Channel – A Guide for Design”,

- especially if we want to use the IALA paper “SG 3 – 4 – 1, Navigation Accuracy by the use of buoys and leading lights in restricted waterways” or if any update or further investigation might be required,

- if we can talk about separation distance and number of buoys without regarding their size, shape, colour and conspicuity (IALA Buoyage System)

- and if we can use the same paper for finding the minimum comprehensive mix of AtoN.