# Fairway buoyage

## Basic principles of buoyage

Two different principles are used for small craft track buoyage: leading marking and edge marking. Usually, fairway buoyage employs a combination of these two principles.

Leading marking means marking the fairway line in nature with leading boards, leading beacons, or similar marks.

Edge marking means marking the edges of the fairway area or maritime obstacles limiting the fairway area in nature with (usually) floating aids to navigation. Edge marking features two approaches: critical shallows marking and fairway area marking.

Hence, fairway area marking means more or less systematic marking of the edges of the fairway area in nature with floating aids to navigation. Usually, this means that a buoy is located at the bend points of the fairway edge line. Fairway area marking is a suitable basis for marking especially when the area to be used by small craft must be marked precisely. This can have several reasons, among the most common of which are that:

- The fairway runs continuously in a narrow and possibly dredged channel.

- The water area has not been surveyed over a broader area.

- Other waterway users (e.g., fishers) need exact fairway area location information.

For the user, fairway area marking is a better optical guide than critical shallows marking, so in principle it is easier to use.

Critical shallows marking refers to a situation in which the fairway area is marked so that each shallows limiting the fairway area is marked with separate aids to navigation. With critical shallows marking, the actual locations that are dangerous to the user are marked instead of the theoretical bend points of the fairway area. Critical shallows marking is better for sailing since it does not unnecessarily limit the usable water area. Moreover, the number of buoys used in critical shallows marking is easier to limit since placing ‘unnecessary’ buoys in full-depth areas is not required.

For the user, critical shallows marking is safer since the dangerous locations are specifically marked while with fairway area marking a dangerous location can be outside the theoretical edge line between two buoys.

In practice, small craft track buoyage in most cases involves a combination and compromise of these two principles, with the focus perhaps being more on critical shallows marking. In the future, when navigation software and chart plotters enter more common use among boaters, fairway area information can be seen directly on a display, decreasing the importance of fairway area marking. Then it is perhaps more important that especially the most critical and dangerous shallows are marked.

### Marking principles, according to user group

Travel on small craft tracks occurs in two main ways, by sailing or by motoring. Due to the considerable differences between the two travelling methods, both groups have different preferences regarding buoyage. A sailor desires as wide a fairway as possible, to enable travelling on the fairway in different wind conditions. Due to the high speed involved, a motor boater wants the buoyage to be as clear as possible, continuous, and visible from far away.

This means that no general instructions can be given for channels for sailing since sailors travel according to their own objectives in full-depth water areas. Small craft track buoyage can be planned according to the needs of motor boaters, supplemented by possible buoyage for sailors' special needs.

A general rule is that on the small craft tracks suitable for sailing (channel depth of more than 1.5 m and vertical clearance of over 12 m), edge marks must principally be located at dangerous rocks and shallows. On open fairway sections suitable for beating and larger open areas, it can be necessary also to mark a dangerous rock even if it is far from the fairway area.

In travel by motor boat, speeds easily can grow high and hence navigation must be as easy as possible since the time for making decisions is short. For safe motor boating it is especially important for the buoyage to be as continuous and clear as possible. Buoyage also should provide as good optical guidance as possible. Buoyage can be made clearer if, for example, the edge buoys are on the same straight line or at the bend points of the edge line.

## Navigation aids used

The aids to navigation used for small craft track buoyage are primarily simple, inexpensive, and low-maintenance devices, such as spars, buoys, cairns, and unlighted leading marks. All of these aids to navigation can be combined relatively freely so that the optimal result is achieved with regard to operation, cost, and safety while specific conditions and landscape issues are taken into consideration.

Whenever possible, clearly visible landmarks and objects, such as masts, towers, chimneys, large boulders, and rocks, should be utilised for buoyage. These landmarks and objects utilised as aids to navigation and buoyage also must be marked on the charts.

As boating takes place mainly during daytime, active lighting of the aids to navigation usually is not required. Due to the high costs and demanding maintenance requirements of lighted aids to navigation, their use should be considered carefully.

The aids to navigation used in small craft tracks are equipped with light reflectors, as are all aids to navigation used in Finland, except rock cairns. Moreover, floating aids to navigation (spars and buoys) are equipped with radar reflectors. Hence, navigating on small craft tracks is possible also at night-time with a hand-held lamp and chart or, when visibility is limited (in fog), via radar and the chart. Small craft track buoyage does not, however, provide as easy navigation during the night as in daytime.

However, the safety equipment used in the aids to navigation, especially for the major small craft tracks (VL4), is such that its use by authorities is as easy and safe as possible. This means that special attention is paid to radar navigation on these fairways and that they are designed to be navigated with on-board light devices also during the night.

The present-day floating aids to navigation keep their location extremely reliably during open water season. If the locations of the floating aids to navigation are checked before each boating season, small craft track buoyage for sheltered areas can well be based fully on the use of floating aids to navigation.

Usually, a fairway can be marked in more than one appropriate way. Cost estimates should be prepared for these alternative marking methods, in which the costs of servicing and maintenance must be taken into particular consideration.

### Spars and buoys

In Finland, the region-A buoyage system of the International Association of Lighthouse Administrations (IALA) is used; this system is described in Annex 1. The system is based on two different methods of marking: lateral marking and cardinal marking. In lateral marking, the fairway is marked with port (red) and starboard (green) side marks according to the specified conventional direction of buoyage. Cardinal marking is based on marks that indicate the side where navigable water lies as a compass direction relative to the mark.

For small craft tracks, mainly lateral marks are used, but cardinal marking is used as well in crossings and other special circumstances. Also, safe water marks are used for some sections of the small craft tracks. The size of the marks is determined by how significant and open the location site is. Usually, the mark diameter is between 160 and 355 mm.

Figure 1. Floating aids to navigation used for small craft tracks (pre-stressed buoy, small craft track buoy, slack mooring buoy)

Three main types of floating aids to navigation are used for small craft tracks: pre-stressed buoys, small craft track buoys, and slack mooring buoys. These types, respectively, are shown from left to right in Figure 1 The diameter of pre-stressed buoys varies from 160 to 500 mm (large buoy). The 160-mm buoy in particular is widely used on boating routes (VL6). The small craft track buoy is an affordable aid to navigation designed specially for small craft tracks, the diameter being from 90 to 160/225 mm. The smallest (90 mm) small craft track buoys are used mainly on inland waters. The slack mooring buoy is clearly heavier and a more expensive solution, 225 or 355 mm in diameter and usually lighted. Slack mooring buoys usually are employed in locations where the visibility of the aids to navigation is especially important or they must be detected from a greater distance than usually.

The difference between a spar and buoy is in the mooring. A spar is moored in a pre-stressed way; that is, its buoyant force keeps the mooring chain tight at all times. By contrast, a buoy is moored with a slack mooring chain so that it floats freely around its mooring location and stays upright through its own internal balance. In addition, the length-to-width ratio of a spar is greater than 5:1. Floating aids to navigation with a lower length-to-width ratio are buoys. When an aid to navigation is shaped like a spar (length-to-width ratio of, for example, 6:1) but is anchored like a buoy (floating freely) it is called a slack mooring buoy. Then, short and wide aids to navigation anchored in a pre-stressed way are known as offshore-type ice buoys.

In approaching the fairway entrance, a narrows, or a similar fairway location with a large number of buoys, detecting the first buoy from the ‘buoy jam’ can be difficult.  In these cases, the first buoy should be selected such that it stands out clearly from the other buoys (for example, with a cardinal mark or a lateral mark that is larger than those of other buoys).  A red lateral mark usually is regarded as the easiest to observe. Table 1 shows theoretical observation distances according to buoy size in moderate visibility conditions.

Table 1. Observation distances of buoys

| Buoy diameter | Observation distance |
| --- | --- |
| 160 mm | 900 m |
| 225 mm | 1,200 m |
| 355 mm | 1,500 m |

All buoys are equipped with light reflectors, which provide optimal reflectivity for night use. Similarly, the buoys feature an integrated radar reflector for radar navigation in poor visibility conditions.

### Cairns

More than 2,200 cairns are in use in Finland. Most of these are on inland waters, and especially used for small craft tracks.

Cairns cannot be used for positioning the vessel in relation to the fairway space. They are intended for coarse positioning to the accuracy of an island and hence help the boater to remain on the correct route. Due to their good visibility, cairns also are used as direction signs for crossing large open areas.

Traditionally, cairns have consisted of piles of rocks, log cribs, or similar structures that were whitewashed or painted white and provided positioning marks for boaters.

Figure 2. Dayboard

For small craft track usage, a cairn constructed of board, called a dayboard and shown in Figure 2, has been developed. A dayboard is supported on legs, and its top board section is reflective so that it can be seen clearly in daytime, and it also is easy to detect at night-time due to its large reflective surface. Dayboards usually have a letter marking to aid in positioning. The letter is indicated on the chart. Separate instructions regarding dimensioning of dayboards have been issued by the Finnish Maritime Administration (FMA Bulletin, issue 15/1984).

### Leading marks

Leading marks are pairs of boards that indicate the fairway line when they are aligned. Sizes of leading marks vary considerably depending on the purpose and location (area of 1 to 200 m2, height of 2 to 40 m). The leading mark pairs are identified with the terms 'upper board' (higher, further from the fairway line) and 'lower board' (lower, closer). Usually, leading marks are constructed of steel or a wooden frame to which a board of sufficient size (determined by the observation distance) is attached. The exact position of the fairway line is indicated on the board by colour (a stripe differing from the background) or via a symbol (for example, a triangle). Figure 3 shows an example of a leading mark.

Figure 3. Leading mark

For small craft tracks, leading marks can be attached to or painted on a suitable existing natural structure.

Of all small craft tracks, leading marks usually are used only for major small craft tracks. However, if the number of edge marks can be reduced significantly (decreasing total buoyage costs) through the use of leading marks, it can be considered also for lower classes of fairway.

The Finnish Maritime Administration has issued instructions on dimensioning and locating leading marks ('Linjalaskenta', FMA, 1998).

## Selection of buoyage system

Careful consideration is required in selecting the buoyage system. Both cardinal and lateral systems have their benefits. Buoyage may never conflict with the existing buoyage, such that, for example, direction of buoyage in lateral system is changed in the middle of the fairway. Hence, cardinal marking is recommended for crossing areas.

According to principles specified by IALA, lateral marking is used for indicating the starboard and port sides of the defined fairway / fairway area, on the basis of the conventional direction of buoyage. Cardinal marks indicate to which side of the mark the deep navigable water lies. In Finland, lateral and cardinal marks are used as follows:

Lateral system is used for

- Indicating the edges of fairways for which the conventional direction of buoyage can be clearly defined and especially when edge marking is relatively frequent.

Cardinal system is used for

- Fairway crossing areas (lateral marks can be interpreted in conflicting ways).

- On fairways for which buoyage is based primarily on marking of single shallow areas and when distances between marks are long (can also apply to sections of fairways using lateral system).

- Marking of single shallows/rocks away from the fairway.

- Marking of so-called widening areas of fairways and other special navigational areas (the fairway otherwise can be laterally marked).

Also, the basic principles for the lateral system's direction of buoyage must be considered when one selects the conventional direction of buoyage: ”The conventional direction of buoyage is always indicated on charts and in navigation instructions. The conventional direction of buoyage is the direction the vessel enters harbour from seaward or the direction towards headwaters. On fairways along the coast, the conventional direction of buoyage is from west to east in the Gulf of Finland, and from south to north in the Finnish Archipelago and the Gulf of Bothnia.”  The dividing line for sea area is at Utö. In practice, the basic principle means that for sea areas the rule is from west to east, and it is from south to north for all areas except for fairways leading to harbours. However, if the conventional direction of buoyage is changed in the middle of the fairway (for example, in the Finnish Archipelago), the changing point must be clearly indicated with either a lateral gate or a cardinal mark.

All in all, conventional directions of buoyage for lateral marking are:

- from seaward to harbour

- towards headwaters

- west to east in the Gulf of Finland

- south to north in the Gulf of Bothnia

In areas with a large number of single buoys – that is, the buoys are not set up as gates – the cardinal system has clear benefits. In navigation of the area without a chart, the information value of a single lateral mark is very low, but a cardinal mark indicates safe water directly. On the other hand, with a fairway using closely spaced buoys, interpreting cardinal marks is considerably slower and the fairway is not as clear as the same fairway with lateral marks. Hence, a relatively frequent lateral marking works better especially for fast travel, but for slower navigation (for example, sailing) using relatively well-spaced cardinal markings has its advantages.

## Buoyage methods – examples

There are some easily adaptable basic buoyage methods to be used for small craft tracks. Below, these are presented by way of examples. These examples are for guidance only and created only for these instructions, not taken from existing locations. In addition, the scale in the examples is only for rough guidance.

### Edge marking

A small craft track can be marked by using only floating aids to navigation (spars and buoys) positioned on the edge lines. Edge marking should be continuous throughout the fairway length, although, whenever required, single cardinal marks (for example, a single mark in the middle of a large open area) can be used for a laterally marked fairway.

In some cases, edge marking must be designed to be both-sided so that the buoys form gates. A gate array improves observation of the buoyage and indicates clearly the available fairway space. Both-sided edge marking is not required for navigation on tracks intended solely for boating. Hence, a gate array should be used only in special cases, including, for example:

- If depth point surveys or sweepings are carried out for limited areas, a gate array can be more cost-effective than expansion of the survey area.

- If the fairway space must be limited to be exceptionally narrow compared with the rest of the fairway for some reason (dredged narrows, fishing ground, rock, etc.).

Figure 4. Principle of edge marking

Figure 4 shows an example of a single-edge marking solution: in a relatively shallow and rocky area, a gate array has to be used since the fairway is very narrow. Otherwise, the only lateral marks are used by positioning marks at bend points or points where critical shallows come close to the fairway area.

The situation in Figure 4 is exceptional also since the area is shallow and rocky, causing the fairway to be very narrow. Edge lines are indicated in the figure (via a dashed line) to illustrate the principle of edge marking.

### Edge marking supplemented with cairns

Although a small craft track usually can be marked by using only buoys, the use of supplementing cairns can be justified in many cases. Cairns can be seen from far away, ice does not move or break them, and they can be used in confirming position and direction of buoyage. Also, positioning of buoys is easier with the help of cairns. Figure 5 shows an example in which cairns are used for supplementing edge marking.

Although the use of cairns seldom eliminates the need for buoys, supplementing edge marking with cairns can be justified. On the other hand, if there are no critical locations near the fairway area, cairns can be used for thinning out the number of buoys, due to the better visibility of cairns. Moreover, cairns can effectively indicate, for example, direction of buoyage (a cairn as a bow mark), the point of an island, or a rock by which the fairway passes. Also, a diverging point on a fairway can be indicated with a cairn. In the long run, cairns can save on service and maintenance costs since the maintenance interval for cairns is significantly longer than that for buoys.

Figure 5. Edge marking supplemented by cairns

Figure 5 shows a situation in which both lateral and cardinal marking are used, supplemented with two cairns. At the crossing area of the two fairways where lateral marking is not possible, a south cardinal is used. It indicates the crossing area clearly and is not open to interpretations as to the side on which the buoy should be passed.

### Leading marking

Small craft track buoyage is based primarily on edge marking.  Leading marks are used mainly for major small craft tracks. Since the small craft tracks are not lighted, special attention must be paid to the visibility of leading marks. Unlighted leading marks easily can go unobserved or do not stand out sufficiently from their background. When leading marks are used, observation distances exceeding 5 km should be avoided since at long distances lines the visibility of the boards is impaired even if the size of the board is increased. Moreover, the required mast height for the boards becomes unreasonably high for lower sites due to the low observation height.

### Use of safe water marks

A safe water mark indicates that there is navigable water all around and also under its position. Safe water marks also can be used to indicate the fairway line of a small craft track. The use of safe water marks requires the full-depth water area to be sufficiently large for boating, and any dangerous shallows even further away from the fairway are marked with, for example, a cardinal mark or isolated danger mark. Safe water marks usually are applicable for replacing edge marking for crossing a wide open area or otherwise broad water areas. In planning the location of the mark, it should be checked that the intended location is not too deep.

Figure 6A and B. Two line marking examples

Figure 6A shows an example of a situation in which leading marks are used. In addition, the point of an island at the south end of the fairway is marked with a cairn. Leading marks can be used to avoid the use of floating aids to navigation, which would otherwise be required for the open water area and which are difficult to construct, especially in deep water.

Figure 6B shows a crossing of a wide open area. In this case, safe water marks are used. The solution in Figure 6B assumes that in travel northward the turning point after crossing of the open area can be indicated with sufficient accuracy via a cairn.

In the case shown in Figure 6B, the diameter of the safe water marks must be sufficient for the marks to be visible in the wave conditions occurring on the site.

### Buoy jams

In some cases, usually in very narrow straits and similar locations, a problem is generated by the high number of buoys and especially in detection of the first buoy when the area is approached. Figure 7 shows two different solutions to this ‘buoy jam’ problem. In the upper figure, the first mark (when approached from the east) is a cardinal mark, while all other marks are lateral marks.

Figure 7. Buoy jam, two examples

The lower figure shows another way to solve the problem. Since a safe water mark is on the fairway when approached from the east, the buoy jam is automatically approached from the right direction. Hence, detecting the first mark immediately from the buoy jam is not essential but can well happen later. However, this is not an appropriate solution if there is a lot of boat traffic approaching the buoy jam from other directions from the fairway. The same result can be achieved by using leading marks.

A third option would be to use a larger mark at the position of the first green starboard side mark when approached from the east. This would in practice be possible if there is a relatively wide open area on the east side so that the first (larger) mark detected further away leads the boater to the right direction and given that the depth allows mooring of a large mark.

### Locations outside the fairway area

In some cases, it can be justified to use aids to navigation to indicate dangerous locations that are far from the fairway area. This is necessary especially where there is considerable boat traffic that uses other than the fairway area (traffic to summer cottages or fishing) or, for example, crossing a wide open area (beating possibility for sailboats). In these kinds of situations, it can be justified to mark a dangerous shallows even far from the fairway.

Figure 8. Shallows outside the fairway area

Similarly, at the crossing area of two fairways it can be justified to indicate the crossing using a mark that is positioned such that it protects the most dangerous shallows in the area and is not close to either of the fairways. This situation is illustrated in Figure 8.

If the mark were positioned close to the crossing, there would be a possibility of a boater coming from the south and turning to the eastbound fairway, cutting the crossing, and hitting the shallows, now covered by the north cardinal in Figure 8. When a decision is made to position a mark far from the fairway area, it must be ensured that there is no area shallower than the confirmed depth between the fairway and the mark.

In planning of positioning similar to the situation shown in Figure 8, it should be noted that, even if the fairway area does not reach the north cardinal, the whole area between the fairway and the north cardinal must be confirmed. Hence, in this case the marks and the cairn indicate the confirmed area, which is considerably larger than the fairway area.

Locations outside the fairway area should be marked sparingly since users easily interpret all area near the marks as being fairway area. Hence, careful consideration is required for marks positioned outside the fairway area.