IALA S-201

Product Specification

Draft  0.0.7 – July 2017

IALA AtoN Product Specification

ANNEX B2

Schema S201.xsd

| Elements | Complex types | Simple types |
| --- | --- | --- |
| Aggregation | Aggregation | beaconShape |
| AidsToNavigation | AidsToNavigation | buildingShape |
| Association | Association | buoyShape |
| BeaconCardinal | BeaconCardinal | categoryOfCardinalMark |
| BeaconIsolatedDanger | BeaconIsolatedDanger | categoryOfFogSignal |
| BeaconLateral | BeaconLateral | categoryOfInstallationBuoy |
| BeaconSafeWater | BeaconSafeWater | categoryOfLandmark |
| BeaconSpecialPurposeGeneral | BeaconSpecialPurposeGeneral | categoryOfLateralMark |
| BuoyCardinal | BuoyCardinal | categoryOfLight |
| BuoyInstallation | BuoyInstallation | categoryOfNavigationLine |
| BuoyIsolatedDanger | BuoyIsolatedDanger | categoryOfOffshorePlatform |
| BuoyLateral | BuoyLateral | categoryOfPile |
| BuoySafeWater | BuoySafeWater | categoryOfRadarTransponderBeacon |
| BuoySpecialPurposeGeneral | BuoySpecialPurposeGeneral | categoryOfRecommendedTrack |
| DataCoverage | contactAddress | categoryOfSiloTank |
| DataSet | DataCoverage | categoryOfSpecialPurposeMark |
| Daymark | DatasetType | categoryOfTemporalVariation |
| Equipment | Daymark | colour |
| FeatureType | Equipment | colourPattern |
| FogSignal | FeatureType | condition |
| GenericBeacon | FogSignal | Decimal0.0To360.0 |
| GenericBuoy | GenericBeacon | exhibitionConditionOfLight |
| IALA_EnvironmentObservationEquipment | GenericBuoy | function |
| IALA_Lighthouse | IALA_EnvironmentObservationEquipment | IALA_aidAvailabilityCategory |
| InformationType | IALA_Lighthouse | IALA_categoryOfAggregation |
| Landmark | IMemberType | IALA_categoryOfAssociation |
| Light | InformationTypeType | ISO3166-alpha2 |
| LightFloat | Landmark | ISO639-2 |
| LightVessel | Light | ISO639-3 |
| LocalDirectionOfBuoyage | LightFloat | lightCharacteristic |
| MetaFeatureType | LightVessel | lightVisibility |
| NavigationalSystemOfMarks | LocalDirectionOfBuoyage | marksNavigationalSystemOf |
| NavigationLine | MemberType | multiplicityOfLightsType |
| OffshorePlatform | MetaFeatureType | natureOfConstruction |
| Pile | NavigationalSystemOfMarks | NonNegativeDecimal |
| QualityOfNonBathymetricData | NavigationLine | NonNegativeDecimal.1 |
| RadarReflector | OffshorePlatform | product |
| RadarTransponderBeacon | Pile | qualityOfPosition |
| RecommendedTrack | QualityOfNonBathymetricData | qualityOfSoundingMeasurement |
| RetroReflector | RadarReflector | radarConspicuous |
| SiloTank | RadarTransponderBeacon | signalGeneration |
| SoundingDatum | RecommendedTrack | status |
| SpatialUncertainty | RetroReflector | techniqueOfSoundingMeasurement |
| StructureObject | S100_TruncatedDate | topmarkDaymarkShape |
| Topmark | SiloTank | trafficFlow |
| VerticalDatum | SoundingDatum | verticalDatum |
|  | SpatialUncertainty | visuallyConspicuous |
|  | StructureObject |  |
|  | surveyDateRange |  |
|  | Topmark |  |
|  | VerticalDatum |  |

element Aggregation

| diagram |  |
| --- | --- |
| type | Aggregation |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation IALA_categoryOfAggregation peer |
| annotation | documentation. An implementation that uses the named role method for associations might not use the featureAssociation and informationAssociation content tags because they duplicate the link.. |

element AidsToNavigation

| diagram |  |
| --- | --- |
| type | AidsToNavigation |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer |
| annotation | documentation. A visual, acoustical, or radio device, external to a ship, designed to assist in determining a safe course or a vessel's position, or to warn of dangers and/or obstructions. Aids to navigation usually include buoys, beacons, fog signals, lights, radio beacons, leading marks, radio position fixing systems and GNSS which are chart-related and assist safe navigation.. |

element Association

| diagram |  |
| --- | --- |
| type | Association |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation IALA_categoryOfAssociation peer |
| annotation | documentation. An implementation that uses the named role method for associations might not use the featureAssociation and informationAssociation content tags because they duplicate the link.. |

element BeaconCardinal

| diagram |  |
| --- | --- |
| type | BeaconCardinal |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child beaconShape colour colourPattern condition radarConspicuous visuallyConspicuous elevation height marksNavigationalSystemOf natureOfConstruction verticalLength verticalAccuracy objectNameInNationalLanguage objectName status geometry categoryOfCardinalMark |

element BeaconIsolatedDanger

| diagram |  |
| --- | --- |
| type | BeaconIsolatedDanger |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child beaconShape colour colourPattern condition radarConspicuous visuallyConspicuous elevation height marksNavigationalSystemOf natureOfConstruction verticalLength verticalAccuracy objectNameInNationalLanguage objectName status geometry |

element BeaconLateral

| diagram |  |
| --- | --- |
| type | BeaconLateral |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child beaconShape colour colourPattern condition radarConspicuous visuallyConspicuous elevation height marksNavigationalSystemOf natureOfConstruction verticalLength verticalAccuracy objectNameInNationalLanguage objectName status geometry categoryOfLateralMark |

element BeaconSafeWater

| diagram |  |
| --- | --- |
| type | BeaconSafeWater |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child beaconShape colour colourPattern condition radarConspicuous visuallyConspicuous elevation height marksNavigationalSystemOf natureOfConstruction verticalLength verticalAccuracy objectNameInNationalLanguage objectName status geometry |

element BeaconSpecialPurposeGeneral

| diagram |  |
| --- | --- |
| type | BeaconSpecialPurposeGeneral |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child beaconShape colour colourPattern condition radarConspicuous visuallyConspicuous elevation height marksNavigationalSystemOf natureOfConstruction verticalLength verticalAccuracy objectNameInNationalLanguage objectName status geometry categoryOfSpecialPurposeMark |

element BuoyCardinal

| diagram |  |
| --- | --- |
| type | BuoyCardinal |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child buoyShape colour colourPattern radarConspicious marksNavigationalSystemOf natureOfconstuction objectNameInNationalLanguage objectName status verticalAccuracy verticalLength IALA_typeOfBuoy geometry categoryOfCardinalMark |

element BuoyInstallation

| diagram |  |
| --- | --- |
| type | BuoyInstallation |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child buoyShape colour colourPattern radarConspicious marksNavigationalSystemOf natureOfconstuction objectNameInNationalLanguage objectName status verticalAccuracy verticalLength IALA_typeOfBuoy geometry categoryOfInstallationBuoy |

element BuoyIsolatedDanger

| diagram |  |
| --- | --- |
| type | BuoyIsolatedDanger |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child buoyShape colour colourPattern radarConspicious marksNavigationalSystemOf natureOfconstuction objectNameInNationalLanguage objectName status verticalAccuracy verticalLength IALA_typeOfBuoy geometry |

element BuoyLateral

| diagram |  |
| --- | --- |
| type | BuoyLateral |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child buoyShape colour colourPattern radarConspicious marksNavigationalSystemOf natureOfconstuction objectNameInNationalLanguage objectName status verticalAccuracy verticalLength IALA_typeOfBuoy geometry categoryOfLateralMark |

element BuoySafeWater

| diagram |  |
| --- | --- |
| type | BuoySafeWater |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child buoyShape colour colourPattern radarConspicious marksNavigationalSystemOf natureOfconstuction objectNameInNationalLanguage objectName status verticalAccuracy verticalLength IALA_typeOfBuoy geometry |

element BuoySpecialPurposeGeneral

| diagram |  |
| --- | --- |
| type | BuoySpecialPurposeGeneral |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child buoyShape colour colourPattern radarConspicious marksNavigationalSystemOf natureOfconstuction objectNameInNationalLanguage objectName status verticalAccuracy verticalLength IALA_typeOfBuoy geometry categoryOfSpecialPurposeMark |

element DataCoverage

| diagram |  |
| --- | --- |
| type | DataCoverage |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation maximumDisplayScale minimumDisplayScale geometry |

element DataSet

| diagram |  |
| --- | --- |
| type | DatasetType |
| children | gml:boundedBy DatasetIdentificationInformation DatasetStructureInformation Point MultiPoint Curve CompositeCurve OrientableCurve Surface Polygon imember member |
| identity constraints | Name; Refer; Selector; Field(s); Annotation. unique; UNIQUE_FOID; ./member/*/S100:featureObjectIdentifier; S100:agency S100:featureIdentificationNumber S100:featureIdentificationSubdivision; documentation. each FOID must be unique.. |

element Daymark

| diagram |  |
| --- | --- |
| type | Daymark |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_remoteMonitoringSystem IALA_remotelyMonitored parent geometry categoryOfSpecialPurposeMark colour colourPattern elevation height natureOfConstruction objectNameInNationalLanguage objectName status topmarkDaymarkShape verticalAccuracy verticalDatum verticalLength |

element Equipment

| diagram |  |
| --- | --- |
| type | Equipment |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_remoteMonitoringSystem IALA_remotelyMonitored parent geometry |
| annotation | documentation. An implementation that uses the named role method for associations might not use the featureAssociation and informationAssociation content tags because they duplicate the link.. |

element FeatureType

| diagram |  |
| --- | --- |
| type | FeatureType |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation |

element FogSignal

| diagram |  |
| --- | --- |
| type | FogSignal |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_remoteMonitoringSystem IALA_remotelyMonitored parent geometry categoryOfFogSignal signalFrequency signalGeneration signalGroup signalPeriod signalSequence status valueOfMaximumRange IALA_signalOutput IALA_typeOfBattery |

element GenericBeacon

| diagram |  |
| --- | --- |
| type | GenericBeacon |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child beaconShape colour colourPattern condition radarConspicuous visuallyConspicuous elevation height marksNavigationalSystemOf natureOfConstruction verticalLength verticalAccuracy objectNameInNationalLanguage objectName status geometry |

element GenericBuoy

| diagram |  |
| --- | --- |
| type | GenericBuoy |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child buoyShape colour colourPattern radarConspicious marksNavigationalSystemOf natureOfconstuction objectNameInNationalLanguage objectName status verticalAccuracy verticalLength IALA_typeOfBuoy geometry |

element IALA_EnvironmentObservationEquipment

| diagram |  |
| --- | --- |
| type | IALA_EnvironmentObservationEquipment |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_remoteMonitoringSystem IALA_remotelyMonitored parent geometry status height IALA_typeOfEnvironmentObservationEquipment IALA_typeOfBattery |

element IALA_Lighthouse

| diagram |  |
| --- | --- |
| type | IALA_Lighthouse |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child categoryOfLandmark colour colourPattern condition radarConspicuous visuallyConspicuous elevation function height natureOfConstruction objectNameInNationalLanguage objectName status verticalAccuracy verticalDatum verticalLength IALA_mannedStructure geometry |
| annotation | documentation. Restriction of geometry compared to super-type (Landmark) is enforced by a Schematron rule. |

element InformationType

| diagram |  |
| --- | --- |
| type | InformationTypeType |
| children | informationAssociation invInformationAssociation |
| used by | complexType; IMemberType. |

element Landmark

| diagram |  |
| --- | --- |
| type | Landmark |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child categoryOfLandmark colour colourPattern condition radarConspicuous visuallyConspicuous elevation function height natureOfConstruction objectNameInNationalLanguage objectName status verticalAccuracy verticalDatum verticalLength IALA_mannedStructure geometry |

element Light

| diagram |  |
| --- | --- |
| type | Light |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_remoteMonitoringSystem IALA_remotelyMonitored parent geometry categoryOfLight colour exhibitionConditionOfLight height lightCharacteristic lightVisibility marksNavigationalSystemOf multiplicityOfLights objectNameInNationalLanguage objectName orientation sectorLimitOne sectorLimitTwo signalGroup signalPeriod signalSequence status verticalAccuracy valueOfNominalRange verticalDatum IALA_typeOfLight IALA_typeOfBattery IALA_valueOfGeographicalRange IALA_valueOfLuminousRange |

element LightFloat

| diagram |  |
| --- | --- |
| type | LightFloat |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child colour colourPattern radarConspicuous visuallyConspicuous horizontalAccuracy horizontalLength horizontalWidth natureOfConstruction objectNameInNationalLanguage objectName status verticalAccuracy verticalLength IALA_mannedStructure geometry |

element LightVessel

| diagram |  |
| --- | --- |
| type | LightVessel |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child colour colourPattern radarConspicuous visuallyConspicuous horizontalAccuracy horizontalLength horizontalWidth natureOfConstruction objectNameInNationalLanguage objectName status verticalAccuracy verticalLength IALA_mannedStructure geometry |

element LocalDirectionOfBuoyage

| diagram |  |
| --- | --- |
| type | LocalDirectionOfBuoyage |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation orientation geometry |

element MetaFeatureType

| diagram |  |
| --- | --- |
| type | MetaFeatureType |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation |

element NavigationalSystemOfMarks

| diagram |  |
| --- | --- |
| type | NavigationalSystemOfMarks |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation marksNavigationalSystemOf geometry |

element NavigationLine

| diagram |  |
| --- | --- |
| type | NavigationLine |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer categoryOfNavigationLine orientation status navigableTrack geometry |
| annotation | documentation. An implementation that uses the named role method for associations might not use the featureAssociation and informationAssociation content tags because they duplicate the link.. |

element OffshorePlatform

| diagram |  |
| --- | --- |
| type | OffshorePlatform |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child categoryOfOffshorePlatform colour colourPattern condition radarConspicuous visuallyConspicuous height natureOfConstruction objectNameInNationalLanguage product objectName status verticalAccuracy verticalDatum verticalLength IALA_mannedStructure geometry |

element Pile

| diagram |  |
| --- | --- |
| type | Pile |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child categoryOfPile colour colourPattern condition visuallyConspicuous height verticalAccuracy verticalDatum verticalLength geometry |

element QualityOfNonBathymetricData

| diagram |  |
| --- | --- |
| type | QualityOfNonBathymetricData |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation categoryOfTemporalVariation directionUncertainty horizontalDistanceUncertainty horizontalPositionUncertainty information informationInNationalLanguage textualDescription textualDescriptionInNationalLanguage verticalUncertainty surveyDateRange geometry |

element RadarReflector

| diagram |  |
| --- | --- |
| type | RadarReflector |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_remoteMonitoringSystem IALA_remotelyMonitored parent geometry height status verticalAccuracy verticalDatum |

element RadarTransponderBeacon

| diagram |  |
| --- | --- |
| type | RadarTransponderBeacon |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_remoteMonitoringSystem IALA_remotelyMonitored parent geometry categoryOfRadarTransponderBeacon objectNameInNationalLanguage objectName radarWaveLength sectorLimitOne sectorLimitTwo signalGroup signalSequence status valueOfNominalRange IALA_typeOfBattery |

element RecommendedTrack

| diagram |  |
| --- | --- |
| type | RecommendedTrack |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer categoryOfRecommendedTrack depthRangeMinimumValue depthRangeMaximumValue objectNameInNationalLanguage objectName orientation qualityOfSoundingMeasurement soundingAccuracy status techniqueOfSoundingMeasurement trafficFlow verticalDatum navigationLine geometry |
| annotation | documentation. An implementation that uses the named role method for associations might not use the featureAssociation and informationAssociation content tags because they duplicate the link.. |

element RetroReflector

| diagram |  |
| --- | --- |
| type | RetroReflector |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_remoteMonitoringSystem IALA_remotelyMonitored parent geometry colour colourPattern height marksNavigationalSystemOf status verticalAccuracy verticalDatum |

element SiloTank

| diagram |  |
| --- | --- |
| type | SiloTank |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child buildingShape categoryOfSiloTank colour colourPattern condition radarConspicuous visuallyConspicuous elevation height natureOfConstruction objectNameInNationalLanguage objectName product status verticalAccuracy verticalDatum verticalLength geometry |

element SoundingDatum

| diagram |  |
| --- | --- |
| type | SoundingDatum |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation verticalDatum geometry |

element SpatialUncertainty

| diagram |  |
| --- | --- |
| type | SpatialUncertainty |
| children | informationAssociation invInformationAssociation qualityOfPosition positionalAccuracy |

element StructureObject

| diagram |  |
| --- | --- |
| type | StructureObject |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child |
| annotation | documentation. An implementation that uses the named role method for associations might not use the featureAssociation and informationAssociation content tags because they duplicate the link.. |

element Topmark

| diagram |  |
| --- | --- |
| type | Topmark |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_remoteMonitoringSystem IALA_remotelyMonitored parent geometry colour colourPattern height marksNavigationalSystemOf status topmarkDaymarkShape verticalAccuracy verticalDatum verticalLength |

element VerticalDatum

| diagram |  |
| --- | --- |
| type | VerticalDatum |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation verticalDatum geometry |

complexType Aggregation

| diagram |  |
| --- | --- |
| type | extension of FeatureType |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation IALA_categoryOfAggregation peer |
| used by | element; Aggregation. |
| annotation | documentation. Used to identify an aggregation of two or more objects. This aggregation may be named.. documentation. content of categoryOfAggregation should be put in information attribute when converting to S-57.. |

element Aggregation/IALA_categoryOfAggregation

| diagram |  |
| --- | --- |
| type | IALA_categoryOfAggregation |

element Aggregation/peer

| diagram |  |
| --- | --- |
| type | gml:ReferenceType |
| annotation | documentation. The role given to an object of equal status in the relationship.. |

complexType AidsToNavigation

| diagram |  |
| --- | --- |
| type | extension of FeatureType |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer |
| used by | element; AidsToNavigation. complexTypes; Equipment NavigationLine RecommendedTrack StructureObject. |
| annotation | documentation. A visual, acoustical, or radio device, external to a ship, designed to assist in determining a safe course or a vessel's position, or to warn of dangers and/or obstructions. Aids to navigation usually include buoys, beacons, fog signals, lights, radio beacons, leading marks, radio position fixing systems and GNSS which are chart-related and assist safe navigation.. |

element AidsToNavigation/idCode

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. Definition required. |

element AidsToNavigation/dateEnd

| diagram |  |
| --- | --- |
| type | S100_TruncatedDate |
| children | gDay gMonth gYear gMonthDay gYearMonth date |
| annotation | documentation. The latest date on which an object (e.g., a buoy) will be present.. |

element AidsToNavigation/dateStart

| diagram |  |
| --- | --- |
| type | S100_TruncatedDate |
| children | gDay gMonth gYear gMonthDay gYearMonth date |
| annotation | documentation. The earliest date on which an object (e.g., a buoy) will be present.. |

element AidsToNavigation/periodEnd

| diagram |  |
| --- | --- |
| type | S100_TruncatedDate |
| children | gDay gMonth gYear gMonthDay gYearMonth date |
| annotation | documentation. The end of the active period for a seasonal object (e.g., a buoy).. |

element AidsToNavigation/periodStart

| diagram |  |
| --- | --- |
| type | S100_TruncatedDate |
| children | gDay gMonth gYear gMonthDay gYearMonth date |
| annotation | documentation. The start of the active period for a seasonal object (e.g., a buoy).. |

element AidsToNavigation/information

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. Textual information about the object. (max 300 characters). |

element AidsToNavigation/informationInNationalLanguage

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. Textual information in national language characters. (max 300 characters). |

element AidsToNavigation/textualDescriptionInNationalLanguage

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. The file name of an external text file that contains the text in a national language.. |

element AidsToNavigation/textualDescription

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. file reference to text file. |

element AidsToNavigation/scaleMinimum

| diagram |  |
| --- | --- |
| type | xs:positiveInteger |
| annotation | documentation. The minimum scale at which the object may be used, e.g., for ECDIS presentation.. |

element AidsToNavigation/sourceDate

| diagram |  |
| --- | --- |
| type | xs:dateTime |
| annotation | documentation. The production date of the source, e.g., the date of measurement.. |

element AidsToNavigation/sourceIndication

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. Information about the source of the object.. documentation. Indication: country (c2): Two letter code according to ISO 3166 (refer to Annex A to S-57 Appendix A) authority (c2): A string of two alphanumeric characters (refer to Annex A to S-57 Appendix A), e.g. German Bundesamt für Seeschiffahrt und Hydrographie = DE; US National Imagery and Mapping Agency = U1. procedure (c4): digitized = digi scanned = scan alpha/numeric input = alph Format: c2,c2,c4 (mandatory) Example: DK,D1,digi. |

element AidsToNavigation/pictorialRepresentation

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. file reference to text file. |

element AidsToNavigation/IALA_inspectionFrequency

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. Definition required. |

element AidsToNavigation/IALA_inspectionRequirements

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. Definition required. |

element AidsToNavigation/IALA_AtoNMaintenanceRecord

| diagram |  |
| --- | --- |
| type | xs:anyURI |
| annotation | documentation. Definition required. |

element AidsToNavigation/IALA_installationDate

| diagram |  |
| --- | --- |
| type | S100_TruncatedDate |
| children | gDay gMonth gYear gMonthDay gYearMonth date |
| annotation | documentation. Definition required. |

element AidsToNavigation/peer

| diagram |  |
| --- | --- |
| type | gml:ReferenceType |
| annotation | documentation. Encodes the association to Aggregation AND Association. In principle the role names for the two target classes should be different as should the names at the two ends of each, but they are all currently (September 2015) "peer" in the application schema.. |

complexType Association

| diagram |  |
| --- | --- |
| type | extension of FeatureType |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation IALA_categoryOfAssociation peer |
| used by | element; Association. |
| annotation | documentation. Used to identify an association between two or more objects. The association may be named. documentation. content of categoryOfAssociation should be put in information attribute when converting to S-57.. |

element Association/IALA_categoryOfAssociation

| diagram |  |
| --- | --- |
| type | IALA_categoryOfAssociation |

element Association/peer

| diagram |  |
| --- | --- |
| type | gml:ReferenceType |
| annotation | documentation. The role given to an object of equal status in the relationship.. |

complexType BeaconCardinal

| diagram |  |
| --- | --- |
| type | extension of GenericBeacon |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child beaconShape colour colourPattern condition radarConspicuous visuallyConspicuous elevation height marksNavigationalSystemOf natureOfConstruction verticalLength verticalAccuracy objectNameInNationalLanguage objectName status geometry categoryOfCardinalMark |
| used by | element; BeaconCardinal. |
| annotation | documentation. A beacon is a prominent, specially constructed object forming a conspicuous mark as a fixed aid to navigation or for use in hydrographic survey (IHO Dictionary, S-32, 5th Edition, 420).. documentation. A cardinal beacon is used in conjunction with the compass to indicate where the mariner may find the best navigable water. It is placed in one of the four quadrants (North, East, South and West), bounded by inter-cardinal bearings from the point marked. (UKHO NP 735, 5th Edition). |

element BeaconCardinal/categoryOfCardinalMark

| diagram |  |
| --- | --- |
| type | categoryOfCardinalMark |
| facets | Kind; Value; Annotation. enumeration; north cardinal mark; documentation. Quadrant bounded by the true bearing NW-NE taken from the point of interest it should be passed to the north side of the mark... enumeration; east cardinal mark; documentation. Quadrant bounded by the true bearing NE-SE taken from the point of interest it should be passed to the east side of the mark... enumeration; south cardinal mark; documentation. Quadrant bounded by the true bearing SE-SW taken from the point of interest it should be passed to the south side of the mark... enumeration; west cardinal mark; documentation. Quadrant bounded by the true bearing SW-NW taken from the point of interest it should be passed to the west side of the mark... |

complexType BeaconIsolatedDanger

| diagram |  |
| --- | --- |
| type | extension of GenericBeacon |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child beaconShape colour colourPattern condition radarConspicuous visuallyConspicuous elevation height marksNavigationalSystemOf natureOfConstruction verticalLength verticalAccuracy objectNameInNationalLanguage objectName status geometry |
| used by | element; BeaconIsolatedDanger. |
| annotation | documentation. A beacon is a prominent specially constructed object forming a conspicuous mark as a fixed aid to navigation or for use in hydrographic survey (IHO Dictionary, S-32, 5th Edition, 420).. documentation. An isolated danger beacon is a beacon erected on an isolated danger of limited extent, which has navigable water all around it. (UKHO NP735, 5th Edition). |

complexType BeaconLateral

| diagram |  |
| --- | --- |
| type | extension of GenericBeacon |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child beaconShape colour colourPattern condition radarConspicuous visuallyConspicuous elevation height marksNavigationalSystemOf natureOfConstruction verticalLength verticalAccuracy objectNameInNationalLanguage objectName status geometry categoryOfLateralMark |
| used by | element; BeaconLateral. |
| annotation | documentation. A beacon is a prominent specially constructed object forming a conspicuous mark as a fixed aid to navigation or for use in hydrographic survey (IHO Dictionary, S-32, 5th Edition, 420).. documentation. A lateral beacon is used to indicate the port or starboard hand side of the route to be followed. They are generally used for well defined channels and are used in conjunction with a conventional direction of buoyage. (UKHO NP 735, 5th Edition). |

element BeaconLateral/categoryOfLateralMark

| diagram |  |
| --- | --- |
| type | categoryOfLateralMark |
| facets | Kind; Value; Annotation. enumeration; port-hand lateral mark; documentation. indicates the port boundary of a navigational channel or suggested route when proceeding in the 'conventional direction of buoyage'... enumeration; starboard-hand lateral mark; documentation. indicates the starboard boundary of a navigational channel or suggested route when proceeding in the 'conventional direction of buoyage'... enumeration; preferred channel to starboard lateral mark; documentation. at a point where a channel divides, when proceeding in the 'conventional direction of buoyage', the preferred channel (or primary route) is indicated by a modified port-hand lateral mark... enumeration; preferred channel to port lateral mark; documentation. at a point where a channel divides, when proceeding in the 'conventional direction of buoyage', the preferred channel (or primary route) is indicated by a modified starboard-hand lateral mark... |

complexType BeaconSafeWater

| diagram |  |
| --- | --- |
| type | extension of GenericBeacon |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child beaconShape colour colourPattern condition radarConspicuous visuallyConspicuous elevation height marksNavigationalSystemOf natureOfConstruction verticalLength verticalAccuracy objectNameInNationalLanguage objectName status geometry |
| used by | element; BeaconSafeWater. |
| annotation | documentation. A safe water beacon is a prominent specially constructed object forming a conspicuous mark as a fixed aid to navigation or for use in hydrographic survey (IHO Dictionary, S-32, 5th Edition, 420). A safe water beacon may be used to indicate that there is navigable water around the mark. (UKHO NP735, 5th Edition). |

complexType BeaconSpecialPurposeGeneral

| diagram |  |
| --- | --- |
| type | extension of GenericBeacon |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child beaconShape colour colourPattern condition radarConspicuous visuallyConspicuous elevation height marksNavigationalSystemOf natureOfConstruction verticalLength verticalAccuracy objectNameInNationalLanguage objectName status geometry categoryOfSpecialPurposeMark |
| used by | element; BeaconSpecialPurposeGeneral. |
| annotation | documentation. A beacon is a prominent specially constructed object forming a conspicuous mark as a fixed aid to navigation or for use in hydrographic survey (IHO Dictionary, S-32, 5th Edition, 420).. documentation. A special purpose beacon is primarily used to indicate an area or feature, the nature of which is apparent from reference to a chart, Sailing Directions or Notices to Mariners. (UKHO NP 735, 5th Edition) Beacon in general: A beacon whose appearance or purpose is not adequately known.. |

element BeaconSpecialPurposeGeneral/categoryOfSpecialPurposeMark

| diagram |  |
| --- | --- |
| type | categoryOfSpecialPurposeMark |
| facets | Kind; Value; Annotation. enumeration; firing danger area mark; documentation. a mark used to indicate a firing danger area, usually at sea... enumeration; target mark; documentation. any object toward which something is directed. The distinctive marking or instrumentation of a ground point to aid its identification on a photograph... enumeration; marker ship mark; documentation. a mark marking the position of a ship which is used as a target during some military exercise... enumeration; degaussing range mark; documentation. a mark used to indicate a degaussing range... enumeration; barge mark; documentation. a mark of relevance to barges... enumeration; cable mark; documentation. a mark used to indicate the position of submarine cables or the point at which they run on to the land... enumeration; spoil ground mark; documentation. a mark used to indicate the limit of a spoil ground.. enumeration; outfall mark; documentation. a mark used to indicate the position of an outfall or the point at which it leaves the land... enumeration; ODAS (Ocean-Data-Acquisition-System); documentation. Ocean Data Acquisition System.. enumeration; recording mark; documentation. a mark used to record data for scientific purposes... enumeration; seaplane anchorage mark; documentation. a mark used to indicate a seaplane anchorage... enumeration; recreation zone mark; documentation. a mark used to indicate a recreation zone... enumeration; private mark; documentation. a privately maintained mark... enumeration; mooring mark; documentation. a mark indicating a mooring or moorings... enumeration; LANBY (Large Automatic Navigational Buoy); documentation. a large buoy designed to take the place of a lightship where construction of an offshore light station is not feasible... enumeration; leading mark; documentation. aids to navigation or other indicators so located as to indicate the path to be followed. Leading marks identify a leading line when they are in transit... enumeration; measured distance mark; documentation. a mark forming part of a transit indicating one end of a measured distance... enumeration; notice mark; documentation. a notice board or sign indicating information to the mariner... enumeration; TSS mark (Traffic Separation Scheme); documentation. a mark indicating a traffic separation scheme... enumeration; anchoring prohibited mark; documentation. a mark indicating an anchoring prohibited area... enumeration; berthing prohibited mark; documentation. a mark indicating that berthing is prohibited... enumeration; overtaking prohibited mark; documentation. a mark indicating that overtaking is prohibited... enumeration; two-way traffic prohibited mark; documentation. a mark indicating a one-way route... enumeration; reduced wake mark; documentation. a mark indicating that vessels must not generate excessive wake... enumeration; speed limit mark; documentation. a mark indicating that a speed limit applies... enumeration; stop mark; documentation. a mark indicating the place where the bow of a ship must stop when traffic lights show red... enumeration; general warning mark; documentation. a mark indicating that special caution must be exercised in the vicinity of the mark... enumeration; sound ship's siren mark; documentation. a mark indicating that a ship should sound its siren or horn... enumeration; restricted vertical clearance mark; documentation. a mark indicating the minimum vertical space available for passage... enumeration; maximum vessel's draught mark; documentation. a mark indicating the maximum draught of vessel permitted... enumeration; restricted horizontal clearance mark; documentation. a mark indicating the minimum horizontal space available for passage... enumeration; strong current warning mark; documentation. a mark warning of strong currents... enumeration; berthing permitted mark; documentation. a mark indicating that berthing is allowed... enumeration; overhead power cable mark; documentation. a mark indicating an overhead power cable... enumeration; channel edge gradient mark; documentation. a mark indicating the gradient of the slope of a dredge channel edge... enumeration; telephone mark; documentation. a mark indicating the presence of a telephone... enumeration; ferry crossing mark; documentation. a mark indicating that a ferry route crosses the ship route often used with a 'sound ship's siren' mark... enumeration; pipeline mark; documentation. a mark used to indicate the position of submarine pipelines or the point at which they run on to the land... enumeration; anchorage mark; documentation. a mark indicating an anchorage area... enumeration; clearing mark; documentation. a mark used to indicate a clearing line... enumeration; control mark; documentation. a mark indicating the location at which a restriction or requirement exists... enumeration; diving mark; documentation. a mark indicating that diving may take place in the vicinity... enumeration; refuge beacon; documentation. a mark providing or indicating a place of safety... enumeration; foul ground mark; documentation. a mark indicating a foul ground... enumeration; yachting mark; documentation. a mark installed for use by yachtsmen... enumeration; heliport mark; documentation. a mark indicating an area where helicopters may land... enumeration; GPS mark; documentation. a mark indicating a location at which a GNSS position has been accurately determined... enumeration; seaplane landing mark; documentation. a mark indicating an area where sea-planes land... enumeration; entry prohibited mark; documentation. a mark indicating that entry is prohibited... enumeration; work in progress mark; documentation. a mark indicating that work (generally construction) is in progress... enumeration; mark with unknown purpose; documentation. a mark whose detailed characteristics are unknown... enumeration; wellhead mark; documentation. a mark indicating a borehole that produces or is capable of producing oil or natural gas... enumeration; channel separation mark; documentation. a mark indicating the point at which a channel divides separately into two channels... enumeration; marine farm mark; documentation. a mark indicating the existence of a fish, mussel, oyster or pearl farm/ culture... enumeration; artificial reef mark; documentation. a mark indicating the existence or the extent of an artificial reef... enumeration; IALA_jetski prohibited; documentation. A mark indicating a jetski prohibited area.. |

complexType BuoyCardinal

| diagram |  |
| --- | --- |
| type | extension of GenericBuoy |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child buoyShape colour colourPattern radarConspicious marksNavigationalSystemOf natureOfconstuction objectNameInNationalLanguage objectName status verticalAccuracy verticalLength IALA_typeOfBuoy geometry categoryOfCardinalMark |
| used by | element; BuoyCardinal. |
| annotation | documentation. A cardinal buoy is used in conjunction with the compass to indicate where the mariner may find the best navigable water. It is placed in one of the four quadrants (North, East, South and West), bounded by inter-cardinal bearings from the point marked.. |

element BuoyCardinal/categoryOfCardinalMark

| diagram |  |
| --- | --- |
| type | categoryOfCardinalMark |
| facets | Kind; Value; Annotation. enumeration; north cardinal mark; documentation. Quadrant bounded by the true bearing NW-NE taken from the point of interest it should be passed to the north side of the mark... enumeration; east cardinal mark; documentation. Quadrant bounded by the true bearing NE-SE taken from the point of interest it should be passed to the east side of the mark... enumeration; south cardinal mark; documentation. Quadrant bounded by the true bearing SE-SW taken from the point of interest it should be passed to the south side of the mark... enumeration; west cardinal mark; documentation. Quadrant bounded by the true bearing SW-NW taken from the point of interest it should be passed to the west side of the mark... |

complexType BuoyInstallation

| diagram |  |
| --- | --- |
| type | extension of GenericBuoy |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child buoyShape colour colourPattern radarConspicious marksNavigationalSystemOf natureOfconstuction objectNameInNationalLanguage objectName status verticalAccuracy verticalLength IALA_typeOfBuoy geometry categoryOfInstallationBuoy |
| used by | element; BuoyInstallation. |
| annotation | documentation. A buoy is a floating object moored to the bottom in a particular place, as an aid to navigation or for other specific purposes. (IHO Dictionary, S-32, 5th Edition, 565).. documentation. An installation buoy is a buoy used for loading tankers with gas or oil. (IHO Chart Specifications, M-4). |

element BuoyInstallation/categoryOfInstallationBuoy

| diagram |  |
| --- | --- |
| type | categoryOfInstallationBuoy |
| facets | Kind; Value; Annotation. enumeration; catenary anchor leg mooring (CALM); documentation. incorporates a large buoy which remains on the surface at all times and is moored by 4 or more anchors. Mooring hawsers and cargo hoses lead from a turntable on top of the buoy, so that the buoy does not turn as the ship swings to wind and stream... enumeration; single buoy mooring (SBM or SPM); documentation. a mooring structure used by tankers to load and unload in port approaches or in offshore oil and gas fields. The size of the structure can vary between a large mooring buoy and a manned floating structure. Also known as single point mooring (SPM).. |

complexType BuoyIsolatedDanger

| diagram |  |
| --- | --- |
| type | extension of GenericBuoy |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child buoyShape colour colourPattern radarConspicious marksNavigationalSystemOf natureOfconstuction objectNameInNationalLanguage objectName status verticalAccuracy verticalLength IALA_typeOfBuoy geometry |
| used by | element; BuoyIsolatedDanger. |
| annotation | documentation. A buoy is a floating object moored to the bottom in a particular place, as an aid to navigation or for other specific purposes. (IHO Dictionary, S-32, 5th Edition, 565).. documentation. A isolated danger buoy is a buoy moored on or above an isolated danger of limited extent, which has navigable water all around it. (UKHO NP735, 5th Edition). |

complexType BuoyLateral

| diagram |  |
| --- | --- |
| type | extension of GenericBuoy |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child buoyShape colour colourPattern radarConspicious marksNavigationalSystemOf natureOfconstuction objectNameInNationalLanguage objectName status verticalAccuracy verticalLength IALA_typeOfBuoy geometry categoryOfLateralMark |
| used by | element; BuoyLateral. |
| annotation | documentation. A buoy is a floating object moored to the bottom in a particular place, as an aid to navigation or for other specific purposes. (IHO Dictionary, S-32, 5th Edition, 565).. documentation. A lateral buoy is used to indicate the port or starboard hand side of the route to be followed. They are generally used for well defined channels and are used in conjunction with a conventional direction of buoyage. (UKHO NP 735, 5th Edition). |

element BuoyLateral/categoryOfLateralMark

| diagram |  |
| --- | --- |
| type | categoryOfLateralMark |
| facets | Kind; Value; Annotation. enumeration; port-hand lateral mark; documentation. indicates the port boundary of a navigational channel or suggested route when proceeding in the 'conventional direction of buoyage'... enumeration; starboard-hand lateral mark; documentation. indicates the starboard boundary of a navigational channel or suggested route when proceeding in the 'conventional direction of buoyage'... enumeration; preferred channel to starboard lateral mark; documentation. at a point where a channel divides, when proceeding in the 'conventional direction of buoyage', the preferred channel (or primary route) is indicated by a modified port-hand lateral mark... enumeration; preferred channel to port lateral mark; documentation. at a point where a channel divides, when proceeding in the 'conventional direction of buoyage', the preferred channel (or primary route) is indicated by a modified starboard-hand lateral mark... |

complexType BuoySafeWater

| diagram |  |
| --- | --- |
| type | extension of GenericBuoy |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child buoyShape colour colourPattern radarConspicious marksNavigationalSystemOf natureOfconstuction objectNameInNationalLanguage objectName status verticalAccuracy verticalLength IALA_typeOfBuoy geometry |
| used by | element; BuoySafeWater. |
| annotation | documentation. A buoy is a floating object moored to the bottom in a particular place, as an aid to navigation or for other specific purposes. (IHO Dictionary, S-32, 5th Edition, 565).. documentation. A safe water buoy is used to indicate that there is navigable water around the mark. (UKHO NP735, 5th Edition). |

complexType BuoySpecialPurposeGeneral

| diagram |  |
| --- | --- |
| type | extension of GenericBuoy |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child buoyShape colour colourPattern radarConspicious marksNavigationalSystemOf natureOfconstuction objectNameInNationalLanguage objectName status verticalAccuracy verticalLength IALA_typeOfBuoy geometry categoryOfSpecialPurposeMark |
| used by | element; BuoySpecialPurposeGeneral. |
| annotation | documentation. A buoy is a floating object moored to the bottom in a particular place, as an aid to navigation or for other specific purposes. (IHO Dictionary, S-32, 5th Edition, 565).. documentation. A special purpose buoy is primarily used to indicate an area or feature, the nature of which is apparent from reference to a chart, Sailing Directions or Notices to Mariners (UKHO NP 735, 5th Edition). Buoy in general: A buoy whose appearance or purpose is not adequately known.. |

element BuoySpecialPurposeGeneral/categoryOfSpecialPurposeMark

| diagram |  |
| --- | --- |
| type | categoryOfSpecialPurposeMark |
| facets | Kind; Value; Annotation. enumeration; firing danger area mark; documentation. a mark used to indicate a firing danger area, usually at sea... enumeration; target mark; documentation. any object toward which something is directed. The distinctive marking or instrumentation of a ground point to aid its identification on a photograph... enumeration; marker ship mark; documentation. a mark marking the position of a ship which is used as a target during some military exercise... enumeration; degaussing range mark; documentation. a mark used to indicate a degaussing range... enumeration; barge mark; documentation. a mark of relevance to barges... enumeration; cable mark; documentation. a mark used to indicate the position of submarine cables or the point at which they run on to the land... enumeration; spoil ground mark; documentation. a mark used to indicate the limit of a spoil ground.. enumeration; outfall mark; documentation. a mark used to indicate the position of an outfall or the point at which it leaves the land... enumeration; ODAS (Ocean-Data-Acquisition-System); documentation. Ocean Data Acquisition System.. enumeration; recording mark; documentation. a mark used to record data for scientific purposes... enumeration; seaplane anchorage mark; documentation. a mark used to indicate a seaplane anchorage... enumeration; recreation zone mark; documentation. a mark used to indicate a recreation zone... enumeration; private mark; documentation. a privately maintained mark... enumeration; mooring mark; documentation. a mark indicating a mooring or moorings... enumeration; LANBY (Large Automatic Navigational Buoy); documentation. a large buoy designed to take the place of a lightship where construction of an offshore light station is not feasible... enumeration; leading mark; documentation. aids to navigation or other indicators so located as to indicate the path to be followed. Leading marks identify a leading line when they are in transit... enumeration; measured distance mark; documentation. a mark forming part of a transit indicating one end of a measured distance... enumeration; notice mark; documentation. a notice board or sign indicating information to the mariner... enumeration; TSS mark (Traffic Separation Scheme); documentation. a mark indicating a traffic separation scheme... enumeration; anchoring prohibited mark; documentation. a mark indicating an anchoring prohibited area... enumeration; berthing prohibited mark; documentation. a mark indicating that berthing is prohibited... enumeration; overtaking prohibited mark; documentation. a mark indicating that overtaking is prohibited... enumeration; two-way traffic prohibited mark; documentation. a mark indicating a one-way route... enumeration; reduced wake mark; documentation. a mark indicating that vessels must not generate excessive wake... enumeration; speed limit mark; documentation. a mark indicating that a speed limit applies... enumeration; stop mark; documentation. a mark indicating the place where the bow of a ship must stop when traffic lights show red... enumeration; general warning mark; documentation. a mark indicating that special caution must be exercised in the vicinity of the mark... enumeration; sound ship's siren mark; documentation. a mark indicating that a ship should sound its siren or horn... enumeration; restricted vertical clearance mark; documentation. a mark indicating the minimum vertical space available for passage... enumeration; maximum vessel's draught mark; documentation. a mark indicating the maximum draught of vessel permitted... enumeration; restricted horizontal clearance mark; documentation. a mark indicating the minimum horizontal space available for passage... enumeration; strong current warning mark; documentation. a mark warning of strong currents... enumeration; berthing permitted mark; documentation. a mark indicating that berthing is allowed... enumeration; overhead power cable mark; documentation. a mark indicating an overhead power cable... enumeration; channel edge gradient mark; documentation. a mark indicating the gradient of the slope of a dredge channel edge... enumeration; telephone mark; documentation. a mark indicating the presence of a telephone... enumeration; ferry crossing mark; documentation. a mark indicating that a ferry route crosses the ship route often used with a 'sound ship's siren' mark... enumeration; pipeline mark; documentation. a mark used to indicate the position of submarine pipelines or the point at which they run on to the land... enumeration; anchorage mark; documentation. a mark indicating an anchorage area... enumeration; clearing mark; documentation. a mark used to indicate a clearing line... enumeration; control mark; documentation. a mark indicating the location at which a restriction or requirement exists... enumeration; diving mark; documentation. a mark indicating that diving may take place in the vicinity... enumeration; refuge beacon; documentation. a mark providing or indicating a place of safety... enumeration; foul ground mark; documentation. a mark indicating a foul ground... enumeration; yachting mark; documentation. a mark installed for use by yachtsmen... enumeration; heliport mark; documentation. a mark indicating an area where helicopters may land... enumeration; GPS mark; documentation. a mark indicating a location at which a GNSS position has been accurately determined... enumeration; seaplane landing mark; documentation. a mark indicating an area where sea-planes land... enumeration; entry prohibited mark; documentation. a mark indicating that entry is prohibited... enumeration; work in progress mark; documentation. a mark indicating that work (generally construction) is in progress... enumeration; mark with unknown purpose; documentation. a mark whose detailed characteristics are unknown... enumeration; wellhead mark; documentation. a mark indicating a borehole that produces or is capable of producing oil or natural gas... enumeration; channel separation mark; documentation. a mark indicating the point at which a channel divides separately into two channels... enumeration; marine farm mark; documentation. a mark indicating the existence of a fish, mussel, oyster or pearl farm/ culture... enumeration; artificial reef mark; documentation. a mark indicating the existence or the extent of an artificial reef... enumeration; IALA_jetski prohibited; documentation. A mark indicating a jetski prohibited area.. |

complexType contactAddress

| diagram |  |
| --- | --- |
| children | deliveryPoint cityName administrativeDivision country postalCode |
| used by | element; StructureObject/contactAddress. |
| annotation | documentation. Direction or superscription of a letter, package, etc., specifying the name of the place to which it is directed, and optionally a contact person or organisation who should receive it. (Oxford English Dictionary, 2nd Ed., adapted).. |

element contactAddress/deliveryPoint

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. Details of where post can be delivered such as the apartment, name and/or number of a street, building or PO Box.. |

element contactAddress/cityName

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. The name of a town or city.. |

element contactAddress/administrativeDivision

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. Administrative division is a generic term for an administrative region within a country at a level below that of the sovereign state.. |

element contactAddress/country

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. The name of a nation. (Adapted from The American Heritage Dictionaries). |

element contactAddress/postalCode

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. Known in various countries as a postcode, or ZIP code, the postal code is a series of letters and/or digits that identifies each postal delivery area.. |

complexType DataCoverage

| diagram |  |
| --- | --- |
| type | extension of MetaFeatureType |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation maximumDisplayScale minimumDisplayScale geometry |
| used by | element; DataCoverage. |
| annotation | documentation. A geographical area that describes the coverage and extent of spatial types. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 1, Page 1.210, November 2000).. |

element DataCoverage/maximumDisplayScale

| diagram |  |
| --- | --- |
| type | xs:positiveInteger |
| annotation | documentation. The largest intended viewing scale for the data.. |

element DataCoverage/minimumDisplayScale

| diagram |  |
| --- | --- |
| type | xs:positiveInteger |
| annotation | documentation. The smallest intended viewing scale for the data.. |

element DataCoverage/geometry

| diagram |  |
| --- | --- |
| type | SurfacePropertyType |
| children | surfaceProperty |

complexType DatasetType

| diagram |  |
| --- | --- |
| type | extension of gml:AbstractFeatureType |
| children | gml:boundedBy DatasetIdentificationInformation DatasetStructureInformation Point MultiPoint Curve CompositeCurve OrientableCurve Surface Polygon imember member |
| used by | element; DataSet. |
| annotation | documentation. Dataset element for dataset as "GML document". |

element DatasetType/DatasetIdentificationInformation

| diagram |  |
| --- | --- |
| type | DataSetIdentificationType |
| children | encodingSpecification encodingSpecificationEdition productIdentifier productEdition applicationProfile datasetFileIdentifier datasetTitle datasetReferenceDate datasetLanguage datasetAbstract datasetTopicCategory |
| annotation | documentation. Dataset identification information. |

element DatasetType/DatasetStructureInformation

| diagram |  |
| --- | --- |
| type | DataSetStructureInformationType |
| children | datasetCoordOriginX datasetCoordOriginY datasetCoordOriginZ coordMultFactorX coordMultFactorY coordMultFactorZ nInfoRec nPointRec nMultiPointRec nCurveRec nCompositeCurveRec nSurfaceRec nFeatureRec |
| annotation | documentation. Dataset structure information. |

element DatasetType/imember

| diagram |  |
| --- | --- |
| type | IMemberType |
| children | InformationType |
| annotation | documentation. intended for S100 information types. Extension of GML practice, not addressed by ISO 19136.. |

element DatasetType/member

| diagram |  |
| --- | --- |
| type | MemberType |
| children | gml:AbstractFeature |
| annotation | documentation. intended for technical GML 3.2 requirement for making the dataset a "GML document" and clause 21.3 of the OGC GML standard. |

complexType Daymark

| diagram |  |
| --- | --- |
| type | extension of Equipment |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_remoteMonitoringSystem IALA_remotelyMonitored parent geometry categoryOfSpecialPurposeMark colour colourPattern elevation height natureOfConstruction objectNameInNationalLanguage objectName status topmarkDaymarkShape verticalAccuracy verticalDatum verticalLength |
| used by | element; Daymark. |
| annotation | documentation. The identifying characteristics of an aid to navigation which serve to facilitate its recognition against a daylight viewing background. On those structures that do not by themselves present an adequate viewing area to be seen at the required distance, the aid is made more visible by affixing a daymark to the structure. A daymark so affixed has a distinctive colour and shape depending on the purpose of the aid. (IHO Dictionary, S-32, 5th Edition, 1248). |

element Daymark/categoryOfSpecialPurposeMark

| diagram |  |
| --- | --- |
| type | categoryOfSpecialPurposeMark |
| facets | Kind; Value; Annotation. enumeration; firing danger area mark; documentation. a mark used to indicate a firing danger area, usually at sea... enumeration; target mark; documentation. any object toward which something is directed. The distinctive marking or instrumentation of a ground point to aid its identification on a photograph... enumeration; marker ship mark; documentation. a mark marking the position of a ship which is used as a target during some military exercise... enumeration; degaussing range mark; documentation. a mark used to indicate a degaussing range... enumeration; barge mark; documentation. a mark of relevance to barges... enumeration; cable mark; documentation. a mark used to indicate the position of submarine cables or the point at which they run on to the land... enumeration; spoil ground mark; documentation. a mark used to indicate the limit of a spoil ground.. enumeration; outfall mark; documentation. a mark used to indicate the position of an outfall or the point at which it leaves the land... enumeration; ODAS (Ocean-Data-Acquisition-System); documentation. Ocean Data Acquisition System.. enumeration; recording mark; documentation. a mark used to record data for scientific purposes... enumeration; seaplane anchorage mark; documentation. a mark used to indicate a seaplane anchorage... enumeration; recreation zone mark; documentation. a mark used to indicate a recreation zone... enumeration; private mark; documentation. a privately maintained mark... enumeration; mooring mark; documentation. a mark indicating a mooring or moorings... enumeration; LANBY (Large Automatic Navigational Buoy); documentation. a large buoy designed to take the place of a lightship where construction of an offshore light station is not feasible... enumeration; leading mark; documentation. aids to navigation or other indicators so located as to indicate the path to be followed. Leading marks identify a leading line when they are in transit... enumeration; measured distance mark; documentation. a mark forming part of a transit indicating one end of a measured distance... enumeration; notice mark; documentation. a notice board or sign indicating information to the mariner... enumeration; TSS mark (Traffic Separation Scheme); documentation. a mark indicating a traffic separation scheme... enumeration; anchoring prohibited mark; documentation. a mark indicating an anchoring prohibited area... enumeration; berthing prohibited mark; documentation. a mark indicating that berthing is prohibited... enumeration; overtaking prohibited mark; documentation. a mark indicating that overtaking is prohibited... enumeration; two-way traffic prohibited mark; documentation. a mark indicating a one-way route... enumeration; reduced wake mark; documentation. a mark indicating that vessels must not generate excessive wake... enumeration; speed limit mark; documentation. a mark indicating that a speed limit applies... enumeration; stop mark; documentation. a mark indicating the place where the bow of a ship must stop when traffic lights show red... enumeration; general warning mark; documentation. a mark indicating that special caution must be exercised in the vicinity of the mark... enumeration; sound ship's siren mark; documentation. a mark indicating that a ship should sound its siren or horn... enumeration; restricted vertical clearance mark; documentation. a mark indicating the minimum vertical space available for passage... enumeration; maximum vessel's draught mark; documentation. a mark indicating the maximum draught of vessel permitted... enumeration; restricted horizontal clearance mark; documentation. a mark indicating the minimum horizontal space available for passage... enumeration; strong current warning mark; documentation. a mark warning of strong currents... enumeration; berthing permitted mark; documentation. a mark indicating that berthing is allowed... enumeration; overhead power cable mark; documentation. a mark indicating an overhead power cable... enumeration; channel edge gradient mark; documentation. a mark indicating the gradient of the slope of a dredge channel edge... enumeration; telephone mark; documentation. a mark indicating the presence of a telephone... enumeration; ferry crossing mark; documentation. a mark indicating that a ferry route crosses the ship route often used with a 'sound ship's siren' mark... enumeration; pipeline mark; documentation. a mark used to indicate the position of submarine pipelines or the point at which they run on to the land... enumeration; anchorage mark; documentation. a mark indicating an anchorage area... enumeration; clearing mark; documentation. a mark used to indicate a clearing line... enumeration; control mark; documentation. a mark indicating the location at which a restriction or requirement exists... enumeration; diving mark; documentation. a mark indicating that diving may take place in the vicinity... enumeration; refuge beacon; documentation. a mark providing or indicating a place of safety... enumeration; foul ground mark; documentation. a mark indicating a foul ground... enumeration; yachting mark; documentation. a mark installed for use by yachtsmen... enumeration; heliport mark; documentation. a mark indicating an area where helicopters may land... enumeration; GPS mark; documentation. a mark indicating a location at which a GNSS position has been accurately determined... enumeration; seaplane landing mark; documentation. a mark indicating an area where sea-planes land... enumeration; entry prohibited mark; documentation. a mark indicating that entry is prohibited... enumeration; work in progress mark; documentation. a mark indicating that work (generally construction) is in progress... enumeration; mark with unknown purpose; documentation. a mark whose detailed characteristics are unknown... enumeration; wellhead mark; documentation. a mark indicating a borehole that produces or is capable of producing oil or natural gas... enumeration; channel separation mark; documentation. a mark indicating the point at which a channel divides separately into two channels... enumeration; marine farm mark; documentation. a mark indicating the existence of a fish, mussel, oyster or pearl farm/ culture... enumeration; artificial reef mark; documentation. a mark indicating the existence or the extent of an artificial reef... enumeration; IALA_jetski prohibited; documentation. A mark indicating a jetski prohibited area.. |

element Daymark/colour

| diagram |  |
| --- | --- |
| type | colour |
| facets | Kind; Value; Annotation. enumeration; white. enumeration; black. enumeration; red. enumeration; green. enumeration; blue. enumeration; yellow. enumeration; grey. enumeration; brown. enumeration; amber. enumeration; violet. enumeration; orange. enumeration; magenta. enumeration; pink. |

element Daymark/colourPattern

| diagram |  |
| --- | --- |
| type | colourPattern |
| facets | Kind; Value; Annotation. enumeration; horizontal stripes; documentation. Straight bands or stripes of differing colours oriented horizontally. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; vertical stripes; documentation. Straight bands or stripes of differing colours oriented vertically. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; diagonal stripes; documentation. Straight bands or stripes of differing colours oriented diagonally (i.e. not horizontally or vertically). (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; squared; documentation. Often referred to as checker plate, where alternate colours are used to create squares similar to a chess or draught board. The pattern may be straight or diagonal. (S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; stripes (direction unknown); documentation. Straight bands or stripes of differing colours oriented in an unknown direction. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; border stripe; documentation. A band or stripe of colour which is displayed around the outer edge of the feature, which may also form a border to an inner pattern or plain colour. (S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; IALA_single colour; documentation. One solid colour of uniform coverage.. enumeration; IALA_rectangle; documentation. A four-sided shape that is made up of two pairs of parallel lines and that has four right angles, on a different coloured background.. enumeration; IALA_triangle; documentation. a shape that is made up of three lines and three angles, on a different coloured background.. |

element Daymark/elevation

| diagram |  |
| --- | --- |
| type | xs:decimal |
| annotation | documentation. The altitude of the ground level of an object, measured from a specified vertical datum.. |

element Daymark/height

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. The value of the vertical distance to the highest point of the object, measured from a specified vertical datum.. |

element Daymark/natureOfConstruction

| diagram |  |
| --- | --- |
| type | natureOfConstruction |
| facets | Kind; Value; Annotation. enumeration; masonry. enumeration; concreted. enumeration; loose boulders. enumeration; hard surfaced. enumeration; unsurfaced. enumeration; wooden. enumeration; metal. enumeration; glass reinforced plastic (GRP). enumeration; painted. enumeration; IALA_fiberglass; documentation. Constructed from fiberglass.. enumeration; IALA_plastic; documentation. Constructed from plastic.. |

element Daymark/objectNameInNationalLanguage

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. Name of object in national language characters.. |

element Daymark/objectName

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. The individual name of an object.. |

element Daymark/status

| diagram |  |
| --- | --- |
| type | status |
| facets | Kind; Value; Annotation. enumeration; permanent. enumeration; occasional. enumeration; recommended. enumeration; not in use. enumeration; periodic/intermittent. enumeration; reserved. enumeration; temporary. enumeration; private. enumeration; mandatory. enumeration; extinguished. enumeration; illuminated. enumeration; historic. enumeration; public. enumeration; synchronized. enumeration; watched. enumeration; un-watched. enumeration; existence doubtful. enumeration; IALA_confirmed; documentation. made certain as to truth, accuracy, validity, availability, etc. - www.dictionary.com.. enumeration; IALA_candidate; documentation. Item selected for an action.. enumeration; IALA_under modification; documentation. Item that is in the process of being modified.. enumeration; IALA_candidate for modification; documentation. Item selected for modification.. enumeration; IALA_under removal/deletion; documentation. Item in the process of being removed or deleted.. enumeration; IALA_removed/deleted; documentation. Item that has been removed or deleted.. |

element Daymark/topmarkDaymarkShape

| diagram |  |
| --- | --- |
| type | topmarkDaymarkShape |
| facets | Kind; Value; Annotation. enumeration; cone, point up. enumeration; cone, point down. enumeration; sphere. enumeration; 2 spheres. enumeration; cylinder (can). enumeration; board. enumeration; x-shape (St. Andrew's cross). enumeration; upright cross (St George's cross). enumeration; cube, point up. enumeration; 2 cones, point to point. enumeration; 2 cones, base to base. enumeration; rhombus (diamond). enumeration; 2 cones (points upward). enumeration; 2 cones (points downward). enumeration; besom, point up (broom or perch). enumeration; besom, point down (broom or perch). enumeration; flag. enumeration; sphere over rhombus. enumeration; square. enumeration; rectangle, horizontal; documentation. where the two longer opposite sides are standing horizontally. A rectangle is a plane figure with four right angles and four straight sides, opposite sides being parallel and equal in length.. enumeration; rectangle, vertical; documentation. where the two longer opposite sides are standing vertically. A rectangle is a plane figure with four right angles and four straight sides, opposite sides being parallel and equal in length.. enumeration; trapezium, up. enumeration; trapezium, down. enumeration; triangle, point up. enumeration; triangle, point down. enumeration; circle. enumeration; two upright crosses (one over the other). enumeration; T-shape. enumeration; triangle pointing up over a circle. enumeration; upright cross over a circle. enumeration; rhombus over a circle. enumeration; circle over a triangle pointing up. enumeration; other shape. enumeration; IALA_tubular. |

element Daymark/verticalAccuracy

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal.1 |
| facets | Kind; Value; Annotation. minInclusive; 0.0. fractionDigits; 1. |
| annotation | documentation. The best estimate of the vertical accuracy of heights, vertical distances and vertical clearances, excluding sounding measurements.. documentation. Expected input: The one-dimensional error. The error is assumed to be positive and negative. The plus/minus character shall not be encoded. Minimum value: 0 Indication: Unit: defined in the HUNI subfield of the DSPM record or in the HUNITS attribute of the M_UNIT meta object class, e.g. metre (m) Resolution: 0.1 m or 0.1 ft Format: xx.x Example: 1.2 for an error of 1.2 metres.. |

element Daymark/verticalDatum

| diagram |  |
| --- | --- |
| type | verticalDatum |
| facets | Kind; Value; Annotation. enumeration; Mean low water springs. enumeration; Mean lower low water springs. enumeration; Mean sea level. enumeration; Lowest low water. enumeration; Mean low water. enumeration; Lowest low water springs. enumeration; Approximate mean low water springs. enumeration; Indian spring low water. enumeration; Low water springs. enumeration; Approximate lowest astronomical tide. enumeration; Nearly lowest low water. enumeration; Mean lower low water. enumeration; Low water. enumeration; Approximate mean low water. enumeration; Approximate mean lower low water. enumeration; Mean high water. enumeration; Mean high water springs. enumeration; High water. enumeration; Approximate mean sea level. enumeration; High water springs. enumeration; Mean higher high water. enumeration; Equinoctial spring low water. enumeration; Lowest astronomical tide. enumeration; Local datum. enumeration; International Great Lakes Datum 1985. enumeration; Mean water level. enumeration; Lower low water large tide. enumeration; Higher high water large tide. enumeration; Nearly highest high water. enumeration; Highest astronomical tide (HAT). |

element Daymark/verticalLength

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. The total vertical length of an object.. documentation. References: INT 1: IE 5; IL 21.3; M-4: 303; Minimum Value: 0 Indication: Unit:defined in the data set metadata, e.g. metre (m) Resolution: 0.1 m or 0.1 ft Format: xxx.x Example: 24.5 for a vertical length of 24.5 metres.. documentation. Remarks: For floating objects: the vertical distance from the surface of water to the highest point of that object. For fixed objects: the vertical distance from seabed or ground to the highest point of that object. For objects on top of other objects: the vertical distance from the lowest to the highest point of that object. Vertical length measurements do not require a datum.. |

complexType Equipment

| diagram |  |
| --- | --- |
| type | extension of AidsToNavigation |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_remoteMonitoringSystem IALA_remotelyMonitored parent geometry |
| used by | element; Equipment. complexTypes; Daymark FogSignal IALA_EnvironmentObservationEquipment Light RadarReflector RadarTransponderBeacon RetroReflector Topmark. |
| annotation | documentation. The implements used in an operation or activity.. |

element Equipment/IALA_remoteMonitoringSystem

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. Definition required. |

element Equipment/IALA_remotelyMonitored

| diagram |  |
| --- | --- |
| type | xs:boolean |
| annotation | documentation. Definition required. |

element Equipment/parent

| diagram |  |
| --- | --- |
| type | gml:ReferenceType |
| annotation | documentation. For association to StructureObject specializations. Note that multiplicity in this XML schema is 0 rather than 1 compared to the UML model because this schema offers an alternative means of encoding the association. For strict conformance to the UML model, the existence should be checked with a constraint rule.. |

element Equipment/geometry

| diagram |  |
| --- | --- |
| type | PointPropertyType |
| children | pointProperty |

complexType FeatureType

| diagram |  |
| --- | --- |
| type | extension of AbstractFeatureType |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation |
| used by | element; FeatureType. complexTypes; Aggregation AidsToNavigation Association MetaFeatureType. |
| annotation | documentation. Generalized feature type which carry all the common attributes. |

complexType FogSignal

| diagram |  |
| --- | --- |
| type | extension of Equipment |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_remoteMonitoringSystem IALA_remotelyMonitored parent geometry categoryOfFogSignal signalFrequency signalGeneration signalGroup signalPeriod signalSequence status valueOfMaximumRange IALA_signalOutput IALA_typeOfBattery |
| used by | element; FogSignal. |
| annotation | documentation. A warning signal transmitted by a vessel, or aid to navigation, during periods of low visibility. Also, the device producing such a signal.. documentation. (IHO Dictionary, S-32, 5th Edition, 1890). |

element FogSignal/categoryOfFogSignal

| diagram |  |
| --- | --- |
| type | categoryOfFogSignal |
| facets | Kind; Value; Annotation. enumeration; explosive; documentation. a signal produced by the firing of explosive charges... enumeration; diaphone; documentation. a diaphone uses compressed air and generally emits a powerful low-pitched sound, which often concludes with a brief sound of suddenly lowered pitch, termed the 'grunt'... enumeration; siren; documentation. a siren uses compressed air and exists in a variety of types which differ considerably in their sound and power... enumeration; nautophone; documentation. a horn having a diaphragm oscillated by electricity.. enumeration; reed; documentation. a reed uses compressed air and emits a weak, high pitched sound... enumeration; tyfon; documentation. a diaphragm horn which operates under the influence of compressed air or steam.. enumeration; bell; documentation. a ringing sound with a short range... enumeration; whistle; documentation. a distinctive sound made by a jet of air passing through an orifice... enumeration; gong; documentation. a sound produced by vibration of a disc when struck... enumeration; horn; documentation. a horn uses compressed air or electricity to vibrate a diaphragm and exists in a variety of types which differ greatly in their sound and power... |

element FogSignal/signalFrequency

| diagram |  |
| --- | --- |
| type | xs:int |
| annotation | documentation. The frequency of a signal.. |

element FogSignal/signalGeneration

| diagram |  |
| --- | --- |
| type | signalGeneration |
| facets | Kind; Value; Annotation. enumeration; automatically. enumeration; by wave action. enumeration; by hand. enumeration; by wind. |

element FogSignal/signalGroup

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. The number of signals, the combination of signals or the morse character(s) within one period of full sequence.. |

element FogSignal/signalPeriod

| diagram |  |
| --- | --- |
| type | xs:decimal |
| annotation | documentation. The time occupied by an entire cycle of intervals of light and eclipse.. |

element FogSignal/signalSequence

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. The sequence of times occupied by intervals of light and eclipse for all 'light characteristics' except for occulting where the sequence of times is occupied by intervals of eclipse and light.. |

element FogSignal/status

| diagram |  |
| --- | --- |
| type | status |
| facets | Kind; Value; Annotation. enumeration; permanent. enumeration; occasional. enumeration; recommended. enumeration; not in use. enumeration; periodic/intermittent. enumeration; reserved. enumeration; temporary. enumeration; private. enumeration; mandatory. enumeration; extinguished. enumeration; illuminated. enumeration; historic. enumeration; public. enumeration; synchronized. enumeration; watched. enumeration; un-watched. enumeration; existence doubtful. enumeration; IALA_confirmed; documentation. made certain as to truth, accuracy, validity, availability, etc. - www.dictionary.com.. enumeration; IALA_candidate; documentation. Item selected for an action.. enumeration; IALA_under modification; documentation. Item that is in the process of being modified.. enumeration; IALA_candidate for modification; documentation. Item selected for modification.. enumeration; IALA_under removal/deletion; documentation. Item in the process of being removed or deleted.. enumeration; IALA_removed/deleted; documentation. Item that has been removed or deleted.. |

element FogSignal/valueOfMaximumRange

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. The extreme distance at which an object can be seen or a signal detected.. |

element FogSignal/IALA_signalOutput

| diagram |  |
| --- | --- |
| type | xs:decimal |
| annotation | documentation. Given in dB. |

element FogSignal/IALA_typeOfBattery

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. Types of storage battery for light e.g. Hi-Ca 100, PS-250E, LDA-400, VGS-350. |

complexType GenericBeacon

| diagram |  |
| --- | --- |
| type | extension of StructureObject |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child beaconShape colour colourPattern condition radarConspicuous visuallyConspicuous elevation height marksNavigationalSystemOf natureOfConstruction verticalLength verticalAccuracy objectNameInNationalLanguage objectName status geometry |
| used by | element; GenericBeacon. complexTypes; BeaconCardinal BeaconIsolatedDanger BeaconLateral BeaconSafeWater BeaconSpecialPurposeGeneral. |
| annotation | documentation. A fixed artificial navigation mark that can be recognised by its shape, colour, pattern, topmark or light character, or a combination of these. It may carry various additional aids to navigation.. |

element GenericBeacon/beaconShape

| diagram |  |
| --- | --- |
| type | beaconShape |
| facets | Kind; Value; Annotation. enumeration; stake, pole, perch, post; documentation. an elongated wood or metal pole, embedded in the bottom to serve as a navigational aid or a support for a navigational aid... enumeration; withy; documentation. a tree without roots stuck or spoiled into the bottom of the sea to serve as a navigational aid... enumeration; beacon tower; documentation. a solid structure of the order of 10 metres in height used as a navigational aid... enumeration; lattice beacon; documentation. a structure consisting of strips of metal or wood crossed or interlaced to form a structure to serve as an aid to navigation or as a support for an aid to navigation... enumeration; pile beacon; documentation. a long heavy timber(s) or section(s) of steel, wood, concrete, etc., forced into the seabed to serve as an aid to navigation or as a support for an aid to navigation... enumeration; cairn; documentation. a mound of stones, usually conical or pyramidal, raised specifically for maritime navigation... enumeration; buoyant beacon; documentation. a tall spar-like beacon fitted with a permanently submerged buoyancy chamber, the lower end of the body is secured to seabed sinker either by a flexible joint or by a cable under tension... |

element GenericBeacon/colour

| diagram |  |
| --- | --- |
| type | colour |
| facets | Kind; Value; Annotation. enumeration; white. enumeration; black. enumeration; red. enumeration; green. enumeration; blue. enumeration; yellow. enumeration; grey. enumeration; brown. enumeration; amber. enumeration; violet. enumeration; orange. enumeration; magenta. enumeration; pink. |

element GenericBeacon/colourPattern

| diagram |  |
| --- | --- |
| type | colourPattern |
| facets | Kind; Value; Annotation. enumeration; horizontal stripes; documentation. Straight bands or stripes of differing colours oriented horizontally. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; vertical stripes; documentation. Straight bands or stripes of differing colours oriented vertically. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; diagonal stripes; documentation. Straight bands or stripes of differing colours oriented diagonally (i.e. not horizontally or vertically). (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; squared; documentation. Often referred to as checker plate, where alternate colours are used to create squares similar to a chess or draught board. The pattern may be straight or diagonal. (S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; stripes (direction unknown); documentation. Straight bands or stripes of differing colours oriented in an unknown direction. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; border stripe; documentation. A band or stripe of colour which is displayed around the outer edge of the feature, which may also form a border to an inner pattern or plain colour. (S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; IALA_single colour; documentation. One solid colour of uniform coverage.. enumeration; IALA_rectangle; documentation. A four-sided shape that is made up of two pairs of parallel lines and that has four right angles, on a different coloured background.. enumeration; IALA_triangle; documentation. a shape that is made up of three lines and three angles, on a different coloured background.. |

element GenericBeacon/condition

| diagram |  |
| --- | --- |
| type | condition |
| facets | Kind; Value; Annotation. enumeration; under construction. enumeration; ruined. enumeration; under reclamation. enumeration; wingless. enumeration; planned construction. |

element GenericBeacon/radarConspicuous

| diagram |  |
| --- | --- |
| type | radarConspicuous |
| facets | Kind; Value; Annotation. enumeration; radar conspicuous. enumeration; not radar conspicuous. enumeration; radar conspicuous (has radar reflector). |

element GenericBeacon/visuallyConspicuous

| diagram |  |
| --- | --- |
| type | visuallyConspicuous |
| facets | Kind; Value; Annotation. enumeration; visually conspicuous. enumeration; not visually conspicuous. |

element GenericBeacon/elevation

| diagram |  |
| --- | --- |
| type | xs:decimal |
| annotation | documentation. The altitude of the ground level of an object, measured from a specified vertical datum.. |

element GenericBeacon/height

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. The value of the vertical distance to the highest point of the object, measured from a specified vertical datum.. |

element GenericBeacon/marksNavigationalSystemOf

| diagram |  |
| --- | --- |
| type | marksNavigationalSystemOf |
| facets | Kind; Value; Annotation. enumeration; IALA A. enumeration; IALA B. enumeration; no system. enumeration; other system. |

element GenericBeacon/natureOfConstruction

| diagram |  |
| --- | --- |
| type | natureOfConstruction |
| facets | Kind; Value; Annotation. enumeration; masonry. enumeration; concreted. enumeration; loose boulders. enumeration; hard surfaced. enumeration; unsurfaced. enumeration; wooden. enumeration; metal. enumeration; glass reinforced plastic (GRP). enumeration; painted. enumeration; IALA_fiberglass; documentation. Constructed from fiberglass.. enumeration; IALA_plastic; documentation. Constructed from plastic.. |

element GenericBeacon/verticalLength

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. The total vertical length of an object.. |

element GenericBeacon/verticalAccuracy

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal.1 |
| facets | Kind; Value; Annotation. minInclusive; 0.0. fractionDigits; 1. |
| annotation | documentation. The best estimate of the vertical accuracy of heights, vertical distances and vertical clearances, excluding sounding measurements. The one-dimensional error. The error is assumed to be positive and negative. The plus/minus character shall not be encoded.. |

element GenericBeacon/objectNameInNationalLanguage

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. Name of object in national language characters.. |

element GenericBeacon/objectName

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. The individual name of an object.. |

element GenericBeacon/status

| diagram |  |
| --- | --- |
| type | status |
| facets | Kind; Value; Annotation. enumeration; permanent. enumeration; occasional. enumeration; recommended. enumeration; not in use. enumeration; periodic/intermittent. enumeration; reserved. enumeration; temporary. enumeration; private. enumeration; mandatory. enumeration; extinguished. enumeration; illuminated. enumeration; historic. enumeration; public. enumeration; synchronized. enumeration; watched. enumeration; un-watched. enumeration; existence doubtful. enumeration; IALA_confirmed; documentation. made certain as to truth, accuracy, validity, availability, etc. - www.dictionary.com.. enumeration; IALA_candidate; documentation. Item selected for an action.. enumeration; IALA_under modification; documentation. Item that is in the process of being modified.. enumeration; IALA_candidate for modification; documentation. Item selected for modification.. enumeration; IALA_under removal/deletion; documentation. Item in the process of being removed or deleted.. enumeration; IALA_removed/deleted; documentation. Item that has been removed or deleted.. |

element GenericBeacon/geometry

| diagram |  |
| --- | --- |
| type | PointPropertyType |
| children | pointProperty |

complexType GenericBuoy

| diagram |  |
| --- | --- |
| type | extension of StructureObject |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child buoyShape colour colourPattern radarConspicious marksNavigationalSystemOf natureOfconstuction objectNameInNationalLanguage objectName status verticalAccuracy verticalLength IALA_typeOfBuoy geometry |
| used by | element; GenericBuoy. complexTypes; BuoyCardinal BuoyInstallation BuoyIsolatedDanger BuoyLateral BuoySafeWater BuoySpecialPurposeGeneral. |
| annotation | documentation. A floating object moored to the bottom in a particular (charted) place, as an aid to navigation or for other specific purposes.. |

element GenericBuoy/buoyShape

| diagram |  |
| --- | --- |
| type | buoyShape |
| facets | Kind; Value; Annotation. enumeration; conical (nun, ogival); documentation. the upper part of the body above the water-line, or the greater part of the superstructure, has approximately the shape or the appearance of a pointed cone with the point upwards... enumeration; can (cylindrical); documentation. the upper part of the body above the water-line, or the greater part of the superstructure, has the shape of a cylinder, or a truncated cone that approximates to a cylinder, with a flat end uppermost... enumeration; spherical; documentation. the upper part of the body above the water-line, or the greater part of the superstructure, has the shape of a part of a sphere... enumeration; pillar; documentation. the upper part of the body above the water-line, or the greater part of the superstructure is a narrow vertical structure, pillar or lattice tower... enumeration; spar (spindle); documentation. the upper part of the body above the water-line, or the greater part of the superstructure, has the form of a pole, or of a very long cylinder, floating upright... enumeration; barrel (tun); documentation. the upper part of the body above the water-line, or the greater part of the superstructure, has the form of a barrel or cylinder floating horizontally... enumeration; super-buoy; documentation. a very large buoy, generally more than 5m in diameter... enumeration; ice buoy; documentation. a specially constructed shuttle shaped buoy which is used in ice conditions... |

element GenericBuoy/colour

| diagram |  |
| --- | --- |
| type | colour |
| facets | Kind; Value; Annotation. enumeration; white. enumeration; black. enumeration; red. enumeration; green. enumeration; blue. enumeration; yellow. enumeration; grey. enumeration; brown. enumeration; amber. enumeration; violet. enumeration; orange. enumeration; magenta. enumeration; pink. |

element GenericBuoy/colourPattern

| diagram |  |
| --- | --- |
| type | colourPattern |
| facets | Kind; Value; Annotation. enumeration; horizontal stripes; documentation. Straight bands or stripes of differing colours oriented horizontally. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; vertical stripes; documentation. Straight bands or stripes of differing colours oriented vertically. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; diagonal stripes; documentation. Straight bands or stripes of differing colours oriented diagonally (i.e. not horizontally or vertically). (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; squared; documentation. Often referred to as checker plate, where alternate colours are used to create squares similar to a chess or draught board. The pattern may be straight or diagonal. (S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; stripes (direction unknown); documentation. Straight bands or stripes of differing colours oriented in an unknown direction. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; border stripe; documentation. A band or stripe of colour which is displayed around the outer edge of the feature, which may also form a border to an inner pattern or plain colour. (S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; IALA_single colour; documentation. One solid colour of uniform coverage.. enumeration; IALA_rectangle; documentation. A four-sided shape that is made up of two pairs of parallel lines and that has four right angles, on a different coloured background.. enumeration; IALA_triangle; documentation. a shape that is made up of three lines and three angles, on a different coloured background.. |

element GenericBuoy/radarConspicious

| diagram |  |
| --- | --- |
| type | radarConspicuous |
| facets | Kind; Value; Annotation. enumeration; radar conspicuous. enumeration; not radar conspicuous. enumeration; radar conspicuous (has radar reflector). |

element GenericBuoy/marksNavigationalSystemOf

| diagram |  |
| --- | --- |
| type | marksNavigationalSystemOf |
| facets | Kind; Value; Annotation. enumeration; IALA A. enumeration; IALA B. enumeration; no system. enumeration; other system. |

element GenericBuoy/natureOfconstuction

| diagram |  |
| --- | --- |
| type | natureOfConstruction |
| facets | Kind; Value; Annotation. enumeration; masonry. enumeration; concreted. enumeration; loose boulders. enumeration; hard surfaced. enumeration; unsurfaced. enumeration; wooden. enumeration; metal. enumeration; glass reinforced plastic (GRP). enumeration; painted. enumeration; IALA_fiberglass; documentation. Constructed from fiberglass.. enumeration; IALA_plastic; documentation. Constructed from plastic.. |

element GenericBuoy/objectNameInNationalLanguage

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. Name of object in national language characters.. |

element GenericBuoy/objectName

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. The individual name of an object.. |

element GenericBuoy/status

| diagram |  |
| --- | --- |
| type | status |
| facets | Kind; Value; Annotation. enumeration; permanent. enumeration; occasional. enumeration; recommended. enumeration; not in use. enumeration; periodic/intermittent. enumeration; reserved. enumeration; temporary. enumeration; private. enumeration; mandatory. enumeration; extinguished. enumeration; illuminated. enumeration; historic. enumeration; public. enumeration; synchronized. enumeration; watched. enumeration; un-watched. enumeration; existence doubtful. enumeration; IALA_confirmed; documentation. made certain as to truth, accuracy, validity, availability, etc. - www.dictionary.com.. enumeration; IALA_candidate; documentation. Item selected for an action.. enumeration; IALA_under modification; documentation. Item that is in the process of being modified.. enumeration; IALA_candidate for modification; documentation. Item selected for modification.. enumeration; IALA_under removal/deletion; documentation. Item in the process of being removed or deleted.. enumeration; IALA_removed/deleted; documentation. Item that has been removed or deleted.. |

element GenericBuoy/verticalAccuracy

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal.1 |
| facets | Kind; Value; Annotation. minInclusive; 0.0. fractionDigits; 1. |
| annotation | documentation. The best estimate of the vertical accuracy of heights, vertical distances and vertical clearances, excluding sounding measurements. The one-dimensional error. The error is assumed to be positive and negative. The plus/minus character shall not be encoded.. |

element GenericBuoy/verticalLength

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. The total vertical length of an object.. |

element GenericBuoy/IALA_typeOfBuoy

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. Types of light buoy e.g. LANBY-100, LS-35, LL-30, LL-28, LL-26, LL-26(M), LL-24, LS-24, LSP-24, LT-10 Types of buoy e.g. U-17C(P), U-17S(P), U-17C(S), U-17S(S), UR-17C(P), UR-17S(P), UR-17C(S), UR-17S(S). |

element GenericBuoy/geometry

| diagram |  |
| --- | --- |
| type | PointPropertyType |
| children | pointProperty |

complexType IALA_EnvironmentObservationEquipment

| diagram |  |
| --- | --- |
| type | extension of Equipment |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_remoteMonitoringSystem IALA_remotelyMonitored parent geometry status height IALA_typeOfEnvironmentObservationEquipment IALA_typeOfBattery |
| used by | element; IALA_EnvironmentObservationEquipment. |
| annotation | documentation. A sensor used to observe the environment.. |

element IALA_EnvironmentObservationEquipment/status

| diagram |  |
| --- | --- |
| type | status |
| facets | Kind; Value; Annotation. enumeration; permanent. enumeration; occasional. enumeration; recommended. enumeration; not in use. enumeration; periodic/intermittent. enumeration; reserved. enumeration; temporary. enumeration; private. enumeration; mandatory. enumeration; extinguished. enumeration; illuminated. enumeration; historic. enumeration; public. enumeration; synchronized. enumeration; watched. enumeration; un-watched. enumeration; existence doubtful. enumeration; IALA_confirmed; documentation. made certain as to truth, accuracy, validity, availability, etc. - www.dictionary.com.. enumeration; IALA_candidate; documentation. Item selected for an action.. enumeration; IALA_under modification; documentation. Item that is in the process of being modified.. enumeration; IALA_candidate for modification; documentation. Item selected for modification.. enumeration; IALA_under removal/deletion; documentation. Item in the process of being removed or deleted.. enumeration; IALA_removed/deleted; documentation. Item that has been removed or deleted.. |

element IALA_EnvironmentObservationEquipment/height

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. The value of the vertical distance to the highest point of the object, measured from a specified vertical datum.. |

element IALA_EnvironmentObservationEquipment/IALA_typeOfEnvironmentObservationEquipment

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. Type of sensor used to observe the environment. e.g. Anemometer, fog monitor, etc. |

element IALA_EnvironmentObservationEquipment/IALA_typeOfBattery

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. Types of storage battery for light e.g. Hi-Ca 100, PS-250E, LDA-400, VGS-350. |

complexType IALA_Lighthouse

| diagram |  |
| --- | --- |
| type | extension of Landmark |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child categoryOfLandmark colour colourPattern condition radarConspicuous visuallyConspicuous elevation function height natureOfConstruction objectNameInNationalLanguage objectName status verticalAccuracy verticalDatum verticalLength IALA_mannedStructure geometry |
| used by | element; IALA_Lighthouse. |
| annotation | documentation. A distinctive structure on or off a coast exhibiting a major light designed to serve as an aid to navigation. (IHO S-32 - 2822, 5th Edition, 1994).. |

complexType IMemberType

| diagram |  |
| --- | --- |
| type | extension of gml:AbstractFeatureMemberType |
| children | InformationType |
| used by | element; DatasetType/imember. |
| annotation | documentation. data format construct for dataset member S-100 information types. |

complexType InformationTypeType

| diagram |  |
| --- | --- |
| type | extension of AbstractInformationType |
| children | informationAssociation invInformationAssociation |
| used by | element; InformationType. complexType; SpatialUncertainty. |
| annotation | documentation. Generalized information type which carry all the common attributes. |

complexType Landmark

| diagram |  |
| --- | --- |
| type | extension of StructureObject |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child categoryOfLandmark colour colourPattern condition radarConspicuous visuallyConspicuous elevation function height natureOfConstruction objectNameInNationalLanguage objectName status verticalAccuracy verticalDatum verticalLength IALA_mannedStructure geometry |
| used by | element; Landmark. complexType; IALA_Lighthouse. |
| annotation | documentation. A prominent object at a fixed location which can be used in determining a location or a direction.. documentation. A relatively permanent structure, roofed and usually walled. It is designed for some particular use which it may be important to indicate. (Digital Geographic Information Working Group, Oct.87). |

element Landmark/categoryOfLandmark

| diagram |  |
| --- | --- |
| type | categoryOfLandmark |
| facets | Kind; Value; Annotation. enumeration; cairn; documentation. a mound of stones, usually conical or pyramidal, raised as a landmark or to designate a point of importance in surveying... enumeration; cemetery; documentation. an area of land for burying the dead... enumeration; chimney; documentation. a vertical structure containing a passage or flue for discharging smoke and gases... enumeration; dish aerial; documentation. a parabolic aerial for the receipt and transmission of high frequency radio signals... enumeration; flagstaff (flagpole); documentation. a staff or pole on which flags are raised... enumeration; flare stack; documentation. a tall structure used for burning-off waste oil or gas. Normally showing a flame and located at refineries... enumeration; mast; documentation. a straight vertical piece of timber or a hollow cylinder... enumeration; windsock; documentation. a tapered fabric sleeve mounted so as to catch and swing with the wind, thus indicating the wind direction... enumeration; monument; documentation. a structure erected or maintained as a memorial to a person or event... enumeration; column (pillar); documentation. a cylindrical or slightly tapering body of considerably greater length than diameter erected vertically... enumeration; memorial plaque; documentation. a slab of metal, usually ornamented, erected as a memorial to a person or event... enumeration; obelisk; documentation. a tapering shaft usually of stone or concrete, square or rectangular in section, with a pyramidal apex... enumeration; statue; documentation. a representation of a human, animal or fantasy figure in marble, bronze, etc... enumeration; cross; documentation. a monument, or other structure in form of a cross... enumeration; dome; documentation. a landmark comprising a hemispherical or spheroidal shaped structure.. enumeration; radar scanner; documentation. a device used for directing a radar beam through a search pattern.. enumeration; tower; documentation. a relatively tall structure which may be used for observation, support, storage or communication etc... enumeration; windmill; documentation. a wind driven system of vanes attached to a tower like structure (excluding wind-generated power plants)... enumeration; windmotor; documentation. a modern structure for the use of windpower... enumeration; spire/minaret; documentation. a tall conical or pyramid-shaped structure often built on the roof or tower of a building, especially a church or mosque... enumeration; large rock or boulder on land; documentation. an isolated rocky formation or a single large stone.. |

element Landmark/colour

| diagram |  |
| --- | --- |
| type | colour |
| facets | Kind; Value; Annotation. enumeration; white. enumeration; black. enumeration; red. enumeration; green. enumeration; blue. enumeration; yellow. enumeration; grey. enumeration; brown. enumeration; amber. enumeration; violet. enumeration; orange. enumeration; magenta. enumeration; pink. |

element Landmark/colourPattern

| diagram |  |
| --- | --- |
| type | colourPattern |
| facets | Kind; Value; Annotation. enumeration; horizontal stripes; documentation. Straight bands or stripes of differing colours oriented horizontally. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; vertical stripes; documentation. Straight bands or stripes of differing colours oriented vertically. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; diagonal stripes; documentation. Straight bands or stripes of differing colours oriented diagonally (i.e. not horizontally or vertically). (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; squared; documentation. Often referred to as checker plate, where alternate colours are used to create squares similar to a chess or draught board. The pattern may be straight or diagonal. (S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; stripes (direction unknown); documentation. Straight bands or stripes of differing colours oriented in an unknown direction. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; border stripe; documentation. A band or stripe of colour which is displayed around the outer edge of the feature, which may also form a border to an inner pattern or plain colour. (S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; IALA_single colour; documentation. One solid colour of uniform coverage.. enumeration; IALA_rectangle; documentation. A four-sided shape that is made up of two pairs of parallel lines and that has four right angles, on a different coloured background.. enumeration; IALA_triangle; documentation. a shape that is made up of three lines and three angles, on a different coloured background.. |

element Landmark/condition

| diagram |  |
| --- | --- |
| type | condition |
| facets | Kind; Value; Annotation. enumeration; under construction. enumeration; ruined. enumeration; under reclamation. enumeration; wingless. enumeration; planned construction. |

element Landmark/radarConspicuous

| diagram |  |
| --- | --- |
| type | radarConspicuous |
| facets | Kind; Value; Annotation. enumeration; radar conspicuous. enumeration; not radar conspicuous. enumeration; radar conspicuous (has radar reflector). |

element Landmark/visuallyConspicuous

| diagram |  |
| --- | --- |
| type | visuallyConspicuous |
| facets | Kind; Value; Annotation. enumeration; visually conspicuous. enumeration; not visually conspicuous. |

element Landmark/elevation

| diagram |  |
| --- | --- |
| type | xs:decimal |
| annotation | documentation. The altitude of the ground level of an object, measured from a specified vertical datum.. |

element Landmark/function

| diagram |  |
| --- | --- |
| type | function |
| facets | Kind; Value; Annotation. enumeration; harbour-master's office. enumeration; custom office. enumeration; health office. enumeration; hospital. enumeration; post office. enumeration; hotel. enumeration; railway station. enumeration; police station. enumeration; water-police station. enumeration; pilot office. enumeration; pilot lookout. enumeration; bank office. enumeration; headquarters for district control. enumeration; transit shed/warehouse. enumeration; factory. enumeration; power station. enumeration; administrative. enumeration; educational facility. enumeration; church. enumeration; chapel. enumeration; temple. enumeration; pagoda. enumeration; shinto shrine. enumeration; buddhist temple. enumeration; mosque. enumeration; marabout. enumeration; lookout. enumeration; communication. enumeration; television. enumeration; radio. enumeration; radar. enumeration; light support. enumeration; microwave. enumeration; cooling. enumeration; observation. enumeration; timeball. enumeration; clock. enumeration; control. enumeration; airship mooring. enumeration; stadium. enumeration; bus station. |

element Landmark/height

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. The value of the vertical distance to the highest point of the object, measured from a specified vertical datum.. |

element Landmark/natureOfConstruction

| diagram |  |
| --- | --- |
| type | natureOfConstruction |
| facets | Kind; Value; Annotation. enumeration; masonry. enumeration; concreted. enumeration; loose boulders. enumeration; hard surfaced. enumeration; unsurfaced. enumeration; wooden. enumeration; metal. enumeration; glass reinforced plastic (GRP). enumeration; painted. enumeration; IALA_fiberglass; documentation. Constructed from fiberglass.. enumeration; IALA_plastic; documentation. Constructed from plastic.. |

element Landmark/objectNameInNationalLanguage

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. Name of object in national language characters.. |

element Landmark/objectName

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. The individual name of an object.. |

element Landmark/status

| diagram |  |
| --- | --- |
| type | status |
| facets | Kind; Value; Annotation. enumeration; permanent. enumeration; occasional. enumeration; recommended. enumeration; not in use. enumeration; periodic/intermittent. enumeration; reserved. enumeration; temporary. enumeration; private. enumeration; mandatory. enumeration; extinguished. enumeration; illuminated. enumeration; historic. enumeration; public. enumeration; synchronized. enumeration; watched. enumeration; un-watched. enumeration; existence doubtful. enumeration; IALA_confirmed; documentation. made certain as to truth, accuracy, validity, availability, etc. - www.dictionary.com.. enumeration; IALA_candidate; documentation. Item selected for an action.. enumeration; IALA_under modification; documentation. Item that is in the process of being modified.. enumeration; IALA_candidate for modification; documentation. Item selected for modification.. enumeration; IALA_under removal/deletion; documentation. Item in the process of being removed or deleted.. enumeration; IALA_removed/deleted; documentation. Item that has been removed or deleted.. |

element Landmark/verticalAccuracy

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal.1 |
| facets | Kind; Value; Annotation. minInclusive; 0.0. fractionDigits; 1. |
| annotation | documentation. The best estimate of the vertical accuracy of heights, vertical distances and vertical clearances, excluding sounding measurements. The one-dimensional error. The error is assumed to be positive and negative. The plus/minus character shall not be encoded.. |

element Landmark/verticalDatum

| diagram |  |
| --- | --- |
| type | verticalDatum |
| facets | Kind; Value; Annotation. enumeration; Mean low water springs. enumeration; Mean lower low water springs. enumeration; Mean sea level. enumeration; Lowest low water. enumeration; Mean low water. enumeration; Lowest low water springs. enumeration; Approximate mean low water springs. enumeration; Indian spring low water. enumeration; Low water springs. enumeration; Approximate lowest astronomical tide. enumeration; Nearly lowest low water. enumeration; Mean lower low water. enumeration; Low water. enumeration; Approximate mean low water. enumeration; Approximate mean lower low water. enumeration; Mean high water. enumeration; Mean high water springs. enumeration; High water. enumeration; Approximate mean sea level. enumeration; High water springs. enumeration; Mean higher high water. enumeration; Equinoctial spring low water. enumeration; Lowest astronomical tide. enumeration; Local datum. enumeration; International Great Lakes Datum 1985. enumeration; Mean water level. enumeration; Lower low water large tide. enumeration; Higher high water large tide. enumeration; Nearly highest high water. enumeration; Highest astronomical tide (HAT). |

element Landmark/verticalLength

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. The total vertical length of an object.. |

element Landmark/IALA_mannedStructure

| diagram |  |
| --- | --- |
| type | xs:boolean |
| annotation | documentation. An expression of the feature being permanently manned or not. |

element Landmark/geometry

| diagram |  |
| --- | --- |
| type | PointCurveSurfacePropertyType |
| children | pointProperty curveProperty surfaceProperty |

complexType Light

| diagram |  |
| --- | --- |
| type | extension of Equipment |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_remoteMonitoringSystem IALA_remotelyMonitored parent geometry categoryOfLight colour exhibitionConditionOfLight height lightCharacteristic lightVisibility marksNavigationalSystemOf multiplicityOfLights objectNameInNationalLanguage objectName orientation sectorLimitOne sectorLimitTwo signalGroup signalPeriod signalSequence status verticalAccuracy valueOfNominalRange verticalDatum IALA_typeOfLight IALA_typeOfBattery IALA_valueOfGeographicalRange IALA_valueOfLuminousRange |
| used by | element; Light. |
| annotation | documentation. A luminous or lighted aid to navigation. (adapted from IHO Dictionary, S-32, 5th Edition, 2766). |

element Light/categoryOfLight

| diagram |  |
| --- | --- |
| type | categoryOfLight |
| facets | Kind; Value; Annotation. enumeration; directional function; documentation. a light illuminating a sector of very narrow angle and intended to mark a direction to follow... enumeration; leading light; documentation. a light associated with other lights so as to form a leading line to be followed... enumeration; aero light; documentation. an aero light is established for aeronautical navigation and may be of higher power than marine lights and visible from well offshore... enumeration; air obstruction light; documentation. a light marking an obstacle which constitutes a danger to air navigation... enumeration; fog detector light; documentation. a light used to automatically determine conditions of visibility which warrant the turning on or off of a sound signal... enumeration; flood light; documentation. a broad beam light used to illuminate a structure or area... enumeration; strip light; documentation. a light whose source has a linear form generally horizontal, which can reach a length of several metres... enumeration; subsidiary light; documentation. a light placed on or near the support of a main light and having a special use in navigation... enumeration; spotlight; documentation. a powerful light focused so as to illuminate a small area... enumeration; front; documentation. term used with leading lights to describe the position of the light on the lead as viewed from seaward... enumeration; rear; documentation. term used with leading lights to describe the position of the light on the lead as viewed from seaward... enumeration; lower; documentation. term used with leading lights to describe the position of the light on the lead as viewed from seaward... enumeration; upper; documentation. term used with leading lights to describe the position of the light on the lead as viewed from seaward... enumeration; moiré effect; documentation. a short range (up to 2km) type of directional light. Sodium lighting gives a yellow background to a screen on which a vertical black line will be seen by an observer on the centre line... enumeration; emergency; documentation. a light available as a backup to a main light which will be illuminated should the main light fail... enumeration; bearing light; documentation. a light which enables its approximate bearing to be obtained without the use of a compass... enumeration; horizontally disposed; documentation. a group of lights of identical character and almost identical position, that are disposed horizontally... enumeration; vertically disposed; documentation. a group of lights of identical character and almost identical position, that are disposed vertically... |

element Light/colour

| diagram |  |
| --- | --- |
| type | colour |
| facets | Kind; Value; Annotation. enumeration; white. enumeration; black. enumeration; red. enumeration; green. enumeration; blue. enumeration; yellow. enumeration; grey. enumeration; brown. enumeration; amber. enumeration; violet. enumeration; orange. enumeration; magenta. enumeration; pink. |

element Light/exhibitionConditionOfLight

| diagram |  |
| --- | --- |
| type | exhibitionConditionOfLight |
| facets | Kind; Value; Annotation. enumeration; light shown without change of character; documentation. a light shown throughout the 24 hours without change of character... enumeration; daytime light; documentation. a light which is only exhibited by day... enumeration; fog light; documentation. a light which is exhibited in fog or conditions of reduced visibility... enumeration; night light; documentation. a light which is only exhibited at night... |

element Light/height

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. The value of the vertical distance to the highest point of the object, measured from a specified vertical datum.. |

element Light/lightCharacteristic

| diagram |  |
| --- | --- |
| type | lightCharacteristic |
| facets | Kind; Value; Annotation. enumeration; fixed. enumeration; flashing. enumeration; long-flashing. enumeration; quick-flashing. enumeration; very quick-flashing. enumeration; ultra quick-flashing. enumeration; isophased. enumeration; occulting. enumeration; interrupted quick-flashing. enumeration; interrupted very quick-flashing. enumeration; interrupted ultra quick-flashing. enumeration; morse. enumeration; fixed/flash. enumeration; flash/long-flash. enumeration; occulting/flash. enumeration; fixed/long-flash. enumeration; occulting alternating. enumeration; long-flash alternating. enumeration; flash alternating. enumeration; group alternating. enumeration; quick-flash plus long-flash. enumeration; very quick-flash plus long-flash. enumeration; ultra quick-flash plus long-flash. enumeration; alternating. enumeration; fixed and alternating flashing. |

element Light/lightVisibility

| diagram |  |
| --- | --- |
| type | lightVisibility |
| facets | Kind; Value; Annotation. enumeration; high intensity. enumeration; low intensity. enumeration; faint. enumeration; intensified. enumeration; unintensified. enumeration; visibility deliberately restricted. enumeration; obscured. enumeration; partially obscured. |

element Light/marksNavigationalSystemOf

| diagram |  |
| --- | --- |
| type | marksNavigationalSystemOf |
| facets | Kind; Value; Annotation. enumeration; IALA A. enumeration; IALA B. enumeration; no system. enumeration; other system. |

element Light/multiplicityOfLights

| diagram |  |
| --- | --- |
| type | multiplicityOfLightsType |
| facets | Kind; Value; Annotation. minInclusive; 2. |
| annotation | documentation. The number of lights of identical character that exist as a co-located group.. documentation. Minimum Value: 2; Indication: Unit: none; Resolution: 1; Format: xx; Example: 5. |

element Light/objectNameInNationalLanguage

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. Name of object in national language characters.. |

element Light/objectName

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. The individual name of an object.. |

element Light/orientation

| diagram |  |
| --- | --- |
| type | Decimal0.0To360.0 |
| facets | Kind; Value; Annotation. minInclusive; 0.0. maxInclusive; 360.0. fractionDigits; 1. |
| annotation | documentation. The angular distance measured from true north to the major axis of the object.. |

element Light/sectorLimitOne

| diagram |  |
| --- | --- |
| type | Decimal0.0To360.0 |
| facets | Kind; Value; Annotation. minInclusive; 0.0. maxInclusive; 360.0. fractionDigits; 1. |
| annotation | documentation. The sector limit 1 specifies the first limit of the sector. The order of sector limit 1 and sector limit 2 is clockwise around the central object (e.g., a light).. |

element Light/sectorLimitTwo

| diagram |  |
| --- | --- |
| type | Decimal0.0To360.0 |
| facets | Kind; Value; Annotation. minInclusive; 0.0. maxInclusive; 360.0. fractionDigits; 1. |
| annotation | documentation. The sector limit 2 specifies the second limit of the sector. The order of sector limit 1 and sector limit 2 is clockwise around the central object (e.g., a light).. |

element Light/signalGroup

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. The number of signals, the combination of signals or the morse character(s) within one period of full sequence.. |

element Light/signalPeriod

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. The time occupied by an entire cycle of intervals of light and eclipse.. |

element Light/signalSequence

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. The sequence of times occupied by intervals of light and eclipse for all 'light characteristics' except for occulting where the sequence of times is occupied by intervals of eclipse and light.. |

element Light/status

| diagram |  |
| --- | --- |
| type | status |
| facets | Kind; Value; Annotation. enumeration; permanent. enumeration; occasional. enumeration; recommended. enumeration; not in use. enumeration; periodic/intermittent. enumeration; reserved. enumeration; temporary. enumeration; private. enumeration; mandatory. enumeration; extinguished. enumeration; illuminated. enumeration; historic. enumeration; public. enumeration; synchronized. enumeration; watched. enumeration; un-watched. enumeration; existence doubtful. enumeration; IALA_confirmed; documentation. made certain as to truth, accuracy, validity, availability, etc. - www.dictionary.com.. enumeration; IALA_candidate; documentation. Item selected for an action.. enumeration; IALA_under modification; documentation. Item that is in the process of being modified.. enumeration; IALA_candidate for modification; documentation. Item selected for modification.. enumeration; IALA_under removal/deletion; documentation. Item in the process of being removed or deleted.. enumeration; IALA_removed/deleted; documentation. Item that has been removed or deleted.. |

element Light/verticalAccuracy

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal.1 |
| facets | Kind; Value; Annotation. minInclusive; 0.0. fractionDigits; 1. |
| annotation | documentation. The best estimate of the vertical accuracy of heights, vertical distances and vertical clearances, excluding sounding measurements. The one-dimensional error. The error is assumed to be positive and negative. The plus/minus character shall not be encoded.. |

element Light/valueOfNominalRange

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. The nominal range at which an object can be seen or a signal detected.. |

element Light/verticalDatum

| diagram |  |
| --- | --- |
| type | verticalDatum |
| facets | Kind; Value; Annotation. enumeration; Mean low water springs. enumeration; Mean lower low water springs. enumeration; Mean sea level. enumeration; Lowest low water. enumeration; Mean low water. enumeration; Lowest low water springs. enumeration; Approximate mean low water springs. enumeration; Indian spring low water. enumeration; Low water springs. enumeration; Approximate lowest astronomical tide. enumeration; Nearly lowest low water. enumeration; Mean lower low water. enumeration; Low water. enumeration; Approximate mean low water. enumeration; Approximate mean lower low water. enumeration; Mean high water. enumeration; Mean high water springs. enumeration; High water. enumeration; Approximate mean sea level. enumeration; High water springs. enumeration; Mean higher high water. enumeration; Equinoctial spring low water. enumeration; Lowest astronomical tide. enumeration; Local datum. enumeration; International Great Lakes Datum 1985. enumeration; Mean water level. enumeration; Lower low water large tide. enumeration; Higher high water large tide. enumeration; Nearly highest high water. enumeration; Highest astronomical tide (HAT). |

element Light/IALA_typeOfLight

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. Types of Light : e.g. Bulb type(250mm, 300mm, 400mm), LED type(Integral type, 200, 200HI, 250, 300, 350). |

element Light/IALA_typeOfBattery

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. Types of storage battery for light e.g. Hi-Ca 100, PS-250E, LDA-400, VGS-350. |

element Light/IALA_valueOfGeographicalRange

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. Definition required. |

element Light/IALA_valueOfLuminousRange

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. Definition required. |

complexType LightFloat

| diagram |  |
| --- | --- |
| type | extension of StructureObject |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child colour colourPattern radarConspicuous visuallyConspicuous horizontalAccuracy horizontalLength horizontalWidth natureOfConstruction objectNameInNationalLanguage objectName status verticalAccuracy verticalLength IALA_mannedStructure geometry |
| used by | element; LightFloat. |
| annotation | documentation. A boat-like structure used instead of a light buoy in waters where strong streams or currents are experienced, or when a greater elevation than that of a light buoy is necessary.. |

element LightFloat/colour

| diagram |  |
| --- | --- |
| type | colour |
| facets | Kind; Value; Annotation. enumeration; white. enumeration; black. enumeration; red. enumeration; green. enumeration; blue. enumeration; yellow. enumeration; grey. enumeration; brown. enumeration; amber. enumeration; violet. enumeration; orange. enumeration; magenta. enumeration; pink. |

element LightFloat/colourPattern

| diagram |  |
| --- | --- |
| type | colourPattern |
| facets | Kind; Value; Annotation. enumeration; horizontal stripes; documentation. Straight bands or stripes of differing colours oriented horizontally. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; vertical stripes; documentation. Straight bands or stripes of differing colours oriented vertically. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; diagonal stripes; documentation. Straight bands or stripes of differing colours oriented diagonally (i.e. not horizontally or vertically). (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; squared; documentation. Often referred to as checker plate, where alternate colours are used to create squares similar to a chess or draught board. The pattern may be straight or diagonal. (S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; stripes (direction unknown); documentation. Straight bands or stripes of differing colours oriented in an unknown direction. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; border stripe; documentation. A band or stripe of colour which is displayed around the outer edge of the feature, which may also form a border to an inner pattern or plain colour. (S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; IALA_single colour; documentation. One solid colour of uniform coverage.. enumeration; IALA_rectangle; documentation. A four-sided shape that is made up of two pairs of parallel lines and that has four right angles, on a different coloured background.. enumeration; IALA_triangle; documentation. a shape that is made up of three lines and three angles, on a different coloured background.. |

element LightFloat/radarConspicuous

| diagram |  |
| --- | --- |
| type | radarConspicuous |
| facets | Kind; Value; Annotation. enumeration; radar conspicuous. enumeration; not radar conspicuous. enumeration; radar conspicuous (has radar reflector). |

element LightFloat/visuallyConspicuous

| diagram |  |
| --- | --- |
| type | visuallyConspicuous |
| facets | Kind; Value; Annotation. enumeration; visually conspicuous. enumeration; not visually conspicuous. |

element LightFloat/horizontalAccuracy

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. The best estimate of the horizontal accuracy of horizontal clearance and distances.. |

element LightFloat/horizontalLength

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. A measurement of the longer of two linear axis.. |

element LightFloat/horizontalWidth

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. A measurement of the shorter of two linear axis.. |

element LightFloat/natureOfConstruction

| diagram |  |
| --- | --- |
| type | natureOfConstruction |
| facets | Kind; Value; Annotation. enumeration; masonry. enumeration; concreted. enumeration; loose boulders. enumeration; hard surfaced. enumeration; unsurfaced. enumeration; wooden. enumeration; metal. enumeration; glass reinforced plastic (GRP). enumeration; painted. enumeration; IALA_fiberglass; documentation. Constructed from fiberglass.. enumeration; IALA_plastic; documentation. Constructed from plastic.. |

element LightFloat/objectNameInNationalLanguage

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. Name of object in national language characters.. |

element LightFloat/objectName

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. The individual name of an object.. |

element LightFloat/status

| diagram |  |
| --- | --- |
| type | status |
| facets | Kind; Value; Annotation. enumeration; permanent. enumeration; occasional. enumeration; recommended. enumeration; not in use. enumeration; periodic/intermittent. enumeration; reserved. enumeration; temporary. enumeration; private. enumeration; mandatory. enumeration; extinguished. enumeration; illuminated. enumeration; historic. enumeration; public. enumeration; synchronized. enumeration; watched. enumeration; un-watched. enumeration; existence doubtful. enumeration; IALA_confirmed; documentation. made certain as to truth, accuracy, validity, availability, etc. - www.dictionary.com.. enumeration; IALA_candidate; documentation. Item selected for an action.. enumeration; IALA_under modification; documentation. Item that is in the process of being modified.. enumeration; IALA_candidate for modification; documentation. Item selected for modification.. enumeration; IALA_under removal/deletion; documentation. Item in the process of being removed or deleted.. enumeration; IALA_removed/deleted; documentation. Item that has been removed or deleted.. |

element LightFloat/verticalAccuracy

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal.1 |
| facets | Kind; Value; Annotation. minInclusive; 0.0. fractionDigits; 1. |
| annotation | documentation. The best estimate of the vertical accuracy of heights, vertical distances and vertical clearances, excluding sounding measurements. The one-dimensional error. The error is assumed to be positive and negative. The plus/minus character shall not be encoded.. |

element LightFloat/verticalLength

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. The total vertical length of an object.. |

element LightFloat/IALA_mannedStructure

| diagram |  |
| --- | --- |
| type | xs:boolean |

element LightFloat/geometry

| diagram |  |
| --- | --- |
| type | PointPropertyType |
| children | pointProperty |

complexType LightVessel

| diagram |  |
| --- | --- |
| type | extension of StructureObject |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child colour colourPattern radarConspicuous visuallyConspicuous horizontalAccuracy horizontalLength horizontalWidth natureOfConstruction objectNameInNationalLanguage objectName status verticalAccuracy verticalLength IALA_mannedStructure geometry |
| used by | element; LightVessel. |
| annotation | documentation. A distinctively marked vessel anchored or moored at a charted point, to serve as an aid to navigation. By night, it displays a characteristic light(s) and is usually equipped with other devices, such as fog signal, submarine sound signal, and radio-beacon, to assist navigation. Also called light ship. (IHO Dictionary, S-32, 5th Edition, 2828,2829). |

element LightVessel/colour

| diagram |  |
| --- | --- |
| type | colour |
| facets | Kind; Value; Annotation. enumeration; white. enumeration; black. enumeration; red. enumeration; green. enumeration; blue. enumeration; yellow. enumeration; grey. enumeration; brown. enumeration; amber. enumeration; violet. enumeration; orange. enumeration; magenta. enumeration; pink. |

element LightVessel/colourPattern

| diagram |  |
| --- | --- |
| type | colourPattern |
| facets | Kind; Value; Annotation. enumeration; horizontal stripes; documentation. Straight bands or stripes of differing colours oriented horizontally. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; vertical stripes; documentation. Straight bands or stripes of differing colours oriented vertically. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; diagonal stripes; documentation. Straight bands or stripes of differing colours oriented diagonally (i.e. not horizontally or vertically). (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; squared; documentation. Often referred to as checker plate, where alternate colours are used to create squares similar to a chess or draught board. The pattern may be straight or diagonal. (S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; stripes (direction unknown); documentation. Straight bands or stripes of differing colours oriented in an unknown direction. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; border stripe; documentation. A band or stripe of colour which is displayed around the outer edge of the feature, which may also form a border to an inner pattern or plain colour. (S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; IALA_single colour; documentation. One solid colour of uniform coverage.. enumeration; IALA_rectangle; documentation. A four-sided shape that is made up of two pairs of parallel lines and that has four right angles, on a different coloured background.. enumeration; IALA_triangle; documentation. a shape that is made up of three lines and three angles, on a different coloured background.. |

element LightVessel/radarConspicuous

| diagram |  |
| --- | --- |
| type | radarConspicuous |
| facets | Kind; Value; Annotation. enumeration; radar conspicuous. enumeration; not radar conspicuous. enumeration; radar conspicuous (has radar reflector). |

element LightVessel/visuallyConspicuous

| diagram |  |
| --- | --- |
| type | visuallyConspicuous |
| facets | Kind; Value; Annotation. enumeration; visually conspicuous. enumeration; not visually conspicuous. |

element LightVessel/horizontalAccuracy

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. The best estimate of the horizontal accuracy of horizontal clearance and distances.. |

element LightVessel/horizontalLength

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. A measurement of the longer of two linear axis.. |

element LightVessel/horizontalWidth

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. A measurement of the shorter of two linear axis.. |

element LightVessel/natureOfConstruction

| diagram |  |
| --- | --- |
| type | natureOfConstruction |
| facets | Kind; Value; Annotation. enumeration; masonry. enumeration; concreted. enumeration; loose boulders. enumeration; hard surfaced. enumeration; unsurfaced. enumeration; wooden. enumeration; metal. enumeration; glass reinforced plastic (GRP). enumeration; painted. enumeration; IALA_fiberglass; documentation. Constructed from fiberglass.. enumeration; IALA_plastic; documentation. Constructed from plastic.. |

element LightVessel/objectNameInNationalLanguage

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. Name of object in national language characters.. |

element LightVessel/objectName

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. The individual name of an object.. |

element LightVessel/status

| diagram |  |
| --- | --- |
| type | status |
| facets | Kind; Value; Annotation. enumeration; permanent. enumeration; occasional. enumeration; recommended. enumeration; not in use. enumeration; periodic/intermittent. enumeration; reserved. enumeration; temporary. enumeration; private. enumeration; mandatory. enumeration; extinguished. enumeration; illuminated. enumeration; historic. enumeration; public. enumeration; synchronized. enumeration; watched. enumeration; un-watched. enumeration; existence doubtful. enumeration; IALA_confirmed; documentation. made certain as to truth, accuracy, validity, availability, etc. - www.dictionary.com.. enumeration; IALA_candidate; documentation. Item selected for an action.. enumeration; IALA_under modification; documentation. Item that is in the process of being modified.. enumeration; IALA_candidate for modification; documentation. Item selected for modification.. enumeration; IALA_under removal/deletion; documentation. Item in the process of being removed or deleted.. enumeration; IALA_removed/deleted; documentation. Item that has been removed or deleted.. |

element LightVessel/verticalAccuracy

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal.1 |
| facets | Kind; Value; Annotation. minInclusive; 0.0. fractionDigits; 1. |
| annotation | documentation. The best estimate of the vertical accuracy of heights, vertical distances and vertical clearances, excluding sounding measurements. The one-dimensional error. The error is assumed to be positive and negative. The plus/minus character shall not be encoded.. |

element LightVessel/verticalLength

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. The total vertical length of an object.. |

element LightVessel/IALA_mannedStructure

| diagram |  |
| --- | --- |
| type | xs:boolean |

element LightVessel/geometry

| diagram |  |
| --- | --- |
| type | PointPropertyType |
| children | pointProperty |

complexType LocalDirectionOfBuoyage

| diagram |  |
| --- | --- |
| type | extension of MetaFeatureType |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation orientation geometry |
| used by | element; LocalDirectionOfBuoyage. |
| annotation | documentation. An area within which the navigational system of marks has been established in relation to a specific direction.. |

element LocalDirectionOfBuoyage/orientation

| diagram |  |
| --- | --- |
| type | Decimal0.0To360.0 |
| facets | Kind; Value; Annotation. minInclusive; 0.0. maxInclusive; 360.0. fractionDigits; 1. |
| annotation | documentation. The angular distance measured from true north to the major axis of the object.. |

element LocalDirectionOfBuoyage/geometry

| diagram |  |
| --- | --- |
| type | SurfacePropertyType |
| children | surfaceProperty |
| annotation | documentation. Permitted spatial primitives. |

complexType MemberType

| diagram |  |
| --- | --- |
| type | extension of gml:AbstractFeatureMemberType |
| children | gml:AbstractFeature |
| used by | element; DatasetType/member. |
| annotation | documentation. data format construct for dataset members. |

complexType MetaFeatureType

| diagram |  |
| --- | --- |
| type | extension of FeatureType |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation |
| used by | element; MetaFeatureType. complexTypes; DataCoverage LocalDirectionOfBuoyage NavigationalSystemOfMarks QualityOfNonBathymetricData SoundingDatum VerticalDatum. |
| annotation | documentation. Generalized feature type which carry all the common attributes. |

complexType NavigationalSystemOfMarks

| diagram |  |
| --- | --- |
| type | extension of MetaFeatureType |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation marksNavigationalSystemOf geometry |
| used by | element; NavigationalSystemOfMarks. |
| annotation | documentation. An area within which a specific system of navigational marks applies and/or a common direction of buoyage. |

element NavigationalSystemOfMarks/marksNavigationalSystemOf

| diagram |  |
| --- | --- |
| type | marksNavigationalSystemOf |
| facets | Kind; Value; Annotation. enumeration; IALA A. enumeration; IALA B. enumeration; no system. enumeration; other system. |

element NavigationalSystemOfMarks/geometry

| diagram |  |
| --- | --- |
| type | SurfacePropertyType |
| children | surfaceProperty |
| annotation | documentation. Permitted spatial primitives. |

complexType NavigationLine

| diagram |  |
| --- | --- |
| type | extension of AidsToNavigation |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer categoryOfNavigationLine orientation status navigableTrack geometry |
| used by | element; NavigationLine. |
| annotation | documentation. A navigation line is a straight line extending towards an area of navigational interest and generally generated by two navigational aids or one navigational aid and a bearing. (Service Hydrographique et Océanographique de la Marine, France). |

element NavigationLine/categoryOfNavigationLine

| diagram |  |
| --- | --- |
| type | categoryOfNavigationLine |
| facets | Kind; Value; Annotation. enumeration; clearing line; documentation. a straight line that marks the boundary between a safe and a dangerous area or that passes clear of a navigational danger... enumeration; transit line; documentation. a line passing through one or more fixed marks... enumeration; leading line bearing a recommended track; documentation. a line passing through one or more clearly defined objects, along the path of which a vessel can approach safely up to a certain distance off... |

element NavigationLine/orientation

| diagram |  |
| --- | --- |
| type | Decimal0.0To360.0 |
| facets | Kind; Value; Annotation. minInclusive; 0.0. maxInclusive; 360.0. fractionDigits; 1. |
| annotation | documentation. The angular distance measured from true north to the major axis of the object.. |

element NavigationLine/status

| diagram |  |
| --- | --- |
| type | status |
| facets | Kind; Value; Annotation. enumeration; permanent. enumeration; occasional. enumeration; recommended. enumeration; not in use. enumeration; periodic/intermittent. enumeration; reserved. enumeration; temporary. enumeration; private. enumeration; mandatory. enumeration; extinguished. enumeration; illuminated. enumeration; historic. enumeration; public. enumeration; synchronized. enumeration; watched. enumeration; un-watched. enumeration; existence doubtful. enumeration; IALA_confirmed; documentation. made certain as to truth, accuracy, validity, availability, etc. - www.dictionary.com.. enumeration; IALA_candidate; documentation. Item selected for an action.. enumeration; IALA_under modification; documentation. Item that is in the process of being modified.. enumeration; IALA_candidate for modification; documentation. Item selected for modification.. enumeration; IALA_under removal/deletion; documentation. Item in the process of being removed or deleted.. enumeration; IALA_removed/deleted; documentation. Item that has been removed or deleted.. |

element NavigationLine/navigableTrack

| diagram |  |
| --- | --- |
| type | gml:ReferenceType |
| annotation | documentation. Encodes the RecommendedTrack/NavigationLine association. |

element NavigationLine/geometry

| diagram |  |
| --- | --- |
| type | CurvePropertyType |
| children | curveProperty |

complexType OffshorePlatform

| diagram |  |
| --- | --- |
| type | extension of StructureObject |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child categoryOfOffshorePlatform colour colourPattern condition radarConspicuous visuallyConspicuous height natureOfConstruction objectNameInNationalLanguage product objectName status verticalAccuracy verticalDatum verticalLength IALA_mannedStructure geometry |
| used by | element; OffshorePlatform. |
| annotation | documentation. A permanent offshore structure, either fixed or floating, used in the production of oil or natural gas. (IHO Dictionary, S-32, 5th Edition, 3895). |

element OffshorePlatform/categoryOfOffshorePlatform

| diagram |  |
| --- | --- |
| type | categoryOfOffshorePlatform |
| facets | Kind; Value; Annotation. enumeration; oil derrick/rig; documentation. a temporary mobile structure, either fixed or floating, used in the exploration stages of oil and gas fields... enumeration; production platform; documentation. a term used to indicate a permanent offshore structure equipped to control the flow of oil or gas. It does not include entirely submarine structures... enumeration; observation/research platform; documentation. a platform from which one's surroundings or events can be observed, noted or recorded such as for scientific study... enumeration; articulated loading platform (ALP); documentation. a metal lattice tower, buoyant at one end and attached at the other by a universal joint to a concrete filled base on the sea bed. The platform may be fitted with a helicopter platform, emergency accommodation and hawser/hose retrieval... enumeration; single anchor leg mooring (SALM); documentation. a rigid frame or tube with a buoyancy device at its upper end , secured at its lower end to a universal joint on a large steel or concrete base resting on the sea bed, and at its upper end to a mooring buoy by a chain or wire... enumeration; mooring tower; documentation. a platform secured to the sea bed and surmounted by a turntable to which ships moor... enumeration; artificial island; documentation. a man-made structure usually built for the exploration or exploitation of marine resources, marine scientific research, tidal observations, etc... enumeration; floating production, storage and off-loading vessel (FPSO); documentation. an offshore oil/gas facility consisting of a moored tanker/barge by which the product is extracted, stored and exported... enumeration; accommodation platform; documentation. a platform used primarily for eating, sleeping and recreation purposes... enumeration; navigation, communication and control buoy (NCCB); documentation. a floating structure with control room, power and storage facilities, attached to the sea bed by a flexible pipeline and cables... |

element OffshorePlatform/colour

| diagram |  |
| --- | --- |
| type | colour |
| facets | Kind; Value; Annotation. enumeration; white. enumeration; black. enumeration; red. enumeration; green. enumeration; blue. enumeration; yellow. enumeration; grey. enumeration; brown. enumeration; amber. enumeration; violet. enumeration; orange. enumeration; magenta. enumeration; pink. |

element OffshorePlatform/colourPattern

| diagram |  |
| --- | --- |
| type | colourPattern |
| facets | Kind; Value; Annotation. enumeration; horizontal stripes; documentation. Straight bands or stripes of differing colours oriented horizontally. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; vertical stripes; documentation. Straight bands or stripes of differing colours oriented vertically. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; diagonal stripes; documentation. Straight bands or stripes of differing colours oriented diagonally (i.e. not horizontally or vertically). (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; squared; documentation. Often referred to as checker plate, where alternate colours are used to create squares similar to a chess or draught board. The pattern may be straight or diagonal. (S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; stripes (direction unknown); documentation. Straight bands or stripes of differing colours oriented in an unknown direction. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; border stripe; documentation. A band or stripe of colour which is displayed around the outer edge of the feature, which may also form a border to an inner pattern or plain colour. (S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; IALA_single colour; documentation. One solid colour of uniform coverage.. enumeration; IALA_rectangle; documentation. A four-sided shape that is made up of two pairs of parallel lines and that has four right angles, on a different coloured background.. enumeration; IALA_triangle; documentation. a shape that is made up of three lines and three angles, on a different coloured background.. |

element OffshorePlatform/condition

| diagram |  |
| --- | --- |
| type | condition |
| facets | Kind; Value; Annotation. enumeration; under construction. enumeration; ruined. enumeration; under reclamation. enumeration; wingless. enumeration; planned construction. |

element OffshorePlatform/radarConspicuous

| diagram |  |
| --- | --- |
| type | radarConspicuous |
| facets | Kind; Value; Annotation. enumeration; radar conspicuous. enumeration; not radar conspicuous. enumeration; radar conspicuous (has radar reflector). |

element OffshorePlatform/visuallyConspicuous

| diagram |  |
| --- | --- |
| type | visuallyConspicuous |
| facets | Kind; Value; Annotation. enumeration; visually conspicuous. enumeration; not visually conspicuous. |

element OffshorePlatform/height

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. The value of the vertical distance to the highest point of the object, measured from a specified vertical datum.. |

element OffshorePlatform/natureOfConstruction

| diagram |  |
| --- | --- |
| type | natureOfConstruction |
| facets | Kind; Value; Annotation. enumeration; masonry. enumeration; concreted. enumeration; loose boulders. enumeration; hard surfaced. enumeration; unsurfaced. enumeration; wooden. enumeration; metal. enumeration; glass reinforced plastic (GRP). enumeration; painted. enumeration; IALA_fiberglass; documentation. Constructed from fiberglass.. enumeration; IALA_plastic; documentation. Constructed from plastic.. |

element OffshorePlatform/objectNameInNationalLanguage

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. Name of object in national language characters.. |

element OffshorePlatform/product

| diagram |  |
| --- | --- |
| type | product |
| facets | Kind; Value; Annotation. enumeration; oil. enumeration; gas. enumeration; water. enumeration; stone. enumeration; coal. enumeration; ore. enumeration; chemicals. enumeration; drinking water. enumeration; milk. enumeration; bauxite. enumeration; coke. enumeration; iron ingots. enumeration; salt. enumeration; sand. enumeration; timber. enumeration; sawdust/wood chips. enumeration; scrap metal. enumeration; liquified natural gas (LNG). enumeration; liquified petroleum gas (LPG). enumeration; wine. enumeration; cement. enumeration; grain. |

element OffshorePlatform/objectName

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. The individual name of an object.. |

element OffshorePlatform/status

| diagram |  |
| --- | --- |
| type | status |
| facets | Kind; Value; Annotation. enumeration; permanent. enumeration; occasional. enumeration; recommended. enumeration; not in use. enumeration; periodic/intermittent. enumeration; reserved. enumeration; temporary. enumeration; private. enumeration; mandatory. enumeration; extinguished. enumeration; illuminated. enumeration; historic. enumeration; public. enumeration; synchronized. enumeration; watched. enumeration; un-watched. enumeration; existence doubtful. enumeration; IALA_confirmed; documentation. made certain as to truth, accuracy, validity, availability, etc. - www.dictionary.com.. enumeration; IALA_candidate; documentation. Item selected for an action.. enumeration; IALA_under modification; documentation. Item that is in the process of being modified.. enumeration; IALA_candidate for modification; documentation. Item selected for modification.. enumeration; IALA_under removal/deletion; documentation. Item in the process of being removed or deleted.. enumeration; IALA_removed/deleted; documentation. Item that has been removed or deleted.. |

element OffshorePlatform/verticalAccuracy

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal.1 |
| facets | Kind; Value; Annotation. minInclusive; 0.0. fractionDigits; 1. |
| annotation | documentation. The best estimate of the vertical accuracy of heights, vertical distances and vertical clearances, excluding sounding measurements. The one-dimensional error. The error is assumed to be positive and negative. The plus/minus character shall not be encoded.. |

element OffshorePlatform/verticalDatum

| diagram |  |
| --- | --- |
| type | verticalDatum |
| facets | Kind; Value; Annotation. enumeration; Mean low water springs. enumeration; Mean lower low water springs. enumeration; Mean sea level. enumeration; Lowest low water. enumeration; Mean low water. enumeration; Lowest low water springs. enumeration; Approximate mean low water springs. enumeration; Indian spring low water. enumeration; Low water springs. enumeration; Approximate lowest astronomical tide. enumeration; Nearly lowest low water. enumeration; Mean lower low water. enumeration; Low water. enumeration; Approximate mean low water. enumeration; Approximate mean lower low water. enumeration; Mean high water. enumeration; Mean high water springs. enumeration; High water. enumeration; Approximate mean sea level. enumeration; High water springs. enumeration; Mean higher high water. enumeration; Equinoctial spring low water. enumeration; Lowest astronomical tide. enumeration; Local datum. enumeration; International Great Lakes Datum 1985. enumeration; Mean water level. enumeration; Lower low water large tide. enumeration; Higher high water large tide. enumeration; Nearly highest high water. enumeration; Highest astronomical tide (HAT). |

element OffshorePlatform/verticalLength

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. The total vertical length of an object.. |

element OffshorePlatform/IALA_mannedStructure

| diagram |  |
| --- | --- |
| type | xs:boolean |
| annotation | documentation. Definition required. |

element OffshorePlatform/geometry

| diagram |  |
| --- | --- |
| type | PointOrSurfacePropertyType |
| children | pointProperty surfaceProperty |

complexType Pile

| diagram |  |
| --- | --- |
| type | extension of StructureObject |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child categoryOfPile colour colourPattern condition visuallyConspicuous height verticalAccuracy verticalDatum verticalLength geometry |
| used by | element; Pile. |
| annotation | documentation. A long heavy timber or section of steel, wood, concrete, etc.. forced into the earth which may serve as a support, as for a pier, or a free standing pole within a marine environment. (Adapted from IHO Dictionary, S-32, 5th Edition, 3840). |

element Pile/categoryOfPile

| diagram |  |
| --- | --- |
| type | categoryOfPile |
| facets | Kind; Value; Annotation. enumeration; stake; documentation. an elongated wood or metal pole embedded in the bottom to serve as a marker or support... enumeration; post; documentation. a vertical piece of timber, metal or concrete forced into the earth or sea bed... enumeration; tripodal; documentation. a single structure comprising 3 or more piles held together (sections of heavy timber, steel or concrete), and forced into the earth or sea bed... |

element Pile/colour

| diagram |  |
| --- | --- |
| type | colour |
| facets | Kind; Value; Annotation. enumeration; white. enumeration; black. enumeration; red. enumeration; green. enumeration; blue. enumeration; yellow. enumeration; grey. enumeration; brown. enumeration; amber. enumeration; violet. enumeration; orange. enumeration; magenta. enumeration; pink. |

element Pile/colourPattern

| diagram |  |
| --- | --- |
| type | colourPattern |
| facets | Kind; Value; Annotation. enumeration; horizontal stripes; documentation. Straight bands or stripes of differing colours oriented horizontally. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; vertical stripes; documentation. Straight bands or stripes of differing colours oriented vertically. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; diagonal stripes; documentation. Straight bands or stripes of differing colours oriented diagonally (i.e. not horizontally or vertically). (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; squared; documentation. Often referred to as checker plate, where alternate colours are used to create squares similar to a chess or draught board. The pattern may be straight or diagonal. (S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; stripes (direction unknown); documentation. Straight bands or stripes of differing colours oriented in an unknown direction. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; border stripe; documentation. A band or stripe of colour which is displayed around the outer edge of the feature, which may also form a border to an inner pattern or plain colour. (S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; IALA_single colour; documentation. One solid colour of uniform coverage.. enumeration; IALA_rectangle; documentation. A four-sided shape that is made up of two pairs of parallel lines and that has four right angles, on a different coloured background.. enumeration; IALA_triangle; documentation. a shape that is made up of three lines and three angles, on a different coloured background.. |

element Pile/condition

| diagram |  |
| --- | --- |
| type | condition |
| facets | Kind; Value; Annotation. enumeration; under construction. enumeration; ruined. enumeration; under reclamation. enumeration; wingless. enumeration; planned construction. |

element Pile/visuallyConspicuous

| diagram |  |
| --- | --- |
| type | visuallyConspicuous |
| facets | Kind; Value; Annotation. enumeration; visually conspicuous. enumeration; not visually conspicuous. |

element Pile/height

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. The value of the vertical distance to the highest point of the object, measured from a specified vertical datum.. |

element Pile/verticalAccuracy

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal.1 |
| facets | Kind; Value; Annotation. minInclusive; 0.0. fractionDigits; 1. |
| annotation | documentation. The best estimate of the vertical accuracy of heights, vertical distances and vertical clearances, excluding sounding measurements. The one-dimensional error. The error is assumed to be positive and negative. The plus/minus character shall not be encoded.. |

element Pile/verticalDatum

| diagram |  |
| --- | --- |
| type | verticalDatum |
| facets | Kind; Value; Annotation. enumeration; Mean low water springs. enumeration; Mean lower low water springs. enumeration; Mean sea level. enumeration; Lowest low water. enumeration; Mean low water. enumeration; Lowest low water springs. enumeration; Approximate mean low water springs. enumeration; Indian spring low water. enumeration; Low water springs. enumeration; Approximate lowest astronomical tide. enumeration; Nearly lowest low water. enumeration; Mean lower low water. enumeration; Low water. enumeration; Approximate mean low water. enumeration; Approximate mean lower low water. enumeration; Mean high water. enumeration; Mean high water springs. enumeration; High water. enumeration; Approximate mean sea level. enumeration; High water springs. enumeration; Mean higher high water. enumeration; Equinoctial spring low water. enumeration; Lowest astronomical tide. enumeration; Local datum. enumeration; International Great Lakes Datum 1985. enumeration; Mean water level. enumeration; Lower low water large tide. enumeration; Higher high water large tide. enumeration; Nearly highest high water. enumeration; Highest astronomical tide (HAT). |

element Pile/verticalLength

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. The total vertical length of an object.. |

element Pile/geometry

| diagram |  |
| --- | --- |
| type | PointPropertyType |
| children | pointProperty |

complexType QualityOfNonBathymetricData

| diagram |  |
| --- | --- |
| type | extension of MetaFeatureType |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation categoryOfTemporalVariation directionUncertainty horizontalDistanceUncertainty horizontalPositionUncertainty information informationInNationalLanguage textualDescription textualDescriptionInNationalLanguage verticalUncertainty surveyDateRange geometry |
| used by | element; QualityOfNonBathymetricData. |
| annotation | documentation. An area within which a uniform assessment of the quality of the non-bathymetric data exists. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 1, Page 1.208, November 2000).. |

element QualityOfNonBathymetricData/categoryOfTemporalVariation

| diagram |  |
| --- | --- |
| type | categoryOfTemporalVariation |
| facets | Kind; Value; Annotation. enumeration; unassessed; documentation. Temporal variation not assessed or cannot be determined.. enumeration; event; documentation. No new hydrographic survey conducted after an event (e.g. hurricane, earthquake, volcanic eruption, landslide, etc), which is considered likely to have changed the seafloor significantly... enumeration; likely to change; documentation. Continuous or frequent change (e.g. river siltation, sand waves, seasonal storms, ice bergs, etc)... enumeration; likely to change, but significant shoaling unlikely; documentation. Likely to change but significant shoaling unlikely.. enumeration; unlikely to change; documentation. Significant change to the seafloor is not expected... |

element QualityOfNonBathymetricData/directionUncertainty

| diagram |  |
| --- | --- |
| type | Decimal0.0To360.0 |
| facets | Kind; Value; Annotation. minInclusive; 0.0. maxInclusive; 360.0. fractionDigits; 1. |
| annotation | documentation. The best estimate of the accuracy of a bearing.. |

element QualityOfNonBathymetricData/horizontalDistanceUncertainty

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. The best estimate of the horizontal accuracy of horizontal clearance and distances. The expected input is the radius of the two-dimensional error. The error is assumed to be positive and negative. The plus/minus character shall not be encoded.. |

element QualityOfNonBathymetricData/horizontalPositionUncertainty

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. The best estimate of the accuracy of a position. The expected input is the maximum of the two-dimensional error. The error is assumed to be positive and negative. The plus/minus character shall not be encoded.. |

element QualityOfNonBathymetricData/information

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. Textual information about the object.. documentation. (max 300 characters). |

element QualityOfNonBathymetricData/informationInNationalLanguage

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. Textual information in national language characters.. documentation. (max 300 characters). |

element QualityOfNonBathymetricData/textualDescription

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. file reference to text file. |

element QualityOfNonBathymetricData/textualDescriptionInNationalLanguage

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. The file name of an external text file that contains the text in a national language.. |

element QualityOfNonBathymetricData/verticalUncertainty

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. The best estimate of the vertical uncertainty of heights, vertical distances and vertical clearances, excluding sounding measurements. The one-dimensional error. The error is assumed to be positive and negative. The plus/minus character shall not be encoded.. |

element QualityOfNonBathymetricData/surveyDateRange

| diagram |  |
| --- | --- |
| type | surveyDateRange |
| children | dateEnd dateStart |

element QualityOfNonBathymetricData/geometry

| diagram |  |
| --- | --- |
| type | SurfacePropertyType |
| children | surfaceProperty |

complexType RadarReflector

| diagram |  |
| --- | --- |
| type | extension of Equipment |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_remoteMonitoringSystem IALA_remotelyMonitored parent geometry height status verticalAccuracy verticalDatum |
| used by | element; RadarReflector. |
| annotation | documentation. A device capable of, or intended for, reflecting radar signals. (IHO Dictionary, S-32, 5th Edition, 4147) A radar reflector is usually a >tetrahedron or pentagonal corner reflector (...) to facilitate reflection towards the sender. (International Maritime Dictionary, 2nd Ed.). |

element RadarReflector/height

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. The value of the vertical distance to the highest point of the object, measured from a specified vertical datum.. |

element RadarReflector/status

| diagram |  |
| --- | --- |
| type | status |
| facets | Kind; Value; Annotation. enumeration; permanent. enumeration; occasional. enumeration; recommended. enumeration; not in use. enumeration; periodic/intermittent. enumeration; reserved. enumeration; temporary. enumeration; private. enumeration; mandatory. enumeration; extinguished. enumeration; illuminated. enumeration; historic. enumeration; public. enumeration; synchronized. enumeration; watched. enumeration; un-watched. enumeration; existence doubtful. enumeration; IALA_confirmed; documentation. made certain as to truth, accuracy, validity, availability, etc. - www.dictionary.com.. enumeration; IALA_candidate; documentation. Item selected for an action.. enumeration; IALA_under modification; documentation. Item that is in the process of being modified.. enumeration; IALA_candidate for modification; documentation. Item selected for modification.. enumeration; IALA_under removal/deletion; documentation. Item in the process of being removed or deleted.. enumeration; IALA_removed/deleted; documentation. Item that has been removed or deleted.. |

element RadarReflector/verticalAccuracy

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal.1 |
| facets | Kind; Value; Annotation. minInclusive; 0.0. fractionDigits; 1. |
| annotation | documentation. The best estimate of the vertical accuracy of heights, vertical distances and vertical clearances, excluding sounding measurements. The one-dimensional error. The error is assumed to be positive and negative. The plus/minus character shall not be encoded.. |

element RadarReflector/verticalDatum

| diagram |  |
| --- | --- |
| type | verticalDatum |
| facets | Kind; Value; Annotation. enumeration; Mean low water springs. enumeration; Mean lower low water springs. enumeration; Mean sea level. enumeration; Lowest low water. enumeration; Mean low water. enumeration; Lowest low water springs. enumeration; Approximate mean low water springs. enumeration; Indian spring low water. enumeration; Low water springs. enumeration; Approximate lowest astronomical tide. enumeration; Nearly lowest low water. enumeration; Mean lower low water. enumeration; Low water. enumeration; Approximate mean low water. enumeration; Approximate mean lower low water. enumeration; Mean high water. enumeration; Mean high water springs. enumeration; High water. enumeration; Approximate mean sea level. enumeration; High water springs. enumeration; Mean higher high water. enumeration; Equinoctial spring low water. enumeration; Lowest astronomical tide. enumeration; Local datum. enumeration; International Great Lakes Datum 1985. enumeration; Mean water level. enumeration; Lower low water large tide. enumeration; Higher high water large tide. enumeration; Nearly highest high water. enumeration; Highest astronomical tide (HAT). |

complexType RadarTransponderBeacon

| diagram |  |
| --- | --- |
| type | extension of Equipment |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_remoteMonitoringSystem IALA_remotelyMonitored parent geometry categoryOfRadarTransponderBeacon objectNameInNationalLanguage objectName radarWaveLength sectorLimitOne sectorLimitTwo signalGroup signalSequence status valueOfNominalRange IALA_typeOfBattery |
| used by | element; RadarTransponderBeacon. |
| annotation | documentation. A transponder beacon transmitting a coded signal on radar frequency, permitting an interrogating craft to determine the bearing and range of the transponder. Also called racon. (IHO Dictionary, S-32, 5th Edition, 4137). |

element RadarTransponderBeacon/categoryOfRadarTransponderBeacon

| diagram |  |
| --- | --- |
| type | categoryOfRadarTransponderBeacon |
| facets | Kind; Value; Annotation. enumeration; ramark, radar beacon transmitting continuously; documentation. a radar marker beacon which continuously transmits a signal appearing as a radial line on a radar screen, the line indicating the direction of the beacon. Ramarks are intended primarily for marine use. The name 'ramark' is derived from the words radar marker... enumeration; racon, radar transponder beacon; documentation. a radar beacon which returns a coded signal which provides identification of the beacon, as well as range and bearing. The range and bearing are indicated by the location of the first character received on the radar screen. The name 'racon' is derived from the words radar beacon... enumeration; leading racon/radar transponder beacon; documentation. a radar beacon that may be used (in conjunction with at least one other radar beacon) to indicate a leading line... |

element RadarTransponderBeacon/objectNameInNationalLanguage

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. Name of object in national language characters.. |

element RadarTransponderBeacon/objectName

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. The individual name of an object.. |

element RadarTransponderBeacon/radarWaveLength

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. The distance between two successive peaks (or other points of identical phase) on an electromagnetic wave in the radar band of the electromagnetic spectrum.. |

element RadarTransponderBeacon/sectorLimitOne

| diagram |  |
| --- | --- |
| type | Decimal0.0To360.0 |
| facets | Kind; Value; Annotation. minInclusive; 0.0. maxInclusive; 360.0. fractionDigits; 1. |
| annotation | documentation. The sector limit 1 specifies the first limit of the sector. The order of sector limit 1 and sector limit 2 is clockwise around the central object (e.g., a light).. |

element RadarTransponderBeacon/sectorLimitTwo

| diagram |  |
| --- | --- |
| type | Decimal0.0To360.0 |
| facets | Kind; Value; Annotation. minInclusive; 0.0. maxInclusive; 360.0. fractionDigits; 1. |
| annotation | documentation. The sector limit 2 specifies the second limit of the sector. The order of sector limit 1 and sector limit 2 is clockwise around the central object (e.g., a light).. |

element RadarTransponderBeacon/signalGroup

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. The number of signals, the combination of signals or the morse character(s) within one period of full sequence.. |

element RadarTransponderBeacon/signalSequence

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. The sequence of times occupied by intervals of light and eclipse for all 'light characteristics' except for occulting where the sequence of times is occupied by intervals of eclipse and light.. |

element RadarTransponderBeacon/status

| diagram |  |
| --- | --- |
| type | status |
| facets | Kind; Value; Annotation. enumeration; permanent. enumeration; occasional. enumeration; recommended. enumeration; not in use. enumeration; periodic/intermittent. enumeration; reserved. enumeration; temporary. enumeration; private. enumeration; mandatory. enumeration; extinguished. enumeration; illuminated. enumeration; historic. enumeration; public. enumeration; synchronized. enumeration; watched. enumeration; un-watched. enumeration; existence doubtful. enumeration; IALA_confirmed; documentation. made certain as to truth, accuracy, validity, availability, etc. - www.dictionary.com.. enumeration; IALA_candidate; documentation. Item selected for an action.. enumeration; IALA_under modification; documentation. Item that is in the process of being modified.. enumeration; IALA_candidate for modification; documentation. Item selected for modification.. enumeration; IALA_under removal/deletion; documentation. Item in the process of being removed or deleted.. enumeration; IALA_removed/deleted; documentation. Item that has been removed or deleted.. |

element RadarTransponderBeacon/valueOfNominalRange

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. The nominal range at which an object can be seen or a signal detected.. |

element RadarTransponderBeacon/IALA_typeOfBattery

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. Definition required. |

complexType RecommendedTrack

| diagram |  |
| --- | --- |
| type | extension of AidsToNavigation |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer categoryOfRecommendedTrack depthRangeMinimumValue depthRangeMaximumValue objectNameInNationalLanguage objectName orientation qualityOfSoundingMeasurement soundingAccuracy status techniqueOfSoundingMeasurement trafficFlow verticalDatum navigationLine geometry |
| used by | element; RecommendedTrack. |
| annotation | documentation. A track recommended to all or only certain vessels. (IHO Dictionary, S-32, 5th Edition, 5576). |

element RecommendedTrack/categoryOfRecommendedTrack

| diagram |  |
| --- | --- |
| type | categoryOfRecommendedTrack |
| facets | Kind; Value; Annotation. enumeration; based on a system of fixed marks; documentation. a straight route (known as a recommended track, range or leading line), which comprises at least two structures (usually beacons or daymarks) and/or natural features, which may carry lights and/or top-marks. The structures/features are positioned so that when observed to be in line, a vessel can follow a known bearing with safety... enumeration; not based on a system of fixed marks; documentation. a route (known as a recommended track or preferred route) which is not based on a series of structures or features in line... |

element RecommendedTrack/depthRangeMinimumValue

| diagram |  |
| --- | --- |
| type | xs:decimal |
| annotation | documentation. The minimum (shoalest) value of a depth range. Where the area dries, the value is negative.. |

element RecommendedTrack/depthRangeMaximumValue

| diagram |  |
| --- | --- |
| type | xs:decimal |
| annotation | documentation. The maximum (deepest) value of a depth range. Where the area dries, the value is negative.. |

element RecommendedTrack/objectNameInNationalLanguage

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. Name of object in national language characters.. |

element RecommendedTrack/objectName

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. The individual name of an object.. |

element RecommendedTrack/orientation

| diagram |  |
| --- | --- |
| type | Decimal0.0To360.0 |
| facets | Kind; Value; Annotation. minInclusive; 0.0. maxInclusive; 360.0. fractionDigits; 1. |
| annotation | documentation. The angular distance measured from true north to the major axis of the object.. |

element RecommendedTrack/qualityOfSoundingMeasurement

| diagram |  |
| --- | --- |
| type | qualityOfSoundingMeasurement |
| facets | Kind; Value; Annotation. enumeration; depth known. enumeration; depth unknown. enumeration; doubtful sounding. enumeration; unreliable sounding. enumeration; no bottom found at value shown. enumeration; least depth known. enumeration; least depth unknown, safe clearance at value shown. enumeration; value reported (not surveyed). enumeration; value reported (not confirmed). enumeration; maintained depth. enumeration; not regularly maintained. |

element RecommendedTrack/soundingAccuracy

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. The best estimate of the accuracy of the sounding data. The maximum of the one-dimensional error. The error is assumed to be positive and negative. The plus/minus character shall not be encoded.. |

element RecommendedTrack/status

| diagram |  |
| --- | --- |
| type | status |
| facets | Kind; Value; Annotation. enumeration; permanent. enumeration; occasional. enumeration; recommended. enumeration; not in use. enumeration; periodic/intermittent. enumeration; reserved. enumeration; temporary. enumeration; private. enumeration; mandatory. enumeration; extinguished. enumeration; illuminated. enumeration; historic. enumeration; public. enumeration; synchronized. enumeration; watched. enumeration; un-watched. enumeration; existence doubtful. enumeration; IALA_confirmed; documentation. made certain as to truth, accuracy, validity, availability, etc. - www.dictionary.com.. enumeration; IALA_candidate; documentation. Item selected for an action.. enumeration; IALA_under modification; documentation. Item that is in the process of being modified.. enumeration; IALA_candidate for modification; documentation. Item selected for modification.. enumeration; IALA_under removal/deletion; documentation. Item in the process of being removed or deleted.. enumeration; IALA_removed/deleted; documentation. Item that has been removed or deleted.. |

element RecommendedTrack/techniqueOfSoundingMeasurement

| diagram |  |
| --- | --- |
| type | techniqueOfSoundingMeasurement |
| facets | Kind; Value; Annotation. enumeration; found by echo-sounder. enumeration; found by side scan sonar. enumeration; found by multi-beam. enumeration; found by diver. enumeration; found by lead-line. enumeration; swept by wire-drag. enumeration; found by laser. enumeration; swept by vertical acoustic system. enumeration; found by electromagnetic sensor. enumeration; photogrammetry. enumeration; satellite imagery. enumeration; found by levelling. enumeration; swept by side-scan sonar. enumeration; computer generated. |

element RecommendedTrack/trafficFlow

| diagram |  |
| --- | --- |
| type | trafficFlow |
| facets | Kind; Value; Annotation. enumeration; inbound. enumeration; outbound. enumeration; one-way. enumeration; two-way. |

element RecommendedTrack/verticalDatum

| diagram |  |
| --- | --- |
| type | verticalDatum |
| facets | Kind; Value; Annotation. enumeration; Mean low water springs. enumeration; Mean lower low water springs. enumeration; Mean sea level. enumeration; Lowest low water. enumeration; Mean low water. enumeration; Lowest low water springs. enumeration; Approximate mean low water springs. enumeration; Indian spring low water. enumeration; Low water springs. enumeration; Approximate lowest astronomical tide. enumeration; Nearly lowest low water. enumeration; Mean lower low water. enumeration; Low water. enumeration; Approximate mean low water. enumeration; Approximate mean lower low water. enumeration; Mean high water. enumeration; Mean high water springs. enumeration; High water. enumeration; Approximate mean sea level. enumeration; High water springs. enumeration; Mean higher high water. enumeration; Equinoctial spring low water. enumeration; Lowest astronomical tide. enumeration; Local datum. enumeration; International Great Lakes Datum 1985. enumeration; Mean water level. enumeration; Lower low water large tide. enumeration; Higher high water large tide. enumeration; Nearly highest high water. enumeration; Highest astronomical tide (HAT). |

element RecommendedTrack/navigationLine

| diagram |  |
| --- | --- |
| type | gml:ReferenceType |
| annotation | documentation. The role given to the navigation line(s) that is generally formed between two or more objects, or by one object and a bearing.. |

element RecommendedTrack/geometry

| diagram |  |
| --- | --- |
| type | CurvePropertyType |
| children | curveProperty |

complexType RetroReflector

| diagram |  |
| --- | --- |
| type | extension of Equipment |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_remoteMonitoringSystem IALA_remotelyMonitored parent geometry colour colourPattern height marksNavigationalSystemOf status verticalAccuracy verticalDatum |
| used by | element; RetroReflector. |
| annotation | documentation. A means of distinguishing unlighted marks at night. Retro-reflective material is secured to the mark in a particular pattern to reflect back light. (Adapted from the UKHO NP735, 5th Edition).. |

element RetroReflector/colour

| diagram |  |
| --- | --- |
| type | colour |
| facets | Kind; Value; Annotation. enumeration; white. enumeration; black. enumeration; red. enumeration; green. enumeration; blue. enumeration; yellow. enumeration; grey. enumeration; brown. enumeration; amber. enumeration; violet. enumeration; orange. enumeration; magenta. enumeration; pink. |

element RetroReflector/colourPattern

| diagram |  |
| --- | --- |
| type | colourPattern |
| facets | Kind; Value; Annotation. enumeration; horizontal stripes; documentation. Straight bands or stripes of differing colours oriented horizontally. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; vertical stripes; documentation. Straight bands or stripes of differing colours oriented vertically. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; diagonal stripes; documentation. Straight bands or stripes of differing colours oriented diagonally (i.e. not horizontally or vertically). (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; squared; documentation. Often referred to as checker plate, where alternate colours are used to create squares similar to a chess or draught board. The pattern may be straight or diagonal. (S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; stripes (direction unknown); documentation. Straight bands or stripes of differing colours oriented in an unknown direction. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; border stripe; documentation. A band or stripe of colour which is displayed around the outer edge of the feature, which may also form a border to an inner pattern or plain colour. (S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; IALA_single colour; documentation. One solid colour of uniform coverage.. enumeration; IALA_rectangle; documentation. A four-sided shape that is made up of two pairs of parallel lines and that has four right angles, on a different coloured background.. enumeration; IALA_triangle; documentation. a shape that is made up of three lines and three angles, on a different coloured background.. |

element RetroReflector/height

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. The value of the vertical distance to the highest point of the object, measured from a specified vertical datum.. |

element RetroReflector/marksNavigationalSystemOf

| diagram |  |
| --- | --- |
| type | marksNavigationalSystemOf |
| facets | Kind; Value; Annotation. enumeration; IALA A. enumeration; IALA B. enumeration; no system. enumeration; other system. |

element RetroReflector/status

| diagram |  |
| --- | --- |
| type | status |
| facets | Kind; Value; Annotation. enumeration; permanent. enumeration; occasional. enumeration; recommended. enumeration; not in use. enumeration; periodic/intermittent. enumeration; reserved. enumeration; temporary. enumeration; private. enumeration; mandatory. enumeration; extinguished. enumeration; illuminated. enumeration; historic. enumeration; public. enumeration; synchronized. enumeration; watched. enumeration; un-watched. enumeration; existence doubtful. enumeration; IALA_confirmed; documentation. made certain as to truth, accuracy, validity, availability, etc. - www.dictionary.com.. enumeration; IALA_candidate; documentation. Item selected for an action.. enumeration; IALA_under modification; documentation. Item that is in the process of being modified.. enumeration; IALA_candidate for modification; documentation. Item selected for modification.. enumeration; IALA_under removal/deletion; documentation. Item in the process of being removed or deleted.. enumeration; IALA_removed/deleted; documentation. Item that has been removed or deleted.. |

element RetroReflector/verticalAccuracy

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal.1 |
| facets | Kind; Value; Annotation. minInclusive; 0.0. fractionDigits; 1. |
| annotation | documentation. The best estimate of the vertical accuracy of heights, vertical distances and vertical clearances, excluding sounding measurements. The one-dimensional error. The error is assumed to be positive and negative. The plus/minus character shall not be encoded.. |

element RetroReflector/verticalDatum

| diagram |  |
| --- | --- |
| type | verticalDatum |
| facets | Kind; Value; Annotation. enumeration; Mean low water springs. enumeration; Mean lower low water springs. enumeration; Mean sea level. enumeration; Lowest low water. enumeration; Mean low water. enumeration; Lowest low water springs. enumeration; Approximate mean low water springs. enumeration; Indian spring low water. enumeration; Low water springs. enumeration; Approximate lowest astronomical tide. enumeration; Nearly lowest low water. enumeration; Mean lower low water. enumeration; Low water. enumeration; Approximate mean low water. enumeration; Approximate mean lower low water. enumeration; Mean high water. enumeration; Mean high water springs. enumeration; High water. enumeration; Approximate mean sea level. enumeration; High water springs. enumeration; Mean higher high water. enumeration; Equinoctial spring low water. enumeration; Lowest astronomical tide. enumeration; Local datum. enumeration; International Great Lakes Datum 1985. enumeration; Mean water level. enumeration; Lower low water large tide. enumeration; Higher high water large tide. enumeration; Nearly highest high water. enumeration; Highest astronomical tide (HAT). |

complexType S100_TruncatedDate

| diagram |  |
| --- | --- |
| children | gDay gMonth gYear gMonthDay gYearMonth date |
| used by | elements; surveyDateRange/dateEnd AidsToNavigation/dateEnd surveyDateRange/dateStart AidsToNavigation/dateStart AidsToNavigation/IALA_installationDate AidsToNavigation/periodEnd AidsToNavigation/periodStart. |
| annotation | documentation. built in date types from W3C XML schema, implementing S-100 truncated date. |

element S100_TruncatedDate/gDay

| diagram |  |
| --- | --- |
| type | xs:gDay |

element S100_TruncatedDate/gMonth

| diagram |  |
| --- | --- |
| type | xs:gMonth |

element S100_TruncatedDate/gYear

| diagram |  |
| --- | --- |
| type | xs:gYear |

element S100_TruncatedDate/gMonthDay

| diagram |  |
| --- | --- |
| type | xs:gMonthDay |

element S100_TruncatedDate/gYearMonth

| diagram |  |
| --- | --- |
| type | xs:gYearMonth |

element S100_TruncatedDate/date

| diagram |  |
| --- | --- |
| type | xs:date |

complexType SiloTank

| diagram |  |
| --- | --- |
| type | extension of StructureObject |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child buildingShape categoryOfSiloTank colour colourPattern condition radarConspicuous visuallyConspicuous elevation height natureOfConstruction objectNameInNationalLanguage objectName product status verticalAccuracy verticalDatum verticalLength geometry |
| used by | element; SiloTank. |
| annotation | documentation. An enclosed container, used for storage (Digital Geographic Information Working Group, Oct.87). |

element SiloTank/buildingShape

| diagram |  |
| --- | --- |
| type | buildingShape |
| facets | Kind; Value; Annotation. enumeration; high-rise building; documentation. a building having many storeys... enumeration; pyramid; documentation. a polyhedron of which one face is a polygon of any number of sides, and the other faces are triangles with a common vertex... enumeration; cylindrical; documentation. shaped like a cylinder, which is a solid geometrical figure generated by straight lines fixed in direction and describing with one of its points a closed curve, especially a circle... enumeration; spherical; documentation. shaped like a sphere, which is a body the surface of which is at all points equidistant from the centre... enumeration; cubic; documentation. a shape the sides of which are six equal squares a regular hexahedron... |

element SiloTank/categoryOfSiloTank

| diagram |  |
| --- | --- |
| type | categoryOfSiloTank |
| facets | Kind; Value; Annotation. enumeration; silo in general; documentation. a generally cylindrical tower used for storing fodder or grain... enumeration; tank in general; documentation. a fixed structure for storing liquids... enumeration; grain elevator; documentation. a storage building for grain. Usually a tall frame, metal or concrete structure with an especially compartmented interior... enumeration; water tower; documentation. a tower with an elevated container used to hold water... |

element SiloTank/colour

| diagram |  |
| --- | --- |
| type | colour |
| facets | Kind; Value; Annotation. enumeration; white. enumeration; black. enumeration; red. enumeration; green. enumeration; blue. enumeration; yellow. enumeration; grey. enumeration; brown. enumeration; amber. enumeration; violet. enumeration; orange. enumeration; magenta. enumeration; pink. |

element SiloTank/colourPattern

| diagram |  |
| --- | --- |
| type | colourPattern |
| facets | Kind; Value; Annotation. enumeration; horizontal stripes; documentation. Straight bands or stripes of differing colours oriented horizontally. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; vertical stripes; documentation. Straight bands or stripes of differing colours oriented vertically. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; diagonal stripes; documentation. Straight bands or stripes of differing colours oriented diagonally (i.e. not horizontally or vertically). (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; squared; documentation. Often referred to as checker plate, where alternate colours are used to create squares similar to a chess or draught board. The pattern may be straight or diagonal. (S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; stripes (direction unknown); documentation. Straight bands or stripes of differing colours oriented in an unknown direction. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; border stripe; documentation. A band or stripe of colour which is displayed around the outer edge of the feature, which may also form a border to an inner pattern or plain colour. (S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; IALA_single colour; documentation. One solid colour of uniform coverage.. enumeration; IALA_rectangle; documentation. A four-sided shape that is made up of two pairs of parallel lines and that has four right angles, on a different coloured background.. enumeration; IALA_triangle; documentation. a shape that is made up of three lines and three angles, on a different coloured background.. |

element SiloTank/condition

| diagram |  |
| --- | --- |
| type | condition |
| facets | Kind; Value; Annotation. enumeration; under construction. enumeration; ruined. enumeration; under reclamation. enumeration; wingless. enumeration; planned construction. |

element SiloTank/radarConspicuous

| diagram |  |
| --- | --- |
| type | radarConspicuous |
| facets | Kind; Value; Annotation. enumeration; radar conspicuous. enumeration; not radar conspicuous. enumeration; radar conspicuous (has radar reflector). |

element SiloTank/visuallyConspicuous

| diagram |  |
| --- | --- |
| type | visuallyConspicuous |
| facets | Kind; Value; Annotation. enumeration; visually conspicuous. enumeration; not visually conspicuous. |

element SiloTank/elevation

| diagram |  |
| --- | --- |
| type | xs:decimal |
| annotation | documentation. The altitude of the ground level of an object, measured from a specified vertical datum.. |

element SiloTank/height

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. The value of the vertical distance to the highest point of the object, measured from a specified vertical datum.. |

element SiloTank/natureOfConstruction

| diagram |  |
| --- | --- |
| type | natureOfConstruction |
| facets | Kind; Value; Annotation. enumeration; masonry. enumeration; concreted. enumeration; loose boulders. enumeration; hard surfaced. enumeration; unsurfaced. enumeration; wooden. enumeration; metal. enumeration; glass reinforced plastic (GRP). enumeration; painted. enumeration; IALA_fiberglass; documentation. Constructed from fiberglass.. enumeration; IALA_plastic; documentation. Constructed from plastic.. |

element SiloTank/objectNameInNationalLanguage

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. Name of object in national language characters.. |

element SiloTank/objectName

| diagram |  |
| --- | --- |
| type | xs:string |
| annotation | documentation. The individual name of an object.. |

element SiloTank/product

| diagram |  |
| --- | --- |
| type | product |
| facets | Kind; Value; Annotation. enumeration; oil. enumeration; gas. enumeration; water. enumeration; stone. enumeration; coal. enumeration; ore. enumeration; chemicals. enumeration; drinking water. enumeration; milk. enumeration; bauxite. enumeration; coke. enumeration; iron ingots. enumeration; salt. enumeration; sand. enumeration; timber. enumeration; sawdust/wood chips. enumeration; scrap metal. enumeration; liquified natural gas (LNG). enumeration; liquified petroleum gas (LPG). enumeration; wine. enumeration; cement. enumeration; grain. |

element SiloTank/status

| diagram |  |
| --- | --- |
| type | status |
| facets | Kind; Value; Annotation. enumeration; permanent. enumeration; occasional. enumeration; recommended. enumeration; not in use. enumeration; periodic/intermittent. enumeration; reserved. enumeration; temporary. enumeration; private. enumeration; mandatory. enumeration; extinguished. enumeration; illuminated. enumeration; historic. enumeration; public. enumeration; synchronized. enumeration; watched. enumeration; un-watched. enumeration; existence doubtful. enumeration; IALA_confirmed; documentation. made certain as to truth, accuracy, validity, availability, etc. - www.dictionary.com.. enumeration; IALA_candidate; documentation. Item selected for an action.. enumeration; IALA_under modification; documentation. Item that is in the process of being modified.. enumeration; IALA_candidate for modification; documentation. Item selected for modification.. enumeration; IALA_under removal/deletion; documentation. Item in the process of being removed or deleted.. enumeration; IALA_removed/deleted; documentation. Item that has been removed or deleted.. |

element SiloTank/verticalAccuracy

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal.1 |
| facets | Kind; Value; Annotation. minInclusive; 0.0. fractionDigits; 1. |
| annotation | documentation. The best estimate of the vertical accuracy of heights, vertical distances and vertical clearances, excluding sounding measurements. The one-dimensional error. The error is assumed to be positive and negative. The plus/minus character shall not be encoded.. |

element SiloTank/verticalDatum

| diagram |  |
| --- | --- |
| type | verticalDatum |
| facets | Kind; Value; Annotation. enumeration; Mean low water springs. enumeration; Mean lower low water springs. enumeration; Mean sea level. enumeration; Lowest low water. enumeration; Mean low water. enumeration; Lowest low water springs. enumeration; Approximate mean low water springs. enumeration; Indian spring low water. enumeration; Low water springs. enumeration; Approximate lowest astronomical tide. enumeration; Nearly lowest low water. enumeration; Mean lower low water. enumeration; Low water. enumeration; Approximate mean low water. enumeration; Approximate mean lower low water. enumeration; Mean high water. enumeration; Mean high water springs. enumeration; High water. enumeration; Approximate mean sea level. enumeration; High water springs. enumeration; Mean higher high water. enumeration; Equinoctial spring low water. enumeration; Lowest astronomical tide. enumeration; Local datum. enumeration; International Great Lakes Datum 1985. enumeration; Mean water level. enumeration; Lower low water large tide. enumeration; Higher high water large tide. enumeration; Nearly highest high water. enumeration; Highest astronomical tide (HAT). |

element SiloTank/verticalLength

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. The total vertical length of an object.. |

element SiloTank/geometry

| diagram |  |
| --- | --- |
| type | PointOrSurfacePropertyType |
| children | pointProperty surfaceProperty |

complexType SoundingDatum

| diagram |  |
| --- | --- |
| type | extension of MetaFeatureType |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation verticalDatum geometry |
| used by | element; SoundingDatum. |
| annotation | documentation. The horizontal plane or tidal datum to which soundings have been reduced. Also called datum for sounding reduction.. |

element SoundingDatum/verticalDatum

| diagram |  |
| --- | --- |
| type | verticalDatum |
| facets | Kind; Value; Annotation. enumeration; Mean low water springs. enumeration; Mean lower low water springs. enumeration; Mean sea level. enumeration; Lowest low water. enumeration; Mean low water. enumeration; Lowest low water springs. enumeration; Approximate mean low water springs. enumeration; Indian spring low water. enumeration; Low water springs. enumeration; Approximate lowest astronomical tide. enumeration; Nearly lowest low water. enumeration; Mean lower low water. enumeration; Low water. enumeration; Approximate mean low water. enumeration; Approximate mean lower low water. enumeration; Mean high water. enumeration; Mean high water springs. enumeration; High water. enumeration; Approximate mean sea level. enumeration; High water springs. enumeration; Mean higher high water. enumeration; Equinoctial spring low water. enumeration; Lowest astronomical tide. enumeration; Local datum. enumeration; International Great Lakes Datum 1985. enumeration; Mean water level. enumeration; Lower low water large tide. enumeration; Higher high water large tide. enumeration; Nearly highest high water. enumeration; Highest astronomical tide (HAT). |

element SoundingDatum/geometry

| diagram |  |
| --- | --- |
| type | SurfacePropertyType |
| children | surfaceProperty |
| annotation | documentation. Permitted spatial primitives. |

complexType SpatialUncertainty

| diagram |  |
| --- | --- |
| type | extension of InformationTypeType |
| children | informationAssociation invInformationAssociation qualityOfPosition positionalAccuracy |
| used by | element; SpatialUncertainty. |
| annotation | documentation. Information object for uncertainty of individual features. |

element SpatialUncertainty/qualityOfPosition

| diagram |  |
| --- | --- |
| type | qualityOfPosition |
| facets | Kind; Value; Annotation. enumeration; surveyed. enumeration; unsurveyed. enumeration; inadequately surveyed. enumeration; approximate. enumeration; position doubtful. enumeration; unreliable. enumeration; reported (not surveyed). enumeration; reported (not confirmed). enumeration; estimated. enumeration; precisely known. enumeration; calculated. |

element SpatialUncertainty/positionalAccuracy

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. The best estimate of the accuracy of a position. The expected input is the maximum of the two-dimensional error. The error is assumed to be positive and negative. The plus/minus character shall not be encoded.. |

complexType StructureObject

| diagram |  |
| --- | --- |
| type | extension of AidsToNavigation |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_aidAvailabilityCategory contactAddress child |
| used by | element; StructureObject. complexTypes; GenericBeacon GenericBuoy Landmark LightFloat LightVessel OffshorePlatform Pile SiloTank. |
| annotation | documentation. Something (such as a house, tower, bridge, etc.) that is built by putting parts together and that usually stands on its own. |

element StructureObject/IALA_aidAvailabilityCategory

| diagram |  |
| --- | --- |
| type | IALA_aidAvailabilityCategory |
| facets | Kind; Value; Annotation. enumeration; Category 1; documentation. An AtoN or system of AtoN that is considered by the Competent Authority to be of vital navigational significance. Recommendation O-130 – Categorisation and Availability Objectives for Short Range Aids to Navigation December 2004 - Revised June 2011.. enumeration; Category 2; documentation. An AtoN or system of AtoN that is considered by the Competent Authority to be of important navigational significance. Recommendation O-130 – Categorisation and Availability Objectives for Short Range Aids to Navigation December 2004 - Revised June 2011.. enumeration; Category 3; documentation. An AtoN or system of AtoN that is considered by the Competent Authority to be of necessary navigational significance. Recommendation O-130 – Categorisation and Availability Objectives for Short Range Aids to Navigation December 2004 - Revised June 2011.. |
| annotation | documentation. A Category denoting the significance of an Aid to Navigation, expressed in terms of the probability that an AtoN or system of AtoN, as defined by the Competent Authority, is performing its specified function at any randomly chosen time. This is expressed as a percentage of total time that an AtoN or system of AtoN should be performing their specified function. adapted from the IALA Guideline No. 1035 To Availability and Reliability of Aids to Navigation - Theory and Examples.. |

element StructureObject/contactAddress

| diagram |  |
| --- | --- |
| type | contactAddress |
| children | deliveryPoint cityName administrativeDivision country postalCode |

element StructureObject/child

| diagram |  |
| --- | --- |
| type | gml:ReferenceType |
| annotation | documentation. The role given to the subordinate object(s) in a relationship. |

complexType surveyDateRange

| diagram |  |
| --- | --- |
| children | dateEnd dateStart |
| used by | element; QualityOfNonBathymetricData/surveyDateRange. |
| annotation | documentation. The complex attribute describes the period of the hydrographic survey, as the time between its sub-attributes. |

element surveyDateRange/dateEnd

| diagram |  |
| --- | --- |
| type | S100_TruncatedDate |
| children | gDay gMonth gYear gMonthDay gYearMonth date |
| annotation | documentation. The latest date on which an object (e.g., a buoy) will be present.. |

element surveyDateRange/dateStart

| diagram |  |
| --- | --- |
| type | S100_TruncatedDate |
| children | gDay gMonth gYear gMonthDay gYearMonth date |
| annotation | documentation. The earliest date on which an object (e.g., a buoy) will be present.. |

complexType Topmark

| diagram |  |
| --- | --- |
| type | extension of Equipment |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation idCode dateEnd dateStart periodEnd periodStart information informationInNationalLanguage textualDescriptionInNationalLanguage textualDescription scaleMinimum sourceDate sourceIndication pictorialRepresentation IALA_inspectionFrequency IALA_inspectionRequirements IALA_AtoNMaintenanceRecord IALA_installationDate peer IALA_remoteMonitoringSystem IALA_remotelyMonitored parent geometry colour colourPattern height marksNavigationalSystemOf status topmarkDaymarkShape verticalAccuracy verticalDatum verticalLength |
| used by | element; Topmark. |
| annotation | documentation. A characteristic shape secured at the top of a buoy or beacon to aid in its identification. (IHO Dictionary, S-32, 5th Edition, 5548). |

element Topmark/colour

| diagram |  |
| --- | --- |
| type | colour |
| facets | Kind; Value; Annotation. enumeration; white. enumeration; black. enumeration; red. enumeration; green. enumeration; blue. enumeration; yellow. enumeration; grey. enumeration; brown. enumeration; amber. enumeration; violet. enumeration; orange. enumeration; magenta. enumeration; pink. |

element Topmark/colourPattern

| diagram |  |
| --- | --- |
| type | colourPattern |
| facets | Kind; Value; Annotation. enumeration; horizontal stripes; documentation. Straight bands or stripes of differing colours oriented horizontally. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; vertical stripes; documentation. Straight bands or stripes of differing colours oriented vertically. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; diagonal stripes; documentation. Straight bands or stripes of differing colours oriented diagonally (i.e. not horizontally or vertically). (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; squared; documentation. Often referred to as checker plate, where alternate colours are used to create squares similar to a chess or draught board. The pattern may be straight or diagonal. (S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; stripes (direction unknown); documentation. Straight bands or stripes of differing colours oriented in an unknown direction. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; border stripe; documentation. A band or stripe of colour which is displayed around the outer edge of the feature, which may also form a border to an inner pattern or plain colour. (S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; IALA_single colour; documentation. One solid colour of uniform coverage.. enumeration; IALA_rectangle; documentation. A four-sided shape that is made up of two pairs of parallel lines and that has four right angles, on a different coloured background.. enumeration; IALA_triangle; documentation. a shape that is made up of three lines and three angles, on a different coloured background.. |

element Topmark/height

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. The value of the vertical distance to the highest point of the object, measured from a specified vertical datum.. |

element Topmark/marksNavigationalSystemOf

| diagram |  |
| --- | --- |
| type | marksNavigationalSystemOf |
| facets | Kind; Value; Annotation. enumeration; IALA A. enumeration; IALA B. enumeration; no system. enumeration; other system. |

element Topmark/status

| diagram |  |
| --- | --- |
| type | status |
| facets | Kind; Value; Annotation. enumeration; permanent. enumeration; occasional. enumeration; recommended. enumeration; not in use. enumeration; periodic/intermittent. enumeration; reserved. enumeration; temporary. enumeration; private. enumeration; mandatory. enumeration; extinguished. enumeration; illuminated. enumeration; historic. enumeration; public. enumeration; synchronized. enumeration; watched. enumeration; un-watched. enumeration; existence doubtful. enumeration; IALA_confirmed; documentation. made certain as to truth, accuracy, validity, availability, etc. - www.dictionary.com.. enumeration; IALA_candidate; documentation. Item selected for an action.. enumeration; IALA_under modification; documentation. Item that is in the process of being modified.. enumeration; IALA_candidate for modification; documentation. Item selected for modification.. enumeration; IALA_under removal/deletion; documentation. Item in the process of being removed or deleted.. enumeration; IALA_removed/deleted; documentation. Item that has been removed or deleted.. |

element Topmark/topmarkDaymarkShape

| diagram |  |
| --- | --- |
| type | topmarkDaymarkShape |
| facets | Kind; Value; Annotation. enumeration; cone, point up. enumeration; cone, point down. enumeration; sphere. enumeration; 2 spheres. enumeration; cylinder (can). enumeration; board. enumeration; x-shape (St. Andrew's cross). enumeration; upright cross (St George's cross). enumeration; cube, point up. enumeration; 2 cones, point to point. enumeration; 2 cones, base to base. enumeration; rhombus (diamond). enumeration; 2 cones (points upward). enumeration; 2 cones (points downward). enumeration; besom, point up (broom or perch). enumeration; besom, point down (broom or perch). enumeration; flag. enumeration; sphere over rhombus. enumeration; square. enumeration; rectangle, horizontal; documentation. where the two longer opposite sides are standing horizontally. A rectangle is a plane figure with four right angles and four straight sides, opposite sides being parallel and equal in length.. enumeration; rectangle, vertical; documentation. where the two longer opposite sides are standing vertically. A rectangle is a plane figure with four right angles and four straight sides, opposite sides being parallel and equal in length.. enumeration; trapezium, up. enumeration; trapezium, down. enumeration; triangle, point up. enumeration; triangle, point down. enumeration; circle. enumeration; two upright crosses (one over the other). enumeration; T-shape. enumeration; triangle pointing up over a circle. enumeration; upright cross over a circle. enumeration; rhombus over a circle. enumeration; circle over a triangle pointing up. enumeration; other shape. enumeration; IALA_tubular. |

element Topmark/verticalAccuracy

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal.1 |
| facets | Kind; Value; Annotation. minInclusive; 0.0. fractionDigits; 1. |
| annotation | documentation. The best estimate of the vertical accuracy of heights, vertical distances and vertical clearances, excluding sounding measurements.. documentation. Expected input: The one-dimensional error. The error is assumed to be positive and negative. The plus/minus character shall not be encoded. Minimum value: 0 Indication: Unit: defined in the HUNI subfield of the DSPM record or in the HUNITS attribute of the M_UNIT meta object class, e.g. metre (m) Resolution: 0.1 m or 0.1 ft Format: xx.x Example: 1.2 for an error of 1.2 metres.. |

element Topmark/verticalDatum

| diagram |  |
| --- | --- |
| type | verticalDatum |
| facets | Kind; Value; Annotation. enumeration; Mean low water springs. enumeration; Mean lower low water springs. enumeration; Mean sea level. enumeration; Lowest low water. enumeration; Mean low water. enumeration; Lowest low water springs. enumeration; Approximate mean low water springs. enumeration; Indian spring low water. enumeration; Low water springs. enumeration; Approximate lowest astronomical tide. enumeration; Nearly lowest low water. enumeration; Mean lower low water. enumeration; Low water. enumeration; Approximate mean low water. enumeration; Approximate mean lower low water. enumeration; Mean high water. enumeration; Mean high water springs. enumeration; High water. enumeration; Approximate mean sea level. enumeration; High water springs. enumeration; Mean higher high water. enumeration; Equinoctial spring low water. enumeration; Lowest astronomical tide. enumeration; Local datum. enumeration; International Great Lakes Datum 1985. enumeration; Mean water level. enumeration; Lower low water large tide. enumeration; Higher high water large tide. enumeration; Nearly highest high water. enumeration; Highest astronomical tide (HAT). |

element Topmark/verticalLength

| diagram |  |
| --- | --- |
| type | NonNegativeDecimal |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. The value of the vertical distance to the highest point of the object, measured from a specified vertical datum.. documentation. Minimum Value: 0. documentation. References: INT 1: IC 14; IE 4; IK 10-11; M-4: 302; 352.4; 421.1-2;. documentation. Indication: Unit: defined in the data set metadata Resolution: 0.1 m or 0.1 ft Format: xxx.x Example: 73 for a height of 73 metres. Remarks: Height must not be used for floating objects.. |

complexType VerticalDatum

| diagram |  |
| --- | --- |
| type | extension of MetaFeatureType |
| children | gml:boundedBy featureObjectIdentifier informationAssociation featureAssociation invFeatureAssociation verticalDatum geometry |
| used by | element; VerticalDatum. |
| annotation | documentation. Any level surface from which to reference elevations. Also called datum level, reference level, reference plane, levelling datum, datum for heights. (Adapted from IHO Dictionary – S-32).. |

element VerticalDatum/verticalDatum

| diagram |  |
| --- | --- |
| type | verticalDatum |
| facets | Kind; Value; Annotation. enumeration; Mean low water springs. enumeration; Mean lower low water springs. enumeration; Mean sea level. enumeration; Lowest low water. enumeration; Mean low water. enumeration; Lowest low water springs. enumeration; Approximate mean low water springs. enumeration; Indian spring low water. enumeration; Low water springs. enumeration; Approximate lowest astronomical tide. enumeration; Nearly lowest low water. enumeration; Mean lower low water. enumeration; Low water. enumeration; Approximate mean low water. enumeration; Approximate mean lower low water. enumeration; Mean high water. enumeration; Mean high water springs. enumeration; High water. enumeration; Approximate mean sea level. enumeration; High water springs. enumeration; Mean higher high water. enumeration; Equinoctial spring low water. enumeration; Lowest astronomical tide. enumeration; Local datum. enumeration; International Great Lakes Datum 1985. enumeration; Mean water level. enumeration; Lower low water large tide. enumeration; Higher high water large tide. enumeration; Nearly highest high water. enumeration; Highest astronomical tide (HAT). |

element VerticalDatum/geometry

| diagram |  |
| --- | --- |
| type | SurfacePropertyType |
| children | surfaceProperty |
| annotation | documentation. Permitted spatial primitives. |

simpleType beaconShape

| type | restriction of xs:string |
| --- | --- |
| used by | element; GenericBeacon/beaconShape. |
| facets | Kind; Value; Annotation. enumeration; stake, pole, perch, post; documentation. an elongated wood or metal pole, embedded in the bottom to serve as a navigational aid or a support for a navigational aid... enumeration; withy; documentation. a tree without roots stuck or spoiled into the bottom of the sea to serve as a navigational aid... enumeration; beacon tower; documentation. a solid structure of the order of 10 metres in height used as a navigational aid... enumeration; lattice beacon; documentation. a structure consisting of strips of metal or wood crossed or interlaced to form a structure to serve as an aid to navigation or as a support for an aid to navigation... enumeration; pile beacon; documentation. a long heavy timber(s) or section(s) of steel, wood, concrete, etc., forced into the seabed to serve as an aid to navigation or as a support for an aid to navigation... enumeration; cairn; documentation. a mound of stones, usually conical or pyramidal, raised specifically for maritime navigation... enumeration; buoyant beacon; documentation. a tall spar-like beacon fitted with a permanently submerged buoyancy chamber, the lower end of the body is secured to seabed sinker either by a flexible joint or by a cable under tension... |
| annotation | documentation. The shape a beacon exhibits. |

simpleType buildingShape

| type | restriction of xs:string |
| --- | --- |
| used by | element; SiloTank/buildingShape. |
| facets | Kind; Value; Annotation. enumeration; high-rise building; documentation. a building having many storeys... enumeration; pyramid; documentation. a polyhedron of which one face is a polygon of any number of sides, and the other faces are triangles with a common vertex... enumeration; cylindrical; documentation. shaped like a cylinder, which is a solid geometrical figure generated by straight lines fixed in direction and describing with one of its points a closed curve, especially a circle... enumeration; spherical; documentation. shaped like a sphere, which is a body the surface of which is at all points equidistant from the centre... enumeration; cubic; documentation. a shape the sides of which are six equal squares a regular hexahedron... |
| annotation | documentation. The specific shape of the building.. |

simpleType buoyShape

| type | restriction of xs:string |
| --- | --- |
| used by | element; GenericBuoy/buoyShape. |
| facets | Kind; Value; Annotation. enumeration; conical (nun, ogival); documentation. the upper part of the body above the water-line, or the greater part of the superstructure, has approximately the shape or the appearance of a pointed cone with the point upwards... enumeration; can (cylindrical); documentation. the upper part of the body above the water-line, or the greater part of the superstructure, has the shape of a cylinder, or a truncated cone that approximates to a cylinder, with a flat end uppermost... enumeration; spherical; documentation. the upper part of the body above the water-line, or the greater part of the superstructure, has the shape of a part of a sphere... enumeration; pillar; documentation. the upper part of the body above the water-line, or the greater part of the superstructure is a narrow vertical structure, pillar or lattice tower... enumeration; spar (spindle); documentation. the upper part of the body above the water-line, or the greater part of the superstructure, has the form of a pole, or of a very long cylinder, floating upright... enumeration; barrel (tun); documentation. the upper part of the body above the water-line, or the greater part of the superstructure, has the form of a barrel or cylinder floating horizontally... enumeration; super-buoy; documentation. a very large buoy, generally more than 5m in diameter... enumeration; ice buoy; documentation. a specially constructed shuttle shaped buoy which is used in ice conditions... |
| annotation | documentation. The shape a buoy exhibits.. |

simpleType categoryOfCardinalMark

| type | restriction of xs:string |
| --- | --- |
| used by | elements; BeaconCardinal/categoryOfCardinalMark BuoyCardinal/categoryOfCardinalMark. |
| facets | Kind; Value; Annotation. enumeration; north cardinal mark; documentation. Quadrant bounded by the true bearing NW-NE taken from the point of interest it should be passed to the north side of the mark... enumeration; east cardinal mark; documentation. Quadrant bounded by the true bearing NE-SE taken from the point of interest it should be passed to the east side of the mark... enumeration; south cardinal mark; documentation. Quadrant bounded by the true bearing SE-SW taken from the point of interest it should be passed to the south side of the mark... enumeration; west cardinal mark; documentation. Quadrant bounded by the true bearing SW-NW taken from the point of interest it should be passed to the west side of the mark... |
| annotation | documentation. Cardinal marks are classified according to the quadrant of space they occupy.. |

simpleType categoryOfFogSignal

| type | restriction of xs:string |
| --- | --- |
| used by | element; FogSignal/categoryOfFogSignal. |
| facets | Kind; Value; Annotation. enumeration; explosive; documentation. a signal produced by the firing of explosive charges... enumeration; diaphone; documentation. a diaphone uses compressed air and generally emits a powerful low-pitched sound, which often concludes with a brief sound of suddenly lowered pitch, termed the 'grunt'... enumeration; siren; documentation. a siren uses compressed air and exists in a variety of types which differ considerably in their sound and power... enumeration; nautophone; documentation. a horn having a diaphragm oscillated by electricity.. enumeration; reed; documentation. a reed uses compressed air and emits a weak, high pitched sound... enumeration; tyfon; documentation. a diaphragm horn which operates under the influence of compressed air or steam.. enumeration; bell; documentation. a ringing sound with a short range... enumeration; whistle; documentation. a distinctive sound made by a jet of air passing through an orifice... enumeration; gong; documentation. a sound produced by vibration of a disc when struck... enumeration; horn; documentation. a horn uses compressed air or electricity to vibrate a diaphragm and exists in a variety of types which differ greatly in their sound and power... |
| annotation | documentation. Classification of the various means of generating the fog signal.. |

simpleType categoryOfInstallationBuoy

| type | restriction of xs:string |
| --- | --- |
| used by | element; BuoyInstallation/categoryOfInstallationBuoy. |
| facets | Kind; Value; Annotation. enumeration; catenary anchor leg mooring (CALM); documentation. incorporates a large buoy which remains on the surface at all times and is moored by 4 or more anchors. Mooring hawsers and cargo hoses lead from a turntable on top of the buoy, so that the buoy does not turn as the ship swings to wind and stream... enumeration; single buoy mooring (SBM or SPM); documentation. a mooring structure used by tankers to load and unload in port approaches or in offshore oil and gas fields. The size of the structure can vary between a large mooring buoy and a manned floating structure. Also known as single point mooring (SPM).. |
| annotation | documentation. Classification of fixed installation buoy. |

simpleType categoryOfLandmark

| type | restriction of xs:string |
| --- | --- |
| used by | element; Landmark/categoryOfLandmark. |
| facets | Kind; Value; Annotation. enumeration; cairn; documentation. a mound of stones, usually conical or pyramidal, raised as a landmark or to designate a point of importance in surveying... enumeration; cemetery; documentation. an area of land for burying the dead... enumeration; chimney; documentation. a vertical structure containing a passage or flue for discharging smoke and gases... enumeration; dish aerial; documentation. a parabolic aerial for the receipt and transmission of high frequency radio signals... enumeration; flagstaff (flagpole); documentation. a staff or pole on which flags are raised... enumeration; flare stack; documentation. a tall structure used for burning-off waste oil or gas. Normally showing a flame and located at refineries... enumeration; mast; documentation. a straight vertical piece of timber or a hollow cylinder... enumeration; windsock; documentation. a tapered fabric sleeve mounted so as to catch and swing with the wind, thus indicating the wind direction... enumeration; monument; documentation. a structure erected or maintained as a memorial to a person or event... enumeration; column (pillar); documentation. a cylindrical or slightly tapering body of considerably greater length than diameter erected vertically... enumeration; memorial plaque; documentation. a slab of metal, usually ornamented, erected as a memorial to a person or event... enumeration; obelisk; documentation. a tapering shaft usually of stone or concrete, square or rectangular in section, with a pyramidal apex... enumeration; statue; documentation. a representation of a human, animal or fantasy figure in marble, bronze, etc... enumeration; cross; documentation. a monument, or other structure in form of a cross... enumeration; dome; documentation. a landmark comprising a hemispherical or spheroidal shaped structure.. enumeration; radar scanner; documentation. a device used for directing a radar beam through a search pattern.. enumeration; tower; documentation. a relatively tall structure which may be used for observation, support, storage or communication etc... enumeration; windmill; documentation. a wind driven system of vanes attached to a tower like structure (excluding wind-generated power plants)... enumeration; windmotor; documentation. a modern structure for the use of windpower... enumeration; spire/minaret; documentation. a tall conical or pyramid-shaped structure often built on the roof or tower of a building, especially a church or mosque... enumeration; large rock or boulder on land; documentation. an isolated rocky formation or a single large stone.. |
| annotation | documentation. Classification of prominent cultural and natural features in the landscape. |

simpleType categoryOfLateralMark

| type | restriction of xs:string |
| --- | --- |
| used by | elements; BeaconLateral/categoryOfLateralMark BuoyLateral/categoryOfLateralMark. |
| facets | Kind; Value; Annotation. enumeration; port-hand lateral mark; documentation. indicates the port boundary of a navigational channel or suggested route when proceeding in the 'conventional direction of buoyage'... enumeration; starboard-hand lateral mark; documentation. indicates the starboard boundary of a navigational channel or suggested route when proceeding in the 'conventional direction of buoyage'... enumeration; preferred channel to starboard lateral mark; documentation. at a point where a channel divides, when proceeding in the 'conventional direction of buoyage', the preferred channel (or primary route) is indicated by a modified port-hand lateral mark... enumeration; preferred channel to port lateral mark; documentation. at a point where a channel divides, when proceeding in the 'conventional direction of buoyage', the preferred channel (or primary route) is indicated by a modified starboard-hand lateral mark... |
| annotation | documentation. Classification of lateral buoys. |

simpleType categoryOfLight

| type | restriction of xs:string |
| --- | --- |
| used by | element; Light/categoryOfLight. |
| facets | Kind; Value; Annotation. enumeration; directional function; documentation. a light illuminating a sector of very narrow angle and intended to mark a direction to follow... enumeration; leading light; documentation. a light associated with other lights so as to form a leading line to be followed... enumeration; aero light; documentation. an aero light is established for aeronautical navigation and may be of higher power than marine lights and visible from well offshore... enumeration; air obstruction light; documentation. a light marking an obstacle which constitutes a danger to air navigation... enumeration; fog detector light; documentation. a light used to automatically determine conditions of visibility which warrant the turning on or off of a sound signal... enumeration; flood light; documentation. a broad beam light used to illuminate a structure or area... enumeration; strip light; documentation. a light whose source has a linear form generally horizontal, which can reach a length of several metres... enumeration; subsidiary light; documentation. a light placed on or near the support of a main light and having a special use in navigation... enumeration; spotlight; documentation. a powerful light focused so as to illuminate a small area... enumeration; front; documentation. term used with leading lights to describe the position of the light on the lead as viewed from seaward... enumeration; rear; documentation. term used with leading lights to describe the position of the light on the lead as viewed from seaward... enumeration; lower; documentation. term used with leading lights to describe the position of the light on the lead as viewed from seaward... enumeration; upper; documentation. term used with leading lights to describe the position of the light on the lead as viewed from seaward... enumeration; moiré effect; documentation. a short range (up to 2km) type of directional light. Sodium lighting gives a yellow background to a screen on which a vertical black line will be seen by an observer on the centre line... enumeration; emergency; documentation. a light available as a backup to a main light which will be illuminated should the main light fail... enumeration; bearing light; documentation. a light which enables its approximate bearing to be obtained without the use of a compass... enumeration; horizontally disposed; documentation. a group of lights of identical character and almost identical position, that are disposed horizontally... enumeration; vertically disposed; documentation. a group of lights of identical character and almost identical position, that are disposed vertically... |
| annotation | documentation. Classification of different light types. |

simpleType categoryOfNavigationLine

| type | restriction of xs:string |
| --- | --- |
| used by | element; NavigationLine/categoryOfNavigationLine. |
| facets | Kind; Value; Annotation. enumeration; clearing line; documentation. a straight line that marks the boundary between a safe and a dangerous area or that passes clear of a navigational danger... enumeration; transit line; documentation. a line passing through one or more fixed marks... enumeration; leading line bearing a recommended track; documentation. a line passing through one or more clearly defined objects, along the path of which a vessel can approach safely up to a certain distance off... |
| annotation | documentation. Classification of route guidance given to vessels. |

simpleType categoryOfOffshorePlatform

| type | restriction of xs:string |
| --- | --- |
| used by | element; OffshorePlatform/categoryOfOffshorePlatform. |
| facets | Kind; Value; Annotation. enumeration; oil derrick/rig; documentation. a temporary mobile structure, either fixed or floating, used in the exploration stages of oil and gas fields... enumeration; production platform; documentation. a term used to indicate a permanent offshore structure equipped to control the flow of oil or gas. It does not include entirely submarine structures... enumeration; observation/research platform; documentation. a platform from which one's surroundings or events can be observed, noted or recorded such as for scientific study... enumeration; articulated loading platform (ALP); documentation. a metal lattice tower, buoyant at one end and attached at the other by a universal joint to a concrete filled base on the sea bed. The platform may be fitted with a helicopter platform, emergency accommodation and hawser/hose retrieval... enumeration; single anchor leg mooring (SALM); documentation. a rigid frame or tube with a buoyancy device at its upper end , secured at its lower end to a universal joint on a large steel or concrete base resting on the sea bed, and at its upper end to a mooring buoy by a chain or wire... enumeration; mooring tower; documentation. a platform secured to the sea bed and surmounted by a turntable to which ships moor... enumeration; artificial island; documentation. a man-made structure usually built for the exploration or exploitation of marine resources, marine scientific research, tidal observations, etc... enumeration; floating production, storage and off-loading vessel (FPSO); documentation. an offshore oil/gas facility consisting of a moored tanker/barge by which the product is extracted, stored and exported... enumeration; accommodation platform; documentation. a platform used primarily for eating, sleeping and recreation purposes... enumeration; navigation, communication and control buoy (NCCB); documentation. a floating structure with control room, power and storage facilities, attached to the sea bed by a flexible pipeline and cables... |
| annotation | documentation. Classification of an offshore raised structure. |

simpleType categoryOfPile

| type | restriction of xs:string |
| --- | --- |
| used by | element; Pile/categoryOfPile. |
| facets | Kind; Value; Annotation. enumeration; stake; documentation. an elongated wood or metal pole embedded in the bottom to serve as a marker or support... enumeration; post; documentation. a vertical piece of timber, metal or concrete forced into the earth or sea bed... enumeration; tripodal; documentation. a single structure comprising 3 or more piles held together (sections of heavy timber, steel or concrete), and forced into the earth or sea bed... |
| annotation | documentation. Classification of pile, driven into the earth as a foundation or support for a structure. |

simpleType categoryOfRadarTransponderBeacon

| type | restriction of xs:string |
| --- | --- |
| used by | element; RadarTransponderBeacon/categoryOfRadarTransponderBeacon. |
| facets | Kind; Value; Annotation. enumeration; ramark, radar beacon transmitting continuously; documentation. a radar marker beacon which continuously transmits a signal appearing as a radial line on a radar screen, the line indicating the direction of the beacon. Ramarks are intended primarily for marine use. The name 'ramark' is derived from the words radar marker... enumeration; racon, radar transponder beacon; documentation. a radar beacon which returns a coded signal which provides identification of the beacon, as well as range and bearing. The range and bearing are indicated by the location of the first character received on the radar screen. The name 'racon' is derived from the words radar beacon... enumeration; leading racon/radar transponder beacon; documentation. a radar beacon that may be used (in conjunction with at least one other radar beacon) to indicate a leading line... |
| annotation | documentation. Classification of radar transponder beacon based on fuctionality.. |

simpleType categoryOfRecommendedTrack

| type | restriction of xs:string |
| --- | --- |
| used by | element; RecommendedTrack/categoryOfRecommendedTrack. |
| facets | Kind; Value; Annotation. enumeration; based on a system of fixed marks; documentation. a straight route (known as a recommended track, range or leading line), which comprises at least two structures (usually beacons or daymarks) and/or natural features, which may carry lights and/or top-marks. The structures/features are positioned so that when observed to be in line, a vessel can follow a known bearing with safety... enumeration; not based on a system of fixed marks; documentation. a route (known as a recommended track or preferred route) which is not based on a series of structures or features in line... |
| annotation | documentation. Classification of track based on defining navigational marks. |

simpleType categoryOfSiloTank

| type | restriction of xs:string |
| --- | --- |
| used by | element; SiloTank/categoryOfSiloTank. |
| facets | Kind; Value; Annotation. enumeration; silo in general; documentation. a generally cylindrical tower used for storing fodder or grain... enumeration; tank in general; documentation. a fixed structure for storing liquids... enumeration; grain elevator; documentation. a storage building for grain. Usually a tall frame, metal or concrete structure with an especially compartmented interior... enumeration; water tower; documentation. a tower with an elevated container used to hold water... |
| annotation | documentation. Classification based on the product. |

simpleType categoryOfSpecialPurposeMark

| type | restriction of xs:string |
| --- | --- |
| used by | elements; BeaconSpecialPurposeGeneral/categoryOfSpecialPurposeMark BuoySpecialPurposeGeneral/categoryOfSpecialPurposeMark Daymark/categoryOfSpecialPurposeMark. |
| facets | Kind; Value; Annotation. enumeration; firing danger area mark; documentation. a mark used to indicate a firing danger area, usually at sea... enumeration; target mark; documentation. any object toward which something is directed. The distinctive marking or instrumentation of a ground point to aid its identification on a photograph... enumeration; marker ship mark; documentation. a mark marking the position of a ship which is used as a target during some military exercise... enumeration; degaussing range mark; documentation. a mark used to indicate a degaussing range... enumeration; barge mark; documentation. a mark of relevance to barges... enumeration; cable mark; documentation. a mark used to indicate the position of submarine cables or the point at which they run on to the land... enumeration; spoil ground mark; documentation. a mark used to indicate the limit of a spoil ground.. enumeration; outfall mark; documentation. a mark used to indicate the position of an outfall or the point at which it leaves the land... enumeration; ODAS (Ocean-Data-Acquisition-System); documentation. Ocean Data Acquisition System.. enumeration; recording mark; documentation. a mark used to record data for scientific purposes... enumeration; seaplane anchorage mark; documentation. a mark used to indicate a seaplane anchorage... enumeration; recreation zone mark; documentation. a mark used to indicate a recreation zone... enumeration; private mark; documentation. a privately maintained mark... enumeration; mooring mark; documentation. a mark indicating a mooring or moorings... enumeration; LANBY (Large Automatic Navigational Buoy); documentation. a large buoy designed to take the place of a lightship where construction of an offshore light station is not feasible... enumeration; leading mark; documentation. aids to navigation or other indicators so located as to indicate the path to be followed. Leading marks identify a leading line when they are in transit... enumeration; measured distance mark; documentation. a mark forming part of a transit indicating one end of a measured distance... enumeration; notice mark; documentation. a notice board or sign indicating information to the mariner... enumeration; TSS mark (Traffic Separation Scheme); documentation. a mark indicating a traffic separation scheme... enumeration; anchoring prohibited mark; documentation. a mark indicating an anchoring prohibited area... enumeration; berthing prohibited mark; documentation. a mark indicating that berthing is prohibited... enumeration; overtaking prohibited mark; documentation. a mark indicating that overtaking is prohibited... enumeration; two-way traffic prohibited mark; documentation. a mark indicating a one-way route... enumeration; reduced wake mark; documentation. a mark indicating that vessels must not generate excessive wake... enumeration; speed limit mark; documentation. a mark indicating that a speed limit applies... enumeration; stop mark; documentation. a mark indicating the place where the bow of a ship must stop when traffic lights show red... enumeration; general warning mark; documentation. a mark indicating that special caution must be exercised in the vicinity of the mark... enumeration; sound ship's siren mark; documentation. a mark indicating that a ship should sound its siren or horn... enumeration; restricted vertical clearance mark; documentation. a mark indicating the minimum vertical space available for passage... enumeration; maximum vessel's draught mark; documentation. a mark indicating the maximum draught of vessel permitted... enumeration; restricted horizontal clearance mark; documentation. a mark indicating the minimum horizontal space available for passage... enumeration; strong current warning mark; documentation. a mark warning of strong currents... enumeration; berthing permitted mark; documentation. a mark indicating that berthing is allowed... enumeration; overhead power cable mark; documentation. a mark indicating an overhead power cable... enumeration; channel edge gradient mark; documentation. a mark indicating the gradient of the slope of a dredge channel edge... enumeration; telephone mark; documentation. a mark indicating the presence of a telephone... enumeration; ferry crossing mark; documentation. a mark indicating that a ferry route crosses the ship route often used with a 'sound ship's siren' mark... enumeration; pipeline mark; documentation. a mark used to indicate the position of submarine pipelines or the point at which they run on to the land... enumeration; anchorage mark; documentation. a mark indicating an anchorage area... enumeration; clearing mark; documentation. a mark used to indicate a clearing line... enumeration; control mark; documentation. a mark indicating the location at which a restriction or requirement exists... enumeration; diving mark; documentation. a mark indicating that diving may take place in the vicinity... enumeration; refuge beacon; documentation. a mark providing or indicating a place of safety... enumeration; foul ground mark; documentation. a mark indicating a foul ground... enumeration; yachting mark; documentation. a mark installed for use by yachtsmen... enumeration; heliport mark; documentation. a mark indicating an area where helicopters may land... enumeration; GPS mark; documentation. a mark indicating a location at which a GNSS position has been accurately determined... enumeration; seaplane landing mark; documentation. a mark indicating an area where sea-planes land... enumeration; entry prohibited mark; documentation. a mark indicating that entry is prohibited... enumeration; work in progress mark; documentation. a mark indicating that work (generally construction) is in progress... enumeration; mark with unknown purpose; documentation. a mark whose detailed characteristics are unknown... enumeration; wellhead mark; documentation. a mark indicating a borehole that produces or is capable of producing oil or natural gas... enumeration; channel separation mark; documentation. a mark indicating the point at which a channel divides separately into two channels... enumeration; marine farm mark; documentation. a mark indicating the existence of a fish, mussel, oyster or pearl farm/ culture... enumeration; artificial reef mark; documentation. a mark indicating the existence or the extent of an artificial reef... enumeration; IALA_jetski prohibited; documentation. A mark indicating a jetski prohibited area.. |
| annotation | documentation. Classification of an aid to navigation which signifies some special purpose. |

simpleType categoryOfTemporalVariation

| type | restriction of xs:string |
| --- | --- |
| used by | element; QualityOfNonBathymetricData/categoryOfTemporalVariation. |
| facets | Kind; Value; Annotation. enumeration; unassessed; documentation. Temporal variation not assessed or cannot be determined.. enumeration; event; documentation. No new hydrographic survey conducted after an event (e.g. hurricane, earthquake, volcanic eruption, landslide, etc), which is considered likely to have changed the seafloor significantly... enumeration; likely to change; documentation. Continuous or frequent change (e.g. river siltation, sand waves, seasonal storms, ice bergs, etc)... enumeration; likely to change, but significant shoaling unlikely; documentation. Likely to change but significant shoaling unlikely.. enumeration; unlikely to change; documentation. Significant change to the seafloor is not expected... |
| annotation | documentation. An assessment of the likelihood of change within an area since last survey.. |

simpleType colour

| type | restriction of xs:string |
| --- | --- |
| used by | elements; Daymark/colour GenericBeacon/colour GenericBuoy/colour Landmark/colour Light/colour LightFloat/colour LightVessel/colour OffshorePlatform/colour Pile/colour RetroReflector/colour SiloTank/colour Topmark/colour. |
| facets | Kind; Value; Annotation. enumeration; white. enumeration; black. enumeration; red. enumeration; green. enumeration; blue. enumeration; yellow. enumeration; grey. enumeration; brown. enumeration; amber. enumeration; violet. enumeration; orange. enumeration; magenta. enumeration; pink. |
| annotation | documentation. The property possessed by an object of producing different sensations on the eye as a result of the way it reflects or emits light.. |

simpleType colourPattern

| type | restriction of xs:string |
| --- | --- |
| used by | elements; Daymark/colourPattern GenericBeacon/colourPattern GenericBuoy/colourPattern Landmark/colourPattern LightFloat/colourPattern LightVessel/colourPattern OffshorePlatform/colourPattern Pile/colourPattern RetroReflector/colourPattern SiloTank/colourPattern Topmark/colourPattern. |
| facets | Kind; Value; Annotation. enumeration; horizontal stripes; documentation. Straight bands or stripes of differing colours oriented horizontally. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; vertical stripes; documentation. Straight bands or stripes of differing colours oriented vertically. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; diagonal stripes; documentation. Straight bands or stripes of differing colours oriented diagonally (i.e. not horizontally or vertically). (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; squared; documentation. Often referred to as checker plate, where alternate colours are used to create squares similar to a chess or draught board. The pattern may be straight or diagonal. (S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; stripes (direction unknown); documentation. Straight bands or stripes of differing colours oriented in an unknown direction. (Adapted from S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; border stripe; documentation. A band or stripe of colour which is displayed around the outer edge of the feature, which may also form a border to an inner pattern or plain colour. (S-57 Edition 3.1, Appendix A – Chapter 2, Page 2.113, November 2000)... enumeration; IALA_single colour; documentation. One solid colour of uniform coverage.. enumeration; IALA_rectangle; documentation. A four-sided shape that is made up of two pairs of parallel lines and that has four right angles, on a different coloured background.. enumeration; IALA_triangle; documentation. a shape that is made up of three lines and three angles, on a different coloured background.. |
| annotation | documentation. A regular repeated design containing more than one colour. |

simpleType condition

| type | restriction of xs:string |
| --- | --- |
| used by | elements; GenericBeacon/condition Landmark/condition OffshorePlatform/condition Pile/condition SiloTank/condition. |
| facets | Kind; Value; Annotation. enumeration; under construction. enumeration; ruined. enumeration; under reclamation. enumeration; wingless. enumeration; planned construction. |
| annotation | documentation. The various conditions of buildings and other constructions.. |

simpleType Decimal0.0To360.0

| type | restriction of xs:decimal |
| --- | --- |
| used by | elements; QualityOfNonBathymetricData/directionUncertainty Light/orientation NavigationLine/orientation RecommendedTrack/orientation LocalDirectionOfBuoyage/orientation Light/sectorLimitOne RadarTransponderBeacon/sectorLimitOne Light/sectorLimitTwo RadarTransponderBeacon/sectorLimitTwo. |
| facets | Kind; Value; Annotation. minInclusive; 0.0. maxInclusive; 360.0. fractionDigits; 1. |
| annotation | documentation. Type for value interval [0.0, 360.0]. Also limits value to 1 decimal place.. |

simpleType exhibitionConditionOfLight

| type | restriction of xs:string |
| --- | --- |
| used by | element; Light/exhibitionConditionOfLight. |
| facets | Kind; Value; Annotation. enumeration; light shown without change of character; documentation. a light shown throughout the 24 hours without change of character... enumeration; daytime light; documentation. a light which is only exhibited by day... enumeration; fog light; documentation. a light which is exhibited in fog or conditions of reduced visibility... enumeration; night light; documentation. a light which is only exhibited at night... |
| annotation | documentation. The outward display of the light. |

simpleType function

| type | restriction of xs:string |
| --- | --- |
| used by | element; Landmark/function. |
| facets | Kind; Value; Annotation. enumeration; harbour-master's office. enumeration; custom office. enumeration; health office. enumeration; hospital. enumeration; post office. enumeration; hotel. enumeration; railway station. enumeration; police station. enumeration; water-police station. enumeration; pilot office. enumeration; pilot lookout. enumeration; bank office. enumeration; headquarters for district control. enumeration; transit shed/warehouse. enumeration; factory. enumeration; power station. enumeration; administrative. enumeration; educational facility. enumeration; church. enumeration; chapel. enumeration; temple. enumeration; pagoda. enumeration; shinto shrine. enumeration; buddhist temple. enumeration; mosque. enumeration; marabout. enumeration; lookout. enumeration; communication. enumeration; television. enumeration; radio. enumeration; radar. enumeration; light support. enumeration; microwave. enumeration; cooling. enumeration; observation. enumeration; timeball. enumeration; clock. enumeration; control. enumeration; airship mooring. enumeration; stadium. enumeration; bus station. |
| annotation | documentation. A specific role that describes a feature. |

simpleType IALA_aidAvailabilityCategory

| type | restriction of xs:string |
| --- | --- |
| used by | element; StructureObject/IALA_aidAvailabilityCategory. |
| facets | Kind; Value; Annotation. enumeration; Category 1; documentation. An AtoN or system of AtoN that is considered by the Competent Authority to be of vital navigational significance. Recommendation O-130 – Categorisation and Availability Objectives for Short Range Aids to Navigation December 2004 - Revised June 2011.. enumeration; Category 2; documentation. An AtoN or system of AtoN that is considered by the Competent Authority to be of important navigational significance. Recommendation O-130 – Categorisation and Availability Objectives for Short Range Aids to Navigation December 2004 - Revised June 2011.. enumeration; Category 3; documentation. An AtoN or system of AtoN that is considered by the Competent Authority to be of necessary navigational significance. Recommendation O-130 – Categorisation and Availability Objectives for Short Range Aids to Navigation December 2004 - Revised June 2011.. |

simpleType IALA_categoryOfAggregation

| type | union of (restriction of xs:string, restriction of xs:string) |
| --- | --- |
| used by | element; Aggregation/IALA_categoryOfAggregation. |
| annotation | documentation. Named aggregations between two or more aids to navigation and/or navigationally relevant features.. |

simpleType IALA_categoryOfAssociation

| type | union of (restriction of xs:string, restriction of xs:string) |
| --- | --- |
| used by | element; Association/IALA_categoryOfAssociation. |
| annotation | documentation. Named associations between two or more aids to navigation and/or navigationally relevant features.. |

simpleType ISO3166-alpha2

| type | restriction of xs:string |
| --- | --- |
| facets | Kind; Value; Annotation. pattern; \w{2}. |
| annotation | documentation. stub for ISO 3166 alpha2 country codes. |

simpleType ISO639-2

| type | restriction of xs:string |
| --- | --- |
| facets | Kind; Value; Annotation. pattern; \w{3}. |
| annotation | documentation. stub for ISO 639-2 language codes. |

simpleType ISO639-3

| type | restriction of xs:string |
| --- | --- |
| facets | Kind; Value; Annotation. pattern; \w{3}. |
| annotation | documentation. stub for ISO 639-3 language codes. |

simpleType lightCharacteristic

| type | restriction of xs:string |
| --- | --- |
| used by | element; Light/lightCharacteristic. |
| facets | Kind; Value; Annotation. enumeration; fixed. enumeration; flashing. enumeration; long-flashing. enumeration; quick-flashing. enumeration; very quick-flashing. enumeration; ultra quick-flashing. enumeration; isophased. enumeration; occulting. enumeration; interrupted quick-flashing. enumeration; interrupted very quick-flashing. enumeration; interrupted ultra quick-flashing. enumeration; morse. enumeration; fixed/flash. enumeration; flash/long-flash. enumeration; occulting/flash. enumeration; fixed/long-flash. enumeration; occulting alternating. enumeration; long-flash alternating. enumeration; flash alternating. enumeration; group alternating. enumeration; quick-flash plus long-flash. enumeration; very quick-flash plus long-flash. enumeration; ultra quick-flash plus long-flash. enumeration; alternating. enumeration; fixed and alternating flashing. |
| annotation | documentation. The typical behaviour associated with the light. |

simpleType lightVisibility

| type | restriction of xs:string |
| --- | --- |
| used by | element; Light/lightVisibility. |
| facets | Kind; Value; Annotation. enumeration; high intensity. enumeration; low intensity. enumeration; faint. enumeration; intensified. enumeration; unintensified. enumeration; visibility deliberately restricted. enumeration; obscured. enumeration; partially obscured. |
| annotation | documentation. The specific visibility of a light, with respect to the light's intensity and ease of recognition.. |

simpleType marksNavigationalSystemOf

| type | restriction of xs:string |
| --- | --- |
| used by | elements; GenericBeacon/marksNavigationalSystemOf GenericBuoy/marksNavigationalSystemOf Light/marksNavigationalSystemOf RetroReflector/marksNavigationalSystemOf Topmark/marksNavigationalSystemOf NavigationalSystemOfMarks/marksNavigationalSystemOf. |
| facets | Kind; Value; Annotation. enumeration; IALA A. enumeration; IALA B. enumeration; no system. enumeration; other system. |
| annotation | documentation. The system of navigational buoyage a region complies with.. |

simpleType multiplicityOfLightsType

| type | restriction of xs:positiveInteger |
| --- | --- |
| used by | element; Light/multiplicityOfLights. |
| facets | Kind; Value; Annotation. minInclusive; 2. |
| annotation | documentation. The number of lights of identical character that exist as a co-located group. Integer (minimum 2).. |

simpleType natureOfConstruction

| type | restriction of xs:string |
| --- | --- |
| used by | elements; Daymark/natureOfConstruction GenericBeacon/natureOfConstruction Landmark/natureOfConstruction LightFloat/natureOfConstruction LightVessel/natureOfConstruction OffshorePlatform/natureOfConstruction SiloTank/natureOfConstruction GenericBuoy/natureOfconstuction. |
| facets | Kind; Value; Annotation. enumeration; masonry. enumeration; concreted. enumeration; loose boulders. enumeration; hard surfaced. enumeration; unsurfaced. enumeration; wooden. enumeration; metal. enumeration; glass reinforced plastic (GRP). enumeration; painted. enumeration; IALA_fiberglass; documentation. Constructed from fiberglass.. enumeration; IALA_plastic; documentation. Constructed from plastic.. |
| annotation | documentation. The buildings primary construction material. |

simpleType NonNegativeDecimal

| type | restriction of xs:decimal |
| --- | --- |
| used by | elements; Daymark/height IALA_EnvironmentObservationEquipment/height Pile/height GenericBeacon/height OffshorePlatform/height Landmark/height Topmark/height Light/height SiloTank/height RetroReflector/height RadarReflector/height LightVessel/horizontalAccuracy LightFloat/horizontalAccuracy QualityOfNonBathymetricData/horizontalDistanceUncertainty LightVessel/horizontalLength LightFloat/horizontalLength QualityOfNonBathymetricData/horizontalPositionUncertainty LightVessel/horizontalWidth LightFloat/horizontalWidth Light/IALA_valueOfGeographicalRange Light/IALA_valueOfLuminousRange SpatialUncertainty/positionalAccuracy RecommendedTrack/soundingAccuracy FogSignal/valueOfMaximumRange RadarTransponderBeacon/valueOfNominalRange Light/valueOfNominalRange Daymark/verticalLength GenericBeacon/verticalLength LightVessel/verticalLength LightFloat/verticalLength OffshorePlatform/verticalLength Pile/verticalLength SiloTank/verticalLength Landmark/verticalLength Topmark/verticalLength GenericBuoy/verticalLength QualityOfNonBathymetricData/verticalUncertainty. simpleType; NonNegativeDecimal.1. |
| facets | Kind; Value; Annotation. minInclusive; 0.0. |
| annotation | documentation. Type for decimal values with minimum 0.0.. |

simpleType NonNegativeDecimal.1

| type | restriction of NonNegativeDecimal |
| --- | --- |
| used by | elements; Daymark/verticalAccuracy GenericBeacon/verticalAccuracy GenericBuoy/verticalAccuracy Landmark/verticalAccuracy Light/verticalAccuracy LightFloat/verticalAccuracy LightVessel/verticalAccuracy OffshorePlatform/verticalAccuracy Pile/verticalAccuracy RadarReflector/verticalAccuracy RetroReflector/verticalAccuracy SiloTank/verticalAccuracy Topmark/verticalAccuracy. |
| facets | Kind; Value; Annotation. minInclusive; 0.0. fractionDigits; 1. |
| annotation | documentation. Type for decimal values with minimum 0.0 and at most 1 decimal place. |

simpleType product

| type | restriction of xs:string |
| --- | --- |
| used by | elements; OffshorePlatform/product SiloTank/product. |
| facets | Kind; Value; Annotation. enumeration; oil. enumeration; gas. enumeration; water. enumeration; stone. enumeration; coal. enumeration; ore. enumeration; chemicals. enumeration; drinking water. enumeration; milk. enumeration; bauxite. enumeration; coke. enumeration; iron ingots. enumeration; salt. enumeration; sand. enumeration; timber. enumeration; sawdust/wood chips. enumeration; scrap metal. enumeration; liquified natural gas (LNG). enumeration; liquified petroleum gas (LPG). enumeration; wine. enumeration; cement. enumeration; grain. |
| annotation | documentation. The various substances which are transported, stored or exploited.. |

simpleType qualityOfPosition

| type | restriction of xs:string |
| --- | --- |
| used by | element; SpatialUncertainty/qualityOfPosition. |
| facets | Kind; Value; Annotation. enumeration; surveyed. enumeration; unsurveyed. enumeration; inadequately surveyed. enumeration; approximate. enumeration; position doubtful. enumeration; unreliable. enumeration; reported (not surveyed). enumeration; reported (not confirmed). enumeration; estimated. enumeration; precisely known. enumeration; calculated. |
| annotation | documentation. The degree of reliability attributed to a position. |

simpleType qualityOfSoundingMeasurement

| type | restriction of xs:string |
| --- | --- |
| used by | element; RecommendedTrack/qualityOfSoundingMeasurement. |
| facets | Kind; Value; Annotation. enumeration; depth known. enumeration; depth unknown. enumeration; doubtful sounding. enumeration; unreliable sounding. enumeration; no bottom found at value shown. enumeration; least depth known. enumeration; least depth unknown, safe clearance at value shown. enumeration; value reported (not surveyed). enumeration; value reported (not confirmed). enumeration; maintained depth. enumeration; not regularly maintained. |
| annotation | documentation. The reliability of the value of a sounding.. |

simpleType radarConspicuous

| type | restriction of xs:string |
| --- | --- |
| used by | elements; GenericBuoy/radarConspicious GenericBeacon/radarConspicuous Landmark/radarConspicuous LightFloat/radarConspicuous LightVessel/radarConspicuous OffshorePlatform/radarConspicuous SiloTank/radarConspicuous. |
| facets | Kind; Value; Annotation. enumeration; radar conspicuous. enumeration; not radar conspicuous. enumeration; radar conspicuous (has radar reflector). |
| annotation | documentation. A feature which returns a strong radar echo.. |

simpleType signalGeneration

| type | restriction of xs:string |
| --- | --- |
| used by | element; FogSignal/signalGeneration. |
| facets | Kind; Value; Annotation. enumeration; automatically. enumeration; by wave action. enumeration; by hand. enumeration; by wind. |
| annotation | documentation. The mechanism used to generate a fog signal.. |

simpleType status

| type | restriction of xs:string |
| --- | --- |
| used by | elements; Daymark/status IALA_EnvironmentObservationEquipment/status FogSignal/status GenericBeacon/status GenericBuoy/status Landmark/status Light/status LightFloat/status LightVessel/status NavigationLine/status OffshorePlatform/status RadarReflector/status RadarTransponderBeacon/status RecommendedTrack/status RetroReflector/status SiloTank/status Topmark/status. |
| facets | Kind; Value; Annotation. enumeration; permanent. enumeration; occasional. enumeration; recommended. enumeration; not in use. enumeration; periodic/intermittent. enumeration; reserved. enumeration; temporary. enumeration; private. enumeration; mandatory. enumeration; extinguished. enumeration; illuminated. enumeration; historic. enumeration; public. enumeration; synchronized. enumeration; watched. enumeration; un-watched. enumeration; existence doubtful. enumeration; IALA_confirmed; documentation. made certain as to truth, accuracy, validity, availability, etc. - www.dictionary.com.. enumeration; IALA_candidate; documentation. Item selected for an action.. enumeration; IALA_under modification; documentation. Item that is in the process of being modified.. enumeration; IALA_candidate for modification; documentation. Item selected for modification.. enumeration; IALA_under removal/deletion; documentation. Item in the process of being removed or deleted.. enumeration; IALA_removed/deleted; documentation. Item that has been removed or deleted.. |
| annotation | documentation. The condition of an object at a given instant in time.. |

simpleType techniqueOfSoundingMeasurement

| type | restriction of xs:string |
| --- | --- |
| used by | element; RecommendedTrack/techniqueOfSoundingMeasurement. |
| facets | Kind; Value; Annotation. enumeration; found by echo-sounder. enumeration; found by side scan sonar. enumeration; found by multi-beam. enumeration; found by diver. enumeration; found by lead-line. enumeration; swept by wire-drag. enumeration; found by laser. enumeration; swept by vertical acoustic system. enumeration; found by electromagnetic sensor. enumeration; photogrammetry. enumeration; satellite imagery. enumeration; found by levelling. enumeration; swept by side-scan sonar. enumeration; computer generated. |
| annotation | documentation. Survey method used to obtain depth information. |

simpleType topmarkDaymarkShape

| type | restriction of xs:string |
| --- | --- |
| used by | elements; Daymark/topmarkDaymarkShape Topmark/topmarkDaymarkShape. |
| facets | Kind; Value; Annotation. enumeration; cone, point up. enumeration; cone, point down. enumeration; sphere. enumeration; 2 spheres. enumeration; cylinder (can). enumeration; board. enumeration; x-shape (St. Andrew's cross). enumeration; upright cross (St George's cross). enumeration; cube, point up. enumeration; 2 cones, point to point. enumeration; 2 cones, base to base. enumeration; rhombus (diamond). enumeration; 2 cones (points upward). enumeration; 2 cones (points downward). enumeration; besom, point up (broom or perch). enumeration; besom, point down (broom or perch). enumeration; flag. enumeration; sphere over rhombus. enumeration; square. enumeration; rectangle, horizontal; documentation. where the two longer opposite sides are standing horizontally. A rectangle is a plane figure with four right angles and four straight sides, opposite sides being parallel and equal in length.. enumeration; rectangle, vertical; documentation. where the two longer opposite sides are standing vertically. A rectangle is a plane figure with four right angles and four straight sides, opposite sides being parallel and equal in length.. enumeration; trapezium, up. enumeration; trapezium, down. enumeration; triangle, point up. enumeration; triangle, point down. enumeration; circle. enumeration; two upright crosses (one over the other). enumeration; T-shape. enumeration; triangle pointing up over a circle. enumeration; upright cross over a circle. enumeration; rhombus over a circle. enumeration; circle over a triangle pointing up. enumeration; other shape. enumeration; IALA_tubular. |
| annotation | documentation. The shape a topmark or day mark exhibits. |

simpleType trafficFlow

| type | restriction of xs:string |
| --- | --- |
| used by | element; RecommendedTrack/trafficFlow. |
| facets | Kind; Value; Annotation. enumeration; inbound. enumeration; outbound. enumeration; one-way. enumeration; two-way. |

simpleType verticalDatum

| type | restriction of xs:string |
| --- | --- |
| used by | elements; Daymark/verticalDatum Landmark/verticalDatum Light/verticalDatum OffshorePlatform/verticalDatum Pile/verticalDatum RadarReflector/verticalDatum RecommendedTrack/verticalDatum RetroReflector/verticalDatum SiloTank/verticalDatum Topmark/verticalDatum SoundingDatum/verticalDatum VerticalDatum/verticalDatum. |
| facets | Kind; Value; Annotation. enumeration; Mean low water springs. enumeration; Mean lower low water springs. enumeration; Mean sea level. enumeration; Lowest low water. enumeration; Mean low water. enumeration; Lowest low water springs. enumeration; Approximate mean low water springs. enumeration; Indian spring low water. enumeration; Low water springs. enumeration; Approximate lowest astronomical tide. enumeration; Nearly lowest low water. enumeration; Mean lower low water. enumeration; Low water. enumeration; Approximate mean low water. enumeration; Approximate mean lower low water. enumeration; Mean high water. enumeration; Mean high water springs. enumeration; High water. enumeration; Approximate mean sea level. enumeration; High water springs. enumeration; Mean higher high water. enumeration; Equinoctial spring low water. enumeration; Lowest astronomical tide. enumeration; Local datum. enumeration; International Great Lakes Datum 1985. enumeration; Mean water level. enumeration; Lower low water large tide. enumeration; Higher high water large tide. enumeration; Nearly highest high water. enumeration; Highest astronomical tide (HAT). |
| annotation | documentation. Vertical datum used for measuring elevations of points on the earth's surface. Its the datum to which both heights and soundings are referred.. |

simpleType visuallyConspicuous

| type | restriction of xs:string |
| --- | --- |
| used by | elements; GenericBeacon/visuallyConspicuous Landmark/visuallyConspicuous LightFloat/visuallyConspicuous LightVessel/visuallyConspicuous OffshorePlatform/visuallyConspicuous Pile/visuallyConspicuous SiloTank/visuallyConspicuous. |
| facets | Kind; Value; Annotation. enumeration; visually conspicuous. enumeration; not visually conspicuous. |
| annotation | documentation. Relating to a features ability to be clearly seen and identified. |