IALA Guideline No. XXXX

on

Light Sources with Integrated Power Systems

(Solar LED Lanterns)

Edition 1

December 2007

AISM Association of Internationale de Signalisation Maritime  IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone +33 1 34 51 70 0 Telefax +33 1 34 51 82 05

E-mail - iala-aism@wanadoo.fr      Internet -  http://iala-aism.org

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

Table of Contents

1	SUMMARY	4

2	OVERVIEW	4

3	INSIDE A SOLAR LED LANTERN	4

4	SOLAR PANELS	5

5	BATTERIES	6

6	LEDs	6

7	LENS / REFLECTOR	7

8	ELECTRONIC MODULE	7

9	CONSTRUCTION	8

10	VENTING	8

11	AUTONOMY	8

12	FLASH SEQUENCES	9

13	INTENSITY AND RANGE	9

14	PRODUCT SPECIFICATIONS	9

14.1	Light Output	9

14.2	Range	10

14.3	Vertical Beam Divergence	10

14.4	Horizontal Output	10

14.5	Autonomy	10

14.6	Latitude Range	10

14.7	Temperature Range	11

14.8	On/Off Level	11

14.9	Illumination Source	11

14.10	Chromaticity	11

14.11	Flash Sequences	11

14.12	Power Management	11

14.13	Additional Important Specifications	11

15	SIMULATION TOOLS	12

16	MAINTENANCE	12

17	TESTING METHODS	12

IALA GUIDELINE FOR LIGHT SOURCES WITH INTEGRATED POWER SYSTEMS

(SOLAR LED LANTERNS)

# SUMMARY

This guideline provides a general informative overview of Integrated Power System (Solar LED lantern) technology, construction and operation.

# OVERVIEW

Solar LED lanterns have a number of advantages.  They are typically smaller, incorporate modern technologies, are durable, reliable, cost-effective, completely self-contained and require no external power source.  Advances in light emitting diode (LED), photovoltaic (Solar Panels) and battery technology create an effective and complementary combination of these three components for modern Solar LED lanterns.  In order to operate efficiently, the design of these lanterns must account for a wide range of solar conditions (i.e. sunlight available to charge the lantern) while maintaining a specified optical output over the expected operating lifetime.

# INSIDE A SOLAR LED LANTERN

A lantern is composed of six key components (See figure 1):

1) Solar Panels to capture light and convert into electricity

2) Rechargeable batteries to store the solar panel electricity

3) LED to convert the stored electricity into bright light output

4) Lens to direct and optionally colour the light output

5) Electronic control to manage the charging/storage/light process

6) Sealed durable housing to enclose and protect the components

Figure 1:  Cutaway of a marine solar LED lantern

Photo courtesy of Carmanah Technologies Corp.

The apparent simplicity of a solar LED lantern is, however, deceptive.  Each part of a lantern’s construction undergoes extensive testing, qualifying and engineering to ensure it is optimally matched with the design to provide the best performance and maximum lifetime possible.

# SOLAR PANELS

Solar panels are made of special semi-conductor materials such as crystalline silicon arranged in “cells” that generate a voltage when exposed to light.  The energy of incoming light in the form of “photons” is used to promote electrons into a higher energy state from which they will flow as current  through the circuit back to their original lower energy state.  Cells will have different efficiencies (ability to convert light into electricity) depending on how the atoms of the material are arranged.   Mono-crystalline cells are 14% to 17% efficient; poly-crystalline are 12% to 14% efficient while amorphous cells are around 5% to 7% efficient.  As the only source of power for the solar LED lantern, optimal performance and design of the solar panel is very important.  For example, by their nature the output voltage of solar panels declines as temperature rises.  Better designs accommodate for this behaviour by providing additional cells to provide elevated voltage output under normal conditions so that adequate voltage is available in hotter conditions.  Another element of design is the solar panel’s orientation.  A solar panel generates the highest voltage when its face is pointed directly at the sun.  Some manufacturers tilt their solar panels for this reason, while others may opt for flat configurations.  If a solar panel is obscured for some reason, whether because of buoy metalwork, bird droppings, bird deterrents, debris or other causes, its performance will be degraded.

# BATTERIES

Rechargeable batteries are a critical component of solar LED lanterns.  There are several different technologies for rechargeable batteries including Lead Acid, Nickel-Cadmium, Nickel-Metal-Hydride and Lithium-Ion.  Each storage technology has different performance parameters such as $/Watt-hour, cycle life (number of complete discharge/recharges possible), operating temperatures, charge methods, self discharge rates, and charging efficiencies.  The choice of which battery type to use is part of a manufacturer’s overall design to optimize performance, lifetime and durability.  Lead-Acid is one of the more popular choices for manufacturers because of its wide operating temperature range and charging/discharging performance.  Batteries can be sensitive to impact shock and a good design will employ effective mounting hardware and padding to minimize those effects.  Of all the components of a solar lantern, after a number of years it is the batteries that will likely fail first.  Manufacturers will therefore design for easy battery replacement with a minimum of disruption to the sealing of the lantern.

# LEDs

Light Emitting Diodes (LEDs) are another type of semi-conductor device, but in a manner opposite to solar cells, LEDs emit light when a current runs through them.  Electrons flowing through the LED drop in energy, releasing that energy as “photons” – particles of light.  The most important feature of LEDs is their efficiency at converting electricity to light unlike traditional incandescent lights that convert most of their electricity into heat.  It is this efficiency that makes solar LED lanterns, with their strict and lean power reserves, possible.  A second major feature of LEDs is their long lifetimes.  Incandescent lights, experiencing heat cycling and having fragile filaments, have short life spans compared to solid-state LEDs which can often exceed 100,000 hours of life.  A long lifetime translates into higher reliability and fewer servicing visits for solar LED lanterns.  LEDs can be manufactured from a wide variety and mixture of elemental materials such as silicon, gallium, indium and nitrogen.  Different combinations are used to produce different colours of light, and filtering is usually not needed to achieve a desired colour.  LED light has the advantage of chromatic purity where the colour peaks narrowly at a single frequency of light.  Solar LED lantern manufacturers often obtain their LEDs from the same manufacturer, and therefore generally see equivalent LED output performance for brightness and visibility range.  Even though they are a product of modern manufacturing, the nature of LEDs results in them often having large variances in properties such as flux (light output), forward voltages and colour.  Solar LED lantern manufacturers can apply an additional quality control technique called “binning” to carefully sort and categorize LEDs that are optimally suited for use with each other.  Electrical power ratings for LEDs vary from a few milliwatts up to 5W.  Depending on the power of LEDs used, lanterns may have many LEDs of lower brightness or a few ultra-bright LEDs.  They can be arranged into groups or arrays to improve intensity and light source geometry.  When used in groups or arrays, wiring arrangements can vary considerably, but are typically series/parallel.  In the unlikely event of an LED failure, the arrangement will have consequences on the number of LEDs that may not operate.  Contrary to popular opinion, LEDs do emit heat, and it is an important consideration for high-intensity LED design.  The heat in an LED is confined to the junction - the meeting point of dissimilar materials inside an LED semi-conductor where the light is emitted from.  Hotter junction temperatures result in lowered brightness for LEDs and even shifting colour.  Good design will there employ effective heat conduction techniques including used metal-based circuit boards to conduct heat away from the LEDs better than conventional boards.

# LENS / REFLECTOR

Like incandescent lanterns, Solar LED lanterns employ a lens and/or reflector to shape the output of the light into a desired profile.  Many lanterns use off-the-shelf or traditional Fresnel-type lenses which have been modified for LED use.  Some newer versions of lens/reflector technology shape the beam by reflection off engineered surfaces.  The primary purpose of a well-designed lens/reflector arrangement is to precisely control beam vertical divergence (e.g. +/- 3.5 degrees above/below horizontal) and azimuthal consistency over 360 degrees horizontally with minimal effect on the beams brightness or colour.  Multi-LED systems having discretely placed LEDs arranged in a circle will have intensity peaks or nodes aligned with the LEDs, and a good lens design will help smooth the intensity profile.  All lenses must be UV (ultraviolet)-resistant as the haze created by UV degradation of the lens would create unacceptable reduction in beam brightness.

# ELECTRONIC MODULE

The single most important part of a solar LED lantern that makes it more than a simple assembly of components is an intelligent electronic control module.  While implementations of this vary among manufacturers, the electronic controller of most modern lanterns will offer the following functions:

- Power consumption monitoring

- Automatic light control to reduce intensity when low battery charges are sensed

- Intelligent monitoring and charging of batteries at the most effective voltages and currents

- Monitor and meter solar panel voltages

- Detect and respond to changing ambient lux levels using either photo sensors or interpretation of solar panel voltages

- Modulate current to the LEDs to provide variable intensity

- Respond to programming requests using a variety of methods including infrared programmers, waterproof switches or rotary/DIP switches on the electronic module

- Report back to users on internal statistics such as battery charge state

- Activate sleep or ship modes that conserve battery power while retaining programming information

- Output any of more than 200 flash sequences including steady on

- Power width modulation of LED duty cycle to control power consumption, range and autonomy values

- Minimization of quiescent (baseline between flashes) current

The main algorithm that an electronic module will need to satisfy is to ensure that, over a suitable period of time, energy in (from the solar panel) equals energy out (as LED light and circuit losses).  When the equation is not met, the electronic module must react to manage the lantern’s resources to ensure continued operation at acceptable levels.

# CONSTRUCTION

The marine environment can be very hard on lanterns, and it’s important that they be of strong construction and resistant to damage by passing ships, accidental sinking and other mishaps.  Plastic construction has an advantage of lower manufacturing cost over machined, cast, or extruded metal but at the expense of ruggedness.  Plastics, like lenses, may suffer cracking and brittleness by prolonged exposure to UV light.  Except for venting (covered later), solar LED lanterns will need to be sealed and resistant to water ingress according to standards such as IP67.

# VENTING

As a result of the chemical processes occurring during charging, rechargeable batteries can generate flammable hydrogen gas.  Solar LED lanterns should be properly vented to ensure the hydrogen gas can exit the lantern before accumulating to excessive levels.  There are usually at least two vents in a lantern – one near the top and one near the bottom – to allow convection currents to flow through the housing and draw in fresh air.  The vent holes can be waterproofed with materials such as gortex to allow efficient gas transport while denying the ingress of water.  Vents should be placed in such a way that mounting the lantern to a structure does not block them.

# AUTONOMY

A critical performance factor, directly related to battery capacity, is the autonomy of a solar LED lantern.  Autonomy is the number of nights the light functions without any solar charging.  It is an important parameter in environments where prolonged cloud cover and rain can lead to the lantern drawing upon its battery resources instead of having sunlight available for operation.  An estimate for autonomy can be calculated using the amp-hours rating of the batteries and the lantern’s nightly amp-hour draw.  For example, if the batteries have a fully charged capacity of 50 amp-hours, and the solar LED lantern consumes 2 amp-hours per night, then the rated autonomy is 50 ÷ 2 = 25 nights.  In real operation, other factors such as ambient temperature and battery degradation over time due to charge cycling are also considerations in autonomy calculations.  Differing flash sequence duty cycles will draw different currents and lead to autonomy variations within a single lantern.  Electronic modules may be capable of creating consistent autonomy across different flash sequences through modulating current to the LEDs.

# FLASH SEQUENCES

Solar LED lanterns can usually be programmed for over 200 different flash sequences in use around the world.  Methods for programming vary between manufacturers.  Some manufacturers will offer programming of flash sequences at the factory prior to shipment.

# INTENSITY AND RANGE

Perhaps the single-most important quality of a solar LED lantern is the intensity of the light and the resulting range at which it can be seen.  Without concern for energy management, LEDs could be driven at their maximum currents for maximum light output.  The art of solar LED lantern design, however, must balance the current of LEDs (and hence the intensity and range) with power consumption and battery charge levels.  In practice, the electronic module may be able to finely control and modulate LED currents to ensure the power consumption can be matched by the solar panels’ charging of the batteries while still maintaining its performance as a lantern with a particular range in nautical miles.  Solar LED lanterns should be capable of adjusting their intensity both by human operator and by the internal electronics in response to changing solar light and battery charge conditions.  Similar to the method for creating consistency of autonomy across various flash sequence duty cycles, another design approach is to engineer the lantern so that its range is consistent across different flash sequences and pulse durations.  Due to a human perceptual effect, there is a non-linear relationship between the durations of light flashes and the apparent intensity to an observer.  Intelligent electronic control can account for these differences by employing Schmidt-Clausen equations and ensure that any particular lantern will have the same range (i.e. a given intensity at a given distance) for all flash sequences.

# PRODUCT SPECIFICATIONS

Every manufacturer of solar LED lanterns will condense their products performance into a concise set of specifications for publication as specification sheets and will include some or all of (exact terminology will vary):

## Light Output

Expressed as candela, this is a measure of the lantern’s brightness and is often indicated as “Effective Intensity”.  It is “effective” because it is usually stated for a particular flash sequence and, as discussed in Intensity/Range above, flashing lights appear less intense to the human eye than at steady intensity.  Output measurements are taken over 360-degrees horizontally and determined using IALA criteria that the IALA rating is that value at which 90% or more of the data points meet or exceed the value.  There is a general trend among solar LED lantern manufacturers to better express light output by stating performance in intensity (candela) rather than range (nautical miles). Some manufacturers combine the performance of their colours into one candela specification that, by including the lowest performer (usually amber), de-rates the other colours.  Colour-by-colour candela specifications are more informative.

## Range

Expressed in nautical miles, range is calculated by Allard’s Law - a function of candela output, atmospheric transmissivity and illuminance threshold for the human eye.

## Vertical Beam Divergence

Divergence measures the vertical containment of the lantern’s beam by tilting a lantern up and down through a range of values around horizontal.  The beam width is determined by Full Width Half Maximum or FWHM – the points at which the beam intensity falls to one half of the intensity at its peak (which would correspond with horizontal).

## Horizontal Output

Usually “360 degrees”, this is simply the azimuthal range over which horizontal output is measured while rotating the lantern.

## Autonomy

Expressed in either hours or days, this is the theoretical amount of time the solar LED lantern can continue to operate normally without any solar energy input (starting from a full battery charge).

## Latitude Range

Expressed as a range of degrees latitude (e.g. +50° to -50°), this is a rule-of-thumb range in which the solar LED lantern can be expected to operate normally.  In actual practice, local insolation (sunlight) data is required to conclusively determine whether a lantern will have continuous normal operation.

## Temperature Range

Expressed as a temperature range in Celsius and Fahrenheit within which the lantern is capable of operating normally or possibly at minimal function.  Solar panel, battery, LED and electronic performance all degrade to varying degrees with high temperatures.

## On/Off Level

Expressed in lux, this is the ambient light levels at which the lantern will turn on or off.  Manufacturers will frequently use solar panel voltages to estimate lux levels, while advanced designs will use dedicated photosensors.

## Illumination Source

A brief description of the number, type and possibly the arrangement of LEDs providing the optical output of the lantern.

## Chromaticity

Usually expressed as being within certain IALA general or preferred regions of the CIE colour chart.  Chromaticity is measured with a spectrometer and expressed as x and y co-ordinates on the CIE colour chart.  The x and y coordinates are then mapped into IALA regions on the chart.

## Flash Sequences

Expressed as the number of flash sequences the lantern can be programmed for.  This should include a steady state.  Some manufacturers offer custom flash sequences.

## Power Management

Provides information on the electronic control module and its methods/technology for managing the lantern’s power balance (to maintain energy in = energy out)

## Additional Important Specifications

- Solar Panels – type, number, arrangement, wattage, UV protection

- Battery – type, number, amp-hours capacity, brand

- Lens Material – UV protection

- Battery venting – number, locations, type of seal (e.g. gortex)

- Sealing – type of seals, waterproof standards (e.g. IP67, Nema 6)

- Weight – kilograms and pounds

- Construction – materials and grade

- Mounting – number and bolt patterns

- Lifetimes - serviceable lifetimes in years

- Environmental – compliance with environmental protections standards such as RoHS and WEEE

- Weight

- Dimensions

- Vibration and Shock

- EMC/ESD - electromagnetic interference from sources such as VHF/UHF antennae and electric discharge from lighting (25kVolts)

- Resistance to icing, wind, salt spray

# SIMULATION TOOLS

The performance of a solar LED lantern is affected by many factors.  Manufacturers will usually create custom solar simulation tools to aid in the determination of whether a particular lantern will work in an anticipated location.  Inputs to the simulator will include such things as location (latitude, longitude), solar panel specifications (current, voltage), and battery storage capacity (amp-hours), energy consumed by the light, flash sequences, output (candela) and the solar radiation data for the selected location.  Comprehensive solar radiation data is available from a number of online sources including NASA.  Depending on the level of conservatism desired in the simulation, historical averages and minimums allow the designer to establish safe and reasonable specification limits.  Simulations can be done in less than 15 minutes, and it’s therefore good practice to have one done prior to purchasing and deploying lanterns in a particular location.

# MAINTENANCE

While solar LED lanterns are designed as integrated units with a goal to be maintenance-free, certain maintenance events such as battery replacement every 5 years or so may be necessary.  Manufacturers will have different approaches in their designs and methods.  Replacement gaskets should always be included when the sealing on a lantern needs to be broken for maintenance.  Wiring connectors should only connect one way, and the system should be protected against reverse polarity if accidental battery voltage reversal does occur.  No special tool sets should be required, and the design should use simple conveniences such as battery handles.  The plastic or polycarbonate construction of the lenses, body and other parts should not require special cleaning materials and should be chemically inert as much as possible.

# TESTING METHODS

Each manufacturer of solar LED lanterns must, at some regular interval in their manufacturing, sample production units for performance and quality testing.  The testing may be outsourced to external service providers or done in house.  Whether in-house or outsourced, manufacturers should have their products tested in a standards-based (e.g. IALA) testing facility with rigorous, regular testing of lanterns to ensure performance and quality compliance.  As with any manufacturing process, the application of modern statistical process control ensures consistency in the finished lanterns.  Statistical tools such as adequate sample sizing, Gage Repeatability and Reproducibility (GRR) and Cpk should be used.  Appropriate sample selection is critical to obtaining a statistically-relevant population of lanterns whose test results may be considered representative of the population as a whole.  An important point to make about testing intensity is the measured intensity of the lantern will be higher if measured while flashing than if measured while in a steady-on state.  Some manufacturers set their lanterns into a steady, unmodulated state while measuring candela.  This method is experimentally convenient, but because of the unnecessary extra heating of the LED junction and reduced light output and colour shift because of the heat, the manufacturer may measure a result lower than would have resulted with a flashing sequence (which would run cooler).

***********************