MOORINGS

The proposal is to provide a new guideline on moorings for floating aids combining the content of  ‘Practical notes on the use of mooring chains for floating aids to navigation’ 06/1989, ‘Recommendation on the design of normal moorings’ E-107 and ‘Guideline on synthetic mooring lines ‘1024.

A large part of the text is available from the existing papers and I anticipate that new information on composite moorings will emerge from the workshop. However a number of new illustrations will be required to produce a document of suitable quality.

The following notes provide the chapter headings and some notes on their content.

--------------------------------

DRAFT GUIDELINE ON MOORINGS

Their component parts, design and maintenance.

1.  INTRODUCTION

Utilise ‘general’ sections from E-107 and 06/1989 to describe the function of the mooring in maintaining a floating AtoN on station and then explain when it may be advantageous to use chain, rope or rubber cord as the main component of the mooring.

- MOORING COMPONENTS

Utilise relevant parts of the three papers to describe the components of the mooring (chain, rope, shackles, swivels, sinkers etc) and to explain the terminology used to describe the sections of the mooring (bridle, thrash, ground chain etc).

The importance of component specification in mooring life and maintenance planning could be included here.

I propose that details of chain proportions, shackle sizes and types of steel used for chain and other mooring components should be included as an appendix.

- MOORING DESIGN

Explain the principles of mooring design generally as per. Recommendation E-107 ie. that the mooring must resist the combination of wind, wave and water flow on the buoy and the submerged components of the mooring and also include the effects that the static and dynamic forces from the mooring will have on the buoy.

The details of how to calculate the loads in a chain catenary mooring and a composite rope/chain mooring could form an appendix and I believe there is information from the Netherlands on measured mooring loads in shallow water buoy moorings.

- MOORING MAINTENANCE

This could be based on the short section from Recommendation E-107 with additional information on handling rope and rubber cord moorings.