EE9 9 Action Items for the IALA Secretariat

(EEP10/02/01)

Author: Sabina Schollmeier

Date: 04.10.07

Item 44 and 58: surface colours

At the end of 2006 the TTC defined new regions for surface colours basically used on buoys. The result is that we don’t use and recommend any RAL colours for red and green. There is only RAL number we apply: RAL 1023 (“traffic yellow”). Especially for the use of ordinary colours we emphasize the use of very bright (high luminance factor) and saturated colours to archive a high distance of recognition.

Further information can be found in the “Letter to the Group: IALA EEP Surface Colour “point 6. In this document you will also find more about the steps of perception (point 2). It has already been delivered to all members of the “surface colour group” in December 2006.

As an example we present a colour fan with fluorescent and preferred ordinary colours with a higher luminance factor for red and green. These colours meet our new specification. The colour-coordinates and the luminance factor can be found on the backside of each plate.

Review:

The Federal Ministry of Transport, Building and Urban Affairs entitled us to standardise the specifications for surface colours, to establish a method for testing colours and to publish and recommend products. In a first step we determined new regions and luminance factors for surface colours used on buoys for a high recognisablitity. Our next steps will be the determination of colours regions for plastic, retroreflective sheeting and signs.

It should be mentioned that the measurements and the approval of colour products by the TTC are obligatory. A list of approved colour products is available.

(Remark: There are only one or two RAL oder British Standard Colours that meet die IALA specification E 108 but because of there poor recognisability we don’t want to recommend them for buoys.)

Item 52: Experience of sequencing and synchronising lights

In September 2006 the TTC discussed with the local office of Bremerhaven several ideas to increase the conspicuity of the lighted buoys in the approach of Bremerhaven. This was necessary because a new container terminal causes high background lighting and therefore the lighted buoys could hardly be seen against the relatively bright background.

Situation:

In the approach of Bremerhaven 6 pairs of lighted buoys are concerned. The distance of the lighted buoys is between 1.5 km and 2 km. All of them are in pairs and they have three different characters due to different meaning and their position.

In Germany lateral marks are allowed to use only selected characters according to a national regulation (Erlass vom 24.11.77):

General lateral marks: fl (flash) or oc (occulting)

Lateral mark to show “change of course”: q (quick) 0.5+ (0.5)

The time schedule of the characters for each pair of buoys is shown in fig.1.

Proposals:

Different proposals according the character of the buoys were made:

oc 4s: 3 + (1)), fl. 4s: (1 + (3) and iso 4s: (2 + (2). To install a “runway effect” each buoy should get the same character. These ideas could be simulated by a program on screen.

Result:

According to the demands of the marines (position identification by flash character) it was decided that all pairs of buoys shall be kept with their own characters.

- Due to the relatively great distances between the buoys and the weather condition (light haze) during the test period only the next pair was visible. The situation that you can observe all 6 buoys pairs at the same time can be simulated on the screen only.

- According to this experience that only one pair can be seen no further experiments were made. A “runway light” as used on airports was not examined and would bring no advantage in this case.

- All lights shall be synchronized: they start at the same time (“point of synchronisation” in fig.1) with their characters so that you see the next pair of buoys switching on and off at the same time.

Summary:

In this application the most important result is that you get the best improvement by synchronizing a pair of buoys. There was nearly no benefit of synchronizing a whole chain of buoys.

Remark:

Visual aids like buoys and the GPS system are considered to be redundant to each other. In case of a synchronisation by GPS a certain dependency arises so that in case of a failure of the GPS system the advantage of the synchronisation is lost.

This is especially remarkable for leading lights.

Q : 0,5s + (0,5)s

Oc(2) 9s : (1)s + 2s + (1)s + 5s

Fl(2) 9s : 1s + (2)s + 1s + (5)s

Pair: 41/42

Pair: 43/44

Pair: 45/46

Pair: 47/48

Pair: 49/50

Pair: 51/52

Oc(2) 9s : (1)s + 2s + (1)s + 5s

Fl(2) 9s : 1s + (2)s + 1s + (5)s

Q : 0,5s + (0,5)s

Point of Synchronization

Fig. 1: Time schedule for the pairs of buoys