HISTORY OF FLOATING AIDS TO NAVIGATION

THE FIRST BUOYS

I have been unable to find a definite starting point for the use of floating Aids to Navigation.

There were probably some forms of floating marks moored to navigational hazards far back in history, possibly simply pieces of wood moored by rope to a block of stone. Anchors are recorded by the Greeks c 500 B.C. (Anchors M E Upham) along with mooring chains also Julius Caesar noted that the Veneti tribe in Brittany had iron anchors with iron chains. Both Greek and Roman period anchors had attachment points in the crown for a cable to connect to a marker buoy.

The first mention of navigation buoys in Europe was in Spain, La Compasso de Navigare, sailing directions of 1295 for the river leading to Seville.

(Noted in historic report to USCG, also UK book. Can Spanish service help with more detail?)

The buoys being somewhere near Chipiona. There is no record of the construction of these buoys or their moorings.

German book illustrations. 1585.  1568.  1675. 1751

Sea buoys are referred to in Texel in 1358 and small illustrations of buoys appear on charts around 1500.

More detailed illustrations of individual buoys are found in documents from 1585, 1625, 1751 and 1810. These illustrations show a common form of conical buoy made from tapered wooden staves and bound with iron hoops. The mooring being attached to the point of the cone and the flat, circular end being the top of the buoy.

This type of iron bound wooden construction is similar to that used for making barrels and is in English , called cooperage.

Some buoys were similar in form to conventional barrels but they seem to have been in the minority.

The early charters of Trinity House refer to 'seamarks buoyage and beaconage' ( Elizabeth 1  1621 )  and references are made to buoys and mooring chains.

1800's  wooden buoys

Illustrations of wooden buoys shows the basic conical form being used for more than 300 years.

German bi-conical buoy

These buoys were also constructed in a bi-conical form presumably to provide a better daymark. The technology of wooden buoy construction seems to have been more developed in Holland and Germany. There are records of the Trinity House of Hull ( UK ) buying buoys from Holland in 1621 and Hamburg in 1682. It would be interesting to know why the considerable expense was justified to import what seem to be very simple products given that the construction of wooden barrels from curved staves was widespread.

'A Sea Grammar' 1627 Cptn John Smith, a compendium of sea terms, refers to buoys (then spelt boyes) as being short pieces of wood used to mark the position of anchors or 'can boyes' that were much larger and moored 'to give mariners warning of dangers'. The term 'can buoys' is confusing as illustrations show that this term was used to describe the conical wooden buoys then in common use.

MOORING

German buoy/mooring

1585

Buoys were moored with specially made square stone sinkers or with old, worn mill stones. Some were moored with rope, that must have had a very short service life, but by the 17th century many were moored with chain. This is 250 years before chain came into general use for mooring ships. German illustrations show buoy, sinker and chain in 1585.

The chain used was very different to modern chain. It was of course entirely hand forged and the individual links were generally very long. In some cases the links were up to 0.5 m long and in the form of rods with an eye at each end, rather than oval links (UK). There is a  German reference to 0.3 m to 0.5 m chain links made from 22mm diameter bar.

Sinker/chain

Long links provide faster and hence cheaper construction for a given length of chain and reduce the number of welded joints that were the weakest areas of the chain.

LIGHTVESSELS

Nore lightvessel picture from chart

This is an inset from a 1732 chart that illustrates the first lightvessel at the 'Nore' station in the Thames estuary (UK). Sea trade was increasing and a lighted aid was required to enable ships to establish their position at night in the featureless estuary of the Thames. Personnel were required to tend the lights and so a small fishing vessel was converted to provide this first lighted seamark.

A German website ( Feurship) notes that vessels had been deployed before this time with some form of light on deck to mark harbour entrances but not permanently moored as sea marks.

Buoys on Nore chart

This chart was produced one year after the light vessel was established. The chart also shows several conical buoys.

Nore  Lightvessel model

This is believed to have been an accurate model of the original 'Nore' vessel that set a large flag as a daymark and candle lanterns at the end of the yards at night. The crew tended the rope moorings and sailed the vessel when the mooring failed.

Lightvessels increased in number as their effectiveness became apparent. Their AtoN technology generally followed that used in lighthouses. ie light and fog signals.

It is interesting that in the account of the building of the Eddystone lighthouse by Smeaton in 1758, he particularly notes that their use of chain moorings for securing their support vessel was special and unusual at this time. The chain was made for this mooring by a Mr. Wilson of Blackwall (London). The difficulties with handling this chain, which had 6 inch long links, on board ship, is described in some detail. Trinity House had one spare chain mooring at this time, presumably a lightvessel mooring which they loaned to Smeaton. He mentions that the Eddystone mooring support buoy was made of layers of cork and that swivels were incorporated in the moorings.

THE DEVELOPMENT OF BUOYS

Moving ahead to the 19th Century, commercial ships are larger, deeper draft and faster. There is the need to identify the deep-water channels and to navigate restricted waterways at night and in poor visibility. Buoys have become larger and more numerous. Trinity House had 40 buoys in 1796 and 400 by 1860! Lattice top marks and flags are in use as daymarks in the early 1800s but they are carried on what are essentially the same wooden buoys as were in use 200 years before.

Note: Netherlands book on buoys - J.A.Van Wallenburg.  ALs't diep verloopt versetmen de bakens : de ontwikkeling van tonnen en boeien door de eeuwen heen.    Can Sipke obtain a copy?

German conical buoy

Double skin and steel banding

As wooden buoys were made larger they had to be made from increasingly thick timber to be sufficiently strong and hence became heavier. Illustrations from this period suggest that some wooden buoys had two layers of external planking and had an internal bulkhead, presumably to prevent the buoy sinking if minor leakage occurred. The shapes that could be made by this cooperage process were also very limited. As we move into the 1800’s the technology of manufacturing iron storage vessels and iron ships hulls developed at a rapid pace and this technology was soon used to make iron buoys.

TH Buoy depot picture. Many wooden buoys

The fist riveted iron buoys followed the shape of wooden buoys  but they would have been lighter and hence could support more mooring weight. In the UK the first iron buoy was supplied to Trinity House by Lenox in 1845. The ability of iron to be worked into many complex shapes allowed a variety of buoy shapes to be developed that in some cases included the daymark shape in the buoy body and allowed the buoy to be sub-divided with bulkheads to maintain buoyancy in case of collision damage.

TH BUOY Drawing    1860?

This drawing shows a variety of buoy types in use by Trinity House in the mid 19th Century when Britain seems to have been at the forefront of developments in buoys. Note that a bell buoy is included. These quickly came in to use as they provided the only means of providing warnings to mariners in conditions of restricted visibility.

Bell buoy

The hemispherical hull in this illustration would increase the rolling motion of the buoy to maximise bell action.

There had been a visit to TH by the US service in 1845 ( USCG report). It was particularly noted that  standard  shapes and colours were used for buoys throughout the UK. (apparently not the case in the US) . The TH buoys remained on station usually for 6 months. They were moored to stone sinkers with chain moorings and ex-lightvessel chain was often used. We can surmise that lightvessels were then usually moored with chain.

There was a French lighthouse service visit to Trinity House in 1855 to inspect developments in iron buoys and this resulted in the French Service developing a standard iron service buoy and a bellbuoy.

2 x French buoy booklet pictures

In the US Mr Courtney (an English émigré) developed the automatic whistle buoy in 1880. The vertical motion of the buoy in the water drives air through a central tube to operate a whistle. This principle was rapidly developed and is still in worldwide use.

Courtney buoy diagram

LIGHTED BUOYS

Lightvessels had developed into substantial vessels with large daymarks. The lanterns were usually kept at deck level during the day and hoisted up the mast at night. Lighting was provided by Argand lamps with individual reflectors that were positioned around the mast and individually gimballed. The 19th Century had seen many developments in artificial lighting using vegetable and mineral oils, gas and electricity but the problems associated with operating any system on a buoy, continually moving around at sea were considerable

Some sophisticated self contained oil (paraffin)  lanterns were developed by Wigham in Ireland and Bourdellsche in France

Drawings In German Book.

These were primarily used on beacons but also on some buoys.

picture  of Wingham lantern on Uk Naval buoy in Portsmouth Harbour

They do not seem to have been used extensively on buoys and would probably only have worked at very sheltered buoy stations.

The first major buoy lighting success was by a German, Julius Pintsch. He had developed a gas lighting system for railway carriages.. A stable gas, referred to as 'oil gas' was distilled from a variety of vegetable and mineral compounds and compressed into steel gas holders. The gas was burned in an open flame burner that would have produced a rather yellow flame. 52,000  rail carriage lighting units had been produced by 1893 and had been marketed around the world.

The Pintsch buoy had the 'oil gas'  stored in the  buoy body at a pressure of 6 atmospheres. The first of these lighted buoys were produced in 1877 and were used in the waterways leading to St Petersburg, for which Mr Pintsch received the award of the order of Stanislaus from the Tsar. By 1880  Pintsch buoys were in use in England in the Thames estuary.

The Pintsch Engineering Co. utilised spherical steel buoy bodies developed from mines they had produced in the Franco Prussian War ( Pintsch Archive) and  they incorporated these into a high focal plane tail tube buoy.

Patent drawing of first Pintsch buoy

Pintsch buoy    Photo Pintsch archive

Surprisingly in 1880 electrically lighted buoys were installed in New York harbour. These were spar buoys with ‘Swan’ filament lamps, only 2 years after the filament lamp had been invented. Coloured filters were used to identify individual buoys. Initially a DC supply was used.

Note: USCG report? Book

However the voltage loss in over 6 miles of cable that delivered the electrical power to the buoys must have caused serious problems. The system was changed to AC in 1895 but  the buoys were changed to more reliable gas lights in 1904. A similar system was trialled on the river Jade in Germany in 1896.

US buoy photo

German buoy drawing

This used 500V AC underwater supply cable with a transformer on the buoy supplying a lantern with a main and standby lamp. Discontinued in 1901 to be replaced by more reliable gas buoys.

An electric buoy was patented in England in 1903 see Pat. 26249 with drawing. but there is no record of being used in service.

Electric buoy patent picture

This illustration shows I believe a Pints gas buoy in the river Thames that had a 30 day service period.

Pintsch buoy on Thames

These Pintsch buoys were soon in use around the world supported by the network of gas distillation plants that Pintsch had established for their railway lighting equipment.. A clockwork occulting mechanism was introduced in 1883, which reduced gas consumption and gave a 3-month service life.

Note: Any details of lanterns, flashers etc?

Services around the world produced their own "oil gas" under licence from Pintsch. These included the UK, FRANCE, USA .The process was based on high temperature distillation from crude oil or oil bearing shale in iron retorts heated in coal fired furnaces.

Note: Details of TH plant in 1911 are available.

Acetylene gas was discovered in 1892. It has the particular advantage that with a correctly designed burner it can burn in air to produce a white light. Burning equal quantities of gas the acetylene flame will provide a light of approximately 7x the intensity of an open flame oil gas light. Acetylene is capable of operating very fast flash times and hence gas consumption is greatly reduced when compared with oil gas. However acetylene is extremely explosive and liable to detonation in widely varying ratios of acetylene to air. Initially there was no practical means of safely storing acetylene.

The Wilson buoy from 1900 attempted to overcome these storage problems by having an on-board acetylene production plant. The acetylene was produced by the reaction of calcium carbide and water within the buoy. These buoys were introduced by some services, particularly US and Canada. But there were several disastrous explosions when the buoys were being serviced and operational problems due to the water freezing in cold climates.

Drawing of Wilson buoy

Photo of Wilson buoy

Major advances were made in the early 1900’s by the AGA company in Sweden and in particular by the engineer Gustav Dalen in the safe storage of acetylene. The acetylene gas was dissolved in acetone which was then absorbed in a porous solid within a steel cylinder . Dalen also developed complex, but highly reliable gas flashers. The flasher mechanically reduced and regulated the gas pressure and fed pulses of gas to the burner so that the required flashing character can be produced.

Original AGA buoy drawing

The gas storage and the gas flashers provided the means to operate buoys and remote beacons, unattended usually for at least a year. These became the industry standard in many parts of the world and some are still in use today.

A selection of AGA buoy pictures

AGA BUOY LANTERN

This type of gas lantern remained in use for nearly 100years. Gustav Dalen received the Nobel Prize for Physics in 1912 for his contribution to the safety of navigation.

AGA buoy lantern

Some countries continued to use oil gas until the 1930’s with Authorities having their own gas distillation plants. As the production of various petroleum based gasses (LPG) developed oil gas was gradually replaced with propane or butane that could be stored in liquid form at moderate pressure. Petroleum gas lanterns and flashers

Picture German propane buoy 1937

developed in similar ways to acetylene lanterns and incandescent mantles were introduced to improve intensity and provide a white light. The world’s lighthouse services divided between the use of acetylene or propane/butane  as the gas for their buoys for the next 70 years or so.

Picture of 1912 buoys

This drawing is from the 1912 International Navigation Congress in Philadelphia where buoy lighting with various types of gas was a major topic. The buoys are essentially similar to tail tube buoys still in use. Note that one of the buoys has an under-water bell. This is an example of an almost forgotten area of floataid technology.

Wyllie illustration of leadsman

We forget that the fast steam vessels in the early 20th century had no on-board navigation aids beside their compass, sextant, chronometer and leadline.

The picture shows a leadsman sounding the water depth on a battleship in 1904. Some assistance was provided with the introduction of submarine bells. These take advantage of the fact that sounds travels much faster in water than it does in air and the reflection and refraction problems that make  sound propagation in air so uncertain have little effect on propagation in water. Large ships were equipped with underwater listening devices (hydrophone) that could detect underwater bells at a range of 10 miles and make a good estimate of the direction of the bell.

Buoy bells were operated by the rolling motion of the buoy and were thus dependant on sea state. Lightvessels had bells that were mechanically operated and later electrical oscillators. These could be synchronised  with the lightvessel’s fog signal (whistle, siren or diaphone). The approaching ship could then note the time difference between the reception of the airborne signal and the under-water signal and be able to accurately calculate, for the first time, their distance from the lightvessel. This principal was further developed when the underwater fog signals were synchronised with the first radio beacons.

Note: Dates

Underwater fog bell

Lightvessel designs were refined until in the 1930’s they had rotating lights, diesel generators, compressed air fog signals and radio beacons. The major difference that existed from one country to another was that some Authorities had self- propelled vessels and some had vessels with no propulsion machinery that had to be towed to station. Authorities with many floating AtoN needed a considerable fleet of support vessels to tow lightvessels to and from station, change crews over and to lay and re-gas buoys.

Picture of Lanby

The next major advance in floating AtoN technology took place in the 1970’s when the US Coastguard made the brave step of replacing manned lightvessels with automatically operated very large buoys. These LNB’s were 13m diameter very shallow hulls with 13m towers. They were developed from a meteorological buoy designed for open ocean operation. The hull contained fuel tanks, two long running diesel alternators to power the light and fog signal and a very large cabinet of electrical control equipment and radio systems. The original US buoys and those built later in the UK were extremely unreliable and very difficult to maintain.

The engineering effort that went into solving the problems highlighted by the first LNB’s eventually established confidence in the operation of remotely operated diesel power systems, remote control and monitoring systems and the unattended moorings of the LNB and lead to the steady automation of the manned lightvessel fleet and the offshore lighthouses.

TH tender picture with buoys and Light Vessel

In 1990 we see a tender removing one automatic lightvessel from station and about to replace it with a newly refitted vessel. Note that the buoys on deck have no lanterns fitted. The gas lanterns will be operating in the lantern test space on the foredeck to ensure their characters are correct and will only be fitted to their buoys just before laying. The sea time of the tender fleet will have been substantially reduced as they no longer have to carry crews and their supplies to lightvessels and offshore lighthouses as many of these have been automated  but they are still re-gassing buoys and  re-fuelling  the lightvessels and maintaining moorings.

ELECRIC BUOY LIGHTS

Some Services changed from gas buoy lights to primary battery powered electric lights in the 1950’s and 60’s. These required the regular replacement of primary battery packs. The small filament lamps, usually carried in automatic lamp changers were never entirely reliable at exposed buoy stations due to

Picture of US multi lamp changer?

the inherent fragile nature of the incandescent lamp filament and the problems associated with the electro mechanical lamp changer, moving contacts,vibration, corrosion etc.

Some authorities had used secondary batteries that were regularly recharged by the servicing vessel but these had relatively short service periods and a history of serious accidents due to the accidental ignition of battery gases during the charging process.

SOLAR POWER

The introduction of solar modules to recharge secondary batteries was probably the most significant step in lighted buoy technology. There were progressive solar module developments in the 1980’s and the problems of mounting solar modules, charge regulation and choice of battery type were solved and confidence was gained in the long term operation of solar systems. Many gas buoys and primary battery buoys were then converted to solar power with electric lanterns thus removing the need for ships to change heavy gas bottles or replace packs of primary batteries.

Solar buoy picture

LED LIGHTING

LED picture

Solar power systems soon demonstrated their reliability and the next development was the LED lantern. The LED provides a very efficient conversion of electrical power to light. The LED will last for many years and consequently the need to regularly replace filament lamps, on station has gone. There is the added advantage that the light output of coloured LEDs are equal or better than white LEDs, this is compared with coloured light produced by filtering light from a filament lamp where 30 to 50% of the light is lost in the filter. This has resulted in smaller solar modules being required for a given light output so that for short range buoys the lantern can be combined in one housing with the solar modules and battery.

LED lantern

CONCLUSION

Many buoys are now made from some form of moulded plastic rather than steel, but most are moored in  a similar  way that they were 400 years ago.

The few remaining lightvessels have taken advantage of low powered AtoN technology and have also been converted to solar power. It will be interesting to see how many and what types of floating AtoN will remain at the end of this century.