### Lithium Batteries

Different lithium technologies have different safety features.

#### General advantages

- Low weight (typically 1/3 of Lead acid) and high-energy availability in a small space.  Sealed operation allows operation even under water.

- Fast recharge may allow for PV heavy and battery light solutions

- High life cycle rates (typically 7 times that of lead acid) can offer a long service life for AtoN applications

#### General disadvantages

- Production of dangerous gases with consequent environmental concerns, including explosions, prevents this type of battery from being recommended unless a rigorous safety regime is introduced;

- Disposal of Lithium batteries is extremely difficult and costly however this service may be built into the purchase price.

#### Advances

- The advance of this type of battery is mostly driven by the automotive industry and is moving ahead very quickly. The most likely battery to apply to AtoN will be the Lithium Iron Phosphate (LiFePO4) as developments with this type of cell appear to be producing a cell best suited to AtoN application. This is due to safe operating parameters, low weight and high power density figures.