- EEP 3 / 6e) / 3

- DRAFT 1

IALA GUIDE TO AVAILABILITY AND RELIABILITY

- THEORY AND EXAMPLES

- IALA EEP COMMITTEE

Month 200?

- INTRODUCTION

CONTENTS

1	INTRODUCTION	5

1.1	Background (purpose and scope)	5

1.2	Relationship between reliability and availability	5

2	RELIABILITY OF SYSTEMS	7

2.1	Introduction	7

2.2	Relationship between preventive maintenance and reliability	8

2.3	Blocks in series	9

2.4	Blocks in parallel, passive redundancy	10

2.4.1	Passive redundancy. Without repair	10

2.4.2	Passive redundancy. With repair	10

2.5	Blocks in parallel, active redundancy	11

2.5.1	Active redundancy without repair when only one block has failed	11

2.5.2	Active redundancy with repair of a failed item	11

3	PROOF OF FORMULAS	13

3.1	Blocks in series	13

3.1.1	Passive redundancy without repair	15

3.1.2	Passive redundancy, with repair	16

3.1.3	Active redundancy without repair	17

3.1.4	Active redundancy with repair of a failed item	19

3.1.5	Relationship between preventive maintenance and reliability	21

4	EXAMPLES	22

4.1	Methodology to assess Failure Response Time (FRT) impact on service Availability	22

4.1.1	Computation of failure response time	22

4.1.2	Conclusion	22

4.2	Computation of Mean Time to Repair for selected redundancy and repair policies	23

4.2.1	Detailed computation of MTTR values presented in the above table	24

5	QUALITY MANAGEMENT SYSTEMS AND RELIABILITY	28

5.1	Basic terms	28

5.1.1	Quality	28

5.1.2	Quality assurance	28

5.1.3	Quality control	28

5.1.4	Specification	29

5.2	Drafting specification	29

5.2.1	Target specification	29

5.2.2	Functional specification	29

5.2.3	Product specification	29

5.2.4	Materials specification	30

5.2.5	Process specification	30

5.2.6	Inspection specification	30

5.2.7	Test specification	30

5.2.8	Acceptance specification	30

5.2.9	Installation specification	30

5.2.10	Use specification	30

5.2.11	Maintenance specification	30

5.2.12	Disposal specification	30

5.3	Specification data	31

5.4	Maintenance	31

5.4.1	Corrective Maintenance	31

5.4.2	Preventative Maintenance	31

5.4.3	Inspections	32

5.5	The application of quality assurance	32

5.6	Selection of a supplier	32

5.7	Lighthouse authorities as suppliers	33

ANNEX 1 -	A SHORT INTRODUCTION TO PROBABILITY THEORY	35

A1.1	Axioms of mathematical probability	35

A1.2	Addition rule for mutually exclusive events	35

A1.3	Addition rule for arbitrary events	35

A1.4	Conditional probability	36

A1.5	Multiplication rule (Theorem 4)	37

A1.6	Poisson Distribution	37

ANNEX II - DEFINITIONS AND COMMENTS	38

# INTRODUCTION

## Background (purpose and scope)

In spite of the efforts made within IALA, it has not proved possible to assess the importance of an aid on a rational and universally accepted and applicable basis.

Notwithstanding this difficulty one can seek to produce guidance applicable to the availability of aids, firstly, because examination of the data concerning the operation of aids to navigation gathered by IALA revealed the possibility of such an approach and, secondly, because the needs for such guidance is evident.

Changing conditions of navigation, and the development of new aids, are leading to a reassessment of what were considered to be established policies and to a search for the foundations of a more effective use of resources. Engineers encountered difficulties in defining the level of sophistication of an aid when no objectives were laid down other than navigational requirements.

The guidelines, which it is sought to produce, are simple. They consist of paying special attention to operational performance and of recording data permitting the calculation of an indicator of service rendered. The value of this indicator should be set for each project and cases of aids for which the indicator value is unsatisfactory with regard to the objectives should be closely examined.

It is accepted that this examination could lead to the aid concerned being abolished in order to concentrate available resources on a reduced number of aids providing an acceptable level of service.

The following cases are treated:

- blocks in series

- passive redundancy without repair

- passive redundancy with repair

- active redundancy without repair

- active redundancy with repair

The outlined methodology may be used to calculate the predicted reliability of a single aid to navigation consisting of several statistically independent subsystems each with its own level of reliability expressed as Mean Time Between Failure (MTBF).

The methodology may also be used to calculate the predicted reliability of a system of aids consisting of a number of individual aids.

The outlined mathematical model may furthermore be used by system designers to calculate the required reliability of systems and subsystems to ensure that the availability objectives set by management can be obtained.

## Relationship between reliability and availability

Use of the "AVAILABILITY" parameter is a good method for defining the level of service a navigator can expect from an aid to navigation. Furthermore, the numerical value of availability can be used to determine, in precise quantifiable terms, the sum total of all the relevant characteristics of the design, engineering, procurement and quality assurance procedures involved in the provision of aids to navigation, together with the necessary logistics and maintenance.

DEFINITION:	The AVAILABILITY is the probability that an aid or a system of aids to navigation is performing a required function under stated conditions at any randomly chosen instant in time.

It should be emphasized that, within the framework of what follows, availability concerns only the ability of an aid to operate as advertised in nautical documents and does not consider external factors such as reduced visibility.

It is true to say that at the design stage the intensity of a light will be chosen according to visibility conditions prevailing locally.

It is also true to say that a light of say 1000 candelas (cd) operating as advertised will have the same luminous output irrespective of the visibility and that it is the duty of the mariner to adjust his expectations and behavior according to the weather conditions.

The parameter "AVAILABILITY" is intended to be used as a management tool, not as a measure of an act of God. In other words, because he meteorological visibility is greater than 10 NM 90% of the time one year and 95% of the time the next year, the Lighthouse Authority should not be automatically satisfied with its performance.

The availability (A) may be calculated by dividing the total time during which the aid has been operating correctly (Up Time) by the total time during which the function to be performed by the aid should have been fulfilled (Mission Time).

A=UpTimeMissionTime	eq. 1

Percentage availability is also used. It is defined as 100 times the availability:

A%=100A	eq. 2

An appropriate period of time should be used for the calculation of the availability of a single aid. This period will have to be sufficiently long, say longer than 2 years, if the calculated availability is intended to represent the long-term aid performance.

DEFINITION: 	System RELIABILITY expressed as Mean Time Between Failures (MTBF) is the average period of time wherein the probability of system failure approaches or equals 1 ????

In order to achieve a reasonable value for MTBF the system of interest must be observed over a large number of system failures. ????

The unique parameter used hereafter to characterize the reliability is the Mean Time Between Failures (MTBF) which is also the parameter used in data collection by most Lighthouse Authorities.

The availability of an aid to navigation may also be calculated by dividing the MTBF by the sum of the MTBF and the Mean Time to Repair (MTTR).

A=MTBFMTBF + MTTR	eq. 3

If we rewrite eq. 3 to obtain MTBF we get:

MTBF=MTTRA(1−A)	eq. 4

which is useful for system designers to calculate the required reliability (MTBF) of systems and subsystems to ensure that the availability objectives set by management can be obtained with a given MTTR.

A typical failure scenario will look like this:

- SECTION TO BE COMPLETED -

# RELIABILITY OF SYSTEMS

## Introduction

In spite of the value of the methods whereby the availability of the total equipment is expressed as a function of the availability of its components, and the MTBF is derived from that availability, we have concentrated on more simple and straightforward methods whereby the predicted MTBF of an equipment is calculated as a function of the MTBF of its components.

This has been possible only because of the very special nature of aids to navigation. In general the time to respond to a failure depends more on the local conditions, and whether the failure has caused a casualty which requires immediate attention.

In aids to navigation business, there is generally only one redundant element. For the sake of simplicity and clarity attention is restricted to this case although the formulae can often be generalized to a system with a higher level of redundancy.

Some basic assumptions have to be made (…in order to comply with the basic rules of probability theory):

- All systems of interest may be modeled by a number of subsystems (blocks) connected in series or parallel or as a combination of both.

- The various blocks are statistically independent.

- For a given block the MTTR is far shorter than the MTBF.

- The number of blocks is few, which in practice means that the difference between the MTBF computed as the mean of up times, and the MTBF computed as the ratio between mission time and the number of failures can be neglected. Under these hypotheses the difference between the MTBF and the mean time to failure MTTF, can also be neglected.

- The probability that a device starts when activated or the probability that a switching device operates correctly is assumed to be constant.

- A repaired item is equal to a new item in terms of MTBF.

- An item put into service from time to time (e.g. a standby element) has the same MTBF in terms of its own mission time, as one operating continuously.

Probability laws are continuous with a large spectrum. This eliminates some very peculiar cases encountered when the probability laws concentrate on few values.

## Relationship between preventive maintenance and reliability

If the life duration of a component follows a probability law for which the more probable duration periods are concentrated around a given value m (see the next figure) then it is clear that the replacement of the component after T hours will be beneficial in terms of enhanced reliability but in the case of expensive components there might be better ways to solve the problem, and a cost benefit analysis should be carried out.

f(t)

t

T          m

- The on site MTBF of a component, which is regularly changed after T hours of operation, is equal to:

MTBF=MTBF0+T1−p0p0	eq. 5

where MTBF0 is the average working time of a component that fails before T hours of operation, and p0 the probability that such a situation occurs.

## Blocks in series

- The system operates if, and only if, each of the blocks is in good running order.

The operating time may be presented as follows:

The MTBF of the system (MTBFSYS) is then given by the following formula:

1MTBFSYS=1MTBFX+1MTBFY+.............+1MTBFi

where MTBF i is the Mean Time Between Failures of the i´th block.

The MTTR of the system (MTTRSYS) is given by the following formula:

MTTRSYS=MTBFSYSn=1iMTTRnMTBFn

## Blocks in parallel, passive redundancy

### Passive redundancy. Without repair

Block Y is put into service only if block X fails, and block X is repaired only after failure of block Y.

The succession of up times and down times is as follows:

The MTBF of the system (MTBFSYS) is then given by the following formula:

MTBFSYS=MTBFX+(1−p)MTBFY

where p is the probability that Y doesn't start operating correctly when X fails:

### Passive redundancy. With repair

- The block Y is put into service only if the block X fails and operates only while X is under repair.

The MTBF of the system (MTBF SYS) is then given by the following formula:

MTBFSYS=MTBFX+ MTTRXp+(1−p)MTTRXMTBFY

where p is the probability that Y does not start operating correctly when X fails.

## Blocks in parallel, active redundancy

### Active redundancy without repair when only one block has failed

X and Y run at the same time when X or Y alone is enough to produce the function required from S (a failure of the system occurs only when X and Y are out of order).

If X and Y are identical then if the time between failure was fixed (i.e. failures don't occur at random) MTBFSYS would be equal to MTBFX and the redundancy would not carry advantages.

Active redundancy will prove beneficial if the dispersion of the running times is of some importance.

If X and Y are identical with a constant failure rate MTBFSYS is given by the following formula:

MTBFSYS=3−2p2MTBFX

where, p, is the probability that the second block becomes ineffective as soon as a failure occurs on one block.

### Active redundancy with repair of a failed item

X and Y operate simultaneously when X or Y alone would be enough to ensure that the function required from the system S is performed.

The operating times may be represented as follows:

MTBFSYS is given by the following formula:

MTBFSYS=11MTTRX+1MTTRY1−ISIS     where   IS=MTTRXMTBFX+MTTRX∗MTTRYMTBFY+MTTRY

# PROOF OF FORMULAS

## Blocks in series

We shall assume that all blocks stop operating when one of them is out of order and that the equipment has been operating for a very long period of time T and has had a great number of failures, N, comprising Nl failures of block 1, N2 failures of block 2 …. Ni failures of block i.

The total number of failures (N) in a system consisting of i blocks all in series may be expressed as follows:

N=n=1iNn

- The MTBF of the system is equal to

TN=TN1+ N2+.....+ Ni

i.e. the total time divided by the total number of failures.

Let MTBF i be the MTBF of block number i

MTBF i=  TNi

then

NT=N1T+N2T+....+NiT

that is to say

1MTBFSYS=1MTBFX+1MTBFY+.............+1MTBFi

The invert of the MTBF of a system with all blocks in series is equal to the sum of inverses of the MTBF of each block.

A very popular demonstration of this formula is given hereafter:

The probability that two independent events occur during a trial is equal to the product of the individual probability of each event. Then the probability that a system with i blocks in series does not fail during a period of time t is equal to

R(t)=e−(λXt)+e−(λYt)+......+e−(λit)

If each block has a constant failure rate.

It results from the property of the exponential that

R(t)=e−n=1i(λnt)

so, the total failure rate SYS of the system is equal to

R(t)=n =1iλnt

- Note: With the same notations as before

MTTRSYS=n =1iNn MTTRnN=n=1iTMTBFn MTTRnTMTBFSYS

And thus:

MTTRSYS= MTBFSYSn =1iMTTRnMTBFn

### Passive redundancy without repair

The succession of up times and down times is as follows:

By definition

MTBFSYS=1nn=1i(Xn+Yn)

where n is large and is the total number of failures of the system. So:

MTBFSYS=1nn=1iXn+1nn=1iYn=MTBFX+ MTBFY

If Y has a probability equal to, p, not to start operating correctly when X falls then it is only in 100 * ( 1-p ) % of the cases that the operating time of Y will be added to that of X.

It follows that :

MTBFSYS=MTBFX+(1+p)MTBFY

### Passive redundancy, with repair

Let n be the number of failures of X during a long period of time equal to n * (MTBF X + MTTR X).

The block Y will have to operate during the down times of X. The total down during the period in question will be equal to NX * MTTR X, and there will be on average a failure of Y, (and so of the system) every MTBF Y hours (Counted on the basis of the down time of X).

Then the number of failures NSYS of the system S will be very close to

NSYS Approx=NX* MTBFXMTBFY

- MTBFSYS=NX(MTBFX+ MTTRX)NSYS≈NX(MTBFX+MTTRX)NSYS ApproxBy definition:

- Thus follows:

- MTBFSYS≈MTBFX2MTTRX+MTBFXIf X and Y have the same statistical characteristics:

- If we assume that there is a probability p that the block Y does not start when activated then the same demonstration as before leads to the following formula:

NSYSApprox=Np+N(1−p)MTTRXMTBFY

And

MTBFSYS≈N ( MTBFX+ MTTRX)NSYSAapprox=MTBFX+MTTRXp+(1−p)MTTRXMTBFY

* Strictly speaking this should be noted MTTFSYS, and the demonstration stands only when after repair of y both X and Y are in a good running order (under basic hypothesis, the approximation is very good).

### Active redundancy without repair

If p = 0 the proof is rather simple:

X and Y being statistically independent then P(S  t) = P(X  t) * P(Y  t)

If in addition X and Y are identical with a constant failure rate then:

P(S≤t)=1−e−λt1−e−λt=1−2e−λt+e−2λt

- The reliability function R(t) is then equal to:

- R(t)=2e−λt−e−2λt

- and

- MTBFSYS=0∞2e−λt−e−2λtdt=2e0λ−e02λ=32∗1λ    where   1λ=MTBF

- That is to say

- MTBFSYS=32MTBFX

If d 0 the proof is more complicated and is only given hereafter for those familiar with the theory of probability.

- Let FX and FY be respectively the states where X and Y are in good running order at t.

- The event "the failure of one element does not cause that of the other" will be quoted as FB.

- Let FX,FY  and  FB be the events complementary to FX, FY FB respectively.

Then by applying the theorem of "total probabilities" (1) it follows that the probability P(S t) of a failure of the system occurring before t is given by:

P(S≤t)=P(FB)P(FX∪FY)+P(FB)P(FX∩FY)=P(FB)P(FX)+P(FY)−P(FX)P(FY)+P(FB)P(FX)P(FY)P(FB)=p   and   P(FB)=(1-p)P(FX)=(1−e−λt)   and   P(FY)=(1−e−λt)0∞P(S≤t) dt =MTBFSYS

A simple calculation shows that

MTBFSYS=3−2p2MTBFX

One should note that the same method would permit calculation of MTBFSYS when the 2 blocks do not have the same failure rate.

### Active redundancy with repair of a failed item

The operating times may be represented as follows:

Let us calculate IS = (1 - A SYS) where A SYS is the availability of the system:

IS=MTTRXMTBFX+MTTRX∗MTTRYMTBFY+MTTRY

As by definition :

IS=MTTRSYSMTBFSYS+MTTRSYS

by looking at the operating diagram one can notice that the model for calculating MTTRSYS is equivalent to one with blocks in series where time to repair, is substituted for operating time (up time) and vice versa and where it is assumed that a block continues to operate even when the other has failed.

Having done that, the basic hypotheses are not preserved since mean time to repair is greater than MTBF, and then the demonstration given in 2.1 can't be applied.

However, one can show that the following formula is still valid :

1MTTRSYS=1MTTRX+1MTTRY

and so

MTBFSYS=11MTTRX+1MTTRY1−ISIS

- If X and Y are identical    MTTRSYS=MTTRX2   and

- MTBFSYS=MTTRX21+MTBFXMTTRX2−1

- The above formula can be generalized to the case where i blocks are in active redundancy.

If the i blocks are identical the formulae are:

and

MTBFSYS=MTTRXi1+MTBFXMTTRXi−1

Notes:

- A more efficient policy being in general applied then the whole system S has failed; the above MTTRSYS should be considered only as a mean to calculate MTBFSYS.

- Under the basic hypothesis it can be shown that the probability that the system S still operates at time t is close to e−tMTBFSYS

### Relationship between preventive maintenance and reliability

The proof is quite easy for those familiar with the theory of probability:

By definition:   P0=0tF(t)dt

Let Pq be the probability that the equipment fails after having been replaced q times then

Pq=P0(1−P0)q

since q components have been changed after having been operating correctly during T hours and the q + l component failed before T hours of operation.

- MTBF0=oTtf(t)dtoTf(t)dt

- And

- MTBF=i=0∞(iT+MTBF0)Pi=MTBF00∞Pi+T0∞iPi

- In this expression  0∞Pi=1  and   0∞iPi=P00∞i(1−P0)i=1−P0P0

- It follows that

- MTBF=MTBF0+T1−P0P0

# EXAMPLES

## Methodology to assess Failure Response Time (FRT) impact on service Availability

### Computation of failure response time

Failure Response Time (FRT) can be derives from the availability (A) relationship between MTBF and MTTR:

A=MTBFMTBF+MTTR					Equation (1)

Solving for MTTR yields:

A=MTBF(1−A)MTBF+MTTR					Equation (2)

Use as an example the following case where values are given:

- A = 0.998

- MTBF							= 14 000 h.

- Mean Time To Report (MTT Report)			=   5 h.

- Mean Time To Prepare (MTT Prepare)		=   4 h.

- Mean Time To Transport (MTT Transport)		= 11.5 h.

(Includes possible loss of time due to bad weather)

- Mean Time To Repair on Site (MTT Repair on Site)	=   2 h.

Recall that:

MTTR =	Mean Failure Response Time (MFRT) + MTT Report + MTT Prepare + MTT Transport + MTT repair on site.		Equation (3)

Inserting the assumed values yields:

MTTR =	5 + 4 + 11.5 + 2 + MFRT

MTTR =	22.5 + MFRT

Substituting this value for MTTR in equation (2) yields:

22.5+MFRT=14000(1−0.998)0.988; 22.5 + MFRT = 28.1; MFRT = 28.1 – 22.5

MFRT = 	5.6 hours.

### Conclusion

For the given values of MTBF and repair times, a Mean Failure Response Time of 5,6 hours selected by management would permit the lighthouse service to maintain the stated level of service (A = 0,998) to the mariner.

If the computed value for MFRT is negative, it means the MTBF is too low or the total MTTR is too high to provide the stated availability. The MTBF situation can be helped by specifying higher quality equipment, increasing equipment redundancy, or increasing preventive maintenance. The MTTR situation can be helped by reducing the various time components in the MTTR total.

It is possible that a certain aid to navigation does not seem to be very important and as a result may have been assigned a low availability and a large MFRT. Due to a failure of this aid, a vessel may be misled, run aground and be heavily damaged and severely injure the environment. The resulting cost could be enormous. A Lighthouse service should keep this in mind when setting an availability objective.

## Computation of Mean Time to Repair for selected redundancy and repair policies

Consider the two aid systems represented by the block diagrams below, one without redundancy and the other with redundancy. Using the relationships developed in Part I Chapter 2 of this Guide, MTTR’s are computed for differing redundancy, maintenance policies, and number of planned maintenance visits annually.

For each case the availability objective (A) is chosen to be 0,995, and the MTBF of each light system block is given to be 4000 hours. The 4000 hours interval is nominally one year for these daylight controlled lights. Computed values for MTTR are shown in the table below. A more detailed presentation of each computation is shown on the following pages in this annex.

NORMAL

LIGHT

NORMAL

LIGHT

MTBX  =  4000  h

MTBF  =  4000 h

STANDBY

LIGHT

NO REDUNDANCY

PASSIVE REDUNDANCY

MAXIMUM MTTR TO ACHIEVE AVAILABILITY OBJECTIVE OF 0,995.

|  | CASE 1 |  | CASE 2 |  |  | CASE 3 |
| --- | --- | --- | --- | --- | --- | --- |
|  | NO REDUNDANCY | PASSIVE REDUNDANCY REPAIR ONLY WHEN BOTH LIGHTS EXTINGUISHED | PASSIVE REDUNDANCY REPAIR AT REGULAR VISIT OR WHEN BOTH LIGHTS EXTINGUISHED ONE VISIT EVERY: |  |  | REDUNDANCY NO REGULAR VISIT MTTR SAME FOR ONE OR BOTH LIGHTS EXTINGUISHED |
|  |  |  | 2000 h | 1000 h | 500 h |  |
| COMPUTED MTTR | 20 h | 40 h | 100 h | 180 h | 340 h | 300 h |

This simple example shows that maintenance policies and the existence of standby equipment will affect considerably the MTTR objective necessary to meet an availability objective.

This is of paramount importance because the cost of maintenance is likely to be far higher when the Mean Time To Repair a failure is set at 20 hours than when it is set at 300 hours (note that in both cases the number of visits to the aid, per year, is the same). One reason for this is that more flexibility is given to the maintenance force and that there are fewer panic maintenance operations.

It should also be realised that paying visits every 2000h or 500h to an aid may not be excessively costly. This will be the case, for example, if the distance between the aids marking an area is short compared with that between an aid and the maintenance facility; and if it is accepted that a visit is paid to each aid when travel to one of them is rendered necessary by circumstances.

### Detailed computation of MTTR values presented in the above table

The general technical hypotheses are given as follows:

- Availability objective, A: 	0,995;

- 2 technical choices:		no redundancy (normal light),

passive redundancy (normal & standby);

- MTBF of each light (normal & standby light) = 4000 h.

Equation terminology: 	p is the probability that the standby light does not start operating correctly when the normal light fails.

MTBF and MTTR denote general availability attributes.

Addition of a suffix denotes corresponding subsystem values:

MTBFX is MTBF for subsystem X

MTTRY is MTTR for subsystem Y

MTBFSYS is MTBF for entire system

MTTRSYS is MTTR for entire system.

The general formulas for cases are:

Availability (A) = MTBF(MTBF+MTTR)

MTTR = MTBF(1−A)A

MTBFSYS  = (MTBFX+MTTRX)(p+(1−p)MTTRXMTTBFY

With p = 0, the above formula is reduced to:

MTBFSYS  = MTBFYMTTFX(MTBFX+MTTRX)

Case 1:

- No redundancy.

MTTR = MTBF(1−A)A

MTTR = 4000(1−0.995)0.995

MTTR = 20,1 hours.

B.	Redundancy; repairs take place only when the two lights (normal & standby) are extinguished. This is a category of passive redundancy without repair.

MTBFSYS=MTBFX+(1−p)MTBFY

Assume p = 0

MTBFSYS  = 4000 + (1-0) 4000

MTBFSYS  = 8000 hours.

MTTRSYS = MTBFSYS(1−A)A

MTTRSYS = MTBFSYS(1−0.995)0.995

MTTRSYS = 40,2 hours

Case 2:

Redundancy: repairs take place at a regular visit or when the two lights (normal & standby) are extinguished; visit planned at selected intervals. This is a category of passive redundancy with repair. The formulas below are used to solve the problem.

MTTR = V (Visit interval) / 2 (see 2.8 in Part I)

MTBFSYS  = MTBFYMTTFX(MTBFX+MTTRX)

- One visit every 2000 h.

MTTRX = 2000 / 2 = 1000 h.

MTBFSYS  = 40001000(40000+1000) = 20,000

MTTR = 20,000 x (1-0,995) / 0,995

MTTR = 100 hours.

- One visit every 1000 h.

The same procedure is followed as in A.

MTTRX = 1000 /2 = 500 h.

MTBFSYS  = 4000500(40000+500) = 36,000h

MTTR = 36 000 x (1-0,995) / 0,995

MTTR = 180 hours.

- One visit every 500 h.

The same procedure is followed as in A.

MTTRX = 500 / 2 = 250 h.

MTBFSYS  = 4000250(40000+250) = 68,000h

MTTR = 68 000 x (1-0,0995) / 0,995

MTTR = 340 hours.

Case 3:

Redundancy; no regular visit, Mean Time To Repair is the same when only one light is out and when the two lights (normal & standby) are extinguished. This is a category of passive redundancy with repair.

MTBFSYS=MTBFyMTTRx(MTBFX+MTTRX)	Formula 2.3 with p = 0

MTBFSYS=4000MTTRX(4000+MTTRX)

A=MTBFSYSMTBFSYS+MTTR

0.995=MTBFSYSMTBFSYS+MTTR

The MTTR of the system is given to be equal to MTTRX in this case, so:

0.995=MTBFSYSMTBFSYS+MTTR or

MTBFSYS = (MTBFSYS + MTTRX) x 0.995 or

MTBFSYS (1-0.995) = 0.995 MTTRX

MTBFSYS=0.995MTTRX0.005 = 199 MTTRX

199 MTTRX=4000MTTRX(4000+MTTRX) or

199 (MTTRX)² = 16000000 + 4000 MTTRX or

199 (MTTRX)² - 4000 MTTRX – 16000000 = 0 or

divide by 199, we have:

(MTTRX)² - 20.1 MTTRX – 80402 = 0

[General solution x=−b±b2−4ac2a]

MTTRX=20.1±20.12+4x804022

MTTRX = 293,78 hours (approximately 300 hours)

# QUALITY MANAGEMENT SYSTEMS AND RELIABILITY

The application of quality assurance in industry is well known. With the industrial revolution specifications and standards became a necessity. On the one hand these avoided the proliferation of, for example, different screw threads, which had to conform to a standard if interchangeability was to be achieved and on the other hand materials could be ordered, or items manufactured, to an agreed quality.

Quality, however, can mean different things to different people and a good deal of effort on the part of standards organisations throughout the world has been expended in defining and agreeing the meaning of terms. Some of these terms have been included in this guide. The specific definitions given are based upon the International Standard BS EN ISO 9000 (2000): Quality management systems – Fundamentals and vocabulary.

## Basic terms

To avoid the misunderstandings, which can occur over the meanings of words, the following terms are defined in accordance with ISO 9000.

### Quality

‘The degree to which a set of inherent characteristics fulfils needs or expectations that are stated, generally implied or obligatory’. The word quality is often used for several distinct purposes. One is in a comparative sense or degree of excellence whereby products may be ranked on a relative basis; sometimes referred to as "grade". Another is in a quantitative sense as used in manufacturing product release and for technical evaluations; sometimes referred to as "quality level". A third is in the sense of fitness for purpose, which relates the evaluation of a product to its ability to satisfy a given need. In the context of this paper the "fitness for the purpose" sense is used.

### Quality assurance

‘All those parts of the coordinated activities to direct and control an organization with regard to quality that focus on providing confidence that quality requirements will be fulfilled’. There are, of course, many more terms relating to quality assurance and these can be found in appropriate national standard as required. For the purpose of this guide quality assurance is defined as embracing all those coordinated activities, concerned with providing confidence that quality requirements will be fulfilled , rather than in the narrower sense only of the provision of proof associated with the word "assurance". Thus quality assurance includes the determination and assessment of quality.

### Quality control

‘All those parts of the coordinated activities to direct and control an organization with regard to quality that are focused on fulfilling quality requirements’. It includes that aspect of quality assurance that concerns the practical means of securing product or service quality as set out or implied in the specification for a product or a process.  The term may be applied to the system of control or service being controlled.

### Specification

"The document and data defining the needs or expectations that are stated, generally implied or obligatory’.

## Drafting specification

Before drafting a new specification it is advisable to ascertain, from the national and international standards organisations, whether specifications already exist, which will meet the requirements. If it is necessary to draft a new specification it is essential to follow a logical method. The definition of quality covers all aspects of a product or service and it follows that the specification should cover all the features and characteristics. In practice there is considerable diversity in the contents and details given in specifications and it cannot be assumed that specifications for all the products in the same general category will always contain the same requirements from the quality point of view.

The twelve types of specification are described in the following headings, which correspond to the different ways in which the quality of a product or service may be defined. In practice the information given under these headings may be collated and treated as a single comprehensive specification but in the initial drafting it is recommended that they be treated separately so that each is given its full specialised attention. This subject is of paramount importance in the achievement of quality; in many cases poor products or services are the result of inadequate, ambiguous or imprecise specifications.

### Target specification

The document that describes the primary purpose of an item and gives essential guidance concerning such matters as style, grade, performance, appearance, conditions of use including any health and safety considerations, characteristics, packaging, conformity, reliability, maintenance, etc. The target specification is often the result of a feasibility study and forms the basis of the design. It is sometimes known as the design "brief" or the "primary specification."

### Functional specification

The document that describes in detail characteristics with regard to its intended capability. As far as possible this specification should be written in quantitative terms.

### Product specification

The document that describes, for manufacturing purposes, the item concerned.

### Materials specification

The document that describes in detail the materials, components or supplies used in the manufacture of the item.

### Process specification

For discrete items, it describes in detail the method of assembling or producing an item. For bulk commodities, it describes in detail the procedures and operations to be carried out on the materials used. For plant, it describes in detail the control of facilities used in the treatment or sequence of treatments, of the item or commodity.

### Inspection specification

Describes in detail the methods of inspection including, if necessary, the basis for subsequent action.

### Test specification

Describes in detail the method of conducting tests including, if necessary, the criteria for assessing the result. It may contain clauses for conformity and reliability assessment.

### Acceptance specification

Describes in detail the criteria for acceptance of a product.

### Installation specification

Describes in detail the procedures for installing the product; including if necessary, the procedures for unpacking and preparation prior to installation.

### Use specification

Describes in detail the method of bringing into use, operating, controlling and adjusting the product. This may take the form of an instruction manual.

### Maintenance specification

Describes in detail the procedures and circumstances for carrying out maintenance. This may take the form of an instruction manual.

### Disposal specification

Describes in detail the methods and precautions to be observed in the discarding or otherwise disposing of a product when it has failed or is no longer required for any reason. The essential features of this document are usually given in the form of a warning notice displayed on the product.

## Specification data

It may not be easy to quantify some of the parameters in the specification. One of the most difficult is the definition of the environmental conditions that must be met. Consider, for example, equipment for use on a navigation buoy; acceleration values will vary considerably between different designs of buoys. Ideally tests should be carried out to establish the correct parameters and thus ensure that the specification is neither too strict nor too lax.

It will be noted that the target specification contains the reliability requirements; here again these may be difficult to quantify in the absence of sound data. Figures are available of failure rates of components but it is essential to be satisfied that the conditions, under which the figures were obtained, were relevant to the case under study. There is a need for a data bank of failure rates and reliability based on experience under lighthouse conditions.

## Maintenance

Operating and maintenance departments should be represented on the team engaged on drawing up new specifications and have the opportunity to comment on any revision of existing specifications. In particular the maintainability, that is how easy the system can be returned to service following a failure, like reliability will have a major affect on availability.

Identification of maintainability requirement during the system life cycle is essential to ensure a successful system design. Factors that affect the overall contribution that maintainability has on system availability will include the geographic location, distance of the AtoN from the maintenance center and the availability of transportation, resources and spares. However, some of these may be mitigated to a degree by the maintenance practices and procedures employed by the Lighthouse Authority.

In general maintenance can be divided into three types, corrective maintenance, preventive maintenance and inspections.

### Corrective Maintenance

Corrective maintenance usually involves the repair or replacement of a system component/module to restore the system to a fully operating condition in as short a time as possible. It involves the detection of the fault by the technician, the replacement or repair of the faulty component and finally the testing of the system to prove its satisfactory operation.

### Preventative Maintenance

Preventative maintenance is the process by which components or modules are replaced before they fail in order to support continuous operation without failure. The affect of preventative maintenance on availability can be of major importance since with preventative maintenance system down time can be significantly reduced because it does not include the time to travel to site but only the repair time on site. The schedule for preventative maintenance is based upon reliability statistics and historical data to determine the key system components that are most likely to wear out.

### Inspections

The primary objective of Inspection is to detect unrevealed failures. If such failures are found then corrective maintenance would be carried out to rectify the problem.

## The application of quality assurance

It is self evident that if the full benefit is to be derived from quality assurance then some formal organisation is required within the Lighthouse Authority to provide the necessary control and delegation of responsibility. It should not be assumed that this requires some expensive superstructure supervising the existing organisation. The quality assurance arrangements should be set up within the existing organisation. It is often a matter only of rationalising and documenting what is being done already. Guides are available to assist in the setting up of the necessary arrangements and these are adaptable to the needs and resources of the particular authorities.

To quote from the British Standards. "The traditional approach to quality control relied heavily on post production inspection and correction and dealt with both design and production problems after the event when they were most difficult and expensive to put right. Quality assurance is based on the principle that prevention is better than cure; it being more economical to get things right in the first place. Quality assurance provides complete control of quality from planning the design, through production and purchasing, into service ..." It follows from this that quality assurance is not just the responsibility of the supplier but also equally that of the purchaser.

## Selection of a supplier

An important factor in achieving reliability and maintainability is the supplier’s ability to manage and deliver the project. The factors discussed below should be considered during the tender review process and a scoring system devised to aid comparison between suppliers

A supplier’s capability may be assessed at five levels:

Level 1 (Basic) where the contracting process is informal and unpredictable. If a successful result is achieved it will depend on a few individuals and good luck.

Level 2 (Documented) where critical processes are documented suggesting repeatability and some learning from experience, hence risks are reduced. Depends upon the degree to which the procedures reflect Senior Management policies.

Level 3 (Managed) where the processes are documented and procedures are followed. If the procedures are based upon processes that are known to be effective within the organisation then the process risks will be low. However, the process will not yield optimum solutions because there is no quantitative feedback mechanism necessary to optimise the process.

Level 4 (Measured) in addition to level 3 capabilities the quantitative controls of processes and product quality must be in place. These will include goals for process performance and measuring project performance. These measurements will be fed back and adjustments made to procedures and process to optimise performance.

Level 5 (Self-Improvement) where proactive improvement is institutionalised. Processes are continually being evaluated in a way that is most cost effective for the project.

The supplier’s design process needs to be assessed and the purchaser needs to look at the companies historical design record. They need to ask if the supplier has been able to design equipment in the past without major design faults; is the quality of the design staff that will be working on the contract appropriate and is the quality of the design procedures to be used on the project adequate.

The design phase of the project will also be influenced by its complexity. Is the specification clear and unambiguous, are the requirements novel with untried technologies, are there complex software based control systems? The time schedule permitted for design will also have an impact on the design phase of the project.

Although the production process cannot improve the reliability and maintainability of a product inherent in its design it can have a negative effect. As with the design process similar factors need to be assessed, for example, has the manufacturer produced similar equipment in the past, are the production staff suitably trained and experienced, is there a quality assurance function in the manufacturing process including inspection and testing.

## Lighthouse authorities as suppliers

So far this section has dealt with the situation of the Lighthouse Authority as the purchaser, it is as well to consider the Authority as supplier of the service to the mariner. It will have been noted that quality assurance applies equally by definition to services as well as to products and in the role of supplier the Authority has a duty to ensure the quality of the service to the mariner.

It is only rarely that the opportunity arises for the mariner to contribute to the formation of the specification and it is by no means certain that he would wish to do so. Therefore the Authority is placed in the position of having to write the specification for the service provided as well as applying the necessary quality control.

At first these	comments may 	appear superfluous since all Lighthouse Authorities make great efforts to maintain the highest standards. However, in these days of advanced and complex technology it is worth considering the application of formal quality assurance techniques in order to ensure the best use of money and resources. This is after all only the logical extension of reliability and availability techniques. The setting up of a quality assurance structure, within an Authority, is similar to that required by a manufacturer and is largely a matter of defining responsibility and exercising the necessary control.

The modern trend towards product liability requires serious consideration of formal quality assurance procedures as a form of insurance. The exchange of specifications and standards between Lighthouse Authorities may materially assist in achievement of quality assurance.

# ANNEX 1 -	A SHORT INTRODUCTION TO PROBABILITY THEORY

## A1.1	Axioms of mathematical probability

If E is any event in a sample space S, then:

0≤P(E)≤1	(Axiom 1)

To the entire sample space S, there corresponds:

P(S)=1	(Axiom 2)

If A and B are mutually exclusive events, then:

P(A∪B)=P(A)+P(B) 	(Axiom 3)

This may be represented graphically as follows:

If the sample space is infinite, we must replace Axiom 3 with:

If E1, … Em are mutually exclusive events, then:

P(E1∪E2∪....∪Em)=P(E1)+P(E2)+....+P(Em)	(Axiom 3*)

## A1.2	Addition rule for mutually exclusive events

If E1, … Em are mutually exclusive events, then:

P(E1∪E2∪....∪Em)=P(E1)+P(E2)+....+P(Em)	(Theorem 1)

## A1.3	Addition rule for arbitrary events

If A and B are any events in a sample space S, then:

P(A∪B)=P(A)+P(B)−P(A∩B)	(Theorem 2)

This may be represented graphically as follows:

Furthermore, an event and its complement EC are mutually exclusive and E  EC = S.

Using axioms 3 and 2 we thus have:

P(E∪EC)=P(E)+P(EC)=1

This may be represented graphically as follows:

This yelds

Probabilities of an event and its complement EC in a sample space S vare related by the formula:

P(E)=1−P(EC)	(Theorem 3)

This formula may be used if the computation of P(EC) is simpler that P(E).

## A1.4	Conditional probability

Often it is required to find the probability of an event B under the condition that event A occurs. This probability is called the conditional probability of B given A and is denoted by P(BA).

In this case A serves as a new (reduced) sample space, and that probability is the fraction of P(A) which corresponds to A  B. Thus:

P(B|A)=P(A∩B)P(A)	[P(A)  0]

Similarly, the conditional probability of A given B is:

P(A|B)=P(B∩A)P(B)	[P(B)  0]

Solving the last two equations (7 and 8) for P(A  B), we obtain:

## A1.5	Multiplication rule (Theorem 4)

If A and B are events in a sample space S and P(A)  0, P(B)  0 then:

P(A∩B)=P(A)P(B|A)=P(B)P(A|B)

If the events A and B are such that:

P(A∩B)=P(A)P(B)

they are called independent events. Assuming P(A)  0, P(B)  0, we see from (7-9) that in this case:

P(A|B)=P(A)P(B|A)=P(B)

which means that the probability of A does not depend on the occurrence or nonocurrence of B and conversely. This justifies the terminology.

## A1.6	Poisson Distribution

It seems reasonable to use Poisson……

The discrete distribution with the probability function:

f(x)=μxx!e−μ

is called the Poisson distribution, named after Siméon Denis.Poisson (1781-1840).

( Mean  = 5 )

# ANNEX II - DEFINITIONS AND COMMENTS

Failure Response is the action taken by a lighthouse service after a failure has been reported. It may take the form of an immediate repair action, a repair within a certain period of time, another action such as a notice to mariners, or a combination of the latter with one of the former.

Failure Response Time (FRT) is the time wherein a lighthouse service must react to a failure. The FRT depends on the intended availability (A) of an aid or system of aids, the risk which may be caused by a failure (such as economic consequences to a port, environmental damage and involved cost, etc.), and the type of failure.

Aid Availability established by a service depends in part on the attributes of the aid itself. This includes the importance of the aid, determined by its purpose, type and number of users, configuration of the waterway, and equipment on the aid. The selected level of availability will also depend on other aids or information sources in the vicinity, maintenance and Mean Time Between Failure (MTBF), and Mean Time To Repair (MTTR). Several methods exist to determine the importance of an aid including computation according to a quantitative methodology, or from practice or experience in the local area of interest. Pilots and VTS communication centers/stations can be valuable information sources.

Mean Time Between Failures (MTBF) is simply the length of time between aid failures. Maintenance and MTBF are interactive. If more preventive (planned) maintenance is rendered, the MTBF increases.

Mean Time To Repair (MTTR) is the time it takes to restore an aid to normal operation after it fails. MTTR is composed of reporting time (or time to detect), preparation time, transportation time, repair time on site, and failure response time.

Mean Time To Report varies, depending on whether the aid is manned or remotely controlled; if not, the time will depend on the attentiveness of marines who can make reports from vessels passing a failed aid.

Mean Time To Prepare is the time required for preparations such as alerting the crew, collecting tools and materials, making the tender ready to sail, etc.

Mean Time To Transport is the time required to transit from the base to the failed aid and includes time lost due to sea and weather conditions.

Mean Time To Repair on site is the time required on site for the repair crew to actually restore a failed aid to its original function.

References

MOD R&M/6/4000/17.A 	‘A Process Capability Model (PCM) for Reliability and Maintainability (R&M)’

BS EN ISO 9000:200	‘Quality Management Systems – Fundamentals and Vocabulary’

ANNEX ?

MAXIMUM MEAN TIME TO REPAIR (MTTR), AND NUMBER OF FAILURES PER YEAR, COMPATIBLE WITH OBJECTIVES

To draw the tables a Poisson distribution was assumed both for the time between failures and for the time to repair.

Maximum acceptable Mean Time To Repair

Maximum acceptable Mean Time To Repair (MTTR) for a given objective MTTRo, of the MTTR and a 10% probability of rejecting unduly the hypotheses that repair times comply with the objective.

| NUMBER OF FAILURES | MTTR (factual) ------------------- MTTRo |
| --- | --- |
| 1 2 3 4 5 6 7 10 15 | 2,3 1,95 1,77 1,67 1,60 1,55 1,50 1,40 1,34 |

Maximum acceptable Mean Time To repair (MTTR) for given objective MTTRo, of the MTTR and a 1% probability of rejecting unduly the hypotheses that repair times comply with the objective.

| NUMBER OF FAILURES | MTTR (factual) ------------------- MTTRo |
| --- | --- |
| 1 2 3 4 5 6 7 10 15 | 4,6 3,32 2,8 2,5 2,32 2,18 2,07 1,88 1,70 |

Maximum acceptable number of failures per year

Probability for having at the most x failures in one year.

| x MTBF unit=year | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 0,716 | 0,955 | 0,995 | 0,9996 | 0,9999 |  |  |  |
| 2 | 0,606 | 0,909 | 0,986 | 0,998 | 0,9998 | 0,9999 |  |  |
| 1,5 | 0,51 | 0,856 | 0,97 | 0,995 | 0,9994 | 0,9999 |  |  |
| 1 | 0,368 | 0,736 | 0,92 | 0,98 | 0,996 | 0,9994 |  |  |
| 0,5 | 0,135 | 0,406 | 0,677 | 0,857 | 0,947 | 0,983 | 0,995 | 0,9989 |
| 0,33 | 0,05 | 0,199 | 0,423 | 0,647 | 0,815 | 0,916 | 0,967 | 0,988 |

Example of a table that can be extracted from the previous one:

Maximum acceptable number of failures per year for a given objective MTBFo of the Mean Time Between Failures and a  1% probability of rejecting unduly the hypothesis that the times between failures comply with the objective.

| MTBFo unit=year | Maximum acceptable number of failures in one year |
| --- | --- |
| 3 2 1,5 1 0,5 (6 month) 0,33 (4 month) | 2 3 3 4 6 7 |