| From: EEP | EEP5 / output/ 03 |
| --- | --- |
| To: ANM | Date: Oct. 7, 2004 |

Liaison Note

Availability Objectives Recommendation

- Introduction

The Draft IALA Recommendation on Availability Objectives for Aids to Navigation was presented to IALA Council, Session 34.  This draft recommendation was not accepted, and it was identified that further work was required.

- Background

The EEP provided input to the ANM Committee on the creation of the Draft Recommendation for Availability Objectives for Aids to Navigation.  The EEP Committee reviewed the recommendation during EEP5, and provides the attached revisions.

- Action Requested

The ANM Committee is invited to review the attached revisions and implement the changes as appropriate.

AISM Association of Internationale de Signalisation Maritime                IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

IALA Recommendation O-###

On

Categorisation and Availability Objectives for Aids to Navigation

DRAFT

Edition 1

May 2004

20ter, rue Schnapper     -     78100     Saint Germain en Laye        -       France

Telephone +33 1 34 51 70 0 Telefax +33 1 34 51 82 05 Telex 695499 ialaism f

E-mail          :         iala-aism@wanadoo.fr      Internet      :      hjttp://iala-aism.org

DRAFT Recommendation on Categorisation and Availability Objectives for  Aids to Navigation Services

(Recommendation O-###)

THE COUNCIL,

RECALLING that one of the aims of the Association is to foster safe, economic and efficient movement of vessels by the improvement and harmonisation of aids to navigation world-wide;

RECALLING ALSO that IMO SOLAS Chapter V, Regulation 14, includes the requirement that Contracting Governments undertake to arrange for the establishment and maintenance of such aids to navigation as, in their opinion, the volume of traffic justifies and the degree of risk required;

RECALLING FURTHER that IMO Resolution A.953(23) identifies the required signal availability for world-wide radionavigation systems used in harbour entrances, harbour approaches and coastal waters is dependent upon the volume of traffic and the degree of risk in the area concerned;

RECOGNISING that IALA Recommendation E-105 (May 1998) on “The need to follow national and international standards” advises in Recommends (2), that purchasing authorities include availability and quality requirements in their specifications when procuring Aids to Navigation equipment;

RECOGNISING ALSO the importance of describing the management objectives for the operational performance levels of Aids to Navigation services provided to mariners and the need to provide guidance to National Members on suitable and realistic levels of operational performance;

NOTING that it is possible to identify the required level of availability during the design phase of Aids to Navigation services by taking into account the known theoretical relationship between individual component reliability and system availability;

NOTING ALSO that risk management practices enables the assessment of availability of an aid to navigation or system based upon its criticality to the mariner and for the protection of the marine environment;

HAVING CONSIDERED the proposal of the ANM Committee, prepared in consultation with the EEP Committee;

ADOPTS the Availability Objectives for Aids to Navigation, given in the Annex to this Recommendation; and

RECOMMENDS that National Members and other authorities providing Aids to Navigation categorise each aid to navigation or system in accordance with the categories set out in the Annex to this Recommendation.

*    *    *

ANNEX

AVAILABILITY FOR AIDS TO NAVIGATION

# Introduction

Availability for Aids to Navigation has traditionally been linked to the size and complexity of the aid to navigation concerned – for example, major lighthouses have been rated as Category 1 and light buoys as Category 3.  While this correlation has been relevant in the past for Aids to Navigation (AtoN), changes occurring in navigational safety requirements and in the technologies used in AtoN have indicated a need to review the basis on which availability is defined. .

The adoption of contemporary risk management practices enables AtoN management authorities to define, preferably in consultation with mariners and other stakeholders, the availability requirements for the aid to navigation concerned, and to assess its future categorisation based upon its navigational significance.  The resulting categorisation of the AtoN may result in some traditional higher category aids being downgraded and, alternatively, the potential for lower category aids to be upgraded.

For the purpose of this recommendation, Availability is defined as:

The probability that an Aid to Navigation as defined by the Competent Authority is performing its specified function at any randomly chosen time, expressed as a percentage of total time that an Aid to Navigation should be performing its specified function.

## Considerations

The Availability objective, should be based on a risk assessment methodology that assesses the navigational significance of an AtoN, taking into consideration factors such as:

- Waterway significance,

- Areas of environmentally sensitivity,

- Nature and type of cargo,

- Nature and type of navigation,

- Traffic density,

- Aids to Navigation coverage,

- Climate,

- National concerns.

## Assessment Aspects

The Availability objective, and the Availability achieved in practice, both depend on aspects such as:

- Existing technology,

- Logistics,

- Redundancy,

- Accessibility,

- Other navigational services available to the mariner including, Pilotage, VTS, GNSS, etc.

The Availability objective should, wherever practicable, be defined or confirmed in consultation with mariners and other stakeholders who use the particular AtoN to assist in their navigation.

Formal procedures for collecting, processing and recording Availability data should be established. Refer to related documentation list below.

# Categories for Aids to Navigation

Category 1: 	An Aid to Navigation (AtoN) that is considered by the Competent Authority to be of vital navigational significance. For example, lighted aids to navigation and racons that are considered essential for marking landfalls, primary routes, channels, waterways or new dangers.

Category 2: 	An AtoN that is considered by the Competent Authority to be of important navigational significance. For example, it may include any lighted aids to navigation and Racons that mark secondary routes and those used to supplement the marking of primary routes.

Category 3: 	An AtoN that is considered by the Competent Authority to be of necessary navigational significance.

# Availability Objectives for Aids to Navigation

The table below provides overall availability objectives for each category of the aids to navigation as provided by the Competent Authority.

| CATEGORY | OVERALL AVAILABILITY OBJECTIVE | CALCULATION PERIOD |
| --- | --- | --- |
| 1 | 99.8% | Availability Objectives are calculated over a three-year continuous period, unless otherwise specified |
| 2 | 99.0% |  |
| 3 | 97.0% |  |

It is emphasised that, where availability of an individual AtoN consistently falls below 95%, it could be a hazard to navigation and consideration should be given to the discontinuance of that AtoN if availability cannot be improved.

# Calculation of Availability

- The availability of an AtoN should be calculated using the following equation, using the most accurate timing available:

- Availability= Total Time−Down TimeTotal Time expressed in percentage (%)

Total Time is the time that an AtoN should be performing its specified function.

- Down Time is the sum of the periods during which the Aid to Navigation is unable to perform its specified function.  It does not include those periods when the mariner has been notified of a discrepancy by prior publication through a Preliminary Notice to Mariners. [include reference to the actual time (2 months) as per IHO for Preliminary Notice to Mariners]

- Related Documentation

The latest editions of the following IALA documents are relevant to the identification of availability objectives for aids to navigation or systems:

- IALA Aids to Navigation Guide (Navguide),

- IALA Guidelines on Risk Management,

- IALA Guidelines on Availability and Reliability [as revised – amend this if approved by Council],

- IALA Guidelines on Levels of Service.

Reference – as adapted from the IALA Guidelines on Availability and Reliability of Aids to Navigation.

At least 2 months notice as defined by the International Hydrographic Organization (IHO) Technical Resolution F3.5.