### [Possible Annex for Certification Guidelines ] ANNEX 2 - Product Templates

# Purpose of Product Templates

The purpose of a Product Template is

- To list the important parameters which should be specified for that type of product or group of products;

- To identify the testing standards / procedures which are acceptable to IALA for measuring each of the listed parameters, or the computational method which should be used for predicted, or similar, parameters.

The benefit of using the same Product Template for all similar IALA certified products is to ensure that products can be compared using the same parameters, with results derived from consistent test procedures.

Product Templates are not generally intended to specify performance levels, nor to set limits to parameter values. The evaluation of one or more products is the responsibility of the user or purchaser of the product or products.

# Content of Product Templates

Each Product Template should contain

- A list of the product types or groups of products to which the Product Template applies

- A table listing the important parameters which should be specified for that type of product or group of products,  and the test standards / procedures which are acceptable to IALA for measuring each of those listed parameters. Note that more than one test standard / procedure may be acceptable, and that any one of these test methods listed for a parameter may be employed to test that parameter

# Product types and product groups

The following list indicates the product types and product groups for which Product Templates are required.

- Product Template 1 - Lanterns for buoys and light-beacons, including enclosed rotating beacons

- Product Template 2 - Lighthouse optical systems and components, for use inside lantern houses

- Product Template 3 - Energy sources and related components, including energy generators, energy storage, and regulation or control devices

- Product Template 4 - Buoys, buoy moorings, and structures for land use and for offshore

- Product Template 5 - Radio-navigation equipment and AIS products, including AtoN AIS and AIS base station networks

# IALA AtoN Product Certification Template

# Product Template 1 - Lanterns for buoys and light-beacons, including enclosed rotating beacons

To be used in conjunction with the IALA Guidelines on the Certification of Marine Aids to Navigation Products.

# TABLE OF PERFORMANCE TEST STANDARDS

| Parameter | Test Specification | Compliance |
| --- | --- | --- |
| Peak intensity | IALA Recommendation E122 |  |
| Vertical divergence | IALA Recommendation E122 |  |
| Horizontal divergence | IALA Recommendation E122 |  |
| Light Chromaticity | IALA Recommendation E122 |  |
| Character/ Code | IALA Recommendation E110 |  |
| Power supply voltage range | IEC 60945 section 7, state voltage range |  |
| Power consumption, quiescent current | Standard not yet identified |  |
| Power consumption, full load current | Standard not yet identified |  |
| EMC Compatibility | IEC 60050-161 |  |
| Temperature range | IEC 60068-2-1; IEC 60068-2-2 |  |
| Vibration | IEC 60945 section 8.7 |  |
| Humidity | IEC 60068-2-30 |  |
| Immersion | IEC 60529 Table3, second character 7 |  |
| Salt tolerance | IEC 60068-2-52 |  |
| UV tolerance | IEC 60060-2-5 |  |
| Equipment manuals | IEC 60945 section 14 |  |
| Marking and identification | IEC 60945 section 15 |  |

Other Parameters to be included, specified by manufacturer:

- Physical size

- Weight

- Design lifetime

- Lens material

- Light source type

# IALA AtoN Product Certification Template

Product Template 2 - Lighthouse optical systems and components, for use inside lantern houses

To be used in conjunction with the IALA Guidelines on the Certification of Marine Aids to Navigation Products.

# TABLE OF PERFORMANCE TEST STANDARDS

| Parameter | Test Specification | Compliance |
| --- | --- | --- |

Other Parameters to be included, specified by manufacturer:

- Xx

- Xx

- Xx

- xx