## EEP Committee, WG4, Item 4.3, Task 14

LED Life tests in Netherlands Sipke Hoekstra

# Lantern ZENI  ZL-LS 221 G401 green	12 Volt - 6 W

Average intensity = 36,6 cd (fixed) measured  18-11-1996   in service  19-03-1997

Average intensity = 37,4 cd (fixed) measured  16-10-2001

After almost 5 years in service, with a character LFL 8, there is no degradation of the intensity. The lantern is still on the buoy and the performance is oké. I think it is interesting to measure the lantern once more at the end of the year.

Lantern

Nr86	3,6 W  	40% character     Green     Average intensity   79,6 cd  fixed 	08-02-2000

74,5 	   	21-02-2002

71,1              	08-03-2004

Nr92	3,6 W	40	                Green		      	79,4		08-02-2000

74,2		08-03-2004

Nr71	6,0 W *              fixed	           Green		                 112,2		08-02-2000

102,6		19-03-2004

*the lantern is measured at 6,0 W but is installed as a harbour light with 3,6 W.

As you can see the degradation differs a lot from lantern to lantern.

I think there is a lot of development of leds, more intensity, less consumption etc. Do the manufactures install for instance every year the latest generation of leds in LED lanterns? If so we will not get a good idee of the degradation of the lanterns. What is the policy of ZENI on this paticular item? Have you got some more information from colleagues and manufacturers? I' am interested to know the results.

# Summary of above by Mike Card

" It is not easy to generate data from field tests of lanterns in service, to show degradation of intensity with time, as initial lantern performance is not usually measured before a lantern is placed into use. In addition the degradation depends on a number of factors including diode current and flash character. The following data is taken from tests conducted in the Netherlands.

| Lantern | Initial intensity, candela, and date | Part life intensity and date | Period, years | Intensity as % of initial |
| --- | --- | --- | --- | --- |
| Green, Long Flash 8 sec. | 36.6, 1997-03-19 | 37.4, 2001-10-16 | 4.6 | 102% |
| Green, 40% character | 79.6, 2000-02-08 | 71.1, 2004-03-08 | 4.1 | 90% |
| Green, 40% character | 79.4, 2000-02-08 | 74.2, 2004-03-08 | 4.1 | 93% |
| Green, fixed character | 112.2, 2000-02-08 | 102.6, 2004-03-19 | 4.1 | 91% |