| IALA COUNCIL 34th session | May 24-26, 2004 Shanghai People’s Republic of China |
| --- | --- |

6 – IALA COMMITTEES

6.6	ANM COMMITTEE

6.6.2	Draft Recommendation on Availability of Aids to Navigation

The Draft Recommendation emphasises a shift in the categorization of AtoN from the size/range of the aid to its significance to navigation.  The Draft Recommendation has been developed with extensive consultation between ANM and EEP Committees.

The Council is invited to Approve the Draft Recommendation on Availability of Aids to Navigation.

Recommendation Not Approved.

Comments from Report of Council Session 34:

    - Draft Recommendation on Availability Objectives for Aids to Navigation

The Council in general found that the document was confusing, and leading to a number of questions, especially on the definition of categories, the basis for calculating the requested availability, and as to what it should be applied: individual aids or systems of aids? (They are both mentioned).

The Council decided that the document should be sent back to the Committee to better explain what the categories are and how they are defined.

AISM Association of Internationale de Signalisation Maritime                IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

IALA Recommendation O-###

On

Availability of Aids to Navigation Services

DRAFT

Edition 1

May 2004

20ter, rue Schnapper     -     78100     Saint Germain en Laye        -       France

Telephone +33 1 34 51 70 0 Telefax +33 1 34 51 82 05 Telex 695499 ialaism f

E-mail          :         iala-aism@wanadoo.fr      Internet      :      hjttp://iala-aism.org

DRAFT Recommendation Availability of Aids to Navigation Services

(Recommendation O-###)

THE COUNCIL,

RECALLING that one of the aims of the Association is to foster safe, economic and efficient movement of vessels by the improvement and harmonisation of aids to navigation world-wide;

RECALLING ALSO that IMO SOLAS Chapter V, Regulation 14, includes the requirement that Contracting Governments undertake to arrange for the establishment and maintenance of such aids to navigation as, in their opinion, the volume of traffic justifies and the degree of risk required;

RECALLING FURTHER that IMO Resolution A.953(23) identifies the required signal availability for world-wide radionavigation systems used in harbour entrances, harbour approaches and coastal waters is dependent upon the volume of traffic and the degree of risk in the area concerned;

RECOGNISING that IALA Recommendation E-105 (May 1998) on “The need to follow national and international standards” advises in Recommends (2), that purchasing authorities include reliability and quality requirements in their specifications when procuring Aids to Navigation equipment;

RECOGNISING ALSO the importance of describing the management objectives for the operational performance levels of Aids to Navigation services provided to mariners and the need to provide guidance to National Members on suitable and realistic levels of operational performance;

NOTING that it is possible to identify the required level of availability during the design phase of Aids to Navigation services by taking into account the known theoretical relationship between individual component reliability and system availability;

NOTING ALSO that risk management practices enables the assessment of availability of an aid to navigation or system based upon its criticality to the mariner and for the protection of the marine environment;

HAVING CONSIDERED the proposal of the ANM Committee, prepared in consultation with the EEP Committee;

ADOPTS the Availability for Aids to Navigation Services, given in the Annex to this Recommendation; and

RECOMMENDS that National Members and other authorities providing Aids to Navigation categorise each aid to navigation or system in accordance with the categories set out in the Annex to this Recommendation.

*    *    *

ANNEX

AVAILABILITY OF AIDS TO NAVIGATION SERVICES

# Introduction

Availability of Aids to Navigation Services have traditionally been linked to the size and complexity of the aid to navigation or system concerned – for example, major lighthouses have been rated as Category 1 and light buoys as Category 3.  While this correlation has been relevant in the past for Aids to Navigation (AtoN) and systems, changes occurring in navigational safety requirements and in the technologies used in AtoN have indicated a need to review the basis on which availability is defined. .

The adoption of contemporary risk management practices enables AtoN management authorities to define, preferably in consultation with mariners and other stakeholders, the availability requirements for the aid to navigation or system concerned, and to assess its future categorisation based upon its navigational significance.  The resulting categorisation of the AtoN or system may result in some traditional higher category aids being downgraded and, alternatively, the potential for lower category aids to be upgraded.

For the purpose of this recommendation, Availability is defined as:

The probability that an Aid to Navigation or a system of aids to navigation as defined by the Competent Authority is performing its specified function at any randomly chosen time, expressed as a percentage of total time that an Aid to Navigation or system should be performing its specified function.

## Considerations

The Availability should be based on a risk assessment methodology that assesses the navigational significance of an AtoN or system, taking into consideration factors such as:

- Waterway significance,

- Areas of environmentally sensitivity,

- Nature and type of cargo,

- Nature and type of navigation,

- Traffic density,

- Aids to Navigation coverage,

- Climate,

- National concerns.

## Assessment Aspects

The Availability also depends on aspects such as:

- Existing technology,

- Logistics,

- Redundancy,

- Accessibility,

- Other navigational services available to the mariner including, Pilotage, VTS, GNSS, etc.

The Availability should, wherever practicable, be defined or confirmed in consultation with mariners and other stakeholders who use the particular AtoN or system to assist in their navigation.

Formal procedures for collecting, processing and recording availability data should be established.

# Categories of Aids to Navigation

| CATEGORY | AVAILABILITY OBJECTIVE | CALCULATION PERIOD |
| --- | --- | --- |
| 1 | 99.8% | Availability Objectives are calculated over a three-year continuous period, unless otherwise specified |
| 2 | 99.0% |  |
| 3 | 97.0% |  |

Category 1: 	An Aid to Navigation (AtoN) or system of AtoN that is considered by the Competent Authority to be of vital navigational significance. For example, lighted aids to navigation and racons that are considered essential for marking landfalls, primary routes, channels, waterways or new dangers.

Category 2: 	An AtoN or system of AtoN that is considered by the Competent Authority to be of important navigational significance. For example, it may include any lighted aids to navigation and Racons that mark secondary routes and those used to supplement the marking of primary routes.

Category 3: 	An AtoN or system of AtoN that is considered by the Competent Authority to be of necessary navigational significance.

# Notes on Availability

- It is emphasised that, where availability of an AtoN consistently falls below 95%, then consideration should be given to the discontinuance of that AtoN;

- The availability of an AtoN should be calculated using the following equation, using the most accurate time available:

- Availability= Total Time−Down TimeTotal Time expressed in percentage (%)

- Where Total Time is the time that AtoN or System should be performing its specified function and Down Time is the period of time during which the aid to navigation is in the failure state as defined in Notes 3 & 4.

- The IALA Navguide, latest edition, should be consulted for further information on the method of calculating the availability.

- A failure state is the malfunction of an aid to navigation or system of AtoN to display its correct characteristics or to be on its assigned position for its intended use by the mariner. As such, a failure of a technical function is not necessarily considered an aid to navigation discrepancy. For example, if the main power supply has failed but the light continues to function at normal intensity on standby power, this is not considered a failure, since the aid to navigation continues to provide its correct characteristics to the mariner.

- The failure state may be caused by equipment malfunction, or scheduled or unscheduled maintenance work. However, if the unavailability of the aid to navigation has been advised by prior publication of a Preliminary Notice to Mariners, then the failure state should not be considered downtime.

# Related Documentation

The latest editions of the following IALA documents are relevant to the identification of availability objectives for aids to navigation or systems:

- IALA Aids to Navigation Guide (Navguide),

- IALA Guidelines on Risk Management,

- IALA Guidelines on Availability and Reliability,

- IALA Guidelines on Levels of Service.

Reference – as adapted from the IALA Guidelines on Availability and Reliability of Aids to Navigation.

At least 2 months notice as defined by the International Hydrographic Organization (IHO) Technical Resolution F3.5.