Input paper for the following Committee(s): 					Purpose of paper:

□  ARM	□  ENG		□  PAP					x  Input

X  ENAV	□  VTS								□  Information

Agenda item

Technical Domain / Task Number 2…………………………………

Author(s) / Submitter(s)：	Donghai Navigation Safety Administration, Shanghai

Shanghai Aerospace Electronic Technology Institute

# VDE-TER Multipoint Communication Field Tests in East China Sea

# Background

In order to promote the establishment of standards and the construction of marine data exchange systems, Shanghai Aerospace Electronic Technology Institute, together with Donghai Navigation Safety Administration (DNSA) MOT held three times of field tests on VDE-TER in Luchao Port and Yangshan in China eastern sea during 2019 and 2020. The test verified the performance and function of physical layer and link layer that meet the IALA G1139 2020 VDES standard. The test used one VDE-TER station and three ship stations. The performance of the system was tested in different communication distance, different speed and different heading. The communication process tests include ship to shore, shore to ship, ship to shore to ship were carried out. The test was carried out on different communication processes, information capacity, shore-to-ship distance, speed, and heading. The document introduces the test result of six frame formats on physical layer and seven communication processes of the link layer.

# system architecture

In this phase of the test, according to IALA G1139 2020, four typical physical layer frame formats (LINK ID11, ID12、ID14、ID15) have been implemented, with link layer functions. One shore and three ships in a single cell, ship-shore, shore-ship, ship- Shore-to-ship communication tests were carried out to verify physical layer communication performance and link layer communication function.

Figure 1  VDE-TER test system layered architecture diagram

- Physical layer: The test verifies the functions and performance of the newly developed VDES shore station and ship stations in real scenarios. The test verifies the physical layer performance of the four typical physical layer frame formats LINK ID11, ID12, ID13, ID14 that meet the G1139 standard, including receiving sensitivity and PER, etc.

- Link layer: verified 7 kinds of communication processes including broadcast, addressed message, and short message that meet with the G1139 standard, and verify functions such as time slot allocation and scheduling.

- Transport layer: Use TCP/IP protocol to realize communication between PC and VDE in both ship station and shore station.

- Application layer: PC-side software is used to realize data management of sending and receiving data among one shore station and three ships stations.

Figure 2  VDE-TER system connection diagram

- Test overview

  - Test equipment

The test equipment mainly includes 1 VDE shore station, 3VDE ship stations , VHF antenna, GPS antenna, ground test equipment (laptop) and related instruments and meters, etc. The connection relation of the ground test equipment is shown in Figure 3.

Figure 3  VDES terminal equipment connection diagram

  - Test environment

The test was carried out in the sea area along the Luchaogang-Donghai Bridge-Yangshan Island, as shown in the figure below. The VDES shore station is placed in the Luchao Port AIS station building, and the AIS base station antenna is used for signal transmission and reception. The antenna height is about 60m.

Figure 4  Test Environment

  - Experiment Procedure

The aim of this test includes: physical layer communication performance verification and link layer communication verification. According to the plan, the main test process is as follows:

(1) Route 1, ship 22015 took the test. The test ship departed from Yangshan Port (the distance between Yangshan Port and Luchao Port AIS Base Station is about 35km), first sailed to the southeast to 45km away from Luchao Port AIS Base Station, and then turned around to the direction of Luchao Port AIS Base Station Navigate to a distance of 2km from the ship's shore, and then return to Yangshan Port to test the physical layer PER. The route is shown in Figure 5.

Figure 5  Ship track map 1

(2) Route 2: The test ship 80009 took the test and sailed from Yangshan Port. It sailed southeast to a distance of 50km from Luchao Port AIS base station, then turned around and sailed towards Luchao Port AIS base station, to a distance of 5km from the ship's shore, and then returned to Yangshan Port for physical layer PER test. The route is shown in Figure 6.

Figure 6  Ship track map 2

(3) Route three, test ship 22015 (VDES terminal: ship 3), test ship 80009 (VDES terminal: ship 2) departed from Yangshan Port, and test ship 80001 (VDES terminal: ship 4) departed from Luchao Port. The three ships sailed within a certain distance range to carry out link layer tests. The specific distance ranges are shown in Figure 7 and Table 1.

Figure 7  Ship track map 3

Table 1 Link layer test ship station information

| Ship Name | Ship ID | Distance from shore station |
| --- | --- | --- |
| 22015 | Ship 2 | 30.3 km~ 49.8 km |
| 80009 | Ship 1 | 21.4 km~32.7 km |
| 80001 | Ship 3 | 2km~ 6.9 km |

- Test data recording and analysis

  - Physical layer performance test

    - Physical layer test data record

A total of 37 sets of uplink/downlink Link ID11 test data, 43 sets of Link ID12 test data, 21 sets of Link ID14 test data, and 19 sets of Link ID15 test data were collected in this experiment. The details are as follows.

Table 2  Link ID11 Test Record

| Number |  | Speed（knot） |  |  | Distance(km) | Packets send |  |  | Packets received |  | PER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| downlink |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 5.3 | 2.4 |  |  | 2000 | 1988 |  | 0.60% |  |  |
| 2 |  | 8.3 | 6.6 |  |  | 2000 | 1992 |  | 0.40% |  |  |
| 3 |  | 8.3 | 7 |  |  | 2000 | 1996 |  | 0.20% |  |  |
| 4 |  | 8 | 8.4 |  |  | 2000 | 1808 |  | 9.60% |  |  |
| 5 |  | 9.4 | 16.9 |  |  | 2000 | 1990 |  | 0.50% |  |  |
| 6 |  | 9.4 | 17.4 |  |  | 2000 | 1996 |  | 0.20% |  |  |
| 7 |  | 9.4 | 22 |  |  | 2000 | 1970 |  | 1.50% |  |  |
| 8 |  | 9.4 | 24 |  |  | 2000 | 1976 |  | 1.20% |  |  |
| 9 |  | 7 | 26.8 |  |  | 2000 | 1782 |  | 10.90% |  |  |
| 10 |  | 7 | 30.3 |  |  | 2000 | 1848 |  | 7.60% |  |  |
| 11 |  | 7.8 | 32.6 |  |  | 2000 | 1916 |  | 4.20% |  |  |
| 12 |  | 7.5 | 33.6 |  |  | 2000 | 1864 |  | 6.80% |  |  |
| 13 |  | 0 | 34.7 |  |  | 2000 | 1964 |  | 1.80% |  |  |
| 14 |  | 0 | 34.7 |  |  | 2000 | 1918 |  | 4.10% |  |  |
| 15 |  | 8.9 | 43.6 |  |  | 2000 | 1944 |  | 2.80% |  |  |
| 16 |  | 10.9 | 49.1 |  |  | 2000 | 1910 |  | 4.50% |  |  |
| 17 |  | 10.9 | 50 |  |  | 2000 | 1920 |  | 4.00% |  |  |
| uplink |  |  |  |  |  |  |  |  |  |  |  |
| 18 |  | 8.6 | 2.1 |  |  | 2000 | 1994 |  | 0.30% |  |  |
| 19 |  | 8.6 | 2.3 |  |  | 2000 | 1996 |  | 0.20% |  |  |
| 20 |  | 8.6 | 7.5 |  |  | 2000 | 1994 |  | 0.30% |  |  |
| 21 |  | 8.3 | 8 |  |  | 2000 | 1990 |  | 0.50% |  |  |
| 22 |  | 8 | 9.9 |  |  | 2000 | 1988 |  | 0.60% |  |  |
| 23 |  | 8 | 10.3 |  |  | 2000 | 1990 |  | 0.50% |  |  |
| 24 |  | 8 | 11 |  |  | 2000 | 1986 |  | 0.70% |  |  |
| 25 |  | 9.4 | 18 |  |  | 2000 | 1978 |  | 1.10% |  |  |
| 26 |  | 7 | 25.2 |  |  | 2000 | 1810 |  | 9.50% |  |  |
| 27 |  | 7 | 25.6 |  |  | 2000 | 1844 |  | 7.80% |  |  |
| 28 |  | 7 | 31.6 |  |  | 2000 | 1864 |  | 6.80% |  |  |
| 29 |  | 7.2 | 34.4 |  |  | 2000 | 1812 |  | 9.40% |  |  |
| 30 |  | 7.5 | 34.8 |  |  | 2000 | 1854 |  | 7.30% |  |  |
| 31 |  | 8.1 | 35.3 |  |  | 2000 | 1936 |  | 3.20% |  |  |
| 32 |  | 8.1 | 35.6 |  |  | 2000 | 1976 |  | 1.20% |  |  |
| 33 |  | 8 | 38.8 |  |  | 2000 | 1914 |  | 4.30% |  |  |
| 34 |  | 8.9 | 43.6 |  |  | 2000 | 1896 |  | 5.20% |  |  |
| 35 |  | 9.8 | 45.6 |  |  | 2000 | 1840 |  | 8.00% |  |  |
| 36 |  | 8.9 | 47 |  |  | 2000 | 1978 |  | 1.10% |  |  |
| 37 |  | 8.9 | 50 |  |  | 2000 | 1954 |  | 2.30% |  |  |

Table 3  Link ID12 Test Record

| Number |  | Speed（knot） |  | Distance(km) | Packets send |  |  | Packets received | PER |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| downlink |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 5.3 |  | 2.9 |  |  | 2000 | 1994 |  |  | 0.30% |
| 2 |  | 5.6 |  | 2.9 |  |  | 2000 | 1988 |  |  | 0.60% |
| 3 |  | 8.6 |  | 4.5 |  |  | 2000 | 1790 |  |  | 10.50% |
| 4 |  | 7.5 |  | 4.8 |  |  | 2000 | 1998 |  |  | 0.10% |
| 5 |  | 7.5 |  | 5.6 |  |  | 2000 | 1998 |  |  | 0.10% |
| 6 |  | 8.3 |  | 8.4 |  |  | 2000 | 1982 |  |  | 0.90% |
| 7 |  | 9.4 |  | 14.6 |  |  | 2000 | 1950 |  |  | 2.50% |
| 8 |  | 9.4 |  | 15 |  |  | 2000 | 1992 |  |  | 0.40% |
| 9 |  | 9.4 |  | 16.9 |  |  | 2000 | 1986 |  |  | 0.70% |
| 10 |  | 7 |  | 22.9 |  |  | 2000 | 1748 |  |  | 12.60% |
| 11 |  | 9.4 |  | 26.3 |  |  | 2000 | 1996 |  |  | 0.20% |
| 12 |  | 9.4 |  | 26.6 |  |  | 2000 | 1996 |  |  | 0.20% |
| 13 |  | 9.4 |  | 30 |  |  | 2000 | 1986 |  |  | 0.70% |
| 14 |  | 5 |  | 30.3 |  |  | 2000 | 1864 |  |  | 6.80% |
| 15 |  | 9.4 |  | 30.6 |  |  | 2000 | 1970 |  |  | 1.50% |
| 16 |  | 9.4 |  | 31.1 |  |  | 2000 | 1990 |  |  | 0.50% |
| 17 |  | 7 |  | 33.4 |  |  | 2000 | 1898 |  |  | 5.10% |
| 18 |  | 0 |  | 34.7 |  |  | 2000 | 1812 |  |  | 9.40% |
| 19 |  | 0 |  | 34.7 |  |  | 2000 | 1818 |  |  | 9.10% |
| 20 |  | 7 |  | 40 |  |  | 2000 | 1878 |  |  | 6.10% |
| 21 |  | 7 |  | 40.2 |  |  | 2000 | 1956 |  |  | 2.20% |
| 22 |  | 9.4 |  | 45.9 |  |  | 2000 | 1916 |  |  | 4.20% |
| 23 |  | 8.9 |  | 45.4 |  |  | 2000 | 1796 |  |  | 10.20% |
| 24 |  | 10.9 |  | 50 |  |  | 2000 | 1808 |  |  | 9.60% |
| 25 |  | 10.9 |  | 50.1 |  |  | 2000 | 1880 |  |  | 6.00% |
| uplink |  |  |  |  |  |  |  |  |  |  |  |
| 26 |  | 8.6 |  | 2.8 |  |  | 2000 | 1998 |  |  | 0.10% |
| 27 |  | 8.6 |  | 3.2 |  |  | 2000 | 1994 |  |  | 0.30% |
| 28 |  | 0 |  | 8.4 |  |  | 2000 | 1994 |  |  | 0.30% |
| 29 |  | 8 |  | 13 |  |  | 2000 | 1982 |  |  | 0.90% |
| 30 |  | 8 |  | 13.4 |  |  | 2000 | 1986 |  |  | 0.70% |
| 31 |  | 7 |  | 24.5 |  |  | 2000 | 1790 |  |  | 10.50% |
| 32 |  | 9.4 |  | 27 |  |  | 2000 | 1986 |  |  | 0.70% |
| 33 |  | 7.1 |  | 32.1 |  |  | 2000 | 1858 |  |  | 7.10% |
| 34 |  | 5 |  | 32.4 |  |  | 2000 | 1942 |  |  | 2.90% |
| 35 |  | 7.2 |  | 34 |  |  | 2000 | 1838 |  |  | 8.10% |
| 36 |  | 7 |  | 39.5 |  |  | 2000 | 1838 |  |  | 8.10% |
| 37 |  | 7 |  | 43 |  |  | 2000 | 1742 |  |  | 12.90% |
| 38 |  | 7 |  | 43.1 |  |  | 2000 | 1868 |  |  | 6.60% |
| 39 |  | 8.9 |  | 45.2 |  |  | 2000 | 1896 |  |  | 5.20% |
| 40 |  | 5 |  | 45.5 |  |  | 2000 | 1780 |  |  | 11.00% |
| 41 |  | 5 |  | 50 |  |  | 2000 | 1934 |  |  | 3.30% |
| 42 |  | 0 |  | 50.1 |  |  | 2000 | 1980 |  |  | 1.00% |
| 43 |  | 3 |  | 50.4 |  |  | 2000 | 1894 |  |  | 5.30% |

Table 4  Link ID14 Test Record

| Number | Speed（knot） | Distance(km) | Packets send | Packets received | PER |
| --- | --- | --- | --- | --- | --- |
| downlink |  |  |  |  |  |
| 1 | 5.3 | 2.4 | 2000 | 1978 | 1.10% |
| 2 | 8.3 | 7 | 2000 | 1998 | 0.10% |
| 3 | 9.4 | 16.9 | 2000 | 1990 | 0.50% |
| 4 | 9.4 | 22 | 2000 | 1978 | 1.10% |
| 5 | 7 | 26.8 | 2000 | 1785 | 10.75% |
| 6 | 7.8 | 32.6 | 2000 | 1945 | 2.75% |
| 7 | 0 | 34.7 | 2000 | 1911 | 4.45% |
| 8 | 8.9 | 43.6 | 2000 | 1944 | 2.80% |
| 9 | 10.9 | 49.1 | 2000 | 1913 | 4.35% |
| 10 | 10.9 | 50 | 2000 | 1910 | 4.50% |
| uplink |  |  |  |  |  |
| 11 | 8.6 | 2.1 | 2000 | 1980 | 1.00% |
| 12 | 8 | 9.9 | 2000 | 1981 | 0.95% |
| 13 | 8 | 10.3 | 2000 | 1980 | 1.00% |
| 14 | 9.4 | 18 | 2000 | 1978 | 1.10% |
| 15 | 7 | 25.2 | 2000 | 1830 | 8.50% |
| 16 | 7.5 | 34.8 | 2000 | 1854 | 7.30% |
| 17 | 8 | 38.8 | 2000 | 1924 | 3.80% |
| 18 | 8.9 | 43.6 | 2000 | 1896 | 5.20% |
| 19 | 9.8 | 45.6 | 2000 | 1830 | 8.50% |
| 20 | 8.9 | 47 | 2000 | 1958 | 2.10% |
| 21 | 8.9 | 50 | 2000 | 1950 | 2.50% |

Table 5  Link ID15 Test Record

| Number |  | Speed（knot） |  | Distance(km) | Packets send |  |  | Packets received | PER |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| downlink |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 5.3 |  | 2.9 |  |  | 2000 | 1984 |  |  | 0.80% |
| 2 |  | 8.3 |  | 8.4 |  |  | 2000 | 1982 |  |  | 0.90% |
| 3 |  | 9.4 |  | 15 |  |  | 2000 | 1992 |  |  | 0.40% |
| 4 |  | 7 |  | 22.9 |  |  | 2000 | 1848 |  |  | 7.60% |
| 5 |  | 9.4 |  | 26.6 |  |  | 2000 | 1996 |  |  | 0.20% |
| 6 |  | 9.4 |  | 30 |  |  | 2000 | 1976 |  |  | 1.20% |
| 7 |  | 0 |  | 34.7 |  |  | 2000 | 1882 |  |  | 5.90% |
| 8 |  | 7 |  | 40.2 |  |  | 2000 | 1936 |  |  | 3.20% |
| 9 |  | 8.9 |  | 45.4 |  |  | 2000 | 1816 |  |  | 9.20% |
| 10 |  | 10.9 |  | 50.1 |  |  | 2000 | 1780 |  |  | 11.00% |
| uplink |  |  |  |  |  |  |  |  |  |  |  |
| 11 |  | 8.6 |  | 3.2 |  |  | 2000 | 1994 |  |  | 0.30% |
| 12 |  | 0 |  | 8.4 |  |  | 2000 | 1984 |  |  | 0.80% |
| 13 |  | 8 |  | 13 |  |  | 2000 | 1982 |  |  | 0.90% |
| 14 |  | 7 |  | 24.5 |  |  | 2000 | 1790 |  |  | 10.50% |
| 15 |  | 9.4 |  | 27 |  |  | 2000 | 1956 |  |  | 2.20% |
| 16 |  | 5 |  | 32.4 |  |  | 2000 | 1942 |  |  | 2.90% |
| 17 |  | 7 |  | 39.5 |  |  | 2000 | 1838 |  |  | 8.10% |
| 18 |  | 8.9 |  | 45.2 |  |  | 2000 | 1886 |  |  | 5.70% |
| 19 |  | 0 |  | 50.1 |  |  | 2000 | 1760 |  |  | 12.00% |

    - Data Analysis

A total of 37 sets of Link ID11 test data were collected in the experiment, including 17 sets of downlink (shore-ship) data and 20 sets of uplink (ship-shore) data. Take multiple samples within the same distance to draw the relationship between unpacking rate and distance, as shown in Figure 8.

Figure 8 Relationship of distance and PER of LinkID 11

A total of 43 sets of Link ID12 test data were collected in the experiment, including 25 sets of downlink (shore-ship) data and 18 sets of uplink (ship-shore) data. Take multiple samples within the same distance to draw the relationship between the unpacking rate and the distance, as shown in Figure 9.

Figure 9 Relationship of distance and PER of LinkID 12

A total of 37 sets of Link ID11 test data were collected in the experiment, including 17 sets of downlink (shore-ship) data and 20 sets of uplink (ship-shore) data. Take multiple samples within the same distance to draw the relationship between PER and distance, as shown in Figure 10.

Figure 10 Relationship of distance and PER of LinkID 14

A total of 43 sets of Link ID12 test data were collected in the experiment, including 25 sets of downlink (shore-ship) data and 18 sets of uplink (ship-shore) data. Take multiple samples within the same distance to draw the relationship between PER and distance, as shown in Figure 11.

Figure 11 Relationship of distance and PER of LinkID 15

Link ID11, ID12, ID14, ID15 uplink/downlink can stably achieve PER 15% within the visible range of the shore-based antenna tower from 0 to 50km.

  - Link layer performance test

    - Link layer test data record

Take ship-shore addressing communication process as an example to show test data records of link layer communication tests. See Table 6 and Figure 12 Figure 13 Figure 14.

#### Table 6 Ship to shore addressed message data record

| Number | Source Ship | Shore Station | Remark |
| --- | --- | --- | --- |
| 1 | Ship 2： “hello，岸站！！！hello，岸站！！！hello，岸站！！！hello，岸站！！！hello，岸站！！！” | Success 24.2km | Figure11 |
| 2 | Ship 2： “站台，80009已收到！ 站台，80009已收到！ 站台，80009已收到！” | Success 24.2km |  |
| 3 | Ship 2： “80009已收到，满载而归！！ 80009已收到，满载而归！！ 80009已收到，满载而归！！” | Success 24.2km |  |
| 4 | Ship 3： “22015收到，准备返航！！ 22015收到，准备返航！！ 22015收到，准备返航！！” | Success 36.8km |  |
| 5 | Ship 3： “岸站，22015请求寻址通信 岸站，22015请求寻址通信 岸站，22015请求寻址通信” | Success 39.4km | Figure12 |
| 6 | Ship 3： “岸站，22015现在40km处，风浪尚可 岸站，22015现在40km处，风浪尚可 岸站，22015现在40km处，风浪尚可” | Success 40.2km |  |
| 7 | Ship 4： “80001已准备就绪 80001已准备就绪” | Fail 2km |  |
| 8 | Ship 4： “hello，VDE！！！80001，呼叫岸站，收到吗？” | Success 5.4km |  |
| 9 | Ship 4： “hello，VDE！！！80001，呼叫岸站，收到吗？15:23，时间” | Success 6.9km | Figure13 |

Figure 12 Ship2 to shore addressed message

Figure 13 Ship3 to shore addressed message

Figure 14 Ship4 to shore addressed message

    - Data Analysis

Table 7 Link layer communication test success rate

| Test Item | Ship | Communication Distance (From shore station) |  | Number of Test | Success Rate |
| --- | --- | --- | --- | --- | --- |
| Shore originated broadcast | Ship 1 | 21.5 km - 32.7 km |  | 3/3 | 100% |
|  | Ship 2 | 32.4 km – 46.8 km |  | 3/2 | 66.7% |
|  | Ship 3 | 5.4 km |  | 1/1 | 100% |
| Total |  |  |  | 7/6 | 85.7% |
| Shore to ship addressed message | Ship 1 | 21.4 km – 24.2 km |  | 3/3 | 100% |
|  | Ship 2 | 30.3 km – 40.9 km |  | 3/3 | 100% |
|  | Ship 3 | 2 km |  | 2/2 | 100% |
| Total |  |  |  | 8/8 | 100% |
| Shore to ship short message | Ship 1 | 24 km |  | 2/2 | 100% |
|  | Ship 2 | 41.4 km |  | 2/2 | 100% |
|  | Ship 3 | 2 km |  | 1/1 | 100% |
| Total |  |  |  | 5/5 | 100% |
| Ship to shore addressed message | Ship 1 | 24 km – 24.2 km |  | 3/3 | 100% |
|  | Ship 2 | 36.8 km – 40.2 km |  | 3/3 | 100% |
|  | Ship 3 | 2 km – 6.9 km |  | 3/2 | 66.7% |
| Total |  |  |  | 9/8 | 88.9% |
| Ship to shore short message | Ship 1 | 24 km |  | 1/1 | 100% |
|  | Ship 2 | 40 km |  | 2/2 | 100% |
|  | Ship 3 | 2 km |  | 3/3 | 100% |
| Total |  |  |  | 6/6 | 100% |
| Ship originated broadcast | Ship 2 | 42 km |  | 1/1 | 100% |
|  | Ship 3 | 5.4 km |  | 1/1 | 100% |
| Total |  |  |  | 2/2 | 100% |
| Ship to shore to ship addressed message | Ship1-ship2 | Ship1 | 28.5 km | 1/1 | 100% |
|  |  | Ship2 | 46 km – 46.4 km |  |  |
|  | Ship2-ship1 | Ship1 | 27 km – 30 km | 3/2 | 66.7% |
|  |  | Ship2 | 43.4 km – 49.8 km |  |  |
|  | Ship3-ship1 | Ship1 | 30.9 km | 1/1 | 100% |
|  |  | Ship3 | 6.8 km |  |  |
|  | Ship2-ship3 | Ship2 | 42 km | 1/1 | 100% |
|  |  | Ship3 | 5 km |  |  |
| Total |  |  |  | 6/5 | 83.3 % |
| Total |  |  |  | 44/41 | 93.2% |

The link layer communication test was carried out 44 times (including 3 failed) tests, covering 7 communication processes. The shore-to-ship communication distance ranged from 2km to 49.8km, and the average communication success rate was 93.2%.

- Conclusion

In this VDE-TER multipoint communication test, 4 typical physical layer frame formats and 7 typical link layer communication process that comply with the latest G1139 Recommendation were tested and verified. The test results show that the unpacking rate of the four physical layer frame formats Link ID11, Link ID12, Link ID14, and Link ID15 can reach less than 15% of PER in the communication range of the test base station antenna height. The test has verified 7 link layer communication processes including ship-shore, shore-ship, ship-shore-ship within the communication range covered by the base station, and the average communication success rate is 93.2%, laying a good foundation for the application of VDES business Foundation.

- Demand for committees

We sincerely hope that the organizing committee and peer experts could pay attention to the experimental conclusions of this experiment and advance the research of the VDES system on this basis. We also hope that the organizing committee and experts to give constructive guidance on VDE-SAT and VDE-TER frequency reuse and switching issues and VDES application and application services.

Leave open if uncertain