Report to The IALA Council on the Future of Maritime Radio Communications

[review for formatting and consistency of language]

Introduction

In 2004 the IALA Council approved a new work item for the Radionavigation Committee, “Develop Guidance on the Future of Maritime Radio Communications”. This initial report reviews the driving factors, new technologies and the maritime applications likely to be affected. It is proposed that the study should continue during the next Committee cycle.

Drivers

- Spectrum: Increasing pressure on the spectrum for commercial mobile telecom applications. In particular radar designers are being pushed towards technology that will use less bandwidth and reduce spurious emissions. This will eventually free up spectrum and make possible alternative uses of adjoining bands. This has major consequences for racon design.

- Increasing need for a common communication platform for two-way data communication between ship and shore.

- There is a growing requirement for Internet access on ships, at sea as well as in port.

- e-Navigation: great interest is being shown in the potential for integrating sources of navigation information to provide the user with the optimum, relevant data on a single display, e.g. the IALA e-ANSI project – communicating AtoN status information to the navigator.

- Commercial pressures, e.g. 3G providers, are increasingly anxious to find markets for their services. This may provide opportunities for new ways of communicating and presenting navigation information to the user.

- Non-safety related communication pressures on existing systems, e.g. messaging on AIS

- ITU is developing technical requirements for MF/HF data exchange services as a possible alternative to NBDP (radio telex) and has also initiated studies on VHF digital technology.

Candidate Technologies

- New services are envisaged for the broadcast over IP networks of Maritime Safety Information (MSI), using HF and satellite services such as Inmarsat BGAN (Broadband Global Area Network). Other IP network systems might also be applicable.

- UWB (Ultra Wide Band) – Spread spectrum applications include short range radars and wireless networks. This technology presents opportunities, but may also pose a threat to existing services, since it raises the noise level.

- Digital radio technology, including CDMA and TDMA, is applicable to many uses in maritime data communication, where spectrum is scarce and error-free communication is required. At the same time discussion are taking place regarding the possible sub-division of maritime mobile communication bands, especially VHF.

Administrative measures

- A major factor in the growing problems for maritime radio communication is the lack of radio spectrum. There are several regulatory measures that could be applied, e.g. subdivision of present channels as well as applying channel sharing between services. The introduction of digital technologies may also help to overcome these problems. Because of the substantial user base in some applications,  these measures will require a long lead-time as well as a long implementation time. This highlights the need to inform ITU of future spectrum requirements for maritime services.

Maritime Applications

- Communication

  - AIS:

    - ITU has assigned two VHF frequencies, 161.975 MHz and 162.025 MHz worldwide for AIS purposes on a primary, non-exclusive basis. Interference problems in some parts of the world have indicated that more protection may be required.

    - There is increasing pressure to utilise AIS for non-safety related messages thus potentially overloading the AIS channels. This type of communication is important but should be taken care of by other communication methods than AIS. Agreement is needed on the technology to use.

  - Long Range Identification and Tracking (LRIT) [Nick to provide text]

  - SAR Search and Rescue applications need priority. The current attitude among  regulators is to allow market demand to dictate spectrum allocation. If allowed to go unchallenged, this approach could lead to serious degradation of SAR services and consequent risk to life.

  - VTS

    - VTS-stations handle a large amount of voice traffic on several different VHF-channels. The problem is the severe lack of free VHF-channels, increasing VHF-traffic and new VHF service requirements. The situation is particularly bad in areas of overlapping coverage from different VTS-areas, even from different countries. The introduction of AIS may reduce the need for VTS VHF voice traffic.

  - NAVTEX Discussions are taking place on the future of NAVTEX. The development or eventual replacement of NAVTEX will have an effect on maritime services and available spectrum.

  - GMDSS The long gestation period of GMDSS saw many problems with training and false alarms. These appear to have been addressed by maritime administrations, but there is likely to be continuing pressure for improvement.

  - SafetyNet This Inmarsat MSI service would be one possible replacement for NAVTEX.

  - Maritime Voice Communication on VHF: The main development here is likely to be reduced channel spacing and the move towards digital technology

  - Maritime Data Communication

- Radionavigation

  - GNSS

    - Additional signals on GPS – L2C, L5 and eventually GPS 3 will increase the applications and improve accuracy and integrity

    - It is planned to restore GLONASS to a full 18 satellite constellation by 2008

    - Galileo is scheduled to become operational by 2008 and will provide several levels of service on multiple signals

  - DGNSS

    - DGNSS can be accomplished using several different methods: IALA DGNSS, AIS, Eurofix, e-Loran, SBAS

    - The frequency band 283.5-325 kHz is allocated for Radio-navigation, but regulatory changes may be needed to ensure its continued availability for the  IALA DGNSS

    - AIS may be used as a communication channel for a DGNSS service

    - Loran transmissions may be used as a communication channel for a DGNSS service, i.e. Eurofix or 9th pulse modulation

    - Additional means of radio communication may be required for fast update rate DGNSS applications

- Radar

  - VTS

    - Shore based radars are likely to be affected by changes in technology required to meet more stringent limits on spurious and out of band emissions

  - Radar AtoN

    - The racon continues to be useful on both the X-band (3 cm) and S-band (10 cm).

    - However, IMO MSC 79 has issued new radar performance requirements that no longer require S-band (10 cm) radars to trigger or detect racons. New ITU rules are forcing changes to S-band (10 cm) radars and existing racons will not respond to new S-band (10 cm) radars.

    - It is too soon to know which direction future radar development is going to take and therefore it is too soon to decide which direction future racon development should take. CIRM is forming a special interest working group to study this problem.

Recommendations

The Council is invited to note this report and approve the inclusion of this item in the future work program of the Committee.[bring forward to cover note to council as well] This would involve continued studies and periodic reporting on the future developments in maritime radio communications and preparation of standards as appropriate.

References

- RNAV22/6/1, Draft Submission to WRC07 –AIS matters/J. Hersey

- RNAV22/4/3, IMO Report – COMSAR9 / J.C.LeClair

- RNAV22/9/1, Future of Radar Aids to Navigation [update to RNAV23 paper]

- RNAV22/9/3, Report from CIRM / P.Mueller[don’t need this when updating RNAV22/9/1 to RNAV23 paper]