# GPS Vulnerability to high-powered radar

## Introduction

In May of 2000, reports of electronic equipment damage on commercial maritime vessels were received by the US Department of the Navy.  Reports indicated that officers on board maritime surface vessels were reporting to both the US Coast Guard and US Navy that their on-board commercial GPS units were no longer functioning after instances of close operating distances with Naval War Ships.  An investigation into the potential for air surveillance radars to interfere with or damage commercial GPS determined that there is a serious potential for commercial GPS burnout from high power microwave radars commonly found on Naval War Ships. Investigation revealed no previously reported connection between radars and commercial GPS, no apparent allowance for such effects in the design standard for commercial GPS, no operating guidance for damage avoidance. It was also noted that commercial use of GPS is spreading quickly globally, and that some commercial GPS units are experiencing a high failure rate, for reasons unknown.

In August 2003 a joint USCG/USN investigation was initiated to collect data on the Electromagnetic Vulnerability (EMV) of various commercial GPS units by conducting a controlled test using selected frequencies and power levels of common high power naval radars.  EMV test results substantiated the investigation. Test results will be used as rationale for the modification of the International Electrotechnical Commission (IEC) Receiver Equipment Performance Standard 61108-1. Continuing efforts are underway to educate GPS manufacturers, Maritime operators, Naval system designers, and Military surface ships on the potential for commercial GPS failure and make recommendations that could reduce susceptibility and failures, protect future consumers and save on replacement costs.

Commercial GPS Design Characteristics

Typical Examples:

• Power Handling 1 Watt & Antenna Gain –1 dBi @ 90degree (deg) elevation (el)

• Power Handling 1 Watt CW, 15-20 dB Filtering & Antenna Gain 2 to –3.8 dBi

• In Band Power Handling 0.5 Watt:1575 +/- 100 MHz, 50 dB min Antenna Gain, 90 deg el +7.5 dBi, 20 deg –1.5 dBi, 5 deg –5.5 dBi, 0 deg –7 dBi

GPS antennas require a broad beam width typically 160 deg. At GPS frequency, gain can increase at higher elevations.

Analysis of naval radar outputs through simple calculations suggests that (air surveillance) radars have the potential to generate peak power high enough to damage GPS antennas at significant distances assuming 1 Watt power handling and 0dBi gain.

beam width typically 160 degrees

Two Specific Interference tests required by IEC for compliance not be reduced by turning off GPS

• Subject to 3W/m2 @ 1636.5MHz for 10 min. and recover within 5 min. after unwanted signal is removed.

• Subject to burst of 10 pulses, each 1.0 to 1.5 ms long with duty cycle 1600:1 @ 2.9 to 3.1 GHz at power of about 7.5 kW/m2 for 10 min. and recover within 5 min. after unwanted signal is removed.

Test results

Antenna failure within 20 seconds appears to indicate failure as a result of energy/heat within a single pulse burst rather than accumulative effect over time.

##### Recommendations for Damage Avoidance

- Commercial maritime operators to be advised to purchase GPS Receivers with higher power handling capacity & IMO Compliancy

- If possible, have commercial ships stand off at Least 1 n.m. From Military Vessels

- Covering GPS Antenna with conductive material or stowing antenna when in the vicinity of high power radars reduces chance of equipment damage

- Burnout susceptibility will not be reduced by turning off GPS receivers as perceived by some commercial operators

ed by some commercial operators

###### Summary

• Testing proved that some commercial GPS Antennas with active electronics are susceptible to high power radar emissions

- Determining which ones are susceptible is an unknown

• Military radars are getting larger and transmitting more power NOT Less

## References

RTCM 2004 Annual Assembly Proceedings. Commercial GPS Susceptibility. S.Williams.

Received Levels for Different Navy Radars