| Source: Document 8B/441 Annex 8 | INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- | --- |
|  | RADIOCOMMUNICATION STUDY GROUPS | Document 8B/TEMP/224-E |
|  |  | 11 September 2006 |
|  |  | English only |

Working Party 8B

DRAFT revision of RECOMMENDATION itu-r m.585-3

Summary of revision

Recommendation ITU-R M.585-3 provides guidance for assigning maritime mobile service identities (MMSI) for ship stations. The draft revision to this recommendation has added text to provides guidance for assigning MMSI to coast stations, aircraft participating in search and rescue operations, aids to navigation, and craft associated with a parent ship.

This proposed draft revision:

–	changes the name of summary section to scope, modified text in this section to include aircraft, ATON, and craft associated with another vessel

–	modifies considering d) to include aircraft, ATON, and craft associated with another vessel

–	adds new recommends 3

–	modifies Recommendations 1, 2 and 3

–	modifies Annex 1

–	moves Annex 2 to Annex 6

–	adds a new Annex 2, Assignment of identification to coast station

–	adds a new Annex 3, Assignment of identification to aircraft

–	adds a new Annex 4, Assignment of identification to AIS Aids to Navigation (AtoN)

–	adds a new Annex 5, Assignment of identification to a parent ship.

DRAFT revision of RECOMMENDATION itu-r m.585-3*

Assignment and use of maritime mobile service identities

(1982-1986-1990-2003)

Scope

This Recommendation provides guidance to administrations for the assignment and conservation of maritime mobile service identities (MMSIs). Formats for ship stations, coast stations, aircraft participating in search and rescue operations, aids to navigation, and craft associated with a parent ship are described as well as some limitations which constrain assignments for ships which utilize the satellite services of the global maritime distress and safety system (GMDSS). Guidance is provided for future mobile satellite systems and the reuse of decommissioned numbers.

The ITU Radiocommunication Assembly,

considering

a)	the need for a unique ship identity for safety and telecommunication purposes;

b)	that the unique number should be the maritime mobile service identity (MMSI);

c)	the need for this identity to be usable with automated radiocommunication systems;

d)	that the identities assigned to ship stations, coast stations, aircraft participating in search and rescue operations, aids to navigation, craft associated with a parent ship, and used for establishing group calls should be of a similar nature;

e)	Article 19 of the Radio Regulations);

f)	that it is possible to use the MMSI to establish a telephone call to a ship after routing through the public switched networks to an appropriate coast station;

g)	that the public switched networks in many countries have restrictions on the maximum number of digits that may be dialled or keyed to identify the called ship station and the routing coast station, which would prevent the translation of the complete MMSI directly into a diallable number for the ship that is compliant with ITU-T Recommendation E.164;

h)	that whatever restrictions may be required should, in the interests of the development of automatic shore-to-ship operations, be as few as possible;

j)	that mobile-satellite systems enable the maritime community to participate in or interwork with international public correspondence telecommunication systems on a fully automatic basis, utilising the numbering, naming and addressing scheme pertaining to the service being used;

k)	that the current generation of mobile-satellite systems participating in the Global Maritime Distress and Safety System have signalling and routing characteristics requiring ships using these networks to have an MMSI ending with three zeroes;

l)	that the numbering scheme specified for future generations of mobile-satellite systems participating in the Global Maritime Distress and Safety System will be designed to meet the needs of the international public correspondence service and is unlikely to offer the facility to incorporate any part of the MMSI in a diallable number for a ship,

noting

that Article 19 contains provisions for the assignment of MMSIs,

recommends

1	that ships complying with the International Convention for the Safety of Life at Sea, 1974, as amended, and other ships equipped with automated radiocommunication systems, including Automatic Identification Systems (AIS), Digital Selective Calling (DSC), and/or carrying alerting devices of the Global Maritime Distress and Safety System (GMDSS) should be assigned maritime mobile service identities in accordance with Annexes 1, 2, 3, 4 or 5 to this Recommendation, as appropriate;

2	that ship, coast stations, and aircraft participating in search and rescue operations using digital selective calling equipment in accordance with Recommendation ITU-R M.493 should use their 9-digit numerical identities transmitted as a 10 digit address/self-identity, normally with a digit 0 added at the end of the identity (see also Recommendation ITU-R M.1080);

3	that ship, coast stations, and non-shipborne stations using Automatic Identification System (AIS) equipment in accordance with Recommendation ITU-R M.1371 should use their 9-digit numerical identities;

4	for the purpose of ensuring compatibility with the Global Maritime Distress and Safety System , the numbers, names and addresses of ship earth stations participating in international telecommunication services should be made readily available to all authorized entities by the telecommunication service providers concerned;

5	that the guidance given in Annex 6 to this Recommendation should be employed regarding the reuse of MMSI numbers, particularly those with three trailing zeroes.

Annex 1

Assignment of identification to ship station

1	Ships participating in the maritime radio services mentioned in recommends 1 should be assigned a nine digit unique ship station identity in the format M1I2D3X4X5X6X7X8X9 where the first three digits represent the Maritime Identification Digits (MID) and X is any figure from 0 to 9. The MID denotes the geographical area of the administration responsible for the ship station so identified.

2	Restrictions may apply with respect to the maximum number of digits, which can be transmitted on some national telex and/or telephone networks for the purpose of ship station identification.

3	The maximum number of digits that could be transmitted over the national networks of many countries for the purpose of determining ship station identity was six. The digits carried on the network to represent the ship station identity are referred to as the “ship station number” in this text and in the relevant ITU-R Recommendations. The use of the techniques described below should have made it possible for the coast stations of such countries to engage in the automatic connection of calls to ship stations.

To obtain the required nine digit ship station identity a series of trailing zeros would have to be added automatically to the ship station number by the coast station in order to complete a shore-originated telephone call, for example, carried over the public switched telephone network:

Ship station number	Ship station identity

M1I2D3X4X5X6	M1I2D3X4X5X6070809

4	In accordance with the above, and the relevant ITU-T Recommendations, a numbering plan was instituted for Inmarsat standard B, C and M systems participating in the GMDSS, which also requires that MMSIs with three trailing zeroes be assigned to ships fitting standard B, C and M ship earth stations.

5	The above restrictions do not necessarily apply to Inmarsat Standard C systems,  as they are not diallable terminals from the public switched telephone network but are only data terminals.

6	With respect to Inmarsat standard B and M systems and as long as the above restrictions apply, ships reasonably expected to be affected by the above limitations should only be assigned ship station identities with X7X8X9  000.

7	Group ship station call identities for calling simultaneously more than one ship are formed as follows:

01M2I3D4X5X6X7X8X9

where the first figure is zero and X is any figure from 0 to 9. The MID represents only the territory or geographical area of the administration assigning the group ship station call identity and does not therefore prevent group calls to fleets containing more than one ship nationality.

8	With the evolution of global mobile-satellite systems, ships earth stations are able to participate in international public correspondence telecommunication services. Ship earth stations having this functionality may be assigned international telecommunication numbers that have no direct correspondence with the ship station MMSI. Those authorized to assign the numbers, names and addresses associated with such ship earth stations should maintain a record of the cross reference relationships with the MMSI, for example in an appropriate database. For the purposes of GMDSS the details of these relationships should be made available to authorized entities such as but not limited to the Rescue Coordination Centres (RCCs). Such availability should be on an automatic basis, 24 hours per day 365 days per year.

Annex 2

Assignment of identification to coast station

1	Coast stations and other stations on land participating in the maritime radio services mentioned in recommends 2 should be assigned a nine-digit unique coast station identity in the format 0102M3I4D5X6X7X8X9 where the digits 3, 4 and 5 represent the Maritime Identification Digit (MID) and X is any figure from 0 to 9. The MID reflects the territory or geographical area in which the coast station or coast earth station is located.

2	As the number of coast stations decreases in many countries, an administration may wish to assign MMSI numbers of the format above to harbour radio stations, pilot stations and other stations participating in the maritime radio services. The stations concerned should be located on land or on an island in order to use the 00MIDXXXX format.

3	The administration may use the sixth digit to further differentiate between certain specific uses of this class of MMSI number, as shown in the example applications below:

a)	00MID1XXX		Coast radio stations

b)	00MID2XXX 	Harbour radio stations

c)	00MID3XXX		Pilot stations, etc.

4	This format scheme creates blocks of 999 numbers for each category of stations, however the method is optional and should be used only as a guidance. Many other possibilities exist if the administration concerned wishes to augment the scheme.

5	Group coast station call identities for calling simultaneously more than one coast station are formed as a subset of coast station identities, as follows:

0102M3I4D5X6X7X8X9

where the first two figures are zeros and X is any figure from 0 to 9. The MID represents only the territory or geographical area of the administration assigning the group coast station call identity. The identity may be assigned to stations of one administration which are located in only one geographical region as indicated in the relevant ITU-T Recommendations.

6	The combination 0102M3I4D506070809 should be reserved for a Group Coast Station Identity and should address all 00MIDXXXX stations within the administration. The administration may further augment this use with additional Group Call identities, i.e. 00MID1111, etc.

7	For the purpose of the GMDSS the details of these MMSI numbers assignments should be made available to authorized entities such as, but not limited to, Rescue Coordination Centres (RCCs). Such availability should be on an automatic basis, 24 hours per day 365 days per year.

8	The combination 010293949506070809 is reserved for the All Coast Stations Identity and should address all VHF 00XXXXXXX stations. It is not applicable to MF or HF coast stations.

Annex 3

Assignment of identification to aircraft

1	When an aircraft is required to use maritime mobile service identities for the purposes of conducting search and rescue communications with stations in the maritime mobile service, the responsible administration should assign a nine digit unique aircraft identity, in the format 111213M4I5D6X7X8X9 where the digits 4, 5 and 6 represent the Maritime Identification Digit (MID) and X is any figure from 0 to 9. The MID represents only the territory or geographical area of the administration assigning the aircraft call identity.

2	The format shown above will accommodate 999 aircraft per MID. If the administration concerned has more Search and Rescue (SAR) aircraft than 999 they may use an additional country code (MID) if it is already assigned by the ITU.

3	The administration may use the seventh digit to differentiate between certain specific uses of this class of MMSI number, as shown in the example applications below:

a)	111MID1XX		Fixed-wing aircraft

b)	111MID5XX 		Helicopters

4	This format scheme creates blocks of 99 numbers for each of the category of stations, however, the method shown here is optional.

5	The combination 111213M4I5D6070809 should be reserved for a Group Aircraft Identity and should address all 111MIDXXX stations within the administration. The administration may further augment this with additional Group Call identities, i.e. 111MID111, etc.

6	For the purpose of the GMDSS the details of these MMSI assignments should be made available to authorized entities such as, but not limited to, Rescue Coordination Centres (RCCs). Such availability should be on an automatic basis, 24 hours per day 365 days per year.

7	The MMSIs assigned to SAR aircraft should also be available from the ITU MARS database (see No. 20.16).

Annex 4

Assignment of identification to AIS Aids to Navigation (AtoN)

1	When a means of automatic identification is required for a station aiding navigation at sea, the responsible administration should assign a nine digit unique number in the format 9192M3I4D5X6X7X8X9 where the digits 3, 4 and 5 represent the Maritime Identification digit (MID) and X is any figure from 0 to 9. The MID represents only the territory or geographical area of the administration assigning the call identity for the navigational aid.

2	The format shown above applies to unmanned AIS AtoN floating in the water and virtual AIS AtoN belonging to aids to navigation systems; however, aids to navigation systems mounted ashore on fixed land or on an island should be assigned an identification number in the format given in Annex 2.

3	The format scheme shown above will accommodate 10 000 AtoN per MID. If the administration concerned has more than 10 000 they may use an additional country code (MID) if it is already assigned by the ITU giving a further 10 000 identities.

4	The administration may use the sixth digit to differentiate between certain specific uses of the MMSI number, as shown in the example applications below:

a)	99MID1XXX		Physical AIS AtoN

b)	99MID6XXX 	Virtual AIS AtoN

5	This format scheme creates blocks of 999 numbers for each category of station, however the method shown here is optional and should be used only as a guidance.

6	In addition to the use of the sixth digit to differentiate between specific navigational aids as explained above, the seventh digit may be used for national purposes, to define areas where the AIS AtoN are located or types of AIS AtoN to the discretion of the administration concerned.

7	The details of these MMSI number assignments should be made available to the International Association of Marine Aids to Navigation and Lighthouse Authorities (IALA) and appropriate national authorities.

8	The assigned MMSI to aids of navigation should also be available from the ITU MARS database (see No. 20.16).

Annex 5

Assignment of identification to craft associated with a parent ship

1	Devices used on craft associated with a parent ship, need unique identification. These devices which participate in the maritime mobile service should be assigned a nine digit unique number in the format 9182M3I4D5X6X7X8X9 where the digits 3, 4 and 5 represent the Maritime Identification digit (MID) and X is any figure from 0 to 9. The MID represents only the territory or geographical area of the administration assigning the call identity for the craft associated with a parent ship.

2	This numbering format is only valid for devices on board crafts associated with a parent ship. A craft may carry multiple devices for which an MMSI is required. These devices may be located in lifeboats, life-rafts, MOB-boats or other craft belonging to a parent ship.

3	A unique MMSI should be assigned for each device onboard a craft associated with a parent ship and will have to be separately registered and linked to the MMSI of the parent ship.

4	The format scheme shown above will accommodate 10 000 devices of this type per MID. If the administration concerned has more than 10 000 they may use an additional country code (MID) if it is already assigned by the ITU giving a further 10 000 identities.

5	The assigned MMSI to these devices should also be available from the ITU MARS database (see No. 20.16).

Annex 6

Guidance on the conservation and management of MMSI numbers

1	Administrations should employ the following measures to manage the limited MMSI numbering resource, particularly for the reuse of MMSIs with three trailing zeroes, in order to avoid depletion of MIDs and the corresponding MMSI number series:

a)	implement effective national procedures for MMSI assignment and registration taking into account the requirements of Resolution 340 (WRC-97);

b)	provide the Bureau with regular updates of assigned numbers in conformity with No. 20.16,

c)	ensure that the period from the expiration of the ship station licence associated with the number assignment until the date of reassignment of that number is sufficient for the changes to be incorporated in the relevant ITU service publications, e.g., Lists V and VIIa, taking into account the standard intervals between successive published editions;

d)	ensure that when ships move from the flag of registration of one administration to that of another administration, all of the assigned means of ship station identification, including the MMSI number, are reassigned as appropriate and that the changes are notified to the Bureau as soon as possible (see No. 20.16).

2	It is essential for a lapsed number assignment to remain dormant for a number of years before taking it back into use again, in order to avoid confusion over the origin of distress communications or over the responsible parties for billing and reconciling accounts for maritime radiocommunications.

3	The objective is to ensure that a period of five years or a period spanning two successive editions of List VIIa, whichever is the greater, should pass before a lapsed MMSI number is reused and entered into national and international databases pursuant to Resolution 340 (WRC-97) and No. 20.16.

4	Administrations could also apply the above procedures to MMSI numbers assigned with 2, 1 or no trailing zeroes in the interests of the long term conservation of MMSI and MID resources. However these number formats are normally not critical to the assignment of an additional MID to an administration (see Section VI of Article 19).

______________

*	 	This Recommendation should be brought to the attention of IALA, ICAO, IHO and IMO.

IMO Resolution A.888(21) requires that distress priority communications in these systems should, as far as possible, be routed automatically to an RCC.