e-NAV10	Input paper

Agenda item	8

Task Number	6

Author(s)	N.Ward

New Work Item Proposal for IMO to Prepare Performance Standards for Multi-system Shipborne Receivers

# 1	SUMMARY

This paper represents the first draft of a proposed request to IMO MSC90, prepare new Performance Standards for Multi-System Shipborne PNT receivers relevant to anticipated developments within e-Navigation.

## 1.1	Purpose of the document

The committee is requested to develop the paper and submit as an input to IMO MSC90.

## 1.2	Related documents

This paper is related to input papers regarding the draft receiver performance standard (to NAV58), discussion of the Maritime Integrated PNT System, and resulting Gap Analysis.

# 2	BACKGROUND

With PNT defined as one of the core elements of e-Navigation, it is necessary to provide updated and consolidated receiver performance standards to meet these requirements.

# 3	DISCUSSION

The draft document requests that IMO undertake a new work item, to develop modern PNT receiver performance standards. It is envisaged that this work would be undertaken at and between the NAV58 and NAV59 meetings.

# 4	REFERENCES

- IMO Resolution A.915(22) Revised Maritime Policy and Requirements for a Future Global Navigation Satellite System (GNSS).

# 5	ACTION REQUESTED OF THE COMMITTEE

The Committee is requested to:

- Finalise an input to MSC 90, requesting that IMO develop Performance Standards for a multi-system receiver.

- New Work Item Proposal for IMO MSC90

- .

| INTERNATIONAL MARITIME ORGANIZATION |  | E |
| --- | --- | --- |

|  | IMO |  |  |
| --- | --- | --- | --- |
| MARITIME SAFETY COMMITTEE 90th session Agenda item [23] |  | MSC 90/[23]/XX xx Jan 2012 Original: ENGLISH |  |

WORK PROGRAMME

Sub-Committee on Safety of Navigation

New Work Item Proposal to Prepare Performance Standards for Multi-system Shipborne Receivers

Submitted by [Denmark], [France], [Netherlands], [Norway], [IALA], [CIRM]

| SUMMARY |  |
| --- | --- |
| Executive summary: | This document proposes that a new Work Item be approved for the Safety of Navigation Sub-Committee, to prepare performance standards for maritime receivers that can use a number of different radio-navigation systems, or combinations of systems. |
| Strategic Direction: | 5.2 |
| High-level Action: | 5.2.4 |
| Planned output: | 5.2.4.3, 5.2.4.4 |
| Action to be taken: | Paragraph 24 |
| Related documents: | Res. A.915(22), Res. A.953(23), Res. MSC 252(83), MSC.112(73), MSC.113(73), MSC.233(82). |

1	The following proposal is submitted in accordance with the Guidelines of the

organization and method of work of the Committees (MSC-MEPC.1/Circ.2), taking into account

the High-level Action Plan for the Organization (resolution A.990 (25)) and circular letter No.2831 regarding the new format for IMO documents.

Background

2	IMO Res. A.915(22) envisaged the availability of Global Navigation Satellite Systems (GNSS) with “worldwide conformity of the signals processed by the user receivers to predetermined operational performance standards” and stated as a requirement that  “Augmentation provisions should be harmonised worldwide to avoid the necessity of carrying more than one ship-borne receiver” and that “development and use of integrated receivers using future GNSS(s) and terrestrial systems is recommended.” Res. A.953(23) stated that “ship-borne receiving equipment should be suitable for operating either with a world-wide radio-navigation system, or with radio-navigation systems which cover the area in which the ship trades.” and that “Ship-borne receiving equipment should conform to the relevant performance standards not inferior to those adopted by the Organization.”

3	MSC Resolution 252(83) sets out the Performance Standards for Integrated Navigation Systems. These standards make reference to performance standards for ship-borne receiver equipment (MSC.112(73); MSC.113(73); MSC.233(82)) and state that ‘The integrity of information should be verified by comparison of the data derived independently from at least two sensors and/or sources, if available.’

4	Several different GNSS are available or about to be deployed, as well as augmentations and terrestrial Electronic Position Fixing Systems (EPFS). These sources of navigation data will be compatible and inter-operable and receiving equipment will use them in combination, rather than individually. Performance standards have previously been developed for standalone receiving equipment using single sources and these would not be applicable to combined receiving equipment. Additional performance standards have been developed for augmentation systems receiving equipment (DGNSS). Rather than develop separate standards for each and every combination of navigation data source that might be used, it is proposed that one multi-system standard be developed.

5	It is generally accepted that GNSS are vulnerable to accidental and intentional interference, because of their extremely low received signal levels. This vulnerability has been demonstrated in trials carried out in the UK during 2008 & 2009.

6	The use of multiple systems, which are independent and complementary, will help to counter the risks of dependency on a single GNSS.  Each GNSS and terrestrial system, with augmentations where applicable, should satisfy the accuracy, availability, integrity, and continuity performance requirements for maritime sector applications, e.g. oceanic, coastal, harbour entrance and approach manoeuvers.  The use of multiple systems would allow users to retain the safety, security, and economic benefits they derive from the use of GNSS, even when one or more services are disrupted.

Scope of the proposal

7	To develop new Performance Standards for Ship-borne Receiving Equipment enabling  full use to be made of the availability, continuity and integrity, as well as increased accuracy, available from the combination of systems (GNSS and terrestrial) and augmentations now becoming available.

It is noted that revision of Resolution A.915(22) may also be necessary to make it compatible with the recent revision of A.953(23) and the trend towards the use of multiple systems.

Compelling Need

8	Existing performance standards for shipborne receiving equipment are intended for standalone sensors using single sources of navigation data. These standards will not be suitable for the combined receiving equipment now becoming available.

9	ECDIS depends on an input from a navigation data source, preferably two. Other onboard sensors, including radars, gyrocompasses and GMDSS equipment may also have navigation data inputs, resulting in alarms when the input is lost.

GNSS signals are vulnerable to disruption by environmental and man-made interference (accidental or deliberate). Using different, independent, but compatible transmissions can ensure availability and continuity of Position, Navigation and Timing (PNT) services with the accuracy and integrity required for all phases of navigation, even when individual GNSS services are disrupted. Ensuring availability of a timing source from more than one GNSS or terrestrial system is likely to be increasingly important as more onboard equipment becomes dependent on accurate timing.

Analysis of the issues involved, having regard to both the costs to the maritime industry, as well as the associated legislative and administrative burden, at global level

10	Several national and regional authorities have committed to providing GNSS and terrestrial sources for navigation by all transport sectors, as well as applications that depend upon precise timing references, such as certain communication service providers. There is no direct cost to the maritime industry from the provision of these sources, which are generally supported by the national and regional authorities involved. The primary burden to the maritime community is therefore the development and implementation of appropriate performance and test standards for receiving equipment. The cost of this will be minimized by having one set of performance standards, which do not need updating for every new source, or combination of sources. IMO need to provide such performance standards so that performance requirement and test standards can be developed.  Manufacturers need these standards to design, build, and test to obtain type-approval for receivers. Commonality of these standards will lower the costs of testing, which must be passed on to the user.

Benefits which would accrue from the proposal

11	GNSS signals can provide position-fixing services that can be expected to meet the likely accuracy, integrity, availability and continuity requirements. The ability to use several different GNSS and their augmentations, particularly if combined with terrestrial sources will help to ensure that these requirements can always be met, even when the service from an individual GNSS is subject to interruption.

12	A single set of performance standards for all sources and combinations of sources will streamline the process of introducing new sources of navigation data and should simplify the task of manufacturers in developing new receiving equipment.

13.	The Maritime Safety Committee (in MSC 85/26) has described the need for resilience of e-navigation. It states: “e-navigation systems should be resilient and take into account issues of data validity, plausibility and integrity for the systems to be robust, reliable and dependable. Requirements for redundancy, particularly in relation to position fixing systems should be considered.” An integrated, multi-source ship-borne receiving equipment is most likely to meet this need.

Priority and target completion date

14	GNSS has become an indispensable part of the maritime transportation system. Diversity of sources is essential to help prevent maritime casualties and maritime pollution incidents, in situations of reduced levels of service from particular GNSS.  Developing standards that will enable this diversity is therefore proposed as a high priority project with completion by 2013.

Is the subject of the proposal within the scope of IMO's objectives?

15	IMO is responsible for performance standards for shipborne navigational equipment. Up to date standards are essential to ensure safety of navigation.

How is the proposed item related to the scope of the Strategic plan for the Organization and how does it fit into the High-level action plan2

16	The proposed work item will enhance technical, operational and safety management standards - Strategic Direction: Section 5.2.

17	It fits into Section 5.2.4. of the High-level action plan - Keep under review measures to improve navigational safety, including e-navigation, ships routeing, ship reporting systems, vessel traffic services, requirements and standards for shipborne navigational aids and systems.

18	It would contribute to Section 5.2.4.3.  - Update and development of worldwide radionavigation systems.

19	It would also contribute to Section 5.2.4.4 – Strategic review and policy framework for e-Navigation. In this regard e-Navigation should address systems that provide a continuity of position-fixing services, if the primary source becomes unusable.

Do adequate industry standards exist?

20	No performance standards exist for integrated, multi-source ship-borne receiving equipment

Do the benefits justify the proposed action?

21	 The benefits described in paragraphs 11, 12 and 13 justify the proposed action.

Identification of which committee/subsidiary body(ies) are essential to complete the work

22	The work should be assigned to the Sub-Committee on Safety of Navigation (NAV).

Estimation of the number of sessions needed to complete the work

23	Two sessions of NAV Sub-Committee are estimated to be necessary to complete the work.

Action requested of the Committee

24	The Committee is invited to include this proposal in the agenda of the NAV Sub-Committee as a new work item, in accordance with the provisions for updating resolutions.

2 IMO Res. A.990(25)

The requirements for general navigation in the ocean, coastal, port approach and restricted waters operations, as specified in resolution A.915(22).