e-NAV10	Information paper

Agenda item	12

Task Number

- Author(s)	Norwegian Coastal Administration, Jeppesen

- 22 July 2011

- Notice of Arrival and Pilot Request Specification

- Norwegian Coastal Administration

- Jeppesen

| Version Number | Date | Author | Purpose |
| --- | --- | --- | --- |
| Draft 1 | 2011-03-21 | Jarle Hauge, Raphael Malyankar | Initial Discussion Draft |
| Draft 3 | 2011-0406 | Jarle Hauge, Raphael Malyankar | revisions |
| Draft 4 | 2011-0722 | RMM | Added data product and metadata descriptions |

Contents

1. Overview	4

1.1. Introduction	4

1.2. References	4

1.3. Terms, Definitions and Abbreviations	4

1.4. General Data Product Description	5

1.5. Notice of Arrival and Pilotage Requests Information Product Specification Metadata	5

1.6. Use case/story	6

2. Specification Scope	7

3. Data Product Identification	7

4. Data Content and Structure	7

4.1. Introduction	7

4.2. Application Schema	9

4.3. Feature Catalogue	11

4.4. Definition Sources	14

4.5. Feature Types	14

5. Data Product Format	15

5.1. Introduction	15

5.2. Spatial Data	15

5.3. Cells	15

6. Data Product Delivery	15

6.1. Introduction	15

6.2. Exchange set	16

6.3. Data set	16

6.4. Support files	17

6.5. Exchange catalogue	17

6.6. Data integrity	17

7. Metadata	17

7.1. Exchange set metadata	18

7.2. Dataset metadata	18

7.3. Exchange catalogue file metadata	21

8. Data Encoding	21

9. Sections Omitted from this Product Specification	21

Annex A. Named Types	22

A.1. Feature Types	22

A.2. Information Types	27

Annex B. Property Types	32

# Overview

## Introduction

Kystverket (Norwegian Coastal Administration) being the National Competent Authority for the European SafeSeaNet (SSN) in Norway and thereby maintains a vessel and voyage reporting system intended for use by commercial marine traffic arriving and departing Norwegian ports (SafeSeaNet Norway, http://www.shiprep.no). The system is intended as a single-window for filing Notices of Arrival, pilotage requests, and other forms required of commercial shipping.

The Notification of Arrival and Pilotage information product specification describes the data model used in this system. Reporting during passage to VTS centres, on arrival at radio calling-in points, and under ship reporting systems is not included.

Note: The current document is a “proof-of-concept” document that is intended to explore the feasibility of defining such specifications for maritime information other than charts, nautical publications, or similar information.  It does not contain all the obligatory information a ship is obliged to report on arrival/departure. Creating a comprehensive product specification for “Notifications of Arrival” is not the goal of this exercise.

## References

[ISO19115]	ISO 19115:2003. Geographic information – metadata. International Standards Organisation, 2003.

[S-57]	IHO Transfer Standard for Digital Hydrographic Data, Special Publication No. S-57, Edition 3.1, November 2000 (as updated by Supplement 2). International Hydrographic Bureau, Monaco.

[S-100]	Universal Hydrographic Data Model. IHO Special Publication No. S-100, Edition 1.0.0, January 2010. International Hydrographic Bureau, Monaco.

[S-100U1]	Draft update to Universal Hydrographic Data Model, IHO Special Publication No. S-100. (Under development, January 2011.)

[S-101]	Electronic Navigational Chart Product Specification. IHO Special Publication No. 101, (Draft), International Hydrographic Bureau, Monaco.

[SW]	Wiki maintained by Capt. Schröder-Fürstenberg for Standardization of Nautical Publications Working Group discussions, URL: http://www.fuerstenberg-dhg.de/mediawki/index.php

## Terms, Definitions and Abbreviations

### Terms and Definitions

The terms and definitions in S-100 V. 1.0.0 § 1-3 and Annex 1 apply to this document. The following additional terms are used.

Cell			A cell is a geographical area where the information in a dataset applies.

### Abbreviations

The abbreviations defined in S-100 V. 0.0.3 § 0-2 are used in this document. The following abbreviations are also used:

ECDIS				Electronic Chart Display Information Systems

ENC				Electronic Navigation Charts

GML				Geography Markup Language

IHO				International Hydrographic Organisation

IMO				International Maritime Organisation

NCA				Norwegian Coastal Administration (Kystverket)

PSC				Port State Control

SNPWG			Standardisation of Nautical Publications Working Group

TSMAD			Transfer Standard Maintenance and Development Working Group

## General Data Product Description

Title:		Notice of Arrival and Pilotage Request Information

Abstract:	Notice of Arrival and Pilotage Request Information (NO&PR) is an XML product produced for reports required of vessels arriving and departing Norwegian ports. Its primary function is for use in the Norwegian ship reporting system and related systems. The NO&PR contains the information required by regulatory requirements applying to commercial shipping within the area of coverage.  It also contains information required for filing pilotage requests.
Note that the NO&PR product specification does not meet all reporting requirement set out in EU and Norwegian legislation.

Content:	A conformant data set may contain features associated with the information on regulations, notifications of arrival/departure, pilotage, communications, and safety information. The specific content is defined by the NO&PR Feature Catalogue and the NO&PR Application Schema.

Spatial Extent:

Description: 	Areas where regulations pertaining to ship reporting information for vessels arriving or departing Norwegian ports are applicable.

East Bounding Longitude: 180

West Bounding Longitude: -180

North Bounding Latitude: 90

South Bounding Latitude -90

Specific Purpose: 	This document describes data that are exchanged between vessel or agent and government authorities, ports, and pilotage services in connection with arrival and departure reporting requirements, requests for pilotage, and port entry. It also includes information on pilot services, communications, reporting, and other information needed for arrival/departure reports and pilotage planning.

## Notice of Arrival and Pilotage Requests Information Product Specification Metadata

Title:		Notice of Arrival and Pilotage Requests Information Product Specification

Version:	0.0.0

Date:		18 March 2011

Language:	English

Classification:	Unclassified

Contact:	Jeppesen GmbH

(Address)

Telephone: + 49 …

Fax: +49 …

Email: Raphael.Malyankar@jeppesen.com/Jarle.Hauge@kystverket.no

URL:		www.jeppesen.com

Identifier:	NOAPR

Maintenance:	Changes to this product specification are coordinated by the authors.

## Use case/story

A ship is heading for a port in “a country”, i.e.: Haugesund/Norway. According to legislation the vessel is supposed to deliver an electronic Notice of arrival (NOA) 24 hour before arrival to the port of destination. The ship is also in some cases obligated to have a pilot onboard the ship when entering fairways and the port of destination.

The Norwegian Coastal Administration (NCA) is operating a Web Service (WS) which handles NOA’s and pilot requests.

The ship has an “office” computer system which is used in the charthouse or “back bridge” to avoid interference of paperwork with navigation. This system is capable of monitoring passage wither independently or by means of a link with the ECDIS and runs the necessary software for communicating with the WS, filing notices, etc.

The following actions are taken:

- During passage planning, an alert was set to contact the WS when the ship was 24+ hours from arrival in port (the 24-hour reporting requirement plus a margin for completion of the paperwork)..

- The alert goes off at the proper time.

- The ship sends a request (in XML via the Web service) to the NCA for arrival obligations. The request is done automatically by an application on the ship.

- The WS replies to the ship with the mandatory information elements which must be submitted upon arrival.

- The information sent by the WS appears on the ship’s office computer.

  - The application also shows the regulations pertaining to arrival in Norwegian waters in general and at Haugesund port in particular, if any (both national and local regulations). These regulations are extracted from the Nautical Publications (NPUB) S100-compliant dataset installed on the computer.

  - The application running on the “office computer” also displays an outline map of the area showing the relevant features (e.g., port, harbour plan, berths, pilot boarding places in the neighbourhood, radio calling-in points). This information combines selected features from the S100 ENC and NPUB datasets installed on the computer.

- The master fills out the necessary information about ship, cargo, and crew, and ETA in port, consulting the regulations and notes displayed by the system for explanations as needed. Information about cargo is pre-filled by the application which has the data stored in a local database, where it was entered when the ship loaded cargo.

- The master specifies a pilot boarding place by clicking one of the Pilot boarding Place objects in the display, and the system computes an estimate of ETA at the boarding place.

- The master verifies the information and the reply with ship/cargo/crew information, pilot boarding place, and ETA at the boarding place and port is sent to the NCA via the web service.

- The NCA WS responds OK/NotOK.

- If OK, the ship has submitted a NOA and requested a pilot.

This user story can of course be more elaborate and detailed, and other functionality such as automatic update from a subscription service, contact with the ship’s agent, etc., can be hypothesized.

# Specification Scope

Notice of Arrival and Pilotage requests data products are homogeneous in extent and do not vary for different parts of the data.  The NO&PR product specification describes one data product and requires only one scope which is described below;

Scope identification:  		Notice of Arrival and Pilotage Requests datasets.

Hierarchical level: 		(TBD)

Hierarchical level name: 	(TBD)

Level description:		information applies to the dataset

Extent:	EX_GeographicExtent  - Global coverage of maritime areas.
EX_TemporalExtent and EX_VerticalExtent are not defined for this product specification.

# Data Product Identification

Title:					NotificationOfArrival & PilotRequest

Alternative Title:			NOA&PR

Abstract:	When an S-10x PN is produced it must be in accordance with the rules defined in the PN product specification. S-10x details specifications intended to enable Hydrographic Offices to produce a consistent PI, and manufacturers to use that data efficiently in an ECDIS to satisfy IMO Performance Standards for ECDIS.

Topic Category:			Transportation

Geographic Description:	Areas where Notification Of Arrival and Pilotage Request for marine navigation is applicable.

Spatial Resolution:		Display Scale

Purpose:	The data shall be used with the purpose of creating a Notification Of Arrival and requesting a pilot to a National Competent Authority, and explain why use of such service is required or recommended.

Language:		English, with additional languages optional.

Classification:		Unclassified

Spatial Representation Type:		Vector

Point of Contact:		Producing Hydrographic Office

Use Limitations:			Not to be used without ENC and NPUB data

# Data Content and Structure

## Introduction

NO&PR is based on the General Feature Model (GFM) from S-100. The GFM is the conceptual model and the implementation is defined in the Feature Catalogue. A brief summary of the building blocks of the GFM follows. More information is available in the S-100 specification [S100].

A feature is an abstraction of real world phenomena, e.g., charted features such as a restricted area, harbour area, navigation aid, etc. In the S-100 model, GF_FeatureType is a metaclass that is instantiated as classes that represent individual feature types. A certain feature type is the class used for all instances of that feature type. The instances of a class that represents an individual feature type are called feature instances.  Feature types are equivalent to classes and feature instances are equivalent to objects.

An information type is an identifiable object that can be associated with features in order to carry information pertaining to the associated features. A regulation pertaining to passage requirements for a fairway is an example of an information type which is associated with the geographic feature type representing the fairway.  S100_GF_InformationType is the class intended to be the metaclass for information types in S-100.

Simple Attributes can be enumerations, codelists or simple types (e.g. integer or character string).

Complex attributes are properties of a feature which can be divided into multiple sub attributes and are used where objects have properties that better fit a hierarchical structure.  They provide a better construct for encoding list attributes on objects such as light sectors.

Aggregations are used represent features that are related to each other.

A feature association is a relationship that links instances of one feature type with instances of the same or a different feature type. Each relationship has a name and two roles thus giving a more detailed representation of the real world relationships within the dataset.

An NO&PR product is a feature-based product containing objects related to filing notices of a ship’s arrival or departure and pilot requests. The features and properties are as defined in the IHO standard S-57, the IHO feature concept dictionary for S-101 and the SNPWG Nautical publications dictionary (under development at the SNPWG Wiki [SW]).  Spatial objects are encoded as vector entities which are derived from the geometry element GM_Object (from the IHO S-100 framework standard and ISO 19107). Spatial objects can be of type Point, Curve (line) or surface (area). Figure 1 provides a partial overview of the domain model. The model consists of the following packages:

- definitions of the spatial objects (package “Geometry”)

- generalized domain classes, attributes, enumerations, and sub-packages (package “Domain Objects”)

- definitions from the IHO nautical publications model (package NPUB)

- definitions from the IHO ENC model (package HYDRO)

- definitions for notices of arrival and departure (package “Notices”)

- definitions for pilotage requests (package “Requests”)

Also shown in the figure are the complex attributes and enumerations defined in each package (only the nautical publications and ENC models define them).

Figure 1. Domain Model

This rest of this section contains the product Application Schema expressed in UML. The associated Feature Catalogue is given in Appendices A-C.  The Feature Catalogue provides a full description of each feature type including its attributes, attribute values and relationships in the data product.

## Application Schema

Figure 2 is a model of the notices of arrival and departure and pilotage request. Each notice is associated with a port area (which provides the name of the port as the objectName attribute of PortArea) as the port or arrival or departure. Association with an additional port area as the “Next Port of Call” is optional. The figure also shows a pilotage request which can optionally be linked to a pilot boarding place.

Figure 2. Notices of arrival/departure and pilotage requests

Figure 3 shows a model of a single voyage. A voyage is done by a Vessel and must have a Port of Call and Port of Departure. It may have an optional Next Port of Call. All are represented by the PortArea object from the Nautical Publications register.

Figure 3. Voyage

Figure 4 shows the notices and pilot request transmitted by a ship. The message consists of vessel information and an arrival notice, departure notice, or pilot request. Vessel information is required, and at least one of the notices or request must be present.

Figure 4. Messages sent by a ship

## Feature Catalogue

Name: Notice Of Arrival and Pilotage Requests Feature Catalogue

Scope: Catalogue containing objects associated with notices of arrival and pilot requests.

Field of application: Marine navigation

Version Number: 0.1

Version Date: 18 March 2011

Producer: Jeppesen/NCA

### Summary of Types

The table below lists the objects (feature and information types) and the attributes used in this product specification.

- The “Register Dictionary” column tells where the item is defined

  - HYDRO for objects/attributes which are part of ENCs

  - NPUB for those which are part of nautical publications, and

  - NOAPR for those defined for the NOAPR product).

- The “Index” column states whether the entity is a geographic object (“Feature”), information object, or attribute.

- Alpha code is a 6-character acronym assigned to the object and version date serves to identify which version of the object is used in this specification.

The table lists all the feature types first, followed by the information types and attributes.

From our point of view the Voyage is a central object. The voyage has a unique identifier and is related to a LOCATION object, in this case departure- and arrival port, sometimes also called PortOfDeparture and PortOfCall. There might in some cases be a relation towards the NextPortOfCall, which will be the next planned port after the one the ship is calling for.

In this product specification, ports LOCATIONs are represented by the Port Area geographic object defined in the Nautical Publications register.

Vessel is also considered to be an important information type. The Pilotage Request object will hold the necessary information (from a ships point view) to enable the navigator to request a pilot.

A few other geographic and information objects which may be useful in ship report messages or the overlay of this dataset with ENC or NPUB data are also included. Examples are Pilot Boarding Place, Pilot Service, and Contact Details.

The feature catalogue is given in Appendices A-B.

| Register Dict. | Index | Alpha code | Name | Version Date |
| --- | --- | --- | --- | --- |
| HYDRO | Feature | PILBOP | Pilot Boarding Place | 2000-11-01 |
| NPUB | Feature | PLTSRV | Pilot Service | 2009-06-19 |
| NPUB | Feature | PRTARE | Port Area |  |
| HYDRO | Feature | RDOCAL | Radio calling-in point | 2000-11-01 |
| NOAPR | Feature | VOYAGE | Voyage |  |
| NOAPR | Information | ARRINF | Arrival Information |  |
| NPUB | Information | CONDET | Contact Details | 2009-06-19 |
| NOAPR | Information | DEPINF | Departure Information |  |
| NPUB | Information | NWKDAY | Non-standard working day |  |
| NOAPR | Information | PILREQ | Pilotage Request |  |
| NOAPR | Information | VESSEL | Vessel |  |
| NOAPR | Attribute | ACTDRF | Actual Draft |  |
| NOAPR | Attribute | ACTHGT | Actual Height |  |
| NPUB | Attribute | ADMDIV | Administrative division | 2009-06-19 |
| NOAPR | Attribute | AGTNAM | Billing Agent name |  |
| NPUB | Attribute | CALNAM | Call Name | 2009-06-19 |
| HYDRO | Attribute | CALSGN | Call Sign | 2000-11-01 |
| NPUB | Attribute | CATCGO | Category of Cargo | 2009-06-19 |
| NPUB | Attribute | CATDHC | Category of dangerous or hazardous cargo or ballast | 2009-06-19 |
| NPUB | Attribute | CATPLT | Category of Pilot | 2000-06-19 |
| NPUB | Attribute | CATPBP | Category of Pilot Boarding Place | 2009-06-19 |
| NPUB | Attribute | CATVSL | Category of Vessel | 2009-06-19 |
| NPUB | Attribute | CITYNM | City Name | 2009-06-19 |
| HYDRO | Attribute | COMCHA | Communication Channel | 2000-11-01 |
| NPUB | Attribute | CONTRY | Country | 2009-06-19 |
| NOAPR | Attribute | CONTR3 | Country Code 3-character |  |
| HYDRO | Attribute | DATEND | Date end | 2000-11-01 |
| HYDRO | Attribute | DATSTA | Date start | 2000-11-01 |
| NPUB | Attribute | DELPNT | Delivery Point | 2009-06-19 |
| NPUB | Attribute | DSTNTN | Destination | 2009-06-19 |
| NOAPR | Attribute | DISNAM | Dispatch Department Name |  |
| NPUB | Attribute | EMAILS | Email Address | 2009-06-19 |
| NPUB | Attribute | NUMFAX | Fax number | 2009-06-19 |
| NOAPR | Attribute | HELREQ | Helicopter Required |  |
| NOAPR | Attribute | VSLIMO | IMO Number |  |
| HYDRO | Attribute | INFORM | Information | 2000-11-01 |
| NPUB | Attribute | ADRNET | Internet Address | 2009-06-19 |
| NOAPR | Attribute | LOCCOD | Location Code |  |
| NOAPR | Attribute | LOCNAM | Location Name |  |
| NPUB | Attribute | MMSICO | Maritime Mobile Service Identity (MMSI) Code | 2009-06-19 |
| NOAPR | Attribute | MULPLT | Multiple Pilots |  |
| HYDRO | Attribute | NATION | Nationality | 2000-11-01 |
| NPUB | Attribute | NUMTOR | Number Telex over Radio (TOR) |  |
| NOAPR | Attribute | NUMCRW | Number of Crew |  |
| NOAPR | Attribute | NUMPAS | Number of passengers |  |
| HYDRO | Attribute | OBJNAM | Object Name | 2000-11-01 |
| NPUB | Attribute | PRFPIL | Preference of Pilot Boarding Place | 2009-06-19 |
| HYDRO | Attribute | PEREND | Periodic Date End | 2000-11-01 |
| HYDRO | Attribute | PERSTA | Periodic Date Start | 2000-11-01 |
| HYDRO | Attribute | PICREP | Pictorial Representation | 2000-11-01 |
| HYDRO | Attribute | PILDST | Pilot District | 2000-11-01 |
| NPUB | Attribute | PLTMOV | Pilot Movement | 2000-11-01 |
| NPUB | Attribute | PLTQFC | Pilot Qualification | 2009-06-19 |
| NPUB | Attribute | PLTRQS | Pilot Request | 2009-06-19 |
| NOAPR | Attribute | PSTNAM | Pilot Station Department Name |  |
| NPUB | Attribute | PLTVSL | Pilot Vessel | 2009-06-19 |
| NPUB | Attribute | POSCOD | Postal Code | 2009-06-19 |
| NOAPR | Attribute | PLTQAY | Quay Information |  |
| NPUB | Attribute | RMTPLT | Remote Pilot | 2009-06-19 |
| HYDRO | Attribute | SCAMAX | Scale maximum | 2000-11-01 |
| HYDRO | Attribute | SCAMIN | Scale minimum | 2000-11-01 |
| NPUB | Attribute | SVAPRC | Service Access Procedure | 2009-06-19 |
| HYDRO | Attribute | SORDAT | Source Date | 2000-11-01 |
| HYDRO | Attribute | SORIND | Source Indication | 2000-11-01 |
| HYDRO | Attribute | STATUS | Status | 2000-11-01 |
| NPUB | Attribute | ADRTLG | Telegraph Address | 2009-06-19 |
| NPUB | Attribute | NUMTEL | Telephone Number | 2009-06-19 |
| NPUB | Attribute | NUMTLX | Telex number |  |
| NPUB | Attribute | NMTLOW | Telephone Number Outside Working Hours | 2009-06-19 |
| HYDRO | Attribute | TXTDSC | Textual Description | 2000-11-01 |
| NOAPR | Attribute | TOWPLT | Towing Pilotage |  |
| NOAPR | Attribute | VSLETA | Vessel’s estimated time of arrival |  |
| NOAPR | Attribute | VSLETD | Vessel’s estimated time of departure |  |
| NOAPR | Attribute | HAZMAT | Vessel has hazardous material |  |
| NOAPR | Attribute | VOYGID | Voyage ID |  |

## Definition Sources

IMDG	International Maritime Dangerous Goods (IMDG) Code

IMO A.851(20)	General Principles For Ship Reporting Systems And Ship Reporting Requirements, Including Guidelines For Reporting Incidents Involving Dangerous Goods, Harmful Substances And/Or Marine Pollutants. IMO Resolution A 851(20) adopted 27 November 1997

ISO 639-1	Codes for the representation of names of languages - Part 1: Alpha-2 code. International Standards Organisation, 2002. URL: http://www.infoterm.info/standardization/iso_639_1_2002.php retrieved 13 July 2009.

ISO 639-2	Codes for the representation of names of languages - Part 2: Alpha-3 code. International Standards Organisation, 1998. URL: http://www.loc.gov/standards/iso639-2/ retrieved 13 July 2009

ISO 3166-1	Codes for the representation of names countries and their subdivisions - Part 1: Country codes. International Standards Organisation.

MARPOL 73/78	International Convention for the Prevention of Pollution from Ships, modified by Protocol of 1978. http://www.imo.org/

## Feature Types

### Abstract feature types

Abstract feature types define classes which are used as generalizations of feature classes. Abstract types cannot have instances. The feature types derived from an abstract type inherit the properties of their parents unless explicitly overridden

### Geographic Feature Types

NOAPR is designed to provide both spatial and non-geospatial information. The spatial information provided by routeing guides is limited to overviews of relatively large areas or stretches of the coastline, depicting the location and spatial relationships of major hazards, major navigation aids, routeing measures and traffic schemes.  Non-spatial information includes text summaries of general material about areas, navigation regulations, hazards, pilotage, and ship reporting and ship routeing. This information may be associated with relatively large areas or large stretches of the coast, different administrative jurisdictions, or smaller areas or points of special interest such as ports or congested waters.

### Aggregated Feature Types

Feature with a use type of aggregated can have multiple associations to other feature types.  No aggregations are specified in this specification.

### Information Types

Information types are identifiable pieces of information in a cell that can be shared between other features. They have attributes like all feature types but have no geometry of their own. Information types may reference other information types and may reference feature types,.

# Data Product Format

## Introduction

This clause specifies the encoding of NOAPR datasets. A complete description of the data records, fields, and subfields is omitted from this exploration but in a formal specification would be provided in an Annex to this specification.

The clauses which follow, concerning encoding of latitude, longitude, and depth are taken from the latest draft S101 product specification in order to maintain consistency with ENC data.

### Encoding of Latitude and Longitude

Coordinates are stored as integers.  Latitude and longitude are converted to integers using a multiplication factor held in the Data Set Structure Information field under CMFX and CMFY (Coordinate Multiplication Factor(s) for x- and y-coordinates).

These coordinate multiplication factors must be set to 10000000 (107) for all datasets.

EXAMPLE	A longitude = 42.0000 is converted into X = longitude * CMFX = 42.0000 * 10000000 = 	420000000.

### Encoding of Depths

Depths are converted from decimal metres to integers by means of the Coordinate Multiplication factor for z-coordinate (CMFZ).  This product limits the resolution to two decimal places and therefore the CMFZ must be set to 100.

Depths are not transmitted in NOAPR datasets.

## Spatial Data

The spatial schema for spatial data shall be the same as defined in the S-100 standard and used by ENC product specification, S-101.

## Cells

Notices of arrival/departure and pilotage requests are linked to ports, and the previous or next port of call may near or distant, in the same or another nation. Further, notices and pilot requests are at least as much administrative and legal data as geographic information. Under the circumstances, division of data according to the national territory (including territorial waters) of a state may be more natural and more convenient to data queries than a grid-based division of space. Further subdivisions can be made if necessary according to port locations and administrative subdivisions, corresponding, for example, to different local offices of the competent national authority, or the volume of traffic at different ports. It may also be necessary to separate notices by time or by obsolete or pending status.

The proposed cell structure is therefore spatiotemporal, based on the location of the port of arrival and the date stamp of the notice or the effective date (date of arrival, departure, or arrival at the pilot boarding place).

# Data Product Delivery

## Introduction

This clause specifies the encoding and delivery mechanisms for NOAPR information.  Data which conforms to this product specification must be delivered by means of an exchange set.

NOAPR products consist of an exchange set which consists of one or more data sets plus accompanying metadata. Each data set consists of one or more Notices of Arrival or Pilot Requests. The diagram below shows the components of an exchange set.

Figure 5. Data product components

## Exchange set

NOAPR datasets will be grouped into exchange sets. Each exchange set will consist of one or more datasets with an associated XML metadata file and a single Exchange Catalogue XML file containing metadata.

Note: As of the writing of this specification, S100 appears to be internally inconsistent as to whether a dataset must always be contained in an exchange set.

Units of Delivery:		Exchange Set

Transfer Size:			Unlimited

Medium Name:		One of the following: HTTP; FTP; Email attachment

Other Delivery Information:

Each exchange set has a single exchange catalogue which contains the discovery metadata for each data set.

An exchange set is encapsulated into a form suitable for transmission either on hard or soft media by a mapping called an encoding. An encoding translates each of the elements of the exchange set into a logical form suitable for writing to media and for transmission online. An encoding may also define other elements in addition to the exchange set contents (i.e media identification, data extents etc.) and also may define commercial constructs such as encryption and compression methods.

This product specification defines a single example encoding for exchange sets which is described in clause 8. This encoding provides a file based encoding for an exchange set with no encrypted or compressed contents.  This is not a complete specification of the encoding. A complete encoding would include error-detecting and data authentication elements.

The encoding encapsulates exchange set elements as follows:

- NOAPR datasets – XML encoding of features/attributes and their associated geometry and metadata. Defined further in Annex [TBD].

- Exchange Catalogue – the XML encoded representation of exchange set catalogue features (discovery metadata).

Note that the exchange set definition does not allow for support files, but they are considered necessary, adding them to this specification can be done exactly as envisioned in S-100 and demonstrated in the S-101 specification for ENCs.

## Data set

Four kinds of data set may be contained within an exchange set:

- New notice or pilot request: Contains new notices or requests which have not been submitted before by the filing vessel. A new notice or request must be submitted every time the vessel arrives or departs or requests a pilot.

- Updated notice or pilot request: An updated notice or request updates a previously filed notice/request with new information such as a change in the arrival time. Updated notices are originated by the vessel and must refer to a previously filed notice/request.

- Response: Responses to notices or pilot requests are originated by the National Competent Authority or Pilot Agency and sent to the vessel in response to a new, updated or supplemental notice or request.

- Supplemental notice or pilot request: Supplemental notices or requests are submitted by the vessel in response to requests for additional information by the National Competent Authority.

Datasets shall not exceed 10MB.

A dataset is a collection of features and information objects comprising a single communication from or to a vessel and the National Competent Authority. Datasets may contain references to other datasets (for example, a supplemental notice may contain a reference to the original notice and the response received from the National Competent Authority.

## Support files

Support files are not included in this specification, since they are believed to be unnecessary. Should it be necessary to add them, perhaps to allow for attachments to Notices of Arrival/Departure or Pilot Requests, they can be defined in accordance with the S-100 standard as is done in the drafts of the ENC standard S101.

## Exchange catalogue

The exchange catalogue acts as the table of contents for the exchange set. The catalogue file of the exchange set must be named CATALOG.NPC. No other file in the exchange set may be named CATALOG.  The contents of the exchange catalogue are described in Clause 7.3.

## Data integrity

### Data integrity measures

Data integrity shall be assured by means of Cyclic Redundancy Check (CRC) values computed for every file in the exchange set. The algorithm shall be the CRC-32 algorithm (a 32-bit Cyclic Redundancy Check algorithm) defined in ANSI/IEEE Standard 802.3.

### Processing

Encoding is defined by the following generating polynomial:

G(x) = x 32 + x 26 + x 23 + x 22 + x 16 + x 12 + x 11 + x 10 + x 8 + x 7 + x 5 + x 4 + x 2 + x + 1

Processing is applied to relevant files as they appear in the exchange set.

The CRC value of the file is defined by the following process:

1. The first 32 bits of the data are complemented.

2. The n bits of the data are then considered to be the coefficients of a polynomial M(x) of degree n-1.

3. M(x) is multiplied by x32 and divided by G(x), producing a remainder R(x) of degree <31.

4. The coefficients of R(x) are considered to be a 32-bit sequence.

5. The bit sequence is complemented and the result is the CRC.

# Metadata

The metadata description is based on the S-100 metadata document section, which is a profile of the ISO 19115 standard.  Two types of metadata are described in this product specification; exchange set metadata and dataset metadata. The exchange set metadata imports the dataset discovery metadata.

Figure 6. Metadata for exchange set and datasets

## Exchange set metadata

Exchange set metadata contains metadata about the contents of the exchange set and metadata about the data distributor. It identifies the product specifications governing the files and resources comprising the exchange set  and the compiler of the exchange set. Exchange set metadata may be provided as a separate resource or as part of the catalogue file (see clause 7.3).

| Name | Cardinality | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| ExchangeSetMetadata | - |  | - | - |
| metadataFileIdentifier | 1 |  | CharacterString |  |
| metadataPointOfContact | 1 |  | CI_ResponsibleParty | Party responsible for compilation of exchange set |
| metadataDateStamp | 1 |  | Date |  |
| metadataLanguage | 1 | English, Norwegian | CharacterString | All data sets conforming to this PS must use English or Norwegian language. |
| metadataCharacterSet | 0..1 | utf8 | MD_CharacterSetCode | Use code for UTF8 |
| issueDate | 1 |  | Date | Date on which the exchange set was issued/transmitted. |
| productSpecification | 1 | NOAPR version 1.0.0 | S100_ProductSpecification | This must be encoded as NOAPR |
| productSpecificationDependency | 2 | S-101 product specification version, Nautical Publications specification version | S100_ProductSpecification | The ENC and the NPUBs product specifications which define the ENC and NPUB objects and attributes used in this dataset |
| comment | 0..1 |  | CharacterString | Any additional Information |

Note: Types with CI_, EX_, and MD_ prefixes are from packages defined in ISO 19115 and adapted by S-100.

## Dataset metadata

Dataset metadata contains information about the contents of a dataset, the party responsible for the dataset (normally the creator of the dataset), and the specifications governing it. Dataset metadata may be provided as a separate file or as part of the exchange catalogue file.

| Name | Cardinality | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| DataSetDiscoveryMetadata | - |  | - | - |
| metadataFileIdentifier | 1 |  | CharacterString |  |
| metadataPointOfContact | 1 |  | CI_ResponsibleParty | Contact information for the vessel or authority |
| metadataDateStamp | 1 |  | Date | When the dataset was created |
| metadataLanguage | 1 | English, Norwegian | CharacterString | All data sets conforming to this PS must use English or Norwegian language |
| fileName | 1 |  | CharacterString | Dataset file name |
| filePath | 1 |  | CharacterString | Full path from the exchange set root directory |
| description | 1 |  | CharacterString | One of the following: 1. Notice of arrival 2. Pilot request 3. Both notice of arrival and pilot request. NATIONAL LANGUAGE enabled |
| dataProtection | 1 | {1} to {2} | CharacterString | 1. Encrypted 2. Unencrypted |
| purpose | 1 | {1} to {4} | CharacterString | 1. New 2. Update 3. Response 4. Supplemental |
| specificUsage | 1 | {1} to {2} | Integer | 1, Notices of Arrival: A dataset containing a new or updated notice of arrival as required by local or national regulations 2. A new or updated request for making pilotage arrangements |
| editionNumber | 1 | {1} | Integer | The dataset edition. Required for s100 but not applicable to NOAPR, therefore it is set to 1 for all datasets. |
| updateNumber | 1 |  | CharacterString | Update number 0 is assigned to a new request or notice. Subsequent updates or supplements are assigned numbers 1, 2, 3… |
| issueDate | 1 |  | Date | Date on which the dataset was generated. |
| productSpecification | 1 | NOAPR version 1.0.0 | S100_ProductSpecification | This must be encoded as NOAPR |
| productSpecificationDependency | 2 | S-101 and Nautical Publications specification versions | S100_ProductSpecification | The ENC and the NPUBs product specification which define the ENC and NPUB objects and attributes used in this dataset |
| producingAgency | 1 |  | CI_ResponsibleParty | Party responsible for generating the dataset. |
| horizontalDatum | 1 | WGS84 | CharacterString | The datum for latitude/longitude. EPSG:4326 |
| verticalDatum | 1 | {1} to {30} | Integer | 1. Mean low water springs (29 other values) |
| soundingDatum | 1 | {1} to {30} | Integer | 1. Mean low water springs (29 other values) |
| dataType | 1 | other | S-100_DataFormat | S-100 v.1.0.0 lists only ISO8211, GML, and “other” formats |
| otherDataTypeDescription | 0..1 |  | CharacterString |  |
| boundingBox | 0..1 |  | EX_GeographicBoundingBox | Either boundingBox or geographicDescription must be given. |
| boundingPolygon | 0..* |  | EX_BoundingPolygon |  |
| geographicDescription | 0..1 | Norway | EX_GeographicDescription | Descriotion of the location for notices or requests, in the form of the country of the national authority. Either boundingBox or geographicDescription must be given. |
| comment | 0..1 |  | CharacterString | Any additional Information |
| cyclicRedundancyCheck | 1 |  | NonNegativeInteger | Check value for the dataset |
| layerId | 1 | {1} | integer | Identifies the relationship to other S100 information on a graphical display. All NOAPR data is scale independent 1. Scale Independent |

Note: Types with CI_, EX_, and MD_ prefixes are from packages defined in ISO 19115 and adapted by S-100. Types with S100_ prefix are from packages defined in S-100.

This specification retains the core elements of metadata as given in S-100 and adds elements for describing dependencies on other product specifications and verbal descriptions of the location. Elements believed to by inapplicable (e.g., ”classification” (secret, etc.)) have been removed. The metadata elements for display, optimum, and minimum scales have also been removed as they are not relevant to notices or arrival/departure and pilot requests. The bounding box and bounding polygon elements are optional since they are not expected to contribute to anticipated needs for dataset discovery. It would be possible to define rules for using these metadata elements if future needs arise, for example, requiring that these values correspond to national territorial waters or admnistrative subdivisions.

## Exchange catalogue file metadata

The catalogue file is defined in an XML schema and encoded in XML. The metadata pertaining to that file is documented below.

| Name | Cardinality | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| metadataFileIdentifier | 1 |  | CharacterString |  |
| metadataPointOfContact | 1 |  | CI_ResponsibleParty |  |
| metadataDateStamp | 1 |  | Date |  |
| metadataLanguage | 1 | English | CharacterString | All data sets conforming to this PS must use English or Norwegian language. The catalogue file must be in English. |
| name | 1 | CATALOG.NPC | CharacterString | Catalogue filename |
| abstract | 1 |  | CharacterString | Description of what the exchange catalogue contains |
| productSpecification | 1 |  | S100_ProductSpecification | NOAPR product specification Version Number |
| productSpecificationDependency | 2 | S-101 and, Nautical Publications specifications | S100_ProductSpecification | The ENC and the NPUBs product specification which define the ENC and NPUB objects and attributes used |
| comment | 0..1 |  | CharacterString | Any additional Information |

Note: Types with CI_, EX_, and MD_ prefixes are from packages defined in ISO 19115 and adapted by S-100. Types with S100_ prefix are from packages defined in S-100.

# Data Encoding

The encoding and exchange format for NOAPR products will be XML conforming to the schema located at [schema location URL to be provided, preferably as part of the registry]. The catalogue file shall be an XML file conforming to the catalogue schema at [URL to be provided, preferably as part of the registry].

Note: Detailed descriptions of structure and XML schemas have been omitted for the sake of minimising effort expended on this exploration and in the interest of reducing the length of this document, but can easily be developed along with a formal specification. If an ISO-8211 conformant encoding must be selected, this section would be replaced by a description of that encoding instead, which would be based on Part 10 of S-100 version 1.0.0. It would be similar to the encoding under development as part of the ENC product specification S-101..

# Sections Omitted from this Product Specification

An S-100-style product specification would also have the following sections, omitted from this example specification for the sake or brevity or due to not being relevant:

Coordinate Reference Systems

Data Quality

Data Maintenance

Portrayal

Additional Information

Annex A. Named Types

This annex contains the geographic objects (feature types) and information objects (information types in the feature catalogue).

A.1. Feature Types

Geo Object Class: Pilot boarding place 					Alpha code: PILBOP

Camel case: PilotBoardingPlace					Abstract type: False
Definition: The meeting place to which the pilot comes out. (IHO Chart Specs, M-4)

References: INT 1: IT 1.1-4; M-3: ? M-4: 491.1 2;

Remarks: No remarks.			Spatial Objects: Point (GM_Point); Area (GM_Polygon)

Distinction: No distinctions.

| Attribute | Camel case | Alpha code | Cardinality | Sequential |
| --- | --- | --- | --- | --- |
| Call Sign | callSign | CALSGN | 1 |  |
| Category of pilot boarding place | categoryOfPilotBoardingPlace | CATPBP | 1 |  |
| Category of vessel | categoryOfVessel | CATVSL | 0..* | False |
| Communication Channel | communicationChannel | COMCHA | 1..* | False |
| Date, end | dateEnd | DATEND | 0..1 |  |
| Date, start | dateStart | DATSTA | 0..1 |  |
| Destination | destination | DSTNTN | 0..* | False |
| Notice Time | noticeTime | NTCTIM | 0..* | True |
| Object Name | objectName | OBJNAM | 0..* | False |
| Periodic date end | periodicDateEnd | PEREND | 0..1 |  |
| Periodic date start | periodicDateStart | PERSTA | 0..1 |  |
| Pilot district | pilotDistrict | PILDST | 0..* | False |
| Pilot movement | pilotMovement | PLTMOV | 0..* | False |
| Pilot request | pilotRequest | PLTRQS | 0,,* | True |
| Pilot vessel | pilotVessel | PLTVSL | 0..1 |  |
| Preference of pilot boarding place | preferenceOfPilotBoardingPlace | PRFPIL | 1 |  |
| Status | status | STATUS | 0..* | False |
| Service access procedure | serviceAccessProcedure | SVAPRC | 0..* | False |
| Information | information | INFORM | 0..* | False |
| Textual description | textualDescription | TXTDSC | 0..* |  |

Geo Object Class: Pilot service 						Alpha code: PLTSRV

Camel case: PilotService						Abstract type: False

Supertype: Abstract Feature Type
Definition: The area where pilotage services are available.  Pilotage is a service provided by a person who directs the movements of a vessel through pilot waters, usually a person who has demonstrated extensive knowledge of channels, aids to navigation, dangers to navigation, etc., in a particular area and is licensed for that area. (adapted from IHO Dictionary, S-32, 5th Edition, 3843)

References: INT 1: not specified; M-3: Chapter C Section C 2.8;  M-4: not specified;

Remarks: The name of this object may be the same as the Pilot District of the associated PILBOPs.

Spatial Objects: Area (GM_Polygon)

Distinction: No distinctions.

| Attribute | Camel case | Alpha code | Cardinality | Sequential |
| --- | --- | --- | --- | --- |
| Category of pilot | categoryOfPilot | CATPLT | 1..* | False |
| Notice Time | noticeTime | NTCTIM | 0..* | False |
| Object Name | objectName | OBJNAM | 0..* | False |
| Pilot district | pilotDistrict | PILDST | 0..* | False |
| Pilot qualification | pilotQualification | PLTQFC | 0..1 |  |
| Pilot request | pilotRequest | PLTRQS | 0..1 |  |
| Remote pilot | remotePilot | RMTPLT | 0..1 |  |
| Service access procedure | serviceAccessProcedure | SVAPRC | 0..* | False |
| Information | information | INFORM | 0..* | False |
| Textual description | textualDescription | TXTDSC | 0..* | False |

| Information feature | Camel case | Alpha code | Cardinality |
| --- | --- | --- | --- |
| Contact details | ContactDetails | CONDET | 0..* |

Geo Object Class: Port area						Alpha code: PRTARE

Camel case: PortArea							Abstract type: False
Definition: The port and surrounding sea and land areas in which there are services, designated areas and facilities, such as pilotage, outer anchorages, storages yards and warehousing, all associated with shipping.

References:	INT 1: IN 49; M-3: Chapter C Section C 2.8 M-4: 430.1;

Remarks: The name of this object may be the same as the Pilot District of the associated PILBOPs.

Spatial Objects: Area (GM_Polygon)

Distinction: dock area; harbour area (administrative);

| Attribute | Camel case | Alpha code | Cardinality | Sequential |
| --- | --- | --- | --- | --- |
| Object Name | objectName | SRVFBG | 0..1 |  |
| Status | status | STATUS | 0..* | false |
| Information | information | INFORM | 0..* | False |
| Textual description | textualDescription | TXTDSC | 0..* | False |

Geo Object Class: Radio calling-in point					Alpha code: RDOCAL

Camel case: RadioCallingInPoint					Abstract type: False

Definition:  Also called radio reporting points, they have been established in certain busy waterways and port approaches to assist traffic control. On passing these points or crossing a defined line defined types of vessels or vessels carrying specified cargoes are required to report on VHF to a Traffic Control Centre. (adapted from IHO Chart Specifications, M-4)

References: INT 1: IM 40; M-4: 488;

Remarks:  The attribute “orientation” (ORIENT) encodes the orientation of the traffic flow at that point.

Distinction: radio station; pilot boarding place;

Attributes:

| Attribute | Camel case | Alpha code | Cardinality | Sequential |
| --- | --- | --- | --- | --- |
| Development | categoryOfCargo | CATCGO | 0..1 |  |
| Category of Vessel | categoryOfVessel | CATVSL | 0..1 |  |
| Communications channel | comunicationsChannel | COMCHA | 0..* |  |
| Object Name | objectName | SRVFBG | 0..1 |  |
| Orientation | orientation | ORIENT | 0..1 |  |
| Status | status | STATUS | 0..* | false |
| Information | information | INFORM | 0..* | False |
| Textual description | textualDescription | TXTDSC | 0..* | False |

Geo Object Class: Voyage						Alpha code: VOYAGE

Camel Case: Voyage							Abstract type: False

Definition: A description of a specific voyage in terms of an identifier and the ports of departure and call,

References: unspecified;

Remarks: No remarks. 
Distinction: none

| Attribute | Camel case | Alpha code | Cardinality | Sequential |
| --- | --- | --- | --- | --- |
| Voyage ID | voyageID | VOYGID | 1 |  |

Associated geographic features:

| Feature type | Camel case | Alpha code | Cardinality |
| --- | --- | --- | --- |
| Port Area | PortArea | PRTARE | 2..3 |

Associated Information Types:

| Information feature | Camel case | Alpha code | Cardinality |
| --- | --- | --- | --- |
| Vessel | Vessel | VESSEL | 1 |

A.2. Information Types

Information Object Class: Arrival information					Alpha code: ARRINF

Camel Case: ArrivalInformation						Abstract type: False
Definition: Information which must be filed by a vessel before arriving in port.

References: unspecified;

Remarks: No remarks. 
Distinction: Departure information

| Attribute | Camel case | Alpha code | Cardinality | Sequential |
| --- | --- | --- | --- | --- |
| Actual Draft | actualDraft | ACTDRF | 1 |  |
| Actual Height | actualheight | ACTHGT | 1 |  |
| Nationality | nationality | NATION | 1 |  |
| Country Code 3-character | countryCode3 | CONTY3 | 1 |  |
| Country | country | CONTRY | 1 |  |
| Vessel’s Estimated Time of Arrival | vesselETA | VSLETA | 1 |  |
| Location Code | locationCode | LOCCOD | 1 |  |
| Location Name | locationName | LOCNAM | 1 |  |
| Number of Crew | numberOfCrew | NUMCRW | 1 |  |
| Number of Passengers | numberOfPassengers | NUMPAS | 1 |  |

Associated geographic features:

| Feature type | Camel case | Alpha code | Cardinality |
| --- | --- | --- | --- |
| Port Area | PortArea | PRTARE | 2..3 |

Associated information features:

| Information type | Camel case | Alpha code | Cardinality |
| --- | --- | --- | --- |
| Vessel | Vessel | VESSEL | 1 |

Information Object Class: Contact Details					Alpha code: CONDET

Camel Case: ContactDetails							Abstract type: False
Definition: Information on how to reach a person or organisation by postal, internet, telephone, telex and radio systems.

References: M-3: unspecified;

Remarks: No remarks. 
Distinction: No distinctions.

| Attribute | Camel case | Alpha code | Cardinality | Sequential |
| --- | --- | --- | --- | --- |
| Call name | callName | CALNAM | 0..1 |  |
| Call sign | callSign | CALSGN | 1 |  |
| Communication channel | communicationChannel | COMCHA | 1..* | False |
| Object Name | objectName | OBJNAM | 0..* | False |
| Delivery point | deliveryPoint | DELPNT | 0..* | False |
| City name | cityName | CITYNM | 0..1 |  |
| Administrative division | administrativeDivision | ADMDIV | 0..1 |  |
| Postal code | postalCode | POSCOD | 0..1 |  |
| Country | country | CONTRY | 0..1 |  |
| Email address | emailAddress | EMAILS | 0..1 |  |
| Telephone number | telephoneNumber | NUMTEL | 0..1 |  |
| Telephone number outside working hours | telephoneNumberOutsideWorkingHours | NMTLOW | 0..1 |  |
| Fax number | faxNumber | NUMFAX | 0..1 |  |
| Telex number | telexNumber | NUMTLX | 0..1 |  |
| Internet address | internetAddress | ADRNET | 0..1 |  |
| Telegraph address | telegraphAddress | ADRTLG | 0..1 |  |
| Maritime Mobile Service Identity (MMSI) Code | maritimeMobileServiceIdentityCode | MMSICO | 0..1 |  |
| Information | information | INFOML | 0..* | False |
| Textual description | textualDescription | TXTDSC | 0..* | False |

Information Object Class: Departure information					Alpha code: DEPINF

Camel Case: DepartureInformation						Abstract type: False
Definition: Information which must be filed by a vessel before departing a port.

References: unspecified;

Remarks: No remarks. 
Distinction: Arrival information

| Attribute | Camel case | Alpha code | Cardinality | Sequential |
| --- | --- | --- | --- | --- |
| Nationality | nationality | NATION | 1 |  |
| Country Code 3-character | countryCode3 | CONTY3 | 1 |  |
| Country | country | CONTRY | 1 |  |
| Vessel’s Estimated Time of Departure | vesselETD | VSLETD | 1 |  |
| Vessel has Hazardous Material | hasHazMat | HAZMAT | 1 |  |
| Location Code | locationCode | LOCCOD | 1 |  |
| Location Name | locationName | LOCNAM | 1 |  |

Associated geographic features:

| Feature type | Camel case | Alpha code | Cardinality |
| --- | --- | --- | --- |
| Port Area | PortArea | PRTARE | 2 |

Associated information features:

| Information type | Camel case | Alpha code | Cardinality |
| --- | --- | --- | --- |
| Vessel | Vessel | VESSEL | 1 |

Information Object Class: Pilotage request					Alpha code: PILREQ

Camel Case: PilotageRequest							Abstract type: False
Definition: Information which must be filed by a vessel when requesting a pilot.

References: unspecified;

Remarks: The two values of the locationName attribute, indicates the start and end location of the pilotage in same order that the values appear in the data set.
Distinction: none

| Attribute | Camel case | Alpha code | Cardinality | Sequential |
| --- | --- | --- | --- | --- |
| Billing Agent Name | billingAgentName | AGTNAM | 1 |  |
| Dispatch Department Name | dispatchDepartmentName | DISNAM | 0..1 |  |
| Helicopter Required | helicopterRequired | HELREQ | 1 |  |
| Vessel’s Estimated Time of Arrival | vesselETA | VSLETA | 1 |  |
| Towing Pilotage | isTowingPilotage | TOWPLT | 1 |  |
| Location Name | locationName | LOCNAM | 2 | True |
| Multiple Pilots | moreThanOnePilot | MULPLT | 1 |  |
| Pilot Station Department Name | pilotStationDepartmentName | PSTNAM | 0..1 |  |
| Quay Information | quayInformation | PLTQAY | 1 |  |

Associated geographic features:

| Feature type | Camel case | Alpha code | Cardinality |
| --- | --- | --- | --- |
| Port Area | PortArea | PRTARE | 1 |

Associated information features:

| Information type | Camel case | Alpha code | Cardinality |
| --- | --- | --- | --- |
| Contact Details | ContactDetails | CONDET | 1 |
| Vessel | Vessel | VESSEL | 1 |

Information Object Class: Vessel						Alpha code: VSLINF

Camel Case: Vessel								Abstract type: False
Definition: A description of the vessel.

References: unspecified;

Remarks: No remarks. 
Distinction: none

| Attribute | Camel case | Alpha code | Cardinality | Sequential |
| --- | --- | --- | --- | --- |
| Call sign | callSign | CALSGN | 1 |  |
| IMO number | vslIMONumber | VSLIMO | 1 |  |
| Object Name | objectName | OBJNAM | 1 |  |
| Maritime Mobile Service Identity (MMSI) Code | maritimeMobileServiceIdentityCode | MMSICO | 1 |  |

Associated Information Types

| Information feature | Camel case | Alpha code | Cardinality |
| --- | --- | --- | --- |
| Arrival information | ArrivalInformation | ARRINF | 0..1 |
| Departure information | DepartureInformation | DEPINF | 0..1 |
| Pilotage Request | PilotageRequest | PILREQ | 0..1 |

Annex B. Property Types

This annex contains the attributes.

Attribute: Actual Draft 						Alpha code: ACTDRF

Attribute type: simple

Camel case: actualDraft 						Data type: float 
Definition:  The actual draft of the vessel 
Units: metres

Resolution: 0.1
Remarks:

No remarks.

Attribute: Actual Height 						Alpha code: ACTHGT

Attribute type: simple

Camel case: actualHeight 						Data type: float 
Definition:  The actual height of the vessel 
Units: metres

Resolution: 0.1
Remarks:

No remarks.

Attribute: Administrative division					Alpha code: ADMDIV

Attribute type: Simple

Camel case: administrativeDivision					Data Type: text

Definition: Administrative division is a generic term for an administrative region within a country at a level below that of the sovereign state.

Remarks: admdiv is used in the context of contact details.

Distinction: ADMARE

Attribute: Billing Agent Name						Alpha code: AGTNAM

Attribute type: Simple

Camel case: locationName						Data Type: String

Definition: Indicates the name of the location

Constraints: Length must be no more than 50 characters.

References: none

Remarks: --

Attribute: Call name							Alpha code: CALNAM

Attribute type: Simple

Camel case: callName							Data Type: text

Definition: The designated call name of a station, e.g. radio station, radar station, pilot. This is the name used when calling a radio station by radio i.e. "Singapore Pilots".

Distinction:  CALSGN - The designated call-sign of a radio station i.e. "WWVB" for Fort Collins, Colorado.

References: INT 1: not specified; M-3: M-4: not specified;

Remarks:  No remarks.

Attribute: Call sign							Alpha code: CALSGN

Attribute type: Simple

Camel case: callSign							Data Type: text

Definition: The designated call-sign of a radio station.

References: INT 1: not specified; M-4: not specified;

Remarks: No remarks.

Attribute: Category of cargo						Alpha code: CATCGO

Attribute type: Simple

Camel case: categoryOfCargo						Data Type: Enumeration

Values:

| Code | Name | Definition |
| --- | --- | --- |
| 1 | bulk | Normally dry cargo which is transported to and from the vessel on conveyors |
| 2 | container | One of a number of standard sized cargo carrying units, secured using standard corner attachments and bars |
| 3 | general | Break bulk cargo normally loaded by crane |
| 4 | liquid | Any cargo loaded by pipeline |
| 5 | passenger | A fee paying traveller |
| 6 | livestock | Live animals carried in bulk |
| 7 | dangerous or hazardous | Dangerous or hazardous cargo as described by the IMO International Maritime Dangerous Goods code |

References: : INT 1: unspecified; M-4: unspecified;

Remarks: If item 7 is used, the nature of dangerous or hazardous cargoes can be amplified with category of dangerous or hazardous cargo

Attribute: Category of dangerous or hazardous cargo or ballast	Alpha code: CATDHC

Attribute type: Simple

Camel case: categoryOfDangerousOrHazardousCargo			Data Type: Enumeration

Values:

| Code | Name | Definition |
| --- | --- | --- |
| 1 | Class 1; Division 1.1 | Explosives, Division 1: substances and articles which have a mass explosion hazard |
| 2 | Class 1; Division 1.2 | Explosives, Division 2: substances and articles which have a projection hazard but not a mass explosion hazard |
| 3 | Class 1; Division 1.3 | Explosives, Division 3: substances and articles which have a fire hazard and either a minor blast hazard or a minor projection hazard or both, but not a mass explosion hazard |
| 4 | Class 1; Division 1.4 | Explosives, Division 4: substances and articles which present no significant hazard |
| 5 | Class 1; Division 1.5 | Explosives, Division 5: very insensitive substances which have a mass explosion hazard |
| 6 | Class 1; Division 1.6 | Explosives, Division 6: extremely insensitive articles which do not have a mass explosion hazard |
| 7 | Class 2.1 | Gases, flammable gases |
| 8 | Class 2.2 | Gases, non-flammable, non-toxic gases |
| 9 | Class 2.3 | Gases, toxic gases |
| 10 | Class 3 | flammable liquids |
| 11 | Class 4.1 | flammable solids, self-reactive substances and desensitized explosives |
| 12 | Class 4.2 | substances liable to spontaneous combustion |
| 13 | Class 4.3 | substances which, in contact with water, emit flammable gases |
| 14 | Class 5.1 | oxidizing substances |
| 15 | Class 5.2 | organic peroxides |
| 16 | Class 6.1 | toxic substances |
| 17 | Class 6.2 | infectious substances |
| 18 | Class 7 | Radioactive material |
| 19 | Class 8 | Corrosive substances |
| 20 | Class 9 | Miscellaneous dangerous substances and articles |
| 21 | Harmful Substances in packaged form | Harmful substances are those substances which are identified as marine pollutants in the International Maritime Dangerous Goods Code (IMDG Code). Packaged form is defined as the forms of containment specified for harmful substances in the IMDG Code. (MARPOL (73/78) Annex III) |

References: International Maritime Dangerous Goods (IMDG) Code

Remarks: Substances (including mixtures and solutions) and articles subject to the provisions of the International Maritime Dangerous Goods (IMDG) Code are assigned to one of the classes 1-9 according to the hazard or the most predominant of the hazards they present. Some of these classes are subdivided into divisions. These classes or divisions are as listed in IDs 1 : 20 above. (Adapted from IMDG code www.imo.org).

Attribute: Category of pilot						Alpha code: CATPLT

Attribute type: Simple

Camel case: categoryOfPilot						Data Type: Enumeration

Values:

| Code | Name | Definition |
| --- | --- | --- |
| 1 | pilot | pilot licenced to conduct vessels during approach from sea to a specified place which may be a handover place, an anchorage or alongside |
| 2 | deep sea | pilot licenced to conduct vessels over extensive sea areas |
| 3 | harbour | pilot who is licenced to conduct vessels from a specified place, such as a handover area or anchorage into a harbour |
| 4 | bar | pilot licensed to conduct vessels over a bar to or from a handover with a river pilot (for example as used in USA) |
| 5 | river | pilot licensed to conduct vessels from and to specified places, along the course of a river (for example as used in Rio Amazonas and Rio de La Plata) |
| 6 | channel | pilot licensed to conduct vessels from and to specified places, along the course of a channel. (for example as used in Rio Amazonas and Rio de La Plata) |
| 7 | lake | pilot licensed to conduct vessels from and to specified places on a great lake. (for example as used in the Lago de Maracaibo in Venezuela) |

Remarks: No remarks.

Attribute: Category of pilot boarding place				Alpha code: CATPBP

Attribute type: Simple

Camel case: categoryOfPilotBoardingPlace				Data Type: Enumeration

Values:

| Code | Name | Definition | References |
| --- | --- | --- | --- |
| 1 | boarding by pilot-cruising vessel | pilot boards from a cruising vessel | INT 1: IT 1.1-3; M-4: 491.1 |
| 2 | boarding by helicopter | pilot boards by helicopter which comes out from the shore | INT 1: IT 1.4; M-4: 491.2 |
| 3 | pilot comes out from shore | pilot boards from a vessel which comes out from the shore on request | INT 1: IT 1.1-3; M-4: 491.1 |

References: ?

Remarks: No remarks

Attribute: Category of vessel						Alpha code: CATVSL

Attribute type: Simple

Camel case: categoryOfVessel						Data Type: Enumeration

Values:

| Code | Name | Definition |
| --- | --- | --- |
| 1 | general cargo vessel | a vessel designed to carry general cargo |
| 2 | container carrier | a vessel designed to carry ISO containers |
| 3 | tanker | a vessel designed to carry bulk liquid or gas, including LPG and LNG |
| 4 | bulk carrier | a vessel designed to carry bulk solid material |
| 5 | passenger vessel | a vessel designed to carry passengers; often a cruise ship |
| 6 | roll-on roll-off | a vessel designed to allow road vehicles to be driven on and off; often a ferry |
| 7 | refrigerated cargo vessel | a vessel designed to carry refrigerated cargo |
| 8 | fishing vessel | a vessel designed to catch or hunt fish |
| 9 | service | a vessel which provides a service such as a tug, anchor handler, survey or supply vessel |
| 10 | warship | a vessel designed for the conduct of military operations |

References: none

Remarks: none

Attribute: City name							Alpha code: CITYNM

Attribute type: Simple

Camel case: cityName							Data Type: text

Definition: The name of a town or city

Remarks: No remarks

Attribute: Communication channel					Alpha code: COMCHA

Attribute type: Simple

Camel case: communicationChannel					Data Type: text

Definition: A channel number assigned to a specific radio frequency, frequencies or frequency band.

Constraints:

| Length | 4 |
| --- | --- |
| Structure | Each VHF-channel should be indicated by 2 digits and up to 2 characters (A-Z) |

References: INT 1: IM 40; M-4: 488;

Remarks: The attribute “communication channel” encodes the various VHF-channels used for communication. The indication of several VHF-channels is possible through use of multiplicity > 1.

Attribute: Country							Alpha code: CONTRY

Attribute type: Simple

Camel case: country							Data Type: text

Definition: The name of a nation

References: Adapted from The American Heritage Dictionaries

Remarks: Keep a standard for country names under review.

Attribute: Country Code 3-character					Alpha code: CONTR3

Attribute type: Simple

Camel case: countryCode3						Data Type: string

Definition: The attribute “country code 3-character” indicates the nationality of the specific object.

Constraints:

| Length | 3 |
| --- | --- |
| Structure | The value must conform to ISO 3166 |

References: ISO 3166

Remarks: No remarks.

Attribute: Date end							Alpha code: DATEND

Attribute type: Simple

Camel case: dateEnd							Data Type: Date

Definition: The attribute “date end” indicates the latest date on which an object (e.g. a buoy) will be present.

Constraints:

| Other | CCYYMMDD, consisting of 4 digits for the calendar year (CCYY), 2 digits for the month (MM) (e.g. April = 04) and 2 digits for the day (DD), according to ISO 8601:1988 |
| --- | --- |

References: ?

Remarks: This attribute is to be used to indicate the removal or cancellation of an object at a specific date in the future.  See also “periodic date end”. Example: 19961007 for 07 October 1996 as ending date.

Attribute: Date start							Alpha code: DATSTA

Attribute type: Simple

Camel case: dateStart							Data Type: Date

Definition: The attribute “date, start” indicates the earliest date on which an object (e.g. a buoy) will be present.

Constraints:

| Other | CCYYMMDD, consisting of 4 digits for the calendar year (CCYY), 2 digits for the month (MM) (e.g. April = 04) and 2 digits for the day (DD), according to ISO 8601:1988 |
| --- | --- |

References: ?

Remarks: This attribute is to be used to indicate the deployment or implementation of an object at a specific date in the future. See also “periodic date start”. Example: 19960822 for 22 August 1996 as starting date.

Attribute: Delivery point						Alpha code: DELPNT

Attribute type: Simple

Camel case: deliveryPoint						Data Type: text

Definition: Details of where post can be delivered such as the apartment, name and/or number of a street, building or PO Box

References: none

Remarks: This could be repeated if there is more than one address item required in addition to the city name.

Attribute: Destination							Alpha code: DSTNTN

Attribute type: Simple

Camel case: destination						Data Type: text

Definition: The place or general direction to which a vessel is going or directed.

References: none

Remarks: In addition to a placename of a port, harbour area or terminal, the place could include generalities such as “The north-west”, or “upriver”.

Attribute: Dispatch Department Name				Alpha code: DISNAM

Attribute type: Simple

Camel case: dispatchDepartmentName					Data Type: String

Definition: Indicates the name of the location

Constraints: Length must be no more than 50 characters.

References: none

Remarks: --

Attribute: Email address						Alpha code: EMAILS

Attribute type: Simple

Camel case: emailAddress						Data Type: text

Definition: An address assigned to an organisation or person to send or receive electronic mail. Example: steven.smith@domain.com

References: --

Remarks: No remarks.

Attribute: Fax number							Alpha code: NUMFAX

Attribute type: Simple

Camel case: faxNumber							Data Type: text

Definition: A number assigned to a fax machine. Example: + 49 381 4563769

References: not specified

Remarks: The telephone number should be written according to the ITU Recommendation ITU-T E.123. Only spaces should be used to visually separate groups of numbers in international notation.

Attribute: Helicopter Required					Alpha code: HELREQ

Attribute type: Simple

Camel case: helicopterRequired					Data Type: Boolean (?)

Definition: Whether a helicopter is required

References: none

Remarks: --

Attribute: IMO Number						Alpha code: VSLIMO

Attribute type: Simple

Camel case: vslIMONumber						Data Type: int

Definition: The IMO number for a vessel.

References: none

Remarks: --

Attribute: Information							Alpha code: INFORM

Attribute type: Simple

Camel case: information							Data type: text

Definition: Textual information about the object in a single language.

References: INT 1: IA 16; M-4: 242.3-5;

Remarks: --

This attribute should be used, for example, to hold the information that is shown on paper charts by cautionary and explanatory notes.

No formatting of text is possible within INFORM. If formatted text is required, then the attribute

TXTDSC must be used.

Attribute: Internet address						Alpha code: ADRNET

Attribute type: Simple

Camel case: internetAddress						Data Type: text

Definition: An Internet address (for example, http://www.hmco.com/trade/), usually consisting of the access protocol (http), the domain name (www.hmco.com), and optionally the path to a file or resource residing on that server (trade).

References: The American Heritage Dictionaries

Remarks: The address could be a website or an ftp site.

Attribute: Location Code						Alpha code: LOCCOD

Attribute type: Simple

Camel case: locationCode						Data Type: String

Definition: Indicates the code of the location

Constraints: Length must be 5 characters.

References: none

Remarks: --

Attribute: Location Name						Alpha code: LOCNAM

Attribute type: Simple

Camel case: locationName						Data Type: String

Definition: Indicates the name of the location

Constraints: Length must be no more than 50 characters.

References: none

Remarks: --

Attribute: Maritime Mobile Service Identity (MMSI) Code		Alpha code: MMSICO

Attribute type: Simple

Camel case: maritimeMobileServiceIdentityCode				Data Type: Integer

Definition: The Maritime Mobile Service Identity (MMSI) Code is formed of a series of nine digits which are transmitted over the radio path in order to uniquely identify ship stations, ship earth stations, coast stations, coast earth stations, and group calls. These identities are formed in such a way that the identity or part thereof can be used by telephone and telex subscribers connected to the general telecommunications network principally to call ships automatically.

Constraints:

| length | 9 |
| --- | --- |

References: Adapted from USCG which in turn adapted it from Appendix 43 of the International Telecommunications Union Radio Regulations

Unit of measure: None

Remarks: Format: XXXXXXXXX		Example: 366777490

Attribute: Multiple Pilots						Alpha code: MULPLT

Attribute type: Simple

Camel case: moreThanOnePilot					Data Type: Boolean

Definition: Whether the pilotage requires more than one pilot

References: none

Remarks: --

Attribute: Nationality							Alpha code: NATION

Attribute type: Simple

Camel case: nationality							Data Type: text

Definition: The attribute “nationality” indicates the nationality of the specific object.

Constraints:

| Length | 2 |
| --- | --- |
| Structure | The value must conform to ISO 3166 |

References: ISO 3166

Remarks: No remarks.

Attribute: Number of Crew						Alpha code: NUMCRW

Attribute type: Simple

Camel case: numberOfCrew						Data Type: Boolean (?)

Definition: ???

References: none

Remarks: --

Attribute: Number of Passengers					Alpha code: NUMPAS

Attribute type: Simple

Camel case: numberOfPassenger					Data Type: int

Definition: The number of passengers on board

References: none

Remarks: --

Attribute: Number telex over radio (TOR) 				Alpha code: NUMTOR

Attribute type: simple

Camel case: numberTelexOverRadio 					Data type: Integer 
Definition:  A special number to contact a radio station via wireless telex 
Units: None

Resolution: 1
Remarks:

No remarks.

Attribute: Object name						Alpha code: OBJNAM

Attribute type: Simple

Camel case: objectName						Data Type: text

Definition: The individual name of an object.

References: INT 1: ID 7, IF 19, IN 12.2-3; M-4: 371; 323.1-2; 431.2-3; 431.5;

Remarks: no remarks

Attribute: Periodic date end						Alpha code: PEREND

Attribute type: Simple

Camel case: periodicDateEnd						Data Type: Date

Definition: The end of the active period for a seasonal object (e.g. a buoy). See also “date end”.

Constraints:

| Structure | The value should be encoded using 4 digits for the calendar year (CCYY), 2 digits for the month (MM) (e.g. April = 04) and 2 digits for the day (DD). CCYYMMDD (full date); --MMDD (same day each year); --MM (same month each year) This conforms to ISO 8601:1988. |
| --- | --- |
| Other | If an object has either of its PERSTA/PEREND attribute values non-null, the other must also be non-null. |

References: ISO 8601:1988

Remarks: Example: --1015 for an ending date of 15 October each year

Attribute: Periodic date start						Alpha code: PERSTA

Attribute type: Simple

Camel case: periodicDateStart						Data Type: Date

Definition: The start of the active period for a seasonal object (e.g. a buoy). See also “date start”.

Constraints:

| Structure | The value should be encoded using 4 digits for the calendar year (CCYY), 2 digits for the month (MM) (e.g. April = 04) and 2 digits for the day (DD). CCYYMMDD (full date); --MMDD (same day each year); --MM (same month each year) This conforms to ISO 8601:1988. |
| --- | --- |
| Other | If an object has either of its PERSTA/PEREND attribute values non-null, the other must also be non-null. |

References: ISO 8601:1988

Remarks: Example: --04 for an operation starting in April each year

Attribute: Pictorial representation					Alpha code: PICREP

Attribute type: Simple

Camel case: pictorialRepresentation					Data Type: text

Definition: Indicates whether a pictorial representation of the object is available. The string encodes the file name of an external graphic file (pixel/vector) as permitted in the list of allowed support formats.

References: INT 1: IE 3.1-2; M-4: 456.5; 457.3;

Remarks: The “pictorial representation” could be a drawing or a photo.

Attribute: Pilot district							Alpha code: PILDST

Attribute type: Simple

Camel case: pilotDistrict						Data Type: text

Definition: The name assigned to the area within which a particular pilotage service operates.

References: INT 1: IT 1.2; M-4: 491.1-2;

Remarks: No remarks.

Attribute: Pilot movement						Alpha code: PLTMOV

Attribute type: Simple

Camel case: pilotMovement						Data Type: Enumeration

Definition: The embarkation or disembarkation activity of a pilot. This attribute specifies whether pilots embark and/or leave the vessel.

Values:

| Code | Name | Definition |
| --- | --- | --- |
| 1 | embarkation | The place where vessels not being navigated according to a pilot’s instructions pick up a pilot while in transit from sea to a port or restricted waters for future navigation under pilot instructions. |
| 2 | disembarkation | The place where vessels being navigated under a pilot’s instructions in transit from sea to a port or constricted waters drop the pilot and proceed without being subject to pilot instructions. |
| 3 | pilot change | The place where vessels being navigated under a pilot’s instructions drop off the pilot and pick up a different pilot for future navigation under pilot’s instructions. |

References: unspecified;

Remarks: If the Pilot boarding place is used at a port for embarkation and disembarkation, this attribute is not used.

Attribute: Pilot qualification						Alpha code: PLTQFC

Attribute type: Simple

Camel Case: pilotQualification						Data Type: Enumeration

Definition: --

Values:

| Code | Name | Definition |
| --- | --- | --- |
| 1 | government pilot | A pilot service carried out by government pilots. |
| 2 | pilot approved by government | A pilot service carried out by pilots who are approved by government. |
| 3 | state pilot | A pilot that is licensed by the State (USA) and/or their respective pilot association, required for all foreign vessels and all American vessels under registry, bound for a port with compulsory State pilotage. A federal licence is not sufficient to pilot such vessels into the port |
| 4 | federal pilot | A pilot who carries a Federal endorsement, offering services to vessels that are not required to obtain compulsory State pilotage. Services are usually contracted for in advance |
| 5 | company pilot | A pilot provided by a commercial company |
| 6 | local pilot | A pilot with local knowledge but who does not hold a qualification as a pilot |
| 7 | citizen with sufficient local knowledge | A pilot service carried out by a citizen with sufficient local knowledge |
| 8 | citizen with doubtful local knowledge | A pilot service carried out by a citizen whose local knowledge is uncertain |

Remarks: No remarks.

Attribute: Pilot request						Alpha code: PLTRQS

Attribute type: Simple

Camel case: pilotRequest						Data Type: text

Definition: Description of the pilot request procedure.

References: unspecified;

Remarks: No remarks.

Attribute: Pilot Station Department Name				Alpha code: PSTNAM

Attribute type: Simple

Camel case: pilotStationDepartmentName				Data Type: String

Definition: Indicates the name of the location

Constraints: Length must be no more than 50 characters.

References: none

Remarks: --

Attribute: Pilot vessel							Alpha code: PLTVSL

Attribute type: Simple

Camel case: pilotVessel						Data Type: text

Definition: Description of the pilot vessel. The pilot vessel is a small vessel used by a pilot to go to or from a vessel employing the pilot's services. (adapted from Science and Technology Dictionary)

References: unspecified;

Remarks: No remarks.

Attribute: Postal code							Alpha code: POSCOD

Attribute type: Simple

Camel case: postalCode						Data Type: text

Definition: Known in various countries as a postcode, or ZIP code, the postal code is a series of letters and/or digits that identifies each postal delivery area.

References: unspecified;

Remarks: No remarks.

Attribute: Preference of pilot boarding place				Alpha code: PRFPIL

Attribute type: Simple

Camel case: preferenceOfPilotBoardingPlace				Data Type: Enumeration

Definition: This attribute allows for boarding places to be designated as primary or alternate boarding places.

Values:

| Code | Name | Definition |
| --- | --- | --- |
| 1 | Primary | The preferred and published pilot boarding place which is used in normal weather conditions. |
| 2 | Alternate | The pilot boarding place which is used if the primary boarding place is unsuitable, for example because of weather or sea state |

Attribute: Quay Information						Alpha code: PLTQAY

Attribute type: Simple

Camel case: quayInformation						Data Type: String

Definition: Information about the quay, for the special case where the pilot boards at a quay

Constraints: Length must be no more than 50 characters.

References: none

Remarks: No remarks.

Attribute: Remote pilot						Alpha code: RMTPLT

Attribute type: Simple

Camel case: remotePilot						Data Type: Boolean

Definition: Whether remote pilotage is available.

| True | Remote pilot is available | Pilotage is available remotely from shore or other location remote from the vessel requiring pilotage |
| --- | --- | --- |
| False | Remote pilot is not available | Remote pilotage is not available |

References: unspecified;

Remarks: No remarks.

Attribute: Service access procedure					Alpha code: SVAPRC

Attribute type: Simple

Camel case: serviceAccessProcedure					Data Type: text

Definition: A description of the procedure to access the marine service.

References: unspecified;

Remarks: None.

Attribute: Status							Alpha code: STATUS

Attribute type: Simple

Camel case: status							Data Type: Enumeration

Values:

| Code | Label | Definition | References |
| --- | --- | --- | --- |
| 1 | permanent | intended to last or function indefinitely. (The Concise Oxford Dictionary, 7th Edition) |  |
| 2 | occasional | acting on special occasions; happening irregularly. (The Concise Oxford Dictionary, 7th Edition) | INT 1: IP 50; M-4: 473.2; |
| 3 | recommended | presented as worthy of confidence, acceptance, use, etc. (The Macquarie Dictionary, 1988) | INT 1: IN 10; M-4: 431.1; |
| 4 | not in use | no longer used for the purpose intended; disused. | INT 1: IL 14, 44; M-4: 444.7; |
| 5 | periodic/intermittent | recurring at intervals. (The Concise Oxford Dictionary, 7th Edition) | INT 1: IC 21; IQ 71; M-4: 353.3; 460.5; |
| 6 | reserved | set apart for some specific use. (adapted from The Concise Oxford Dictionary, 7th Edition) | INT 1: IN 12.9; |
| 7 | temporary | meant to last only for a time. (The Concise Oxford Dictionary) | INT 1: IP 54; |
| 8 | private | not in public ownership or operation. | INT 1: IQ 70; |
| 9 | mandatory | compulsory; enforced. (The Concise Oxford Dictionary, 7th Edition) |  |
| 11 | extinguished | no longer lit |  |
| 12 | illuminated | lit by floodlights, strip lights, etc. |  |
| 13 | historic | famous in history; of historical interest. (The Concise Oxford Dictionary, 7th Edition) |  |
| 14 | public | belonging to, available to, used or shared by, the community as a whole and not restricted to private use. (adapted from The New Shorter Oxford English Dictionary, 1993) |  |
| 15 | synchronized | occur at a time, coincide in point of time, be contemporary or simultaneous. (The New Shorter Oxford English Dictionary, 1993) |  |
| 16 | watched | looked at or observed over a period of time especially so as to be aware of any movement or change. (adapted from The New Shorter Oxford English Dictionary, 1993) |  |
| 17 | un-watched | usually automatic in operation, without any permanently-stationed personnel to superintend it. (adapted from IHO Dictionary, S-32, 5th Edition, 2814) |  |
| 18 | existence doubtful | an object that has been reported but has not been definitely determined to exist |  |

.

References: --

Remarks: No remarks

Attribute: Telegraph address						Alpha code: ADRTLG

Attribute type: Simple

Camel case: telegraphAddress						Data Type: text

Definition: The telegraphic address assigned to an organisation.

Remarks: No remarks.

Attribute: Telephone number						Alpha code: NUMTEL

Attribute type: Simple

Camel case: telephoneNumber						Data Type: text

Definition: A number assigned to a telephone.

References: unspecified;

Remarks: The telephone number should be written according to the ITU Recommendation ITU-T E.123. Only spaces should be used to visually separate groups of numbers in international notation. The letters "int." are used to indicate internal number extensions.  Example: + 49 381 4563764 int.254

Attribute: Telephone number outside working hours			Alpha code: NMTLOW

Attribute type: Simple

Camel case: telephoneNumberOutsideWorkingHours			Data Type: text

Definition: A number assigned to a service for use outside working hours.

References: unspecified;

Remarks: The telephone number should be written according to the ITU Recommendation ITU-T E.123. Only spaces should be used to visually separate groups of numbers in international notation. The letters "int." are used to indicate internal number extensions.  Example: + 49 172 4019079 int.123

Attribute: Telex number						Alpha code: NUMTLX

Attribute type: Simple

Camel case: telexNumber						Data Type: text

Definition: Numbers assigned to a telex machine as a unique identifier.

References: unspecified;

Remarks: No remarks.

Attribute: Textual description						Alpha code: TXTDSC

Attribute type: Simple

Camel case: textualDescription						Data Type: text

Definition: The file name of an external text file that contains the text.

Remarks: The attribute “textual description” indicates that a file containing text extracted from relevant pilot books or navigational publications is available.

Attribute: Towing Pilotage						Alpha code: TOWPLT

Attribute type: Simple

Camel case: isTowingPilotage						Data Type: Boolean

Definition: Whether the pilotage is a towing pilotage (?)

References: none

Remarks: --

Attribute: Vessel’s ETA						Alpha code: VSLETA

Attribute type: Simple

Camel case: vesselETA							Data Type: DateTime

Definition: Indicates the estimated time of arrival.

References: none

Remarks: --

Attribute: Vessel’s ETD						Alpha code: VSLETD

Attribute type: Simple

Camel case: vesselETD						Data Type: DateTime

Definition: Indicates the estimated time of departure.

References: none

Remarks: --

Attribute: Vessel has Hazardous Material				Alpha code: HAZMAT

Attribute type: Simple

Camel case: hasHazMat						Data Type: Boolean

Definition: Whether the vessel is carrying hazardous cargo as defined by … (IMDG? Norwegian authorities?)

References: none

Remarks: --

Attribute: Voyage ID							Alpha code: VOYGID

Attribute type: Simple

Camel case: voyageID							Data Type: String

Definition: Unique identifier for a voyage.

References: none

Remarks: --

The HYDRO and NPUB registries are located at http://registry.iho.int/, the hypothetical NOAPR registry would be similar to those and contain the information in Annexes A and B.

The Alpha code was recently generalized into an “Alias”, but is retained here for convenience.

This product specification goes slightly beyond the strict limits of current ship reporting, but including these extras allows more interesting applications of the data.