e-NAV2 / 08 / 03

DGPS Re-capitalisation Study

## Introduction

This paper is a summary of a study carried out by Nottingham Scientific Ltd., for the General Lighthouse Authorities (GLA) of the UK & Ireland on the options for modernising the DGPS service. The Committee is invited to note this paper for information purposes.

The GLA DGPS service came into operation in 1998. The DGPS equipment has a typical operational life of 5-10 years and the computers and communications equipment have already been replaced. The Reference Stations and Integrity Monitors (RSIM) currently in use are no longer in production and may not be supported beyond 2008. Also, the communications between RSIM and control station need to be upgraded as they are currently based on proprietary protocols of the RSIM supplier.

Furthermore, the present GLA DGPS system will not meet all the requirements set out in IMO Resolution A.915(22) for Future GNSS.

Four options have been considered:

- Replacement of the existing hardware RSIM

- Replacement with RSIM implemented in software;

- Virtual Reference Station (VRS) approach, using an existing network;

- Integration with Satellite Based Augmentation Systems (EGNOS).

## Timeline

There is a short-term need to update equipment so that present requirements can still be met, but there will also be a longer term need to adapt the system to future requirements, in particular modernised GNSS (GPS L5 and Galileo). Based on estimates of when each of the options is likely to be available, the following timeline, showing availability of options, has been produced to meet these 1st and 2nd generation re-capitalisation requirements.

## Technical Analysis

The timeline shows that some of the options will not be available in time for 1st generation re-capitalisation (L1 DGPS). Options for 2nd generation re-capitalisation (L1/L5 DGNSS) have been considered at this stage because they may influence the choice of 1st generation option.

Based on the availability of technical equipment, the following options have been taken forward for detailed analysis:

1st generation		Procurement of upgraded HW (baseline solution)

Procurement of upgraded HW + SW RSIM

OSNET* VRS (code and RTK**)	* Ordnance Survey Network				** Real Time Kinematic

2nd generation	Procurement of upgraded HW (baseline solution)

Procurement of upgraded HW + SW RSIM

OSNET VRS (code and RTK)

SW Radio Receiver and SW RSIM

EGNOS RTCM

EGNOS RTCA

EGNOS WARTK

## Performance

For 1st generation re-capitalisation (L1 DGPS), performance predictions for the baseline HW RSIM option show that horizontal error and protection levels increase as the distance from the reference station locations increases. The largest errors and protection levels are at the Northern extremities of the area and are mainly due to satellite geometry.

If only the coastal areas are considered (up to 50 nautical miles seaward) then the horizontal accuracy is always better than 9m. This means that the availability of accuracy will meet the 99.8% level from the present requirements.

The same conclusions in terms of performance apply to the SW RSIM and VRS OSNET (code) option with L1 DGPS data.

For 2nd generation re-capitalisation with enhanced L1/L5 DGNSS operations then the performance for the baseline HW RSIM option is significantly improved and the horizontal accuracy is almost always less than 1m. This means that the availability of accuracy is very close to meeting the 99.8% level for future requirements. The majority of the improvement over the L1 DGPS case is due to the improved satellite geometry through the use of two constellations (GPS + Galileo).

The same conclusions in terms of performance apply to the SW RSIM and VRS OSNET (code) options with L1/L5 DGNSS data.

The performances of the EGNOS options (EGNOS RTCM and EGNOS RTCA) are worse than the baseline HW RSIM option for both L1 DGPS and L1/L5 DGNSS applications.

The VRS OSNET (RTK) option and the EGNOS WARTK option are the only ones that are close to reaching the most stringent future accuracy requirement of 0.1m horizontal accuracy with the required availability. However, the coverage area of the VRS OSNET (RTK) option is very limited and the EGNOS WARTK option is not available for L1 DGPS users.

## Risk Analysis

The option with the least risk for 1st generation re-capitalisation is the baseline HW RSIM option. The OSNET VRS code option is by far the riskiest option with several risks that have a high total score.

For upgrade to 2nd generation re-capitalisation then the option with the lowest risk is the SW RSIM option, because it allows more flexibility to upgrade and the hardware to be replaced would be commercial off-the-shelf. There would be more uncertainty with potentially greater cost involved in upgrading the HW and firmware in the HW RSIM option to 2nd generation requirements. Therefore the possibility of transition to the SW RSIM option during the lifetime of the 1st generation system should be considered, if the software becomes available in a proven state at low cost.

The OSNET VRS RTK option is the only one considered that could potentially be used for a high accuracy service. However, it has limited coverage and significant risks, especially on the technical side.

For the additional 2nd generation options the technical risks are generally higher because the technologies involved are less mature. However, it is anticipated that by the time the 2nd generation re-capitalisation program begins these technologies will have advanced to a suitable state. The assessment of the options should be an ongoing process the risk analysis should be updated as more information becomes available.

## Costs

Only the initial costs have been included at this stage. Whole-life costs, including running costs and decommissioning would give a better assessment, preferably taking account of the interaction between 1st and 2nd generation options. However, because of uncertainties and lack of data on many of the options, this was not considered within the scope of this study. Of the 1st generation options, the VRS OSNET (code) option has the lowest procurement costs, but no service charge from OS has been included, because none has been quoted. The baseline HW RSIM option is next lowest and the SW RSIM option is much the highest, on the basis of estimated costs for developing software of this type and complexity. A budgetary quotation for the only known SW RSIM confirms this ranking.

For the additional 2nd generation options the costs are less certain since the solutions are not currently available and so there are no actual costs to base the estimates on. Therefore the assessment of these options should be an ongoing process and should be updated as information becomes available.

## Conclusions

The lowest cost, lowest risk option would be to replace existing hardware with similar, dedicated RSIM based on known, commercially available technology. However, the choice of suppliers is limited and once chosen, it may be difficult to change it to meet emerging requirements.

The flexibility provided by the SW RSIM option could overcome this problem and it should not be ruled out at this stage.

## Recommendations

- The Hardware RSIM option should be adopted for the 1st Generation Re-capitalisation, on grounds of lowest cost and risk.

- Transition to the SW RSIM option during the lifetime of the 1st Generation system should be considered, if suitable proven software becomes available at reasonable cost.

3.	Study of the 2nd Generation options should be a continuing project, running in parallel with the 1st Generation Re-capitalisation work.