From: Finnish Maritime Administration

7 March 2005

(VTT Technical Research Centre of Finland)

To: 	IALA eNavigation Committee

and IALA Secretariat

The Baltic AIS trial - Testing the exchange of AIS binary messages in the Baltic Sea and evaluating the information content of different AIS messages

The international community and the maritime safety, security and environmental authorities as the executing party have a responsibility to undertake the monitoring of vessel traffic and to ensure the sufficient distribution of information to vessel traffic. In addition to this responsibility numerous services need to be provided. These activities are in many respects based on the information available to the authority. AIS is one of the most significant information sources available. While emphasising the need for information, the work load caused to vessels as well as authorities by different reporting requirements should always be considered. By fulfilling their responsibilities, the authorities enhance the safety and efficiency of maritime traffic and contribute to the protection of marine environment.

IMO gave in its circular (SN/Circ. 236) a possibility for the application of seven predefined binary messages. It can be stated that with the implementation of the binary messages the amount and quality of information available for authorities’ is improved, resulting in an improvement in maritime safety. However, before the implementation the usability of these messages should be tested in full scale tests (ship-to-shore) and the information content and usability evaluated. At the same time it would be a favourable moment to evaluate the content of AIS voyage related data and static information. These messages should reflect the needs of information users.

The Finnish Maritime Administration and VTT Technical Research Centre of Finland have launched a study focusing on the above described issues. The Baltic AIS trial project aims to enhance the quality of information exchanged between vessel traffic and authorities by testing the operational and technical usability of binary messages, evaluating the information content of different messages and proposing revisions on present messages, based upon the observations and conclusions made during the project. While defining the revisions it shall be remembered at all times that the load of VHF data link (VDL) must not be increased to any significant degree. The load caused by the low reporting rate of these messages is almost negligible anyway.

We would like to bring this study to the attention of the IALA eNavigation Committee. The Committee will be duly informed about the progress of the study and the results achieved, in detail. The content of the study is defined in more detail in Appendix 1.

Rolf Bäckström

Deputy director

Finnish Maritime Administration, Traffic Department

PROJECT PLAN

Baltic AIS Trial

“AISBaltic”

Contents

1	Background	4

2	Objectives and scope	5

3	Working plan	7

# Background

Automatic Identification System (AIS) is a system for ship identification and tracking, that has the capability of both transmitting and receiving information. AIS can be considered as one of the most significant technical tools developed since the invention of radar. However, its information exchange capacity is limited and International Maritime Organization (IMO) administers the content of the AIS messages to avoid overloading of the AIS frequencies. Various stakeholders have proposed additional needs to the present content of the AIS information and IMO shall in 2008 next open the discussion on the amendments to AIS operation.

Through AIS ships send static and voyage related ship-specific information to other ships and to land-based systems. Information on e.g. ship's identification (name, call sign, IMO number and MMSI number), position, destination, vessel type, etc. is exchanged. The introduction of AIS onboard ships was in many ways a remarkable improvement but the information provided with the system could be defined to better serve the needs of users both onboard and especially ashore.

In addition to the exchange of ship’s static and voyage related information IMO has given a possibility for the application of binary messages. In IMO circular SN/Circ. 236 seven predefined binary messages are provided for test use. Use of these binary messages may enhance the use of AIS but the relevance of the information contained in these messages can be argued.

This project originates from the need to reduce the workload caused on ships bridges and land based authorities by various demands for reporting and to improve the quality of information available. This can be achieved by reducing verbal communication i.e. VHF radio traffic and further the availability of the specific information needed by maritime, SAR and environmental authorities to enhance safety, security and protection of the environment.

Before the work described in this document is even started, several obvious benefits of enhanced quality of AIS information to ships and authorities can be foreseen. One significant positive effect to the safety of navigation is the increase of vessel traffic predictability. Predictability provides for better planning and time for countermeasures against escalating high risk situations. In addition to onboard decision-making, quality information on vessel traffic (ship types, cargoes, routes, incidents, etc.) received in good time enables providing safety related information and prerequisites for the development of new concepts for  intelligent decision-making support systems, risk indicating calculations and strategic traffic planning just to mention a few. Information exchange must be developed before efforts can be placed on developing systems for automatic monitoring and analysing of the information and indicating of relevant observations.

# Objectives and scope

Lack of sufficient information, and availability of information with limited usability.

The present definition of ship’s static and voyage related information included in AIS is mainly the result of work done in the IMO Sub-Committee on Safety of Navigation (NAV) and serves its user well in many respect. The development of vessel traffic monitoring systems and of the international conventions has generated a need to amend and improve this definition. For example the usability of AIS information related to cargo is rather poor. It has also been difficult for the ship officer onboard vessels to understand which of the available options (Fig. 1) for ship type second digit is the correct one for their vessel.  The only variant in the present definition of cargo carried (Fig. 1) is the pollutant category indicator for noxious and liquid substances that is based on the definition found in the International Convention for the Prevention of Pollution from Ships (MARPOL) and in the International Bulk Chemical Code (IBC Code). Thus choosing of the ship type second digit is relevant only vessels with cargo classified to these pollution categories. A problem arises if the vessel carries dangerous cargoes not classified under MARPOL pollutant categories or cargoes classified under more than one pollutant category.

Figure 1 AIS ship type identifiers (IALA 2003)(DG = dangerous goods, HS = harmful substances, MP = marine pollutant)

The revised version of MARPOL Annex II Regulations for the control of pollution by noxious liquid substances in bulk entering into force on the 1.1.2007 has induced another need to update the definition of cargo information in AIS. The revised annex includes a new four-category categorisation system (X, Y, Z & Other substances) for noxious and liquid substances and replaces the old A, B, C and D categorisation that is now used in AIS definition. New classes are not with the same content as the old are. Therefore, after 1.1.2007 part of the definition of what substances are included to AIS ship type second digit is no longer valid.

Presumably, when the AIS information items would be discussed among relevant authorities, more needs for minor adjustments could be found. As an example the need for making the use of United Nations LOCODEs mandatory in the “next port of call” information can be mentioned. This need for improvement has been observed while making data enquiries from the Helsinki Commission (HELCOM) AIS database and from the GOFREP operation: as long as the information concerning port of destination is a free text field the name of a single port continues to be written in numerous inconsistent ways.

The information available in AIS is very limited as shall continue to be so. To improve the amount and quality of information available for authorities from AIS, binary messages may be used. IMO gave in its circular (SN/Circ. 236) a possibility for the application of seven predefined binary messages:

- Meteorological and hydrological data

- Dangerous Cargo Indication

- Fairway Closed

- Tidal window

- Extended ship static and voyage related data

- Number of Persons onboard

- Pseudo-AIS targets

The possibility of using AIS binary messages is welcomed as a means of collecting information required by the authorities almost automatically. In fact, if properly developed systems are available, automatic AIS information acquisition from e.g. named area or an area with defined limits is possible.  This clearly enhances the use of AIS but the present predefined content of these messages seems to provide little added value to the present static and voyage related information.

In addition to the predefined messages, member governments may propose new binary messages to IMO. In this case, member governments should address a demonstrated operational need to the Sub-Committee on the Safety of Navigation (NAV) and provide a proposed format and content of the message.

When planning the testing of binary message exchange it should be remembered that the display capability of binary messages is not part of the mandatory AIS Minimum Keyboard and Display (MKD). In ships not equipped with this capability additional hardware to AIS and dedicated software may be required.

Thus, the objective of the AISBALTIC project is to

- identify the information needs of Baltic Sea states maritime safety, security, environment and SAR authorities that may be fulfilled by the limited information available from AIS,

- define what modifications to static and voyage related information would be essential to fulfil the identified information needs,

- study the possibilities of using AIS binary messages as additional source of information and to critically analyse the present content of these messages to commonly define such message contents that provide for the requirements of the above mentioned authorities,

- identify a demonstrated need for information not presently included in any AIS information and, if such a need is obvious, agree on the format and content of a new binary message including this information,

- test the usability of binary messages in field tests at least in the Gulf of Finland (transmission tests are made between GOFREP Traffic Centre Helsinki Traffic and ships) but not necessarily limited to that area,

- discuss the project with IALA and HELCOM to gain information essential for conducting of this work and to share the results achieved,

- produce a proposal to IMO NAV Sub-Committee in 2008 for amending AIS operation on the basis of the identified needs for modifications to AIS static and voyage related information,  to the predefined binary messages and of the possible new binary message(s) and to report the results of the trial use of binary messages at the Baltic Sea,

- provide means to enhance the SAR, VTS / SRS and environment protection operations and enable more reliable and efficient information exchange by e.g. reducing VHF radio traffic where practical and simultaneously increasing the information available to authorities and

- identify and document any additional results achieved in the project such as the enhanced use of AIS or the means onboard providing for efficient use of AIS and especially the use of binary messages.

# Working plan

Phase 1: January – March 2007

In the first phase of the project a more detailed project plan is finalized by the project partners. Based on the documentation provided by IMO, IALA, IEC and ITU a description of the present state of AIS development and the possibilities for changes are determined. Information on the content of the ongoing related development work e.g. in Russia (use of alternate VHF frequencies) is looked into and possible needs for cooperation discussed. The views of the national maritime and environment authorities, coast guard (including search and rescue operations) and military on the needs for amending the AIS information available and the formats used is discussed and documented. This information is collected to create basic knowledge needed in the second phase of the project. Preparations for the trial tests of binary message exchange between ships and Helsinki Traffic/Helsinki VTS is also started during this phase. Discussions with shipping companies are started to reach cooperation agreements to ensure that sufficient number of vessels participate in the tests.

The result of the first phase is a detailed action plan and documented views of Finnish authorities on the needs for development. In addition, a presentation and an information sheet of the project are made. These documents can be distributed to the authorities of the Baltic Sea countries and Norway when they are invited to cooperate in this project. The project is presented to IALA and HELCOM to gain their support for the development and to obtain valuable information.

If the partners of this project agree that a preliminary questioning (e.g. web-based inquiry) needs to be conducted to gain sufficient information for ThinkThank-session held during the second phase, it shall be completed during the this phase.

Phase 2: April – July 2007

During the second phase material describing the project is delivered to the relevant authorities in all Baltic Sea countries and they are invited to participate in the project in a ThinkTank-session held for defining common views on the needs for AIS information modifications and new binary messages. The ThinkTank-session is conducted as an organized brainstorming and evaluation process with the help of a computer based group decision support system (GDSS). With this working method, it is possible to effectively collect, store and categorize the views of participants and produce summations as the work proceeds. The plans for binary message exchange testing and the required technical facilities are also produced during this phase. Vessels participating in the tests are chosen and information concerning their AIS equipment noted.

The result of the second phase is the material produced before and during the ThinkTank-session. This material is essential to be able to proceed with the work in the third phase.

Phase 3: August 2007 – March 2008

Testing of exchange of binary messages between vessels and Helsinki Traffic/Helsinki VTS is realized during the third phase. If any other country wishes to participate in the testing, they are welcomed and plans for testing are shared. The work concentrated on defining the modification needs to static and voyage related information, to predefined binary messages and on identifying needs for new binary messages continues. For this purpose a second ThinkTank-session is organized. Preliminary results of the testing of exchange of binary messages are reported in this second session. The objective of this session is to reach an agreement on the AIS information modification needs. Based on the agreement, a proposal is prepared to IMO NAV Sub-Committee for amending AIS operation on the basis of the identified needs for modifications to AIS static and voyage related information, to the predefined binary messages and of the possible new binary message(s). Also the results of the trial use of binary messages at the Baltic Sea is reported to IALA, IMO and HELCOM.

The results of the third phase are a report of the testing of exchange of binary messages, a definition of the modification needs to AIS static and voyage related information and to the predefined binary messages and the need to implement new binary message(s). All additional observations related to AIS operation that are produced during the project are also reported. The proposal to IMO shall also be written during the third phase.

SN/Circ. 236 Guidance on the application of binary messages. IMO 28.5.2004.

IALA Guideline No. 1028 On The Automatic Identification (AIS) Volume 1, Part I, Operational Issues (Edition 1.3 December 2004)

SN/Circ. 236 Guidance on the application of binary messages, Annex 1 Guidance on AIS binary messages. IMO 28.5.2004.