ANNEX 9

DRAFT AMENDMENTS TO RESOLUTION A.802(19)

DRAFT PERFORMANCE STANDARDS FOR

SURVIVAL CRAFT AIS SEARCH AND RESCUE TRANSMITTER (AIS-SART STATION)

1	INTRODUCTION

The 9 GHz SAR transponder (SART) AIS Search and Rescue Transmitter (AIS-SART) station, in addition to meeting the requirements of the relevant ITU-R Recommendation and the general requirements set out in resolution A.694(17), should comply with the following performance standards.

2	GENERAL

The AIS-SART station should be capable of indicating the location of a unit in distress on the assisting AIS stations radars by means of a series of equally spaced dots (see resolution A.530(13)) visual indication on the Minimum Keyboard and Display (MKD) and other navigational equipment capable of processing and displaying AIS information such as ECDIS, radar or an integrated navigation system available on board the ship.

2.1	The AIS-SART station should:

.1	be capable of being easily activated by unskilled personnel;

.2	be fitted with means to prevent inadvertent activation;

.3	be equipped with a means which is either visual or audible, or both visual and audible, to indicate correct operation and to alert survivors to the fact that a radar has triggered the SART;

.4	be capable of manual activation and deactivation; provision for automatic activation may be included*;

.5	be provided with an indication of the stand-by condition; be capable of withstanding without damage drops from a height of 20 m into water;

.6	be capable of withstanding without damage drops from a height of 20 m into

water;be watertight at a depth of 10 m for at least 5 min;

.7	be watertight at a depth of 10 m for at least 5 min;maintain water tightness when subjected to a thermal shock of 45C under specified conditions of immersion;

.8	maintain watertightness when subjected to a thermal shock of 45°C under

specified conditions of immersion;be capable of floating if it is not an integral part of the survival craft;

.9	be capable of floating if it is not an integral part of the survival craft;be equipped with buoyant lanyard, suitable for use as a tether, if it is capable of floating;

.10	be equipped with buoyant lanyard, suitable for use as a tether, if it is capable of

floating; not be unduly affected by seawater or oil;

.11	not be unduly affected by seawater or oil;be resistant to deterioration in prolonged exposure to sunlight;

.12	be resistant to deterioration in prolonged exposure to sunlight;be of a highly visible yellow/orange colour on all surfaces where this will assist detection;

.13	be of a highly visible yellow/orange colour on all surfaces where this will assist

detection;have a smooth external construction to avoid damaging the survival craft;

.14	a smooth external construction to avoid damaging the survival craft; andbe provided with an arrangement compatible with the antenna pocket in a survival craft in order to bring the antenna of the AIS-SART station to a level of at least 1 metre above sea level, together with illustrated instructions.

.15	be provided with a pole or other arrangement compatible with the antenna pocket

in a survival craft in order to comply with 2.4, together with illustrated

instructions;transmit its position during operation with an repetition rate of at least 1 minute;

.16	update its position during operation with an update rate of maximum 5 minutes;  and

.17	be capable of being tested for all functionalities by means of a test button.

2.2	The AIS-SART station should have sufficient battery capacity to operate in the stand-by

condition for 96 h and, in addition, following the stand-by period, to provide transponder

transmissions for 8 h when being continuously interrogated with a pulse repetition frequency

of 1 kHz.within a temperature range of -20˚C to +55˚C, and to provide for testing of the functions on the equipment.

2.3	The AIS-SART station should be so designed as to be able to operate under ambient temperatures of -20C to +55C.  It should not be damaged in stowage throughout the temperature range of -30C to +65C +70C.

2.4	The height of the installed antenna of the AIS-SART station should be at least 1 m above sea-level.

2.5	The vertical polar diagram of the antenna and hydrodynamic characteristics of the device

should permit the SART to respond to search radars under heavy swell conditions. The polar

diagram of the antenna should be substantially omnidirectional in the horizontal plane.

Horizontal polarization should be used for transmission and reception. An AIS-SART station in operation should be detectable at a distance of at least 5 nautical miles by another AIS station with an antenna height of 15 meters, complying with resolution MSC.74(69). AIS-SART stations should also be detectable by airborne AIS stations at a distance of up to 50 nautical miles, at a height of 3,000 ft.

2.6	The SART should operate correctly when interrogated at a distance of up to at least 5

nautical miles by a navigational radar complying with resolutions A.477(XII) and A.222(VII),

with an antenna height of 15 m. It should also operate correctly when interrogated at a distance

of up to 30 nautical miles by an airborne radar with at least 10 kW peak output power at a height

of 3,000 ft. If an on-board test is performed using an AIS Class A station, the test should be limited to a minimum number of sequences.

2.7	The AIS-SART station should continue transmission even if the position and time synchronisation from the positioning system is lost or fails.

3	TECHNICAL CHARACTERISTICS

Technical characteristics of the AIS-SART station should be in accordance with Recommendation ITU-R M.628-2. relevant ITU recommendations.

4	LABELLING

In addition to the items specified in resolution A.694(17) on general requirements, the following should be clearly indicated on the exterior of the equipment:

.1	brief operating instructions; and

.2	expiry date for the primary battery used.

___________

* IEC 60945

* If an on-board test is performed using a shipborne 9 GHz radar, activation of the SART should be limited to a few seconds to avoid harmful interference with other shipborne radars and excessive consumption of battery energy.