e-NAV 2 / 10 / 03

The Future of Racons

- Introduction

- This paper is a summary of a study report, prepared by Prof. A.P. Norris for the General Lighthouse Authorities of the UK & Ireland. The Committee is invited to note it for information and consider whether there are consequences for IALA policy and strategy.

# Background

- The International Maritime Organization (IMO) has encouraged the development of marine radars using coherent processing as these give the possibility of improved detection of small targets in clutter. These are sometimes known as New Technology (NT) radars. These radars are not necessarily compatible with existing racons and therefore IMO, after due deliberation, has dropped the requirement for racon compatibility for new S-Band radars installed after 1 July 2008.

- Although racon compatibility at X-band remains a current requirement, there may be future pressure for New Technology to be extended to X-band radars, provided the potentially significant improvements in target detection are realised at S-band. This could necessitate a replacement for existing X-band racons. This study is a preliminary investigation of the available options.

## NT Radar

- Practical realisations of NT radar involve the use of semiconductor based transmitters. These radars utilise pulse compression techniques, transmitting very long pulses of low peak power. These pulses are difficult to detect by existing racons, which also cannot replicate the transmitted waveform.

- The low transmitted peak powers greatly increase transmitter reliability. Also, with the necessary fine control of the transmitted signal, much reduced RF spectrum contamination is generated, compared to a magnetron-based radar. This is of great interest to the International Telecommunication Union (ITU). These additional benefits increase the attractiveness of NT radars.

- The first S-band NT radar was announced by Kelvin Hughes at the Ship and Machinery Exhibition at Hamburg in September 2006.

# Effect on Racons

- There are two major reasons why existing modern (frequency agile) racons do not properly detect NT radars:

1. Low transmitted powers of the radar reduces the effective range of the racon

2. Most existing racons are designed to ignore the long pulses that are typical in length to that used for NT radar.

## Options Studied

- A number of options have been studied, including:

A. Increasing the required sensitivity of racons in conjunction with putting some requirements on NT radars, such as transmitting a ‘racon interrogation pulse’ that exceeds a minimum effective radiated power and meets constraints on its length

B. The design of a Universal Radar Beacon (URB) that can respond to any radar

C. The use of a transponding beacon that is independent of radar and GNSS

# Recommendations

- Option A potentially provides the lowest cost solution but puts significant constraints on radar design. It is recommended that this is given further study

- Option B appears to be future-proof but stretches available technology and is presently expensive and power hungry. However, it may become practical and affordable within the expected timescales needed for a racon replacement programme (10-15 years) and further study is therefore recommended

- Option C appears practical but it would require all vessels to carry additional equipment, which is likely to have a similar cost to AIS. This may create opposition within the shipping community and it is not recommended that this option is explored.