AISM Association of Internationale de Signalisation Maritime  IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

IALA Recommendation A-124

On

Automatic Identification System (AIS) Shore Station and Networking Aspects relating to the AIS Service

Working towards Edition 2

Future

Edition 1.0 issued December 2002

Edition 1.1 issued December 2004

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone +33 1 34 51 70 0 Telefax +33 1 34 51 82 05

E-mail - iala-aism@wanadoo.fr      		Internet -  http://iala-aism.org

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

Recommendation on Automatic Identification System (AlS)

Shore Stations and networking aspects relating to the AIS Service

(Recommendation A-124)

THE COUNCIL:

RECALLING the function of IALA with respect to Safety of Navigation, the efficiency of maritime transport and the protection of the environment;

RECOGNISING that IMO has concluded that AIS will improve the safety of navigation and the protection of the environment;

RECOGNISING ALSO that there is a mandatory carriage requirement for AIS equipment on SOLAS Convention vessels that entered into force on 1 July 2002 and will be complete by [2008];

RECOGNISING FURTHER that the Association has recommended that National Members providing aids to navigation services, including VTS, consider the introduction of AIS facilities in their shore infrastructure ( IALA Recommendation A-123)

NOTING that National Members intending to introduce AIS facilities in their shore infrastructure need information on the operational and technical aspects of such facilities;

NOTING ALSO that IALA has contributed substantially to the development of AIS through collaboration with IMO, ITU and IEC;

CONSIDERING that there is an urgent need for shore based AIS infrastructure to facilitate the interchange of data with shipping;

CONSIDERING ALSO the proposal of the [11th] session of the AIS Committee for the requirement for approval of the draft recommendation and subsequent publishing to the IALA Web site;

APPROVES the “Standard for AIS Shore Stations and networking aspects related to the AIS Service” set out in the annex to this recommendation for consideration by National Members; and,

RECOMMENDS that National Members introducing AIS facilities into their shore infrastructure take into account the “Standard for AIS Shore Stations and networking aspects related to the AIS Service” set out in the annex to this recommendation.

IALA Recommendation on AIS Shore Stations and networking aspects related to the AIS Service

Table of Contents (simplified)

Part I – General

Part II – Basic AIS Services (BAS)

Part III – Aspects relating to the AIS Service architecture

Part IV – Functionality of the Logical AIS Shore Station (LSS) layer

Part V – Functionality of the Physical AIS Shore Station (PSS) layer

Part VI – Functionality of the AIS Service Management (ASM)

Parts VII – [xxx] – Aspects of the Technical Operation of an AIS Service

Annexes and Attachments

Table of Contents

[to be introduced at a later time]

Part I – General

--- whole chapter needs update

--- IALA’s A-123 “Provision of ....” needs to be addressed

--- the reference section needs update in particular => ITU, IEC, IMO specs

--- description on AIS Service architecture relocated to new part III: Aspects relating to the AIS Service architecture

# IMO's provisions for AIS shore infrastructure (A123)

IMO's SOLAS Convention, as revised, Regulation 19, §2.4.5, states with regard to the purpose of the AIS:

"AIS shall

1.	provide automatically to appropriate equipped shore stations, other ships and aircraft information, including ship's identity, type, position, course, speed, navigational status and other safety-related information;

2.	receive automatically such information from similarly fitted ships;

3.	monitor and track ships; and

4.	exchange data with shore-based facilities."

In addition, the IMO Performance Standards for the AIS state:

"1.2	The AIS should improve the safety of navigation by assisting in the efficient navigation of ships, protection of the environment, and operation of Vessel Traffic Services (VTS), by satisfying the following functional requirements:

.1	in a ship-to-ship mode for collision avoidance;

.2	as a means for littoral States to obtain information about a ship and its cargo; and

.3	as a VTS tool, i.e. ship-to-shore (traffic management).

1.3	The AIS should be capable of providing to ships and to competent authorities, information from the ship, automatically and with the required accuracy and frequency, to facilitate accurate tracking. Transmission of the data should be with the minimum involvement of ship's personnel and with a high level of availability.

1.4	The installation, in addition to meeting the requirements of the Radio Regulations, applicable ITU-R Recommendations and the general requirements as set out in resolution A.694(17), should comply with the following performance standards."

From this the provision of IMO for AIS shore infrastructure can be inferred. It should be noted however, that there is neither a stipulation of IMO to any competent authority to implement a VTS nor to implement AIS into existing VTS. However, since IMO stated, that AIS improves the safety of navigation and operation of VTS, competent authorities should consider implementing AIS into VTS.

IMO's provision for AIS shore infrastructure was taken up by ITU-R when creating Recommendation ITU-R M.1371, which included a so-called AIS base station. When drafting a test standard for Class A and Class B shipborne AIS stations, IEC also took the existence and specific role of AIS base and repeater stations into consideration.

# ITU's additional operational requirements for AIS shore infrastructure (A123)

As a peer organisation to IMO, ITU recognised the potential of the AIS also for areas of shore-based application, other than ship reporting and VTS, namely maritime, safety-related information services, Aids-to-Navigation and Search and Rescue:

"The ITU Radiocommunication Assembly considering (...)

d)	that such a system should be used primarily for surveillance and safety of navigation purposes in ship to ship use, ship reporting and vessel traffic services (VTS) applications. It could also be used for other maritime safety related communications, provided that the primary functions were not impaired;

e)	that such a system would be capable of expansion to accommodate future expansion in the numbers of users and diversification of applications, including vessels which are not subject to IMO AIS carriage requirement, Aids-to-Navigation and Search and Rescue." (Recommendation ITU-R M.1371-1)

Hence, the VTS would not be the only shore-based application to which the AIS would be of relevance. Hence, the design of the technical AIS shore infrastructure, in order that it may be used universally, should be designed such that it would not limit the use of AIS information to VTS's needs, only. This statement does not reduce the prominent role of the VTS as the primary shore-based user of the AIS information. It imposes a fundamental technical design philosophy, however, which will be explained in more detail below.

# AIS as a maritime, safety-related information service (A123)

From a VTS or, more generally speaking, from the point of view of a competent authority the AIS provides an information service for shore-based VTS, traffic management schemes, ship reporting systems and other shore-based safety-related services.

This service consists of information delivery between ships and shore and vice versa. Thus the service of information exchange between ships and maritime, safety-related shore services, such as VTS, is one important part of the AIS (refer to SOLAS Regulation 19, §2.4.5, Nr. 4, as cited above). This information comprises, amongst others, the maritime, safety-related data items listed in IMO SOLAS Regulation 19 (see above).

Consequently, approaching the AIS from any shore-based application's point of view, there will be an AIS Service delivered at a functional interface, which will be defined in more detail below.

# The AIS Service: its place within the shore-based technical environment (A123)

Figure 4.1 shows an overview of the functional technical layers needed to process data derived from various shore-based sensor services to present the traffic image to the user. These layers above the different services will be dependent on user requirements and complexity of the infrastructure. This will not be described herein.

Figure 4.1: Functional technical layers

Radio

Direction

Finding

Service

User, e.g.  VTS-Operator or MRCC-Operator.

Provide Human-Machine-Interface

Provide Operational Application(s) for this User

(based on all available ship data)

Ship Data Processing and Storing Services

Long

Range

Monitoring

Service

Ship's Position / Dynamic Data Correlation Service

.....

.....

(additional services)

.....

(additional services)

.....

(addi-tional

services)

AIS Service

Radar Service

.....

(additional

position

related

services)

The AIS Service of a competent authority Comprises all AIS-related functionality below the integration level f AIS-derived information with other information sources or sinks. In a VTS environment the AIS-derived information of the AIS Service would be integrated e.g. with the Radar Service, the Direction Finding Service, Ship Data Processing Services, etc.

According to need, the competent authority may define more than one AIS Service, e.g. East Coast AIS Service and West Coast AIS Service, which may have different properties. When defining more than one AIS Service within one competent authority, it is beneficial to have complete operational, geographical, physical, and data separation between these services to avoid interference. Should a complete separation of different AIS Services, also of different neighbouring competent authorities, not be possible, the competent authority or authorities should take any measure to minimise the potentially harmful interference of their AIS Service.

For simplicity's sake, this document, in its present edition, confines itself to the treatment of one AIS Service in one competent authority, only.

[here also part of Bruce’s paper Sarasota input 10 can go in]

# The functional interface between the shore-based applications and the AIS Service (new Annex 1)

The requirements for the interface between shore-based applications and the AIS Service can be defined in functional terms as follows:

- This functional interface should provide the AIS Service to shore-based applications as a set of Basic AIS Services (BAS).

- The functional interface should facilitate the integration of the AIS Service shore-based applications, such as radar-based VTS environments.

- The functional interface encapsulates the technical details of both the AIS technology and of the layout and local configuration of shore-based AIS. (This is a state-of-the-art engineering principle. It protects application software from unnecessary changes due to configuration or technology improvements within the AIS Service.)

The interface between the AIS Service and its clients e.g. the VTS environment would also comprise a list of available Basic AIS Services (selection or all) and would consist of a definition of all Logical AIS Shore Stations (LSS) set up in this particular VTS environment.

## Basic AIS Services

The purpose and functions of the AIS can be expressed in terms of services provided to the recipient. The most fundamental services of the AIS are called Basic AIS Services (BAS). They make use of the diverse features of the AIS VHF Data Link (VDL; as described in Recommendation ITU-R M.1371-1 in connection with the IALA Recommendation on Technical Clarifications of Recommendation ITU-R M.1371-1) and the diverse features of the different AIS stations. The BAS are described in a common format.

This description of the BAS does not make redundant the referenced documents, i.e. the appropriate international standards, nor do they introduce new system features. However, each description of a BAS integrates, in a comprehensive and highly accurate manner, all information items from various sources into functional terms that are essential to understand what this BAS is delivering. Therefore, the BAS are also the basis for an assessment of the usefulness of the AIS Service for a particular intended application in terms of accuracy, frequency, reliability etc.

The BAS are subdivided into "external" and "internal". The "external" BAS are those BAS which deliver net information about ships and shipboard applications. The "internal" BAS are those which are needed to make use of the special capabilities of the AIS base station with regard to the AIS VDL.

The BAS are the services the AIS delivers to the recipient's side ashore at the interface between the AIS Service and "higher", application-oriented levels. Therefore, their description also takes into account the processing at the fixed AIS stations, the Physical AIS Shore Station (PSS), and the LSS.

## BAS Overview

The following list provides an overview on all defined BAS. The "internal" BAS are contained and managed within the AIS Service and may be triggered and / or configured by higher levels of shore-based infrastructure. All available BAS will be described in detail in the Part II.

[--- there is a more mature insight into the following list available; JHO]

### External BAS

- A_STAT		Static ship data from Class A shipborne mobile AIS stations

- A_DYN		Dynamic ship data from Class A shipborne mobile AIS stations

- A_VOY		Voyage related ship data from Class A shipborne mobile AIS stations

- SAFE_AD	Safety related addressed message

- SAFE_BR	Safety related broadcast message

- INT_TDMA	interrogation via AIS VDL

- INT_DSC		interrogation via DSC Ch 70

- B_DAT		Ship data from Class B shipborne mobile AIS stations

- SAR_DAT	Data from SAR airborne AIS stations

- ATON_DAT	Data from AtoN AIS stations

- TRANS_IAI	Transparent transmission within the International Application Identifier branch

- TRANS_RAI	Transparent transmission within the Regional Application Identifier branch

### Internal BAS

- BASE_DAT	Data on base station (base station's own data)

- ASGN_RATE	Assignment of reporting rate to mobile station(s)

- ASGN_SLOT	Assignment of transmission slots to mobile station(s)

- DGNS_COR	DGNSS corrections to mobiles

- CH_TDMA	Channel management by TDMA

- CH_DSC		Channel management by DSC

- PWR_LEV	Power level setting to mobiles

- FATDMA		(announce) configuration of FATDMA reservations of base station(s)

# Scope of this document (A124 General)

[this section needs update]

## Scope in terms of layers covered

As a conclusion of the above discussion, the scope of this document can be defined in terms of the layers addressed:

This document covers all layers from the ASM  layer downwards to the fixed AIS station's interface to the VHF-/RF-domain equipment, i.e. the so-called radio front end of the fixed AIS stations.

The BAS, which are the interface of the AIS Service and of the LSS to higher levels will be described in the IALA AIS Guidelines (Edition 1.2), Volume II.

This document contains definitions of the following interfaces:

a) interface between the fixed AIS stations and the VHF / RF domain equipment i.e.:
radio interface at antenna plug;

b) interface between the PSS and the fixed AIS stations i.e.: the Presentation Interface of the fixed AIS stations.

The document is subdivided in parts accordingly.

## Scope in terms of depth of description

This document provides functional descriptions of what a particular entity delivers. It does not contain implementation details.

## Scope in terms of functionality separation

[this section appears outdated]

The functionality of different fixed AIS stations, as identified by Recommendation ITU-R M.1371-1, is described separately, although some functionalities may run simultaneously at run-time in one device as indicated by Table 9.1.

Table 9.1: Possible / Impossible combinations of functionality of fixed AIS stations at run-time

|  | AIS base station functionality | AtoN-AIS-station functionality | AIS simplex repeating | AIS duplex repeating |
| --- | --- | --- | --- | --- |
| AIS base station functionality | N/A | Possible | Possible | Not possible |
| AtoN-AIS station functionality | Possible | N/A | Possible | Not possible |
| AIS simplex repeater | Possible | Possible | N/A | Not Possible |

It is possible, in principle, that manufacturers develop fixed AIS stations, which may be configured as AIS duplex repeaters or as any of the above AIS simplex stations at different points in time.

# References (A124 General)

The following documents are referenced throughout the document by the using the abbreviation given in brackets.

## IMO references

AIS shore stations are necessary pre-requisites for IMO's requirement for AIS being a "VTS tool".

- [SOLAS] “IMO Resolution MSC.90(73) Annex 7, Adoption of amendments to the international convention for the safety of life at sea, 1974, as amended”

- [A3] "IMO Resolution MSC.74(69) Annex 3 Recommendation on performance standards for AIS"

- [IMOCircFreqMan] "IMO Sub-Committee on Safety of Navigation (NAV); Liaison statement to ITU-R Working Party 8B" (Report NAV47/13, Annex 15; 26 July 2001) on use of Frequency management in the AIS

- [RNAV] Draft Revision of "IMO Resolution A.815(19):1995 World wide radio navigation system"; needed for requirements of any DGNSS service provided by AIS stations

## ITU references

- [M1371] "Recommendation ITU-R M.1371-1, Technical Characteristics for a Universal Shipborne Automatic Identification System using Time Division Multiple Access in the VHF maritime mobile band."

- [M825] "Recommendation ITU-R M.825-3, Characteristics of a Transponder System using Digital Selective Calling Techniques for use with Vessel Traffic Services and Ship-to-Ship Identification"

- [RR] ITU-R Radio Regulations, Appendix S18

- [M1084] "Recommendation ITU-R M.1084-4, Interim Solutions for Improved Efficiency in the Use of the Band 156-174 MHz by Stations in the Maritime Mobile Service"

- [M823] "Recommendation ITU-R M.823-2; Technical characteristics of differential transmissions for Global Navigation Satellite Systems from maritime radio beacons in the frequency band 283.5-315 kHz in Region 1 and 285-325 kHz in Regions 2 and 3"

- [ReplyFreqMan] Liaison Statement of ITU-R Working Party 8B to IMO with regard to Frequency Management (reply to IMOCircFreqMan).

- [585] “Recommendation ITU-R M.585-2, Assignment and use of maritime mobile service identities”.

- [210] ITU-T E.210, Ship station identification for VHF/UHF and maritime mobile-satellite services.

## IALA references

There are the following IALA references:

- [Clarifications] “IALA Recommendation on Technical Clarifications on Recommendation ITU-R M.1371-1” (edition 1.3)

- [Guidelines] “IALA Guidelines on Universal ship-borne Automatic Identification System (AIS)” (2 volumes, 3 parts).

- [IALABeacons] IALA Recommendation on Maritime Radio Beacons

## IEC references

The IEC test standards apply to the Class A and Class B shipborne mobile AIS stations. Due to the co-operative nature of the AIS as a whole, these IEC test standards need to be considered in the present document to the extent that functionality of these mobile stations require certain AIS shore station functionality. Also, the test section of the present document was informed by the IEC test standards for Class A and Class B shipborne mobile AIS stations. In addition, the Presentation Interface message definitions contained in IEC 61162-series have been used and adapted to the needs of AIS shore stations.

- [61993-2] "Maritime Navigation and Radiocommunication Equipment and Systems – Automatic Identification systems (AIS); Part 2: Class A shipborne equipment of the Universal Automatic Identification system (AIS) – Operational and performance requirements, methods of test and required test results" (2001)

- [62287] "Maritime Navigation and Radiocommunication Equipment and Systems - Class-B ship-borne installation of the universal automatic identification system (AIS) using TDMA techniques – Operational and performance requirements, methods of test and required test results” (future publication)

- [61162-series] "Maritime Navigation and Radiocommunication Equipment and Systems"

- [61162-1] "Part 1: Single talker and single listeners", as revised 2001.

- [61162-2] "Part 2: Single talker and multiple listeners, high-speed transmissions", as revised 2001.

- [61162-3] "Part 3: Multiple talker and multiple listeners, high-speed network bus" (future publication).

- [60950] IEC 60950 “Information Technology Equipment – Safety - Part 1:General Requirements”

Part II – the Basic AIS Services(new Annex 1)

--- included here the former Vol II of the IALA AIS Guidelines (= eNAV1-09-07)

--- needs heavy (!) editing and amendment

--- needs confinement to the technical issues

# Basic AIS Services (new Annex 1)

## Introduction

The purpose of this document is to provide first level understanding of an AIS Service and the Basic AIS Services (BAS). It will present an organization diagram illustrating loose relationships, then consolidate this same information into a relational representation in the form of a cube. Then we will relate this with a method for communicating the Functional Requirements using the Unified Modeling Language (UML).

## Lets start with the organizational representation

We have a Competent Authority (CCG). It has policies, regulations, resources and provides service (public service in our case). To provide this service it needs other resources that work within its management framework (policy, regulations etc.). Our example has Office of Boating Safety (OBS) Service, MCTS Service, AtoN Service – it could also have included all services NWPA, Fleet, SAR etc. Each of these services use other services to deliver their service i.e. MCTS needs Communications Service, Radar Service to get or distribute their information. Each of these services are comprised of basic services. Radar provides the basic service of Radar position, Radar range, point of coincidence. AIS is also a service – it provides Basic AIS Services in the same manner as other services i.e. AIS position, AIS range, AIS speed over ground etc. This illustrates the relationship that exists between all of the services from Basic to General.

Each service on every level can be described – CCG Service, MCTS Service, etc. There are many methods to describe them – verbal, pictorial, text, combinations of multi-media. There is also the possibility to describe or represent the service using a modeling language called UML. This is an established and mature language that can provide the building blocks of description in a language (the L in UML) that is universal (the U in UML is actual Unified but is derived from a series of previous modeling tools hence Unified).

Each service can be represented by a UML “Component”. Each service or Component will have a relationship to the other Components – the AIS Service “belongs to” the MCTS Service and a BAS “belongs to” the AIS Service. Each Component then has only to list the interface criteria for each relationship, one from the point of view of an input the other as an output. These criteria need to match.

It therefore follows that if we build a system of components with their interface we describe a total Service eventually.

CCG Service

Office of Boating Safety

(OBS) Service

MCTS Service

AtoN Service

Ais Service

DR Service

Radar Service

Radar Services

BAS Services

B

AtoN

S

BAS

BAS

BRS

Figure 1

## Relational representation

Figure 2 has taken the organization view of Figure 1 and transformed it into a more “logical” presentation with respect to the relations. The lines in Figure 1 that usually mean “flows to” are replaced by blocks that denote “belongs to”. The third dimension indicates that the system of relationships has to be duplicated for each instance of the service. It should also be noted that there are often multiple interfaces to a Component (manager of it, other services, conditional influence).

CCG Service

Office of Boating Safety

(OBS) Service

MCTS Service

AtoN Service

Ais

Service

Radar

Service

Basic Radar Services

BRS

Ais Service

DR

Service

Basic DR Services

Basic OBS

Services

Basic AIS Services

Basic AtoN Services

Ais Service

BAS

BOBSS

BAS

BRS

BDRS

BAtonS

Site # 1

Figure 2

## So why consider UML?

By taking the time to work in a modeling language we eliminate much ambiguity. The rules are fairly rigid for UML and they are logical. The same can not be said for any oral/written language – even this text that I am writing is being understood differently by each reader. The UML is less ambiguous. Figure 3 shows the symbol for a UML Component with an interface. There are other relational elements available to represent relationships to other Components and if a Component “belongs to” another it can be nested. The purpose of this paper is not to give a lesson on UML, it only attempts to illustrate that it is a language that can serve our purpose. The main reason to use UML lies in the fact that if we describe a service using UML and another Competent Authority provides the same service, it can use the same model (the M in UML) to describe its service and not have to develop a new description. On an international level this could be used to standardize our services.

Figure 3   UML Component with Interface

## Subdivision of the Basic AIS Services

The BAS are subdivided into "external" and "internal". The "external" BAS are those BAS which deliver net information from and to ships and shipboard applications, the direction of the data flow depending on the nature of the BAS.

The "internal" BAS are those which are needed in particular to perform the "system management" functionality as defined by Recommendation ITU-R M.1371-1, Annex 2, Table 13. They do not deliver net information about ships and shipboard applications, but some "internal" BAS may be needed to provide "external" BAS at all under certain circumstances. The "internal" BAS are contained and managed within the AIS Service and may be triggered and / or configured by higher levels of shore-based infrastructure.

## Motivation for the description of the Basic AIS Services

The description of the Basic AIS Services can serve the following purposes.

### The Basic AIS Services as the interface between operational and technical considerations

"External" BAS produce, transfer, and accept information to and from higher levels that are not themselves part of the AIS Service.  The functionality of higher level services and AIS applications are built upon the capabilities of these services. This is illustrated by

The "external" BAS are the services the AIS Service delivers to the recipient's side ashore at the interface between the AIS Service and "higher", application-oriented levels. Therefore, their description also takes into account the processing at the fixed AIS stations, the Physical AIS Shore Station (PSS), the Logical AIS Shore Stations (LSS), and the AIS Service Management (ASM).

Therefore, the BAS are also the basis for an assessment of the usefulness of the AIS Service for a particular intended application in terms of accuracy, frequency, reliability etc.

### The Basic AIS Services as a means to convey consistent relevant information from the recipient's point of view

When making use of the potential of the AIS by setting up an AIS shore infrastructure, which is integrated into e. g. a VTS environment, the VTS (re-)designer, who formerly had not been involved in shipborne equipment issues at all, will now need to receive and use information on the AIS shipborne installation.

This is due to the fact, that the introduction of the AIS as an automated, co-operative system triggers the integration of shipborne and VTS technologies, which formerly existed more or less independently of each other. The same applies – in principle, but more on an operational level with less technical sophistication – to the VTS operator(s).

The situation is further complicated by the fact, that relevant information is distributed in a variety of relevant documents, the number of which may still be growing and which are maintained by various international or regional organisations. It becomes obvious that it may be difficult to identify all relevant documents for a particular issue at hand, keep track of them or even control them to reflect the intent and current status of the AIS.

The BAS descriptions aim at making available relevant information for a given issue at hand for a system (re-)designer using the AIS Service.

This description of the BAS does not make redundant the referenced documents, i.e. the appropriate international standards, nor do they introduce new system features. However, each description of a BAS integrates, in a comprehensive and highly accurate manner, all information items from various sources into functional terms that are essential to understand what this BAS is delivering.

### The Basic AIS Services as the functional, technical interface between the AIS Service and higher technical levels

Besides being the means to assess the operational usefulness and features of the AIS Service to higher levels and applications, the BAS – in their precise technical description – constitute the functional, technical interface between the AIS Service and higher technical levels. This is indicated in Figure 25.1 in regard to the only physical accesssible interface between the higher levels, applications, and the AIS data transfer network. For the same reason, the BAS constitute the major use cases of the Logical AIS Shore Station layer.

### The Basic AIS Services as a means to encapsulate complexity

The BAS descriptions are also the basis for an assessment of the characteristics of a particular AIS service for a particular intended application in terms of accuracy, frequency, reliability, fallback arrangements etc. An individual BAS, considered as one consistent and prudently partitioned set of functionality of the whole AIS Service may be investigated on its own, thus allowing a concentrated assessment of this particular functionality.

This holds true although there are interactions between certain BAS. Since these interactions will be clearly identified, they may be considered one by one, instead of being overwhelmed by the sheer complexity of the AIS Service as a whole.

### The Basic AIS Services as a means to understand and handle complex configuration states of the AIS Service as a whole

The BAS may also serve as a convenient means to understand and handle complex configuration states of the AIS Service as a whole because of the above features of the BAS, in particular because they encapsulate complexity.

## Concluding remarks

As such, Volume II will be of benefit to manufacturers, system integrators, and users alike.  It provides a basic understanding of what services AIS can deliver.  What services it cannot deliver, and what are the proper conditions under which the service can be used.  It will describe the basic AIS services that support the design of AIS based applications that directly serve the needs of the mariner, vessel traffic service, and littoral state.

## The functional interface between the shore-based applications and the AIS Service

The requirements for the interface between shore-based applications and the AIS Service can be defined in functional terms as follows:

A) This functional interface should provide the AIS Service to shore-based applications as a set of Basic AIS Services (BAS).

B) The functional interface should facilitate the integration of the AIS Service shore-based applications, such as radar-based VTS environments.

C) The functional interface encapsulates the technical details of both the AIS technology and of the layout and local configuration of shore-based AIS. (This is a state-of-the-art engineering principle. It protects application software from unnecessary changes due to configuration or technology improvements within the AIS Service.)

The interface between the AIS Service and its clients e.g. the VTS environment would also comprise a list of available Basic AIS Services (selection or all) and would consist of a definition of all Logical AIS Shore Stations (LSS) set up in this particular VTS environment.

#### External BAS

A_STAT		Static ship data from Class A shipborne mobile AIS stations

A_DYN		Dynamic ship data from Class A shipborne mobile AIS stations

A_VOY		Voyage related ship data from Class A shipborne mobile AIS stations

SAFE_AD		Safety related addressed message

SAFE_BR		Safety related broadcast message

INT_TDMA		interrogation via AIS VDL

INT_DSC 		interrogation via DSC Ch 70

B_DAT		Ship data from Class B shipborne mobile AIS stations

SAR_DAT		Data from SAR airborne AIS stations

ATON_DAT		Data from AtoN AIS stations

TRANS_IAI		Transparent transmission within the International Application Identifier branch

TRANS_RAI		Transparent transmission within the Regional Application Identifier branch

#### Internal BAS

BASE_DAT		Data on base station (base station's own data)

ASGN_RATE	Assignment of reporting rate to mobile station(s)

ASGN_SLOT		Assignment of transmission slots to mobile station(s)

DGNS_COR		DGNSS corrections to mobiles

CH_TDMA		Channel management by TDMA

CH_DSC		Channel management by DSC

PWR_LEV		Power level setting to mobiles

FATDMA		(announce) configuration of FATDMA reservations of base station(s)

## Structure of service descriptions

To facilitate the uniform description of each of the services identified in List 1 - Basic AIS Services (section 4.2) the following format is used.

Service Description

Service Overview

- Service Name:

- Purpose and General Description:

"Who, what, where, when, and why? - newspaper like description"

Early decision, "How do we want to pitch this service?"

Define and describe the service

What does the service deliver?

Why is this service important?

Brief description of service products/benefits

When/where is this service used?

Circumstances under which this service should not be used?

- Use:

Recommendations concerning exploitation of this service.  This service could be used to support the following applications: (missions, operations, surveillance,...)

- Risks:

What are risks - business, safety, technical,...?

How significant is their impact?

Are there measures that can limit the negative effects?

[when using this service mistakenly or with wrong data.

Evaluation of these risks.

Possible measures (on higher levels) that can be used to limit possible negative effects of risks]

- Operation and Support:

Describe start ("trigger event"), support [dependencies, external, internal, ...], and end of this service

Prior Initialization needed for this BAS

Time

Location

Platform condition change

Interdependence of this BAS [service] with another [internal service] BAS

Service Details

- Operating Characteristics:

Timing and Behavior with regard to time

Queuing delays

Conditional behavior

Time of Generation

Time elapsed since Transmission (e.g. aboard) to Reception at point of use (e.g. VTS Center) = Latency issues

Update Interval

- Information:

- Detail of service products and support, Data formats, and Data Content - explicit and / or implicit

- example for explicit information: Latitude in degrees; data format: integer value between ...

- example for implicit information: Use of Message #3 instead of Message #1 indicates that the source AIS now operates in assigned mode = implicit acknowledgement of Assigned Mode Command from Base Station

- Shipboard generation of data item: Automatic generation using sources internal to the shipborne AIS; automatic generation using sources external to the shipborne AIS; manual input at installation; manual input during voyage, etc.

- Resolution and / or accuracy of Data Content (e.g. +/- ... m; binary; etc.)

- Default Value of Data Item

- Messages of M.1371-1 used to realize this BAS

- Integrity precautions taken when generated automatically

- Estimated reliability of information with regard to generation of that data item: high, low, etc. (from the recipient's point of view)

- Implicit or explicit integrity information transmitted

- Consequential fault conditions:

technical

- Shipboard/shore prerequisites for use:

- Technical

[examples: DTE (date terminal equipment), AIS unit revision, AIS class A/B/base, external application specific capabilities,...]

- Regulatory

[examples: ...?]

- Side effects:

- positive

- negative

- Fall back arrangements to other AIS services

List in a priority scheme

- Possibilities / necessity to indicate to higher level state of BAS performance

- Logging possibilities

## Basic AIS Service (BAS) Descriptions - External Basic AIS Services

### A_STAT: IMO SOLAS Static Information

#### Service Overview

This basic service broadcasts the following static information about ships.

-MMSI Number

-AIS type number

-Call Sign

-Name of ship

-Type of ship

-Length and beam of ship

-Height above keel

-Reference location of position sensor

-Type of position sensor

-Functionality of entering and displaying AIS data

The AIS mobile station automatically broadcasts the above static ship information except "height over keel" every 6 minutes. To enter and display this information the AIS mobile station provides, at a minimum, a minimal display function.

Use

-The static ship information is used to identify and classify ships.

-The ship's dimensions are important in order to determine and evaluate the maneuvers of a ship.

-The details regarding the ship's position sensor are important in order to assess the accuracy of the position information.

-The "height-over-keel" information together with other data can be used to determine the height over water level.

Risks

-Because static information on board has to be entered manually it is possible that this data may be faulty.

-The limited precision of the ship's dimensions and the location of the position sensor have to be taken into consideration when evaluating this data.

Operation and Support

-Following normal start-up and synchronization this basic service starts automatically.

-After that, all data will be broadcast every 6 minutes.  If any individual data element is changed the entire data string in immediately transmitted.

-This basic service cannot be turned off either by a base station or by the mobile AIS station itself.

a) The MMSI number, the AIS type number, the IMO number, length and beam of ship have been entered during the original installation of the AIS and cannot be changed by the ship's personnel.

b) Call sign, name, type of ship are pre-assigned during the installation phase and should not be changed by the ship's personnel.

Height-over-keel is a basic initial data element and should not be changed by the ship's personnel. A VTS centre can obtain the height-over-keel information only via polling.

c) The types of the installed position sensors and their reference locations have been predetermined at the time of installation of the AIS unit.  During operation the AIS-unit determines automatically which sensor is being used and what data it is to transmit.

Interaction with other Basic AIS Services

- The type of position sensor and its reference location position as well as the extended use of the length and beam of the ship are important parameters for the ships' data which can be obtained by using A_DYN.

### A_DYN: IMO SOLAS Dynamic Information

#### Service Overview

Purpose and General Description:

The BAS transmits dynamic information from mobile AIS Stations onboard SOLAS V vessels. The current dynamic information is derived onboard the vessel.

Data that is derived from the actual traffic situation and is not static or voyage related is defined to be dynamic information.

- Latitude und longitude of geographical position

- Intigrity of position

- Accuracy of the position

- Time tag of position determination

- SOG and COG

- Heading

- ROT (if available)

- Navigational status

The dynamic information is transmitted in defined data frames periodically in fixed intervals. All sensor data is generated automatically and transferred to the AIS-station except the navigational status. The position, SOG and COG derive from an Electronic Positioning Fixing Device (EPFD).

The BAS is always enabled and operates continuously and autonomous. The information is always broadcasted.

The transmission period (time between reports) depends on the navigational status, the SOG and the maneuvering of the vessel. The transmission rate can be 2 times per second for a vessel with a SOG of 23 knots while maneuvering reduces to 3 times per second while anchoring. Only base stations are capable to change the transmission rate by using ASGN_SLOT or ASGN_RATE.

Use:

The dynamic information combined with the data derived from A_STAT (Static information) and A_VOY (Voyage Information) can be used to visualize the traffic situation.

The information transmitted by A_DYN can by used by other mobile stations and base stations. In a VTS centre it can support the radar tracking and ships data processing.

For the first time it will be possible to control traffic of AIS equipped vessel in region not covered by radar.

Risks:

None

Operation and Support:

Two minutes after switch on a class A mobile station is operational and it works in the autonomous mode. Depending on the navigational status, the maneuvering and the speed of the vessel A_DYN will operate continuously in the autonomous mode.

It is not foreseen to turn off the mobile station during anchoring.

Interaction with other BAS:

The A_DYN depends on:

- A_STAT concerning the type of EPFD and concerning the reference of position to calculate the position, the COG and SOG.

- BAS no 29 concerning secondary synchronization on mobile station

### A_VOY: IMO SOLAS Voyage Information

[optional, Exchange Waypoints and Routeplans (both directions) (IAI FI #17+18)]

#### Service Overview

Purpose and General Description

This AIS Basic Service describes the transmission of voyage related ship data. Only SOLAS V ships (class A) have to transmit voyage related ship data.  The basic service broadcasts  voyage related information.  This information includes the following ship data.

-Draft

-Port of Destination

-ETA

-Type of Cargo

-Waypoints/ Routeplans (optional)

-Number of persons on board

The voyage related data, except for the route plans and persons on board, are broadcast every 6 minutes together with the static ship data.  When required these data can additionally be requested from another AIS unit (re Interrogation see INT_TDMA.  The way points/ route plan information as well as the "number of persons on board" are not broadcast regularly. This broadcast takes place only when the ship's personnel initiates it.  Again, if required, other AIS units can poll for this data.

This AIS Basic Service is enabled at all times.

All data is entered manually and is often subject to the discretion of the ship's personnel.

-The draft must always be entered.

-The entry of port of destination and ETA is at the discretion of the captain. Free form text will be used for the port of destination.  Agreements concerning ways of writing, abbreviations, or codes do not exist.

-The entry of ETA is based on date and time at the port of destination.

-Providing information on the type of cargo is required if the responsible agency so demands it.  The designation of a code for dangerous cargo, or a designation in clear text and quantity is not envisaged.

-Waypoints and route plans may be entered directly into the AIS device.  This process requires additional device functionality.

-The number of persons on board cannot be entered directly into the AIS device.  This, too, requires additional device functionality.

Use:

[Note: item is already described adequately under purpose.]

Risks:

-Manual entries depend on the 'human factor" and are, therefore, only partially reliable.  Since an AIS device only forwards data but does execute any verification of the transmitted data, the sensor (here: person = ship's personnel) has to ensure that only valid data is communicated.

-The time of data entry or the respective decision of the captain is not known. The use of the manually entered data includes, therefore, a certain risk that should not be underestimated that some of the data is old not corrected and up-to-date or even missing. Verification of data or missing up-dates is not possible.

-When waypoints are up-dated on board no automatic transmission by the AIS device takes place.

Operation and Support

The transmission of voyage related data, except waypoint information and number of persons on board, takes place regularly every 6 minutes.  The data are being transmitted independently of the routine schedule if specifically polled for.

Interaction with other Basic AIS Services

INT-TDMA (interrogation of the VDL); TRANS_IAI

### SAFE_AD: IMO Short Safety-related Addressed Messages

(with acknowledgement)

#### Service Overview

Purpose and General Description:

[This message can be used to address to a specific recipient information about safety in readable format. This message is automatically acknowledged by the AIS station.  However, this does not mean that the operator has read the message (acknowledgement does not indicate that the operator has been informed). It should be noted that the AIS unit can only process messages in be pilot position, buoy outage information and a temporary speed limitation.  This service should be used prudently]

- Transmission of the onboard operational (navigational) status in the case of safety onboard is affected or endangered.

- The transmission of navigation information which could mutually affect the navigation of vessels

- Transmission of addressed safety related traffic information from the VTS center to a selected vessel

The IMO defines the safety related message in their requirements as tool for traffic control and collision avoidance. The information is generated automatically or manually at any time. The contents of the message is arbitrary ASCII-text. In case of transmission failure the message is repeated 3 times. The number of repeats may be set by an application. An acknowledgement of the receipt is given only to the AIS system layer.

Class A and Class B mobile stations differ in the performance of the message (Class B maximum length is 2 slots, Class A is 5 slots). The message can be displayed in case of Class B on a minimum display if the DTE flag is not set; in case of Class A the minimum display and keyboard is mandatory. This BAS must accessible under all conditions.

## Use

This BAS can be used for transmission of safety related messages from ship to ship and from ship to shore e.g. fire onboard or failure in a technical system needed for navigation. Applications are recommended to automatically generated and transmitted. The VTS center should invent indicators on the application level for e.g. crossing borders of safety areas (shallow waters, restricted areas). This function can be combined with [DE: initiate] automatic transmission of traffic information addressed to the specific vessel.

Risks:

A minimum display and keyboard is required in SOLAS V vessels. The acknowledgement is given only on the VDL level but no acknowledgement for the contents of the message on higher levels. A safety related information may be missed on board. The transmission starts immediately after initiation of the BAS with a maximum delay of [XXX] sec. Long and often repeated messages stress the capacity of the VHF channel. They also may result long lasting transmission. During VHF disturbance these message may be entirely disturbed. Planning of applications with automatic traffic information should be done with respect to the VDL capacity. Operations on highly frequented VDL-channels may require time slot management to control the system

## Operation and Support

Due to the technical feasibilities of the minimum display and keyboard onboard and the  number of occupied slots the safety related messages should be as short as possible. The message is variable in length and contents. The maximum number of time slots is limited to 5.

Interactions with other BAS:

ASGN_SLOT: Control slot allocation of one or more AIS stations

### SAE_BR: IMO Safety-related Broadcast Messages

#### Service Overview

Purpose and General Description:

This message can be used to broadcast information about safety in readable format to all receivers in the area. This message is limited to maximum "chunks of" about 160 alphanumeric characters. Some examples of this information are real time meteorological and hydrological information, traffic situation, position of hazards. This service should be used prudently (explain prudently).

### INT_TDMA: Interrogation via AIS VDL

#### Service Overview

[Any AIS can send an interrogation to the units in the area requesting a position report and static and voyage related data. This would normally be provided automatically but the user may want the information immediately. The base station may be interrogated for DGNSS correction information, if available, channel management information, position information etc. AtoNs may also be interrogated for their aids to navigation report (name, position, type, status …)]

- Purpose and general description:

- The user can interrogate an AIS station for position information and static and voyage related information, providing the interrogator is permitted and the interrogated stations is in the reception range.

- Vice versa every mobile station can be interrogated according the permission of the interrogating station.

- Additionally mobile stations can interrogate base stations for UTC-time, DGNSS-corrections and Channel and frequency management parameters. With the latter the mobile station can be controlled.

- As a maximum 2 stations can be interrogated with each two set of information like position data and static/voyage data.

Use:

- A base station may use the BAS to interrogate stations in between the periodical transmission to get position and static/voyage information only with slight delay. The stations may be SAR Aircraft, other crafts, base stations or AtoN equipped with AIS systems.

- A base station may also get extended information, especially extended static and voyage information from other stations.

- Mobile stations can interrogate a base station for

- UTC time

- DGNSS correction

- Frequency and channel parameters to control their VDL channel

Risks:

- Many interrogation and answers may overload the VDL channel. The AIS system turns itself off at overload.

- Especially for the (extended) ships data [DE: means Static/Voyage Information] there is a possibility that they are not right or current.

Operation and Support:

- The interrogator has to select the information he wants to interrogate. Either the user defines the delay time or the answer or AIS system defines the delay automatically. After transmission of the interrogation the communicating stations control the process of data exchange automatically.

- If being interrogated the AIS system answers automatically without user action (provided the information is available).

- Interaction with other BAS

- The BAS includes the functionalities of the interrogation and the answer to interrogation.

- The following BAS may be started to get dynamic, static and voyage data:

- A_STAT (Static Information)

- A_DYN (Dynamic Information)

- A_VOY (Voyage Information)

- B_DAT (Class B Dynamic Information)

- SAR_DAT (SAR Aircraft Dynamic Information)

- ATON_DAT (A to N Dynamic Information)

### INT_DSC: Interrogation via DSC(Tampa)

#### Service Overview

Purpose and General Description:

A base station can interrogate an AIS Unit requesting a DSC response

### ASGN_SLOT: Control slot allocation of one or more AIS stations (not only mobile) via assignment (Tampa)

#### Service Overview

Purpose and General description:

AIS stations basically operate in 3 different modes: Autonomous Mode, Assigned Mode, Polling Mode (Interrogated). The default is the autonomous mode. The referring reporting intervals are listed in ITU-R M1371 §4.2. The autonomous mode uses the self organized slot allocation.

Only base stations are capable of slot allocation to have access to the VDL capacity for the use of certain services (assigned mode). This way the reception of far away stations is guaranteed.

Use: If a base station only sporadically receives messages for one mobile station, the base station can assign a mobile station specific slot numbers for use to prevent multiple use of one slot number

### ASGN_RATE: Control reporting rates of one or more AIS stations (not only mobile) via assignment (Tampa)

#### Service Overview

Purpose and General Description:

A base station can control the rate of reporting for AIS stations. This would be done to follow more closely the movement of a vessel (increase the number of the reports) or to prevent interference (decrease the number of reports).

### B_DAT: Current vessel status of ships carrying a Class B shipborne mobile AIS device

#### Service Overview

[This service broadcasts specific static and dynamic information about the ship and the position source used by the AIS unit. This information includes the ship's: name, call sign, length, beam, type, position, COG, SOG, HDG.  The AIS unit also broadcasts the reference point on the ship to which the reported position information is referenced. The dynamic information reporting interval can range from as short as every five seconds for a vessel with a speed over 23 knots, or as long as every three minutes for a vessel not moving faster than 2 knots. The static information is contained in a single report that is broadcast every six minutes, or in response to an interrogation from another AIS unit.  The information is used to increase situation awareness during navigation planning and for collision avoidance.  This information can also be used to improve the waterway management function of a VTS.  The service is normally active.]

Purpose and general description:

This service is used for transmission of current vessel status from non SOLAS V ships (CLASS B).

The BAS transmits in broadcast mode dynamic and static information, and the information with which technique and accuracy the position is determined.

The information includes following data about the ship: MMSI- Number, Ships Name, Call Sign, Length and beam, Type of ship, cargo type, geographical position, COG, SOG, HDG and the position reference point.

If only dynamic information is transmitted, AIS message #18 is used

In the table below the possible reporting intervals are listed.

[Insert M1371 table 1b without the last 3 lines- class B only]

If dynamic and static information is transmitted, AIS message #19 is used. In that case the reporting interval is 6 minutes.

When necessary mobile stations can be interrogated for both messages. (see BAS No.8 Interrogation)

The information is used for the purpose of navigation planning and collision avoidance to be aware of other ships in the area and their behaviour.

The fast and precise presentation of the information about the ships behaviour increases the safety on waterways.

The information can be used to improve the display of the traffic situation in a VTS.

As default the BAS is always enabled.

- The information transmitted by messages #18 or #19 is supplied by:

- the AIS station itself

- set during installation like e.g. MMSI number, Ships Name, Type of Ship

- derived from the system like e.g. time tag, availability of the integrity control or the  availability of the data terminal [DE means:  min. display and keyboard], transmission mode, synchronisation status, slot usage etc.

- sensors, e.g. position, position accuracy (external EPFD)

- manual input, e.g. change of static information (e.g. ships name) or sensor failure

- The automatic delivery of data can produce onboard of all ship and in VTS centres an automatic display of the current traffic situation. The quality of display depends on the quality of the delivered data. It should be required that sensors only should deliver valid data because an AIS station only transfers data without examine its plausibility.

- In VTS centres the manual workload associated with the corrections of radar data and ships data can be reduced.

- On the interface the explicit data delivered from the ship and implicit data derived from the system is available.

Use:

The BAS is used to make available the static and dynamic data of non SOLAS V vessels equipped with AIS.

No voyage data is required from Class B AIS stations.

No information is available about ship not carrying an AIS Station respectively the information can only be derived by verbal interrogation via VHF radio and after the data have to be processed manually. It is unknown up to now, how the data will be processed on higher application levels. Nothing at all there is the risk of faults because of manual input. Therefore the traffic situation display of various users may show different information.

In this case the verification of the information by verbal VHF radio interrogation, radar or by observation in very important.

Operation and Support:

After a maximum delay of 2 minutes the BAS is operational and operates until the AIS station is switched off.

Interaction with other BAS:

BAS9  no.7 (User Interface), BAS No. 8 (Interrogation via VDL), [BAS No.9 (Interrogation via DSC???)], [BAS No.16 (DTE-Flag)], [BAS No.22 (Interrogation specific message) ]  [require further work]

### SAR_DAT: Current SAR aircraft status

#### Service Overview

Purpose and General Description:

AIS units can be installed aboard aircraft and voluntarily used to provide information about the aircraft's status in a fashion similar to vessels.  This information is provided in a way that allows it to be easily identified and separated from vessel and other surface information. This facility is intended to support the coordination and communications between surface vessels and aircraft involved in a common search and rescue operation.  This service provides information that includes; aircraft name, call sign, location with altitude, speed, and course.  This capability should only be used for activities that require the immediate coordination of air and surface platforms.  It is not intended to be a general aviation tracking service.  [Note that "aircraft name" and "call sign" require the use of message 5.  This requires an IALA clarification for the use of message 5.]

Risks: The range of an airborne AIS unit is considerably greater than vessel units.  Limiting the power and use of airborne AIS units serves to reduce any impact this extended range might have on the normal use of AIS by surface platforms.

### ATON_DAT: Current AtoN AIS station data

#### Service Overview

Purpose and General Description:

An AIS transponder fitted to an Aid to Navigation will provide the same positive identification as a radar beacon. This service broadcasts specific aids to navigation information about type, name, dimension, position and off-position information. The AIS can broadcast the current status of the aid. It concerns both fixed and floating aids.

### BASE_DAT: Current base station status

#### Service Overview

Purpose and General Description:

The position, UTC time and date, and current status of the base station is transmitted automatically every 10 seconds. This rate can be increased to every 3.33 seconds when necessary for synchronisation.

### TRANS_IAI: Broadcast text telegram (IAI FI #0)

#### Service Overview

Purpose and General Description: This message can be used to broadcast information in readable format to all receivers in the area. This message is limited to maximum "chunks of" about 163 alphanumeric characters. This is used to transmit free text. The use of this feature can be regulated by the competent authority to avoid overloading of the system.

### TRANS_RAI: Not applicable at this time pending and IMO decision.

### DGNS_COR: Provide DGNSS-Corrections via AIS VDL

#### Service Overview

Purpose and General Description: A base station can transmit differential GNSS corrections if it is connected to a DGNSS reference station. This service can be broadcast automatically and is a back-up service for the DGNSS beacons.

### CH_TDMA: Switch AIS VDL frequencies via AIS VDL

#### Service Overview

[If there is a need to switch over one or both of the frequencies in use to another frequency(s), a service message is available to do this. When both channels must be switched, there will be a transition zone to avoid both channels will be switched at the same time.]

Purpose and General Description

This BAS provides GNSS correction factors. These correction factors are transmitted via base stations.  [BT: The German text is unclear as to which correction factor will actually be transmitted by the AIS base station.  An ideal sentence would be:

"A VTS centre selects the correction factors that will be transmitted by the AIS.  The VTS center in turn selects the correction factors to be transmitted according to the following rule…………..etc.]

AIS mobile stations can request this BAS.

This BAS is offered in order to provide vessels DGNSS correction factors under following conditions:

-when the ship's own receiver for DGNSS correction factors (beacon-receiver) has failed or

-when the ship does not have a receiver for DGNSS correction factor in the first place.

Use: This BAS will be employed in order to deliver to all users of a GNSS that is within reach of a specific base station improved position data and integrity data.

Risk: If there is no interface connection between the AIS device and the GNSS receiver then no correction factors can be transmitted.  In that case the position accuracy is only >10 m.

[BT: Don't agree with the last sentence.  The ship's own independent GPS system could well have differential corrections.  Consequently, it would feed differentially corrected position data to the AIS device.]

Operation and Support: When it is desired to transmit DGNSS correction factors via a base station, they can be sent automatically and continuously in the broadcast mode or in response to the polling mode.  The optimum rate of transmission in the broadcast mode has still to be determined.

Interaction with other Basic AIS Services

INT_TDMA

FATDMA

### CH_DSC: Switch AIS VDL frequencies via DSC Channel 70 (Tampa)

#### Service Overview

- Purpose and General Description:

In an area where the international designated AIS channels may not be available and the AIS is not under control of another AIS pair of frequencies, DSC channel 70 will be used to force the AIS unit to switch over to designated frequencies.

### PWR_LEV: Set transmitter power level of mobile AIS stations

#### Service Overview

Purpose and General Description: This BAS ensures that all AIS stations (mobile stations and base stations) within the same coverage area are synchronized.

Timing Process:

There are several synchronization states. These states depend on the availability of UTC.  UTC time can be obtained directly from the ship's own AIS device via the GNSS link or, via the VHF data channel,  from another AIS station (base station, mobile station) that can receive UTC direct.

If these two possibilities are not available, a mobile station can synchronize itself to another base station whose signals it can receive.  A prerequisite for this mode that the other bases station is transmitting the semaphore status i.e. it is receiving the largest number of other AIS stations.  When an AIS mobile station receives signals from several other base stations that indicate semaphore status then it selects the one with the lowest with the lowest MMSI number.

If all previous methods have been exhausted then synchronization will be based on that AIS mobile station that indicates that it receives the largest number of other AIS Stations (AIS mobile station which signals semaphore status).

For the purpose of synchronization a base station sends message 4 at least very 10 sec. For the time during which one or more AIS mobile stations synchronize themselves to that base station, message 4 is transmitted every 3 1/3 seconds.

(For further details on this process see the corresponding flow diagrams)

Messages used: Message 4, 11, 10

Integrity:

A base station transmits regularly, among other data, its synchronization state and UTC time via message 4.

In response to an interrogation via message 10 an AIS mobile station uses message 11 to communicate its synchronization state and UTC time.

In case the base station has no UTC time it can set a  semaphore status or substitute another time reference for UTC.

[BT: The last sentence doesn't sound right, needs to be verified].

Advantages of Semaphore: Cost effective i.e. no expansion of the base station is required.

Disadvantages of Semaphore: No absolute  time reference.  Slot numbers are unknown which can lead to interference problems when transmitting certain messages.

Advantages of Time Standard: Absolute time relationship remains intact.

Disadvantage of Time Standard: Additional investments are required e.g. for Loran-C receivers or Cesium clocks.

Onboard Ship Requirements: None, are contained in the AIS equipment.

Onboard Ship generation: Automatic

Requirements for VTS centres: No special requirements exist. The parameter "Synch State" must be derived from the Communication State.

Risk

Probabilities:

Fault Indications:

Affected Areas: (Technical operation of base station)

Affected Area: (Technical operation of AIS network)

Affected Area: (Technical operation of radar data processing)

Affected Area: (Technical operation of ships' data (static and voyage related only) processing.

Man Machine Interface (MMI) for the mariner is the VTS centre: (The synch state of own base station and, as required, that of the received AIS mobile station can be evaluated and indicated for possibly required consequences .

Data Format:

Precision/ Accuracy:

[BT: Here follow a number of detailed flow diagrams which have to be processed]

#### Service Overview

FATDMA: FATDMA Slot Management

1.39.2.1 Service Overview

Purpose and General Description

The purpose of this BAS is to allow making reservations for time slots for the base stations (for repetitive tasks).

It follows that time slots reserved by base stations will note be used by AIS mobile stations.

A base station requires time slots for the transmission of:

- Base station report to support synchronization processes

- Assignment of time slots to AIS mobile stations

- Assignment of repetition rate for specific AIS mobile stations

- Channel management messages

- DGNSS correction factors

- Safety related messages (non-repetitive)

- Application specific messages (non-repetitive)

Timing behavior

Reaction Time

- Time of generation: configuration

ITU-R M.1371 Message used: Message 20: Data Link Management Message

- 4 blocs of different size, separation distance, timeout

ITU-R M.1371 References: A2, para 3.3.4.3 FTDMA

Reference Document: M.1371, A2, para. 3.3.8.2.16 Message 20

Integrity, Verification, navigational reliability

Reliability is high due to system management function in case of automatic generation

Default value

………….etc.

Part III – Aspects relating to the AIS Service architecture

--- include here material describing the general architecture of the AIS Service
(from Part I and also from fresh sources)

# The Layered Structure of the AIS Service (General/ Functional components)

[this section was formerly in Part I; needs update]

## The functional layers for the AIS Service and the AIS shore infrastructure

The geographical coverage requirement introduces at least five hierarchical, functional layers with different tasks (from top to bottom):

- The layer LSS processes the data derived from the different physical AIS shore stations and provides the Basic AIS Services to the applications during run-time.

- the AIS shore station layer; an AIS shore station may house more than one fixed AIS station;

- the fixed AIS station layer (AIS repeater stations are also located on this layer);

- the layer of VHF-/RF-domain equipment for fixed AIS stations, which comprises antenna(s) and other pieces of on-site infrastructure;

- the AIS VHF Data Link (VDL), which can only be accessed by the VHF-/RF-domain equipment.

Each of these layers may consist of different entities, which in total must deliver the complete functionality of the layer. On the AIS base station layer, for example, the entities may be AIS base stations proper, AIS simplex repeater stations, and / or AIS duplex repeater stations.

In addition, one uppermost logical layer is required, which would be responsible of managing the whole of the AIS Service. This highest logical layer of the AIS Service is called AIS Service Management (ASM).

## Overview on the layered structure of the AIS Service

Figure 6.1 provides an overview on the layered structure of the AIS Service. This figure may be useful to understand the principle of the layered structure of the AIS Service. A Man-Machine-Interface (MMI) is needed to make data available to technical personnel operating and maintaining the AIS Service. In principle, a MMI would be needed for every layer (refer also to footnote below).

Figure 6.1: Layered structure of AIS Service

[there are slight updates necessary to this figure]

Technical Operation /

Technical Maintenance

MMI

Logical AIS Shore Station (LSS)

Status reports to higher levels

MMI

Physical AIS Shore Station (PSS)

MMI

Fixed AIS Stations Layer (Base stations, repeater stations)

MMI

HF components for the VHF maritime mobile Service

MMI

AIS VDL

(TDMA; optionally DSC)

The AIS Service

Co-loca-tion with other VHF services

MMI

Net Data Output From / Input to External Basic AIS Services

AIS Service
Management

Technical Operation / Technical Maintenance (in principle for each layer)

* Note: This symbolic representation does not imply any inference as to the amount of required personnel for the technical operation / technical maintenance of the AIS service. This symbolic representation aims at indicating that human interaction with the largely automated AIS Service is required as the last resort - and in some cases possibly on a regular basis - on all layers of the AIS Service.

Figure 6.2 also provides a representation of the layered structure of the AIS Service. This figure may be helpful to understand the relationship between the pluralities of the different layers.

Figure 6.2: Description of the AIS Service using the internationally standardized UML (Unified Modeling Language)

The UML provides a powerful, internationally standardized and widely distributed notation to convey logical structures and relationships between abstract and concrete objects, it can be used to describe the functionality of the AIS Service. Since it is novel to IALA Recommendations a legend and some explanation are given.

- The numbers denote the multiplicity of possible relationships between the different entities. For example, a logical AIS shore station must have at least one physical AIS shore station associated with it but will have as many as are needed (indicated by the asterisk *). Also, in most cases, a physical AIS station will be used by more than one logical AIS shore station, hence *. However, in very simple AIS Service layouts the logical AIS shore station may be omitted in the AIS Service (but needs to be provided by higher levels or applications instead), so that a physical AIS shore station would not be unrelated to any logical AIS shore station, hence '0'.

- The composition indications

indicates, that the higher level comprises, at least, the items given. For example, the physical AIS shore station  comprises (amongst other devices which may not be of relevance for the discussion at hand) at least one AIS base station. A black square indicates that the existence of the part of the whole is dependent on the whole for its existence or making sense.

Basic AIS Services

AIS Service
Management

owns

owns

1

1,*

owns

Logical

AIS Shore Station

Antenna

RF filter network

1

1

0,1 *

1,*

0 , 1, *

1,*

0... *

1,*

1,*

1,*

1,*

Notes:

* The VHF / RF domain equipment may be "owned" by a different service, such as VHF radio communications and is provided to the AIS Service on a co-location basis (while fulfilling the requirements of the AIS Service).

† The AIS VDL also exists without any AIS shore installation.

1

Physical

AIS Shore Station

Fixed AIS Station

VHF / RF domain

Equipment (*)

AIS VHF data link
(AIS VDL) (†)

assigned to

assigned to

## The AIS VHF Data Link

VHF data link (VDL) is understood as the medium for exchange of data between different AIS stations; by default, using ITU-assigned channels AIS1 and AIS2 in the VHF maritime mobile service band. The channels AIS1 and AIS2 are divided in time slots, 1 minute consists of 2250  slots per channel, giving in total 4500 slots.

In addition, DSC Channel 70 is used for channel management and DSC polling. DSC polling can result in the utilisation of other DSC channels in the VHF maritime mobile band.

## RF-/VHF domain Equipment

RF-/VHF domain equipment consists of the means to establish the VDL between the different AIS stations.  Antennas, cables and filters are components of the RF-/VHF-equipment.

## Fixed AIS Stations Layer

### AIS Base Station

The AIS base station is the most basic AIS-related entity of any AIS shore infrastructure. Conceptually, it is a "bare bones" black-box like device defined by the functional description and interface definitions given in the following chapters. It cannot operate on its own, since does not have the supporting infrastructure of a physical AIS shore station.

The AIS Service in a littoral or port state is operated by the national competent authority for the whole of the national coverage area. This national competent authority has the need to control the AIS Service and the AIS VHF data link, by employing means like channel management, protected slot allocations, and assigned mode. The AIS Base Station allows for the full control of the AIS VHF data link. However, there are other competent authorities responsible for local operations of e. g. individual harbours, locks, marinas, off-shore structures. These regularly have an interest in monitoring the vessel traffic in their locally confined area of competence and exchange limited operational information with ships in regard to their area of competence, e. g. docking scheduling, tugs operation.  For those purposes the concept of a Limited AIS Base Station was developed.”

### AIS Repeaters

The AIS essentially is a simplex system, and the AIS easily provides for a simplex repeater or simplex repeating process. Duplex repeaters may be built, but it may be difficult to fulfil physical layer and link layer requirements of Recommendation ITU-R M.1371-1 with respect to the TDMA AIS VDL.

The main application of simplex repeating is to extend the coverage, or to overcome local obstacles to radio propagation. The main application of duplex repeating is to extend the coverage.

Within the AIS Service, simplex and duplex AIS repeaters are located on the same layer as AIS base stations, i.e. they have direct access to the VHF/RF domain equipment for reception and transmission of VDL messages.

This document describes the simplex repeater station or the simplex repeating process proper, and the duplex repeater station proper. More detailed planning considerations and application notes for both simplex and duplex repeating are given in the IALA AIS Guidelines (Edition 1.1, Volume I, Part II).

## Physical AIS Shore Station (PSS)

The PSS is the most basic AIS-related entity, which can exist on its own in a real physical environment, as opposed to an AIS base station or AIS repeater station.

A PSS is physically fixed or is considered to be "fixed". A PSS may be theoretically mounted on a flying or floating platform, however. The latter cases are excluded from the scope of this document for simplicity's sake.

A PSS consists of at least the following components or functions:

- one AIS base station or one AIS repeater station;

- power supply;

- VHF-/RF-domain equipment, at minimum simply a cable and a VHF antenna;

- if the PSS houses an AIS base station a means to transport data to and from the AIS base station is required (an AIS repeater may operate without this data transport means);

- a means to protect the above component against environmental influence and damage, e.g. a shelter building or a housing case. Thus, a PSS does not necessarily need to be considered large physically.

In addition, a PSS will generally have a UTC source of its own (there may be cases during which the AIS base station may be set up using only the synchronisation provided by the AIS VDL itself, i.e. UTC indirect or even slot synchronisation). This UTC source may be internal to the AIS base station such as a GNSS receiver, or just internal to the PSS, such as an atomic clock or a Loran timing receiver.

In addition, there may be optional AIS-related functions added to the PSS' set-up, e.g. DGNSS correction source, AtoN station's functionality source, remote control equipment, or logging devices. Also, the PSS will, in most cases, comprise a control device, which, for instance, monitors the integrity of operation of some or all devices of that PSS, or which performs filtering functions for those AIS base and repeater stations housed by this PSS.

Co-location issues: From a conceptual point of view, one shelter or housing case may house more than just AIS-related services. Yet, the term "Physical AIS shore station" is used to indicate that the shore station has AIS capability regardless of any non-AIS capability it may have.

## Logical AIS Shore Station (LSS)

A LSS is a software process, which transforms the AIS data flow associated with one or more PSS into a different AIS-related data flow. Every individual transformation process takes into consideration

- operational aspects of the applications using the AIS service, and

- technical aspects which arise when operating a network of PSS.

Details are described in the appropriate Part below.

The software process of a logical AIS shore station can run on any appropriate computer at any appropriate place. It is required, however, that there are reliable data transportation means to and from all associated physical AIS shore stations and to the AIS Service, which interfaces with the applications.

## The AIS Service Management (ASM)

Since the AIS Service of a competent authority will, in most cases, comprise more than one LSS, PSS, and fixed AIS station, there is a need for a top layer, which acts as a controlling entity for the whole of the AIS Service. In particular, the ASM:

- "owns" all the logical and physical shore stations, i.e. it invokes, initialises, configures and terminates the logical and physical shore station software processes at run-time;

- determines the network communication relationships between physical shore stations and their associated logical shore stations for them to use during run-time;

- determines the communication relationships between the logical shore stations and the applications associated with them, i.e. this top level acts as a "switch-board" for the data exchange relationships between the different processes.

A more detailed description is given in Part V below.

Figure 6.3

Example on an AIS Service with clients and providers of AIS Data. This example describes an AIS Service, which includes Physical Shore Stations (PSS), Logical Shore Stations (LSS) and an AIS Service Management (ASM) entity. Several PSS are assigned to at least one LSS, which combines the data from these PSS in order to cover a certain area. In this example the AIS Service includes several LSS. Clients will receive data from the LSS distributing the data of interest to that certain client. The management of the AIS Service is done by the ASM. This entity controls the data flow in the system and configures the different entities in order to fulfil the given requirements.

# Interaction between the components of the AIS Service [Was: Fundamental technical prerequisites] (new Annex 3)

[this section was formerly in Part I; needs update]

From the above considerations the following fundamental technical prerequisites can be deduced, which need to be fulfilled by proper planning (see the appropriate chapters):

- The physical coverage of all physical AIS shore stations of one competent authority should always exceed the required coverage area of all logical AIS shore stations defined by that competent authority, where the same service level in terms of availability, reliability etc. applies.

- Each of the layers of the AIS Service requires a certain amount of processing capacity. The competent authority should provide the needed processing capacity to ensure proper operation of the AIS Service.

- The layered stack does not expressively state transportation of data over distances. Between each layer a transportation process is required; however, the distances and capacity requirements of which are totally dependent on local conditions. It is assumed, that the transportation processes do not constitute a bottleneck which determine the overall functionality. This can be achieved by standard modern technology even with high capacity needs.

- Between each layer there are functional interfaces, which can be defined precisely.

# AIS Data Transfer Network (new Annex 5)

[this section was formerly in Part I; needs update]

The AIS data transfer network is a fully transparent data transportation means used to deliver AIS-derived data. Since all AIS-related functionality has been encapsulated in either the PSS, or the LSS or at the AIS Service level, the AIS data transport network does not comprise any AIS-specific technology. Hence, the AIS data transfer network is a virtual network transporting AIS-related data within a possibly much larger network of the competent authority.

# Employing TCP / IP protocol(new Annex 5? If needed?)

[-- from former Part IV; needs update, if needed at all]

Entities of the AIS Service should - to the maximum extent possible - employ the TCP/IP protocol for AIS data transfer.

TCP/IP packets containing safety-related data, i.e. TCP/IP packets handled by the BAS SAFE_AD or SAFE_BR, should have the highest priority set in the packet header and should be prioritised for transmission.

# Additional fundamental design philosophy statements (General/Service model)

[this section was formerly in Part I; needs update]

The following fundamental design philosophy statement(s) should apply throughout the descriptions in this document:

- For maintenance purposes all fixed AIS stations should have modular construction. The functionality of fixed AIS stations should be able to be upgraded at minimal expense and minimal downtime for the fixed AIS station;

- All fixed AIS devices should have a status monitoring, and the status information should be output via an appropriate interface.

Part IV – Functionality of the Logical AIS Shore Station (LSS) layer

[this chapter needs update]

# Use Case of the Logical AIS Shore Station (LSS) (new Annex 9)

This chapter describes the use cases of the Layer of the Logical AIS Shore Stations recognised so far. The use case list constitutes the basis for appropriate description of the functionalities of the use cases.

The chapter is subdivided in accordance with the three different relationships a LSS can exhibit:

- use cases with regard to the Basic AIS Services

- use cases with regard to the mobile AIS stations which are taken care of by a LSS

- use cases with regard to the configuration of the Physical AIS Shore Stations assigned to a LSS

## Use Cases of the Logical AIS Shore Station with regard to Basic AIS Services

These use cases can be further subdivided into five (5) different categories in accordance with their principle working mechanism.

### BAS-related Use Cases Category I

These use cases share the following common characteristic: The data flow direction is from mobile to fixed. The use cases are implicitly triggered by AIS data input received from the Physical Shore Station layer, which then is processed and forwarded to higher levels. These use cases perform their function upon a trigger event and return to standby until the next trigger event, only.

Status

Configuration

Figure 52.1

Data to client(s)

LSS

data input =

implicit trigger event

Status

Configuration

Use Cases of this category:

The generation of the following BAS constitutes the use cases of this category:

A_STAT  (without any application-specific message processing)

A_DYN

A_VOY 	(without any application-specific message processing)

B_DAT

SAR_DAT

ATON_DAT

SAFE_AD   	(information flow direction mobile-to-fixed)

SAFE_BR   	(information flow direction mobile-to-fixed)

TRANS_IAI 	(information flow direction mobile-to-fixed)

TRANS_RAI 	(information flow direction mobile-to-fixed)

These use cases may also have a diagnostic functionality for individual mobile AIS stations.

### BAS-related Use Cases Category II

These use cases share the following common characteristic: The data flow direction is from fixed to mobile. The use cases are implicitly triggered by AIS data input received from higher levels, which then is processed and forwarded to lower levels, i.e. the Physical Shore Station layer. These use perform their function upon a trigger event and return to standby until the next trigger event, only.

Status

Configuration

Figure 52.2

Data input from client(s) =

implicit trigger event

LSS

Status

Configuration

processed data

Use Cases of this category:

The generation of the following BAS constitutes the use cases of this category:

SAFE_AD   		(information flow direction fixed-to-mobile)

SAFE_BR   		(information flow direction fixed-to-mobile)

TRANS_IAI  		(information flow direction fixed-to-mobile)

TRANS_RAI  	(information flow direction fixed-to-mobile)

DGNS_COR

### BAS-related Use Cases Category III

These use cases share the following common characteristic: These use cases are triggered expressively by a trigger information input received from higher levels (trigger event). This triggers a use case, which assembles a request for certain data, which then is processed and forwarded to lower levels, i.e. the Physical Shore Station layer. Upon receipt of the answer to the data request, the data is processed to be forwarded to higher levels (to the originator of the trigger event). These use cases perform their function upon a trigger event and return to standby until the next trigger event, only. The total information flow direction is fixed-mobile-fixed.

Figure 52.3

Status

Configuration

LSS

Requested Data to Client(s)

Trigger

Information

Status

Configuration

Use Cases of this category:

The generation of the following BAS constitutes the use cases of this category:

INT_TDMA   	(information flow direction fixed-mobile-fixed)

INT_DSC  	(information flow direction fixed-mobile-fixed)

### BAS-related Use Cases Category IV

These use cases share the following common characteristic: The use cases are triggered expressively by a trigger information input received from the Physical AIS Shore Station layer. This constitutes the trigger event. It triggers a use case, which assembles the reply for certain data, which then is processed and forwarded to lower levels, i.e. the Physical Shore Station layer, to be forwarded to the originator of the trigger event. An indication on the request as such may be given to higher levels. These use cases perform their function upon a trigger event and return to standby until the next trigger event, only. The total information flow direction is mobile-fixed-mobile.

Figure 52.4

Status

Configuration

## LSS

Requested

data

Indication

Status

Configuration

Use Cases of this category:

The generation of the following BAS constitutes the use case of this category:

INT_TDMA   (information flow direction mobile-fixed-mobile)

Note: The BAS INT_DSC is allowed only for the information flow direction fixed-mobile-fixed.

### BAS-related Use Cases Category V

These use cases share the following common characteristic: These use cases are triggered expressively by a trigger information input received from higher levels (trigger event). This triggers a use case, which, based on the configuration input from higher levels, generates and forwards certain AIS VDL management commands to the PSS layer. Upon receipt of the (implicit) answers to these management commands, they will be evaluated by the use case, and indications may be forwarded to higher levels (to the originator of the trigger information).

These use cases by default exhibit a continuously and autonomously behaviour, but one-shot behaviour may also be achieved by appropriate trigger information input. The total information flow direction is fixed-mobile-fixed.

Figure 52.5

Status

Configuration

## LSS

Indication to

Trigger Source

Trigger Information

Status

Configuration

Use Cases of this category:

The generation of the following BAS constitute the use cases of this category:

BASE_DAT

ASGN_RATE

ASGN_SLOT

CH_TDMA

CH_DSC

PWR_LEV

FATDMA

## Other Use Cases with regard to BAS

In addition, there are the following use cases of LSS layer with regard to BAS:

- Format transformation of VDL Message format to BAS format (and vice versa)

- Fault detection and management of with regard to the BAS

- Provide MMI for technical operation / maintenance of LSS layer

## Use Cases with regard to the mobile AIS stations which are taken care of by a LSS

While the above use cases are defined in regard to an BAS, the following use cases are defined in regard to regularly more than one BAS for one or more individual mobile AIS stations. The following Use Cases with regard to the mobile AIS have been identified:

- Management of the type-of-mobile-filter functionality of the LSS layer: This use case defines the mix of relevant type-of-mobiles for a given LSS. Types of mobiles could be Class A Shipborne Mobile AIS stations, Class B Shipborne AIS stations, SAR Airborne AIS Stations, and AtoN AIS Stations. 
Higher levels may configure "their" LSS such, that this LSS forwards only data of relevant types-of-mobiles to them. 
Example: An application (higher level) may be interested only in data from Class A and Class B shipborne mobile AIS stations. This application would then configure "its" LSS such, that only Class A and Class B data would be forwarded, but not data from AtoN or SAR Airborne AIS stations. The latter would have been received however, and would possibly be used by different LSS and their associated applications.

- Management of the type-of-ship-filter functionality of the LSS layer: This use case defines the mix of relevant type-of-ships for a given LSS. Types of ship could be determined by evaluating the appropriate data field in Class A and Class B Shipborne Mobile AIS stations reports. 
Higher levels may configure "their" LSS such, that this LSS forwards only data of relevant types-of-ships to them. 
Example: An application (higher level) may be interested only in data from ships identifying themselves as "passenger ships". This application would then configure "its" LSS such, that only data from "passenger ships" would be forwarded, but not data from any other type of ship. The latter would have been received however, and would possibly be used by different LSS and their associated applications.

- Management of the type-of-AtoN-filter functionality of the LSS layer: This use case defines the mix of relevant type-of-AtoN for a given LSS. Types of AtoN could be determined by evaluating the appropriate data field in AtoN AIS station reports. 
Higher levels may configure "their" LSS such, that this LSS forwards only data of relevant types-of-AtoN to them.

- Management of the assignments of responsibilities of a PSS and/or fixed AIS station for a given mobile AIS station:  In particular, but not exclusively, for any information flow direction fixed-to-mobile, it is necessary to determine, which PSS and/or even fixed AIS station would be "responsible" for transmitting data from any BAS to that mobile AIS station. Since mobiles regularly move through the coverage areas of many fixed AIS stations and / or PSS, the assignment of responsibilities must be dynamic. The assignment would be based on the mobile's VDL-related identity.

- Geographical Filtering for information flow direction fixed-to-mobile: Which PSS would be responsible for outgoing data?

- Geographical Filtering for information flow direction mobile-to-fixed: Which LSS would be responsible for data received by a fixed AIS station / PSS?

- Remove duplicate data for information flow direction mobile-to-fixed

- Reduce data rate forwarded by this LSS to higher levels for information flow direction mobile-to-fixed: Upon configuration from higher levels, this use case would filter the data rate forwarded from a given LSS to its associated application. This use case would be employed when an application would have no need of data from mobiles at the rate the data is transmitted by that mobile on the AIS VDL. As a benefit, the reliability of the data flow regularly increases by employing this use case. Note: There is a similar use case defined at the PSS layer, for a situation, when a fixed AIS station / PSS would be sitting at a location with insufficient AIS data transfer network capacity to the associated LSS.

- Fault detection and management of with regard to above use cases

- Provide MMI for technical operation / maintenance of above use cases

## Use Cases with regard to the configuration of the PSS assigned to a LSS

The following represent a class of use cases, completely internal to the shore-based AIS Service. They are relevant for the management of the assignment of LSS and PSS.

- Management of the assignments of regularly more than one PSS to a given LSS

- Configuration management of individual PSS assigned to a given LSS within the margin of configuration rights given to that LSS by the ASM

- Fault detection and management of with regard to above use cases

- Provide MMI for technical operation / maintenance of above use cases

# Part V – The Physical AIS Shore Station (PSS) Layer (new Annex 10)

# General

## Co-location of AIS-base and AIS-repeater station(s) with other maritime mobile services in the same shore station

# Functional definition of an AIS shore station

## Diagnostics and remedial capabilities for one or more base station(s) and /or repeaters in one shore station

## Functionality located in AIS shore station that affects the operation of the AIS Base station

# Multi-base AIS shore stations

# Pre-Processing and Interfacing to the AIS Network (new Annex 5)

# Technical Guidance for Installation of an AIS shore station (new Annex 12)

# Interaction between AIS-related services and other maritime mobile services operating in the same shore station (new Annex 12)

Part VI - Functionality of the AIS Service Management

# Use Cases of the AIS Service Management (ASM) (new Annex 1 and 11)

## Use Cases of the ASM with regard to the management of the BAS assignment to (individual) LSS

The common characteristics of these use cases are such, that they manage the responsibilities of any and all LSS in particular with regard to the internal BAS.

In some cases, the internal BAS control critical features of the VDL. In this case, the ASM must ensure, that responsibilities assigned to different LSS do not interfere with each other. This would mean in some cases, that only one LSS should be given the right to employ certain internal BAS.

The following use cases in this category have been identified:

- Management of the assignment of internal BAS to LSS

- LSS selection based on geographical and possibly other, configurable criteria for incoming data from higher levels for information flow direction fixed-to-mobile

- Frequency management for the whole of the AIS Service: This use cases controls the use of the internal BAS CH_TDMA, CH_DSC, PWR_LEV by LSS layer.

- Management of the AIS Service-wide slot reservation scheme using internal BAS: FATDMA

- Management of the AIS Service-wide assignment functionalities of mobiles using internal BAS: ASGN_RATE, ASGN_SLOT

- Management of the AIS Service-wide distribution of DGNSS corrections: DGNSS corrections are provided by some radio navigation service, e.g. IALA beacon reference stations, outside the AIS Service proper. This use case manages the distribution of the correction data input to the appropriate LSS. It is using the internal BAS DGNS_COR

- Management of identity of the LSS, PSS and / or fixed AIS station: Instead of their own pre-configured MMSI fixed AIS stations may use different MMSI, which may be provided by applications on higher layers and / or by a LSS and / or by a PSS. This use case manages the use of different MMSI’s within the AIS Service

- Diagnostics toolbox for individual AIS mobile stations

- Diagnostics and failure management of entities within the AIS Service

- Fault detection and management of with regard to above use cases

- Provide MMI for technical operation / maintenance of above use cases

## Use Cases with regard to the configuration of any or all individual LSS except BAS

The common characteristics of these use cases are, that they manage the existence and configuration of any or all individual LSS.  Following use cases have been identified:

- Creation of an individual LSS

- Termination of an individual LSS

- Configuration management of an individual LSS with regard to its configurable parameters except BAS-related parameters

- Provide MMI for technical operation / maintenance of all LSS

- Fault detection and fault management for LSS functionality except for BAS-related functionality

- Tools for planned maintenance at LSS level, such as "Garbage Collection"

## Management of the assignments within the AIS Service

The common characteristics of these use cases are, that they all manage the assignments between individual functional entities within the AIS Service:

- Overall management of the assignments of LSS to the / more than one AIS Service

- Overall management of the assignments of PSS to LSS

- Overall management of the assignments of  fixed AIS stations to PSS

- (Supplementary) management functionality of the virtual AIS data transport network within the overall network of the competent authority

- Perform basic configuration of individual PSS upon their initialisation

- Perform basic configuration of individual fixed AIS stations upon their initialisation

- Fault detection and fault management for assignments within the AIS Service

- Provide MMI for technical operation / maintenance of above use cases

## Initialisation and Termination of the AIS Service

The following use cases have been identified:

- Initialisation of the AIS Service as a whole, including set-up of the minimum number of LSS for internal functions of the AIS Service

- Termination of the AIS Service as a whole.

Following Parts – Aspects of the Technical Operation of an AIS Service

[extended updates needed]

Part VII - Guidelines for FATDMA Configuration Set-up of AIS shore stations (new Annex 14)

# Introduction

## Overview

This Annex introduces the configuration of FATDMA of Physical AIS Shore Stations (PSS) by one or more competent authorities in a given area. The rule base for the FATDMA Configuration is Recommendation ITU-R M.1371-1 (and appropriate IALA Technical Clarifications). The FATDMA Configuration also takes into account the standardization done at IEC for AIS base stations (IEC Standard 62320-1 – expected publication 2005), Aids-to-Navigation AIS Stations (IEC Standard 62320-2- under development) Limited Base station (IEC Standard 62320-3- to be developed), Class A mobile AIS stations (IEC Standard 61993-2:2002), and Class B (CSTDMA) mobile AIS stations (IEC Standard 62287 – expected publication 2005). In addition, the IALA-Recommendation A-126 on AIS for Aids-to-Navigation is taken into account. Finally, the layered concept of the AIS Service, as described in previous chapters of this recommendation is used.

## Goal of Annex F

The goal of this Annex can be summarized as followed:

- Provide to competent authorities a comprehensive „cookbook-like“ recipe to create a FATDMA plan

- Provide to adjacent and affected national competent authorities guidance to co-ordinate their joint FATDMA plan in the adjacent area

- Provide to (national / other domestic) competent authorities guidance how to operate their co-ordinated FATDMA plan correctly and optimize the benefit of both mobile AIS stations and the competent authorities in a given area.

- Provide a basis for required and / or stipulated type(s) of components of a competent authority

## Section overview

When setting up a FATDMA plan, there are three dominating aspects which need to be introduced:

- The required usage of FATDMA reserved slots, answering the questions: What would the FATDMA reserved slots used for by AIS stations?

- What geographical criteria apply to co-ordination of FATDMA schedules and schemes when using FATDMA reserved slots in a (larger) given area of a FATDMA plan?

- What rules are applicable for allocating FATDMA reserved slots to national competent authorities and other (domestic) competent authorities? In particular, what rule base governs the allocation of FATDMA schemes internationally?

These three aspects will be dealt with in the above order in the following sections after the introduction of important definitions.

# Introduction to relevant aspects of the FATDMA reservations based on Recommendation ITU-R M.1371-1

The following rules for FATDMA reservations are taken from Recommendation ITU-R M.1371-1 (and IALA Technical Clarifications).

A FATDMA reservation is made by a base station transmitting Data Link Management Message, msg 20.

A FATDMA reservation consists of

- a start slot, the slot where the FATDMA reservation starts in the frame,

- the block size, the number of consecutive reserved slots,

- an increment, the number of slots between the start of each reserved block and

- a time out value, how long the FATDMA reservation is valid.

The term FATDMA block denotes the effect of a FATDMA reservation on the AIS VDL.

Each Data Link Management Message (msg 20) can reserve up to four (4) independent and different FATDMA reservations.

The blocks of FATDMA reservations should be evenly distributed across the frame. The values recommended for "increment" are as follows (refer to Technical Clarifications §2.48.1; msg 20): 2, 3, 5, 6, 9, 10, 15, 18, 25, 30, 45, 50, 75, 90, 125, 225, 250, 375, 450, 750, 1125. Use of one of these values guarantees symmetric slots reservations throughout each frame.

Using the value 0 for "increment" means, that only one reservation is made in the frame.

The lower the "increment" value, the more slots will be reserved.

In normal operation the base station should use FATDMA for its own transmissions. For this, slots have to be reserved by a base station using Data Link Management Message (msg 20). If there are no reserved FATDMA slots available the base station will transmit using RATDMA.

The FATDMA reservation is timed out after 3 minutes (default) in the mobile AIS stations. The time out value for the FATDMA reservation within the mobile AIS station can be configured between 1 and 8 minutes by the Data Link Management Message (msg 20). The FATDMA reservation should be repeated before a time out occurs, if the FATDMA reservation is to be maintained.

The Data Link Management Message applies only to the AIS channel on which it is transmitted.

The effects of an FATDMA reservation on other AIS stations is as follows:

- Mobile stations will not include any FATDMA reserved slots in their candidate slot map at all, if the reserving base station is within 120 nautical miles. If the position of the reserving base station, as derived from message 4, is more than 120 nautical miles away from the mobile, Class A mobile stations should ignore the reservation.

- Other base stations will not include any FATDMA reserved slots in their candidate slot map when using RATDMA. However, at the discretion of the competent authority, a base station may intentionally FATDMA reserve and use the same time slots within 120 nautical miles, if no interference is likely.

# Definition of relevant terms for FATDMA reservations

This section defines relevant terminology used for FATDMA Configuration.

## Fundamental considerations for terminology

In order to make efficient usage of the available AIS VDL time slots in a given area a “proper plan” is needed. This “proper plan” may be called FATDMA Area Slot Map. It has two aspects:

- Setting up the FATMDA Area Slot Map (at planning time)

- Using the FATDMA Area Slot Map (at run-time).

These aspects and the composition of the FATMDA Area Slot Map are illustrated in the following figure. An explanation of the various aspects will be given afterwards (the figure employs the UML notation).

The aspects which should be considered when planning FATDMA reservations on the AIS VDL in a given area, are described below:

- Geographical area aspect: Every FATDMA reservation is done within a defined area.

- Time domain aspect: The actual FATDMA reservations are done using the time domain. The time domain in the AIS is subdivided into time slots. Every FATDMA reservation consists of time slots reserved for a specific use. These FATDMA reservations can be described in terms using absolute time slot terminology and relative time slot terminology as follows:

- absolute slot numbers of FATDMA reservations: At planning time, the FATDMA reservations can only be planned using absolute slot numbers within the slot frame epoch. This requires an absolute reference point, which is the start of a specific frame epoch which in turn is related to the start of an UTC minute or hour. Since UTC is a universal concept, any absolute point in time can be determined thereby.

- Terms related to the relative slot numbers used for actual transmissions of FATDMA reservations: At run-time, the FATDMA reservation is being done be transmitting the Data Link Management Command (msg 20). The parameters given by this command determine the number and location of the slots reserved by mobile stations. The parameters given determine the absolute position of the FATDMA allocated slots by reference to the absolute slot number of that slot, in which the Data Link Management Command was received. Therefore, these terms are relative by nature.

- Usage domain aspects: The usage domain is subdivided into two aspects:

- At planning time: Every FATDMA reservation is used for a purpose. Therefore, the usage aspect of FATDMA reservations must be considered. Each and every FATDMA reserved slot can and must have a designated usage allocated to it; otherwise its reservation would not be justified. This concept is called “Usage Designation”. It is further subdivided into Usage Categories.

- At run-time: The different potential users of FATDMA reservations (“VDL usage stake holders”) are identified as – by default and in accordance with the rules of the AIS VDL – all defined varieties of AIS stations, mobile or fixed. Their usage requirements as well as the overall requirement to maintain the viability of the AIS VDL need to be reflected in the Usage Designations at planning time.

- Roles of participating entities:

- At planning time the national competent authorities of individual countries sets up their FATDMA plan in accordance with the rules in this Annex. Should there be more than one national competent authority adjacent to the same geographical area, co-ordination would be done to guarantee the inter-operability of the individual FATDMA plans and create one joint FATDMA plan.

- At run-time every FATDMA reservation is done by one entity, called base station. Every time slot within this FATDMA reservation can be used by a variety of different entities participating in the AIS VDL, i. e. the AIS base stations proper, the AIS simplex repeaters and the different classes of AIS stations other than AIS base station.

In the following sections, the precise terms will be introduced.

## Terms related to FATDMA reservations in the time domain in general

The following terms are related to FATDMA reservations in the time domain in general, i.e. those terms are part of both the relative as well as the absolute FATMA terminology sets:

- FATDMA block: A FATDMA block consists of consecutive slots.

- FATDMA block size: The FATDMA block size is the number of consecutive slots of a block. The FATDMA block size can be between 1 and 5.

- FATDMA reservation: A FATDMA reservation is an announcement of a base station to create one or more FATDMA block(s) using the FATDMA reservation parameters. The FATDMA reservation is both an activity of a base station as well as the stationary knowledge of receiving mobile AIS stations, namely that the slots contained within the blocks have been reserved by one base station.

- FATDMA reservation parameters: Each FATDMA reservation is described by the following parameters (see figure). The FATDMA offset number is a relative term exclusively and will be explained in the next section.

Figure ....: 
Composition of FATDMA reservation

- FATDMA reservation time out: This value indicates, how many minutes the FATDMA reservation should be considered valid by the mobile AIS stations. The number of minutes results by adding one (1) to the values 0 to 7 in the FATDMA reservation(s) in msg 20. Therefore, a FATMDA reservation can be made with a FATDMA reservation time out between 1 and 8 minutes.

- FATDMA increment: The FATDMA increment is the offset of slots between the first slot of the first FATDMA block and the first slot of the next FATDMA block of the same FATDMA reservation in the same frame. The FATDMA increment is then used repeatedly to create further FATDMA blocks in the same frame until the end of this frame is reached and no further FATDMA block could be accommodated. If the FATDMA increment is set to zero (0), then there will just one FATDMA block reserved by this FATDMA reservation.

- Data Link Management message (msg 20): Actual FATDMA reservations are being made by transmitting the Data Link Management message (msg 20) from a base station to mobile AIS stations.

- FATDMA reservation No N: Each Data Link Management message can include up to four (4) FATDMA reservations. Within the Data Link Management message, the FATDMA reservations are numbered from 1 to 4. The following figure illustrates, that all those four FATDMA reservations in one Data Link Management message inherit the above qualities of a FATDMA reservation, but may be different in terms of actual parameter setting.

Figure ....: 
Inheritance relationship between FATDMA reservation definition in general and individual FATDMA reservations

- Msg-20-slot: The Msg-20-slot is the slot, in which the Data Link Management Message (msg 20) was transmitted. It has an absolute slot number. Starting with the Msg-20-slot number the FATDMA offset number is counted (see below; relative terminology).

### Terms exclusively related to the relative slot number used for actual transmissions of FATDMA reservations

The following terms are exclusively related to the relative slot number used for actual transmission of FATDMA reservations.

- FATDMA offset number: Every FATDMA reservation has a parameter FATDMA offset number. The FATDMA offset number denotes the offset from the slot in which Data Link Management Message (msg 20) was received to the first slot of the first FATDMA block to be reserved by the receiving station.

The relative composition of one (1) FATDMA reservation is illustrated by the following example figure.  In this example the parameters are as follows:

- Msg 20 is transmitted in the slot with an absolute number of n (numbered 1 to n relative to frame start. Hence the Msg-20-slot has the absolute slot number n. Note: The absolute number of the start slot of  the first FATDMA block can only be determined, if the absolute value of the Msg-20-slot is known (which is n+25 in this example).

- FATDMA offset number :			25

- FATDMA block size:		 	5

- FATDMA reservation time-out:	 	3

- FATDMA increment:			375

The following figure illustrates this example.

Figure ....:

Example for one FATDMA reservation within a Data Link Management message transmitted in Msg-20-slot n

FATDMA Offset

Block size

FATDMA Increment

n   n+1                                            n+ 25               n+29                          n+400              n+404                           n+775         n+779

free slot

reserved slot

Additional FATDMA blocks are reserved in this example, each of which has a FATDMA block size of 5, starting at absolut slot numbers: n+1150, n+1525, n+1900 (if end of frame is not reached by then).

### Terms exclusively related to the absolute slot numbers of FATDMA reservations

Certain FATDMA related terms refer to an absolute framework of time and / or slot numbers. In addition, there are FATDMA related terms defined for areas, which may also be considered absolute (see next section).

- FATDMA start slot: The FATDMA start slot is the slot number of the first slot of the first FATDMA block within a frame (refer to Table 8 of Recommendation ITU-R M.1371-1). Therefore, the FATDMA start slot is an absolute term.

- FATDMA epoch: Due to the time out behaviour of FATDMA reservations and due to the need to devise FATDMA plans which minimise the actual number of FATDMA reservations (see next sections), a time span of more than a frame (i. e. one UTC-based minute) needs to be considered. These larger time spans are called FATDMA epochs. Each FATDMA epoch is 6 minutes long. The start of a FATDMA epoch is correlated with the beginning of the UTC hour as indicated by the FATDMA epoch number. Any FATDMA epoch starts with the first second of the first minute correlated with the UTC hour as given by the FATDMA epoch number and ends with the end of second 59 of the 6th minute after the start minute of the epoch.

- FATDMA epoch number: Each FATDMA epoch has assigned a unique number within the duration of one (1) hour. The FATDMA epoch number of each and every of the FATDMA epoch of one hour is correlated to an absolute start minute within the UTC hour as given in the table below:

| FATDMA epoch number x | Start minute of FATDMA epoch Number x | End minute of FATDMA epoch Number x |
| --- | --- | --- |
| 0 | 0 | 5 (included in epoch 0) |
| 1 | 6 | 11 (included in epoch 1) |
| 2 | 12 | 17 (included in epoch 2) |
| 3 | 18 | 23 (included in epoch 3) |
| 4 | 24 | 29 (included in epoch 4) |
| 5 | 30 | 35 (included in epoch 5) |
| 6 | 36 | 41 (included in epoch 6) |
| 7 | 42 | 47 (included in epoch 7) |
| 8 | 48 | 53 (included in epoch 8) |
| 9 | 54 | 59 (included in epoch 9) |

### Relationship between absolute and relative FATDMA terms

The following relationships exist between the above absolute and relative FATDMA related terms. Some FATDMA related terms are used identically, in an absolute as well as a relative context. In those cases, the term appears twice in the same line of the table. If there are different terms used in the same line of the table, the two terms correspond to each other, and a transformation / correlation is needed or performed at least at run-time. The following table gives the appropriate correspondence.

Table: Correspondence between absolute and relative FATDMA terminology.

| Absolute terms used for FATDMA reservations | Relative terms used for FATDMA reservations |
| --- | --- |
| FATDMA plan | - no equivalent - |
| FATDMA schedule | - no equivalent - |
| FATDMA scheme | - no equivalent - |
| FATDMA epoche | - no equivalent - |
| FATDMA reservation | FATDMA reservation |
| FATDMA block | FATDMA block |
| FATDMA start slot | FATDMA offset number |
| FATDMA increment | FATDMA increment |
| FATDMA block size | FATDMA block size |
| Msg20-slot | Msg20-Slot |
| - no equivalent - | Msg20 |

## Terms related to the geographical areas for which FATDMA reservation planning is done

The terms related to the geographical areas for which FATDMA reservations are planned and performed at run-time are arranged in order to assist in the structured mutual FATDMA reservation planning of neighbouring administrations:

- FATDMA plan: A FATDMA plan is the highest level of FATDMA reservation planning. A FATDMA plan comprises all FATDMA schedules (see below), which are considered relevant to the FATDMA plan. Therefore, a FATDMA plan also comprises the area covered by the FATDMA schedules relevant to the FATDMA plan. It is the purpose of this Annex to describe a global FATDMA plan, which translates seamlessly to FATDMA schedules (regional or national) and FATDMA schemes (see below).

- FATDMA schedule: A FATDMA schedule comprises the relevant FATDMA schemes of one (1) or more Physical AIS Shore stations (PSS) in the area under consideration. The FATDMA schemes combined in a FATDMA schedule may be of the same or different competent authorities. Any FATDMA schedule needs to be consistent at its interfaces to adjacent FATDMA schedules.

- FATDMA scheme: A FATDMA scheme comprises the relevant FATDMA reservations of one (1) PSS. Hence, a FATDMA scheme may comprise the FATDMA reservations of one or more transmitting AIS base station(s) of that PSS.

- FATDMA Configuration: This general term denotes all aspects of FATDMA planning and FATDMA reservations made at run-time to be done by one or more competent authorities in a given area. It is an umbrella term.

The following schematic drawing illustrates the relationship between the various terms related to areas (note: each transmitting base station (BS) transmits Data Link Management Message (msg 20) to set up FATDMA reservation(s).

Figure ....: 
Hierarchy of FATDMA plan, FATDMA schedule and FATDMA scheme

FATDMA plan

BS

BS

BS

BS

BS

BS

BS

BS

BS

BS

BS

BS

BS

FATDMA

Scheme

FATDMA

Scheme

FATDMA

Scheme

FATDMA

Scheme

FATDMA

Scheme

FATDMA Schedule

FATDMA Schedule

## Terms related to the usage of FATDMA reserved slots (“Usage Designation”)

This section introduces the different terms for usage of FATDMA reserved slots.

### General considerations in regard to periodicity of transmissions using FATDMA reserved slots

When setting up a FATDMA plan, the required periodicity of a transmission needs to be considered. In regard to periodicity the reserved FATDMA slots can be used for:

- Short term repetitive pattern spanning a number of time slots within the same frame

- Intermediate term repetitive pattern spanning one frame

- Long term repetitive pattern spanning one (1) FATDMA epoch

There may be also one-shot transmissions, which are called spurious transmissions.

### General considerations in regard to timing of transmissions using FATDMA reserved slots

The timing of transmissions using FATDMA reserved slots is largely independent of the periodicity aspect introduced above, and needs to be considered in following ways:

- Urgency of transmission of a message(s): Regardless of their periodicity some AIS messages need to be transmitted with a very short delay. Besides the requirement to provide for prioritised queuing on the PSS Controlling Unit level, the urgency of transmission using FATDMA reserved slots requires the FATDMA plan to provide sufficient closely spaced, general-purpose FATDMA reserved time slots to allow for urgent transmissions.

- Timely availability of FATDMA reserved slots when allowing for dynamic features of FATDMA plans: When introducing dynamic features of a FATDMA plan, there is a  requirement to provide additional FATDMA reserved slots before the actual net data transmission can be made in these FATDMA reserved slots. Additional FATDMA reservations are to considered “timely”, in regard to the intended net data transmission, only if they allow for the following transient processes to be completed:

- Transmission of additional FATDMA reservation(s) by AIS base station(s) to be completed = initiation of dynamic FATDMA reservation

- Allow AIS mobile stations to clear the slots previously allocated by their own transmissions on both AIS channels completely. This process can not be fastened and – for un-garbled operation – will require 8 minutes from transmission of the additional FATDMA reservation.

This consideration substantiates the introductory general rule, that dynamic FATDMA reservations should be avoided or should be minimised.

### Terminology for categories for usage of FATDMA reserved slots

All of the above considerations are simplified when categorising the usages of FATDMA reserved slots into the following categories.

- FATDMA reservation usage category 1: AIS VDL management slots

These FATDMA reserved slots will be used for the transmission of any message, which is necessary for the management of the AIS VDL.

- FATDMA reservation usage category 2: High timing requirements slots

These FATDMA reserved slots will be used for the transmission of messages, which need to be transmitted complying to high timing requirements.

- FATDMA reservation usage category 3: General purpose pre-reserved slots

These FATDMA reserved slots will be used for the transmission of any message, which does not fall under FATDMA reservation usage category 1 or 2.

## Terms related to activities of entities involved in the FATDMA (“VDL Usage Stake Holders”)

### Terms related to roles and activities of entities related to FATDMA

The following terms are related to activities of entities involved in setting up and executing a FATDMA plan.

- FATDMA reservation: The FATDMA reservation, as viewed as an announcement, is an activity of a transmitting AIS base station.

### Roles and activities involved in making and using FATDMA reservations

A FATDMA reservation can be made by one entity participating in the AIS VDL and it will be used by one or more entities participating in the AIS VDL.

A FATDMA reservation can only be made by a base station of a competent authority.

- FATDMA reservation for transmissions of its own data and when acting as a semaphore;

- FATDMA reservation for transmissions of other base station/Limited Base Station (LBS);

- FATDMA reservation for transmissions of simplex repeater station(s)

- FATDMA reservation for transmissions for mobile AIS stations

An existing FATDMA reservation can therefore be used by one of the following entities participating in the AIS VDL:

- own AIS base station

- other AIS base station(s) (of own or other competent authority)

- other Limited AIS base station(s) (of own or other competent authority)

- other simplex repeater station(s) (of own or other competent authority)

- mobile AIS stations operating either in

- assigned mode, hard assignment (Class A, Class B, AtoN AIS stations) or

- responding to an interrogation (assigned by an AIS base station)

- fixed-slot usage AtoN mobile AIS stations

The following figure provides an overview of the different roles in regard to the FATDMA.

Figure ....: 
Activity of AIS base stations and AIS mobile stations for FATDMA reservation

## Mapping of FATDMA related aspects to Layers of the AIS Service

The terms defined in this section can be associated to the different hierarchical layers of the AIS Service. This association indicates on what hierarchical layer of the AIS Service certain correlations between the "absolute" and "relative" realms need to be performed.

In accordance with the definition of the AIS base station in IEC 62320-1 there are two principal modes of operation for base stations:

- the independent mode: the base station in independent mode is independently, i. e. autonomously, performing tasks delegated to it from the PSS layer, such as autonomously creation and transmission of msg 20 upon configuration from the PSS layer. To that end certain configuration PI sentences are used by the PSS. The configuration comprises the generally pre-programmed transmission timing, which is also given in absolute terms. If the independent mode of base stations is employed by a competent authority the PSS needs to set up and supervise the proper operation of all of its base station(s) in independent mode.

- the dependent mode: the base station in dependent mode transmits only pre-composed messages of whatever kind, which it receives from the PSS at a individually pre-determined time slot, the number of which is given in absolute terms. The base station does not compose messages itself when operating in dependent mode. Certain PI sentences are used by the PSS to forward the messages to be transmitted as well as the individually pre-determined time slot for transmission.

In both cases the task of translating from the absolute FATDMA-related terminology used throughout the AIS Service and the relative FATDMA-related terminology used on the AIS VDL is a task of  the PSS, which may be delegated completely to the AIS base station when operating in independent mode. When operating in dependent mode, the AIS base station still translates the transmission timing from the pre-determined absolute slot number it receives to the (relative) point in time, when the pre-determined transmission slot actually occurs.

Table: Association of terms related to FATDMA and Layers of the AIS and the AIS Service

| Term related to FATDMA | AIS VDL | AIS base station proper | PSS Controlling Unit ("PSS") | AIS Service Management | AIS Service as a whole | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| FATDMA Reservation (relative point of view) | X | X | - | - | - | Reservation announcement by base station on the AIS VDL as such; therefore only known to AIS VDL and AIS base station; similar to msg 20 |
| FATDMA Reservation (absolute point of view) | - | X | X | X | X | Absolute equivalent of reservation |
| Msg 20 | X | X (when AIS base station operates in independent mode) | X (only when AIS base station operates in dependent mode) | - | - | Consists of a set of 4 reservations; msg 20 will be composed in AIS base station upon configuration settings from PSS when operating in independent mode or will be composed in PSS when AIS base station operates in dependent mode. |
| Msg-20-slot | X | X | - | - | - | Msg-20-slot is determined by the start slot |
| FATDMA start slot | - | X | X | X | X | Absolute term to be known for FATDMA scheme, schedule and plan; refer to Table 8 of Rec. ITU-R M.1371-1+Techn. Clarifications LME.FTST |
| FATDMA offset number | X | X (when AIS base station operates in independent mode) | X (only when AIS base station operates in dependent mode) | - | - | Relative term; refer to Table 33 of Rec. ITU-R M.1371-1+Techn. Clarifications |
| FATDMA block | X | X | X | X | X | FATDMA blocks are known both in relative and absolute terminology throughout the AIS Service and the AIS VDL |
| FATDMA increment | X | X | X | X | X | If FATDMA increments are used at all, they are known to all layers; refer to Rec. ITU-R M.1371-1 + Techn. Clarifications Table 8, LME.FTI |
| FATDMA block size | X | X | X | X | X | FATDMA block size is known both in relative and absolute terminology throughout the AIS Service and the AIS VDL (refer to Table 8 Rec. ITU-R M.1371-1; LME.FTBS) |
| FATDMA plan | - | -- | - | X | X | AIS VDL only recognizes the effects in terms of concrete reservations |
| FATDMA scheme | - | - | X | X | X | Group of all reservations of one PSS |
| FATDMA schedule | - | - | - | X | X | Group of all reservations of one or all PSS in given area |

# Fundamental philosophy statements for FATDMA Configuration

Before any details for FATDMA Configuration can be considered, some fundamental “philosophy” statements need to be set up.

## Rationale for fundamental usage rules of FATDMA

The complexity of the FATDMA increases in proportion to

- the slot usage requirements by different competent authorities in their envisaged coverage areas,

- the number of transmitting entities using the FATDMA reserved slots, and

- their relative geographical proximity.

Therefore, fundamental usage rules for FATDMA plans, FATDMA schemes and FATDMA schedules need to be established to resolve the complexity of FATDMA configuration in any given area.

## Guiding principles for setting up FATDMA plans

There are several possibilities to create such FATDMA plans, FATDMA schemes and FATDMA schedules. The following principles were adhered to when developing the FATDMA plan of this Annex:

### Rules in regard to the applicability of the FATDMA plan

- The rules underlying the FATDMA plan should be as simple as possible.

- The rules underlying the FATDMA plan should be globally applicable. The global applicability would provide for a fair and equal basis for setting up the FATDMA plans.

- The resulting FATDMA plan should cover the overwhelming majority of FATDMA configurations world-wide in just one planning step.

- When adjacent countries are required to co-ordinate this should require only the minimum number of parties involved, i. e. only the actually affected countries would treat the co-ordination issue independently of other co-ordination issues with other countries in a different area.

- There should not be a need to frequent or even run-time co-ordination between adjacent and affected countries.

- The most fundamental assumption is, that adjacent and affected national administrations are willing to protect the joint operation of the AIS VDL and to achieve its smooth operation in their area of (joint) responsibility. It is assumed that all affected national administrations will exhibit due diligence to arrive at a solution where co-ordination is needed.

- It is also assumed, that not all aspects of co-ordination can be satisfactorily achieved by FATMDA planning alone. Other mitigation methods consistent with the AIS system layout as a whole would be needed, and those mitigation methods are hinted at in this Annex but are fully described in other chapters of this document.

### Rules to minimise the usage of the VDL by FATDMA reservations

- The number of FATDMA reserved slots should be minimised in any given area. This would translate into the following consecutive rules:

- The FATDMA Configuration should be such, that FATDMA reservations are only done when there is a current (run-time) need for FATDMA reserved slots.

- The FATDMA block size should not exceed 5 consecutive slots (refer to Technical Clarifications §2.18.1 "Candidate Slots").

- However, this rule should not be applied rigidly, since the sophistication and the complexity of the components of the AIS Service(s) involved should be decreased rather than increased by a properly completed FATDMA plan. This translates into the following rule: The FATDMA plan should impose only a minimum of run-time dynamic FATDMA reservations. A maximum of stationary FATDMA reservations should be achieved.

- The FATDMA plan should provide for all capabilities of the AIS.

- The FATDMA plan should provide a maximum flexibility of AIS VDL operations.

- FATDMA reservations should be done, also using the frequency separation of the two working channels of the AIS (A and B) for the same slots. By default, mobile stations include the slots in the opposite working channel in the same slot only as a last resort into their candidate slot set, hence acting as if a de-facto FATDMA reservation was made in these other slots as well. Not to use these slots for FATDMA reservations would be a waste of available slots.

# FATDMA Reserved Slot Use Cases

This section describes the functional cases that require FATDMA reservations, the parameters that need to be considered, how to go about deciding the slot reservation profile and proposes usage recommendations for each case. The selection of the actual AIS base station reserving the slots will be discussed in another section.

Note: While the main concepts, such as the usage categories, are in place and are mature in this section, the detailed usage recommendations on individual messages should be reviewed and may be amended at the next revision of A-124.

## Rules for usage categories

The following rules, developed from the fundamental rules described in Section 4 apply to the  usage category definitions:

- At least one stationary FATDMA slot should be reserved for each of the defined usage-categories. This is necessary to minimise the dynamics of FATDMA allocation: All FATDMA reserved slots of a FATDMA plan, reserved for a given usage-category, should be reserved in such a way, that the appropriate FATDMA reservations comply exactly with the anticipated periodicity of the messages to be transmitted. To that end the FATDMA plan should take into account

- the required periodicity within one frame, which translates into the appropriate FATDMA increment and

- the usage of the individual minutes of an FATDMA epoch.

- In a single FATDMA reservation with more than one FATDMA reserved slot, the available slots can be used for different categories of usage independently of each other. This means, that slots reserved for different usage-categories can be bundled together in one FATDMA reservation, while their respective individual usage may be completely different.

## Use Cases for FATDMA reservation usage category 1 - AIS VDL management slots

The FATDMA reserved slots of usage category 1 will be used for the transmission of any message, which is necessary for the management of the AIS VDL.

The following use cases have been identified for FATDMA reservation usage category 1:

- Transmission of Data Link Management message  (msg 20) by base station(s)

- Transmission of Base Station Report (msg 4) with default reporting rate by base station(s)

- Transmission of Channel Management message (msg 22) by base station(s)

Each of the above use cases will be dealt with in more detail in consecutive sections.

### Transmission of Data Link Management Messages (msg 20)

When defining the usage of Data Link Management messages (msg 20),  the following aspects should be taken into account:

- At least one FATDMA reservation must be made before FATDMA reserved slots become available. The transmission of a Data Link Management message 20 is needed to make any FATDMA reservations in a given area.

- When starting a new FATDMA reservation, i. E. when there has been none previously,

- the FATDMA reserved slots will be available in the next frame after the frame during which the transmission of msg 20 occurred.

- the FATDMA reserved slots can only safely be used after 8 frames have elapsed because mobile AIS stations need to clear the FATDMA reserved slots.

- Depending on the total number of FATDMA reservations needed for a given geographical area, there may be more than one transmission of Data Link Management message 20 needed to accommodate all required FATDMA reservations in that area.

- Every FATDMA reservation times out after 3 minutes (default), if no different time out value is given within the FATDMA reservation. A different value may be between 1 and 8 minutes.

- Should a particular FATDMA reservation be maintained continuously longer than the above pre-set time out value, the FATDMA reservation needs to be refreshed periodically before the FATDMA reservation expires.

- Due to the time out value up to 8 minutes, one refreshment transmission of msg20 per FATDMA epoch would be sufficient to maintain that FATDMA reservation.

- The overlap of 2 minutes between maximum time out pre-set and FATDMA epoch length, would allow the distribution of the required transmissions of msg 20 over the first two frames of any FATDMA epoch, hence allowing for an even distribution of msg 20 transmissions in the first two frames of the first FATDMA epoch of any new continuous FATDMA reservation.

- When updating the refreshment of a continuous FATDMA reservation at the minimum rate, i. e. once per epoch, AIS mobile stations entering the area under consideration would learn about this FATDMA reservation with a delay of up to nearly 6 minutes.

- Note: The periodicity of transmissions of msg 20 should not be confused with the FATDMA increment of one FATDMA reservation. One FATDMA reservation may reserve many FATDMA blocks throughout the same frame, but may time-out after 8 minutes maximum because it was not refreshed.

Based on the above aspects two different use cases of transmission of msg20 can be identified:

5.2.1.1 Transmission of Data Link Management msg 20 to establish a short-lived FATDMA reservation (no repetitive pattern):

Short-lived FATDMA reservations should be used to accommodate dynamic demand for FATDMA reserved slots. 
Usage recommendation:

- When using a short-lived FATDMA reservation, its time-out should be pre-set to the duration of a FATDMA epoch, i. e. 6 minutes (or time-out value 5), maximum.

- Due to the purpose of short-lived FATDMA reservations, it should be carefully considered whether more than one FATDMA block should be reserved by this short lived FATDMA reservation, i. e. the FATDMA increment parameter should be used with care.

5.2.1.2  Transmission of Data Link Management msg 20 to establish a continuous FATDMA reservation (long term repetitive pattern):

Continuous FATDMA reservations should be used to accommodate the stationary demand for FATDMA reserved slots.
Usage recommendation:

- Data Link Management message (msg 20) should be transmitted at least once per epoch to maintain a continuous FATDMA reservation.

- When transmitting msg 20, the time-out value should be set to 6 minutes, thus spanning one entire FATDMA epoch.

- The transmissions of msg 20 for different continuous FATDMA reservations should be evenly distributed over the FATDMA epoch.

- Depending on the general risk assessment of message loss due to VDL traffic loading and / or interference in the given area, there should be [up to 6] transmissions of the Data Link Management message (msg 20) in one FATDMA epoch to maintain the same continuous FATDMA reservation, thus creating redundancy and providing for a more rapid reaction of new mobile AIS stations entering the given area to the FATMDA reservations. Those transmissions of msg 20 for the same continuous FATDMA reservation should also be evenly distributed over the FATDMA epoch, i. e. one msg 20 per 3 frames (2 transmissions per epoch), one msg 20 per 2 frames (3 transmissions per epoch), one msg 20 per one frame (6 transmissions per epoch).

### Transmission of Base Station Report (msg 4) at the default reporting rate

Assignment of the transmission of Base Station Report (msg 4) at the default reporting rate is justified, because a msg 4 with a valid position allows all mobile AIS stations to correlate this position with the transmission(s) of Data Link Management msg 20 using the same MMSI. The correlation is used by a mobile AIS station to determine whether the 120 nm intentional slot reuse rule should be used. This is an essential AIS VDL management rule because it  effectively counteracts the potential blocking of a large number of slots by the reception of non-relevant  msg 20 from distant base stations during times of  good RF propagation.

When defining the usage of Base Station Report (msg 4) at the default reporting rate, the following aspects should be taken into account:

- Table 1B of Recommendation ITU-R M.1371-1 states, that the default reporting rate by a base station should be once per 10 seconds.

- Therefore, the transmission of Base Station Reports exhibits a short-term repetitive pattern.

- This translates into six (6) slots reserved for that purpose per frame for a reporting interval of 375 slots, this translates into a FATDMA increment of 750 (msg 20) in one FATDMA reservation on each channel.

Usage recommendation:

When transmitting Base Station Reports (msg 4) with default reporting rate by one base station,

- there should be one (1) slot reserved with an increment of 750 on each channel, and

- the required FATDMA reserved slots should be allocated by a Continuous FATDMA reservation (see above).

### Transmission of Channel Management Messages (msg 22)

When defining the usage of Channel Management messages (msg 22), the following aspects should be considered:

- The Channel Management message (msg 22) provides the capability to change the  fundamental operation settings of the AIS frequency channels A and B.

- A mobile AIS station will store up to eight (8) different Channel Management settings, which were either received by Channel Management message, by a DSC telecommand or by manual setting aboard.

- If a new Channel Management setting is received, which partly or in total overlaps any of the previous Channel Management settings, the older Channel Management setting will be erased and replaced by the new Channel Management setting.

- Amongst other parameters, every Channel Management setting includes a geographical area (rectangle) for which this parameter set would be activated.

- The AIS mobile station uses the Channel Management setting corresponding to the geographical area in which it is currently located.

- The geographical area given for any Channel Management setting can be very large.

- The transient characteristics of a change in Channel Management settings involve at least the time span of two FATDMA epochs (compare IALA AIS Guidelines, Vol. I, chapter on Channel Management). Therefore, the transmission of Channel Management messages should be considered to exhibit long-term repetitive pattern.

- The timing requirements for the transmission of Channel Management messages as such are relaxed, i. e. the mobile AIS stations act upon a Channel Management setting when they enter the area designated by the Channel Management setting regardless of when they received the Channel Management setting.

- To ensure, that all mobile AIS stations operate on the default AIS frequency settings with the default Channel Management settings, a competent authority should transmit Channel Management messages on a regular basis with the default Channel management settings for its area of responsibility. These transmissions would instruct the mobile AIS stations to “return to default”. Due to the long-term repetitive pattern of the Channel Management message the VDL load created by these “return to default” transmissions would not be high.

Usage Recommendation:

- The geographical area parameter of any Channel Management message (msg 22) should be as small as possible. It may be equal to the size of the geographical area of the FATDMA plan, FATDMA schedule or even FATDMA scheme.

- Channel Management messages should be transmitted at least once per FATDMA epoch, i. e. there should be [one (1)] slot reserved  with an increment of 0 [or 2250].

- Depending on the general risk assessment of message loss due to VDL traffic loading and / or interference in the given area, there should be [up to 6] transmissions of the Channel Management message (msg 22) in one FATDMA epoch, thus creating redundancy and providing for a more rapid reaction of new mobile AIS stations entering the given area. Those transmissions of msg 22 should also be evenly distributed over the FATDMA epoch, i. e. one msg 22 per 3 frames (2 transmissions per epoch), one msg 22 per 2 frames (3 transmissions per epoch), one msg 22 per one frame (6 transmissions per epoch).

- the required FATDMA reserved slots should be allocated by a Continuous FATDMA reservation (see above).

- Even when there is no channel management scheme in place, which deviates from the default Channel Management settings in any regard, the Channel Management message should be transmitted by administrations with default Channel Management settings (return to AIS1 and AIS2 in the geographical area) to erase „wrong“ and „old“ frequency settings in mobile AIS station’s memories.

## Use Cases for FATDMA reservation usage category 2 - High timing requirements slots

These FATDMA reserved slots will be used for the transmission of messages, which need to be transmitted within a defined time period.

The following use cases have been identified for FATDMA reservation usage category 2:

- Transmission of Base Station Report (msg 4) when a base station in the given area is acting as a semaphore

- Transmission of Position Reports of any kind of AIS mobile stations, when assigned a “FATDMA protected” slot (“safe slot” concept)

- Transmission of Safety Related message (msg 12 and 14)

- Transmission of Acknowledgements on received addressed messages (msg 7 and 13) by base station(s)

- Transmission of DGNSS broadcast binary messages (msg 17) with high timing requirements contents, such as integrity alerts, by base station(s)

- Re-transmission of Position Reports (and other messages) of any kind of AIS mobile stations

### Transmission of Base Station Reports when a base station (in the given area) is acting as a semaphore

The assignment of the transmission of Base Station Report (msg 4) with increased reporting rate due to a base station switching into semaphore mode is justified, because the reporting interval is decreased to a value below 4 seconds. This constitutes a high timing requirement.

When defining the usage of Base Station Report (msg 4) with increased reporting rate due to a base station acting as a semaphore, the following aspects should be taken into account:

- Table 1B, footnote 1, of Recommendation ITU-R M.1371-1 states, that the increased reporting rate by a base station should be once per 3 1/3 seconds.

- Therefore, the transmission of Base Station Reports exhibits a short-term repetitive pattern.

- This translates into eighteen (18) slots reserved for that purpose per frame for a reporting interval of 125 slots or into a FATDMA increment of 250 (msg 20) in one FATDMA reservation on each channel.

- Since the requirement for a base station to act as a semaphore is infrequent and transient in nature, one or several short-lived FATDMA reservations may suffice for the duration of the base station acting as a semaphore.

When transmitting Base Station Reports (msg 4) with an increased reporting rate due,

- there should be one (1) slot reserved with an increment of 250 on each channel, and

- the required FATDMA reserved slots should be allocated by one [Short-Lived] FATDMA reservation with time-out value pre-set to maximum, i. e. 8 minutes, and consecutive [Short-Lived] FATDMA reservations with the same parameters as long as the base station is acting as a semaphore (see above).

- Instead, the required FATDMA reserved slots may be taken from a continuously and in advance reserved pool of usage category 2 slots within the same FATDMA scheme, FATDMA schedule or FATDMA plan. Although the latter option would smooth the operation of the VDL by avoiding transient FATDMA reservations, more VDL capacity will be used when it will not actually be needed.

### Position Reports of any kind of AIS mobile stations, when assigned a “FATDMA protected” slot (“safe slot” concept)

To assign the transmission of Position Reports of any kind of mobile AIS stations, which have been assigned to specific “FATDMA protected” slots given by a previous assignment command, to the FATDMA usage category 2 („High timing requirements slots”) is justified, because the position reports will invariably occur in the slots pre-determined by the previous assignment command, but the resulting reporting rate of the mobile AIS station may still be high (Class A mobile stations will allow only to increase the reporting rate up to once per second). This constitutes a high timing requirement.

When defining the usage the following aspects need to be taken into consideration:

- assigned transmissions of mobile stations either hard assignment (msg 2, 18)

Usage recommendation:

When reserving FATDMA protected slots for assigned mode mobile stations

- The number of slots and the increment required depend on the defined reporting rate

- The required amount of slots should be allocated by one [short lived] FATDMA reservation preset to x minutes (depending on the timeout of the assignment)

### Transmission of Safety Related Messages (msgs 12 and 14) by base station(s)

The assignment of the transmission of Safety Related Messages (msgs 12 and 14) is justified, because it is a general operational requirement that safety related messages are being transmitted to the VDL with minimum latency after being released. This constitutes a high timing requirement.

When defining the usage the following aspects need to be taken into consideration:

The number of slots required for safety related messages can dynamically vary between 1 and 5

Usage recommendation:

When reserving FATDMA protected slots for Safety Related Messages

- Safety related messages should be transmitted at least once per FATDMA epoch, i.e. there should be 5 slots reserved (to cover the maximum message length) with an increment of [0 or 2250].

- the required FATDMA reserved slots should be allocated by a Continuous FATDMA reservation (see above).

### Transmission of Acknowledgements on received addressed messages (msg 7 and 13) by base station(s)

The assignment of the transmission of Acknowledgements on received addressed messages (msg 7 and 13) is justified, because the acknowledgement on a correctly received addressed message is required and must be received within 4 seconds by the originating station of the addressed message in order to prevent re-transmission of the same addressed message by the originating station, which would result in unnecessary VDL load. This constitutes a high timing requirement.

When defining the usage the following aspects need to be taken into consideration:

- detailed description of the use case

Usage recommendation:

When reserving FATDMA slots for Acknowledgements of received addressed messages

- Acknowledgement messages should be transmitted at least once per FATDMA frame, i.e. there should be one [1] slot reserved with an increment of [0 or 2250].

- the required FATDMA reserved slots should be allocated by a Continuous FATDMA reservation (see above).

### Transmission of DGNSS broadcast binary messages (msg 17) with high timing requirements contents, such as integrity alerts, by base station(s)

The assignment of the transmission of DGNSS broadcast binary messages (msg 17) with high timing requirements contents, such as integrity alerts, is justified, because of the relevant operational requirement in the field of GNSS.

Note: While the transmission of e. g. integrity alerts of DGNSS message to mobile AIS stations may exhibit a high timing requirement, this may not be true to any other DGNSS message content. Therefore, all DGNSS messages contents should be examined in regard to their timing requirements. DGNSS Message contents identified not to exhibit high timing requirements for transmission should be treated under the usage category 3 (“General purpose pre-reserved slots”).

When defining the usage the following aspects need to be taken into consideration:

- The most stringent demand with respect to the DGNSS corrections update rate is the integrity functionality. This holds true in particular to GPS since S/A was removed from GPS.

- The time for broadcasting a DGNSS integrity warning to the user, from the time the integrity problem is identified within the DGNSS correction source(s), i. e. a DGNSS reference station, to the reception of that DGNSS integrity alert at the user receiver equipment (here: the AIS mobile equipment) should not exceed 10 seconds. This results in a small latency budget for the AIS Service, which in turn translates into a high timing requirement for transmissions of DGNSS integrity alerts.

- There can be more than one integrity alert simultaneously, since more than one satellite may exhibit flawed behaviour. Up to four (4) DGNSS integrity alerts may be included in one DGNSS broadcast message using one (1) FATDMA reserved slot.

- This DGNSS integrity alert message may substitute regular DGNSS correction messages, which have a less stringent timing behaviour (therefore, those are included in FATDMA reservation usage category 3).

- Since the AIS Service does not create the DGNSS corrections and integrity alerts itself but rather receives them from an external DGNSS correction source (for instance the fully monitored IALA beacon reference station system, as stipulated by IALA Recommendation R – 121 Edition 1.1 Dec 2004 ), the replacement of regular DGNSS corrections with DGNSS integrity alerts – as need may be – will be done at the external DGNSS correction source. Therefore, the AIS Service will only be required to provide a transparent means to transmit the DGNSS correction broadcasts received from the external source within proper timing bounds.

- Only part of the above latency budget of 10 seconds can be used by the AIS Service. The other part would be needed by the DGNSS correction source to establish and distribute the integrity warning.

Usage recommendation:

- Provisions should be made to transmit a DGNSS broadcast message containing at least one DGNSS integrity alert, one slot every 6 seconds with an increment of 225, and

- the required FATDMA reserved slots should be allocated by a Continuous FATDMA reservation (see above).

- This would require to use FATDMA reserved slots of FATDMA reservation usage category 2.

- This defines the minimum update rate for DGNSS broadcast messages and the minimum number of FATMDA reserved slots for this service.

- While fulfilling this requirement there are still some alternatives to setup a regular DGNSS correction broadcast scheme. They will be discussed under the appropriate heading in FATDMA reservation usage category 3 below.

### Re-transmission of Position Reports (and other high-timing requirement messages) of any kind of AIS mobile stations

- detailed description of the use case will be provided in a future document release

-- while the re-transmission of position reports of mobile AIS stations may exhibit a high timing requirement, this may not be true to any message, to be re-transmitted. Therefore, all messages which may be transmitted by a class of mobile station should be examined in regard to their timing requirements. Messages identified not to exhibit high timing requirements for re-transmission should be treated under the usage category 3 (“General purpose pre-reserved slots”)

--- refer also to separate chapter in the same part of A-124 (chapter 48) for mechanics of re-transmission

## Use Cases for FATDMA reservation usage category 3 - General purpose pre-reserved slots

These FATDMA reserved slots will be used for the transmission of any message, which does not fall under FATDMA reservation usage category 1 or 2.

The following use cases have been identified for FATDMA reservation usage category 3:

- Transmission of any assignment command (msg 16; [msg 23]) by base station(s)

- Transmission of UTC and Date Inquiry message (msg 10) by base station(s)

- Transmission of DGNSS broadcast binary messages (msg 17) with relaxed timing requirements contents by base station(s)

- Transmission of Binary Messages (msg 6 and 8) by base station(s)

- Transmission of Interrogation message (msg 15) by base station(s)

- Transmission of Aids-to-Navigation reports (msg 21; [new AtoN msg])

- Re-transmission of messages other than position reports and of AIS mobile stations by AIS fixed station(s)

### Transmission of any assignment command (msg 16; [msg 23]) by base station(s)

When defining the usage of any assignment command (msg 16 [or msg 23]), the following aspects should be considered:

- There are two possible options of the assignment command which impact the behaviour of mobile AIS stations: assignment of a new reporting rate behaviour while leaving the actual slot selection to the mobile AIS station (so called “soft” assignment) and assignment of the usage of specific pre-determined slots by the assigned mobile AIS station (so called “hard” assignment).

- Assignment Command (msg 16) and Group Assignment Command (msg 23) differ in that the Assignment Command applies to individual mobile AIS stations and provides for both “soft” and “hard” assignment, while the Group Assignment Command provides the capabilities to address group of mobile AIS stations determined by their respective device class.

- Every assignment times out after 3 minutes (default), if no different time out value is given within the assignment command. A different value may be between 1 and 8 minutes. The effect of the [Go silent] parameter for Class B mobile AIS stations within the Group Assignment Command (msg 23) will last for up to 15 minutes, however.

- All transmissions of assignment commands do not – by default – exhibit a repetitive pattern. Therefore the transmission of assignment commands would require slot reserved in the usage category 3 (“General purpose pre-reserved slots”)

- Should a particular assignment be maintained over a longer period than 8 minutes, the assignment needs to be refreshed periodically before the present assignment expires. In this case, the following should be considered:

- Due to the time out value up to 8 minutes, one refreshment transmission of the assignment command per FATDMA epoch would be sufficient to maintain that assignment.

- When updating the refreshment of an assignment at the minimum rate, i. e. once per epoch, AIS mobile stations entering the area under consideration would learn about this assignment with a delay of up to nearly 6 minutes.

- While a mobile AIS station commanded to change into slot assignment mode (“hard” assignment) exhibits a high timing requirement for the pre-reserved slots, the assignment command as such has no high timing requirement for transmission. [The latency between the trigger event (assignment request) and the reaction shown by the assigned mobile AIS station(s) will be determined not by the timing of the transmission of the assignment command (msg 16 or msg 23) but by the present frame: Any assignment will be effective with the next frame earliest. Hence the latency can be up to one minute, which is the duration of one frame.]

NOTE: this will be checked with Class A, Class B etc. and clarification will be provided, in a future release of this document.

Usage recommendation:

- Assignment commands should be transmitted in [one (1)] of the next available slots reserved in the usage category 3 (“General purpose pre-reserved slots”).

- Should a particular assignment be maintained over a longer period than 8 minutes, the assignment needs to be refreshed periodically before the present assignment expires. In this case, the following recommendation applies:

- The assignment command (msg 16 or msg 23) should be transmitted at least once per epoch for a un-interrupted assignment as long as the desired duration of the assignment.

- When transmitting the assignment command, the time-out value should be set to 6 minutes, thus spanning one entire FATDMA epoch.

- Depending on the general risk assessment of message loss due to VDL traffic loading and / or interference in the given area, there should be [up to 6] transmissions of the assignment command to maintain the un-interrupted assignment, thus creating redundancy and providing for a more rapid reaction of new mobile AIS stations entering the given area to the assignments. Those transmissions of assignment command for the same desired assignment should also be evenly distributed over the FATDMA epoch, i. e. one assignment command per 3 frames (2 transmissions per epoch), one assignment command per 2 frames (3 transmissions per epoch), one assignment command per one frame (6 transmissions per epoch).

### Transmission of UTC and Date inquiry message (msg 10) by base station(s)

- detailed description of the use case will be developed in a future release

### Transmission of DGNSS broadcast binary messages (msg 17) with relaxed timing requirements contents by base station(s)

The DGNSS broadcast binary message (msg 17) is designated for distribution of DGNSS information. Broadcasting DGNSS corrections from AIS base station(s) on the AIS VDL enables the receiving AIS stations to provide position reports with integrity and improved accuracy.

#### Timing requirements depending on DGNSS correction message content

While the transmission of e. g. integrity alerts of DGNSS correction message to mobile AIS stations exhibits a high timing requirement, this may not be true to any other DGNSS correction message content. Therefore, all DGNSS correction messages contents should be examined in regard to their timing requirements. DGNSS Message contents identified not to exhibit high timing requirements for transmission should be treated here under the usage category 3 (“General purpose pre-reserved slots”).

The requirements of DGNSS correction messages containing high timing requirement integrity information were introduced under the usage category 2 (“High timing requirement slots”).

#### Responsibilities of the external DGNSS Correction Source compared to that of the AIS Service

The actual source for the DGNSS correction messages to be transmitted via the AIS is external to the AIS Service. That source, which should be set up and operated in accordance with the appropriate IALA recommendations, originates a constant flow of DGNSS correction messages, which could contain either DGNSS integrity alerts, thus leading to the “high timing requirements” as discussed above, and / or regular DGNSS corrections.

Since the AIS Service does not generate the DGNSS corrections and should not be involved in the sophistication of the generation of DGNSS corrections but rather act like an ideal transmission scheme for those DGNSS correction messages, it is the responsibility of the external DGNSS Correction Source to schedule the correct flow of DGNSS correction messages to the AIS Service, thus inserting DGNSS integrity alerts when required instead of regular DGNSS correction messages. On the other hand, it is the responsibility of the AIS Service to provide FATDMA reserved slots both for the DGNSS integrity alerts (see “high timing requirement section”) and for the regular DGNSS corrections (discussed here).

#### Degradation of DGPS accuracy with age of corrections

An important aspect when transmitting DGNSS corrections via AIS is the additional channel load in consequence of the necessary update rate of the DGNSS corrections, i. e. the DGNSS corrections update rate. Therefore, some consideration must be given to DGNSS correction update rate scenarios.

The following figure shows how the errors in a differentially corrected position fix increase with increasing age of correction in the absence of S/A [reference take from M. Hoppe’s paper]. The age-of-correction and hence the DGNSS broadcast message transmission interval for the same satellite could be extended from a value of less than 10 seconds (which is used in the IALA DGNSS MF beacon system) to about 3 minutes without the errors in position accuracy increasing by more than 0.1 m. Indeed, errors in position accuracy remain below 1m for all DGNSS broadcast message transmission intervals up to 8.5 minutes.

The following table shows the increase of position accuracy error for various intervals.

| Time interval between DGNSS broadcast message transmissions [minutes] | Position accuracy error at end of time interval [95%; in meters; absence of S/A] |
| --- | --- |
| 0.5 | Less than 0.1 |
| 1.0 | Less than 0.1 |
| 2.0 | Less than 0.1 |
| 3.0 | 0.2 |
| 4.0 | 0.4 |
| 5.0 | 0.6 |

#### Resulting scenarios for regular DGNSS correction message transmission using FATDMA reservations usage category 3

From the above considerations follows, that there are some different scenarios possible with regard to the DGNSS correction message broadcast.  There are identified the following five (5) different scenarios (see table below).

| Scenario | Consecutive Number of FATDMA reserved time slots needed per DGNSS correction message | Number of corrected satellites/bits needed (incl. 80 bit for header) | DGNSS correction message update rate | Resulting data age [s] for complete set of corrections | Slots needed for scenario and frame | Resulting AIS VDL Channel load [%] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | One Channel usage | Two Channel usage |
| Scenario 1 | 3 | 12 / 560 bits | 5 | 5 | 36 | 1.6 | 0.8 |
| Scenario 2 | 1 | 2 / 160 bits | 5 | 30 | 12 | 0.53 | 0.26 |
| Scenario 3 | 3 | 12 / 560 bits | 30 | 30 | 6 | 0.26 | 0.13 |
| Scenario 4 | 2 plus 1 | 8 / 400 bits 2 / 160 bits | 30 5 | 5 – 30 | 6 12 | 0.8 | 0.4 |
| Scenario 5 | 2 | 8 / 400 bits | 5 | 5 for 8 GNSS satellites; 10 for 12 GNSS satellites | 24 | 1 | 0.5 |

Scenario 4 implies the reservation of two FATMDA blocks. “FATDMA block 1” uses two (2) time slots of FATDMA reservation usage category 3 to transmit DGNSS corrections for up to 8 GNSS satellites with an update rate of 30 seconds. “FATDMA block 2” uses one (1) time slot of FATDMA reservation usage category 2 with an update rate of 5 seconds; this “FATDMA block 2” would be used for integrity alerts, whenever they appear, or would be used on a regular basis for regular DGNSS corrections.

Scenario 5 implies that the DGNSS correction source provides only DGNSS correction messages, regular and / or integrity warnings, to the 8 most relevant GNSS at the simultaneously.

#### Assessment of the scenarios presented

The pros and cons of the different scenarios are as follows.

##### Scenario 1

Advantages:

- Lowest possible age-of-correction for complete GNSS satellite set

- Integrity alerts already included in DGNSS correction messages:

- No need for additional FATDMA reservation usage category 2 slots (“high timing requirement slots”) due to DGNSS broadcasts

- No need for AIS Service to provide additional algorithm for discerning the different timing requirements, i. e. straight forward connection of AIS Service to DGNSS correction source possible

- No dynamic FATDMA reassignment needed for regular DGNSS correction messages

Disadvantages:

- Largest AIS VDL load of all scenarios considered due to DGNSS correction broadcast

##### Scenario 2

Advantages:

- Low AIS VDL channel load

- No dynamic FATDMA reassignment needed for regular DGNSS correction messages

Disadvantages:

- Relatively high data age for complete set of GNSS satellites

- requirement for additional FATDMA reservation usage category 2 slots(“high timing requirement slots”) when more than 4 GNSS satellites would need integrity alerts simultaneously, the probability of which should be considered low.

- An additional algorithm is needed in AIS Service to make sure that the most critical DGNSS correction data is transmitted with the highest priority for those max. 4 GNSS satellite’s integrity alerts.

##### Scenario 3

Advantages:

- Lowest AIS VDL channel load of all alternatives

- No dynamic FATDMA reassignment needed for regular DGNSS correction messages

Disadvantages:

- Relatively high data age for complete set of GNSS satellites

- Requirement for additional FATDMA reservation usage category 2 slots(“high timing requirement slots”) when any one GNSS satellite would need integrity alert.

##### Scenario 4

Advantages:

- Relatively low AIS VDL channel load

- No dynamic FATDMA reassignment needed for regular DGNSS correction messages

- Low age-of-correction for those DGNSS corrections transmitted using the one FATDMA reservation usage category 2 slot

Disadvantages:

- Requirement for one additional FATDMA reservation usage category 2 slot (“high timing requirement slots”) in any case, for use for regular DGNSS correction broadcast or for GNSS integrity alerts when any one GNSS satellite would need integrity alerts simultaneously.

- An additional algorithm is needed in AIS Service to make sure that the most critical DGNSS correction data is transmitted with the highest priority for those max. 4 GNSS satellite’s integrity alerts.

##### Scenario 5

Advantages:

- Data age is less than 10 seconds

- Integrity alerts already included in DGNSS correction messages:

- No need for additional FATDMA reservation usage category 2 slots (“high timing requirement slots”) due to DGNSS broadcasts

- No need for AIS Service to provide additional algorithm for discerning the different timing requirements, i. e. straight forward connection of AIS Service to DGNSS correction source possible

- No dynamic FATDMA reassignment needed for regular DGNSS correction messages

Disadvantages:

- Second highest AIS channel load of all scenarios considered due to DGNSS correction broadcast

- Maximum of 8 relevant GNSS satellites are corrected.

#### Provision for future additional need for DGNSS broadcast

The selection of one of the above scenarios may also be determined by the different GNSS to which DGNSS corrections should be provided. There may be a need to transmit DGNSS correction messages for GALILEO in the future. For the following reasons any such future requirement can be accommodated as long as the high timing requirements for the appropriate integrity alerts are met by FATDMA reservation usage category 2 time slots.

- The responsibility of the composition of the proper flow of DGNSS correction messages lies with the external DGNSS correction source, regardless to what GNSS the messages apply to.

- The degradation of position accuracy due to age-of-correction is graceful as shown above.

#### Recommended use

All of the above DGNSS correction broadcast scenarios have their trade-offs. All have in common that they do not require dynamic FATDMA reassignment provided that proper FATDMA reservation planning is done beforehand. All above scenarios would fulfil both requirements for DGNSS integrity alerts and regular DGNSS broadcast corrections.

Therefore, a competent authority may chose one of the above scenarios depending on the local situation. A competent authority may also change the DGNSS correction broadcast scenario over time.

### Transmission of Interrogation message (msg 15) by base station(s)

A detailed description of the use case will be developed for a future release of this document.

Refer to ITU 3.3.8.2.11

### Transmission of Binary Messages (msg 6 and 8) by base station(s)

A detailed description of the use case will be developed for a future release of this document.

The number of slots required for safety related messages can dynamically vary between 1 and 5

### Transmission of Aids-to-Navigation Reports (msg 21; [new AtoN msg])

A detailed description of the use case will be developed for a future release of this document.

Usage rules:

- Two slots required for each AtoN message repeated every third frame.

- AtoN AIS stations: AtoN AIS stations may use slots, which have been reserved by a base station by FATDMA; therefore the AtoN AIS station could be configured to broadcast in fixed slots (number given by competent authority of FATDA scheme)

### Re-transmission of messages other than position reports and of AIS mobile stations by AIS fixed station(s)

- detailed description of the use case

- Usage Rules will be developed in a future release of this document

### Setting up General Purpose Pre-reserved Slots

The above cases indicated what transmissions may actually use the General Purpose Pre-Reserved Slots. The recommendations given above assume the existence of a pool of General Purpose Pre-Reserved Slot

- of sufficient size to accommodate the required usage capacity and

- of sufficient repetitive occurrence within one frame to minimize the latency of the actual transmission and provide for relaxed queuing.

Usage Rules will be developed in a future release of this document

# The definition of FATDMA schemes by space, time and frequency usage separation

After the previous section has introduced the purpose of FATDMA reservations on a detailed level and has given the conditions which apply for certain use case categories of FATDMA reservations in some detail, this section deals with the question: What should be the geographical criteria to determine co-ordinated operation?

## The universal grid scheme – separation by space and frequency

The geographical grid defines a global method for selecting a default FATDMA scheme for any area under consideration: It should be noted that the area is the most important aspect, not the actual location of the transmitting entities using FATDMA reserved slots. It is the total of transmitted, i. e. injected, messages of all relevant AIS stations using FATDMA reserved slots, that count in a given area, i. e. cell, not the station and its location as such in the first place.

It is the goal of the geographical grid to ensure, that identical FATDMA schemes by default are geographically separated more than 120 nautical miles, by arranging a set of FATDMA schemes in a geographical grid in such a manner, that no FATDMA scheme is reused within 120 nautical miles.

The geographical grid is meant as a geographical starting point from which further planning and co-ordination must take place, if needed at all.

The geographical grid is created by dividing the world into cells as illustrated below, by an algorithm, based on the WGS84 datum. The precise algorithm for calculating the geographical grid is given in Attachment A to this Annex. The following picture just gives an impression in principle.

The illustration contains a grid with a repetitive pattern of 36 different FATDMA schemes in an 6 x 6 matrix (called a ‘master cell’), where each cell is approximately 30 x 30 nautical miles in size. Employing the two working frequencies A and B of the AIS would yield 72 different FATDMA schemes in each master cell, while each individual cell would have the two FATDMA schemes created by using the very same time slot reservation pattern but only in the mirrored working frequency.

Identical FATDMA schemes are repeated in a distance of the size 120 nm + 2*cell size, i.e. in a distance of 180 nautical miles.

The above considerations yield one number of a FATDMA scheme. To denote the frequency separation, the two FATDMA schemes are identified by adding the Roman numerals “I” and “II” to the number, e. g. 59-I and 59-II. It should be noted that neither FATDMA scheme works exclusively on AIS working channel A or B exclusively, but alternate between these working channels constantly, but in a frequency mirrored manner. Hence, the Roman numerals were chosen to avoid confusion with the working channel designation A and B.

## FATDMA schemes – separation in time

Above the definition was introduced how to arrive at a cell number for the geographical cell, any given location on earth is in. Further the frequency separation was included. Both FATDMA schemes thus derived are identical in regard to their time slot reservation and usage characteristics, but employ these on mutually mirrored frequencies. It is now necessary to define to what absolute FATDMA reservations and what precise usage categories the above FATDMA scheme numbers translate to.

This is done in a table in Attachment B, in accordance with the following rationale:

As described in the section on usage categories, it is desirable that the FATDMA schemes are confined to a certain part of the time domain of the AIS VDL in order to provide sufficient free time slots for mobiles to allocate under all dynamical situations. FATDMA schemes defined here are a repetitive pattern of 5 + 5 + 4 timeslots for every 25 timeslots:

| x | x | x | x | x |  |  |  |  |  | x | x | x | x | x |  |  |  | x | x | x | x |  |  |  | x | x | x | x | x |  |  |  |  |  | x | x | x | x | x |  |  |  | x | x | x | x |  |  |  | x | x | x | x | x |  |  | .... |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  | 5 |  |  |  |  | 10 |  |  |  |  | 15 |  |  |  |  | 20 |  |  |  |  | 25 |  |  |  |  | 30 |  |  |  |  | 35 |  |  |  |  | 40 |  |  |  |  | 45 |  |  |  |  | 50 |  |  |  |  | 55 |  |  |

This repetitive pattern of every 25 timeslots ensures, that mobiles’ transmissions with associated reporting rates of 2 seconds, 3 1/3 seconds, 6 seconds, 10 seconds... like those reporting rates defined for Class A mobiles, (translating into one transmission every 75, 125, 225, 375.. timeslots,) which fits neatly into the remaining timeslots.

These FATDMA slot reservations are the theoretical maximum at planning time and will not be used all simultaneously in the overwhelming majority of cases globally in the same cell. For the few remaining cases, where special considerations are needed, see below.

The above FATDMA schemes are divided into slots assigned to specific usage categories as given in the Attachment B. The assignment to usage categories rest upon the following rationale. Each FATDMA scheme consists of:

- Usage category 1 time slots – VDL management

- Three (3) timeslots per frame and working channel for Base Station report in non-semaphore mode

- One (1) timeslots per frame and working channel for Data Link Management and / or Channel Management messages

- Usage category 2 time slots – High Timing requirements: Six (6) timeslots per frame and working channel for Base Station reports in semaphore mode, other high timing requirement transmissions; these time slots can be used for Usage category 3 (General purpose) in the absence of Usage category 2 messages.

- Usage category 3 time slots – General purpose: Seven (7) timeslots per frame and working channel for general purpose use or possibly for Usage Categories 1 or 2 (if urgent need should appear)

To re-iterate what was said before: Only time slots needed at run-time should be actually reserved.

Consequential action in particular in those (few) cases when the above FATDMA reservations would not suffice: The requirement for implementing queuing techniques (including priority schemes) as a mitigation measure on the part of the sources for transmission (not necessarily the AIS base stations of the national competent authority alone) is obvious.

After this definition part, the following sections will introduce the rules for allocating the above FATDMA schemes, referenced by their number, to competent authorities, both national and domestic competent authorities.

# Allocation of the defined FATDMA schemes to competent authorities

## Summary of the previous definitions and their consequences

The sections so far have introduced the definitions and concepts needed for a FATDMA plan. The means developed for setting up the FATDMA plan can be summarised by the following two parts:

- The geographical grid scheme introduced in the previous section provides for separation of FATDMA reservations in space, taking into account the 120 nautical mile rule.

- The FATDMA schemes, identified by their FATDMA scheme code, provide the combination of separation of FATDMA reservations in time, i. e. different time slots, and frequency, i. e. different working channels A and B, while preserving the alternating use of –those two working channels. The FATDMA scheme layouts have been informed by usage considerations as well, i. e. they assign certain usage categories to certain slots in the FATDMA schemes.

As a consequence, the FATDMA schemes, identified by their FATDMA scheme code, together with the geographical grid, provide a means to allocate FATDMA schemes for each geographical area, isolated from other geographical areas and their allocated FATDMA schemes – at least in a first, and in most cases sufficient step. Also, this is globally applicable.

Also the means exhibit a certain simplicity as it is built only on the concept of an area, as opposed to location: transmitting stations, using FATDMA reservations, inject their messages into that given area, i. e. one geographical grid cell, while the concrete location from where this transmission occurs within that geographical grid cell is less relevant due to the separations in space introduced by the geographical grid scheme.

From a practical point of view, this means that there will be minimum effort required to co-ordinate between adjacent countries and their national competent authorities because of the isolation of the FATDMA schemes in a given area: By introducing the geographical grid, only bi-lateral or tri-lateral co-ordination would be required in areas, where on the geographical grid cell falls into more than one country.

There are consequences that need to be discussed at a later planning stage, if they are relevant at all:

- The upper limit of the VDL loading due to FATDMA reservations can only be assessed in a given area after the concrete FATDMA plan for that area has been established.

- There may be restrictions to the usage of FATDMA reserved slots over time in areas, where a larger number of transmitting stations are planned to inject messages: Not every message to be transmitted may be transmitted at once when a FATDMA reserved slot allocated to that transmitting station arrives; there may be the need for queuing. These restrictions are due to the very nature of the AIS; a mitigation measure in the long run would be to increase the number of working frequencies of the AIS (which would require more than two receiving processes in mobile AIS stations).

- Proper, and even sophisticated coverage planning would be needed in some cases, employing directional antennas, lower antenna heights of the transmitting station(s) and even transmission power attenuation of the transmitting station(s), making use of or at least taking into account the local topography: The goal would be keep the injected message confined to that geographical grid cell if at all possible.

However, it is anticipated that these consequences will only be relevant to a few places globally, thus justifying extra planning efforts at those places (instead of not having a simple global plan for the large majority of cases). In addition, it is good to know that there are mitigation measures in place for areas where the most simple measures would no longer work.

## Stepwise allocation of the defined FATDMA schemes to competent authorities

Based on the above definitions and having considered the consequences, the following simple two-step approach for planning can be taken.

### Step 1: International allocation of FATDMA schemes

#### Step 1A: Check need for international co-ordination

All FATDMA schemes that fall completely – judging by the area of their geographical grid cell - within the EEZ (Exclusive Economical Zone) of a country belong to that country. They are immediately subject to domestic allocation rules (see step 2 below).

By means of the FATDMA scheme definition of the previous section this should cover the overwhelming majority of cases globally already in just one step.

#### Step 1B: Need to international allocation co-ordination arises

There are FATDMA schemes, which fall - by the geographical area of their geographical grid cell - into the adjacent EEZ of different countries. To these only apply the following rules.

- Thus affected countries and only the thus affected countries need to negotiate the allocation on a peer-to-peer basis and create a memorandum of understanding (or similar document) on the usage of the FATDMA scheme in question. They may use one or more of the following options, combining them as deemed necessary.

- Option A: One or more than one country can contribute an unused and immediately adjacent FATDMA scheme from within its own area (EEZ), where it can be guaranteed by that country that this cell will remain unused (e. g. due to topology).

- Option B:  The cell management is assigned solely to one of the two countries, while both may use the FATDMA scheme on a mutually agreed basis.

- Option C: Both countries set up a detailed usage allocation based on the usage categories of the slots of the FATDMA scheme.

- Option D: There may be an alternating use (e. g. based on epoch) of the same slots of a FATDMA scheme. In this case the countries need to agree on the time schedule for the alternating in advance.

- The result of the international allocation co-ordination is communicated internationally and appropriately.

- The affected countries are encouraged to do a maximum of co-ordination at planning time; there should not be a large need for run-time exchange of FATDMA scheduling data between countries.

### Step 2: Domestic allocation of slots out of the FATMA schemes for that country

- The national competent authority uses the internationally allocated FATDMA scheme on a primary basis; other competent authorities of that country use individual slots as a subset out of that FATDMA scheme due to allocation by the national competent authority on a secondary basis.

- The national authority represents the complete set of usage requirements (also of other competent authorities) to the international co-ordination (see step 1).

- The VDL management capabilities of the AIS (by AIS base stations) as well as the usage of AIS repeater stations are completely reserved to the national competent authority. They are exclusively considered part of the primary allocation domain.

- The other competent authorities should exclusively use other classes of AIS stations, such as the AIS AtoN station and / or Limited AIS Base Station, the definition of which are currently under development at IEC.

- There should be memorandum of understanding (or some similar document) between the national competent authority and other domestic competent authority stating the distribution of individual slots and their intended usage within the FATDMA schemes assigned to that country by international allocation.

## FATDMA configuration data exchange between Administrations at run-time

The previous chapters have shown that careful considerations between adjacent competent authorities are needed at planning time. Furthermore, a FATDMA set-up may not be changed frequently under „real life“ conditions: coverage planning and statutory rights of the competent authorities have been taken into account to arrive at the optimum FATDMA plan or FATDMA schedule for a given area.

While a theoretical possibility to exchange run-time data on actual FATDMA schedules (or maybe even FATDMA schemes) exists on the level of the ASM , this option should not be employed widely and is discouraged by this Recommendation to be used at all.

Part VIII - Guidelines for Assigned Mode Operation by an AIS Service (new Annex 15 and 18)

Part IX - Guidelines for DGNSS Correction broadcast from an AIS Service (new Annex 16)

Part X - Guidelines on Retransmission using VDM from an AIS Service (new Annex 9)

The base station receives AIS messages from the VDL and PI.  Received VDL messages are sent to an external application for processing via the PI as VDM sentences.  VDM sentences received via the PI are retransmitted.  These messages are not processed by the base station.  If received VDL messages are retransmitted, some messages need modifications before retransmission.  The external application is responsible for providing the modified VDM sentences.

The process of assembling the binary data to broadcast on the VDL is similar to the process of assembling an encapsulated ABM or BBM. The difference is that the entire content is transmitted. See IEC: 61993-2 for details.  To make the proper adjustments to the “communication state” and “Repeat Indicator” parameters, refer to the chapter on “Functional requirements for the AIS Simplex Repeating”.

General Guidance for the use of position reports; a retransmitted report should be rejected if the time stamp of the position report is identical or older than the current report being used for that unit (identified by MMSI).

A position report shall not be retransmitted:

- if the time stamp indicates that the age of the position report is greater than 30 seconds,

- if the time stamp of the position report is identical or older than the current report being used for that unit (identified by MMSI), OR

- if M.1371-1 section 4.6.1 applies (“Repeat Indicator” equals 3).

Table 46.1: Condition Table governing Retransmission - contains each type of AIS VDL Message

| Message | Name | Comm. state | Instructions General: increment repeat indicator (1371-1, 4.6.1) | Subject to message delay rules (time critical) |
| --- | --- | --- | --- | --- |
| 1 | Class A position report | SOTDMA | Table 25 | yes |
| 2 | Class A position report | SOTDMA | Table 25 | yes |
| 3 | Class A position report | ITDMA | Table 26 | yes |
| 4 | Base Station Report | SOTDMA | (none, not repeated) |  |
| 5 | Static and Voyage related data | - | nil | no |
| 6 | Binary Addressed message | - | nil | yes |
| 7 | Binary acknowledgement | - | nil | yes |
| 8 | Binary broadcast message | - | nil | no |
| 9 | SAR position | ITDMA/SOTDMA | (none, not repeated) |  |
| 10 | UTC/date inquiry | - | (none, not repeated) |  |
| 11 | UTC/date response | SOTDMA | (none, not repeated) |  |
| 12 | Addressed safety related message | - | nil | yes |
| 13 | Safety related acknowledgement | - | nil | yes |
| 14 | Safety related broadcast message | - | nil | no |
| 15 | Interrogation | - | Only repeat interrogations with slot offset zero (0) DO NOT repeat interrogation containing slot offset | yes |
| 16 | Assigned mode command | - | Only repeat if assigning reporting rate DO NOT repeat if assigning slot reporting increment | no |
| 17 | DGNSS broadcast message | - | (None, not repeated) |  |
| 18 | Standard Class B equipment position report | ITDMA/SOTDMA | Table 25 for SOTDMA Table 26 for ITDMA | yes |
| 19 | Extended Class B equipment position report | - | nil | yes |
| 20 | Data link management message | - | (None, not repeated) |  |
| 21 | Aids-to-navigation report | - | nil | no |
| 22 | Channel management | - | --- | no |

Part XI - Guidelines for Secondary Synchronisation by an AIS Service (new Annex 10)

Part XII - Guidelines for Channel Management by an AIS Service (new Annex 17)

Part XIII - Guidelines for the use of DSC-Capabilities with regard to the AIS (new Annex 17)

The following guidelines apply when there are DSC capabilities installed ashore to influence the behaviour of the mobile AIS stations. These guidelines are currently based on Class A specifications, assuming that all other mobile AIS stations (Class B, SAR, AtoN AIS stations) comply to the same behaviour rules provided that they have a DSC component installed. In principle, there are three different way, a DSC capability can be provided from shore:

- the DSC capability is internal to an AIS base station (as an option

- the DSC capability is co-located with one or more AIS base station(s) in an AIS shore station

- the DSC capability is provided from somewhere else (external to the AIS shore station)

## Cmd 101 - 'Switch to VHF-Channel ‘

When receiving this command the mobile AIS station can only switch the DSC TX frequency not the RX as the receiver is fixed at ch 70. If the channel no is 87 or 88 the 'TeleCommand'  is ignored and NO ack is generated for this 'TeleCommand'. Because ITU 825 8.1.2 talks about “all subsequent VTS DSC communications” no timeout is required for DSC-transmissions and Class A AIS will stay on the commanded DSC channel.

There is a risk of ship’s leaving the VTS area and not switching back channels. Shore stations should take that into account and start every communication with a 101 command!

## Cmd 112 - 'Acknowledge message'

Cmd 112 - ‘Acknowledge message’ is used only as an 'are you awake' communications check from the VTS to the shore station. The mobile AIS station will respond by returning a ‘110’ telecommand “message acknowledged”. Symbol 112 should only be used for requesting an acknowledgement (for test purpose; see ITU 825 table 4).

## ExpCmd 104,00 - 'Frequency channel'

This command is supposed to change the DSC 'VHF-working channel'; it does not affect TDMA channels. It should be used by shore stations like symbol 101 (switch to VHF-DSC channel) but with the additional information that can be given in the 4-digit channel number (use of a duplex channel in simplex mode; 12.5 kHz channel spacing is NOT used with DSC). For setting a standard channel symbol 101 should be used, for special channels 104+00.

Shore station are recommended not to request for 104,00  (use as a command only without acknowledgement).

## ExpCmd 104,01 -  'Transmitter Power Level'

This command will affect AIS and DSC transmissions of Class A AIS. Only the values 2 (2 watts) and 12 (12.5 watts) are used. (see Technical clarifications of recommendation ITU-R M.1371-1 (Edition 1.1) dated 01.12.2001, 2.64). The power setting will follow the algorithm described in the clarifications 2.54.1 : power setting is only effective within a regional area. ITU 825 leaves it open to use it as request also; it is recommended to use it as a command only.

## ExpCmd 104 channel management for TDMA

ExpCmds 104,09 104,10 104,11 include a 'High RF interference environment' flag.

In the mobile AIS station there is no requirement to support for reducing the Rx-sensitivity or similar  (this Rx-Flag is NOT included in AIS VDL-msg 22).

The Class A AIS will ignore this information. It is not recommended for Shore stations to use the 'High RF interference environment' flag.

## General DSC-message recommendations

### Commands/Requests

It is possible for a DSC-message to contain both 'commands' and 'requests' in the same message. One example is 101 103 : send the response (position report) on request 103  on another channel (command 101). A message containing more than 4 'TeleCommands' will be completely ignored by a Class-A AIS and no response is sent.

### Acknowledgements

Acknowledgements transmitted by Class-A AIS are composed as follows:

#### Command ack

The acknowledgement should include the received command information (together with EOS=122) to enable the VTS centre to check the correct transmission and reception of the information.

#### Request ack

Requests are acknowledged using the appropriate response messages together with EOS=122

The following symbols are request/response pairs:

Table 49.1

| Request | ack/response | Content |
| --- | --- | --- |
| 101 xx | 101 xx | Channel switching |
| 102 xx | 100 xx xx xx ... | Position |
| 103 | 100 xx xx xx ... | Position |
| 104 xx | 104 xx | Expanded sequence |
| 108 | 124 xx xx | Length of ship |
| 109 | 119 xx xx | Course |
| 111 | 115 xx xx xx ... | Name/identification |
| 112 | 110 | Acknowledgement test |
| 116 | 120 xx xx | Speed |

## Channel Management via DSC Commands

DSC - Channel management commands to a mobile AIS are composed by a shore station as follows:

Using Individual calls:

- Symbol 09 - Single channel ChA = TX+RX

- Symbol 09, 10 - Dual channel, ChA = TX+RX, ChB = TX+RX

- Symbol 09, 11 - Dual channel, ChA = TX+RX, ChB = RX only

- Symbol 09, 12, 13 - Single channel ChA = TX+RX

- Symbol 09, 10, 12, 13 - Dual channel, ChA = TX+RX, ChB = TX+RX

- Symbol 09, 11, 12, 13 - Dual channel, ChA = TX+RX, ChB = RX only

Using Area addressed or all ship calls:

- Symbol 09, 12, 13 - Single channel ChA = TX+RX

- Symbol 09, 10, 12, 13 - Dual channel, ChA = TX+RX, ChB = TX+RX

- Symbol 09, 11, 12, 13 - Dual channel, ChA = TX+RX, ChB = RX only

Channel management commands can be transmitted as all ship, area or individual calls. The addressing mode is defined by the format-specifier and independent of the message data, e.g. the symbols 104+12 or 104+13 (The symbols 12 and 13 are not used for addressing!!!).

If the symbols 12 and 13 are included in the message a regional area shall be added to the Regional area table of the AIS transponder that is handled as described in Annex 2,4.1 of ITU-R M.1371-1.

Shore stations shall transmit all corresponding information with regard to one region in one DSC command (see 61993-2, 7.4.1 and clarifications 2.54.1).  Thus it is not possible to transmit a channel management command using 01,09,10,12,13 in one call to specify an area incl. Channels A+B and power setting without violating the rules. Power setting, if deemed necessary, should be done by TDMA means (msg 22 addressed) afterwards.

In individual calls the symbols 12 and 13 may be omitted by the shore station (1371 A3/5.). In this case the Class A AIS will switch directly to the channels given by symbol 09, 10 or 11 if and as long as the ship is inside a region formerly defined (see clarification on 1371, 2.54.1).

This does mean for direct channel switching:

- an AIS will NOT take action on a VDL msg22 or DSC 104-09,10,11  command if ship’s position is outside defined regions

- AIS will switch back to default channels if ship leaves the defined region in which the switching command has been received.

Shore stations should be aware of the problem of how to get a ship on a certain channel which enters a region without recognising it because it has no position (maybe by VHF voice communication).

## DSC / TDMA priority

1371, annex 3/2.1 says that DSC operations shall not impair TDMA operations. That does include TDMA TX and RX operations! DSC responses of a Class A AIS thus are only done if the sufficient number of subsequent slots (about 20) is not allocated on either channel A or B   (see also clarification 2.14.1 on ITU 1371 3.3.1.2)

RATDMA transmissions falling in the DSC call time window will be suppressed in any case and cannot be received by mobile AIS.

Base stations are recommended not to request acknowledgements on commands (EOS=127 to be used) and should be aware of the fact that mobile AIS might not be able to respond on a DSC call even if commanded so due to TDMA traffic.

## All ship’s calls

There is a requirement for the Class A AIS not to respond or acknowledge on format 116 calls even if response is requested by EOS=117 (IEC 61993-2 ch. 8). Shore stations should not use all ship calls together with an acknowledgement request.

## VTS calls only

An AIS shall not respond or take action on any calls with category other than 103 (VTS call) in order not to disturb operation of GMDSS equipment. (ITU 1371 annex 3, 1.1). An AIS shore station should not transmit DSC calls of other category than 103.

## Channel free check

Shore stations should be aware of the fact that mobile Class A AIS will do a channel free check before DSC transmissions on channel 70 only (not if commanded to respond on other channels).

Part XIV - Capacity Balance of the AIS VDL by AIS Service(s) (new Annex 18)

Annex A: Services Operating on the VDL simultaneously

Prioriti-

ring in accor-

dance with M.1371-1

Prioriti-

ring in accor-

dance with M.1371-1

void

All mobile AIS stations in radio coverage area

All fixed AIS stations

in radio coverage area

-	Regular base station reports

-	Synchronisation support (additional) by base stations

-	Assigned mode control

-	Frequency mode control

-	Safety related messages

-	Interrogations to mobiles

-	VDL-level acknowledgements

-	FATDMA reservations

-	Application specific messages

-	DGNSS corrections

-	Simplex repeated messages

-	Responses to interrogations from mobiles

-	Re-transmit of addressed messages

-	Scheduled ship position reports (all Classes)

-	Static and voyage Related Data Report (scheduled)

-	Safety Related Messages

-	A-to-N Reports (scheduled)

-	SAR Aircraft reports (scheduled)

-	Extra reports (all kind of mobiles) due to assignment

-	Responses to interrogation

-	VDL-level acknowledgements

-	Application specific messages

-	Synchronisation support (additional) by mobiles

-	Interrogations from mobiles

-	Re-transmit of addressed messages

capacity

loss due

to garbling and

interference

4.500 slots/

minute

(theoretical

maximum)

void

capacity

loss due

to garbling and

interference

# Annex B: Interfacing the DGNSS Correction Service to The AIS Base Station

#### B.1 	Functional

Figure B1 shows a functional flow chart about the interfacing between the GNSS correction data source and the AIS mobile station connected to an (D)GNSS receiver.

Figure B1:  Functional flow chart

Binary

RTCM

DGNSS correction source

Msg #17

RTCM

Msg #17

Internal

(D)GNSS

receiver

External

(D)GNSS

receiver

AIS shore station

AIS mobile station

PI

ASCII

(NMEA)

RTCM

Depending on the used DGNSS correction data source (as described in Chapter 2) it can be advantageous to connect the correction data via a binary or via an ASCII interface to the AIS base station.

For solutions where the RTCM output interface of an external DGNSS reference station or a MF beacon receiver is used, it is beneficial to connect the output port directly to a binary RTCM input port of the AIS base station.

The interface is described in detail in the ITU-R M.823 Data Interface Specification (Annex 2). In this case the conversion to VDL Message 17 is done inside the base station, so that no further adaptation between the DGNSS data source and the base station is needed.

In the case that an external DGNSS source is used, that is connected via a network (LAN/WAN), it could be useful to handle the correction data in ASCII format and feed the data via the presentation interface into the AIS base station.

The conversion of the correction data into an appropriate VDL Message 17 is done outside the AIS base station. The input to the presentation interface occurs as IEC 61162-1 VDM sentence with the encapsulated VDL Message 17 data.

B.2 Processing the Received DGNSS Correction Data

Figure B2 shows a flow chart that describes the logical flowing of decoding and encoding the RTCM data stream and the appropriate AIS Message #17.

Figure B2: Logical flow chart

GNSS correction data

Receive RTCM

(Header, MT1, MT3)

D1  D2   D3    ....                       ...         D23   D24  D25   D26  D27  D28  D29   D30

Coded information bits

Parity bits

Check parity and decode received bit stream using algorithm described in

ICD-GPS-200

d1  d2    d3    ....                          ...           d23   d24   d25    d26   d27  d28   d29    d30

Decoded information bits

Parity bits

Cut

parity

d25-d30

d1 .... d8            d9  .. d14                        d15  ..  d24                        d25 ..   d30

Use information bits

d9-d24 to create

AIS MT #17

Cut

Preamble

d1-d81)

Example for first word of RTCM header

Cut parity and preamble

create AIS Msg #17 using table 5 and transmit

Example for first word of RTCM header

Receive AIS Msg #17 and reconstruct RTCM bit stream using table 5

d1 .... d8            d9  .. d14                     d15  ..  d24

Use information bits

d9-d24 to create

RTCM

paste

Preamble

d1-d8

Encode RTCM bitstream and calculate new Parity 2)

using algorithm described in ICD-GPS-200 and output to data to a connected (D)GNSS receiver

D'1  D'2  D'3    ....                         ...          D'23  D'24 D'25D'26 D'27 D'28 D'29 D'30

New coded information bits

New calculated Parity bits

- The preamble is only cut in the first word of the RTCM header. Normally d1-d24 are used to create 
AIS Msg #17

- Normally bits D29 and D30 of the previous word is used to calculate D'1 to D'30 of the new RTCM data word. For initialisation D29 and D30 can set to '0'.

Table B1 provides information how to create AIS Msg #17.

Table B1

| AIS Msg #17 |  | Data source |
| --- | --- | --- |
| Static data |  |  |
| Parameter | Number of Bits |  |
| Message ID | 6 | Set by AIS shore station |
| Repeat Indicator | 2 | Set by AIS shore station |
| Source ID | 30 | Set by AIS shore station |
| Spare | 2 | - |
| Longitude | 18 | Set by AIS shore station (refer to remark below) |
| Latitude | 17 | Set by AIS shore station (refer to remark below) |
| Spare | 5 | - |
| Differential Correction data (header) |  |  |
| Parameter | Number of Bits |  |
| Message Type | 6 | RTCM Header, Word1, d9-d14 |
| Station ID | 10 | RTCM Header, Word1, d15-d24 |
| Z count | 13 | RTCM Header, Word2, d1-d13 |
| Sequence number | 3 | RTCM Header, Word2, d14-d16 |
| N | 5 | RTCM Header, Word2, (d17-d21) - 2 1) |
| Health | 3 | RTCM Header, Word2, d22-d24 |
| Differential Correction data (example for the 1st satellite) |  |  |
| Scale factor | 1 | RTCM Header, Word3, d1 |
| UDRE | 2 | RTCM Header, Word3, d2-d3 |
| Satellite ID | 5 | RTCM Header, Word3, d4-d8 |
| PRC | 16 | RTCM Header, Word3, d9-d24 |
| RRC | 8 | RTCM Header, Word4, d1-d8 |
| Issue of data | 8 | RTCM Header, Word4, d9-d16 |
| Fill bits | 8 (or 16) 2) | RTCM Header, Word4, d17-d24 |

1)The frame length in RTCM header is two more than the number of words (N) following the header

2)16 fill bits are used for RTCM word (N+2) if  number of satellites Ni = 2,5,8 or 11

Remark

One of the elements used in AIS Msg #17 is the reference station position.

Within the IALA DGNSS MF Beacon System the reference station position will enable a beacon receiver to select the nearest, healthy reference station when in automatic mode.

To select the nearest, healthy station the beacon receiver need to be connected to a GNSS receiver to get the current position.  An algorithm is implemented in the beacon receiver that calculates the distance between the receiver and the reference station and takes care of the health status and signal quality of the received correction.

If an AIS mobile station is able to receive more than one source of DGNSS corrections via Msg 17a similar algorithm is needed within the AIS mobile station.

Annex A to this paper (based on IEC 61108-4) describes such an algorithm.

B.3 Electrical Interfacing

Table B2 shows the electrical requirements.

Table B2

| Interface | Electrical protocol |
| --- | --- |
| AIS shore station (see figure 8) (RTCM input) | RS 232 RS 422 LAN, TCP/IP |
| AIS mobile station (see figure 8) (RTCM output) | RS 422 RS 232 |

Annex 2 to this paper provide a “Data interface guidance”. This Annex is part of IEC 61108-4 and describes information about the RTCM digital data enters and exits through an asynchronous full duplex serial input/output port.

#### B.4 DGNSS correction data administration within the AIS base station

Figure B3 shows the data flow from the DGNSS data source up to the transmission within the AIS base station.

Figure B3 - Functional flow chart

DGNSS correction data source

- WAN/LAN

- RS 232

- HF

(e.g. MF radio beacon)

Memory

(LIFO)

RTCM

and/or

PI

AIS base station

The reference receiver generates corrections with a high update rate. Normally a complete set of corrections is available each second.

Dependent on the used communication link the data will be available at the AIS base station with a latency of 1 to 6 seconds. As described in chapter 3 the AIS base station will transmit the AIS VDL Message 17 within 3 time slots with a repetition interval of  5 seconds. To fulfil the demands of integrity and accuracy it is important that the AIS base station will always use the most topical set of correction data that is available at that time.

ANNEX 1 to Annex B

Implementation using a Concurrent Process

Station Status Update

Navigation Process

ANNEX 2 to Annex B:

DATA INTERFACE GUIDANCE

RTCM Input/Output in Binary Format

ITU-R M.823 Data Interface Specification: The GNSS equipment shall be designed in such a way that the ITU-R M.823 digital data link information enters and exits through an asynchronous full duplex serial input / output port.

The American National Standards Institute (ANSI) X3.16(4) and X3.15(5) standards for eight-bit character structure shall govern the rules for serial data transfers. Note that the use of all eight bits in the transfer of serial data precludes the use of 7 bit parity formats. The recommended protocol being 8 bits, no parity, 1 stop bit. The serial data rate shall be selectable at least over the 1200 to 9600 baud range (1200, 2400, 4800, or 9600 baud).

Important Interface Rules:  Although the data is packaged in 8-bit bytes, the interpretation of what each of the 8 bits means is dictated by a combination of what is presented in ITU-R M.823 and the rules that follow.

Byte Format Rule:  A standard 8-bit byte is described as the “8-Bit Environment” in ANSI X3.16(4). This standard assigns the order of the start, stop, and eight data bits: the first data bit transmitted is designated “a8” and the last is “a1”; bit “a1” is designated the least significant bit. This is a source of problems and is discussed in the next section ( see - Most Significant Bit First Rule ). All equipment shall support the use of the “6 of 8” format ( data bits a1 through a6) to transfer the information contained in ITU-R M.823. As an indication that bits a1 through a6 are “message information”, bit a7 shall be set “marking” and a8 shall be set “spacing”. The appropriate mark and space signalling conditions are discussed in ITU-T V.28 (RS232) and ITU-T V.11 (RS422).

Most Significant Bit First Rule:  The Data Link binary information shall always be passed in the order it appears in ITU-R M.823. This is known as most significant bit first. Unfortunately, the ANSI X3.15-1976 standard states that the least significant bit is first. Almost all integrated circuits designed for serial communications follow this convention. The use of X3.15 standard Universal Asynchronous Receivers and Transmitters (UARTs) introduces the need for a “byte roll” prior to leaving the reference station equipment and then again just after entering the GNSS user equipment. The following is from ANSI standard X 3.15-1976(5): “The bit sequence for an ASCII nomenclature (6) b1 through b7 in ascending  (consecutive ) order, or in terms of the 8-bit nomenclature (7) a1 through a8 in ascending  (consecutive ) order.”

The “roll” process is performed on each byte prior to transmission. Rolling means that bits a1 and a6, a2 and a5, a3 and a4 are swapped. This same process is repeated after user equipment accepts each byte.

Bit Slip Rule:  In a typical installation the communications receiver or modem will assemble the received bits into 8-bit bytes. No specific (except bit synchronisation) byte or “word” synchronisation should be assumed. The user equipment shall be required to recover the message synchronisation just as it is responsible for recovering the synchronisation of the satellite navigation data. This simply means that the user equipment designer should not assume there will be any consistent relationship between the word boundaries of this standard’s 30 bits words and the communications channel 8 bit bytes.

# Annex C: Calculations with regard to DSC channel management command

Structure of DSC message (ITU-R M.825-3)

# Message header-tail all ships [addressed / VTS area] message

Dot pattern			  20 bit			  2 symbols

Phasing Sequence		140 bit			14 symbols

Format specifier	  	40 bit			  4 symbols

Address		  	  0 bit [100 / 220 bit]	  0 [10 / 22] symbols]

Category			  20 bit			  2 symbols

Self ID				100 bit			10 symbols

Message content

EOS				  40 bit			  4 symbols

ECC				  20 bit			  2 symbols

Sum (header, tail)		380 bit [480 / 600 bit]	38 [48 / 60] symbols

# Message content

Power level			  60 bit		  6 symbols

primary channel		100 bit		10 symbols

secondary channel		100 bit		10 symbols

Rx mode			100 bit		10 symbols

NE corner of region		160 bit		16 symbols

SW corner of region		160 bit		16 symbols

-----------------------------------------------------------------------------------------------------

max 4 symbols (ch 1+2)		520 bit		52 symbols

# Time for transmission

channels only (all ships)		(380bit+520bit)/1200 bit/s = 0.75 s

power only (all ships)			(380bit+380bit)/1200 bit/s = 0.633 s

channels only (addressed)		(480bit+520bit)/1200 bit/s = 0.833 s

power only (addressed)		(480bit+380bit)/1200 bit/s = 0.716 s

channels only (VTS area call)	(600bit+520bit)/1200 bit/s = 0.933 s

power only (VTS area call)		(600bit+380bit)/1200 bit/s = 0.816 s

Transmitter settling time			= 0.001 s	[20/25ms]

Transmitter release time 			= 0.001 s	[20/25ms]

Note:	The transmitter settling and release times above are as per the AIS transmitter requirements however, could be adapted to suit the DSC transmitter requirement if optional and additional.

[20/25ms]

# Attachment A: Algorithm to calculate the FATDMA geographical grid scheme on a global basis

In order to achieve a well defined global geographical distribution an algorithm is used for calculating the position of each individual cell in the geographical grid. Upon input of any geographical position the algorithm will render the number of the FATDMA scheme valid for that geographical position, i. e. the number of the FATDMA scheme used in that cell.

The reference point of the geographical cell would be the [top left] corner for cells on the northern hemisphere, and the [bottom left] corner for cells on the southern hemisphere.

LAT =  (WGS 84 latitude in decimal degrees)

LON =  (WGS 84 latitude in decimal degrees)

// Note: The grid consists of a repeated pattern of 6 x 6 cells, each cell 30 x 30 nautical miles in size.

SingleCellSizeInNauticalMiles = 30;

NoOfSingleCellsAlongOneSideOfMasterCell = 6;

// Note: The repeatable pattern of 6 x 6 cells is denoted a ‘master cell’

MasterCellSizeInNauticalMiles =

SingleCellSizeInNauticalMiles * NoOfSingleCellsAlongOneSideOfMasterCell;
// Note: At Equator, one degree longitude approximately resembles 60 nm i.e. one minute longitude resembles one nautical mile

NoOfMasterCellsAroundEquator = 360 * 60  / MasterCellSizeInNauticalMiles;

//Note: At any latitude, one minute latitude resembles one nautical mile

MasterCellSizeInDegreesLatitude = MasterCellSizeInNauticalMiles / 60.0d;

//Note: abs(x) denotes the absolute value of x; int(x) denotes truncated integer value of x

MasterCellRowNo = int( abs(LAT) / MasterCellSizeInDegreesLatitude );

MasterCellMeanLatitude = (MasterCellRowNo+0.5) * MasterCellSizeInDegreesLatitude

//Note: cos(x) denotes cosine of x in radians

NoOfMasterCellsAroundMasterCellRow = 
int( NoOfMasterCellsAroundEquator * cos( 2 * π * MasterCellMeanLatitude / 360 ) );

SingleCellWidthInDegrees = 
360 / (NoOfSingleCellsAlongOneSideOfMasterCell * NoOfMasterCellsAroundMasterCellRow);

RowNumberInsideMasterCell = 
int ( abs(LAT) * 60 / SingleCellSizeInNauticalMiles ) - NoOfSingleCellsAlongOneSideOfMasterCell * MasterCellRowNo;

// Note: first Southern Hemisphere is assumed, then compensate if Northern Hemisphere

if (LAT>0)

{ RowNumberInsideMasterCell =

NoOfSingleCellsAlongOneSideOfMasterCell - RowNumberInsideMasterCell - 1;

};

ColumnNumberInsideMasterCell = int (abs(LON) / SingleCellWidthInDegrees);

while (ColumnNumberInsideMasterCell > (NoOfSingleCellsAlongOneSideOfMasterCell – 1 ) )

{ColumnNumberInsideMasterCell = 
ColumnNumberInsideMasterCell – NoOfSingleCellsAlongOneSideOfMasterCell

};

// Note: first positive longitude is assumed, compensate if negative longitude (west of Greenwich)

if (LON<0)

{ ColumnNumberInsideMasterCell =
(NoOfSingleCellsAlongOneSideOfMasterCell - 1) - ColumnNumberInsideMasterCell

};

ResultingCellNumber  = 
RowNumberInsideMasterCell * NoOfSingleCellsAlongOneSideOfMasterCell + ColumnNumberInsideMasterCell + 1

# Attachment B: Absolute slot number and usage category designation for the FATDMA schemes

The following table shows the absolute slot numbers, working channel usage, and usage category designations for the FATDMA schemes.

|  | Channel A |  |  | Channel B |  |  | Usage Configuration | Base station report starting slot (CBM sentence) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cell Reference |  |  |  |  |  |  |  |  |
|  | starting slot | reservation block size | Increment 1: semaphore mode 2: non – semaphore mode | starting slot 1: semaphore mode 2: non – semaphore mode | reservation block size | Increment 1: semaphore mode 2: non – semaphore mode |  |  |
| 1-I | 0 | 1 | 2501 or 7502 | 1251 or 3752 | 1 | 2501 or 7502 | Base Station report* | 0 |
|  | 312 | 1 | 0 or 1125 | 877 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 602 | 0, 1, 2 or 3 | 0 or 1125 | 12 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 1-II | 125 | 1 | 2501 or 7502 | 01 or 5002 | 1 | 2501 or 7502 | Base Station report* | 125 |
|  | 877 | 1 | 0 or 1125 | 312 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 12 | 0, 1, 2 or 3 | 0 or 1125 | 602 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 2-I | 1 | 1 | 2501 or 7502 | 1261 or 3762 | 1 | 2501 or 7502 | Base Station report* | 1 |
|  | 313 | 1 | 0 or 1125 | 878 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 612 | 0, 1, 2 or 3 | 0 or 1125 | 27 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 2-II | 126 | 1 | 2501 or 7502 | 11 or 5012 | 1 | 2501 or 7502 | Base Station report* | 126 |
|  | 878 | 1 | 0 or 1125 | 313 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 27 | 0, 1, 2 or 3 | 0 or 1125 | 612 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 3-I | 10 | 1 | 2501 or 7502 | 1351 or 3852 | 1 | 2501 or 7502 | Base Station report* | 10 |
|  | 314 | 1 | 0 or 1125 | 879 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 619 | 0, 1, 2 or 3 | 0 or 1125 | 37 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 3-II | 135 | 1 | 2501 or 7502 | 101 or 5102 | 1 | 2501 or 7502 | Base Station report* | 135 |
|  | 879 | 1 | 0 or 1125 | 314 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 37 | 0, 1, 2 or 3 | 0 or 1125 | 619 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 4-I | 11 | 1 | 2501 or 7502 | 1361 or 3862 | 1 | 2501 or 7502 | Base Station report* | 11 |
|  | 627 | 1 | 0 or 1125 | 52 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 327 | 0, 1, 2 or 3 | 0 or 1125 | 902 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 4-II | 136 | 1 | 2501 or 7502 | 111 or 5112 | 1 | 2501 or 7502 | Base Station report* | 136 |
|  | 52 | 1 | 0 or 1125 | 627 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 902 | 0, 1, 2 or 3 | 0 or 1125 | 327 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 5-I | 19 | 1 | 2501 or 7502 | 1441 or 3942 | 1 | 2501 or 7502 | Base Station report* | 19 |
|  | 628 | 1 | 0 or 1125 | 53 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 337 | 0, 1, 2 or 3 | 0 or 1125 | 912 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 5-II | 144 | 1 | 2501 or 7502 | 191 or 5192 | 1 | 2501 or 7502 | Base Station report* | 144 |
|  | 53 | 1 | 0 or 1125 | 628 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 912 | 0, 1, 2 or 3 | 0 or 1125 | 337 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 6-I | 20 | 1 | 2501 or 7502 | 1451 or 3952 | 1 | 2501 or 7502 | Base Station report* | 20 |
|  | 629 | 1 | 0 or 1125 | 54 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 352 | 0, 1, 2 or 3 | 0 or 1125 | 887 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 6-II | 145 | 1 | 2501 or 7502 | 201 or 5202 | 1 | 2501 or 7502 | Base Station report* | 145 |
|  | 54 | 1 | 0 or 1125 | 629 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 887 | 0, 1, 2 or 3 | 0 or 1125 | 352 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 7-I | 21 | 1 | 2501 or 7502 | 1461 or 3962 | 1 | 2501 or 7502 | Base Station report* | 21 |
|  | 362 | 1 | 0 or 1125 | 927 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 637 | 0, 1, 2 or 3 | 0 or 1125 | 62 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 7-II | 146 | 1 | 2501 or 7502 | 211 or 5212 | 1 | 2501 or 7502 | Base Station report* | 146 |
|  | 927 | 1 | 0 or 1125 | 362 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 62 | 0, 1, 2 or 3 | 0 or 1125 | 637 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 8-I | 25 | 1 | 2501 or 7502 | 1501 or 4002 | 1 | 2501 or 7502 | Base Station report* | 25 |
|  | 363 | 1 | 0 or 1125 | 928 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 652 | 0, 1, 2 or 3 | 0 or 1125 | 77 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 8-II | 150 | 1 | 2501 or 7502 | 251 or 5252 | 1 | 2501 or 7502 | Base Station report* | 150 |
|  | 928 | 1 | 0 or 1125 | 363 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 77 | 0, 1, 2 or 3 | 0 or 1125 | 652 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |

| Default FATDMA schemes / 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Channel A |  |  | Channel B |  |  | Usage Configuration | Base station report starting slot (CBM sentence) |  |
| Cell Reference |  |  |  |  |  |  |  |  |  |
|  | starting slot | reservation block size | Increment 1: semaphore mode 2: non – semaphore mode | starting slot 1: semaphore mode 2: non – semaphore mode | reservation block size | Increment 1: semaphore mode 2: non – semaphore mode |  |  |  |
| 9-I | 26 | 1 | 2501 or 7502 | 1511 or 4012 | 1 | 2501 or 7502 | Base Station report* | 26 |  |
|  | 364 | 1 | 0 or 1125 | 929 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 662 | 0, 1, 2 or 3 | 0 or 1125 | 87 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 9-II | 151 | 1 | 2501 or 7502 | 261 or 5262 | 1 | 2501 or 7502 | Base Station report* | 151 |  |
|  | 929 | 1 | 0 or 1125 | 364 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 87 | 0, 1, 2 or 3 | 0 or 1125 | 662 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 10-I | 35 | 1 | 2501 or 7502 | 1601 or 4102 | 1 | 2501 or 7502 | Base Station report* | 35 |  |
|  | 677 | 1 | 0 or 1125 | 102 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 369 | 0, 1, 2 or 3 | 0 or 1125 | 937 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 10-II | 160 | 1 | 2501 or 7502 | 351 or 5352 | 1 | 2501 or 7502 | Base Station report* | 160 |  |
|  | 102 | 1 | 0 or 1125 | 677 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 937 | 0, 1, 2 or 3 | 0 or 1125 | 369 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 11-I | 36 | 1 | 2501 or 7502 | 1611 or 4112 | 1 | 2501 or 7502 | Base Station report* | 36 |  |
|  | 678 | 1 | 0 or 1125 | 103 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 377 | 0, 1, 2 or 3 | 0 or 1125 | 952 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 11-II | 161 | 1 | 2501 or 7502 | 361 or 5362 | 1 | 2501 or 7502 | Base Station report* | 161 |  |
|  | 103 | 1 | 0 or 1125 | 678 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 952 | 0, 1, 2 or 3 | 0 or 1125 | 377 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 12-I | 44 | 1 | 2501 or 7502 | 1691 or 4192 | 1 | 2501 or 7502 | Base Station report* | 44 |  |
|  | 679 | 1 | 0 or 1125 | 104 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 387 | 0, 1, 2 or 3 | 0 or 1125 | 962 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 12-II | 169 | 1 | 2501 or 7502 | 441 or 5442 | 1 | 2501 or 7502 | Base Station report* | 169 |  |
|  | 104 | 1 | 0 or 1125 | 679 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 962 | 0, 1, 2 or 3 | 0 or 1125 | 387 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 13-I | 45 | 1 | 2501 or 7502 | 1701 or 4202 | 1 | 2501 or 7502 | Base Station report* | 45 |  |
|  | 402 | 1 | 0 or 1125 | 977 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 687 | 0, 1, 2 or 3 | 0 or 1125 | 112 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 13-II | 170 | 1 | 2501 or 7502 | 451 or 5452 | 1 | 2501 or 7502 | Base Station report* | 170 |  |
|  | 977 | 1 | 0 or 1125 | 402 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 112 | 0, 1, 2 or 3 | 0 or 1125 | 687 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 14-I | 46 | 1 | 2501 or 7502 | 1711 or 4212 | 1 | 2501 or 7502 | Base Station report* | 46 |  |
|  | 403 | 1 | 0 or 1125 | 978 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 702 | 0, 1, 2 or 3 | 0 or 1125 | 119 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 14-II | 171 | 1 | 2501 or 7502 | 461 or 5462 | 1 | 2501 or 7502 | Base Station report* | 171 |  |
|  | 978 | 1 | 0 or 1125 | 403 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 119 | 0, 1, 2 or 3 | 0 or 1125 | 702 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 15-I | 50 | 1 | 2501 or 7502 | 1751 or 4252 | 1 | 2501 or 7502 | Base Station report* | 50 |  |
|  | 404 | 1 | 0 or 1125 | 979 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 712 | 0, 1, 2 or 3 | 0 or 1125 | 127 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 15-II | 175 | 1 | 2501 or 7502 | 501 or 5502 | 1 | 2501 or 7502 | Base Station report* | 175 |  |
|  | 979 | 1 | 0 or 1125 | 404 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 127 | 0, 1, 2 or 3 | 0 or 1125 | 712 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 16-I | 51 | 1 | 2501 or 7502 | 1761 or 4262 | 1 | 2501 or 7502 | Base Station report* | 51 |  |
|  | 727 | 1 | 0 or 1125 | 137 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 412 | 0, 1, 2 or 3 | 0 or 1125 | 987 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 16-II | 176 | 1 | 2501 or 7502 | 511 or 5512 | 1 | 2501 or 7502 | Base Station report* | 176 |  |
|  | 137 | 1 | 0 or 1125 | 727 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 987 | 0, 1, 2 or 3 | 0 or 1125 | 412 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |

| Default FATDMA schemes / 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Channel A |  |  | Channel B |  |  | Usage Configuration | Base station report starting slot (CBM sentence) |  |
| Cell Reference |  |  |  |  |  |  |  |  |  |
|  | starting slot | reservation block size | Increment 1: semaphore mode 2: non – semaphore mode | starting slot 1: semaphore mode 2: non – semaphore mode | reservation block size | Increment 1: semaphore mode 2: non – semaphore mode |  |  |  |
| 17-I | 60 | 1 | 2501 or 7502 | 1851 or 4352 | 1 | 2501 or 7502 | Base Station report* | 60 |  |
|  | 728 | 1 | 0 or 1125 | 138 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 427 | 0, 1, 2 or 3 | 0 or 1125 | 994 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 17-II | 185 | 1 | 2501 or 7502 | 601 or 5602 | 1 | 2501 or 7502 | Base Station report* | 185 |  |
|  | 138 | 1 | 0 or 1125 | 728 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 994 | 0, 1, 2 or 3 | 0 or 1125 | 427 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 18-I | 61 | 1 | 2501 or 7502 | 1861 or 4362 | 1 | 2501 or 7502 | Base Station report* | 61 |  |
|  | 729 | 1 | 0 or 1125 | 139 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 452 | 0, 1, 2 or 3 | 0 or 1125 | 1002 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 18-II | 186 | 1 | 2501 or 7502 | 611 or 5612 | 1 | 2501 or 7502 | Base Station report* | 186 |  |
|  | 139 | 1 | 0 or 1125 | 729 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 1002 | 0, 1, 2 or 3 | 0 or 1125 | 452 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 19-I | 69 | 1 | 2501 or 7502 | 1941 or 4442 | 1 | 2501 or 7502 | Base Station report* | 69 |  |
|  | 462 | 1 | 0 or 1125 | 1012 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 737 | 0, 1, 2 or 3 | 0 or 1125 | 152 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 19-II | 194 | 1 | 2501 or 7502 | 691 or 5692 | 1 | 2501 or 7502 | Base Station report* | 194 |  |
|  | 1012 | 1 | 0 or 1125 | 462 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 152 | 0, 1, 2 or 3 | 0 or 1125 | 737 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 20-I | 70 | 1 | 2501 or 7502 | 1951 or 4452 | 1 | 2501 or 7502 | Base Station report* | 70 |  |
|  | 463 | 1 | 0 or 1125 | 1013 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 744 | 0, 1, 2 or 3 | 0 or 1125 | 162 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 20-II | 195 | 1 | 2501 or 7502 | 701 or 5702 | 1 | 2501 or 7502 | Base Station report* | 195 |  |
|  | 1013 | 1 | 0 or 1125 | 463 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 162 | 0, 1, 2 or 3 | 0 or 1125 | 744 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 21-I | 71 | 1 | 2501 or 7502 | 1961 or 4462 | 1 | 2501 or 7502 | Base Station report* | 71 |  |
|  | 464 | 1 | 0 or 1125 | 1014 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 752 | 0, 1, 2 or 3 | 0 or 1125 | 177 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 21-II | 196 | 1 | 2501 or 7502 | 711 or 5712 | 1 | 2501 or 7502 | Base Station report* | 196 |  |
|  | 1014 | 1 | 0 or 1125 | 464 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 177 | 0, 1, 2 or 3 | 0 or 1125 | 752 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 22-I | 75 | 1 | 2501 or 7502 | 2001 or 4502 | 1 | 2501 or 7502 | Base Station report* | 75 |  |
|  | 762 | 1 | 0 or 1125 | 187 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 437 | 0, 1, 2 or 3 | 0 or 1125 | 1027 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 22-II | 200 | 1 | 2501 or 7502 | 751 or 5752 | 1 | 2501 or 7502 | Base Station report* | 200 |  |
|  | 187 | 1 | 0 or 1125 | 762 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 1027 | 0, 1, 2 or 3 | 0 or 1125 | 437 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 23-I | 76 | 1 | 2501 or 7502 | 2011 or 4512 | 1 | 2501 or 7502 | Base Station report* | 76 |  |
|  | 763 | 1 | 0 or 1125 | 188 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 477 | 0, 1, 2 or 3 | 0 or 1125 | 1037 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 23-II | 201 | 1 | 2501 or 7502 | 761 or 5762 | 1 | 2501 or 7502 | Base Station report* | 201 |  |
|  | 188 | 1 | 0 or 1125 | 763 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 1037 | 0, 1, 2 or 3 | 0 or 1125 | 477 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 24-I | 85 | 1 | 2501 or 7502 | 2101 or 4602 | 1 | 2501 or 7502 | Base Station report* | 85 |  |
|  | 764 | 1 | 0 or 1125 | 189 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 487 | 0, 1, 2 or 3 | 0 or 1125 | 1052 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 24-II | 210 | 1 | 2501 or 7502 | 851 or 5852 | 1 | 2501 or 7502 | Base Station report* | 210 |  |
|  | 189 | 1 | 0 or 1125 | 764 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 1052 | 0, 1, 2 or 3 | 0 or 1125 | 487 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |

| Default FATDMA schemes / 4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Channel A |  |  | Channel B |  |  | Usage Configuration | Base station report starting slot (CBM sentence) |  |
| Cell Reference |  |  |  |  |  |  |  |  |  |
|  | starting slot | reservation block size | Increment 1: semaphore mode 2: non – semaphore mode | starting slot 1: semaphore mode 2: non – semaphore mode | reservation block size | Increment 1: semaphore mode 2: non – semaphore mode |  |  |  |
| 25-I | 86 | 1 | 2501 or 7502 | 2111 or 4612 | 1 | 2501 or 7502 | Base Station report* | 86 |  |
|  | 494 | 1 | 0 or 1125 | 1062 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 777 | 0, 1, 2 or 3 | 0 or 1125 | 202 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 25-II | 211 | 1 | 2501 or 7502 | 861 or 5862 | 1 | 2501 or 7502 | Base Station report* | 211 |  |
|  | 1062 | 1 | 0 or 1125 | 494 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 202 | 0, 1, 2 or 3 | 0 or 1125 | 777 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 26-I | 94 | 1 | 2501 or 7502 | 2191 or 4692 | 1 | 2501 or 7502 | Base Station report* | 94 |  |
|  | 495 | 1 | 0 or 1125 | 1063 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 787 | 0, 1, 2 or 3 | 0 or 1125 | 212 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 26-II | 219 | 1 | 2501 or 7502 | 941 or 5942 | 1 | 2501 or 7502 | Base Station report* | 219 |  |
|  | 1063 | 1 | 0 or 1125 | 495 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 212 | 0, 1, 2 or 3 | 0 or 1125 | 787 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 27-I | 95 | 1 | 2501 or 7502 | 2201 or 4702 | 1 | 2501 or 7502 | Base Station report* | 95 |  |
|  | 496 | 1 | 0 or 1125 | 1064 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 802 | 0, 1, 2 or 3 | 0 or 1125 | 227 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 27-II | 220 | 1 | 2501 or 7502 | 951 or 5952 | 1 | 2501 or 7502 | Base Station report* | 220 |  |
|  | 1064 | 1 | 0 or 1125 | 496 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 227 | 0, 1, 2 or 3 | 0 or 1125 | 802 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 28-I | 96 | 1 | 2501 or 7502 | 2211 or 4712 | 1 | 2501 or 7502 | Base Station report* | 96 |  |
|  | 812 | 1 | 0 or 1125 | 237 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 502 | 0, 1, 2 or 3 | 0 or 1125 | 1102 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 28-II | 221 | 1 | 2501 or 7502 | 961 or 5962 | 1 | 2501 or 7502 | Base Station report* | 221 |  |
|  | 237 | 1 | 0 or 1125 | 812 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 1102 | 0, 1, 2 or 3 | 0 or 1125 | 502 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 29-I | 100 | 1 | 2501 or 7502 | 2251 or 4752 | 1 | 2501 or 7502 | Base Station report* | 100 |  |
|  | 813 | 1 | 0 or 1125 | 238 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 512 | 0, 1, 2 or 3 | 0 or 1125 | 1087 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 29-II | 225 | 1 | 2501 or 7502 | 1001 or 6002 | 1 | 2501 or 7502 | Base Station report* | 225 |  |
|  | 238 | 1 | 0 or 1125 | 813 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 1087 | 0, 1, 2 or 3 | 0 or 1125 | 512 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 30-I | 101 | 1 | 2501 or 7502 | 2261 or 4762 | 1 | 2501 or 7502 | Base Station report* | 101 |  |
|  | 814 | 1 | 0 or 1125 | 239 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 527 | 0, 1, 2 or 3 | 0 or 1125 | 1077 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 30-II | 226 | 1 | 2501 or 7502 | 1011 or 6012 | 1 | 2501 or 7502 | Base Station report* | 226 |  |
|  | 239 | 1 | 0 or 1125 | 814 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 1077 | 0, 1, 2 or 3 | 0 or 1125 | 527 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 31-I | 110 | 1 | 2501 or 7502 | 2351 or 4852 | 1 | 2501 or 7502 | Base Station report* | 110 |  |
|  | 537 | 1 | 0 or 1125 | 1113 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 827 | 0, 1, 2 or 3 | 0 or 1125 | 244 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 31-II | 235 | 1 | 2501 or 7502 | 1101 or 6102 | 1 | 2501 or 7502 | Base Station report* | 235 |  |
|  | 1113 | 1 | 0 or 1125 | 537 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 244 | 0, 1, 2 or 3 | 0 or 1125 | 827 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 32-I | 111 | 1 | 2501 or 7502 | 2361 or 4862 | 1 | 2501 or 7502 | Base Station report* | 111 |  |
|  | 538 | 1 | 0 or 1125 | 1114 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 837 | 0, 1, 2 or 3 | 0 or 1125 | 252 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 32-II | 236 | 1 | 2501 or 7502 | 1111 or 6112 | 1 | 2501 or 7502 | Base Station report* | 236 |  |
|  | 1114 | 1 | 0 or 1125 | 538 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 252 | 0, 1, 2 or 3 | 0 or 1125 | 837 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |

| Default FATDMA schemes / 5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Channel A |  |  | Channel B |  |  | Usage Configuration | Base station report starting slot (CBM sentence) |  |
| Cell Reference |  |  |  |  |  |  |  |  |  |
|  | starting slot | reservation block size | Increment 1: semaphore mode 2: non – semaphore mode | starting slot 1: semaphore mode 2: non – semaphore mode | reservation block size | Increment 1: semaphore mode 2: non – semaphore mode |  |  |  |
| 33-I | 18 | 1 | 2501 or 7502 | 1431 or 3932 | 1 | 2501 or 7502 | Base Station report* | 18 |  |
|  | 552 | 1 | 0 or 1125 | 1112 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 2 | 0, 1, 2 or 3 | 0 or 1125 | 852 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 33-II | 143 | 1 | 2501 or 7502 | 181 or 5182 | 1 | 2501 or 7502 | Base Station report* | 143 |  |
|  | 1112 | 1 or 2 | 0 or 1125 | 552 | 1 or 2 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 852 | 0, 1, 2 or 3 | 0 or 1125 | 2 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 34-I | 43 | 1 | 2501 or 7502 | 1681 or 4182 | 1 | 2501 or 7502 | Base Station report* | 43 |  |
|  | 562 | 1 | 0 or 1125 | 553 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 262 | 0, 1, 2 or 3 | 0 or 1125 | 862 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 34-II | 168 | 1 | 2501 or 7502 | 431 or 5432 | 1 | 2501 or 7502 | Base Station report* | 168 |  |
|  | 553 | 1 | 0 or 1125 | 562 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 862 | 0, 1, 2 or 3 | 0 or 1125 | 262 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 35-I | 68 | 1 | 2501 or 7502 | 1931 or 4432 | 1 | 2501 or 7502 | Base Station report* | 68 |  |
|  | 577 | 1 | 0 or 1125 | 1118 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 277 | 0, 1, 2 or 3 | 0 or 1125 | 869 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 35-II | 193 | 1 | 2501 or 7502 | 681 or 5682 | 1 | 2501 or 7502 | Base Station report* | 193 |  |
|  | 1118 | 1 | 0 or 1125 | 577 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 869 | 0, 1, 2 or 3 | 0 or 1125 | 277 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 36-I | 93 | 1 | 2501 or 7502 | 2181 or 4682 | 1 | 2501 or 7502 | Base Station report* | 93 |  |
|  | 587 | 1 | 0 or 1125 | 993 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 302 | 0, 1, 2 or 3 | 0 or 1125 | 1119 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 36-II | 218 | 1 | 2501 or 7502 | 93 or 593 | 1 | 2501 or 7502 | Base Station report* | 218 |  |
|  | 993 | 1 | 0 or 1125 | 587 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 1119 | 0, 1, 2 or 3 | 0 or 1125 | 302 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |

# Attachment C - Example of Symmetric FATDMA slot Reservation and Refresh

[formerly Annex D]

The following diagram shows the slots marked as “base station” slots (reserved for FATDMA use) following the reception of two VDL Message 20’s.  The first Message 20 is received in slot 1000 of frame “x”, and the second Message 20 is received in slot 879 of frame “x+1”.  The parameter content of each of these messages is listed above each of the diagrams.

The first Message 20 uses the RATDMA Access Scheme.  The top diagram shows the slots marked as “base station” by all mobile AIS stations for the remainder of the “x” frame and the entire “x+1” frame.  Note that the last reserved interval in frame “x+1”is already reduced by one to “6”.

The second Message 20 uses one of the marked “base station” slots of frame “x+1” to refresh the reservations.  Note that the slot earlier set to “6” is now refreshed to “7”.  Also, note the symmetric spacing of the slot reservations creates an exact replication of the reserved slots in all following frames.  There is no unexpected “slot-drift”.  The use of the exact slot increments of 2, 3, 5, 6, 9, 10, 15, 18, 25, 30, 45, 50, 75, 90, 125, 150, 225, 250, 375, 450, 750, or 1125 minimize the possibility of false markings of “base station” slots in all mobile AIS stations, and support the integrity of the VDL.

The Base Station without control functions has limitations with regards to the internal BAS functions. Forward reference is given to IALA AIS guidelines vol2

If DSC has been implemented as part of the AIS service

A message that is initiated by the vessel is said to be broadcast

As is the case when mounted on an AtoN; the point here is, that the physical AIS shore station has a fixed geographical position. If it is mounted on an AtoN the area in which the AtoN is allowed to swing is very small compared to the coverage area of this physical AIS shore station.

Name change currently considered in revision process of Recommendation ITU-R M.1371-1.

The value 2250 should not be used, as it implies a once per frame transmission; the value zero results in the same.

Means to avoid interference could include low, directional antennas and taking advantage of local land topography.

When acting as a semaphore, the reporting rate of a base station increases to a higher update rate. Therefore more FATDMA reserved slots would be needed and the timing requirements for transmissions of Base Station Reports in semaphore mode would be much more stringent. Therefore, this case is being dealt with at FATDMA usage category 2 (“High timing requirements slots” in the section below.

The requirements concerning DGNSS integrity alert (To-To-Alert TTA max. 10 sec) can be met by a maximum of 4 GNSS satellites at the same time.

The requirements concerning DGNSS integrity alert can only be met by provision of additional FATDMA reservation usage category 1 time slots.

Only the 8 most relevant GNSS satellites are corrected.

Name change currently considered in revision process of Recommendation ITU-R M.1371-1.