e-NAV 3 Working Paper

AIS AtoN Symbology

Introduction

Three input papers to e-NAV 3 addressed the presentation of AIS AtoN symbols: the Liaison Note from ANM (e-NAV3/07/01) was originally sent to the Policy Advisory Panel in May and an input paper to IMO NAV 53 was subsequently developed and submitted (NAV 53/13/4). IALA undertook to provide a further report to NAV 54, when results of trials became available, therefore the ANM Liaison Note has been superseded and requires only a formal response. The other two input papers from Japan (e-NAV3/07/2 and 4B) and Denmark (e-NAV3/07/4C) deal with problems relating to the display of AIS AtoN symbols, in particular clutter and obscuring other information. Both papers propose new or modified symbols. The Committee appreciated the problem and the need for solutions, but felt that it was not really within IALA’s remit to propose new symbols. Therefore the Chair and vice Chair undertook to consider the matter and suggest a way forward.

Current situation

The standard representation for AIS AtoNs is a diamond with a cross at the actual position (see Annex). The same symbol is used for real and virtual AtoNs. This derives from the IMO Performance Standard and is implemented in the relevant IEC documents, explicitly or by reference and it would not be practical or desirable to change it.

Proposal

The lack of distinction between Real and Virtual AtoN may be a matter for concern, but a notation could be added to the symbol in accordance with IMO SN/Circ. 243 & IEC 62288, which allow for alphanumeric labelling, for example a V beside the symbol for a Virtual AtoN.

However, this notation should be standardized to avoid confusion.

The e-NAV Committee is invited to comment on this suggestion, which could be put in the form of a Liaison Note to other international organizations.

ANNEX

AIS AtoN Display - Current situation

IMO approved performance standards for the presentation of navigation-related information on shipborne navigational displays in MSC Resolution 191(79) and adopted Guidelines for the Presentation of Navigation Related Symbols, Terms and Abbreviations in Safety of Navigation Circular SN/Circ. 243 in December 2004. This specifies a blue diamond for AIS AtoNs, changing to red if it goes off station. No distinction is made for Virtual AtoNs, except for a limitation on size of diamond and the fact that the real AtoN symbol will be absent. The relevant text reads:

“AIS Aids to navigation (ATON) shall be presented as a diamond with crossed lines centred at the reported position of the ATON. The crossed lines shall not be more than 2 millimetres in length. For a virtual AIS ATON, the sides of the diamond shall be not more than 6 millimetres in length. For a real AIS ATON that is on position, the diamond shall

encompass both the charted ATON symbol and the reported position of the ATON. For a real AIS ATON that is off position, the diamond shall be drawn as for a virtual AIS ATON apart from the charted symbol. The diamond and crossed lines shall be drawn using a thin solid line style.

Recommended colour for an “on position” AIS ATON: blue.

Recommended colour for an “off position” AIS ATON: red.

An AIS ATON marking a fixed structure with dimensions larger than the diamond symbol shall be presented with an additional true scaled outline representative of the structure. In this case, the diamond symbol shall remain positioned at the reported position of the AIS ATON.

AIS ATON may be labelled. Alphanumeric text used to label an AIS ATON shall be the same colour as the AIS ATON symbol.”

This text and the associated graphics are reproduced in IEC Document 62288 – Presentation of navigation-related information on shipborne navigational displays – General requirements – Methods of testing and required test results. This document is at CDV (Committee Draft for Vote) stage, with a closing date of 11 Jan 2008.

The revision of IEC document 61174 Ed.3, ECDIS - Operational and performance requirements, also at CDV with the same closing date, includes the display of AIS targets, but not AtoNs. However, in Annex K: Presentation of navigation information, Annex A of IEC 62288 is referred to as applicable to 61174.

Display of AIS AtoNs is also addressed in IEC Document 62388, Shipborne radar - Performance requirements, methods of testing and required test results and this document is cross-referenced throughout 62288, where radar and chart information appear on the same display.  IEC 62388 is at FDIS (Final Draft International Standard) and can therefore be considered fixed. The diamond with a cross inside is used for AIS AtoNs and no distinction is made for Virtual AtoNs.