Rapporteurs Report

Development in non-radio navigation systems

(Task M2)

1. Background

This report, prepared for the ENAV Committee, provides information about ongoing developments in the field of non-radionavigation systems. Furthermore developments of INS (Integrated Navigations Systems) which combine radio navigation and non-radionavigation systems will be reported. Most of the information is based on published material, conference proceedings and contacts with manufacturers in this area.

2. Statements from eNAV2

In the last rapporteur report of eNAV2 it was stated that the performance of an INS depends not only on the IMU (Inertial Measurement Unit)  sensor (type/technology) but also on the coupling method used together with a GNSS receiver.

Furthermore a paper about the evaluation of integrated GNSS and INS (Inertial Navigation System) for marine navigation was presented during eNAV2 (e-NAV 2 / 06 / 07). This study concentrated on quantifying the accuracy of navigation in the event of a GNSS failure, using different grades of IMU (‘Navigation’, ‘Tactical’ and ‘Low Cost’) and various (simulated) GNSS failure periods. The main results of the study are given below:

- none of the options considered were suitable to maintain navigation accuracy during a major outage of GNSS.

- For ocean areas, both navigation and tactical-grade IMUs will give protection for appreciable outages (over 1 hour and 15 minutes respectively).

- for coastal areas, 15 minutes of good accuracy would be maintained using a navigation-grade IMU

- for the harbour approach phase, accuracy to 10 metres could be obtained for 3.5 minutes with a navigation-grade IMU-based system

- for docking and where sub-10 metre accuracy is required, INS coupled with GNSS does not appear to offer any benefit in the event of a GNSS failure.

Both reports stated further that future MEMS based INS could  be a useful addition to a ship’s navigation system.

3. Potential and limits of a MEMS based INS

The present rapporteur report will try to give an outlook on the potential and limits of MEMS sensors (Micro Electro Mechanics System) used in an INS. MEMS is an enabling technology with a global market. The major growth opportunities will come from automotive and consumer application markets. The idea of an inertial measurement unit (IMU) on a chip and unit costs as low as GPS modules are anticipated in the near future.

MEMS sensors can be classified in terms of detection criterion, e.g. piezoresistive signal pick-off, capacitive and piezoelectric. Table 1 summarizes the motion sensing technique as well as the pros and cons.

| Category | Motion sensing technique | Pros | Cons |
| --- | --- | --- | --- |
| Piezoresistive | Deflection of the piezoresistive material | Low costs Simple interface Wider range | Low precision High sensitivity to temperature |
| Capacitive | Change in capacitance of two electrodes | More accurate Excellent linearity Low temperature sensitivity | Complex Electromagnetic interference Small dynamic range |
| Piezoelectric | Electric charge due to change in motion | Wide dynamic range | Low sensitivity |

Table 1: Classification of MEMS sensors, [1]

An IMU normally provides position, velocity and attitude. Such an IMU consists in general of orthogonal triads of accelerometers and gyroscopes.

3.1 MEMS Accelerometer Development

MEMS inertial accelerometers are produced in large quantities. Hence their costs are currently in a range of several dollars. A number of companies are working on a tri-axis MEMS, mostly for the automotive industry. Nearly all MEMS accelerometers work by measuring the motion of a proof mass against a fixed frame. The main sensing approaches are capacitive or piezoresistive. The power consumption of such sensors are in a range of about 1-10 mW.

3.2 MEMS Gyros Development

Most commercial gyros are vibratory and use the transfer of energy between two vibration modes caused by Coriolis force. They are challenging to manufacture, hence, their cost are higher compared to accelerometers. Current MEMS gyros are larger devices than MEMS accelerometers and can require nearly 10 times more power. Manufacturers employ many different sensing approaches and materials to make MEMS gyros. Most of the available gyros are single axis solutions. The costs of a typical piezoceramic single axis gyro is about 10 dollars.

| Parameter | Gyros | Accelerometer |
| --- | --- | --- |
| Range | ±150 deg/s | ± 5 g |
| Scale factor | 10 mV/(deg/s) | 250 mV/g |
| Non-linearity | 0.1 % of full scale | 0.2 % of full scale |
| Axis misalignment | ± 0.2 deg | ± 0.2 deg |
| Bias error | ±0.5 deg/s | ± 6 mg |
| Bias instability (100 sec) | 0.01 deg/s | 0.2 mg |

Table 2:  Typical specification of up to date gyros and accelerometers, [1]

3.3  MEMS performance

The costs and size of the mentioned MEMS sensors are very encouraging. But for maritime applications it is essential to know if the accuracy of these sensors will be sufficient to support navigation applications. When looking at different publications and presentations on recent conferences it seems that there are arguments in favour and against the navigation capability of MEMS based INS.

The navigation performance of an MEMS based INS depends mostly on the gyro error behaviour. Some of the error components can be compensated with the integrated GNSS receiver and the fine tuning of a Kalman filter process. In spite of compensating some errors within the MEMS sensors the resulting navigation performance with respect to position accuracy will still be dominated by the MEMS-Gyro short time instability.

To assess the usage of a MEMS/GNSS based INS it is interesting to estimate the resulting position drift when the GNSS is blocked for a given time period. Based on an article (Inside GNSS, [1]) such an estimation shows, that the position error can grow up to more than 80 m during a GPS outage of 60 sec. In this article it was further stated that the performance could be improved when information about the vehicle dynamics will be included in the Kalman filter process.

4. Conclusions

- Even for navigational and tactical IMUs it is difficult to maintain navigation accuracy during a major outage of GNSS.

- MEMS is an enabling technology with a global market and a rapid development to smaller size, lower cost and higher performance of the sensors.

- Because of the insufficient short time stability, especially within the MEMS gyro sensors, they can currently only be used to support during short GNSS outages.

- Nevertheless MEMS could  beneficially be used to support an MEMS based INS in a deep or tight coupling method to enhance signal stability and integrity.

- Furthermore MEMS based INS could have a reasonable contribution for providing a stable heading output during GNSS outages.

- The development of MEMS based INS should further be observed.

M. Hoppe, September 2007

[1] 	Naser El-Sheimy, Xiaoji Niu, ”The Promis of MEMS to the Navigation Community”, Inside GNSS, April 2007