PROPOSAL ON THE APPLICATION OF BINARY MESSAGES.

1. Purpose

At NAV49 held in July 2003, seven (7) binary messages were adopted as a trial set of Application Specific Messages utilizing Binary Messages 6 and 8. It was also agreed that the trial messages should be evaluated for their effectiveness and practicability during a trial period of 4 years. Member Governments were encouraged to make the evaluation and, when they recognize the need, to propose new binary messages.

Accordingly, in Japan, a study committee was organized inviting professionals such as a professor, a knowledgeable engineer, a pilot and a captain of large ships, in order to take up a study-and-research work on the binary messages raised by the IMO Sub-committee on Safety of Navigation. In due course, the committee conducted a field test at sea to evaluate the need, effectiveness and practicability of the messages.

This paper will, based on the accomplishment of our committee, propose some new Application of Binary Messages for practical use with expectation that those may be utilized in the field of international application branch.

2. Notes on Studies

The study committee on binary messages carried on the study-and-research work as follows.

2.1. Opinions of Marine Society

A questionnaire was sent out via a web link to shipping agents, maritime organizations, etc, asking their opinions about the formats, contents, etc, of the defined seven (7) binary messages and new parameters they wish to be newly included in appropriate messages.

2.2. NAV49 Trial Set of Message

On the basis of outcome of the questionnaire and findings from other studies, the committee discussed whether the NAV49 trail messages need additional parameters and/or contents to be included and other aspects concerned.

2.3. New Binary Messages

Response to the questionnaire brought up new probable applications of binary messages as desirable for practical use.

The committee rearranged the inputted requests and/or comments and discussed the contents, ie, parameters and formats, of candidate binary messages for possibility to be included in the items of proposal.

3. Proposal

According to the suggestions our committee has presented through studies, discussions and considerations, we propose following aspcpets concerning the binary messages.

3.1. Modification of NAV49 Trial Set of Message

-  Applications

a)  To combine the messages Aplication 1 “Meteorological and Hydrological Data” and Aplication 4 “Tidal Window”;

These are treated separately as Application 1 and 4 and both provide data of natural phenomena, ie, most advantageous and important information for users in the marine traffic area. The integrated message to be proposed will allow the essentials of the two be known simultaneasly by the users.

b)  To modify Application 3 “Fairaway Closed” to “Port and Fairway Closed”;

This modification will allow the message to provide information on a closed port in addition to a closed fairway.

-  Formats

To modify formats of the messages by rearranging parameters;

These modifications are requied in view of making the binary messages to be further widely utilized and the slots to be more effectively used.

Table 1 describes these proposals in detail.

3.2. New binary messages

Not a few applications were presented as desirable messages by the participants in survey through questionnaire on user requirements for the binary message. Among those, the committee selected fourteen (14) probabls as the ones to be proposed for new messages additional to the trials.

Table 2 shows formats and parameters of the proposed applications in detail.

Table 1: Proposal on Modification of Formats of NAV49 Trial Set

| Name of Message | Description | Addressed / Broadcast | (primary) Transmit Station | (primary) Receiv. Station | Message Table No. (*) | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| Meteorological & Hydrological Data And Tidal Window | To arrange parameters as follows, since this message takes a form to integrate the message “Meteorological and Hydrological Data” with “Tidal Window”. - To add place and date/time in LT of observation, weather, notices and warnings. - To add “time at next highest curent speed, time at next turn of current direction, time at turn of current direction after the next, time at next low tide, water level at next low tide, time at next high tide and water level at next high tide” to the current forecast which provides information on the time period of high and low tide. - To delete Parameter “Current measuring level, #3”as the level is not necessary to that level. - Current information should be obtained at and provided for only one observation point. | B | Ba | Mo | Table 1-1 |  |
| Dangerous | To rearrange Parameter as follows so as to reduce the input workload of ship operators. - To delete Parameter “ATD from Last Port of Call”; the actual time of departure is less needed. - To delete Parameter “ETA at Next Port of Call”; the estimated time of arrival may be derived from a voyage related information. | A | Mo | Ba | Table 1-2 |  |
| Cargo Indication | To arrange the contents as follows so as to make the message provide information usable in case ofａport close as well. - To add name of the closed port or fairway. - To add (brief) reason of closing. - To add coordinates of the center point in the closed area. - To add an extending direction of the closed fairway. - To reduce the number of bits for a text information on the start point of closing. - To add and define Parameter “End of closing” in terms of “day, month, hour and minute”. | B | Ba | Mo | Table 1-3 |  |
| Port And Fairway Closed | No change is proposed. | B | Mo | Ba/Mo | Table 1-4 |  |
| Extended Ship Static And Voyage Related Data | No change is proposed. | A | Mo | Ba | Table 1-5 |  |
| Number Of Persons On Board | No change is proposed. | B | Ba | Mo | Table 1-6 |  |

Note 1:  A; Addressed Message, B; Broadcast Message, Ba; Base Station (= Shore Station), Mo; Mobile Station (= Ship Station)

2: (*); These tables are given in the following pages.

Table 2: Proposals on New Binary Messages

| Name of Message | Description | Addressed / Broadcast | (primary) Transmit Station | (primary) Receiv. Station | Massage Table No. (*) | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| Clearance Time To Enter Port | Date, Time and Position | A | Ba | Mo | Table 2-1 | Refer to Proposal from Germany. |
| Rejection Of Approach To Harbor Entrance | Oil spilling, in Dangerous wreck , Heavily damaged | A | Ba | Mo | Table 2-2 |  |
| Derelicts (Drifting Objects) | Time and position of recognition, Name of Derelicts | B | Ba/Mo | Ba/Mo | Teble 2-3 |  |
| Drifting Mines | Time and position of recognition, Drifting direction, | B | Ba/Mo | Ba/Mo | Table 2-4 |  |
| Wreck | Time and position of wreck, Wreck marking buoy, Navigable water-depth | B | Ba | Mo | Table 2-5 |  |
| Area To Be Noticed | Nature of Notice, Reason, Number of the corners of polygon, Positions of corners | B | Ba | Mo | Table 2-6 | New, original Proposal |
| Aids To Navigation | Name, Position, Type, Status, Light character, Date and time of trouble occurred | B | Ba | Mo | Table 2-7 |  |
| Marine Traffic Signal | Station name, Position, Status, Signal in serve, Time and type of next signal | B | Ba | Mo | Table 2-8 |  |
| Fishing Activities | Fishing method, Radius of fishing circle with unit, Position of the center of circle | B | Ba | Mo | Table 2-9 |  |
| Pilotage | On/off board of Pilot, Voyage with or without Pilot, Position and time of Pilot aboard | A | Mo | Ba/Mo | Table 2-10 |  |
| Navigation Support Lines | Name and type of lines, Start and end position of lines | B | Ba | Mo | Table 2-11 |  |
| Berth Data | Name and position of berth | A | Ba | Mo | Table 2-12 |  |
| Extended Voyage Related Data | Contact point on board, Agent and contact point, Name of berth at destination | A | Mo | Ba | Table 2-13 |  |
| Weather Report From Ships | Weather, Visibility, Wind speed, Wind direction and Air pressure observed on ships in navigation | B | Mo | Ba/Mo | Table 2-14 |  |

Note 1: A; Addressed Message, B; Broadcast Message, Ba; Base Station (= Shore Station), Mo; Mobile Station (= Ship Station)

2: (*); These tables are given in the following pages.

Table 1-1: Message “METEOROLOGICAL/HYDROLOGICAL DATA AND TIDAL WINDOW”

| Editorial Classifi- cation | Parameter | No. of bits | Description |
| --- | --- | --- | --- |
|  | Message ID | 6 | Identifier for Message 8 (always 8) |
|  | Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. |
|  | Source ID | 30 | MMSI number of source station |
|  | Spare | 2 | Not used. Should be set to zero. |
|  | IAI | 16 | 11 or 14 |
| Addition | Observation place | 120 | (20 characters) |
| Common | Observation position, Lat | 24 | ** deg **.*** min |
| Common | Observation position, Lon | 25 | *** deg **.*** min |
| Common | Date and time of observation, (UTC) | 11 | ** hour** min |
| Addition | Date and time of observation, (LT) | 11 | ** hour** min |
|  | Average wind speed | 7 | *** konts |
|  | Wind gust | 7 | *** knots |
|  | Wind direction | 9 | *** deg |
|  | Wind gust direction | 9 | ** deg |
|  | Air temperature | 11 | ** deg |
|  | Relaitve humidity | 7 | *** % |
|  | Dew point | 10 | ** deg |
|  | Air pressure | 9 | **** hPa |
|  | Air pressure tendency | 2 | decreasing/steady/increasing |
|  | Horizontal visibility | 8 | **.* nm |
| Addition | Weather | 4 | Fine/Cloudy/Rain/Fog/Snow/Typhoon (Hurricane, Monsoon) |
| Addition | Caution,Warning | 2 | Gale/High seas |
|  | Water level Now | 9 | **.* m |
| Editorial Classifi- cation | Parameter | No. of bits | Description |
|  | Water level trend Now | 2 | decreasing/steady/increasing |
|  | Surface current speed | 8 | **.* knots |
|  | Surface current direction | 9 | *** deg |
|  | Current speed, #2 | 8 | **.* knots(max. 25.0 knots) |
|  | Current direction, #2 | 9 | *** deg |
|  | Current measuring level, #2 | 5 | ** m |
| Deletion | Current speed, #3; Current direction, #3; Current measuring level, #3 | 0 |  |
|  | Significant wave height | 8 | **.* m |
|  | Wave period | 6 | ** sec |
|  | Wave direction | 9 | *** deg |
|  | Swell height | 8 | **.* m |
|  | Swell period | 6 | ** sec |
|  | Swell direction | 9 | *** deg |
|  | Sea state | 4 | 0 ～ 12 |
|  | Water temparature | 10 | **.* deg |
|  | Precipitation (type) | 3 | 0 ～ 8 |
|  | Ice | 2 | Yes／No |
|  | Salinity | 9 | **.* % |
| Addition | Sunrise, (LT) | 11 | ** hour** min |
| Addition | Sunset, (LT) | 11 | ** hour** min |
| Addition | Time of (next) max. Current speed, (LT) | 11 | ** hour** min |
|  | Current direction at (next) observation time | 9 | *** deg |
| Editorial Classifi- cation | Parameter | No. of bits | Description |
|  | (Next) max. current speed | 8 | **.* knots(max. 25.0 knots) |
| Addition | Time of (next) current turn, (LT) | 11 | ** hour** min |
| Addition | Time of current turn after the next, (LT) | 11 | ** hour** min |
| Addition | Time of next low tide, (LT) | 11 | ** hour** min |
| Addition | Water level at next low tide | 9 | **.* m |
| Addition | Time of next high tide, (LT) | 11 | ** hour** min |
| Addition | Water level at next high tide | 9 | **.* m |
| Deletion | Current direction and speed, #1 - #3 | 0 |  |
|  | Subtotal | (568) |  |
|  | Spare bits | 16 | Not used. Should be set to zero. |
|  | Total Number of bits | 584 |  |

| This message is intended to provide meteorological and hydrological data at present in coastal areas. This message is transmitted by a competent authority. |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3 slots | Transmission: Shore station | Reception: Ship station | Broadcast | ― |

Table 1-2: Message “DANGEROUS CARGO INDICATION”

| Editorial Classifi- cation | Parameter | No. of bits | Description |
| --- | --- | --- | --- |
|  | Message ID | 6 | Identifier for the message 6, always 6 |
|  | Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. |
|  | Source ID | 30 | MMSI number of source station |
|  | Sequence number | 2 | 0 - 3 |
|  | Destination ID | 30 | MMSI number of destination station |
|  | Retransmit Flag | １ | Retransmit Flag should be set upon retransmission: 0 = no transmission = defalult; 1 = retramsmitted |
|  | Spare | １ | Not used. Should be set zero. |
|  | IAI | 16 | 12 |
|  | Last Port of Call | 30 | UN Locode（5 characters） |
| Deletion | ATD from Last Port of Call (UTC)） | 0 | Actual Time of Departure; ** month ** day ** hour ** minute |
|  | Next Port of Call | 30 | UN Locode（5 characters） |
| Deletion | ETA at Next Port of Call (UTC) | 0 | Estimated Time of Arrival; ** month ** day ** hour ** minute |
|  | Main Dangerous Good | 120 | （20 characters） |
|  | IMD category of Main Dangerous Good | 24 | （4 characters） |
|  | UN Number of Main Dangerous Good | 13 | 1 - 3363 |
|  | Value of Quantity of Main Dangerous Good | 10 | 1 - 1023 |
|  | Unit of Quantity of Main Dagerous Good | 2 | in kg/tons/kilo-tons |
|  | Subtotal | (317) |  |
|  | Spare | 43 | Not used. Should be set to zero. |
|  | Total Number of bits | 360 |  |

| This message provides information on the main dangerous goods being carried on board. This message is transmitted by an interrogated ship on request from a competent authority. |  |  |  |  |
| --- | --- | --- | --- | --- |
| 2 slots | Transmittion: Ship station | Reception: Shore station | Addressed | Acknowledgement: Requested |

Table 1-3: Message “HABOR AND FAIRWAY CLOSED”

| Editorial Classifi- cation | Parameter | No. of bits | Description |
| --- | --- | --- | --- |
|  | Message ID | 6 | Identifier for Message 8 (Always 8) |
|  | Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. |
|  | Source ID | 30 | MMSI number of source station |
|  | Spare | 2 | Not used. Should be set to zero. |
|  | IAI | 16 | 13 |
| Addition | Name of Harbour or Fairway closed | 120 | （20 characters） |
| Addition | Reason for closing (in brief) | 2 | Accident at sea/Accident on land/others |
|  | Reason for closing (in detail) | 120 | （text） |
| Addition | Center of closed area, Lat | 24 | ** deg **.*** min |
| Additiion | Center of closed zone, Lon | 25 | *** deg **.*** min |
| Addition | (approx.) Direction of Closed Fairway | 3 | East/West/South/North/South to North/East to West |
| Modifi- cation | Start position of closing | 90 | (15 characters) |
| Modifi- cation | End position of closing | 90 | (15 characters） |
|  | Extention of closed area (radius) | 10 | 0 - 1000 |
|  | Unit of extention value | 2 | m/km/nm/cbl |
|  | Starting date and time of closing (LT) | 20 (4+5+5+6) | ** day ** month ** hour ** min |
| Modifi- cation | Ending date and time of closing (LT) | 20 (4+5+5+6) | ** day ** month ** hour ** min |
|  | Subtotal | (582) |  |
|  | Spare | 2 | Not used. Should be set to zero. |
| Editorial Classifi- cation | Parameter | No. of bits | Description |
|  | Total Number of bits | 584 |  |

| This message provides reasons, positions and duration for closing of harbors and fairways. This message is transmitted by a competent authority. |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3 slots | Transmission: Shore station | Reception: Ship station | Broadcast | ― |

Table 1-4: Message “EXTENDED SHIP STATIC AND VOYAGE RELATED DATA”

| Parameter | No. of bits | Discription |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 8 (always 8) |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. |
| Source ID | 30 | MMSI number of source station |
| Spare | 2 | Not used. Should be set to zero. |
| IAI | 16 | 15 |
| Air Drought | 11 | In 1/10m, 2047 = height over keel 204.7 m or greater, 0 = not available = default |
| Spare | 5 | Not used. Should be set to zero. |
| Total Number of bits | 72 | This International Function Message uses one slot. |

| This message is used to report the height over keel of a ship. |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 slot | Transmittion: Ship station | Reception: Shore/Ship station | Broadcast | ― |

Table 1-5: Message “NUMBER OF PERSONS ON BOARD”

| Parameter | No. of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 8 (Always 8) |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. |
| Source ID | 30 | MMSI number of source station |
| Spare | 2 | Not used. Chould be set to zero. |
| IAL | 16 | 16 |
| Number of persons | 13 | Current number of persons on-board, including crew members: 0 - 8191; Default = 0 = not available; 8191 = 8191 or more. |
| Spare | 3 | Not used. Should be set to zero. |
| Total Number of bits | 72 | This International Function Message uses one slot. |

| This message is used to report the number of persons on board. |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 slot | Transmission: Ship station | Reception: Shore station | Addressed | Acknowledgement: Not required |

Table 1-6: Message “PSEUDO-AIS TARGETS”

| Parameter | No. of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 8 (always 8) |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. |
| Source ID | 30 | MMSI number of source station |
| Spare | 2 | Not used.Should be set to zero. |
| IAI | 16 | 17 |
| VTS Target 1 | 120 | Refer to table below; occupies 2 slots |
| VTS Target 2 | 120 | Optional; refer to table below; occupies 2 slots |
| VTS Target 3 | 120 | Optional; refer to table below; occupies 3 slots |
| VTS Target 4 | 120 | Optional; refer to table below; occupies 3 slots |
| Total Number of bits | Max 536 | 3 slots |

Each VTS Target should be structured as follows:

| Parameter | No. of bits | Description |
| --- | --- | --- |
| Type of Target Identifier | 2 | Identifier Type; 0 = The target identifier should be the MMAI number. 1 = The target identifier should be the IMO number. 2 = The target identifier should be the call sign. 3 = Other (default) |
| Target ID | 42 | Target Identifier. The Target ID should depend on Type of Target Identifier above. When call sign is used, it should be inserted using 6-bit ASCII. If Target Identifier is unknown, this field should be set to zero. When MMSI or IMO numberis used, the least significant bit should equal bit zero of the Target ID. |
| Spare | 4 | Not used. Should be set to zero. |
| Latitude | 24 | Latitude in 1/1000 of a minute. |
| Longitude | 25 | Longitude in 1/1000 of a minute. |
| COG | 9 | Course over ground in degrees (0 - 359); 360 = not available = default. |
| Time Stamp | 6 | UTC second when the report was generated (0 - 59, or 60 if time stamp is not available, which should also be the default value.) |
| Parameter | No. of bits | Description |
| ＳＯＧ | 8 | Speed over ground in knots; 0 - 254; 255 = not available = default |
| Total Number of bits | 120 |  |

| This message should be used to transmit VTS targets. This message should be variable in length, based on the amount of VTS targets. The maximum of VTS Targets transmitted in one International FM 16 should be seven (7). Because of the resulting effects of VDL channel loading, the transmission of International FM 16 should be no more than necessary to provide the necessary level of safety. A VTS tartget should be only used, when the position of the target is known. However, the target identity and/or course and/or time stamp and/or speed over ground may be unknown. This message is transmitted by a competent authority. |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3 slots | Transmission: Shore station | Reception: Ship station | Broadcast | ― |

Table 2-1: Message “CLEARANCE TIME TO ENTER PORT”

| Parameter | No. of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 6 (always 6) |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. |
| Source ID | 30 | MMSI number of source station |
| Sequence number | 2 | 0 - 3 |
| Destination ID | 30 | MMSI number of destination station |
| Retransmit flag | 1 | Retransmit flag should be set upon retransmission: 0 = no retransmission = default; 1 = retransmitted |
| Spare | 1 | Not used. Should be set to zero. |
| IAI | 16 | Two-digit number (Should not be overlapped with any of the one used in other message.) |
| Clearance time to enter port, (UTC) | 20 | ** day ** month ** hour ** minute |
| Clearance time to enter port, (LT) | 20 | ** day ** month ** hour ** minute |
| Place | 120 | ( Name of the port and berth, 20 characters) |
| Position, Lat | 24 | *** deg **.*** min |
| Position, Lon | 25 | ***deg **.*** min |
| Subtotal | (297) |  |
| Spare | 63 | Not used. Should be set to zero. |
| Total Number of bits | 360 |  |

| This message is intended to provide specified ships with information on the granted port to call and time to enter. This message is transmitted by a competent authority responsible for control of ships to enter/leave port. |  |  |  |  |
| --- | --- | --- | --- | --- |
| 2 slots | Transmssion: Shore station | Reception: Ship station | Addressed | Acknowledgement: Required |

Table 2-2: Message “REJECTION OF APPROACH TO HARBOR ENTRANCE”

| Parameter | No. of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 6 (always 6) |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. |
| Source ID | 30 | MMSI number of source station |
| Sequence number | 2 | 0 - 3 |
| Destination ID | 30 | MMSI number of destination station |
| Retransmit flag | 1 | Retransmit flag should be set upon retransmission: 0 = no retransmission = default; 1 = retransmitted |
| Spare | 1 | Not used. Should be set to zero. |
| IAI | 16 | Two-digit number (Should not be overlapped with any of the one used in other message.) |
| Place | 120 | Name of Port, (20 characters) |
| Position, Lat | 24 | ** deg **.*** min |
| Position, Lon | 25 | *** deg **.*** min |
| Reason for Rejection | 2 | Oil spilling, in danger to sink, heavily damaged, etc. |
| Sub-total | (259) |  |
| Spare | 101 | Not used. Should be set to zero. |
| Total Number of bits | 360 |  |

| This message provides specified ships with information on rejection of approach to a harbor entrance. This message is transmitted by a competent authority responsible for control of ships to approach a harbor entrance. |  |  |  |  |
| --- | --- | --- | --- | --- |
| 2 slots | Transmission: Shore Station | Reception: Ship Station | Addressed | Acknowledgement: Required. |

Table 2-3: Message “DERELICTS (DRIFTING OBJECTS)”

| Parameter | No. of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Massage 8 (always 8) |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. |
| Source ID | 30 | MMSI number of source station |
| Spare | 2 | Not used. Should be set to zero. |
| IAI | 16 | Two-digit number (Should not be overlapped with any of the one used in other message.) |
| Time of Recognition (UTC) | 20 | ** day ** month ** hour ** minute |
| Time of Recognition (LT) | 20 | ** day ** month ** hour ** minute |
| Place of Recognition | 120 | (20 characters) |
| Position of Recognition, Lat | 24 | ** deg **.*** min |
| Position of Recognition, Lon | 25 | *** deg **.*** min |
| Type of Derelicts (Drifting objects) | 3 | Containers/Lumbers/Drums/Other/Unknown |
| Drifting Direction | 3 | N/NE/E/SE/S/SW/W/NW |
| Subtotal | (271) |  |
| Spare | 89 | Not used. Should be set to zero. |
| Total Number of bits | 360 |  |

| This message is intended to provide information on derelicts recognized at sea. This message is transmitted by a station which recognized the derelicts (drifting objects) at sea. |  |  |  |  |
| --- | --- | --- | --- | --- |
| 2 slots | Transmission: Shore/Ship Station | Reception: Ship/Shore Station | Broadcast | ― |

Table 2-4: Message “DRIFTING MINES”

| Parameter | No. of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 8 (always 8) |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. |
| Source ID | 30 | MMSI number of source station |
| Spare | 2 | Not used. Should be set to zero. |
| IAI | 16 | Two-digit number (Should not be overlapped with any of the one used in other message.) |
| Time of Recognition (UTC) | 20 | ** day ** month ** hour ** minute |
| Time of Recognition (LT) | 20 | ** day ** month ** hour ** minute |
| Place of Recognition | 120 | (20 characters) |
| Position of Recognition, Lat | 24 | ** deg **.*** min |
| Position of Recognition, Lon | 25 | *** deg **.*** min |
| Drifting Direction | 3 | N/NE/E/SE/S/SW/W/NW |
| Subtotal | (268) |  |
| Spare | 92 | Not used. Should be set to zero.- |
| Total Number of bits | 360 |  |

| This message intended to provide information on drifting mines recognized at sea. This message is transmitted by a station which recognized a mine drifting at sea. |  |  |  |  |
| --- | --- | --- | --- | --- |
| 2 slots | Transmission: Shore/Ship Station | Reception: Ship/Shore Station | Broadcast | ― |

Table 2-5: Message “WRECK”

| Parameter | No. of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 8 (always 8) |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. |
| Source ID | 30 | MMSI number of source station |
| Spare | 2 | Not used. Should be set to zero. |
| IAI | 16 | Two-digit number (Should not be overlapped with any of the one used in other message.) |
| Time of Sinking occurred, (UTC) | 20 | ** day ** month ** hour ** minute |
| Time of Sinking occurred, (LT) | 20 | ** day ** month ** hour ** minute |
| Place of Sinking | 120 | (20 characters) |
| Position, Lat | 24 | ** deg **.*** min |
| Position, Lon | 25 | *** deg **.*** min |
| Wreckage Marking Buoy | 2 | Yes/No/Unknown |
| Residual Water-depth (Navigable Water-depth) | 6 | 0 - 62m: 63m≦; 1m step |
| Subtotal | (273) |  |
| Spare | 87 | Not used. Should be set to zero. |
| Total Number of bits | 360 |  |

| This message is intended to provide information on a sunken ship downed by accidents or disasters, including position, navigable water-depth, etc. This message is transmitted by a competent authority. |  |  |  |  |
| --- | --- | --- | --- | --- |
| 2 slots | Transmission: Shore Station | Reception: Ship Station | Broadcast | － |

Table 2-6: Message “AREA TO BE NOTICED”

| Parameter | No. of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier of Message 8 (always 8) |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message bas been repeated. |
| Source ID | 30 | MMSI number of source station |
| Spare | 2 | Not used. Should be set to zero. |
| IAI | 16 | Two-digit number (Should not be overlapped with any of the ones used in other messages.) |
| Navigational Restrictions | 3 | No restriction, Prudent maneuvering required, Navigation and/or Anchoring prohibited, No Overtaking, Two-way navigation prohibited, etc. |
| Reason | 120 | (20 characters) e.g.; Under construction, Divers working, etc. |
| Date and Time of Commencement, (LT) | 20 | ** day ** month ** hour ** minute |
| Date and Time of Expiration, (LT) | 20 | ** day ** month ** hour ** minute |
| Number of Polygonal Apexes | 3 | * (Number) |
| Position 1 , Lat. | 24 | ** deg **.*** min |
| Position 1 , Lon | 25 | *** deg **.*** min |
| Position 2, Lat | 24 | ** deg **.*** min |
| Position 2, Long | 25 | *** deg **.*** min |
| Position 3, Lat | 24 | ** deg **.*** min |
| Position 3, Lon | 25 | *** deg **.*** min |
| Position 4, Lat | 24 | ** deg **.*** min |
| Position 4, Lon | 25 | *** deg **.*** min |
| Position 5, Lat | 24 | ** deg **.*** min |
| Position 5, Lon | 25 | *** deg **.*** min |
| Subtotal | (467) |  |
| Spare | 117 | Not used. Should be set to zero. |
| Total Number of bits | 584 |  |

| This message is intended to provide information on navigational restrictions, positions, etc. of an area to be noticed. This message is transmitted by a competent authority. |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3 slots | Transmission: Shore Station | Reception: Ship Station | Broadcast | ― |

Table 2-7: Message “AIDS TO NAVIGATION”

| Parameter | No. of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 8 (always 8) |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. |
| Source ID | 30 | MMSI number of source station |
| Spare | 2 | Not used. Should be set to zero. |
| IAI | 16 | Two-digit number (Should not be overlapped with any of the ones used in other messages.) |
| Name of AtoN | 120 | (20 characters) |
| Current Position of AtoN, Lat | 24 | ** deg **.*** min |
| Current Position of AtoN, Lon | 25 | *** deg **.*** min |
| Type of AtoN | 2 | Visual, Radio, Audible, Other |
| Status of AtoN | 4 | In service/Light off/Irregular light characters/Collapse/Inclined/Shift/Washed away/Drifting/Fell out/Sunk (Submerged)/Intensity drop/Off-air/Power drop/Poor synchronization/Out of service/Other |
| Nation Code of Light Characters | 10 | Max: 512 codes |
| Light Characters | 14 | Max: 16,384 types |
| Luminous Range | 5 | 0 - 31 nm. |
| Date and Time of Trouble occurred, (UTC) | 20 | ** day ** month ** hour ** minute |
| Expected Date and Time of Recovery, (UTC) | 20 | ** day ** month ** hour ** minute |
| Date and Time of trouble occurred, (LT) | 20 | ** day ** month ** hour ** minute |
| Expected Date and Time of Recovery, (LT) | 20 | ** day ** month ** hour ** minute |
| Subtotal | (340) |  |
| Parameter | No. of bits | Description |
| Spare | 20 | Not used. Should be set to zero. |
| Total Number of bits | 360 |  |

| This message is intended to provide information on operational status, recovery, etc. of Aids to Navigation. This message is transmitted by a competent authority. |  |  |  |  |
| --- | --- | --- | --- | --- |
| 2 slots | Transmission: Shore Station | Reception: Ship Station | Broadcast | ― |

Table 2-8: Message “MARINE TRAFFIC SIGNAL”

| Parameter | No. of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 8 (always 8) |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. |
| Source ID | 30 | MMSI number of source station |
| Spare | 2 | Not used. Should be set to zero. |
| IAI | 16 | Two-digit number (Should not be overlapped with any of the ones used in other messages.) |
| Name of Signal Station | 120 | (20 characters) |
| Position of Station, Lat | 24 | ** deg **.*** min |
| Position of Station, Lon | 25 | *** deg **.*** min |
| Status of Signal | 1 | In regular or irregular service |
| Signal in Service | 5 | Flashing letter of I, O or F, or alternating letters of XI, XO or XF, etc. (*1) |
| Time of next Signal Sift, (UTC) | 11 | ** hour ** minute |
| Time of next Signal Shift, (LT) | 11 | ** hour ** minute |
| Expected Next Signal | 5 | Letter I, O, F, etc. |
| Subtotal | (258) |  |
| Spare | 102 | Not used. Should be set to zero |
| Total Number of bits | 360 |  |

Note:(*1) These are coded signals being adopted in Japan to provide ships with the timing of entering or leaving port.

The code stands for; e.g., I = “in-bound” only acceptable,

O = “out-bound” only acceptable,

F = both “in- and out-bound” acceptable,

XI = Code will shift to “I” in due time,

XO = Code will shift to “O”in due time,

etc.

| This message is intended to provide information on a signal station, status of the control signal now, etc., at the entrance of a harbor or channel where the shipping direction is controlled so that the traffic flow be kept in order. This message is tranmitted by a competent authority. |  |  |  |  |
| --- | --- | --- | --- | --- |
| 2 slots | Transmission: Shore Station | Reception: Ship Station | Broadcast | ― |

Table 2-9: Message “FISHING ACTIVITIES”

| Parameter | No. of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 8 (always 8) |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. |
| Source ID | 30 | MMSI number of source station |
| Spare | 2 | Not used. Should be set to zero. |
| IAI | 16 | Two-digit number (Should not be overlapped with any of the ones used in other messages.) |
| Fishing Method | 2 | Fixed net, Fishing in crowded or groups, Purse seining, etc. |
| Radius of Operating Circle | 10 | 0 - 1000 |
| Unit of Radius above | 2 | m/km/nm/cbl |
| Center of Circle above, Lat | 24 | ** deg **.*** min |
| Center of Circle above, Lon | 25 | *** deg **.*** min |
| Subtotal | (119) |  |
| Spare | 17 | Not used. Should be set to zero. |
| Total Number of bits | 136 |  |

| This message is intended to provide situation, approx. position, etc, of a fishing activity in operation on water adjoining to a fairway. This message is transmitted by a competent authority. |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 slot | Transmission: Shore Station | Reception: Ship Station | Broadcast | ― |

Table 2-10: Message “PILOTAGE”

| Parameter | No. of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 6 (always 6) |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. |
| Source ID | 30 | MMSI number of source station |
| Sequence number | 2 | 0 - 3 |
| Destination ID | 30 | MMSI number of destination station |
| Retransmit flag | 1 | Retransmit flag should be set upon retransmission: 0 = no retransmission = default; 1 = retransmitted |
| Spare | 1 | Not used. Should be set to zero. |
| IAI | 16 | Two-digit number (Should not be overlapped with any of the ones used in other messages.) |
| Pilot | 2 | on-board/off-board/no input |
| Pilot Arrangement | 2 | yes/no/no input |
| Place of Pilot aboard | 120 | 20 characters |
| Position above, Lat | 24 | ** deg **.*** min |
| Position above, Lon | 25 | *** deg **.*** min |
| Estimated Date and Time of Pilot aboard, (UTC) | 20 |  |
| Estimated Date and Time of Pilot aboard, (LT) | 20 |  |
| Subtotal | (301) |  |
| Spare | 59 | Not used. Should be set to zero. |
| Number of bits | 360 |  |

| This message is intended to provide information on arrangement, estimated boarding time, etc. of a pilot. This message is transmitted by a ship station on request from a competent authority. |  |  |  |  |
| --- | --- | --- | --- | --- |
| 2 slots | Transmission: Ship Station | Reception: Ship/Shore Station | Addressed | Acknowledgement: Required |

Table 2-11: Message “NAVIGATION SUPPORT LINES”

| Parameter | No. of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Massage 8 (always 8) |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. |
| Source ID | 30 | MMSI number of source station |
| Spare | 2 | Not used. Should be set to zero. |
| IAI | 16 | Two-digit number (Should not be overlapped with any of the one used in other message.) |
| Name of Line | 102 | (17 characters) |
| Nature of Line | 5 | Guide lines/Lead lines/Limits of Arc visibility and light sectors/Recommended fairways/Deep under-water fairways/Fairways advised through shore radar/General boundaries at sea/Port boundaries/Boundaries of zones in ports/Boundaries of fairways/Boundaries of fishing zones/Boundaries of anchorages/Boundaries of airports/Limits of ice fields/Other/Spare (32 types in total) |
| Line 1 Start Point, Lat | 24 | ** deg **.*** min |
| Line 1 Start Point, Long | 25 | *** deg **.*** min |
| Line 1 End Point, Lat | 24 | ** deg **.*** min |
| Line 1 End Point, Lon | 25 | *** deg **.*** min |
| Line 2 Start Point, Lat | 24 | ** deg **.*** min |
| Line 2 Start Point, Lon | 25 | *** deg **.*** min |
| Line 3 Start Point, Lat | 24 | ** deg **.*** min |
| Line 3 End Point, Lon | 25 | *** deg **.*** min |
| Subtotal | (359) |  |
| Spare | 1 | Not used. Should be set to zero. |
| Total Number of bits | 360 |  |

| This message is intended to provide nomenclature, original function, etc, of lines on a chart which are available as a supporting tool in marine navigation. This message is transmitted by a competent authority. |  |  |  |  |
| --- | --- | --- | --- | --- |
| 2 slots | Transmission: Shore Station | Reception: Ship Station | Broadcast | ― |

Table 2-12: Message “BERTH DATA”

| Parameter | No. of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 6 (always 6) |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. |
| Source ID | 30 | MMSI number of source station |
| Sequence number | 2 | 0-3 |
| Destination ID | 30 | MMSI number of destination station |
| Retransmit flag | 1 | Retransmit flag should be set upon retransmission: 0 = no retransmission = default; 1 = retransmitted |
| Spare | 1 | Not used. Should be set to zero. |
| IAI | 16 | Two-digit number (Should not be overlapped with any of the ones used in other messages.) |
| Name of Berth | 120 | 20 characters |
| Position of Berth, Lat | 24 | ** deg **.*** min |
| Position of Berth, Lon | 25 | *** deg **.*** min |
| Subtotal | (257) |  |
| Spare | 103 | Not used. Should be set to zero. |
| Total Number of bits | 360 |  |

| This message is intended to prevent ships from moving in an indecisive behavior and/or at unnecessary slow speed in port, and subsequently contribute to efficiency of overall marine traffic, by providing information on the arranged berth to the ships of no pilot onboard or unfamiliar with the area. This message is transmitted by a competent authority in charge of controlling ships to enter or leave port. |  |  |  |  |
| --- | --- | --- | --- | --- |
| 2 slots | Transmission: Shore Station | Reception: Ship Station | Addressed | Acknowledgement: Required |

Table 2-13: Message “EXTENDED VOYAGE RELATED DATA”

| Parameter | No. of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 6 (Always 6) |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. |
| Source ID | 30 | MMSI number of source station |
| Sequence number | 2 | 0 - 3 |
| Destination ID | 30 | MMSI number of destination station |
| Retranmit Flag | 1 | Retransmit flag should be set upon retransmission: 0 = no retransmission = default; 1 = retransmitted |
| Spare | １ | Not used. Should be set to zero. |
| IAI | 16 | Two-digit number (Should not be overlapped with any of the ones used in other messages.) |
| Contact point on board | 40 | （Telephone No.; eleven-digit number, Ship telephone No.; eleven-digit number） |
| Berth at next Port of Call | 120 | （20 characters） |
| Shipping Company or Agent | 120 | （20 characters） |
| Contact point above | 54 | (Telephone No.: fifteen-digit number;“Nation Code.; three‐digit”“Ship telephone No.; eleven-digit”“Spare; one-digit”) |
| Subtotal | (422) |  |
| Spare | 162 | Not used. Should be set to zero. |
| Total Number of bits | 584 |  |

| This message is intended to provide shaips with contact point onboard, name of the berth at next port of call, etc. This message is transmitted by a ship station interrogated from a competent authority. |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3 slots | Transmission: Ship station | Reception: Shore station | Addressed | ― |

Table 2-14: Message “WEATHER REPORT FROM SHIPS”

| Parameter | No. of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 8 (Always 8) |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. |
| Source ID | 30 | MMSI number of source station |
| Spare | ２ | Not used. Should be set to zero. |
| IAI | 16 | Two-digit number (Should not be overlapped with any of the ones used in other messages.) |
| Place of Observation | 120 | 20 characters |
| Position of Observation, Lat | 24 | ** deg **.*** min |
| Position of Observation, Lon | 25 | *** deg **.*** min |
| Date and Time of Observation, (UTC) | 11 | * day ** month ** hour ** minute |
| Date and Time of Observation, (LT) | 11 | ** day ** month ** hour ** minute |
| Weather | 4 | Fine/Cloudy/Rain/Fog/Snow/Typhoon (Hurricane, Monsoon) |
| Visibility (View) | 8 | **.* nm |
| Average Wind speed | 7 | *** knots |
| Wind direction | 9 | *** degrees |
| Air pressure | 9 | **** hPa |
| Sub-total | (284) |  |
| Spare | 76 | Not used. Should be set to zero. |
| Total Number of bits | 360 |  |

| This message is intended to provide ships and/or shore institutes with weather information observed on a ship in navigation. |  |  |  |  |
| --- | --- | --- | --- | --- |
| 2 slots | Transmission: Ship Station | Reception: Ship/Shore Station | Broadcast | ― |