e-NAV3 / 12 / 02

| From: VTS26 | VTS26-output-09 |
| --- | --- |
| To: e-NAV03 | September 2007 |

Liaison Note to e-Nav Committee

Binary Messages for use in VTS

1	Introduction

AIS has become an important tool for use in VTS.  AIS has primarily been very valuable in identification of radar targets and tracking of vessels in non-radar coverage areas.  In some instances it has reduced voice communications, for example by making the identification of vessels easier or by automatically providing information that was previously communicated through VHF.  However, the full benefit of AIS capability to VTS beyond these basic applications has yet to be realized, primarily in the area of use of binary messages.

There is need to reduce the workload imposed upon ships bridges and land based authorities by various and repetitive demands for reporting of information, and a need to improve the quality of information available. This can be achieved by reducing verbal communication, i.e. VHF radio traffic.  Binary messages are intended to facilitate ship-to-shore and shore-to-ship information by communicating important information in a common format.

In order to fully use AIS capabilities, VTS must have confidence that information communicated via AIS has been seen and acknowledged as appropriate.  Shipboard equipment must be able to effectively handle binary applications.  This will require equipment beyond the MKD for AIS interface.  ECDIS, ECS or other dedicated software equipment will be required.

In IMO circular SN/Circ. 236, “Guidance on the Application of AIS Binary Messages,” seven predefined binary messages have been provided for test use.  Use of these binary messages has provided a way forward in enhancing the use of AIS but initial experience with the use of these messages has identified that the relevance of the information contained in these messages shows room for improvement, as detailed in Section 3 of this liaison note.  Already it has been found that these predefined messages do not fully serve the information needs of mariners and shoreside authorities and as a result there have been efforts to develop "stand alone" messages in an uncoordinated and duplicative manner.

The loading of the VHF Data Link (VDL) is a well-documented concern.  This concern should not impede the development of binary applications; however, binary applications developed should be done so in an efficient manner and concern for VDL loading should drive coordinated efforts to reduce duplication of effort.

Where coordinated systems of vessel traffic services or integrated shoreside vessel data exchange systems exist, the development of some binary applications to facilitate comprehensive vessel data collection may not be seen as a pressing need.  However, the benefits of binary messages in places without access to extensive and alternative data exchange systems should be kept in consideration.

2	VTS Requirements

Some preliminary work has been done with the limited time available to the IALA VTS Committee to review VTS requirements that may benefit from development of binary applications.  However, the VTS Committee feels that more dedicated work in this area needs to be carried out and should be part of a process of continuous review.  Several national and multi-national efforts are currently underway; these efforts should be coordinated and information from them shared.

The following considerations should be borne in mind in the development of binary applications for VTS use:

•	Reduction of workload on the bridge of ships related to participation in VTS.

•	Reduction of VTS workload, in system maintenance, data gathering and other administrative tasks, in order to maximize the efforts of VTSOs spent on traffic management.

•	While "silent VTS" is unlikely to ever be entirely feasible, more use could be made of binary messaging to communicate information required for shipboard decision making to the mariner.

3	Recommendations on existing binary applications

The following recommendations and comments on existing binary messages specified in IMO SN.1/Circ. 236 are provided based on the limited experience with these messages to date.  These are not all-inclusive:

Application 1 - Meteorological and Hydrological data – The existing message will only send data for multiple parameters at one location.  If additional readings of a similar parameter (e.g., current profile) need to be disseminated, multiple binary messages must be sent, unnecessarily loading the VDL.  Also, if only one parameter is available, the entire two-slot message must be sent, rather than a smaller message containing the available data.  Some parameters do not allow for the required level of accuracy (e.g., water level only to 0.1m).

Application 2 - Dangerous cargo indication – The ability to report multiple dangerous cargoes in one message is required.

Application 3 - Fairway closed – The ability to indicate a polygon or other complex area is required.  Also the application should be considered for use for broader application than solely a closed fairway would allow the use of this message for other purposes that could be specified in the reason field, such as identification of a SAR operation area.

Application 4 - Tidal window – Similar to the Fairway Closed message, this message may be made more general, in that many maritime operations can be defined with a "window" such as passage of drawbridges or entry into one-way channels.

Application 5 - Extended ship static and voyage related data – Use of an entire binary message solely for air draft is inefficient; this information would be better provided in the static and voyage related data message.

Application 6 - Number of persons on board – The persons on board data is valuable and would be made more valuable through addition of information on number of infants or disabled persons on board, for the benefit of SAR and security services.

Application 7 - Pseudo-AIS targets – This is a valuable message for VTS operations. It could be made more valuable through further enhancements, such as allowing VTS to highlight targets or add amplifying information to the target and other enhancements based on an evaluation of VTS operational procedures and requirements.  The limitation of four targets is unnecessarily restrictive.

4	Future Considerations

VTSs have need for information that is not met by the existing binary applications in IMO SN.1/Circ. 236.  The following requirements have been identified; they will need to be updated through a process of continual review:

- Additional and more detailed information on environmental conditions: weather, hydrological, ice coverage

- Areas of interest: Areas to be avoided, hazards, environmentally sensitive areas

- Waterway status: closure, restrictions, width/depth limitations

- Rebroadcast of VTS-designated targets to specified vessels

- Route information –

  - VTS sailing plans

  - Assigned route

  - Recommended ice routes

  - Pilot embarkation and debarkation

- Anticipated vessel encounter list

The following considerations apply to the use of AIS in general, as well as the development of binary applications:

- Vessels should have the ability to request specific data via AIS versus continual broadcast; e.g. a vessel may require current or wind data in a critical maneuvering area more frequently than it is normally broadcast.

- Requested information should be based on the needs in a specific area and on information already collected.  E.g., a shoreside authority should not have to request the entire message 5 if only one information element of the message is required.  The development of the capability to assemble messages from only the required information elements will maximize access to usable information and minimize the load on the VDL.

- User interfaces should facilitate the unambiguous entry of data; e.g., UN LOCODES, UTC time.

- Expanded ship type information is required for use in VTS beyond those already included in the static and voyage related data message (message 5).

- The recent change to MARPOL annex 2 needs to be incorporated into the identification of hazardous cargo.  In updating this message field which currently only allows for the reporting of marine pollutants category, inclusion of additional hazardous cargo categories are required.

5	Action Requested

e-Navigation Committee is requested to incorporate the input provided above in their continuing work on binary messages and that, as appropriate, VTS Committee representation is included on any e-Navigation Committee binary message workgroups.

Currently ongoing testing of binary messages, such as that underway in the Baltic Sea and the United States, indicate that further work is needed, and better coordination of these efforts by Administrations should occur to avoid duplication of effort, conflicting developments and “stand-alone” national or regional standards.  E-Navigation Committee is requested to facilitate and encourage the coordination of ongoing AIS binary message development activities in accordance with IMO SN.1/Circ. 236.

e-Navigation Committee is requested to provide a response to this liaison note commenting on the specific recommendations to promote continuous development of these capabilities.

"Should a Member Government see the need to propose a new binary message, it should address to the Sub-Committee a demonstrated operational need and provide the proposed format and content of the message. Then, the Organization may accept, assign an identifier and publish it in an updated SN circular for the benefit of the maritime industry."