Annex 1/prop.3

Considerations

taken up in

Basic Designing of Electronic Navigation Support System

1. Problems of display by MKD

Most of existing MKDs are assembled with an approx. 5-inch monochrome screen and inherently present following problems when AIS information appears on the display unit.

-  The display unit cannot indicate a large amount of information in one process of presentation due to the small sized screen incorporated.

-  The screen cannot present information containing a longer text, e.g., a safety related information, in one process of presentation due to the small size.

-  The display unit has a difficulty to give clear identification of the superimposed multiple information due to the monochrome screen incorporated.

2. Support Information

Support information for ships must require following items to be communicated:

-  static information; basic information required for operation of the system, such as simplified charts, aids to navigation, remarkable targets, etc.

-  dynamic information; ever-changing information, such as meteorological/hydrological data and current data provided by binary messages, safety related information, information on status of aids to navigation, etc.

3. Requirements for Display Unit

Studies on producing new display unit indicated that attention must be placed on the following aspects.

3.1. AIS information should be kept in as compact as possible.

As a specific idea, it was suggested that the dynamic information only is to be superimposed onto an AIS data on a display unit, whereas the static information is to be provided through an alternative media other than AIS.

3.2. A user equipment, including the display unit, should be easily available and convenient to operate.

In this regard, it was considered that a user tool formed up on the basis of a commercially available personal computer will facilitate a free selection of the equipment and require no specially trained technique.

3.3. A user equipment should have an ability to take care of multiple working languages.

It was recognized that the PC based equipment above will allow operators to choose the language software suitable for communication with probable foreign crew onboard and thus enhance convenience of the users.

3.4. The price of user equipment should be kept at a low level.

It was considered that a user equipment based on a commercially available PC will be assembled so as to have an ability to use reasonably obtainable simplified maps, although high priced ENC will be best for the working charts.

installing

Electronic Navigation Support System

static electronic information

static electronic information

and programs

JGC

User

The electronic information is provided by the Shore station via a radio link.

VTS Center

Shore station

Basic information about e.g., a simplified chart, an AtoN or a remarkable target is to be inputted in PC via CD-ROM.

Ship’s AIS Equipment

Ship

Cable with

conversion adapter

General-purpose PC

Concept of Electronic Navigation Support System