Briefing on GNSS Status - August 2007

This briefing is provided to the e-NAV Committee under Rapporteur Item M3.

GPS

There are 30 usable satellites in the constellation at present. SVN23/PRN32 continues to broadcast, but is set unhealthy. Accuracies are well within the declared (13m 95%) target and monitoring has been improved by an enhanced ground control segment. Satellites being launched now (next due at the end of August) are Block IIR-M, broadcasting the second civil frequency, L2C, which is scheduled to become operational in 2014. The first IIF satellite with the additional L5 signal is due for launch in 2009, with an operational date of 2016 for L5. GPS III is still at the design stage, with first launch scheduled for 2013 and is not due to be fully operational until 2018.

# GLONASS

There are currently 11 operating satellites. Of the three launched on 25 December 2006 only two have been declared operational. The Russian Government is now committed to restoring a full system by late 2009. Recent launches have included the improved GLONASS M satellites with a second civil signal. The new GLONASS-K with a third civil signal on L3, should start launching in 2008. It will also carry differential corrections, integrity information and search and rescue functions. The future GLONASS K-M system is at the requirement definition stage.

# GALILEO

The first test-bed satellite, GIOVE A, continues to broadcast signals, including navigation signals for test purposes. The second, GIOVE B, is still held up by technical problems. The Public Private Partnership to deploy and operate Galileo has failed and a decision is now awaited from the EU Transport Ministers on additional public funding for the deployment phase, with the intention of handing over to a commercial operating consortium later. The official operational date is still 2012.

# COMPASS

China launched the fifth Beidou/Compass navigation satellite in April 2007. This was the first MEO, the previous four being GEOs. It is not yet known whether the intention is to develop a full 30 MEO constellation, or 4 GEOs + 12 MEOs.

# QZSS

The Japanese Aerospace Exploration Agency has published an interface specification for comment, with a baseline constellation of three Highly Elliptical Orbit satellites to begin launching in 2009. It is proposed to transmit L1, L2 and L5 signals, compatible and inter-operable with GPS.

# SBAS

The North American WAAS is operational for non-safety critical applications. The European EGNOS is in its Initial Operations Phase and is claimed to be providing horizontal accuracies better than one metre. It is scheduled to become operational in 2008. The Japanese MSAS and Indian GAGAN systems are still being deployed.

# IALA DGNSS

IALA has recommended that members operating the beacon DGNSS service, which is the international standard for maritime augmentation, should consider modernization to ensure that levels of service can be maintained and future requirements can be met (R-135). This recommendation took into account the potential alternatives, in particular Satellite Based Augmentation Services (SBAS) and the Automatic Identification System (AIS). The simplest option would be replacement of existing hardware with similar dedicated Reference Stations and Integrity Monitors (RSIM), but this would limit the potential for development and there are other possible options for re-engineering the IALA DGNSS system, including software RSIM, Virtual Reference Station Networks and integration with SBAS.

## GNSS Strategy

At some stage (post 2012) there should be two or three GNSS available, providing a total of more than 60 satellites, with inherent integrity monitoring and warning. This number of satellites will make Receiver Autonomous Integrity Monitoring (RAIM) a reliable  proposition. The need for separate (satellite or ground-based) augmentation may become questionable at that stage and it may be appropriate to plan the phase out of existing augmentation systems with those developments in mind. However, the concerns about the vulnerability of GNSS will remain and there will still be a case for alternative electronic position fixing systems and for independent verification and warning systems.