| From: VTS29 |  |
| --- | --- |
| To: e-NAV6 | March 2009 |

Liaison Note to e-Nav Committee

IALA recommendation on the e-Navigation Architecture – The Shore Based Perspective

(Recommendation e-Nav - 101)

The VTS Committee was asked to review the draft subject recommendation. After prolonged discussion it was the sense of the Committee that the user requirements were not taken fully into account in the document. We suggest that first the user requirements of ship and shore based users need to be developed as per the e-Nav3/Output/10 “Methodology for Capturing e-Navigation User Needs”.

It seems that user requirements are abstracted by defining “applications” with their user interfaces. These applications hide all the operational aspects although they will have an impact on the architecture.

A number of issues are likely to be seen as equal to each other where they are not.

For example:

* user interface equals user requirement

* applications are the front for the operational requirements

Because e-Navigation architecture affects both on-board and shore-side users, it is impossible to design a purely shore-side architecture.  The title should be changed to reflect this.

There should be an aim to reduce the overall complexity of the architecture.  If possible, step by step implementation of e-Navigation services would ease the burden on stakeholders, facilitate acceptance, and minimize the impact on operations.

Recommendations specifically related to the content are as follows:

- In Section 2.3, there is a mix of primary driving forces and consequential driving forces. Primary driving forces should be aligned with the definition. 
For example: 
- Safety
	* increased traffic density requires more communication bandwidth and
               decision making tools	 

- Security
            * increased security demands increase the port of call reporting 
               requirements. 

- Efficiency
            * reduced staffing requires more vessels to be handled per VTSO

- Environmental Protection.
            * community awareness drives the need for transparent public information 
	   regarding vessel pollution

- In Section 2.5, “operational requirements need to be analyzed in the framework of the e-Navigation architecture” is back to front. The operational requirements should come first and influence the e-Navigation architecture.

- In Section 4.2, paragraph 8, definitions of “nodes” and “node sites” are needed.

- In Section 4.4, the scope is limited to “shore-based”.  e-Navigation concerns both on-board and the shore side user requirements. Consideration should be given to how the “ship-side” architectural requirements will be handled.  Can the IALA “existing mandate” be referenced?

- In Section 5.1, user requirements and management goals need further definition.

- In Section 5.2 the discussion of VTS and VTMIS should be removed and replaced with a reference to the source definitions in appropriate reference documents such as the VTS Manual or IMO Resolution A.857(20).