Consequentially requiredProposed revision to Rec. ITU-R M.1371-3 Annex 4

Annex 4

Long-range applications

- General

Long range applications should be by interface to other equipment and by broadcast.

- Long Range Applications by Interface to Other Equipment

Class A shipborne mobile equipment should provide a two-way interface for equipment which provides for long-range communications. This interface should comply with IEC 61162 series.

Applications for long-range communications should consider that:

–	The long-range application of AIS must operate in parallel with the VDL. Long-range operation will not be continuous. The system will not be designed for constructing and maintaining real-time traffic images for a large area. Position updates will be in the order of 2-4 times per hour (maximum). Some applications require an update of just twice a day. It can be stated that long-range application forms hardly any workload to the communication system or the transponder and will not interfere with the normal VDL operation.

–	The long-range operational mode will be on interrogation basis only for geographical areas. Base stations shall interrogate AIS systems, initially by geographical area, followed by addressed interrogation. Only AIS information will be included in the reply; e.g. position and static and voyage-related data.

–	The communication system for long-range AIS is not defined in this Recommendation. Inmarsat-C, as part of global maritime distress and safety system (GMDSS) on many vessels, can be a candidate to facilitate the long-range application, but this will not be mandatory. Most of the current Inmarsat-C, but also all other long-range communication systems, do not support the IEC 61162-2 interface. Because the IEC 61162 series will be standard on all future maritime on-board systems, the AIS will be supported by this interface only. This requires for long-range application an active interface box to translate the long-range AIS 61162-2 messages to the required messages suitable for the chosen communication system and vice versa. This active interface can also gather the information which is not available as standard in the AIS. This can be another information system aboard (if installed).

Example configuration:

Operation with Inmarsat-C.

The general set-up of the long-range configuration is in Fig. 25.

Because of the lack of IEC 61162-2 interfaces on long-range communication systems, the configuration shown in Fig. 26 can be used as an interim solution.

- Long Range Applications by Broadcast

Long range AIS receiving systems may receive long range AIS broadcast messages, provided these messages are appropriately structured and transmitted to suit the receiving systems.

  - Packet bit structure for the AIS long range broadcast message

Long range AIS receiving systems require suitable buffering in order to preserve the integrity of the AIS message in the AIS slot boundaries. Table 1 shows a modified packet bit structure that is designed to support reception of AIS messages by satellites with orbital altitudes up to 1000 km.

TABLE 1

Modified packet bit structure for long range AIS message reception

| Slot composition | Bits | Notes |
| --- | --- | --- |
| Ramp up | 8 | Standard |
| Training sequence | 24 | Standard |
| Start flag | 8 | Standard |
| Data field | 96 | Data field is 168 bits for other single-slot AIS messages. This field is shortened by 72 bits to support the long range receiving system buffer |
| CRC | 16 | Standard |
| End flag | 8 | Standard |
| Long range AIS receiving system buffer | 96 | Bit stuffing = 4 bits Synch jitter (mobile station) = 3 bits Synch jitter (mobile/satellite) = 1 bit Propagation time delay difference = 87 bits Spare = 1 bit |
| Total | 256 | Standard (NOTE – Only 160 bits are used in the 17 millisecond transmission) |

  - Long range AIS broadcast message

The long range AIS broadcast message data field is shown in Table 2.

TABLE 2

Data field for long range broadcast message ; Message 27

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for this message ; always 27 |
| Repeat indicator | 2 | Always 3 |
| User ID | 30 | MMSI number |
| Position accuracy | 1 | As defined for Message 1 |
| RAIM flag | 1 | As defined for Message 1 |
| Navigational status | 4 | As defined for Message 1 |
| Longitude | 18 | Longitude in 1/10 min (180º, East = positive, West = negative |
| Latitude | 17 | Latitude in 1/10 min (90º, North = positive, South = negative |
| SOG | 6 | Knots (0-62); 63 = not available = default |
| COG | 9 | Degrees (0-359); 511 = not available = default |
| Status of current GNSS position | 1 | 0 = Position is the current GNSS position; 1 = Reported position is not the current GNSS position = default |
| Spare | 1 | Set to zero, to preserve byte boundaries |
| Total number of bits | 96 |  |

NOTE : There is no time stamp in this message. The receiving system is expected to provide the time stamp when this message is received.

  - Transmission method for the long range AIS broadcast message

The long range AIS broadcast message should be transmitted by the AIS Class A stations at the current power setting using the transmission interval, access scheme, AIS shore station qualifier and designated frequencies specified below. The AIS Class A should perform the long range AIS broadcast as a transmit-only function.

    - Transmission interval

- The nominal transmission interval for the long range AIS broadcast message should be 3 minutes.

    - Access scheme

The access scheme for transmitting the long range AIS broadcast message should be RATDMA. The AIS should consider only the two frequencies in the AIS VDL for qualifying the candidate slots for the transmission of the long range AIS message. The AIS VDL is comprised only of channel A (by default AIS 1) and channel B (by default AIS 2). The long range AIS broadcast on the long range frequencies should not be considered part of the AIS VDL for the purpose of scheduling the RATDMA transmissions of the long range AIS broadcast messages.

    - AIS shore station qualifier

- Transmission of the long range AIS broadcast message should be suppressed when the AIS Class A station is in range of an AIS base station. This should be implemented by reset of the 3-minute timer for the nominal transmission interval upon receipt of a Message 4 (the AIS base station message).

    - Designated frequencies

The long range AIS broadcast message should be transmitted on channel 75 (156.775 MHz) and channel 76 (156.825 MHz). These channels are designated as 0075 and 0076 by Recommendation ITU-R M.1084-4. The transmissions should alternate between these two channels such that each channel is used once every 6 minutes.