Executive Summary

IALA World Wide Radio Communications Plan

International Association of Marine Aids to Navigation and Lighthouse Authorities

The International Association of Marine Aids to Navigation and Lighthouse Authorities (IALA) exists to:

- harmonise standards for aids to navigation systems worldwide;

- facilitate the safe and efficient movement of shipping, and;

- enhance the protection of the maritime environment.

This World Wide Radio Communication Plan (WWRCP) is being developed by IALA to assist in the selection of radio communication systems required to support e-Navigation. The current status of radio communication in the Maritime Mobile Band is discussed and forms the basis of projections for future developments needed to support e-Navigation.

This WWRCP could be the basis for a possible future submission to ITU as a contribution to the continuing efficiency improvements with respect to radio spectrum.

The plan provides guidance to IALA members regarding potential future developments, which will enable members to identify areas requiring resource allocation and research activity.

Glossary of terms

| AIS | Automatic Identification System |
| --- | --- |
| ARPA | Automatic Radar Plotting Aid |
| AtoN(s) | Aid(s) to Navigation |
| CDMA | Code Division Multiple Access |
| CS | Coastal Surveillance (Radar) |
| DGNSS | Differential Global Navigation Satellite System |
| DGPS | Differential Global Positioning System |
| eANSI | Electronic Aids to Navigation System Information |
| ECDIS | Electronic Chart Display Information System |
| EGNOS | European Geostationary Navigation Overlay System |
| eLoran | Enhanced Loran |
| ENC | Electronic Navigation Chart |
| FAA | (US) Federal Aviation Authority |
| GAGAN | GPS-Aided Geo Augmented Navigation (System) (India) |
| GALILEO | European GNSS (not an acronym) |
| GBAS | Ground Based Augmentation System |
| GEO | Geo-stationary Orbit |
| GMDSS | Global Maritime Distress & Safety System |
| GLONASS | Global Navigation Satellite System (Russian Federation) |
| GNSS | Global Navigation Satellite System |
| GPS | Global Positioning System (US) |
| HF | High frequency |
| IALA | International Association of Marine Aids to Navigation and Lighthouse Authorities |
| IBS | Integrated Bridge Systems |
| IEC | International Electrotechnical Commission |
| IGSO | Inclined Geosynchronous Orbit |
| IHO | International Hydrographic Organisation |
| IMO | International Maritime Organization |
| INS | Inertial Navigation System |
| IRNSS | Indian Regional Navigational Satellite System |
| ITU | International Telecommunication Union |
| LORAN | Long Range Navigation system |
| LRIT | Long Range Identification & Tracking |
| MBOC | Multiplex Binary Offset Carrier |
| MEO | Medium Earth Orbit |
| MF | Medium Frequency |
| MSAS | Multi-Satellite Augmentation System (Japan) |
| PNT | Position , Navigation & Timing |
| QZSS | Quasi-Zenith Satellite System |
| RACON | RAdar BeaCON |
| RAIM | Receiver Autonomous Integrity Monitoring |
| RNAV | Radionavigation |
| SBAS | Satellite Based Augmentation System |
| SDCM | System for Differential Corrections and Monitoring (Russian Federation) |
| SOLAS | Safety of Life at Sea (IMO Convention) |
| TDMA | Time Division Multiple Access |
| UHF | Ultra High Frequency |
| VDR | Voyage Data Recorder |
| VHF | Very High Frequency |
| VTS | Vessel Traffic Services |
| WAAS | Wide Area Augmentation System (US) |
| WWRNP | World Wide Radio Navigation Plan |
| WWRNS | World Wide Radio Navigation System |

Contents

|  | Executive Summary | 1 |
| --- | --- | --- |
|  | Glossary of Terms | 2 |
| 1 | Introduction | 4 |
| 1.1 1.2 | General Scope and Objectives | 4 5 |
| 2 | Background to Service Provision | 6 |
| 2.1 2.2 2.3 2.4 2.5 2.6 | General Institutional Regulatory Commercial Operational Technical | 6 6 6 6 7 8 |
| 3 | The IALA World Wide Radio Communication Plan | 9 |
| 3.1 3.2 3.2.1 3.2.1.1 3.2.1.2 3.2.1.3 3.2.2 3.2.2.1 3.2.2.2 3.2.2.3 3.2.2.4 3.2.2.5 3.2.2.6 3.2.2.7 3.3 3.3.1 3.3.2 3.3.3 3.4 3.4.1 3.5 3.5.1 3.5.1.2 3.5.1.3 3.5.1.4 3.5.2 3.5.2.1 3.5.2.2 3.5.2.3 3.5.2.4 3.5.2.5 3.5.2.6 3.5.2.7 | Overview of existing communication systems. Distress and safety Communication MF/HF DSC Voice NBDP VHF DSC Voice EPIRB 121.5MHz Beacon AIS-SART Inmarsat Radar SART Maritime safety Information promulgation NAVTEX NBDP EGC Safety of Navigation AIS General Communication MF/HF Voice Analogue Digital Data Digital VHF Voice Analogue Digital INM-C Data INM-B Voice Digital Data INM-F Voce Digital Data |  |
| 4 |  | 19 |
| 4.1 |  | 19 |
| 5 | Delivering the Plan | 20 |
| 5.1 | Implementation | 20 |

- Introduction

  - general

The International Association of Marine Aids to Navigation and Lighthouse Authorities (IALA) was formed in 1957 as a non-government, non-profit making, technical association that provides a framework for aids to navigation authorities, manufacturers and consultants from all parts of the world to work with a common effort to:

      - harmonise standards for aids to navigation systems worldwide;

      - facilitate the safe and efficient movement of shipping, and;

      - enhance the protection of the marine environment.

The functions of IALA include, among other things:

- developing international cooperation by promoting close working relationships and assistance between members;

- collecting and circulating information on recent developments and matters of common interest;

- liaison with relevant inter-governmental, international and other organisations.  For example, the International Maritime Organization (IMO), the International Hydrographic Organisation (IHO), the International Electrotechnical Commission (IEC), and the International Telecommunication Union (ITU);

- liaison with organisations representing the aids to navigation users;

- addressing emerging navigational technologies, hydrographic matters and vessel traffic management;

e-Navigation

e-Navigation is an International Maritime Organization (IMO) led concept based on the harmonisation of marine navigation systems and supporting shore services driven by user needs.

The working definition of e-Navigation as adopted by IMO is :

“e-Navigation is the harmonised collection, integration, exchange, presentation and analysis of maritime information onboard and ashore by electronic means to enhance berth to berth navigation and related services, for safety and security at sea and protection of the marine environment”

There are 3 key elements or strands that must first be in place before e-Navigation can be realized:

- Electronic Navigation Chart (ENC) coverage of navigational areas,

- a robust electronic positioning (navigation and timing) system (with redundancy), and

- an agreed infrastructure of communications to link ship and shore.

Vision

The aim of IALA is:

Fostering the safe, economic and efficient movement of vessels by improvements and harmonisation of aids to navigation worldwide

while the Vision of IALA is

Safe marine navigation in a world of:

- Larger and faster ships

- Changing economy & technology

- Stringent standards

- Holistic approach (e-NAV)

- Changing waterway use

With this in mind, IALA has taken an initiative as part of the strategy for the future of e-Navigation by developing a World Wide Radio Navigation Plan supported by this  World Wide Radio Communication Plan.

  - scope and objectives

The 3 key elements for e-Navigation are identified above. However, this document focuses mainly on the need for an agreed infrastructure of communications between ships and ashore. It presents IALA’s view on current, developing and future Radio Communication Systems for the maritime sector.

e-Navigation will require appropriately designed radio communication systems for a robust and reliable service to the mariner. This document is aimed at assisting in the formulation of policy for National and International spectrum allocation and usage.

It is also provided for IALA members and other administrations to assist them in offering their proposed use of radio spectrum as part of the on going studies at ITU.

The core objective of this document is to state the IALA vision for the efficient use of Radio Spectrum in the Maritime Mobile Service.

- Background to Service Provision

  - general

The challenge facing radio communication in the maritime sector is pressure on the availability of spectrum from other users and the requirement to relay more and more information over fewer and fewer channels.

Spectrum as a finite resource is under increasing demand from all users globally. Recent trends from national licensing/regulatory authorities have been to use pricing/cost to drive efficiency improvements.

Scenario of private provision of MMS

The moves from analogue to digital and from voice to data show the way forward for future radio communications. Time sharing of voice and data communications on a common channel is a recent technological advance that can be applied to e-Navigation.

Recent advances with TDMA techniques, AIS is the prime example, have shown the way to have several hundred users effectively and efficiently using only two channels to share vast amounts of data.

Selective of appropriate bandwidth given the constraints of required range, data rates and channel availability means that narrow band techniques should be considered.

  - institutional

Significant changes have occurred in the world wide marine institutional environment during the last five years:

In the near future, there will be a need for ever-closer co-operation between IMO, IHO and IALA in pursuit of the emerging e-Navigation concept.

Close co-ordination with ITU will be required to achieve the step change that e-Navigation could deliver.

- 2.3	REGULATORY

The carriage requirement for radio communication equipment comes from IMO. This includes AIS, GMDSS, DSC and voice communications and radar systems.

Radio Spectrum is regulated by ITU this includes not only the frequencies but the technologies and standards for the systems employed.

The WWRCP has meet the requirements of both ITU and IMO when meeting the needs of the maritime industry.

There are moves towards charging “market rates” for the use of radio spectrum in some countries and proposals may be put forward for adoption of this approach internationally.

- 2.4	commercial

Growth of the world fleet places greater stress on the ability of the existing radio communications systems to cope with the traffic. Voice systems that occupy a channel for the duration of the call can be viewed as an inefficient means of relaying information. As the number of vessels increases, a saturation point will be reached where no more calls can be made. Systems using TDMA, such as AIS, can cope with multiple simultaneous users and are much more immune to overloading due to growth in the number of participants.

The option of more channels in the Maritime Mobile Service does not exist so alternatives have to be found to cater for future growth.

- 2.5	OPERATIONAL

There are many changes taking place in the operational environment that present new challenges including:

- IMO’s and IALA’s support of e-Navigation

- the widespread and growing reliance upon GNSS and its role underpinning navigation, situational awareness and communications for e-Navigation;

- Growing deployment of local and specific Traffic Management Schemes to meet ever more stringent requirements at higher capacity levels; and

- the balance between traditional navigation skills and the role of new technological advances such as ECDIS and IBS.

The introduction of GNSS has encouraged mariners to navigate in areas and conditions in which they would not previously have done and the introduction of e-Navigation will further change the way that ships operate.  As part of its introduction, it is essential to understand what happens when key e-Navigation components (e.g. GNSS) fail or are denied.  Getting the human factors part of this right is also critical: before adopting the technology - safety, liability, on-board training and duty of care must all be considered.

e-Navigation means that international bodies (IMO, IHO, IALA etc) must work more closely together as the concept encompasses all their areas of responsibility.

- 2.6	technical

Significant changes to underpinning services and systems are expected over the next two decades:

- development of data communication systems by ITU

- development of AIS satellite detection

- regulatory constraints from ITU, interference

- compatibility with existing systems and services

- application of Wi-Fi and Wi-Max in short range (port)

- adding data services to current voice channels.

- Change from analogue voice to digital voice

- The IALA World Wide Radio Navigation Plan

  - OVERVIEW

This section gives a brief summary of the existing and future Radio communication systems.

The classification follows the ITU format.

.

## Distress and safety Communication

### MF/HF

#### DSC MF/HF is a technique using digital codes which enables a radio station to establish contact with, and transfer information to, another station or group of stations, for distress or general communications over medium to long range distances. DSC is primarily intended to initiate ship-to-ship, ship-to-shore and shore-to-ship radiotelephone and MF/HF radiotelex calls. DSC calls can also be made to individual stations, groups of stations, or "all stations" in one's reach. Each DSC-equipped ship, shore station and group is assigned a unique 9-digit Maritime Mobile Service Identity. DSC distress alerts, which consist of a preformatted distress message, are used to initiate emergency communications with ships and rescue coordination centers. DSC was intended to eliminate the need for persons on a ship's bridge or on shore to continuously guard radio receivers on voice radio channel 2182 kHz used for distress, safety and calling.

#### Voice MF/HF are medium and long range  radio communications which are accomplished by means of analog or digital radiotelephone signals for operational and public correspondence traffic and distress, urgency and safety.

### NBDP (Narrowband Direct Printing) is a technique which automates radio signals to telegraphy.

### VHF

#### DSC is a technique using digital codes which enables a radio station to establish contact with, and transfer information to, another station or group of stations, for distress or general communications within line of site using channel 70 (156.5250). DSC is primarily intended to initiate ship-to-ship, ship-to-shore and shore-to-ship radiotelephone and VHF radiotelex calls. DSC calls can also be made to individual stations, groups of stations, or "all stations" in one's reach. Each DSC-equipped ship, shore station and group is assigned a unique 9-digit Maritime Mobile Service Identity. DSC distress alerts, which consist of a preformatted distress message, are used to initiate emergency communications with ships and rescue coordination centers. DSC was intended to eliminate the need for persons on a ship's bridge or on shore to continuously guard radio receivers on voice VHF channel 16 (156.8 MHz) used for distress, safety and calling.

#### Voice

#### EPIRB Distress radio beacons, also known as emergency beacons, are tracking transmitters which aid in the detection and location of boats, aircraft, and people in distress. They are radiobeacons that interface with Cospas-Sarsat, the international satellite system for search and rescue (SAR). When activated, such beacons send out a distress signal that, when detected by non-geostationary satellites, can be located by triangulation

#### 121.5MHz Beacon The frequency 121.5 MHz is an aeronautical emergency frequency. 121.5 MHz radiobeacons were developed in the mid-seventies for installation on aircraft, as Emergency Locator Transmitters (ELTs). However, they can also be used on board ship as Emergency Position-Indicating Radio Beacons (EPIRBs) or as Personal Locator Beacons (PLBs). These beacons transmit signals that are relayed by Cospas-Sarsat LEOSAR satellites to Cospas-Sarsat LEOLUTs which process the signals to determine the beacon location.

#### AIS-SART The AIS-SART is used to locate a survival craft or distressed vessel by sending updated position reports using a standard AIS class A position report. The position and time synchronization of the AIS-SART is derived from a built in GNSS receiver.

#### Inmarsat Inmarsat is an international satellite communications company providing voice telephony and high speed internet connections for use on land, sea and in the air.

#### Radar SART The radar-SART is used to locate a survival craft or distressed vessel by creating a series of dots on a rescuing ship's radar display. A SART will only respond to a 9 GHz X-band (3 cm wavelength) radar.

## Maritime safety Information promulgation

### NAVTEX (Navigational Telex) is an international, automated system for instantly distributing maritime navigational warnings, weather forecasts and warnings, search and rescue notices and similar information to ships. A small, low-cost and self-contained "smart" printing radio receiver installed in the pilot house of a ship or boat checks each incoming message to see if it has been received during an earlier transmission, or if it is of a category of no interest to the ship's master. The frequency of transmission of these messages is 518 kHz in English, while 490 kHz is use to broadcast in local language. The messages are coded with a header code identified by the using alphabets to represent broadcasting stations, type of messages, and followed by two figures indicating the serial number of the message.

### NBDP (Narrowband Direct Printing)  TOR (Teletype over Radio) a term used to describe a method of sending Maritime Safety Information over the radio and having it automatically printed.

### EGC (Safety NET) The Inmarsat C maritime mobile satellite system has an inherent capability, known as Enhanced Group Call (EGC), which allows broadcast messages to be made to selected groups of ship stations located anywhere within a satellite's coverage. Four geostationary satellites provide worldwide coverage for Safety NET. SafetyNET, along with NAVTEX, is recognized by the GMDSS as the primary means for disseminating maritime safety information. Ships regulated by the Safety of Life at Sea Convention travelling outside areas covered by NAVTEX must carry an Inmarsat C SafetyNET receiver.

## Safety of Navigation

### AIS (Automatic Identification System) is a system used by ships and Vessel Traffic Services (VTS) principally for identification and locating vessels. AIS provides a means for ships to electronically exchange ship data including: identification, position, course, and speed, with other nearby ships and VTS stations. This information can be displayed on a screen display. AIS is intended to assist the vessel's watchstanding officers and allow maritime authorities to track and monitor vessel movements.

## General Communication

### MF/HF	 Medium frequency (MF) refers to radio frequencies (RF) in the range of 300 kHz to 3000 kHz. High frequency (HF) radio frequencies are between 3 and 30 MHz. MF/HF is used for medium to long range radio communications.

#### Voice	Analogue

#### Digital

#### Data	Digital

### VHF	 VHF radio is commonly used for short-range line-of sight (LOS) communications. Maritime VHF communications operate in the 156 -162 MHz radio frequency band.

#### Voice	Analogue

#### Digital

#### INM-C	Data is a two-way, packet data service operated by the telecommunications company Inmarsat. The service is approved for use under the Global Maritime Distress and Safety System (GMDSS), meets the requirements for Ship Security Alert Systems (SSAS) defined by the International Marine Organization (IMO) and is most widely used service in fishing Vessel Monitoring Systems (VMS). The service offers data transfer; e-mail; SMS, crew calling; telex; remote monitoring; tracking (position reporting); chart and weather updates; maritime safety information (MSI); maritime security; GMDSS; and SafetyNET and FleetNET services. The service is operated via an Inmarsat-C Transceiver or a lower-power mini-C Transceiver. Both offering and approved for the same service.

#### INM-B	Voice	Digital The Inmarsat B digital mobile satcoms system provides two-way direct-dial phone, telex, facsimile and data communications at rates up to 9.6kbit/sec to and from anywhere in the world with the exception of the polar regions.

#### Data

#### INM-F	Voice	Digital

#### Data

- 5	Delivering the Plan

- 5.1 	CONCLUSIONS

IALA Members are encouraged to participate in realising the robust position-fixing element of e-Navigation by:

- Contributing to the development of e-Navigation;

- Contributing to the development of GNSS and Space Based Augmentation Systems – by representing marine interests to the suppliers of these systems;

- Continuing to provide Internationally recognised DGNSS Services, by:

  - recapitalising the existing DGNSS infrastructure as required to avoid system obsolescence;

  - reviewing their service provision to take account of GNSS developments;

  - continuing to encourage and support the IMO’s acceptance of IALA radiobeacon DGNSS into the World Wide Radionavigation System (WWRNS); and by

  - investigating the standardisation and delivery of high precision DGNSS to support aids to navigation management, harbour approach and docking;

- Seeking to provide sufficient appropriate levels of contingency support  for GNSS, including:

  - where Loran/Chayka facilities exist within their jurisdiction to retain them in operation and make plans to upgrade them to eLoran capability ;

  - giving full support to the development and standardisation of eLoran, so that the system can be recognised as a component of e-Navigation;

- Responding to Developments in Marine Radar Technology – which may require the replacement of existing radar beacon technology with units adapted to new standard low power radars;

- Developing AIS as an Aid to Navigation – including synthetic and virtual aids to navigation where appropriate;

- Continuing to trial and develop new systems to enhance marine navigation;

- Continuing to provide short range aids to navigation as the ultimate fallback position, based on risk analysis - taking into account all potential changes in the service provision environment. This includes trends, types, volume and mix of traffic, local hazards, areas of traffic convergence/separation, environmental considerations and changes to other risk mitigation measures;

.