- Rapporteurs Report

- Development in non-radio navigation systems

- (Task M2)

- 1. Background

This report, prepared for the e-Navigation Committee, provides information about ongoing developments in the field of non-radionavigation systems. Furthermore developments of INS (Integrated Navigations Systems) which combine radio navigation and non-radionavigation systems will be reported. Most of the information is based on published material, conference proceedings and contacts with manufacturers in this area.

The IALA e-Navigation Committee has developed a “List of Technologies” about present and emerging technical systems to assist the navigation of ships and associated shore-based functions. One of the listed  concepts to support the positioning system of a vessel is the use of an ECDIS map together with echo sounder measurements. The following report will briefly describe a similar system which make use of a map matching algorithm based of a digital terrain model (DTM)  together with measurements taken from a multibeam echo sounder (MBE) or a single beam echo sounder (SBE).

- 2. Terrain Referenced  Navigation

The terrain navigation system (TNS) can be viewed as an independent component of the navigation system whose primary function is to provide position measurements. Such a TNS could  be integrated together with the inertial navigation system (INS) and the connected GNSS receiver. In other systems it might integrate with a dead-reckoning system or serve as an independent position source. Terrain navigation can also enhance the total navigational integrity by providing a position estimate supplementary to INS and GNSS. This is of interest for instance for vessels when they operate in an area where GNSS can be subject to jamming or spoofing. TNS are currently tested and used for navigation in autonomous underwater vehicles (AUVs) and on submarines.

Figure 1 shows the system concept of a TNS  connected to bathymetric sensors and the onboard  navigation system. The illustrated Terrain Correlation Processor consists generally of the following main parts:

- The Terrain Correlator which runs the terrain navigation algorithms on geographically referenced depth data.

- The Geographic Data Producer which  converts body-fixed depth sensor measurements to geographical referenced data using position and attitude data from the INS and

- Map Database which contains the digital terrain model (DTM e.g. bathymetric map)

Figure 1:	Terrain navigation system concept
		(DVL: Doppler Velocity Log; MBE: Multibeam echo sounder; SBE: Single beam echo sounder)

Terrain Correlation Processor

Integrated Navigation System (GNSS, IMU)

Geographic Data

Producer

Map

Database

(DTM)

Terrain

Correlator

DVL

SBE

MBE

Navigation Data (e.g. attitude)

Position

The terrain correlation processor can be connected to INS to run the terrain correlation. Calculated position fix and associated uncertainty is sent back to the navigation system to limit the position error drift. As shown in Figure 2 the terrain correlation processor runs with any sensor providing bathymetric data, for instance multibeam echo sounder, single beam echo sounder (SBE) and/or DVL.

One essential problem for the use of the TNS is that the available digital terrain models (DTM) often have poor resolution and accuracy, except for dedicated areas were accurate high resolution (1 – 10 m horizontal resolution) DTMs have been surveyed. Terrain navigation accuracy depends on algorithm characteristics, sensor accuracy, map accuracy, map resolution and not least terrain suitability. All terrain navigation algorithms need terrain variation to work at all.

- 3. Conclusions

The TNS shortly described in this report seems to be an interesting concept for providing an additional backup position input for maritime navigation. At present this technology is mainly used on AUVs and submarines where GNSS positioning is not available. Nevertheless the concept could also be used as a backup positioning source on maritime vessels, when they are equipped with:

- bathymetric sensors like MBE or SBE and

- digital terrain maps with sufficient resolution accuracy (e.g. in difficult navigation areas) if available.

M. Hoppe, March 2009

Björn Jalving, Magne mandt, Ove Kent Hagen, Norwegian Defence Research Esatblishment,; F Pøhner, Kongsberg Maritime, Norway, “Terrain referenced Navigation of AUVs and submarines using multibeam echo sounders”, UDT, Hawaii, October 2004