e-Navigation Committee							e-NAV 7/5/2

7th Session									21 Sept 2009

GNSS Update

1.	Introduction

This rapporteur’s update on GNSS is provided for e-NAV 7, based on information published in journals and on websites.

2.	GPS

There are 30 operational satellites in the constellation. Around 12 of these are operating without redundancy in one or more of their essential systems, however, some satellites are being held in reserve rather than fully decommissioned so that they can if necessary be re-positioned to provide backup. Deployment of replacement satellites has been held up by problems with components and earlier with the launch vehicle. Block IIR(20)-M, modified to transmit on L5 was launched on March 24 to secure the frequency by the ITU deadline in August. It is transmitting codeless L5, but otherwise is not usable, because of signal anomalies. The last of the eight Block IIR-Ms was launched successfully on August 17, and declared usable on 27 August (SVN 25 was decommissioned a few days earlier).  The launch of IIF satellites has been further delayed by component problems in the L2 transmitter. The six operational IIR-M satellites have been broadcasting the L2C signal, without data or modulation, but newly uploaded software will allow them to provide the new signal with a message structure, which has several advantages over L1.

The next generation GPS IIIA is undergoing its Preliminary Design Review, leading to the development and production of the first two satellites with a planned launch in 2014. However, funding has been cut because of contractual delays.

The April 2009 United States General Audit Office (GAO) GPS Report identified serious programmatic / system management problems such that the probability of having the core 24-satellite constellation may fall to 0.8 over the next few years and hence have a significant impact on GPS end-to-end services, although more recent US statements have downplayed these concerns

3.	GLONASS

17 satellites are in operation, with two in maintenance. A further 6 are expected to be deployed in two launches in September and December this year. The Russian Space Agency (Roscosmos), has declared that GLONASS accuracy will approach that of GPS from 2010 and that the constellation will increase to 30 satellites by 2011.  Of the present constellation, 19 are modernised and 17 are broadcasting a second full civil signal on L2. It is claimed that the present constellation gives 83% global coverage, although the monitoring sites are still limited to Russian territory. The new GLONASS K satellites will start launching in 2010 and CDMA signals on L1 & L5 are promised.

4.	GALILEO

Contracts have been awarded for the launch of the first four operational satellites by the end of 2010 and for procurement of long-lead items to construct the satellite platforms and navigation payloads. The award of the main contract for the satellites is expected before th end of 2009.and contractors have been short-listed for manufacturing the space and ground segments for the operational system.. Suggestions of a simplified system to ensure timely delivery seem to have been resisted by the Commission, which is still committed to the Public Regulated Service, although negotiations with China over spectrum separation have not yet been completed. The first operational satellites are due for launch in 2010, with Full Operational Capability scheduled for 2013.

5.	COMPASS

The frequency plan for the Chinese system, Beidou/Compass, has not yet been finalised, but  launch plans have been announced, with ten satellites due to be launched in the next two years to provide PNT capability in the Asia-Pacific region.

The constellation is eventually planned to comprise 5 geo-synchronous and 30 MEO satellites, providing 10 services, 5 of them open, with a standalone accuracy of 10 m and a real-time wide area differential service giving 1 m. Completion date has recently been announced as 2015.

6.	QZSS

This system is under development by the Japan Aerospace Exploration Agency and will be interoperable and compatible with GPS, providing signals at L1, L1C, L2C and L5, including a sub-meter augmentation with integrity function. Coordination meetings have been held with the EU Galileo signal task force to ensure RF compatibility and efforts are also being made to secure RF compatibility with Compass.

7.	SBAS

The WAAS Performance Standard has been published, defining the signal in space characteristics, navigation message and performance requirements. The service, provided for aircraft approach, is designed to give 16 m horizontal and 20 m vertical accuracy, with 6.2 sec time to alert.

The Safety of Life service of EGNOS is undergoing qualification testing, with certification and declaration of “entry into service” planned for the second half of 2009. Once this service achieves certification under the EU’s Single European Sky regulations it will be available for use by the aviation sector and for maritime and other transport sectors. An operational Signal in Space has been available for the past year and the Open Service should be declared usable shortly. Receivers currently available on the consumer market for leisure craft already use the open signal.

The Japanese MSAS has been commissioned for aviation use, with two MTSATs operating. The Indian GAGAN system has carried out test transmissions with one Inmarsat 4 satellite and a contract has been awarded for the ground stations.