| From: e-NAV Committee | e-NAV7/output/6 |
| --- | --- |
| To: PAP | 25 September 2009 |

Liaison Note

Introduction to the IALA Universal Maritime Data Model (UMDM)

# Introduction

After Council’s approval for the submission to IMO NAV55 of paper NAV55/11/1 (IALA’s work on e-Navigation) the e-NAV Committee have progressed the development of the IALA UMDM, in accordance with the following statement made in the IMO submission:

‘The high-level needs noted in the report of NAV54 include a common maritime information / data structure and IALA is progressing towards an IALA Maritime Data Model and Data Object Identifiers.’

# Current status of work

At Annex A is the introduction to the envisaged IALA UMDM, which if accepted will mean that IALA becomes the registrar of the UMDM.

It is recognised that the procedural aspects of IALA becoming the UMDM registrar and the workload for the IALA Secretariat and e-NAV Committee need to be determined.  This will become clearer as inter-sessional work progresses, which may be as soon as May 2010 if the e-NAV Committee can meet that month.

# Action requested

The Policy Advisory Panel is requested to consider the document at Annex A, with a view to:

- forwarding it to Council, with a request that:

  - the concept be accepted, together with the consequential role of UMDM registrar attaching to IALA;

  - the document be forwarded as an information paper to IMO NAV56;

  - the document be published on the IALA website.

- Introduction to the IALA Universal Maritime Data Model

Introduction

This paper introduces the technical concept of describing maritime information in a user-friendly, simple and consistent manner using the IALA Universal Maritime Data Model, a consequence of the IMO e-Navigation strategy.

The audience for this paper is Maritime and Inland Waterways Communities, such as, Contracting Governments of the IMO, Coast Guards, Port and Harbour Authorities, Waterway Authorities, Manufacturers and Suppliers of ship and shore equipment and software, Service Providers, Specifiers of Equipment and Services, and other interested parties.

Implementation of IMO’s e-Navigation strategy leads to a larger variety and higher volume of information and increased information exchange due to globalization. Consequently there is a need to handle information more effectively in a standardized way. IALA realized that a common data model that expressly includes ship and shore data is essential. IALA designed a data model that meets these characteristics but is also extendable so that it can grow as more end user requirements are identified and captured. This data model is named the IALA Universal Maritime Data Model (UMDM).

Universal Maritime Data Model

The IMO has defined e-Navigation as follows:

E-navigation is the harmonized collection, integration, exchange, presentation and analysis of marine information on board and ashore by electronic means to enhance berth to berth navigation and related services for safety and security at sea and protection of the marine environment.[IMO NAV 54/25 Report Annex 12 Section 1.0]

Specifically for integration and exchange of maritime information the IMO has stated in the same report (italics added):

“Mariners require information pertaining to the planning and execution of voyages, the assessment of navigation risk and compliance with regulation. This information should be accessible from a single integrated system. Shore users require information pertaining to their maritime domain, including static and dynamic information on vessels and their voyages. This information should be provided in an internationally agreed common data structure. Such a data structure is essential for the sharing of information amongst shore authorities on a regional and international basis.”[IMO MSC85 Annex 20 Section 8.2.1]

The first step towards a common data structure is to define the meaning of each and every item in the data structure and the relationships among the items. This is done so that implementers of the data structure have common understanding of items. The means to do this is with a data model. A data model is an inventory of item definitions and blueprints for item relationships, and as such is a tool used to define data structures. The IALA UMDM is a data model specifically designed for maritime data.

The UMDM is an abstract representation of the maritime domain. Specifically, it represents the entities and relationships among the entities that exist in this domain but does not represent processes. The purpose of a data model is to give users a common understanding of the entities and relationships.

The UMDM is neither a database nor an interface. The model contains no details about physical representation of the entities within it. However, the UMDM can be used to guide the development of databases and interfaces, which are the physical representations of entities. By defining a common reference, all implementers can be assured that their understanding of the entities required agrees with the understanding of others.

The UMDM is flexible and extendable for meeting future requirements. New entities can be added to the model by any e-Navigation stakeholder through a process known as registration. Once registered, an entity is available to all stakeholders; there is open, public access. The UMDM also has provisions for proprietary entities that do not require registration. The model is modular and an appropriate subset is easily referenced.

The UMDM can represent any maritime entity, can be extended by the addition of new entities, and is accessible to any e-Navigation stakeholder or implementer, thus it is considered to be universal in the maritime domain.

As such, the UMDM should be unique. A parallel can be made between the UMDM and the Internet. Although it is possible to have multiple global networks, the true benefits of such a network would only be achieved if there is only one global network.

The parallel extends even further as the Internet provides a series of standardized network services from which the client can choose only the ones which he or she requires, e.g, email, ftp, www, etc. Hence, e-Navigation shore-based stakeholders can choose to implement only the UMDM objects that they require, as long as the objects implemented are compliant with the UMDM.

The UMDM will require a governing body to fulfil the role required for the management of the model. The UMDM management role is addressed further in the document.

IALA and e-Navigation

The International Association of Marine Aids to Navigation and Lighthouse Authorities (IALA) was formed in 1957 as a non-government, non-profit making, technical association that provides a framework for aids to navigation authorities, manufacturers and consultants from all parts of the world to work with a common effort to:

- harmonise standards for aids to navigation systems worldwide;

- facilitate the safe and efficient movement of shipping, and;

- further the protection of the maritime environment.

- The main function of IALA is to provide specialist advice and assistance on aids to navigation issues (including technical, organisational and training matters). IALA does this by publishing recommendations and guidelines and organising workshops and symposia.

IMO considers IALA to be a competent body because of its natural role and experience in shore-side matters (IMO MSC86/23/4). Furthermore, IALA previously assisted IMO in developing the e-Navigation strategy (IMO MSC86/26, §23.26). IALA is participating in the implementation plan of the e-Navigation strategy, by capturing user requirements, developing a shore-side architecture and performing gap and cost/ risk analyses (IMO MSC/86/23/4 and IMO draft report of NAV55, §11.11). IALA formed the e-Navigation committee for the purpose of developing recommendations and guidelines on shore-based e-Navigation systems and services.

The IALA e-Navigation Committee’s work includes a suite of recommendations on the shore-side architecture (IALA Recommendation eNAV-101 et al), that is being used by IALA members (and other interested parties) to implement e-Navigation systems and services. One aspect of the IALA e-Navigation architecture is the need for a consistent and unique data model, realized by the IALA Universal Maritime Data Model.

As stated above, the IALA UMDM is universal in nature and can easily be extended to encompass the complete maritime domain. IALA invites the Maritime and Inland Waterways Communities at large to consider using and contributing to the IALA Universal Maritime Data Model.

IALA’s UMDM

IALA will publish a suite of recommendations for the definition and use of the IALA Universal Maritime Data Model. Please see the section Reference Documents, below.

The Universal Data Model concept is composed of these elements:

- UMDM – this is the structure of the data model itself and the data objects it contains, together with a data object definition rule base. In the terminology of UMDM, maritime entities are represented by data objects.

- UDOI – Universal Maritime Data Object Identifier – this is a unique identifier assigned to each object in the UMDM. The UDOI:

- Expedites access of objects

- Facilitates interchange and sharing of objects

- Is used as an index into the Universal Maritime Data Object Registry

- UMOR – Universal Maritime Data Object Registry – this is the mechanism for extending and maintaining the UMDM. There is only one Registry for the UMDM. The Registry is operated by a body known as the Registrar. By providing for organization of objects and their UDOIs, the Registry:

- Eases maintenance of the UMDM

- Facilitates finding and retrieving objects

- Prevents redundancies within the UMDM

- Facilitates global implementation and exchange of objects through standardization

- Rules for management of the UMDM

As previously mentioned, the UMDM requires a governing body to ensure proper management of the model. There may be several management roles to be performed by the governing body, one of which is the Registrar. The different management roles may be delegated at the discretion of the governing body. Hence, the Registrar may not be the same organization as the governing body. The tasks of the governing body always comprise supervision for the Registrar, however.

Initially, IALA will operate as the governing body and Registrar for the IALA UMDM. Once brought to a certain degree of maturity, the IALA UMDM would be offered to IMO as a contribution to the implementation of IMO’s e-Navigation strategy. Then IMO would assume the role as the governing body for the UMDM, thereby guaranteeing the uniqueness of the UMDM within e-Navigation. IALA is prepared to continue as the Registrar for the IMO UMDM. There is precedent for this within IMO, e.g., the registrar for a ship’s IMO number is Lloyd’s Register.

IALA invites the Maritime Waterways Community at large to consider using IALA as the Registrar for the entire Universal Maritime Data Model.

Reference Documents

Draft IALA Recommendation e-NAV 0101 on “The e-Navigation Architecture – the initial shore-based perspective”

Future Draft IALA Recommendation e-NAV 0220 on “The IALA Universal Maritime Data Model (UMDM) – Introduction”

Future Draft IALA Recommendation e-NAV 0221 on “IALA UMDM Objects”.

Information - “knowledge concerning objects, such as facts, events, things, processes or ideas, including concepts, that within a certain context has a particular meaning” (ISO/PAS 16917:2002(E)) or, data made useful by interpretation or analysis.

Data - “representation of facts, concepts or instructions in a formalized manner, suitable or communication, interpretation or processing by humans or by automatic means” (ISO/PAS 16917:2002(E))