Draft Liaison Note to IEC TC 80 AIS WG

Functional requirements for AIS Repeater Station and AIS Limited Base Station

# Introduction

IALA e-NAV committee in its 7 meeting has defined Functional requirements for AIS Repeater Station and AIS Limited Base Station.

# Work Items

Functional requirements for AIS Repeater Station and AIS Limited Base Station for the development of minimum test standards by IEC has been defined.

# Action requested

IEC is requested to:

- Develop a minimum test standard for AIS Repeater Stations and AIS Limited Base Stations.

- Description of a Limited Base Station

# Introduction

## Rationale for the definition of the Limited AIS Base Station

IMO states the purpose of the AIS as follows:

“The AIS should improve the safety of navigation by assisting in the efficient navigation of ships, protection of the environment, and operation of Vessel Traffic Services (VTS), by satisfying the following functional requirements:

- in a ship-to-ship mode for collision avoidance;

- as a means for littoral States to obtain information about the ship and its cargo; and

- as a VTS tool, i. e. ship-to-shore (traffic management).”

This ruling applies to the member states of the IMO, i.e. the contracting governments.  These states can be both port states and littoral states.  By the introduction of the AIS the IMO requests shipping industry to disclose and broadcast potentially sensitive information about ship movements, ship static and voyage related data.  Thus IMO stipulates that only other participating ships and competent authorities, i e. the national competent authority of the member state, may receive and use that information.

In addition, this information may not be used for other purposes than stated above.  The information on the AIS VHF data link is protected by the appropriate stipulations of the Radio Regulations, IMO Resolution MSC.140(76), and additionally by the data protection legislation of the littoral or port states.

In order to use the AIS in its area of competence a littoral or a port state needs to set up a national AIS Service in accordance with the appropriate technical standards created by ITU (Recommendation ITU-R M.1371-3) together with derived technical Recommendations issued by IALA.  The most fundamental means to access the AIS VHF data link for a competent shore authority and thus provide the AIS Service is to deploy and operate AIS Base Station(s).

The AIS Service in a littoral or port state is operated by the national competent authority for the whole of the national coverage area.  This national competent authority has the need to control the AIS Service and the AIS VHF data link, by employing means like channel management, protected slot allocations, and assigned mode.  The AIS Base Station allows for the full control of the AIS VHF data link.

However, there are other competent authorities responsible for local operations of e. g. individual harbours, locks, marinas, off-shore structures.  These regularly have an interest in monitoring the vessel traffic in their locally confined area of competence and exchange limited operational information with ships in regard to their area of competence, e.g. docking scheduling, tugs operation.

The national competent authority is in charge of the full AIS Service functionality, including the control mechanisms.  However, to allow local competent authorities to participate in and benefit from AIS, there are two possible options:

  - connect to the national AIS Service via a shore-based communication link;

  - establish an own locally confined AIS infrastructure with limited capacities.

## General overview on the capabilities and limitations of the LBS

The LBS should be capable of receiving all AIS messages within its radio range.  In the case of addressed AIS messages the LBS only receives messages addressed to itself.  However:

- The LBS may transmit interrogation messages, binary and safety related messages.

- The LBS may transmit Base Station static report but should not provide secondary synchronization.

- The LBS shall also transmit acknowledgements to addressed messages.

- Should the optional Aids-to-Navigation functionality be added then the LBS shall also transmit AtoN-reports.

- The maximum permissible link utilization of an LBS should be 20 slots per frame for all above messages together except acknowledgements.  Interrogation messages should limited to 5 slots in a frame in total.

- The LBS should be prohibited to transmit any controlling messages such as assigned mode commands, DGNSS broadcasts and to perform data link management (FATDMA reservations) and channel management.  The LBS should also not provide secondary synchronisation support, i.e. it should not act as a semaphore.

- Optionally LBS are subject to the channel management messages (TDMA and DSC Channel 70) and assignment messages.  Therefore, the LBS reacts like a mobile station in that it reacts to received Message 20 and 22 and/or DSC-based AIS Channel Management command.

# Comparison of Limited Base Station to Class A Mobile Station and Base Station

Table 1 presents the maximum capabilities of a Limited Base Station in comparison to the Class A Mobile Station and the AIS Base Station.  Limited Base Stations that are less capable than that shown in Table 1 can be achieved by hardware or software configuration means.

- Comparison of transmit capabilities between Limited Base Station Mobile Class A Station and Base Station

| Msg ID | Description | Class A | Limited Base Station1 | Base Station |
| --- | --- | --- | --- | --- |
| 1 | Position Report (SOTDMA) | Yes | nil | nil |
| 2 | Position Report (assigned) | Yes | nil | nil |
| 3 | Position Report (ITDMA) | Yes | nil | nil |
| 4 | Base Station report | nil | Yes2 | Yes |
| 5 | Ship static and voyage related Data | Yes | nil | nil |
| 6 | Binary Addressed Message | Yes | Yes | Yes |
| 7 | Binary Addressed Ack | Yes | Yes | Yes |
| 8 | Binary Broadcast Message | Yes | Yes | Yes |
| 9 | SAR Position Report | nil | Nil | nil |
| 10 | UTC/Date request | Yes | Nil | Yes |
| 11 | UTC/Date | Yes | Nil | nil |
| 12 | Safety Addressed Message | Yes | Yes | Yes |
| 13 | Safety Addressed Ack | Yes | Yes | Yes |
| 14 | Safety Broadcast Message | Yes | Yes | Yes |
| 15 | Interrogation | Yes | Yes | Yes |
| 16 | Assigned mode command | nil | Nil | Yes |
| 17 | DGNSS broadcast | nil | Nil | Yes |
| 18 | Class B position report | nil | Nil | nil |
| 19 | Extended Class B position report | nil | Nil | nil |
| 20 | Data Link Management | nil | Nil | Yes |
| 21 | Aids-to-navigation report | nil | Yes3 | Yes |
| 22 | Channel Management | nil | Nil | Yes |
| 23 | Group assignment command | Nil | Nil | Yes |
| 24a | Static data report | nil | Yes | Yes |
| 25 | Single slot binary message | Yes | Yes | Yes |
| 26 | Multiple slot binary message with ComStat | Yes | Yes | Yes |

Footnote 1: Total Link Load should be less than 20 slots per frame due to RATDMA access scheme.  This applies to Binary Msgs (6, 8), Safety Messages (12, 14), Interrogation (15) and ATON Report (21).  Interrogation messages are limited to 5 slots in a frame.

Footnote 2: No semaphore functionality

Footnote 3: If implemented

All messages will be received and processed accordingly

Other Rules and Limitations of Limited Base Station:

- No retransmission.

- AIS Message 21: There may be a liability issue for the LBS operating authority that applies only if optional AtoN functionality is added to the Limited Base Station.

- The LBS should discard all addressed messages received that are not addressed to the LBS MMSI.  The LBS may have more than one MMSI representing different services connected to the LBS.

- Functional requirements for AIS Repeater Station

(§ reference to ITU-R M. 1371-3)

# Repeater station (§ 2.1.5)

A Repeater station is a non-Controlling Station

# Modes of operation (§ 3.3.2)

For a Repeater there should only be two modes of operation: autonomous and assigned using FATDMA), but no polled mode.

# Repeater Operation (§ 4.6)

The AIS environment may contain one or more repeaters.

A received message requires additional processing before being retransmitted.  The following processing is required:

- Select additional slot(s), required for re-transmitting message(s).

- The communication state of relevant received messages should be changed, and is subject to parameters required by the slot(s) selected for retransmission by the repeater station.

- Repeater behaviour

| Message type | Repeat | Action | Comment |
| --- | --- | --- | --- |
| 1 | yes | Increase repeat indicator Amend ComState |  |
| 2 | yes | Increase repeat indicator Amend ComState |  |
| 3 | yes | Increase repeat indicator Amend ComState |  |
| 4 | no |  | Can’t be used as secondary synch source; Can be done by Base Station with repeater functionality (internally generated) |
| 5 | yes | Increase repeat indicator |  |
| 6 | yes | Increase repeat indicator |  |
| 7 | yes | Increase repeat indicator |  |
| 8 | yes | Increase repeat indicator |  |
| 9 | no |  |  |
| Message type | Repeat | Action | Comment |
| 10 | no |  |  |
| 11 | no |  |  |
| 12 | yes | Increase repeat indicator |  |
| 13 | yes | Increase repeat indicator |  |
| 14 | yes | Increase repeat indicator |  |
| 15 | yes | Increase repeat indicator Disallow repeat of msg 15 with slot offset |  |
| 16 | no |  |  |
| 17 | no |  |  |
| 18 | yes | Increase repeat indicator Amend ComState (check for CS) |  |
| 19 | yes | Increase repeat indicator |  |
| 20 | no |  |  |
| 21 | yes | Increase repeat indicator |  |
| 22 | no |  |  |
| 23 | no |  |  |
| 24 | yes | Increase repeat indicator |  |
| 25 | yes | Increase repeat indicator |  |
| 26 | yes | Increase repeat indicator Amend ComState |  |
| 27 | no |  |  |

# Repeater station use of repeat indicator (§ 4.6.1.2)

The repeated messages always have repeat indicator > 0.

The repeat indicator should be incremented whenever the transmitted message is a repeat of a message already transmitted from another station.

Each time a received message is processed by the repeater station, the repeat indicator value should be incremented by at least one (1) before retransmitting the message.

# Number of repeats (§ 4.6.1.2.1)

a) Using Repeat Indicator

The number of repeats should be a repeater station configurable function, up to three.

The number of repeats should be set to either 1 or 2, indicating the number of further repeats required.

All repeaters within coverage of one another should be set to the same number of repeats, in order to ensure that “Binary acknowledgement” Message 7 and “Safety related acknowledgement” Message 13 are delivered to the originating station.

If the repeat indicator of the received messages equals 3, the message should not be repeated.

b) Using Filter method (only one repeat)

Filter on each received message - only one time repeat (works for messages with time stamp, not for other messages);

A repeater should store its repeated messages between 12 and 30s;

Change repeat indicator to a value other than 0 (check effect on 1371 regarding repeat indicator definition);

# Simplex repeater mode (§ 4.6.3)

This is a store-and-forward process.

In RATDMA mode the message must be repeated within 4 seconds.

In FATDMA mode the message must be repeated within 30 seconds. (Configurable: cautions regarding time delay)

Do not repeat a position report that has been superseded.

If messages can not repeated within the allotted time they are discarded.  (Note: this is because the time stamp does not include the minute information)

Transmissions from the repeater should be on the same channel as the original message.

# Processing functionality (optional) (§ 4.6.3.2)

Filtering should be a function that is configurable by the repeater station.

Filtering of retransmissions should be applied, considering the following as parameters:

- channel

- Message types.

- Message content

- MMSI

- Coverage area.

- Distance/range

- Reporting interval

- VDL load, (channel load threshold)

- Intentional slot reuse

- repeat indicator value

- only one repeat

- age of message

- external input e.g. battery load

# Slot selection (§ 4.6.3.3)

Slot selection should follow the rules according §4.4.1.

In order to assist in slot selection, measurement of received signal strength by the repeater station should be considered.  The received signal strength indicator will indicate when one or more stations are transmitting in the slot (in the previous frame)

# Synchronization

The AIS Simplex Repeater can synchronise on any AIS station that has direct UTC.  Repeater must have UTC direct or UTC indirect and only transmits when sync state is 0 or 1.

# Channel access

RATDMA

FATDMA

The AIS Simplex Repeater should use FATDMA reserved slots for all repeated messages.  If this is not possible, the RATDMA access scheme may be used.

The AIS Simplex Repeater FATDMA allocations can be made by an AIS Base Station on behalf of the AIS Simplex Repeater.  In this case, the AIS Simplex Repeater should be notified of its FATDMA slot reservations for its use by configuration.

Optional SOTDMA

Optional CSTDMA

# Communication State of repeated messages

Applicable for RATDMA and FATDMA access scheme:

SOTDMA Comm.state.

Sync state set to sync state of repeater.

Slot time-out set to zero

Slot increment set to zero

Functional Requirements for Communication State for the Simplex Repeater

The simplex repeater should not change the data content of the message, but should change the SOTDMA communication state or ITDMA if relevant.  The AIS Simplex Repeater should change the SOTDMA communication state as indicated in Table 3

- SOTDMA Communication State of Received Station

| Received Parameter | Simplex Repeater Action |
| --- | --- |
| Synch State | Change to Synch State of Repeater |
| Slot Time Out | Set to zero |
| Slot Offset | Set to zero |

ITDMA Comm state

Sync state set to sync state of repeater.

Slot increment set to zero

Number of slots set to zero

Keep flag set to zero

The AIS Simplex Repeater should change the ITDMA Communication State as indicated in Table 4.

- ITDMA Communication State of Received Station

| Received Parameter | Simplex Repeater Action |
| --- | --- |
| Synch State | Change to Synch State of Repeater |
| Slot Increment | Set to zero |
| Number of Slots | Set to zero |
| Keep Flag | Set to FALSE ( = 0) |

# Interfaces

If applicable the interfaces should be standardised.

# Optional capabilities

Option: Repeater may transmit a msg 6 or msg 8 (Binary message) as status data

Define format ….(similar to msg 6 for an AtoN)

# Functional Requirements for Error Checking for the Simplex Receiver

The AIS Simplex Repeater should only transmit messages with a valid CRC. The repeater should calculate a new CRC for the repeat message prior to transmission.

# Required Parameter Settings for the Physical Layer of the AIS Simplex Repeater Station

The following parameters are required for an AIS Simplex Repeater station.  The Tables 3 to 4 are derived from Recommendation ITU-R M.1371.  For the meaning of the symbols and additional information (footnotes) refer to the appropriate section of Recommendation ITU-R M.1371.

- Required parameter settings for an AIS Simplex Repeater station (part 1)

| Symbol | Parameter Name | Low setting | High setting |
| --- | --- | --- | --- |
| PH.RFR | Regional frequencies | 156.025 MHz | 162.025 MHz |
| PH.CHS | Channel spacing | 25 KHz | 25 kHz |
| PH.AIS1 | AIS 1 (default channel 1) | 161.975 MHz | 161.975 MHz |
| PH.AIS2 | AIS 2 (default channel 2) | 162.025 MHz | 162.025 MHz |
| PH.BR | Bit rate | 9 600 bps | 9 600 bps |
| PH.TS | Training sequence | 24 bits | 24 bits |
| PH.TST | Transmitter settling time (Transmit power within 20% of final value, Frequency stable to within  1.0 kHz of final value) |  1.0 ms |  1.0 ms |
| PH.TXP | Transmit output power ( | 1 Watt | 12.5 Watt |

In addition, the constants of the physical layer of the AIS Simplex Repeater station should comply with the values given in the Tables 6 and 7.

- Required Settings of Physical Layer Constants (part 2)

| Symbol | Parameter name | Value |
| --- | --- | --- |
| PH.DE | Data encoding | NRZI |
| PH.FEC | Forward error correction | Not used |
| PH.IL | Interleaving | Not used |
| PH.BS | Bit scrambling | Not used |
| PH.MOD | Modulation | Bandwidth adapted; GMSK/FM |

- Bandwidth dependent parameters of the Physical Layer of the AIS Simplex Repeater Station

| Symbol | Parameter name | PH.CHB |
| --- | --- | --- |
| PH.TXBT | Transmit BT-product | 0.4 |
| PH.RXBT | Receive BT-product | 0.5 |
| PH.MI | Modulation Index | 0.50 |

# Requirements and recommendations for the TDMA Receivers of the AIS Simplex Repeater Station

Table 7 ITU-R M.1371 should apply to the TDMA receivers.  They may be subject to further regional requirements (Table 9).

- Required receiver characteristics  for Type 1

| Receiver Parameters | 25kHz channels |
| --- | --- |
| Sensitivity | 20% PER for –107 dBm |
| Co-channel rejection | -10 – 0 dB |
| Adjacent channel selectivity | 20 % PER @70 dB |
| Spurious response rejection | 20 % PER @70 dB |
| Intermodulation response rejection and Blocking | 20 % PER @74 dB |

- Required receiver characteristics  for Type 2

| Receiver Parameters | 25kHz channels |
| --- | --- |
| Sensitivity | 20% PER for –107 dBm (*) |
| Co-channel rejection | -10 – 0 dB |
| Adjacent channel selectivity | 20 % PER @70 dB(*) |
| Spurious response rejection | 20 % PER @70 dB(*) |
| Intermodulation response rejection and Blocking | 20 % PER @ 65 dB(*) |

(*) TBD for regional requirements

# Shutdown Procedure for an AIS Repeater station

An automatic transmitter hardware shutdown procedure and indication should be provided in case the transmitter does not discontinue its transmission within [1] second from the start of the repeated message.

# Continuous TX rule

Not more than 12 repeated messages in any UTC second

# Functional Block Diagram of an AIS Simplex Repeater Station

Figure 1 shows the functional block diagram of an AIS simplex repeater station.  The components in dotted lines or the input data in parenthesis are optional.

These functional elements of the AIS simplex repeater station are required (compulsory) in the minimum configuration of an AIS simplex repeater station:

- Two TDMA receiver (optional one receiver)

- One TDMA transmitter

- A controlling unit

- A Built-In-Integrity-Test unit (BIIT)

- A power supply

The following functional elements are optional to the AIS simplex repeater station:

- A Presentation Interface (PI): The Presentation Interface allows to output data from the  AIS simplex repeater station and to input data to the  AIS simplex repeater station.

- An internal sync source

Presentation Interface

Controller

Power

BIIT

AIS RX

AIS RX

AIS TX

Antenna system

Dedicated AIS Simplex Repeater Station

I/O

Internal Sync Source

- Functional Block Diagram of a dedicated Simplex Repeater Station

# Guidance for the presentation of repeated messages in ECDIS/ENC

Guidance for the presentation of repeated messages in ECDIS/ENC should be provided.

# Performance requirements for AIS Simplex Repeater Station

Type 1:

The performance requirements for the interface to the AIS VDL shall be the same as for AIS Base Station:

- Receiver Sensitivity;

- Co-Channel Rejection;

- Adjacent Channel Selectivity;

- Spurious Response Rejection;

- Intermediation Response Rejection and Blocking;

- Transmitter Power;

- Frequency Stability;

- Slot Access.

Type 2:

AtoN based for low traffic area

Optionally the frequency coverage of an AIS Repeater Station may be declared by the manufacturer.

# Configuration of Environments / Repeaters

The following parameters should be configurable for an AIS Repeaters station:

- Operational Channel;

- Transmitter Output Power;

- Channel Load Threshold;

- Filtering parameters;

- Coverage Area;

- Reporting Rate;

- Message Type;

- Access Scheme;

- Repeat Indicator (number of repeats);

- Repeat decision.